3GPP TSG SA WG4#126	S4-231694

Chicago, United States, 13 – 17 November 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | pseudo | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Video Capabilites for MeCAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 06/11/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Media Capabilities for video were underspecified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defines the capabilities for Video aligned with the PD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7, 10.2.3, 10.3.3, 10.4.3, 10.5.3, 10.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | For audio, please refer to S4-231672. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[3]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[4]	3GPP TR 26.857: "5G Media Service Enablers".

[5]	Khronos, "The OpenXR Specification", https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html.

[6]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[7]	ITU-T Recommendation H.264 (08/2021): "Advanced video coding for generic audiovisual services".

[8]	ITU-T Recommendation H.265 (08/2021): "High efficiency video coding".

[9]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[10]	ITU-T Recommendation H.264 (06/2019): "Advanced video coding for generic audiovisual services".

[11]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

=====  CHANGE  =====

# 7	Visual functions and capabilities

[Ed note: eg description of video formats and codecs, same for GPU capabilities and formats]

## 7.1	Decoding capabilities

### 7.1.1	Video decoding

### The following video decoding capabilities are defined in alignment with TS 26.511 [9], clause 4.2:

-	AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [10] bitstreams.

-	AVC-UHD-Dec: the capability to decode H.264/AVC Progressive High Profile Level 5.1 [10] bitstreams for H.264/AVC with the following additional restrictions and requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-	the bitstream does not contain more than 10 slices per picture.

-	HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[11] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1[11] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-8K-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 6.1[11] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1 with the following further limitations:

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432,

-	the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

### 7.1.2	Concurrent decoding capabilities

#### 7.1.2.1	Definition

Simultaneous Video Decoding Instances are defined as follows:

-	For N video bitstreams encoded according to a video codec profile, decoding units flow into the coded picture buffer (CPB) for each stream according to a specified arrival schedule and are delivered by the common Hypothetical Stream Scheduler (HSS) that schedules the N bitstreams for decoding each of the units. For each access unit

-	all data associated with an access unit is removed and decoded instantaneously by the instantaneous decoding process at CPB removal time of the access unit.

-	Each decoded picture is placed in the Decoded Picture Buffer (DPB) for being referenced by the decoding process of this stream as well as for output and cropping.

-	A decoded picture is removed from the DPB at the time that it becomes no longer needed for inter-prediction reference as well as the output time of the access unit is the largest of all decoded pictures remaining in the group of N decoders

-	Then at any point time,

-	each of the individual streams conforms to the signaled profile/level/tier and HRD parameters of the individual stream.

-	The sum of the CPB size conforms to common profile/level/tier signaling

-	The aggregate decoder processing speed (samples per seconds) conforms to common profile/level/tier signaling.

-	The sum of the DPB size conforms to common profile/level/tier signaling

-	The common DPB size conforms to common profile/level/tier signaling

-	A set of N Simultaneous Video Decoding Instances conforms to a common profile/level, if a set of up to N bitstreams encoded to be decodable by the HRD above, is decodable within the timing limits.

In addition, the simultaneous decoding of different video codecs is defined.

#### 7.1.2.2	Relevant Capabilities

Based on this definition, the following Simultaneous Video Decoding Instances are defined

-	AVC-UHD-Dec-4: The capability of supporting up to N=4 Simultaneous Video Decoding Instances with the aggregate capabilities of AVC-UHD-Dec

-	HEVC-UHD-Dec-4: The capability of supporting up to N=4 Simultaneous Video Decoding Instances with the aggregate capabilities of HEVC-UHD-Dec

-	UHD-Dec-4: The capability supporting up to N=4 Simultaneous Video Decoding Instances with either

-	the aggregate capabilities of AVC-UHD-Dec-4, or

-	the aggregate capabilities of HEVC-UHD-Dec-4, or

-	the capability of decoding up to 4 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec

-	HEVC-8K-Dec-8: The capability of supporting up to N=8 Simultaneous Video Decoding Instances with the aggregate capabilities of HEVC-UHD-Dec

-	8K-Dec-8: The capability supporting up to N=8 Simultaneous Video Decoding Instances with either

-	the aggregate capabilities of AVC-UHD-Dec-4, or

-	the aggregate capabilities of HEVC-8K-Dec-8, or

-	the capability of decoding up to

-	8 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec or,

-	4 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-UHD-Dec or HEVC-UHD-Dec

## 7.2	Encoding capabilities

### 7.2.1	Video encoding

The following video encoding capabilities are defined in alignment with TS 26.511 [9], clause 4.3:

-	AVC-FullHD-Enc: the capability to encode a video signal with:

-	up to 245,760 macroblocks per second;

-	up to a frame size of 8,192 macroblocks;

-	up to 240 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bit;

to a bitstream that is decodable by a decoder that is AVC-FullHD-Dec capable as defined in clause 7.1.1

-	HEVC-FullHD-Enc: the capability to encode a video signal with

-	up to 133,693,440 luma samples per second;

-	up to a luma picture size of 2,228,224 samples;

-	up to 240 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 7.1.1.

-	HEVC-UHD-Enc: the capability to encode a video signal with

-	up to 534,773,760 luma samples per second;

-	up to a luma picture size of 8,912,896 samples;

-	up to 480 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 7.1.1.

## 7.3	Scene processing capabilities

## 7.4	Capability exchange

=====  CHANGE  =====

### 10.2.3	Media capabilities support

#### 10.2.3.1	Video

A MeCAR Client of device type 1 shall support decoding capabilities as defined in clause 7.2.1

-	the AVC-UHD-Dec decoding capability.

-	the HEVC-UHD-Dec decoding capability.

-	the AVC-UHD-Dec-4 decoding capability.

-	the HEVC-UHD-Dec-4 decoding capability.

-	the UHD-Dec-4 decoding capability.

A MeCAR Client of device type 1 may support decoding capabilities as defined in clause 7.2.1

-	the HEVC-8K-Dec decoding capability.

-	the 8K-Dec-8 decoding capability.

A MeCAR Client of device type 1 shall support encoding capabilities as defined in clause 7.3.1

-	the AVC-FullHD-Enc encoding capability.

-	the HEVC-FullHD-Enc encoding capability.

A MeCAR Client of device type 1 should support encoding capabilities as defined in clause 7.3.1

-	the HEVC-UHD-Enc encoding capability.

=====  CHANGE  =====

### 10.4.3	Media capabilities support

#### 10.4.3.1	Video

### A MeCAR Client of device type 2 shall support all capabilities defined in clause 10.3.3.1.

=====  CHANGE  =====

### 10.5.3	Media capabilities support

#### 10.5.3.1	Video

A MeCAR Client of device type 2 shall support all capabilities defined in clause 10.4.3.1.

=====  CHANGE  =====

### 10.6.3	Media capabilities support

#### 10.6.3.1	Video

A MeCAR Client of device type 2 shall support all capabilities defined in clause 10.5.3.1.