Title:		         Updates of test for stereo UE in ATIAS Pdoc

Source:	Beijing Xiaomi Mobile Software Co., Ltd

Document for:	Discussion & Agreement

Agenda Item:	7.6

- Introduction

This proposal is to update chapter 4.8 stereo test in ATIAS Pdoc

- Content

……

[

The following method(s) have been incorporated from [5]:

## Spatial perception test for stereo UE in ATIAS

### Definition

interchannel level difference

The interchannel level difference is the sound level of the left channel minus the right channel.

interchannel time difference

The interchannel time difference is the times-of-arrival of the sounds of the left channel minus the right channel.

Central direction:

To create a central direction, interchannel time difference and interchannel level difference should small to avoid noticeable location shift.

Left\Right direction:

The left and right channels should have sufficient difference to make sound images located on the left or right. If the sound source comes from the left direction, the interchannel time difference<0 and\or interchannel level difference >0 in general and vice versa.

### Test Conditions

Free-field propagation conditions

-	The test environment shall contain a free-field volume, wherein free-field sound propagation conditions shall be observed.

-	The free-field sound propagation conditions shall be observed down to a frequency of 200Hz

### Test Configurations

Test signal：

Speech signal according to ITU-T Recommendation P.501 is used.

Editor’s Note:  The influence of processing like echo cancel on stereo audio is still unclear. It should be careful about the differences caused by processing.

Sound source:

HAT and coaxial loudspeaker.

Editor’s Note: Since the UE is most used for speech service, and avoid phase different cause by x-way loudspeaker.

#### Setup for terminals

The setup is referred to TS 26.260 and TS 26.132[2-3]. Including the POI, reference point, etc.

Where the manufacturer gives conditions of use, these will apply for testing. If the manufacturer gives no other requirement, the DUT will be positioned according the reference usage of hand-held hands-free UE in TS 26.132 describing in the following block:

If HATS measurement equipment is used, it shall be configured to the hand-held hands-free UE according to figure 4. The HATS should be positioned so that the HATS Reference Point is at a distance dHF from the centre point of the visual display of the Mobile Station. The distance dHF is specified by the manufacturer. A vertical angle HF may be specified by the manufacturer. Where it is not specified, the nominal distance dHF shall be 42 cm and HF shall be 0º.

Figure 4: Configuration of hand-held hands-free UE relative to the HATS

Measurement points:

Figure 1: Audio capture block diagram for sending direction measurements

Editor’s Note: The test should represent what sound the user will get. Hence, the test operator doesn't need to calibrate the DUT. The result should include all the deviations between components in one device (like the sensitivity difference between a microphone array used in DUT) and deviations between different manufactured batches.

#### Measurement method

- The UE device under test is mounted in the free-field volume such that its reference point is on the axis of the sound source.

Repeat steps b-c) at -90, -30, 0, 30,90 degree :

- The sound source pointed directly toward the reference point of the DUT, measuring interchannel level difference and interchannel time difference.

- Change the angle between sound source and DUT.

#### Interchannel time difference(ITD)

- band-pass filter was applied for the frequency range of 200 – 2000 Hz. The interchannel time difference is determined by cross-correlation analysis between the left channel signal and right channel signal at the electrical access point.

- The cross-correlation function F(τ) between the left channel signal SL(t) and the right channel signal SL(t) at the electrical reference interface is calculated in the time domain.

- The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

#### Interchannel level difference(ILD)

- The left and right channels are aligned in time based on ITD.

- Band-pass filter was applied for the frequency range of 200 – 16000 Hz.

- The level difference between left and right channels are calculated:

ILD=10log(ELeftERight)

#### Sound image (SI)

- Stereo audio only need to consider the left ,center and right direction , Elevation and rear positioning are not essential for stereo. There for the measurement only carry out on 0,±30,±90.

- Sound image indicates the perception location of the image. A sound image value less than zero indicates a position on the left side, while a value greater than zero signifies the right side. An sound image value of exactly zero represents the precise center.

- SI is a function of ITD and ILD as following:

#### SI=fILD,ITD

NOTE: The specific formula is tbd.

| Source azimuth[deg] | requirement |
| --- | --- |
| -90 | ITD>-35ms*1 SI<-[tbd]*2 |
| -30 | ITD<-35ms and SI<-[tbd]*3 |
| 0 | -0.1ms< ITD<0.1ms and -1 dB< ILD<1 dB*4 |
| 30 | ITD<35ms and SI>[tbd] |
| 90 | ITD<35ms SI>[tbd] |

*1: The ITD should less than the threshold of Hass effect.

*2: The sound from ±90 degree should have the sound image close to the edge.

*3: The sound from ±30 degree should have a clearly distinguish between the center positions.

*4: The sound from 0 degree should be precepted as center.

*5: The sound from left and right do not require the same performance, but should have the right direction

interchannel time difference =Delayleft-Delayright

interchannel level difference=Levelleft-Levelright

]

……

- Measurement data

We chose a mobile phone on market that can capture stereo audio for recording, we collected the test data and analysed the spatial perception.

## 3.1 Setup

Audio interface: ADI-2 DAC FS

Sound source: B&K TYPE 5128 HAT

Test sequence: 4 English test sentences (2 female, 2 male) according to ITU-T P.501 is used.

## 3.2 Result

### 3.1.1 ITD (ms)

|  | -90 deg | -30 deg | 0 deg | 30 deg | 90 deg |
| --- | --- | --- | --- | --- | --- |
| female 1.wav | -0.71 | -0.21 | 0.00 | 0.25 | 0.65 |
| female 2.wav | -0.71 | -0.21 | 0.00 | 0.29 | 0.65 |
| male 1.wav | -0.73 | -0.21 | 0.00 | 0.27 | 0.69 |
| male 2.wav | -0.71 | -0.23 | 0.00 | 0.27 | 0.69 |

### 3.1.2 ILD (dB)

|  | -90 deg | -30 deg | 0 deg | 30 deg | 90 deg |
| --- | --- | --- | --- | --- | --- |
| female 1.wav | 3.94 | 3.37 | -0.03 | -3.38 | -3.61 |
| female 2.wav | 3.35 | 1.91 | -0.02 | -2.12 | -3.72 |
| male 1.wav | 4.19 | 2.93 | -0.03 | -3.42 | -4.26 |
| male 2.wav | 3.86 | 2.72 | -0.01 | -2.97 | -3.88 |

Based on subjective perception, sound images from ±90 degrees are precepted near the edge, sound images  from ±30 degrees have a clear distinction between centre positions, and sound images from 0 degree are perceived as cantered.

In this test, ITD and ILD have very regular features:

ITD-90<ITD-30< ITD0<ITD30<ITD90

ILD-90>ILD-30> ILD0>ILD30>ILD90

The value of ITD0 and ILD0 is very close to 0	which aligns with the effects of ITD and ILD on spatial perception.

- Conclusion

Chapter 2 of this proposal offers an updated version of ATIAS Pdoc Chapter 4.8, and Chapter 3 supplies the relevant test data derived from the applied testing methodology.

## References

- S4-231418_Pdoc ATIAS-1 v0.5.0

- S4-230189: Add the spatial perception test for stereo UE in ATIAS, Xiaomi