3GPP TSG-SA WG4 Meeting #126	S4-231739

# Chicago, USA, 13 – 17 November 2023

Source:	NTT

Title:	[FS_eiRTCW] Pseudo-CR on Solution #2 for functional requirements for C-Plane

Spec:	3GPP TR 26.930

Agenda item:	10.9

Document for:	Approval

1. Introduction

Solution#2 was agreed in FS_eiRTCW Permanent Document v600.

2. Reason for Change

Solution#2 needs to be incorporated in TR 26.930 based on the agreement in FS_eiRTCW PD.

3. Proposal

It is proposed to agree on the following changes to 3GPP TR 26.930.

In incorporating the description of FS_eiRCTW PD in TR 26.930, the following modifications are deployed.

-	Correction of editorial errors. (e.g., replacement of capital letter with small letter and refining the words)

-	Adding the following EN in clause 6.3.1

Editor’s Note:	The description of this clause will be aligned with Key Issue #5 and corresponding solution as needed.

* * * First Change * * * *

6.3	Solution #2: Functional requirements for C-Plane

6.3.1	Solution description

This solution addresses Key Issue #2.

This clause identifies requirements for control plane (C-Plane) signalling for WebRTC-based RTC session management supporting inter-operator connection (i.e., collaboration scenario 3 and 4 in 3GPP TS 26.506 [XX]) based on the architecture described in clause 6.2.

Figure 6.3.1-1 shows the C-Plane reference points on the eiRTCW architecture. RTC-4s an RTC-9s are focussed reference points of this study as described in clause 6.2.7.

Figure 6.3.1-1:	Reference points for C-Plane

To support both collaboration scenario 3 and collaboration scenario 4, the functional requirements for RTC-4s on the eiRTCW architecture is required to conform to those specified in 3GPP TS 26.506 [XX] clause 4.3.3.

- RTC-4s supports the exchange of signalling information related to the WebRTC session between two or more WebRTC endpoints using trusted application servers.

To support collaboration scenario 4, RTC-9s is introduced in this study as a new C-Plane interface for signalling information exchange between different operator's networks or between an operator and service provider networks.

Editor’s Note:	The description of this solution will be updated based on the study on Key Issue #5 as needed.

6.3.2	Functional requirements for C-Plane interface

6.3.2.1	General

This clause describes the functional requirements for C-Plane interface to enable WebRTC-based RTC media session management supporting inter-operator connection. The requirements are considered based on following aspects:

1.	Support of WebRTC based RTC services (general aspect);

2.	Transport of signalling message; and

3.	Media session control and management.

6.3.2.2	Functional requirements for support of WebRTC based RTC services

This clause identifies the functional requirements on C-Plane signalling protocol to support WebRTC based RTC services.

1.	It is required to support any WebRTC application (i.e., it should not overfit for a specific use case.).

a.	It is required to support any kind of WebRTC endpoints (e.g., browser).

b.	It is required to allow application specific methods and information elements.

2.	It is required to be web-friendly to support easy deployment in web environments:

a.	by using web technologies such as JSON, etc…

b.	complying with WebRTC standards (e.g., SDP for session description and supporting the exchange of ICE candidates, etc…) defined in IETF and W3C, with an exception for codecs.

3.	It is required to be able to be simple to implement and deploy (e.g., simpler in complexity compared to SIP).

4.	It is required to be able to authenticate and authorise the UE using RTC services.

5.	It is required to protect user privacy and mitigate the linkability and tracking attack caused by unnecessary user information disclosure.

6.3.2.3	Functional requirements for transport of signalling message

This clause identifies the functional requirements on transport of signalling message.

1.	It is required to be web-friendly to support easy deployment in web environments by using web technologies such as WebSocket, etc…

2.	It is required to support the secure exchange of messages supporting integrity-protection and/or encryption.

3.	It is required to be support connection management mechanisms (e.g., keep alive) for reliable exchange of signalling messages.

6.3.2.4	Functional requirements for media session control and management

This clause identifies the functional requirements on media session control and management aspects of signalling message.

1.	It is required to support following methods for media session control.

i.	media session set up

ii.	media session update

iii.	media session disconnection

2.	It is required to support a method for querying information from a connected network. The information includes the service configuration information such as server address.

3.	It is required to be able to set up a media session with any kind of media resources (e.g., WebRTC endpoint on the UE, conference, metaverse).

4.	It is required that an WebRTC endpoint is able to set up multiple media sessions simultaneously.

5.	It is required to support incoming call set up (i.e., be able to receive a media session set up request).

6.	It is required to be able to set up a media session with media resources in different operator network or RTC application provider network. This requirement includes the following cases.

i.	The connected network support RTC AS functionalities. (i.e., connected via NNI)

ii.	The connected network does not support RTC AS functionalities. (i.e., connected via UNI)

7.	It is required to be familiar with existing web-services to exchange media capabilities. It is also required that WebRTC endpoints can notify own media capabilities to a network, and network can handle the notified media capability appropriately.

8.	It is required to support a mechanism to exchange media session related meta data.

9.	It is required to support QoS control of a media session based on the information included in the signalling message related to the media session.

10.	It is required to be ablet to negotiate the use of optional features.

11.	It is required to support the mechanisms for reliable media session management. (e.g., error handling).

12.	It is required to be able to identify the RTC service user uniquely. The identity of the user is able to be associated with multiple devices (WebRTC endpoints) belongs to the user.

13.	It is required to be able to enable communicating parties to verify each other's identity, if required by application.

6.3.3	Protocol stack for C-Plane interface

6.3.3.1	General

This clause studies the appropriate protocol stack for C-plane interfaces, considering the requirements in clause 6.3.2. Especially, the following requirements are considered:

-	It shall support the secure exchange of messages supporting integrity-protection and/or encryption.

-	It shall protect user privacy and mitigate the linkability and tracking attack caused by unnecessary user information disclosure.

-	It should be web-friendly to support easy deployment in web environments

-	by using web technologies such as JSON, WebSockets, etc…

-	complying with WebRTC standards (e.g., SDP for session description and supporting the exchange of ICE candidates) defined in IETF and W3C, with an exception for codecs

-	It shall be simple to implement and deploy (e.g., simpler in complexity compared to SIP).

6.3.3.2	Base Protocol

HTTP (IETF RFC 9110 [XX], IETF RFC 9111 [XX], IETF RFC 9112 [XX], IETF RFC 7235 [XX], and IETF RFC 1113 [XX])/HTTPS and WebSocket (IETF RFC 6455 [XX]) are available options for signalling between UE and WSF so that connection setup procedure could be invoked by JavaScript API as described in clause 3 of IETF RFC 8825 [XX]. Nevertheless, HTTP/HTTPS is less appropriate for two reasons described in clause 1.1 of IETF RFC 6455 [XX]:

-	Server load caused by http transactions (based on request-response)

-	A connection has two sessions each for sending and receiving signalling packets

In addition, when a notification from the network to the UE is required, for such as an incoming call, an HTTP(S) connection is originated from the network side, but this case has some problem. Generally, NAT box is placed between UE and network entities, therefore NAT-traversal problem should be resolved. Besides, in terms of security configuration, UEs often deny incoming TCP (IETF RFC 793 [XX]) connections.

WebSocket fulfils the requirement for secure exchange of signalling messages since WebRTC supports the secure transport over TLS.

For those reasons mentioned above, only WebSocket over TLS is utilized as the base protocol for transport of signalling messages in this study. WebSocket can solve the three problems, server load, number of sessions and the NAT-traversal.

6.3.3.3	Upper Layer Protocol over WebSocket

In IETF RFC 8825 [XX], upper layer protocols over WebSocket are not specified and are thought to be application specific. In the IETF RFC 8825 [XX], SIP (IETF RFC 3261 [XX]) and XMPP (IETF RFC 6120 [XX]) are listed as candidate protocols for C-Plane signalling.

6.3.3.3.1	SIP

Utilizing SIP for C-Plane signalling for WebRTC is already described in clause 5 of 3GPP TS 24.371 [XX]. One of the main advantages of using SIP is the ease of interworking between WebRTC-aware network and IMS network. On the other hand, disadvantages of using SIP are as follows:

-	UE and network must be able to understand both WebRTC and SIP. SIP is not widely used outside of telephony. If SIP must be used in conjunction with WebRTC, the advantage of WebRTC, friendliness to web-based development environments and developers, is to be spoiled.

-	SIP has a strictly managed communication model as SIP dialogue. In principle, the originated signalling is transparently relayed through the network and the terminals manage the dialogue with each other. These characteristics are not compatible with the UE-network relation model, which is the scope of this study.

-	SIP specifies methods divided by each signalling characteristic (i.e., INVITE, ACK, BYE, CANCEL, PRACK, UPDATE, SUBSCRIBE, NOTIFY, REFER, PUBLISH, INFO). Adding control for a new characteristic may need to start from the method definition.

-	Less affinity with cloud environment where HTTP is mainly used. For example, raw values of the IP addresses related to the SIP dialog (consisting a communication path of SIP trapezoid) are in the protocol header or message body, therefore changing communication elements is difficult once the call session is established.

For those reasons above, SIP is not appropriate except for the applications where the interwork to IMS is expected.

6.3.3.3.2	XMPP

There is no specification using XMPP for the upper layer protocol of the C-Plane signalling in 3GPP and no major commercial implementations of WebRTC either. The reason seems that XMPP can be used on its own and does not need to be combined with other protocols. WebSocket encapsulation of XMPP has little benefit except the case that an application using XMPP is implemented using JavaScript.

Therefore, this study will not utilize SIP nor XMPP as the upper layer protocol. More optimal (or WebRTC native) signalling protocols for the upper layer of C-Plane is to be identified in this study.

6.3.3.3.3	Other existing implementations

Among the existing implementations of WebRTC communication, JSON (IETF RFC 8259[XX]) format is mainly used for the upper layer of C-Plane. This is because JSON format is easy to handle in JavaScript. In this study, the potential of JSON for the upper layer protocol of C-Plane signalling is investigated.

In 3GPP specifications, RESTful APIs (such as service-based interface and Northbound APIs) are often defined using OpenAPI 3 (OpenAPI [XX]) and the message-body of the APIs are based on JSON. However, OpenAPI is mainly suitable for RESTful APIs and not suitable for message-driven APIs such as C-Plane signalling over WebSocket. There is another possible API specification for JSON based API. AsyncAPI [XX] (managed by Linux Foundation) is a message/event-driven architecture concept and familiar with message-driven API. For this reason, AsyncAPI [XX] is used for identifying API schemas in this study.

6.3.3.4	Proposed Protocol Stack

The protocol stack for C-Plane interface is shown in Figure 6.3.4.4-1. As described above, JSON based protocol over WebSocket over TLS is an expected solution for C-plane signalling protocol.

WebSocket can be deployed over several versions of HTTP.

-	WebSocket with HTTP/1.1 is specified in IETF RFC 6455 [XX] and used in this study. HTTP/1.1 is not, however, shown in the protocol stack because HTTP/1.1 does not remain after upgrading into WebSocket.

-	WebSocket with HTTP/2 is specified in IETF RFC 8441 [XX] and used in this study. HTTP/2 is shown in the protocol stack because HTTP/2 framing remains after a stream in HTTP/2 connection is upgraded into WebSocket.

-	WebSocket with HTTP/3 (IETF RFC 9114 [XX]) is specified in IETF RFC 9220 [XX] but not used in the current version of this study. The transport protocol used over HTTP/3 needs to be selected in alignment with IETF/W3C discussions.

The sub layers of each protocol are according to the existing specifications.

-	TLS under HTTP/1.1 and HTTP/2 is specified in IETF RFC 8446 [XX].

-	TCP under TLS is specified in IETF RFC 793 [XX].

-	IPv4 and v6 under TCP are specified in IETF RFC 791 [XX](IPv4) and IETF RFC 8200 [XX](IPv6).

Figure 6.3.3.4-1:	Protocol Stack of C-Plane interface

6.3.4	Solution evaluation

There is no misalignment between the functional requirements proposed in clause 6.3.2 and 6.3.3, and those specified in 3GPP SA4 RTC specifications (i.e., 3GPP TS 26.506 [XX] and 3GPP TS 26.113 [XX]), therefore it is proposed to develop the C-Plane signalling protocol based on the proposed functional requirements and protocol stack.

* * * End of Changes * * * *