Source:	Samsung Electronics Co., Ltd.

Title:	pCR on requirements for ARMTSI clients

Document for:	Agreement

Agenda Item:	10.6

- Introduction

AR-MTSI client will inherit most of requirements for MTSI client as it is defined as a DCMTSI client supporting AR capabilities as defined by TS 26.264. This contribution propose to remove some clauses in TS 26.264 related to general requirement applied to both AR-MTSI and MTSI clients.

- Proposed changes

  - Proposed change 1

## 4.1	General

Typical conversational AR scenarios as envisioned in this document consist of an immersive AR call that may include the following conversational components:

-	Real-time speech/audio that can comprise mono, stereo, and/or spatial audio.

-	Real-time 2D video or 360-degree video that can be rendered as rectangular or spherical overlay in the AR experience.

-	A real-time volumetric video of the user or an object that can be rendered in AR or MR.

In addition to the above conversational media, non-real-time objects may be exchanged over the data channel as well.

Both two-party and multiparty calls are possible. The AR experience may be unidirectional, i.e., only one party receives AR media and renders it, or it may be bidirectional, i.e., both parties receive and transmit AR media. The term AR-MTSI client includes both:

-	an AR-MTSI client in terminal which is an AR device as defined in [3] e.g., AR glasses, phone, Head Mounted Display (HMD) that has an XR Runtime for rendering an AR experience.

-	AR MRF that provides support for AR conversational services.

As an AR-MTSI client in terminal is a DCMTSI client in terminal with additional features for AR communication, the following requirements for a MTSI client terminal also apply for an AR-MTSI client in terminal:

-	the interworking requirements in clause 12 of TS 26.114 [2],

-	the jitter buffer management requirements in clause 8 of TS 26.114 [2],

-	the packet loss handling requirements in clause 9 of TS 26.114 [2],

-	the media and rate adaptation requirements in clause 10 and 17 of TS 26.114 [2], and

-	the network preference management object in clause 15 of TS 26.114 [2],

NOTE:	If an AR-MTSI client in terminal supports functionalities for MSMTSI client in terminal as specified in Annex S of TS 26.114 [2], the media and rate adaptation requirements in Annex S.8 of TS 26.114 [2] also apply for an AR-MTSI client in terminal.

  - Proposed change 2

We propose to remove following clauses from TS 26.264 v0.2.0:

- Clause 9 Interworking

- Clause 10 Jitter buffer management

- Clause 11 Packet-lost handling

- Clause 12 Media adaptation

- Clause 13 Network preference management object

- Proposal

We propose to agree on the proposed change in clause 2 and update the TS 26.264 accordingly.