3GPP TSG-SA WG4 Meeting #126	S4-231819

# Chicago, USA, 13 - 17 November 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.966 | CR | - | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_HEVC_Profiles] Latency sensitive multiview applications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_HEVC_Profiles |  |  |  |  |  | Date: |  |  | 30/10/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | So far, a streaming use case has been focused on during the ongoing study, while there can be low-latency real time use cases for such content, for example as noted in S4-231222. This paper introduces such a low latency scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new low delay application scenario is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Low delay applications of multiview content will remain unaddressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ISO/IEC 14496-10:2022: "Information technology — Coding of audio-visual objects — Part 10: Advanced video coding"

[3]	ISO/IEC 23008-2:2015: "Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 2: High efficiency video coding"

[3]	3GPP TR 26.905: "Mobile stereoscopic 3D video".

[4]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[6]	3GPP TS 26.214: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[7]	3GPP TS 26.218: "Virtual Reality (VR) profiles for streaming applications"

[8]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs"

[9]	Vetro, Anthony. "Frame compatible formats for 3D video distribution." In 2010 IEEE International Conference on Image Processing, pp. 2405-2408. IEEE, 2010.

[10]	Hannuksela, Miska M., Ye Yan, Xuehui Huang, and Houqiang Li. "Overview of the multiview high efficiency video coding (MV-HEVC) standard." In 2015 IEEE International Conference on Image Processing (ICIP), pp. 2154-2158. IEEE, 2015.

[11]	ISO/IEC JTC1/SC29/WG11 MPEG2011 M22746, "AVC/MVC anchor coding for MFC", November 2011, Geneva, Switzerland.

[12]	ISO/IEC JTC1/SC29/WG11 N16050, "MV-HEVC Verification Test Report", San Diego, US, Feb. 2016.

[13]	ISO/IEC 14496-15:2022, "Information technology — Coding of audio-visual objects — Part 15: Carriage of network abstraction layer (NAL) unit structured video in the ISO base media file format"

[14]	"HTTP Live Streaming (HLS) authoring specification for Apple devices," https://developer.apple.com/documentation/http-live-streaming/hls-authoring-specification-for-apple-devices

[15]	"ISO Base Media File Format and Apple HEVC Stereo Video Format additions," Version 0.9 (Beta) June 21, 2023

[16]	"Apple HEVC Stereo Video," Interoperability Profile Version 0.9 (Beta) June 21, 2023

[17]	Delbracio, Mauricio, Damien Kelly, Michael S. Brown, and Peyman Milanfar. "Mobile computational photography: A tour." Annual Review of Vision Science 7 (2021): 571-604.

[18]	Camera & Imaging Products Association (CIPA) "Production, Shipment of Digital Still Camera January, January-January in 2017," 2016

[19]	"Smartphones vs Cameras: Closing the gap on image quality," https://www.dxomark.com/smartphones-vs-cameras-closing-the-gap-on-image-quality/

[20]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-I018, "Color format downconversion for test sequence generation," 2003.

[21]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-I019, "Color format upconversion for video display," 2003.

[22]	ISO/IEC 23008-12:2022: "Information technology - MPEG systems technologies - Part 12: Image File Format".

[23]	ISO/IEC 14496-12:2022: "Information technology — Coding of audio-visual objects — Part 12: ISO base media file format".

[24]	"Using HEIF or HEVC media on Apple devices," https://support.apple.com/en-us/HT207022

[25]	"HEIF Imaging," https://source.android.com/docs/core/camera/heif

[26]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[27]	3GPP TR 26.948: "Study on video enhancements in 3GPP multimedia services"

[28]	HTTP Live Streaming (HLS) Authoring Specification for Apple Devices, https://developer.apple.com/documentation/http_live_streaming/http_live_streaming_hls_authoring_specification_for_apple_devices

[29]	Samira Afzal, Vanessa Testoni, Christian Esteve Rothenberg, Prakash Kolan, Imed Bouazizi, “A holistic survey of multipath wireless video streaming”, Journal of Network and Computer Applications, 212: 103581 (2023)

[30]	ISO/IEC JTC1/SC29/WG11 N16051, "SHVC verification test report", February 2016, San Diego, USA.

[31]	ISO/IEC JTC1/SC29/WG11 N16268, "Supplemental SHVC verification test report", June 2016, Geneva, CH.

[xz]	ITU-R/Study Group 6/Document 6/33-E Draft New Recommendation ITU-R BT. "[EVP]: Subjective assessment of video quality using expert viewing protocol (EVP)", 4 February 2016, Geneva.

* * * Next Change * * * *

## 5.x	Scenario #1.2: Low delay applications of stereoscopic 3D video

### 5.x.1	Overview

While scenario #1.1 focuses the use case of streaming of stereoscopic 3D content, there are several other use cases for such content where the latency requirements are stricter compared to the lax latency requirements of the streaming use case. For example, with the advent of modern-era XR devices, video conversational applications exchange stereoscopic 3D content. Some of the other use cases may include the stereoscopic content exchange for split rendering over edge where a (partially) rendered stereoscopic view may be exchanged between the edge cloud server and the device. Such low latency applications will demand different source formats (resolutions, framerates etc.), coding settings, as well as transport considerations to cater for this lower latency requirement.

### 5.x.2	Review of previous work

The evaluation of AVC based stereoscopic 3D coding techniques done in TR 26.905 [3] was primarily focused on download and streaming scenarios. Similarly, most other normative aspects specified had been for download or streaming use cases e.g. in 3GPP DASH in TS 26.247 [3], the 3GPP file format in TS 26.244 [5], IMS in TS 26.114 [6], VR profiles in TS 26.118 [7], and MBMS in TS 26.347 [8]. Reduced resolution frame packing is not sufficient because of the detrimental impact on quality due to resampling, as noted in TR 26.905 [3]. On the other hand, using full resolution frame packing on is not as efficient as multiview based coding because of either the complete loss or limitations of using inter-layer prediction.

Hence in addition to a study on the streaming applications of stereoscopic 3D video content, realtime delivery aspects also need to be studied.

### 5.x.3	Evaluation criteria and metrics

The evaluation for the performance of coding stereoscopic 3D content for low delay applications can be done in alignment with the evaluation for streaming applications. However, low delay configurations instead of random access ones, would need to be considered. Additional criteria include:

- Assessment/discussion of hardware impact; there are two possibilities for this:

  - There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

  - There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementations is provided, or reference to existing demos etc.

- Codec performance evaluation

  - PSNR-based Rate-Distortion (RD) objective performance evaluation, where the RD performance is compared for various solutions with a fixed QP encoding setting to get the plotting data points. A better PSNR-based RD performance is preferred, keeping in view the expected hardware complexity impact.

  - Subjective performance evaluation.

### 5.x.4	Evaluation methodology

#### 5.x.4.1	Objective performance evaluation

For an objective performance evaluation, suitable source test content should be identified that is accepted by video experts as representative content. Some of the important parameters for the content include the resolution, framerate, bit depth, color subsampling, and the duration, of the content in addition to the number of views available. Reference software for a specific solution is to be used with fixed QP encoding settings to generate each plotting point on the PSNR RD curves. The encoding settings (e.g. prediction types IPP or IBB etc.) are to be decided by experts, considering the complexity and latency needs for the scenario. The resulting curves can directly be used for comparison by plotting them together with an anchor, i.e. simulcast encoding of both views, or by computiing the Bjøntegaard Delta (BD) rate metric compared to the anchor.

#### 5.x.4.2	Subjective performance evaluation

The suggested methodology for subjective performance evaluation is to follow the “Expert Viewing Protocol” (EVP) that has been used in JVET for this purpose. The EVP visual evaluation protocol is standardized in [xz] with the following main features:

- 9 experts participate as viewers in each EVP session,

- The “unimpaired” Source video Clip (SRC) is shown once, followed by two Processed Video Sequences (PVSs),

- Experts are required to compare the PVS with the SRC, and to rate them separately.

* * * End of Changes * * * *