Source:	Orange

Title:	Updated proposal for JBM performance testing

## Document for:	Agreement

## Agenda Item:	7.7

- Introduction

We extend the proposal made at SA4#125 [1] with some adjustments of draft requirements, details on profile construction and replacement of existing JBM characterization tests.

A single JBM test profile is proposed; this profile can be found in the attached text file. The PCAP recorded on CMW500 when the profile is used in HEAD acoustics Labcore is also provided for information – preliminary tests were conducted with AMR-WB and EVS calls.

- Proposal

We propose to derive a CR based on Annex A. Note that the existing characterization tests are proposed to be removed. Only NB requirements are defined here, other cases (WB, SWB, FB) can be provided if the NB example is agreeable.

## References

- 3GPP Tdoc S4-231395, Proposed changes for JBM performance testing, Source: Orange

## Annex A: Changes to TS 26.131

Change 1

### 5.12.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE1: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8bis, while meeting the speech quality targets defined.

NOTE2: The UE delay requirements for MTSI-based speech-only with LTE, NR or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8bis, and 80ms for test condition 2 of Table 8bis.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 8bis: UE delay and speech quality requirements for LTE, NR and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 190ms | TS + TR ≤ 230ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

Change 2

#### 5.12.2.1	Wired headset

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE1: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8, while meeting the speech quality targets defined.

NOTE2: The UE delay requirements for MTSI-based speech-only with LTE, NR or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8ter, and 80ms for test condition 2 of Table 8ter.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 8ter: UE delay and speech quality requirements for LTE, NR and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 190ms | TS + TR ≤ 230ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

Change 3

## 5.15	Jitter buffer management behaviour

### 5.15.0	General

The UE delay definition is the same as in clause 5.12.0.

### 5.15.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table X-NB1, while meeting the speech quality targets defined.

NOTE: See NOTE2 in clause 5.12.1 for the derivation of UE delay requirements. By profile construction, an extra delay of 100 ms is added to the delay requirement with an extra margin of 20 ms.

Table X-NB1: UE delay and speech quality requirements for LTE, NR and WLAN access

| Profile | Delay and Loss Profile | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (see Note) |
| --- | --- | --- | --- | --- |
| 0 | Error free condition with minimal jitter | Same as profile 0 in Table 8bis | Same as profile 0 in Table 8bis | No requirement, reference score MOS-LQOREF |
| 1 | artificial_volte_profile | TS + TR ≤ [270ms] | TS + TR ≤ [310ms] | MOS-LQOTEST ≥ MOS-LQOREF – [0.1] |
|  | NOTE: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

### 5.15.2	Headset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table X-NB2, while meeting the speech quality targets defined.

NOTE: See NOTE2 in clause 5.12.2.1 for the derivation of UE delay requirements. By profile construction, an extra delay of 100 ms is added to the delay requirement with an extra margin of 20 ms.

Table X-NB2: UE delay and speech quality requirements for LTE, NR and WLAN access

| Profile | Delay and Loss Profile | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (see Note) |
| --- | --- | --- | --- | --- |
| 0 | Error free condition with minimal jitter | Same as profile 0 in Table 8ter | Same as profile 0 in Table 8ter | No requirement, reference score MOS-LQOREF |
| 1 | artificial_volte_profile | TS + TR ≤ [270ms] | TS + TR ≤ [310ms] | MOS-LQOTEST ≥ MOS-LQOREF – [0.1] |
|  | NOTE: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

#### 5.15.2.1	Wired headset

#### 5.15.2.2	Wireless headset

For further study.

### 5.15.3	Electrical interface UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table X-NB3, while meeting the speech quality targets defined. The delay budget BD to be considered in the performance requirements and objectives depends on the type of electrical interface UE and is given by Table 8quater1.

NOTE: See NOTE2 in clause 5.12.3 for the derivation of UE delay requirements. By profile construction, an extra delay of 100 ms is added to the delay requirement with an extra margin of 20 ms.

Table X-NB3: UE delay and speech quality requirements for LTE, NR and WLAN access

| Profile | Delay and Loss Profile | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (see Note) |
| --- | --- | --- | --- | --- |
| 0 | Error free condition with minimal jitter | Same as profile 0 in Table 8quater2 | Same as profile 0 in Table 8quater2 | No requirement, reference score MOS-LQOREF |
| 1 | artificial_volte_profile | TS + TR ≤ [270+BD ms] | TS + TR ≤ [310+BD ms] | MOS-LQOTEST ≥ MOS-LQOREF – [0.1] |
|  | NOTE: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

## Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

End of changes

## Annex B: Changes to TS 26.132

Change 1

## 7.13	Jitter buffer management behaviour (handset, headset and electrical interface UE)

### 7.13.0	General

For speech-only with LTE, NR or WLAN access, a jitter buffer is used in receiving to handle the variation in packet receiver timing. To minimize the additional latency introduced by the jitter buffer, adaptation is used to minimize delay while preventing packet losses due to packet delivery timing variations. See clause 8 of TS 26.114 [39] for the definition of jitter buffer and minimum performance requirements on JBM.

The test method in clause 7.13.3 is used to assess the requirement on JBM performance.

### 7.13.1	Void

### 7.13.2	Void

### 7.13.3	Delay and speech quality in conditions with packet arrival time variations (handset, headset, electrical interface UE)

The test method is the same as in clause 7.10.4.

The test profiles are defined in Annex F.

Change 2 (Annex E)

Annex E (normative):
Packet delay and loss profiles for UE delay testing of MTSI-based speech with LTE access

# E.1	General

This Annex provides packet delay and loss profiles to be inserted on the IP packets in the test system when testing the UE delay and speech quality for MTSI-based speech with LTE access in jitter and error conditions. The profiles are based on the assumption that one IP packet corresponds to 20ms of the speech sequence. In order to preserve the synchronization of the elements in the profile and the timing of the speech sequence (e.g. the RTP time stamp) when the speech codec does not produce one packet every 20ms, e.g. during speech codec DTX operation, the profiles needs to be sub-sampled in accordance with the rate of production of the packets containg the speech frames.

The variation in the packet arrival time and the loss rate of the speech packets experienced by the receiving UE in an LTE connection will vary depending on several parameters and operating conditions of the LTE network. Standardized characteristics with respect to Packet Delay Budget (PDB) and Packet Error Loss Rate (PELR) for different QoS Class Identifiers (QCI) for an LTE access network are defined in 3GPP TS 23.203 [42]:

-	The PDB of a QCI defines an upper bound for the time that a packet may be delayed between the UE and the PCEF. For a certain QCI the value of the PDB is the same in uplink and downlink. The purpose of the PDB is to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). The PDB shall be interpreted as a maximum delay with a confidence level of 98 percent.

-	The PELR of a QCI defines an upper bound for the rate of SDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in E-UTRAN) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in E-UTRAN). Thus, the PELR defines an upper bound for a rate of non congestion related packet losses. The purpose of the PELR is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in E-UTRAN). For a certain QCI the value of the PELR is the same in uplink and downlink.

The stipulated PDB and PELR as defined in 3GPP TS 23.203 for QCI1 intended for Conversational Voice services results in that on each link 99 percent of the packets will be successfully delivered over an LTE air interface with a maximum delay of less than 80ms with a confidence level of 98 percent.

# E.2	Simulation model for generating packed delay and loss profiles

This clause describes a simulation model that generates packet arrival time variations and packet loss experienced by the receiving UE for MTSI-based speech with an end-to-end LTE access.

The model is derived solely for the purpose of testing the UE delay for MTSI-based speech with LTE access. As discussed in Section E.1, several LTE network parameters have a significant impact on the packet delay variations experienced by the UE. This model includes the effect of the DRX cycle, the BLER on the MAC/PHY layer, the HARQ re-transmission and RMC scheduler [46]. In addition, jitter in the EPC and the effect of the mis-alignment of the DRX-cycle between the uplink and downlink eNBs are included.

The model is described in Table E.1 and operates on the following input parameters

-	Uplink and downlink block error rate (BLER), respectively.

-	Maximum number of HARQ re-transmissions on uplink and dowblink, respectively.

-	DRX cycle length.

-	Time differnce between the uplink and the downlink eNB DRX cycle.

-	Maximum and minimum network delay between the uplink and the downlink eNB.

The random number generator used in the model produces pseudorandom values drawn from the standard uniform distribution on the open interval (0, 1).

The model generates packet delay and loss profiles for two different test system configurations of the UE delay:

-	When the system simulator is transparent in the downlink at the MAC/PHY layer and does not operate in DRX. This approach requires the variations of the delay due to downlink HARQ re-transmissions and mapping to the DRX cycle to be simulated. In this case, the model simulates the delay and jitter profiles for the packets from the antenna of the sending UE to the antenna receiving UE (end-to-end simulation). This is the testing condition used in 3GPP TS 26.132.

-	When the system simulator is configured for error insertion in the downlink at the MAC/PHY layer and DRX operation, and the system simulator implements the HARQ re-transmissions and the mapping of the packets to the DRX cycle time at the downlink. In this case, the model simulates the delay and loss profiles from the sending UE up to the receiving eNB. This testing condition is currently not used in 3GPP TS 26.132.

It should be noted that the model does not fully utilize the PDB for QCI1 as defined in 3GPP TS 23.203 and does not include temporary variations of the packet arrival time variation and the loss rate that may be experienced during e.g. hand-over or congestion. Hence, the packet delay and loss profiles generated by the model do not fully exercise the conditions that the jitter buffer management of the UE may be exposed to in LTE systems and the profiles generated by the model are only intended for the testing of the UE delay in stationary operating conditions.

Table E.1: Simulation model for generating packet delay and loss profiles for MTSI-based speech with LTE access

function [UE1_UE2_dly,UE1_eNB2_dly,plr,comp_e2e,comp] = ... VoLTEDelayProfile_vPHY(BLER_tx, BLER_rx, max_tx, max_rx, drx_cycle_length, mis_eNB1_eNB2, max_net_delay, min_net_delay, nFrames, seed) % BLER_tx : The block error rate in uplink. % BLER_rx : The block error rate in downlink. % max_tx : The maximum number of transmission attempts in uplink. % max_rx : The maximum number of transmission attempts in downlink. % drx_cycle_length : The length of the DRX cycle % mis_eNB1_eNB2 : Scheduling time mis-alignment between eNB1 and eNB2 % max_net_delay : The maximum network delay between eNB1 to eNB2 % min_net_delay : The minimum network delay between eNB1 to eNB2 % nFrames : The number of frames for the simulation % seed : Random number generator seed rng(seed); UE1_UE2_time = zeros(nFrames, 1); UE1_eNB2_time = zeros(nFrames,1); eNB1_eNB2_dly = round(min_net_delay + (max_net_delay-min_net_delay).*rand(nFrames,1)); ack1 = zeros(nFrames,1); ack2 = zeros(nFrames,1); wall_clock = 20; frame = 1; frame_size = 20; simulationTime = nFrames*frame_size; % Calculate the delay from UE1 speech encoder delivery to eNB2. If % transmission to eNB1 is not successful after max_tx attempts, dly = 0 (packet loss) while (wall_clock<=simulationTime) % Set the scheduling time if drx_cycle_length == 0 UE1_scheduling_time=wall_clock; else UE1_scheduling_time=ceil(wall_clock/drx_cycle_length)*drx_cycle_length; end % Add the tx effect for the scheduling time n=0; eNB1_receive_delay = 0; while n < max_tx if (rand(1) < BLER_tx) eNB1_receive_delay = eNB1_receive_delay+8; n=n+1; ack=0; else ack=1; n=max_tx; end end while (wall_clock<=UE1_scheduling_time) UE1_eNB2_time(frame)=ack*(UE1_scheduling_time+eNB1_receive_delay+eNB1_eNB2_dly(frame)); wall_clock=wall_clock+frame_size; ack1(frame)=ack; frame=frame+1; end; end % Translate arrival time to packet delay for UL simulation wall_clock = frame_size*(1:nFrames)'; UE1_eNB2_dly = max(-1, UE1_eNB2_time-wall_clock); % Sort for monotonic arrival time to DL for simulation [UE1_eNB2_time,monotonic_index]=sort(UE1_eNB2_time); % Calculate the delay from eNB2 to UE2 (only for those packets that % successfully arrived at the eNB2!). If transmission to UE2 is not % successful after max_tx attempts, dly = 0; (packet loss) frame = 1; UE2_scheduling_time=mis_eNB1_eNB2; while frame<=nFrames % Add the rx effect for the scheduling time n=0; eNB2_transmit_delay = 0; while n < max_rx if (rand(1) < BLER_rx) eNB2_transmit_delay = eNB2_transmit_delay+8; n=n+1; ack=0; else ack=1; n=max_rx; end end while ((frame<=nFrames)&&(UE1_eNB2_time(frame)<UE2_scheduling_time)) if (UE1_eNB2_time(frame)==-1) UE1_UE2_time(frame)=-1; else UE1_UE2_time(frame)=ack*(UE2_scheduling_time+eNB2_transmit_delay); end ack2(frame)=ack; frame=frame+1; end % Update the scheduling time UE2_scheduling_time=UE2_scheduling_time+drx_cycle_length; end % Re-order for orignal transmit order UE1_UE2_time(monotonic_index) = UE1_UE2_time; % Translate arrival time to packet delay wall_clock = frame_size*(1:nFrames)'; UE1_UE2_dly = max(-1, UE1_UE2_time-wall_clock); % Set compensation values if drx_cycle_length==0 comp_e2e=min_net_delay; comp=min_net_delay; else comp_e2e=min(UE1_UE2_dly(UE1_UE2_dly>0)); comp=min(UE1_eNB2_dly(UE1_eNB2_dly>0)); end % Calculates the overall packet loss from UE1 to UE2 pl=0; for frame=1:nFrames if UE1_UE2_dly(frame)==-1; pl=pl+1; end end plr=pl/nFrames;

# E.3	Packet delay and loss profiles for simulated stationary operating conditions with DRX 20 ms and DRX 40 ms

Three delay and loss profiles simulating the packet delay variations and packet loss that a UE may experience in stationary operating conditions with 20ms, 40ms DRX and 40ms DRX with higher BLER, respectively, are generated from the model with the parameter settings as described in Table E.2. The delay profiles are attached as text files and as binary trace files in the form of TCN network emulator.

Table E.2: Parameter settings for packet delay profile generation.

| Operating condition | Parameter setting | System simulator operation | Delay and Loss profile |
| --- | --- | --- | --- |
| Stationary DRX 20 ms | BLER_tx : 0.1 BLER_rx : 0.1 max_tx : 3 max_rx : 3 drx_cycle_length : 20 mis_eNB1_eNB2 : 10 max_net_delay : 33 min_net_delay : 27 nFrames : 8000 seed : 0 | Transparent system simulator downlink | dly_profile_20msDRX_10pct_BLER_e2e |
|  |  | System simulator generating HARQ re-transmissions and DRX operation on the downlink | dly_profile_20msDRX_10pct_BLER_ue1_to_eNB2 (Not used in 3GPP TS 26.131) |
| Stationary DRX 40 ms | BLER_tx : 0.1 BLER_rx : 0.1 max_tx : 3 max_rx : 3 drx_cycle_length : 40 mis_eNB1_eNB2 : 30 max_net_delay : 33 min_net_delay : 27 nFrames : 8000 seed : 0 | Transparent system simulator downlink | dly_profile_40msDRX_10pct_BLER_e2e (Not used in 3GPP TS 26.131) |
|  |  | System simulator generating HARQ re-transmissions and DRX operation on the downlink | dly_profile_40msDRX_10pct_BLER_ue1_to_eNB2 (Not used in 3GPP TS 26.131) |
| Stationary DRX 40 ms (22% BLER) | BLER_tx : 0.22 BLER_rx : 0.22 max_tx : 3 max_rx : 3 drx_cycle_length : 40 mis_eNB1_eNB2 : 30 max_net_delay : 36 min_net_delay : 24 nFrames : 8000 seed : 0 | Transparent system simulator downlink | dly_profile_40msDRX_22pct_BLER_e2e |

Since the model simulates the packet transmission end-to-end, the delay profiles include a packet delay component that according to the definition of the UE delay in 3GPP TS 26.131 [1] is not attributed to the UE delay. This delay shall be included in the test equipment delay when deriving the UE delay from the delay measurments based on the cross-correlation of the injected and measured signals. Values for the compensation attributed to the packet delay profiles are shown in Table E.3.

Table E.3: Packet delay profiles compensation values.

| Delay and Loss profile | Compensation value |
| --- | --- |
| dly_profile_20msDRX_10pct_BLER_e2e | 30 ms |
| dly_profile_40msDRX_10pct_BLER_e2e | 30 ms |
| dly_profile_40msDRX_22pct_BLER_e2e | 30 ms |

Change 3

# F.1	General

This Annex provides a packet delay and loss profile to be inserted on the IP packets in the test system when testing the UE delay and speech quality for speech-only with LTE, WLAN, or NR access in jitter and error conditions, to evaluate the jitter buffer management behaviour. The profiles are based on the assumption that one IP packet corresponds to 20ms of the speech sequence. In order to preserve the synchronization of the elements in the profile and the timing of the speech sequence (e.g. the RTP time stamp) when the speech codec does not produce one packet every 20ms, e.g. during speech codec DTX operation, the profiles needs to be sub-sampled in accordance with the rate of production of the packets containg the speech frames.

# F.2	Packet delay and loss profiles

Table F.1: Packet delay profiles.

| Delay and Loss profile | Packet loss (%) | Description |
| --- | --- | --- |
| dly_artificial_volte_profile | 0 | See Annex F.3. |
| NOTE: The profile is constructed based on considerations from realistic packet impairments observed on LTE. It does not include any packet loss for better disentanglement between JBM and PLC performance. |  |  |

Since the model simulates the packet transmission end-to-end, the delay profiles include a packet delay component that according to the definition of the UE delay in 3GPP TS 26.131 [1] is not attributed to the UE delay. This delay shall be included in the test equipment delay when deriving the UE delay from the delay measurments based on the cross-correlation of the injected and measured signals. Values for the compensation attributed to the packet delay profiles are shown in Table F.2.

Table F.2: Packet delay profiles compensation values.

| Delay and Loss profile | Compensation value |
| --- | --- |
| dly_artificial_volte_profile | 160 ms |

# F.3	Methodology to construct a profile based on impairments observed in live LTE operation

The following methodology has been used to construct the profile dly_artificial_volte_profile:

Step 0: identification of most sensitive active speech packets

Condition 0 from delay tests is recorded to obtain a reference PCAP file. A single packet loss is applied on each individual active speech packet of the PCAP file and the impaired PCAP file is decoded; the MOS-LQO score is measured for each packet, and the most sensitivity active speech packets are identified with respect to packet loss at a sentence level.

Step 1: set a fixed delay applied to all packets of 160 ms

Step 2: add jitter bursts

Apply an extra delay of 100 ms to 2.81% among most sensitive active speech packets in each sentence; when this extra delay of 100 ms is applied to packet N, the subsequent packets N+1, N+2, N+3 and N+4 are also delayed by respectively 80, 60, 40 and 20 ms to avoid adding packet order impairments

NOTE: There are 4 double sentences repeated 5 times,  there are 4 impaired packets in each sentence, except one sentence with 5 impaired packets because this sentence is longer than other sentences. This results in 165 impaired packets.

Step 3: add packet inversions

Select 2.81% among most sensitive active speech packets in each sentence (outside the set of packets impaired in step 2) and add an extra delay of 21 ms to simulate an inversion; if packet N is selected to be inverted, packet N+1 is not impaired.

Step 4: add duplications

Select 2.81% among most sensitive active speech packets in each sentence (outside the set of packets impaired in steps 2 and 3) and add a duplicate with an extra delay of 1 ms

Change 5 (Attachment)

Replace jbm_characterisation_profiles.zip (including current profiles) by new zip including new profile