| 3GPP TS 26.510 V0.1.0 (2023-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Media delivery; interactions and APIs for provisioning and media session handling; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Functions and roles	12

4.1	Media Application Provider	12

4.2	Media AF	12

4.3	Media Session Handler	12

5	Interactions	12

5.1	Introduction	12

5.2	Provisioning (M1) interactions	14

5.2.1	General	14

5.2.2	Content protocols discovery operations	15

5.2.2.1	General	15

5.2.2.2	Create Content Protocols resource operation	16

5.2.2.3	Retrieve Content Protocols resource operation	16

5.2.2.4	Update Content Protocols resource operation	16

5.2.2.5	Destroy Content Protocols resource operation	16

5.2.3	Provisioning Session operations	16

5.2.3.1	General	16

5.2.3.2	Create Provisioning Session resource operation	16

5.2.3.3	Retrieve Provisioning Session resource operation	17

5.2.3.4	Update Provisioning Session resource operation	17

5.2.3.5	Destroy Provisioning Session resource operation	17

5.2.4	Server Certificate provisioning operations	17

5.2.4.1	General	17

5.2.4.2	Create Server Certificate resource operation	17

5.2.4.3	Reserve Server Certificate resource operation	18

5.2.4.4	Upload Server Certificate resource operation	18

5.2.4.5	Retrieve Server Certificate resource operation	19

5.2.4.6	Update Server Certificate resource operation	19

5.2.4.7	Destroy Server Certificate resource operation	19

5.2.5	Content Preparation Template provisioning operations	20

5.2.5.1	General	20

5.2.5.2	Create Content Preparation Template resource operation	20

5.2.5.3	Retrieve Content Preparation Template resource operation	20

5.2.5.4	Update Content Preparation Template resource operation	20

5.2.5.5	Destroy Content Preparation Template resource operation	21

5.2.6	Edge Resources provisioning operations	21

5.2.6.1	General	21

5.2.6.2	Create Edge Resources Configuration resource operation	21

5.3.6.3	Retrieve Edge Resources Configuration resource operation	22

5.2.6.4	Update Edge Resources Configuration resource operation	22

5.2.6.5	Destroy Edge Resources Configuration resource operation	22

5.2.7	Dynamic Policy provisioning operations	22

5.2.7.1	General	22

5.2.7.2	Create Policy Template resource operation	23

5.2.7.3	Retrieve Policy Template resource operation	24

5.2.7.4	Update Policy Template resource operation	24

5.2.7.5	Destroy Policy Template resource operation	24

5.2.8	Content Hosting provisioning operations	24

5.2.8.1	General	24

5.2.8.2	Create Content Hosting Configuration resource operation	25

5.2.8.3	Retrieve Content Hosting Configuration resource operation	25

5.2.8.4	Update Content Hosting Configuration resource operation	25

5.2.8.5	Destroy Content Hosting Configuration resource operation	25

5.2.8.6	Purge Content Hosting cache operation	26

5.2.9	Content Publishing provisioning operations	26

5.2.9.1	General	26

5.2.9.2	Create Content Publishing Configuration resource operation	26

5.2.9.3	Retrieve Content Publishing Configuration resource operation	26

5.2.9.4	Update Content Publishing Configuration resource operation	26

5.2.9.5	Destroy Content Publishing Configuration resource operation	26

5.2.9.6	Purge Content Publishing cache operation	26

5.2.10	Metrics Reporting provisioning operations	27

5.2.10.1	General	27

5.2.10.2	Create Metrics Reporting Configuration resource operation	27

5.2.10.3	Retrieve Metrics Reporting Configuration resource operation	27

5.2.10.4	Update Metrics Reporting Configuration resource operation	27

5.2.10.5	Destroy Metrics Reporting Configuration resource operation	28

5.2.11	Consumption Reporting provisioning procedures	28

5.2.11.1	General	28

5.2.11.2	Create Consumption Reporting Configuration resource operation	28

5.2.11.3	Retrieve Consumption Reporting Configuration resource operation	28

5.2.11.4	Update Consumption Reporting Configuration resource operation	29

5.2.11.5	Destroy Consumption Reporting Configuration resource operation	29

5.2.12	Event Data Processing Provisioning operations	29

5.2.12.1	General	29

5.2.12.2	Create Event Data Processing Configuration resource operation	29

5.2.12.3	Retrieve Event Data Processing Configuration resource operation	30

5.2.12.4	Update Event Data Processing Configuration resource operation	30

5.2.12.5	Destroy Event Data Processing Configuration resource operation	30

5.3	Network media session handling (M3, M5) interactions	30

5.3.1	Introduction	30

5.3.2	Service Access Information operations	31

5.3.2.1	General	31

5.3.2.2	Create Service Access Information resource	32

5.3.2.3	Retrieve Service Access Information resource operation	32

5.3.2.4	Update Service Access Information resource operation	32

5.3.2.5	Destroy Service Access Information resource operation	32

5.3.3	Dynamic Policy operations	32

5.3.3.1	General	32

5.3.3.2	Create Dynamic Policy Instance resource operation	33

5.3.3.3	Retrieve Dynamic Policy Instance resource operation	33

5.3.3.4	Update Dynamic Policy Instance resource operation	33

5.3.3.5	Destroy Dynamic Policy Instance resource operation	34

5.3.4	Network assistance procedures and operations	34

5.3.4.1	Procedures	34

5.3.4.2	Create Network Assistance Session resource operation	35

5.3.4.3	Retrieve Network Assistance Session resource operation	35

5.3.4.4	Bit rate recommendation request operation	36

5.3.4.5	Delivery boost request operation	36

5.3.4.6	Update Network Assistance Session resource operation	36

5.3.4.7	Destroy Network Assistance Session resource operation	37

5.3.5	Metrics reporting procedures and operations	37

5.3.5.1	Procedures	37

5.3.5.2	Submit metrics report operation	38

5.3.6	Consumption reporting procedures and operations	38

5.3.6.1	Procedures	38

5.3.6.2	Submit consumption report operation	39

5.4	UE media session handling (M6, M11) procedures	40

5.4.1	General	40

5.4.2	Dynamic Policy procedures	40

5.4.3	Network Assistance procedures	40

5.4.4	Metrics reporting procedures	41

5.4.5	Consumption reporting procedures	41

6	3GPP Service URL	42

7	General aspects of APIs	42

7.1	Usage of HTTP	42

7.2.1	HTTP protocol version	42

7.2.2	HTTP resource URIs and paths	42

7.2.3	Usage of HTTP headers	42

7.2.3.1	General	42

7.2.3.2	User Agent identification	43

7.2.3.3	Server identification	43

7.2.3.2	Support for conditional HTTP GET requests	43

7.2.3.5	Support for conditional HTTP POST, PUT, PATCH and DELETE requests	43

7.2.4	HTTP message bodies for API resources	43

7.2.5	HTTP response codes	43

7.2.6	HTTP error response message bodies	43

7.3	Common API data types	44

7.3.1	General	44

7.3.2	Simple data types	44

7.3.3	Structured data types	44

7.3.4	Enumerated data types	44

7.4	Explanation of API data model notation	44

8	Provisioning (M1) APIs	45

8.1	General	45

8.2	Content protocols discovery API	45

8.3	Provisioning Sessions API	45

8.4	Server Certificates provisioning API	45

8.5	Content Preparation Templates provisioning API	45

8.6	Edge Resources provisioning API	45

8.7	Policy Templates provisioning API	45

8.8	Content Hosting provisioning API	45

8.9	Content Publishing provisioning API	45

8.10	Metrics reporting provisioning API	45

8.11	Consumption reporting provisioning API	45

8.12	Event data processing provisioning API	45

9	Network media session handling (M3, M5) APIs	45

9.1	General	46

9.2	Service Access Information API	46

9.3	Dynamic Policies API	46

9.4	Network Assistance API	46

9.5	Metrics Reporting API	46

9.6	Consumption Reporting API	46

10	UE media session handling (M6) APIs	46

10.1	General	46

10.2	Session launch mechanism	46

10.2.1	5G Media Streaming session launch mechanism	46

10.2.1.1	3GPP Service URL format for 5GMS	46

10.2.2	webRTC session launch mechanism	46

10.3	Media session handling APIs for downlink media streaming	46

10.4	Media session handling APIs for uplink media streaming	46

10.5	Media session handling APIs for webRTC	46

11	Usage of existing APIs	47

11.1	Introduction	47

11.2	Usage of 5GC service APIs	47

11.3	Usage of miscellaneous UE-internal APIs	47

Annex A (informative): Index of HTTP REST APIs	47

Annex B (normative): OpenAPI representation of HTTP REST APIs	47

Annex C (normative): Controlled vocabularies of UE data parameters	48

Annex <F> (informative): Change history	49

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

This clause shall start on a new page.

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[23502]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[26501]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[26506]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[29500]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[29501]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[OpenAPI300]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[29571]	3GPP TS 29.571: "Common Data Types for Service Based Interfaces; Stage 3".

[3]	DASH Industry Forum, "Specification of Live Media Ingest", 
https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[4]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	Standard ECMA-262, 5.1 Edition: "ECMAScript Language Specification", June 2011.

[6]	IETF RFC 6234: "US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF)".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[X.509]	ITU-T Recommendation X.509 (2005) | ISO/IEC 9594-8:2005: "Information Technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks".

[10]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[11]	IEEE Standard 1003.1, Issue 7: "The Open Group Base Specifications", 2018.
https://pubs.opengroup.org/onlinepubs/9699919799/

[13]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[15]	3GPP TS 27.007: "AT Command set for User Equipment (UE)".

[X.509]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile", May 2008.

[17]	IETF RFC 7468: "Textual Encodings of PKIX, PKCS, and CMS Structures", April 2015.

[18]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions — Part 1: Country codes".

[19]	ISO 3166-2: "Codes for the representation of names of countries and their subdivisions — Part 2: Country subdivision code".

[25]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[26]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[27]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[28]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[29]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[9]	Void.

[HTTP11]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[HTTP2]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[30]	Void.

[TLS13]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3", August 2018.

[32]	ISO/IEC 23009-1: "Information technology; Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

[33]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[34]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[35]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[37]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[38]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format", December 2017.

[39]	ISO 14496-12: "Information technology – Coding of audio-visual objects – Part 12: ISO base media file format".

[40]	ISO 23000-19: "Information technology – Coding of audio-visual objects – Part 19: Common media application format (CMAF) for segmented media".

[RFC3986]	IETF RFC 3986: "URI Generic Syntax".

[42]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[23558]	3GPP TS 23.558: "Architecture for enabling edge applications".

[24558]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[29558]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[46]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[47]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[48]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[49]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[50]	3GPP TS 29.522: "5G System. Network Exposure Function Northbound APIs; Stage 3".

[26346]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26347]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 26.501 [26501], TS 26.506 [26506] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1], TS 26.501 [26501] or TS 26.506 [26506].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

Media Delivery System: A deployment of a 5GMS System or RTC System.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AF	Application Function

AS	Application Server

EEC	Edge Enabler Client

QoE	Quality of Experience

# 4	Functions and roles

## 4.1	Media Application Provider

Editor's Note: Role of the Media Application Provider in provisioning media features.

## 4.2	Media AF

Editor's Note: Role of the Media AF.

## 4.3	Media Session Handler

Editor's Note: Role of the Media Session Handler.

# 5	Interactions

## 5.1	Introduction

Table 5.1-1 summarises the APIs used to provision and use the various Media Delivery features specified in TS 26.501 [26501] and TS 26.506 [26506].

Table 5.1-1: Summary of APIs relevant to downlink media streaming features

| Media delivery feature | Abstract | Reference point | Operations clause | Relevant APIs |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | API name | API clause |
| Content protocols discovery | Used by the Media Application Provider to interrogate which content ingest protocols are supported by the Media Delivery Systerm. | M1 | 5.2.2 | Content Protocols Discovery API | 8.2 |
| Content hosting | Content is ingested, hosted and distributed by the Media AS according to a Content Hosting Configuration associated with a Provisioning Session. The Media AS may be instantiated in an Edge Data Network. Ingested content may additionally be distributed via eMBMS and/or MBS. | M1 | 5.2.3 | Provisioning Sessions API | 8.3 |
|  |  |  | 5.2.4 | Server Certificates provisioning API | 8.4 |
|  |  |  | 5.2.5 | Content Preparation Templates provisioning API | 8.5 |
|  |  |  | 5.2.6 | Edge Resources provisioning API | 8.6 |
|  |  |  | 5.2.7 | Policy Templates provisioning API | 8.7 |
|  |  |  | 5.2.8 | Content Hosting provisioning API | 8.8 |
|  |  | M5 | 5.4.2 | Service Access Information API | 9.2 |
| Content publishing | Content is contributed to and egested from the Media AS according to a Content Hosting Publishing associated with a Provisioning Session. The Media AS may be instantiated in an Edge Data Network. | M1 | 5.2.3 | Provisioning Sessions API | 8.3 |
|  |  |  | 5.2.4 | Server Certificates provisioning API | 8.4 |
|  |  |  | 5.2.5 | Content Preparation Templates provisioning API | 8.5 |
|  |  |  | 5.2.6 | Edge Resources provisioning API | 8.6 |
|  |  |  | 5.2.7 | Policy Templates provisioning API | 8.7 |
|  |  |  | 5.2.9 | Content Publishing provisioning API | 8.9 |
|  |  | M5 | 5.4.2 | Service Access Information API | 9.2 |
| Dynamic Policy instantiation | The Media Client activates different traffic treatment and charging policies, including Background Data Transfer, selected from a set of Policy Templates provisioned in its Provisioning Session. | M1 | 5.2.3 | Provisioning Sessions API | 8.3 |
|  |  |  | 5.2.7 | Policy Templates provisioning API | 8.7 |
|  |  | M5 | 5.4.2 | Service Access Information API | 9.2 |
|  |  |  | 5.4.3 | Dynamic Policies API | 9.3 |
| Network Assistance | The Media Client requests bit rate recommendations and delivery boosts from the Media AF. | M5 | 5.4.2 | Service Access Information API | 9.2 |
|  |  |  | 5.4.4 | Network Assistance API | 9.4 |
| Metrics reporting | The Media Client submits metrics reports to the Media AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1 | 5.2.3 | Provisioning Sessions API | 8.3 |
|  |  |  | 5.2.10 | Metrics Reporting provisioning API | 8.10 |
|  |  | M5 | 5.4.2 | Service Access Information API | 9.2 |
|  |  |  | 5.4.5 | Metrics Reporting API | 9.5 |
| Consumption reporting | The Media Client submits consumption reports to the Media AF about content consumed from downlink media delivery sessions according to a provisioned Consumption Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1 | 5.2.3 | Provisioning Sessions API | 8.3 |
|  |  |  | 5.2.11 | Consumption Reporting provisioning API | 8.11 |
|  |  | M5 | 5.4.2 | Service Access Information API | 9.2 |
|  |  |  | 5.4.6 | Consumption Reporting API | 9.6 |
| UE data collection, reporting and exposure | UE data related to media delivery is reported to the Data Collection AF instantiated in the Media AF for exposure to Event consumers. | M1 | 5.2.12 | Event Data Processing provisioning API | 8.12 |
|  |  | R2, R4 | 5.6 | Ndcaf_DataReporting service | 12 |
|  |  | R5, R6 | 5.7 | Naf_EventExposure service | 13 |

## 5.2	Provisioning (M1) interactions

### 5.2.1	General

A Media Application Provider may use the operations in this clause to provision the different features offered by the Media Delivery System in the Media AF.

-	For downlink media delivery, these sessions may be DASH streaming sessions, progressive download sessions, background download sessions, or any other type of media streaming or distribution (e.g., HLS) sessions.

-	For uplink media delivery, the content format and delivery protocol are defined by the Media Application Provider, and may be either non-fully standardized or employ standardized HTTP-based streaming of ISO BMFF content fragments as profiled by CMAF [39].

The Provisioning API exposed by the Media AF to the Media Application Provider at reference point M1 offers the following sets of operations:

1.	Discovery of the set of content protocols supported by the Media AS (see clause 5.2.2):

-	For downlink media delivery, discovery of the content ingest protocols available at reference point M2 and the content distribution protocols available at reference point M4.

-	For uplink media delivery, discovery of the content contribution protocols available at reference point M4 and the content egest protocols available at reference point M2.

2.	Provisioning of Provisioning Sessions (see clause 5.2.3) to act as an umbrella for the following provisioning information. Each such Provisioning Session is uniquely identified by a system-dependent Provisioning Session identifier as well as by system-independent service identifier that is subsequently used by a Media-Aware Application to launch media session handling (see clause 10.2) via a 3GPP Service URL (see clause 6).

3.	Provisioning of Server Certificates within the scope of a Provisioning Session (see clause 5.2.4) to be used by the Media AS to assert its identity to the Media Access Function in Media Clients during media delivery sessions at reference point M4.

4.	Provisioning of Content Preparation Templates within the scope of a Provisioning Session (see clause 5.2.5) that can be used by the Media AS to manipulate media content ingested at reference point M2 or contributed at reference point M4.

5.	Provisioning of Edge Resources within the scope of a Provisioning Session (see clause 5.2.6) to be used to instantiate the Media AS as an Edge Application Server (EAS) in an Edge Data Network (EDN) using the APIs specified in TS 29.558 [29558].

5.	Provisioning of Policy Templates within the scope of a Provisioning Session (see clause 5.2.7) that can be applied to M4 downlink/uplink media delivery sessions in order to realise different Service Operation Points as part of the Dynamic Policies feature (see clause 5.4.3).

7.	Provisioning of media delivery by the Media AS within the scope of a Provisioning Session using the abovementioned building blocks:

-	For downlink media delivery, provisioning of the Content Hosting feature of the Media AS (see clause 5.2.8), which offers functionality equivalent to that of a public Content Delivery Network (CDN): content ingest at reference point M2 for onward distribution by the Media AS to Media Clients via reference point M4 or via other distribution systems such as eMBMS or MBS.

After discovering the set of ingest and distribution content protocols supported by the Media AS (see clause 5.2.2), the Media Application Provider may provision a Server Certificate (see clause 5.2.4), Content Preparation Template (see clause 5.2.5) and/or Edge Resources Configuration (see clause 5.2.6) for each Content Hosting distribution configuration to reference. The Media Application Provider may also provision one or more Policy Templates (see clause 5.2.7) to realise Service Operation Points pertaining to downlink media delivery.

-	For uplink media delivery, provisioning of the Content Publishing feature of the Media AS (see clause 5.2.9), including content contribution by Media Clients at reference point M4 and subsequent content egest of content at reference point M2 after optional manipulation by a Content Preparation Template.

After discovering the set of contribution and egest content protocols supported by the Media AS (see clause 5.2.2), the Media Application Provider may provision a Server Certificate (see clause 5.2.4), Content Preparation Template (see clause 5.2.5) and/or Edge Resources Configuration (see clause 5.2.6) for each Content Publishing contribution configuration to reference. The Media Application Provider may also provision one or more Policy Templates (see clause 5.2.7) to realise Service Operation Points relevant to the parent Provisioning Session.

8.	Provisioning of QoE metrics reporting within the scope of a Provisioning Session (see clause 5.2.10) to configure how and how often the Media Client should report Quality of Experience metrics to the Media AF during the course of media delivery sessions at reference point M4.

9.	Provisioning of consumption reporting within the scope of a Provisioning Session (see clause 5.2.11) to configure how often the Media Client should report downlink media consumption to the Media AF during the course of media delivery sessions at reference point M4.

10.	Provisioning of rules for processing of UE data (as defined in TS 26.531 [26531]) related to media delivery sessions by the Data Collection AF instantiated in the Media AF (as defined in clause 4.7 of TS 26.501 [26501]), and for restricting its exposure over reference points R5 and R6 by means of Event Data Processing Configurations and Data Access Profiles for a particular Event ID.

NOTE:	The Network Assistance feature is not provisioned by the Media Application Provider at reference point M1. Instead, it is provisioned at the discretion of the Media Delivery System operator using means beyond the scope of the present document.

### 5.2.2	Content protocols discovery operations

#### 5.2.2.1	General

The set of downlink content ingest and/or uplink content egest protocols supported by the Media AS at reference point M2 is described by the Content Protocols resource exposed by the Media AF at reference point M1, as specified in clause 8.2.3.1. This resource shall exist at all times in the Media AF.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.2.2	Create Content Protocols resource operation

The Create operation is not permitted for the Content Protocols resource. Any usage of the HTTP POST method in relation to its well-known resource URL shall result in the HTTP response 405 (Method Not Allowed).

#### 5.2.2.3	Retrieve Content Protocols resource operation

This operation is used by the Media Application Provider to retrieve from the Media AF a list of downlink content ingest protocols and/or uplink egest protocols supported by the Media AS at reference point M2. The HTTP GET method shall be used for this purpose, citing the well-known URL of the Content Protocols resource.

If the operation is successful, the Media AF shall return a 200 (OK) response that includes a Content Protocols resource in the response message body, as specified in clause 8.2.3.1.

Otherwise, the Media AF shall provide an HTTP response code as specified in clause 7.2.5 and an error response body as specified in clause 7.2.6.

#### 5.2.2.4	Update Content Protocols resource operation

The Update operation is not permitted for the Content Protocols resource. Any usage of the HTTP PUT or PATCH methods in relation to its well-known resource URL shall result in the HTTP response 405 (Method Not Allowed).

#### 5.2.2.5	Destroy Content Protocols resource operation

The Destroy operation is not permitted for the Content Protocols resource. Any usage of the HTTP DELETE method in relation to its well-known resource URL shall result in the HTTP response 405 (Method Not Allowed).

### 5.2.3	Provisioning Session operations

#### 5.2.3.1	General

Prior to configuring media delivery features specified in subsequent clauses of the present document, the Media Application Provider shall create a new Provisioning Session resource in the Media AF at reference point M1. The Media Application Provider shall nominate a globally unique external service identifier that will be used by the Media Session Handler to launch media delivery sessions and this identifier shall be associated with exactly one Provisioning Session in each Media Delivery System.

The operations specified in the following clauses are used to manage a Provisioning Session resource, as specified in clause 8.3.3.1.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.3.2	Create Provisioning Session resource operation

This operation is used by the Media Application Provider to create a new Provisioning Session. The Media Application Provider shall use the HTTP POST method to create a new Provisioning Session.

Upon successful creation, the Media AF shall return a 201 (Created) response message that includes the resource identifier of the newly created Provisioning Session resource (the Provisioning Session identifier) in the body of the HTTP response message and the URL of the resource, including its resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Provisioning Session resource (see clause 8.3.3.1), including any property values set by the Media AF.

This operation may be performed multiple times by a Media Application Provider to create different Provisioning Session resources. Each such resource is assigned a different Provisioning Session identifier by the Media AF.

#### 5.2.3.3	Retrieve Provisioning Session resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Provisioning Session from the Media AF. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Provisioning Session in the request URL.

If the target Provisioning Session exists, the Media AF shall return an HTTP 200 (OK) response with a representation of the Provisioning Session resource in the response message body.

#### 5.2.3.4	Update Provisioning Session resource operation

The Update operation is not permitted for the Provisioning Sessions resource. Any attempt to do so using the HTTP PUT or PATCH methods shall result in the HTTP response 405 (Method Not Allowed).

To achieve an equivalent outcome, the Media Application Provider should instead destroy the existing Provisioning Session resource using the operation specified in clause 5.3.3.5 and create a new one using the operation specified in clause 5.3.3.2.

#### 5.2.3.5	Destroy Provisioning Session resource operation

This operation is used by the Media Application Provider to destroy an existing Provisioning Session. The Media AF shall use the HTTP DELETE method for this purpose, citing the resource identifier of the target Provisioning Session in the request URL.

If the target Provisioning Session resource exists, the Media AF shall return a 200 (OK) HTTP response message with an empty message body. The Media AF shall release any associated resources in the Media AF and Media AS, purge any cached data, and destroy any subresources associated with the target Provisioning Session.

Any subsequent operations citing the resource identifier of a destroyed Provisioning Session should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

### 5.2.4	Server Certificate provisioning operations

#### 5.2.4.1	General

Each X.509 server certificate [X.509] presented by the Media AS to the Media Client at reference point M4 or to the BM-SC at reference point xMB-U is represented by a Server Certificate resource at reference point M1. The Server Certificates Provisioning API specified in clause 8.4 enables a Server Certificate resource to be created within the scope of a Provisioning Session, and subsequently referenced by a Content Hosting Configuration or Content Publishing Configuration created in the scope of the same Provisioning Session. That API supports two alternative provisioning methods for Server Certificate resources: one in which a certificate is generated by the Media Delivery System operator on behalf of the Media Application Provider; the other in which a certificate is generated by the Media Application Provider from a Certificate Signing Request solicited from the Media AF. Both methods shall be supported by implementations of the Media AF.

NOTE:	As a consumer of media from the Media AS in a combined architecture using 5G Media Streaming and eMBMS, the BM-SC needs to be able to trust the content it is receiving comes from a bona fide source. This issue is left to implementation.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.4.2	Create Server Certificate resource operation

This operation is used by the Media Application Provider to request that the Media Delivery System generates a new X.509 certificate on its behalf within the scope of a Provisioning Session. In this case, the certificate’s Common Name (CN) and a single Subject Alternative Name (subjectAltName, see section 4.2.1.6 of RFC 5280 [X.509]) is assigned in a domain under the control of the Media Delivery System operator and the use of supplementary domain name aliases is not supported.

The Media Application Provider shall use the HTTP POST method to create a new Server Certificate resource.

Upon successful creation, the Media AF shall return a 201 (Created) HTTP response message and the URL of the newly created Server Certificate resource, including its resource identifier, shall be provided in the HTTP Location header field. The response message body may optionally convey a copy of the X.509 certificate corresponding to the newly created Server Certificate resource, as specified in clause 8.4.3.2.

NOTE:	The X.509 certificate corresponding to the newly created Server Certificate resource may not be available immediately for interrogation and use. See clause 5.3.4.4 below for more details.

This operation may be performed multiple times by a Media Application Provider to provision different Server Certificate resources within the scope of a Provisioning Session. Each such resource is assigned a different Server Certificate identifier by the Media AF.

#### 5.2.4.3	Reserve Server Certificate resource operation

This operation is used by the Media Application Provider to solicit a Certificate Signing Request from the Media AF for the purpose of generating an X.509 certificate independently of the Media Delivery System. In this case, the subject of the certificate (i.e., its FQDN) is assigned in a domain under the control of the Media Application Provider itself, or that of a third party acting on its behalf. The Media AF shall set the Common Name (CN) and the first Subject Alternative Name (subjectAltName, see section 4.2.1.6 of RFC 5280 [X.509]) in the Certificate Signing Request it generates to the first in a list of FQDNs nominated by the Media Application Provider. The Media Application Provider shall separately arrange for this FQDN to resolve to the address of a Media AS in the target Media Delivery System.

The Media Application Provider may specify additional domain name aliases in its certificate reservation request to the Media AF. If provided, these shall be included in the returned Certificate Signing Request using the Subject Alternative Name (subjectAltName) extension (see section 4.2.1.6 of RFC 5280 [X.509]). In this case, the Media Application Provider is responsible for ensuring that these FQDN aliases resolve to the Common Name of the Media AS in the target Media Application System.

The Media Application Provider shall use the HTTP POST method to create a new Server Certificate. Upon successful creation of the resource, the Media AF shall return a 201 (Created) response message and the URL of the resource, including its resource identifier of the reserved Server Certificate resource, shall be returned in the HTTP Location header. The -HTTP response message shall provide a Certificate Signing Request as specified in clause 8.4.3.1.

This operation may be performed multiple times by a Media Application Provider to provision different Server Certificate resources within the scope of a Provisioning Session. Each such resource is assigned a different Server Certificate identifier by the Media AF.

#### 5.2.4.4	Upload Server Certificate resource operation

This operation is used by a Media Application Provider to upload an X.509 certificate to the Media AF that it has generated in response to a Certificate Signing Request solicited using the reservation operation specified in clause 5.2.4.3 above. The Media Application Provider shall use the HTTP PUT method for this purpose. The Content-Type request header and the body of the HTTP request message shall be as specified in clause 8.4.3.2.

Before accepting the supplied X.509 certificate, the Media AF shall verify that the party originating the upload is the same party that reserved the Server Certificate resource using the operations specified in clause 5.2.4.3 above. If there is a mismatch, the HTTP response 403 (Forbidden) shall be returned.

Attempting to upload an X.509 certificate to a Server Certificate resource URL that has not been reserved using the operation specified in clause 5.2.4.3 above shall elicit a 404 (Not Found) HTTP response.

#### 5.2.4.5	Retrieve Server Certificate resource operation

This operation is used by the Media Application Provider to download a Server Certificate resource from the Media AF for inspection. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Server Certificate in the request URL.

If the requested resource exists and is populated with an X.509 certificate, the Media AF shall return 200 (OK) HTTP response message with a representation of the target Server Certificate in the response message body in accordance with clause 8.4.3.2.

In the case where the X.509 certificate was provisioned by the Media Delivery System on behalf of the Media Application Provider according to clause 5.2.4.2 above, the HTTP response 503 (Service Unavailable) shall be returned until such time as the X.509 certificate is generated and available for download. The optional HTTP response header Retry-After should be included in such a response, indicating when the certificate is expected to become available for inspection and use.

In cases where the X.509 certificate is to be generated by the Media Application Provider from a Certificate Signing Request obtained according to clause 5.2.4.3 above, the HTTP response 204 (No Content) shall be returned until such time as the X.509 certificate has been uploaded using the operation specified in clause 5.2.4.4 above.

#### 5.2.4.6	Update Server Certificate resource operation

The Update operation is not permitted for the Server Certificate resource. Any attempt to do so using the HTTP PUT method shall result in the HTTP response 405 (Method Not Allowed).

Updating a previously uploaded Server Certificate in the Media AF is not permitted for security reasons. To supply a replacement X.509 certificate, for example when a previously supplied certificate is shortly due to expire, the Media Application Provider should instead use one of the operations specified in clause 5.2.4.2 or 5.2.4.3 above to create or reserve a new Server Certificate resource and, once the certificate is available for use, update the Content Hosting Configuration to reference it.

#### 5.2.4.7	Destroy Server Certificate resource operation

This operation is used by the Media Application Provider to remove a Server Certificate resource from a Provisioning Session in the Media AF. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Server Certificate in the request URL.

On success, the HTTP response 204 (No Content) shall be returned with an empty response body and afterwards the identifier of the Service Certificate resource is no longer valid. The party that originally created (see clause 5.2.4.2) or reserved (see clause 5.2.4.3) the Server Certificate resource is responsible for ensuring that the serial number of the destroyed certificate is appropriately revoked.

Only the party that created (see clause 5.2.4.2) or reserved (see clause 5.2.4.3) the Server Certificate resource is permitted to destroy it. Any attempt by another party to destroy a Server Certificate resource shall elicit the HTTP response 405 (Method Not Allowed).

The HTTP response 409 (Conflict) shall be returned if an attempt is made to destroy a Server Certificate resource that is currently referenced by a Content Hosting Configuration or Content Publishing Configuration resource.

Attempting to destroy a Server Certificate resource that has been reserved but never uploaded shall elicit a 200 (OK) HTTP response with an empty message body. In this case, the Media AF should release any resources associated with the reservation.

Any subsequent operations citing the resource identifier of a destroyed Server Certificate should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

### 5.2.5	Content Preparation Template provisioning operations

#### 5.2.5.1	General

For downlink media delivery, the Media AS may be required to process content ingested at reference point M2 before distributing it at reference point M4. For uplink media delivery, the Media AS may be required to process content contributed by  Media Clients before publishing it to the Media Application Provider at reference point M2. These content processing operations are described by a Content Preparation Template resource provisioned in the Media AF by the Media Application Provider at reference point M1, as specified in clause 8.5, and subsequently configured in the Media AS by the Media AF at reference point M3 using an API outside the scope of the present document.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.5.2	Create Content Preparation Template resource operation

This operation is used by the Media Application Provider to register a new Content Preparation Template with a Provisioning Session in the Media AF. The Media Application Provider shall use the HTTP POST method to upload a new Content Preparation Template resource, as specified in clause 8.5.2.1, to the Media AF. The MIME content type of the Content Preparation Template shall be supplied in the Content-Type HTTP request header.

Upon successful creation, the Media AF shall respond with a 201 (Created) response message and the URL of the newly created resource, including its resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Content Preparation Template resource (see clause 8.5.2.1), including any properties assigned by the Media AF.

This operation may be performed multiple times by a Media Application Provider to provision different Content Preparation Template resources within the scope of a Provisioning Session. Each such resource is assigned a different Content Preparation Template identifier by the Media AF.

If the MIME content type indicated in Content-Type is not understood by the Media AF, the creation of the Content Preparation Template resource shall fail with HTTP error response status code415 (Unsupported Media Type).

If the Media AF is unable to provision the resources indicated in the supplied Content Preparation Template, the creation operation shall fail with an HTTP response status code of 503 (Service Unavailable).

#### 5.2.5.3	Retrieve Content Preparation Template resource operation

This operation is used by the Media Application Provider to retrieve the current state of a Content Preparation Template resource from the Media AF. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Server Certificate in the request URL.

If the operation is successful, the Media AF shall respond with 200 (OK) and shall provide a representation of the requested resource in the HTTP message response body. The Content-Type response header shall have the same value as that supplied when the Content Preparation Template resource was created using the operation specified in clause 5.2.5.2.

#### 5.2.5.4	Update Content Preparation Template resource operation

This operation is used by the Media Application Provider to modify or replace an existing Content Preparation Template resource. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Content Preparation Template resource representation shall be provided in the body of the HTTP request message.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message and shall provide a representation of the current state of the resource in the message body to confirm successful update.

If the MIME content type indicated in Content-Type is not acceptable to the Media AF, the creation of the Content Preparation Template resource shall fail with HTTP error response status code 415 (Unsupported Media Type).

If the Media AF is unable to provision the resources indicated in the supplied Content Preparation Template, the update operation shall fail with an HTTP response status code of 503 (Service Unavailable).

#### 5.2.5.5	Destroy Content Preparation Template resource operation

This operation is used by the Media Application Provider to destroy a Content Preparation Template resource. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Content Preparation Template in the request URL.

If the operation is successful, the Media AF shall return a 200 (OK) response message with an empty message body.

If the Content Preparation Template is still referenced by a Content Hosting Configuration or Content Publishing Configuration, the operation shall fail with HTTP error response status code 409 (Conflict).

### 5.2.6	Edge Resources provisioning operations

#### 5.2.6.1	General

These operations are used by the Media Application Provider at reference point M1 to provision edge computing resources in the Media AF for the purpose of instantiating Edge Application Server (EAS) instances of the Media AS in an Edge Data Network (EDN), as defined in TS 23.558 [23558].

NOTE:	The requirements for an edge-enabled 5GMS AF are defined in clause 4.5.2 of TS 26.501 [26501].

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.6.2	Create Edge Resources Configuration resource operation

This operation is used by the Media Application Provider to create a new Edge Resources Configuration resource in the Media AF. The HTTP POST method shall be used for this purpose and the request message body shall be an Edge Resources Configuration resource representation, as specified in clause 8.6.3.1.

-	If the edgeManagmentMode is set to EM_AF_DRIVEN (indicating AF-driven edge resource management), the Media AF is responsible for requesting and managing the required edge resources and for handling EAS relocation in relation to media delivery sessions that fall within the scope of the parent Provisioning Session.

-	If the edgeManagementMode is set to EM_CLIENT_DRIVEN (indicating client-driven edge resource management), the Media AF shall only request edge resources based on requests from the Edge Enabler Client (EEC) instantiated in the Media Session Handler at reference point EDGE-1 (as defined in clause 6 of TS 23.558 [23558]).

If the operation is successful, the Media AF shall generate a resource identifier representing the new Edge Resources Provisioning Configuration. In this case, the Media AF shall respond with a 201 (Created) HTTP response message and the URL of the newly created resource, including its resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Edge Resources Configuration resource (see clause 8.6.3.1), including any properties assigned by the Media AF.

This operation may be performed multiple times by a Media Application Provider to provision different Edge Resources Configuration resources within the scope of a Provisioning Session. Each such resource is assigned a different Edge Resources Configuration identifier by the Media AF.

#### 5.3.6.3	Retrieve Edge Resources Configuration resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Edge Resources Provisioning Configuration resource from the Media AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the Media AF shall return a 200 (OK) response message that includes a representation of the target Edge Resources Configuration resource (see clause 8.6.3.1) in the response message body.

#### 5.2.6.4	Update Edge Resources Configuration resource operation

This operation is invoked by the Media Application Provider to modify the properties of an existing Edge Resources Configuration resource. All writeable properties except edgeManagementMode may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Edge Resources Configuration resource representation shall be provided in the body of the HTTP request message.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message and shall provide a representation of the resulting state of the resource in the message body to confirm successful update.

#### 5.2.6.5	Destroy Edge Resources Configuration resource operation

This operation is used by the Media Application Provider to destroy an Edge Resources Configuration resource in the Media AF. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Edge Resources Configuration in the request URL. This operation makes the configuration unusable for future media delivery sessions, but it does not affect any ongoing media delivery sessions.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty message body.

The HTTP response 409 (Conflict) shall be returned if an attempt is made to destroy an Edge Resources Configuration resource that is currently referenced by a Content Hosting Configuration or Content Publishing Configuration resource.

Any subsequent operations citing the resource identifier of a destroyed Edge Resources Configuration should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

### 5.2.7	Dynamic Policy provisioning operations

#### 5.2.7.1	General

These operations are used by the Media Application Provider to configure Policy Templates for the media delivery sessions of a particular Provisioning Session.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

Figure 5.2.7.1-1 below is a state diagram showing the life-cycle of a Policy Template resource in the Media AF.

Figure 5.2.7.1-1: Policy Template Resource State Diagram

Policy Templates require Media Delivery System operator verification, and a Policy Template resource that is newly created cannot be used immediately.

1.	Upon creation, a Policy Template resource shall be in the PENDING state. Once all mandatory properties are provided, the Media AF triggers validation.

2.	If the Policy Template is not deemed to be valid by the operator of the Media Delivery System, it shall move to the INVALID state, from where it can be updated to remedy the defect.

3.	Once it has been successfully validated by the Media Delivery System operator, a Policy Template resource shall take the READY state, indicating that it may be applied to media delivery sessions.

4.	If it is subsequently updated by the Media Application Provider, a Policy Template resource shall return to the PENDING state, awaiting revalidation by the operator of the Media Delivery System.

5.	Finally, a Policy Template resource may be SUSPENDED by the Media Delivery System operator, e.g., in case of a violation of the usage terms or for some other reasons, which renders it unusable. The update of any property moves the state from SUSPENDED into PENDING and triggers revalidation.

A Policy Template resource may be destroyed when it is in any of the abovementioned states.

The Media AF shall verify the status of a Policy Template resource prior to allowing a Dynamic Policy Instance to instantiate it. Only a Policy Template resource in the READY state is eligible to be instantiated in this way.

The Media AF shall indicate the current state of a Policy Template in the Policy Template resource in machine-readable form as well as indicating a human-readable reason for this state.

#### 5.2.7.2	Create Policy Template resource operation

This operation is used by the Media Application Provider to create a new Policy Template resource. The HTTP POST method shall be used for this purpose.

If the procedure is successful, the 5GMS AF shall generate a resource identifier to uniquely identify the newly created Policy Template resource. In that case, it shall return a 201 (Created) HTTP response message and the URL of the newly created resource, including its resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Policy Template resource (see clause 8.7.3.1), including any property values set by the Media AF.

This operation may be performed multiple times by a Media Application Provider to provision different Policy Template resources within the scope of a Provisioning Session. Each such resource is assigned a different Policy Template identifier by the Media AF.

The default state of a newly created Policy Template resource is PENDING. If all mandatory property values have been provided, the Policy Template resource is eligible for validation, as specified in clause 5.2.7.1.

#### 5.2.7.3	Retrieve Policy Template resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Policy Template resource in the Media AF. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Policy Template in the request URL.

If the operation is successful, the Media AF shall return a 200 (OK) response that includes a representation of the target Policy Template resource (see clause 8.7.3.1) in the response message body.

#### 5.2.7.4	Update Policy Template resource operation

This operation is invoked by the Media Application Provider to modify the properties of an existing Policy Template resource in the Media AF. All available properties except state may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Policy Template resource representation shall be provided in the body of the HTTP request message.

Any update to the Policy Template resource shall automatically change its state back to PENDING, which makes it temporarily unusable by ongoing media delivery sessions. Accordingly, any attempt to instantiate the Policy Template (see clause 5.4.3) shall fail. If all mandatory property values have been provided by the Media Application Provider, the updated Policy Template is eligible for revalidation.

If the operation is successful, the Media AF shall return a 200 (OK) response message that includes a representation of the target Policy Template resource in the response message body, confirming successful update.

Attempts to modify read-only properties of the target Policy Template resource, such as its state, shall be rejected by the Media AF with a 403 (Forbidden) HTTP response.

#### 5.2.7.5	Destroy Policy Template resource operation

This operation is used by the Media Application Provider to destroy a Policy Template resource in the Media AF. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Policy Template in the request URL.

If the operation is successful, the Media AF shall return a 200 (OK) response message with an empty message body.

Currently active media delivery sessions using the destroyed Policy Template, if any exist, shall revert to a default network QoS as a result of destroying the Policy Template resource.

Any subsequent operations citing the resource identifier of a destroyed Policy Template should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

### 5.2.8	Content Hosting provisioning operations

#### 5.2.8.1	General

These operations are used by the Media Application Provider at reference point M1 to provision the Content Hosting feature for downlink media delivery.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.8.2	Create Content Hosting Configuration resource operation

This operation is used by the Media Application Provider at reference point M1 to activate the Content Hosting feature for a particular Provisioning Session. The Media Application Provider shall use the HTTP POST method for this purpose. The request URL shall be a well-known subresource of the Provisioning Session resource and the request message body shall be a Content Hosting Configuration resource representation, as specified in clause 8.8.3.1. There is at most one Content Hosting Configuration at a time for a given Provisioning Session.

-	If the Content Hosting Configuration uses the pull-based content ingest method, i.e., the pull attribute is set to true, then the IngestConfiguration.baseURL property shall be nominated by the Media Application Provider in the request message body. The Media AF shall return the IngestConfiguration.baseURL property value unchanged in its response message body.

-	If the Content Hosting Configuration uses the push-based content ingest method, i.e., the pull attribute is set to false, then the IngestConfiguration.baseURL property shall be nominated by the Media AF and returned in the response message body. It shall not be set by the Media Application Provider in the request message body.

In all cases, the DistributionConfiguration.baseURL property is read-only: it shall always be omitted from the creation request and shall be assigned by the Media AF, allowing the value to be inspected by the Media Application Provider in the returned Content Hosting Configuration resource representation, or by using the operation specified in clause 5.2.8.3 below.

If the operation is successful, the Media AF shall return a 201 (Created) HTTP response message and the request URL shall be returned as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Content Hosting Configuration resource (see clause 8.8.3.1), including any properties assigned by the Media AF.

#### 5.2.8.3	Retrieve Content Hosting Configuration resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Content Hosting Configuration resource from the Media AF. The HTTP GET method shall be used for this purpose.

If the operation is successful, the Media AF shall return a 200 (OK) response message that includes a representation of the target Content Hosting Configuration resource (see clause 8.8.3.1) in the response message body.

#### 5.2.8.4	Update Content Hosting Configuration resource operation

This operation is invoked by the Media Application Provider to modify the properties of an existing Content Hosting Configuration resource. All writeable properties except domainNameAlias may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Content Hosting Configuration resource representation shall be provided in the body of the HTTP request message.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message and shall provide a representation of the current state of the target resource in the message body to confirm successful update.

#### 5.2.8.5	Destroy Content Hosting Configuration resource operation

This operation is used by the Media Application Provider to destroy a Content Hosting Configuration resource in the Media AF and to terminate the related content distribution. The HTTP DELETE method shall be used for this purpose, citing the well-known subresource of the target Provisioning Session in the request URL . As a result, the Media AF shall release any associated network resources, purge any cached content in the Media AS, and remove any corresponding configurations.

If the procedure is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty message body.

#### 5.2.8.6	Purge Content Hosting cache operation

This operation is used by the Media Application Provider to purge content from the Media AS Content Hosting cache. The HTTP POST method shall be used for this purpose with a regular expression describing the media resource URLs to be purged provided in the body of the request. The message request body shall be encoded using the application/x-www-form-urlencoded MIME content type as a key–value pair, with the key being the string pattern and the value being the regular expression.

On receiving a purge request, the Media AF shall immediately invalidate all media resources in the Media AS cache matching the regular expression by declaring them as stale. A subsequent Media Client request at reference point M4 for a purged media resource will trigger the fetching (and possible caching) of the current version from the Media Application Provider's content origin via reference point M2 in case of a Pull-based ingest. For Push-based ingest, M4 requests for purged content shall be responded to with a 404 (Not Found) HTTP response until such time as a new version of the object is published by the Media Application Provider to the Media AS via at reference point M2.

If the procedure is successful, the Media AF shall return one of the following response messages:

-	204 (No Content) if no cache entries were purged, for example because no current cache entries matched the regular expression supplied in the original request. The response message body shall be empty in this case.

-	200 (OK) if some cache entries were purged. The body of the response message shall indicate the total number of cache entries purged in all Media AS instances distributing the content.

The HTTP response 400 (Bad Request) shall be returned in the case where the request message body – or the regular expression contained in it – are found by the Media AF to be syntactically malformed.

### 5.2.9	Content Publishing provisioning operations

#### 5.2.9.1	General

These operations are used by the Media Application Provider at reference point M1 to provision the Content Publishing feature for uplink media delivery.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.9.2	Create Content Publishing Configuration resource operation

This operation is used by the Media Application Provider at reference point M1 to activate the Content Publishing feature for a particular Provisioning Session. The Meida Application Provider shall use the HTTP POST method for this purpose. The request URL shall be a well-known subresource of the Provisioning Session resource and the request message body shall be a Content Publishing Configuration resource representation, as specified in clause 8.8.3.2. There is at most one Content Publishing Configuration at a time for a given Provisioning Session.

-	If the Content Publishing Configuration uses the push-based content ingest method, i.e., the pull attribute is set to false, then the egestConfiguration.baseURL property shall be nominated by the Media Application Provider in the request message body. The Media AF shall return the egestConfiguration.baseURL propertyvaalue unchage  in its response message body.

-	If the Content Publishing Configuration uses the pull-based content ingest method, i.e., the pull attribute is set to true, then the egestConfiguration.baseURL property shall be nominated by the Media AF and returned in the response message body. It shall not be set by the Media Application Provider in the request message body.

If the operation is successful, the Media AF shall return a 201 (Created) HTTP response message and the request URL shall be returned as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Content Publishing Configuration resource (see clause 8.8.3.2), including any properties assigned by the Media AF.

#### 5.2.9.3	Retrieve Content Publishing Configuration resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Content Publishing Configuration resource from the Media AF. The HTTP GET method shall be used for this purpose.

If the operation is successful, the Media AF shall return a 200 (OK) response message that includes a representation of the target Content Publishing Configuration resource (see clause 8.8.3.2) in the response message body

#### 5.2.9.4	Update Content Publishing Configuration resource operation

This operation is invoked by the Media Application Provider to modify the properties of an existing Content Publishing Configuration resource. All writeable properties may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message and shall provide a representation of the current state of the target resource in the message body to confirm successful update.

#### 5.2.9.5	Destroy Content Publishing Configuration resource operation

This operation is used by the Media Application Provider to destroy a Content Publishing Configuration resource and to terminate the related egest of content. The HTTP DELETE method shall be used for this purpose. As a result, the Media AF will release any associated network resources, purge any cached content, and delete any corresponding configurations.

#### 5.2.9.6	Purge Content Publishing cache operation

When pull-based content egest is provisioned in the Content Publishing Configuration, this operation is used by the Media Application Provider to purge content from the Media AS Content Publishing cache. The HTTP POST method shall be used for this purpose with a regular expression describing the media resource URLs to be purged provided in the body of the request. The message request body shall be encoded using the application/x-www-form-urlencoded MIME content type as a key–value pair, with the key being the string pattern and the value being the regular expression.

On receiving a purge request, the Media AF shall immediately invalidate all media resources in the Media AS cache matching the regular expression by declaring them as stale.

If the procedure is successful, the Media AF shall return one of the following response messages:

-	204 (No Content) if no cache entries were purged, for example because no current cache entries matched the regular expression supplied in the original request. The response message body shall be empty in this case.

-	200 (OK) if some cache entries were purged. The body of the response message shall indicate the total number of cache entries purged in all Media AS instances distributing the content.

The HTTP response 400 (Bad Request) shall be returned in the case where the request message body – or the regular expression contained in it – are found by the Media AF to be syntactically malformed.

### 5.2.10	Metrics Reporting provisioning operations

#### 5.2.10.1	General

These operations are used by the Media Application Provider at reference point M1 to provision QoE metrics reporting functionality associated with downlink or uplink media delivery. The Media Application Provider may provision several Metrics Reporting Configurations within the scope of a Provisioning Session with different properties which determine whether and how often the Media Session Handler submits QoE metrics reports to the Media AF as well as the format and contents of these reports. To this end, each Metrics Reporting Configuration shall specify a metrics scheme which may be 3GPP-defined or non-3GPP-defined. This clause defines the basic operations; more details are provided in clause 8.10.

A given Metrics Reporting Configuration is uniquely identified within the scope of its parent Provisioning Session by the metricsReportingConfigurationId property of the corresponding Metrics Reporting Configuration resource, as specified in clause 8.10.3.1.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.10.2	Create Metrics Reporting Configuration resource operation

This operation is used by the Media Application Provider to create a Metrics Reporting Configuration resource within the scope of an existing Provisioning Session. The Media Application Provider shall use the HTTP POST method for this purpose. The request URL shall be a well-known subresource of the Provisioning Session resource and the request message body shall be a Metrics Reporting Configuration resource representation, as specified in clause 8.10.3.1.

Upon successful creation of the resource, the Media AF shall return a 201 (Created) response message and the resource URL for the newly-created Metrics Reporting Configuration resource, including its resource identifier, shall be returned as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Metrics Reporting Configuration resource (see clause 8.10.3.1), including any properties assigned by the Media AF.

This operation may be performed multiple times by a Media Application Provider to provision different Edge Resources Configuration resources within the scope of a Provisioning Session. Each such resource is assigned a different Edge Resources Configuration identifier by the Media AF.

This operation may be performed multiple times by a Media Application Provider to provision different Metrics Reporting Configuration resources within the scope of a Provisioning Session. Each such resource is assigned a different Metrics Reporting Configuration identifier by the Media AF.

#### 5.2.10.3	Retrieve Metrics Reporting Configuration resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Metrics Reporting Configuration resource in the Media AF. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Metrics Reporting Configuration in the request URL.

If successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the target Metrics Reporting Configuration resource (see clause 8.10.3.1) in the response message body.

#### 5.2.10.4	Update Metrics Reporting Configuration resource operation

This operation is invoked by the Media Application Provider to entirely replace or modify certain properties of an existing Metrics Reporting Configuration resource. All available properties may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Metric Reporting Configuration resource representation shall be provided in the body of the HTTP request message.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the current state of the target resource in the message body to confirm successful update.

#### 5.2.10.5	Destroy Metrics Reporting Configuration resource operation

This operation is used by the Media Application Provider to destroy a Metrics Reporting Configuration resource and to terminate the related metrics reporting procedure by Media Clients. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Metrics Reporting Configuration in the request URL. As a result, the Media AF shall release any associated resources, discard any pending metrics reports, and remove any corresponding configurations.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty message body.

Any subsequent operations citing the resource identifier of a destroyed Metrics Reporting Configuration should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

### 5.2.11	Consumption Reporting provisioning procedures

#### 5.2.11.1	General

These operations are used by the Media Application Provider at reference point M1 to activate and to configure consumption reporting functionality associated with downlink media delivery. The Media Application Provider may provision a single Consumption Reporting Configuration within the scope of a Provisioning Session which determines whether and how often the Media Session Handler submits consumption reports to the Media AF. This clause defines the basic operations. More details are provided in clause 8.11.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.11.2	Create Consumption Reporting Configuration resource operation

This operation is used by the Media Application Provider to activate the Consumption Reporting feature for a particular Provisioning Session. The Media Application Provider shall use the HTTP POST method for this purpose. The request URL shall be a well-known subresource of the Provisioning Session resource and the request message body shall be a Consumption Reporting Configuration resource representation, as specified in clause 8.11.3.1. There is at most one Consumption Reporting Configuration at a time for a given Provisioning Session.

Upon successful creation of the resource, the Media AF shall return a 201 (Created) response message and the request URL shall be returned as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Consumption Reporting Configuration resource (see clause 8.11.3.1), including any properties assigned by the Media AF.

#### 5.2.11.3	Retrieve Consumption Reporting Configuration resource operation

This operation is used by the Media Application Provider to obtain the current Consumption Reporting Configuration from the Media AF. The HTTP GET method shall be used for this purpose.

If successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the target Consumption Reporting Configuration resource (see clause 8.11.3.1) in the response message body.

#### 5.2.11.4	Update Consumption Reporting Configuration resource operation

This operation is invoked by the Media Application Provider to modify the current Consumption Reporting Configuration. All available parameters may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Consumption Reporting Configuration resource representation shall be provided in the body of the HTTP request message.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the current state of the Consumption Reporting Configuration resources to confirm successful update.

#### 5.2.11.5	Destroy Consumption Reporting Configuration resource operation

This operation is used by the Media Application Provider to destroy the Consumption Reporting Configuration resource and to terminate the related consumption reporting procedure by Media Clients. The HTTP DELETE method shall be used for this purpose, citing the well-known subresource of the target Provisioning Session in the request URL. As a result, the Media AF shall release any associated resources, purge any cached data, and remove any corresponding configurations.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty message body.

### 5.2.12	Event Data Processing Provisioning operations

#### 5.2.12.1	General

These operations are used by the Media Application Provider at reference point M1 to configure the collection and processing of UE data related to media delivery (as defined in TS 26.531 [46]) and to restrict its exposure over reference points R5 and R6 by configuring the Data Collection AF instantiated in the Media AF (such as that defined in clause 4.7 of TS 26.501 [26501]) with one or more Event Data Processing Configurations and Data Access Profiles for a particular Event ID. The Media Application Provider may provision several Event Data Processing Configurations within the scope of a Provisioning Session with different properties.

Each instance of a Data Access Profile specifies a set of data processing operations to be performed by the Data Collection AF on its collected UE data in order to synthesize the event data to be exposed to a specific Event service consumer entity. In this release, eligible Event service consumer entities of Media Delivery event data are the NWDAF, the Event Consumer AF and the NEF.

The Event Data Processing Provisioning API is specified in clause 8.12.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.2.12.2	Create Event Data Processing Configuration resource operation

This operation is used by the Media Application Provider to create a new Event Data Processing Configuration within the scope of an existing Provisioning Session in the form of one or more Data Access Profiles. The HTTP POST method shall be used for this purpose and the request message body shall be an Event Data Processing Configuration resource representation, as specified in clause 8.12.3.1.

If the operation is successful, the Media AF shall generate a resource identifier representing the new Event Data Processing Configuration. In this case, the Media AF shall return a 201 (Created) response message and the URL of the newly created resource, including its resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Event Data Processing Configuration resource (see clause 8.12.3.1), including any properties assigned by the Media AF.

This operation may be performed multiple times by a Media Application Provider to provision different Event Data Processing Configuration resources within the scope of a Provisioning Session. Each such resource is assigned a different Event Data Processing ConfigurationId identifier by the Media AF.

#### 5.2.12.3	Retrieve Event Data Processing Configuration resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Event Data Processing Configuration resource in the Media AF. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Event Data Processing Configuration in the request URL.

If successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the target Event Data Processing Configuration resource (see clause 8.12.3.1) in the response message body.

#### 5.2.12.4	Update Event Data Processing Configuration resource operation

This operation is invoked by the Media Application Provider to entirely replace or modify certain properties of an existing Event Data Processing Configuration resource. All available properties may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Event Data Processing Configuration resource representation shall be provided in the body of the HTTP request message.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the current state of the target resource in the message body to confirm successful update.

#### 5.2.12.5	Destroy Event Data Processing Configuration resource operation

This operation is used by the Media Application Provider to destroy an existing Event Data Processing Configuration resource and to terminate the related collection of UE data and exposure of events to event consumer subscribers. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Event Data Processing Configuration in the request URL. As a result, the Data Collection AF shall process any reported UE data still outstanding and shall delete any corresponding data collection and reporting client configurations as well as any event subscriptions.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty message body.

Any subsequent operations citing the resource identifier of a destroyed Event Data Processing Configuration should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

## 5.3	Network media session handling (M3, M5) interactions

Editor’s Note: aligned with 4.7 of TS 26.512, but not restricted to downlink streaming.

### 5.3.1	Introduction

This clause specifies the set of operations used by the Media Session Handler within a Media Client to invoke services on the Media AF at reference point M5 relating to downlink or uplink media delivery. A subset of these operations is also exposed by the Media AF to the Media AS at reference point M3.

### 5.3.2	Service Access Information operations

#### 5.3.2.1	General

Service Access Information is the set of parameters and addresses needed by the Media Client to activate reception of a downlink media delivery session or to activate an uplink media delivery session for content contribution.

The Media Session Handler may obtain Service Access Information in one of two ways:

1.	From the Media-aware Application via reference point M6. In this case, the Service Access Information is initially acquired by the Media-aware Application from the Media Application Provider via reference point M8 and the Media-aware Application shall pass the parameters to the Media Session Handler using one of the session launch mechanisms specified in clause 10.2.

2.	From the Media AF via reference point M5. In this case, the Service Access Information is derived by the 5GMSd AF from a Provisioning Session established at reference point M1 and the Media AF exposes this to the Media Session Handler using the operations specified in this clause. At the start of a media delivery session, a minimal set of baseline Service Access Information parameters is passed to the Media Session Handling using one of the session launch mechanisms specified in clause 10.2 and this causes it to fetch the full Service Access Information from the Media AF using the procedure specified in clause 5.3.2.3.

The data model of the Service Access Information resource acquired by the Media Session Handler of the Media Client is specified in clause 9.2.3.

Typically, the Service Access Information for downlink media delivery includes a media entry point (e.g. a URL to a DASH MPD or a URL to a progressive download file) that can be consumed by the Media Access Function and is handed to the Media Access Function via reference point M7.

Service Access Information additionally includes configuration information to allow the Media Session Handler to invoke procedures for dynamic policy (see clause 5.3.3), network assistance (clause 5.3.4), QoE metrics reporting (clause 5.3.5) and consumption reporting (clause 5.3.6).

If an Edge Resources Configuration  with client-driven management (EM_CLIENT_DRIVEN) is provisioned in the applicable Provisioning Session (see clause 5.2.6), the Media AF shall convey the Client Edge Resources Configuration to the Media Session Handler as part of the Service Access Information.

NOTE:	The requirements for an edge-enabled Media Session Handler are defined in clause 4.5.2 of TS 26.501 [26501].

For downlink media streaming exclusively via eMBMS and for hybrid 5GMSd/eMBMS services as defined in clauses 5.10.2 and 5.10.5 respectively of TS 26.501 [26501], the Service Access Information indicates that the Media Client acts as an MBMS-Aware Application.

For dynamically provisioned downlink media streaming via eMBMS as defined in clause 5.10.6 of TS 26.501 [26501], the Media AS creates a presentation manifest that is regularly polled by the Media Player for a potential update. When an eMBMS User Service carrying the 5GMSd content is dynamically provisioned or removed by the 5GMSd AF, the 5GMSd AS shall update the presentation manifest with the locations where the updated manifest and the media segments are now available, for example to add or change to the media server in the MBMS Client.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.3.2.2	Create Service Access Information resource

The Create operation is not permitted for the Service Access Information resource collection. Any usage of the HTTP POST method shall result in the HTTP response 405 (Method Not Allowed).

#### 5.3.2.3	Retrieve Service Access Information resource operation

This operation shall be used by the Media Session Handler to acquire Service Access Information from the Media AF. The Media Session Handler shall use the GET method for this purpose, citing the external service identifier associated with the target Provisioning Session (see clause 5.2.3) in the request URL. The reqest message body shall be empty.

If successful, the Media AF shall reply with a 200 (OK) HTTP response message that includes a representation of the Service Access Information associated with the target media streaming session resource in the response message body, along with HTTP response headers in line with clause 7.2.3.2.

Once it has obtained an initial set of Service Access Information, the Media Session Handler shall periodically check for updated Service Access Information by issuing a conditional HTTP GET request in line with clause 7.2.3.2.

The periodicity of polling for updated Service Access Information shall be guided by the value of the Expires and/or Cache-control: max-age headers that shall be included along with every response message for this operation.

#### 5.3.2.4	Update Service Access Information resource operation

The Update operation is not permitted for members of the Service Access Information resource collection. Any usage of the HTTP PUT or PATCH methods in relation to the URLs of its members shall result in the HTTP response 405 (Method Not Allowed).

#### 5.3.2.5	Destroy Service Access Information resource operation

The Destroy operation is not permitted for members of the Service Access Information resource collection. Any usage of the HTTP DELETE method in relation to the URLs of its members shall result in the 405 (Method Not Allowed) HTTP response.

### 5.3.3	Dynamic Policy operations

#### 5.3.3.1	General

These operations are used by a Media Session Handler to manage Dynamic Policy Instance resources in the Media AF via reference point M5. Instantiating a Policy Template as a dynamic policy requires a Policy Template identifier, a set of Service Data Flow description(s), an optional dynamic QoS specification and potentially other parameters defined in clause 5.7 of TS 26.501 [26501].

-	The Policy Template identifier identifies the desired Policy Template (as previously provisioned per clause 5.2.7.2) to be applied to the specified application flow(s). A Policy Template includes properties such as specific QoS (e.g. background data) or different charging treatments. The Media AF combines the information from the Policy Template with dynamic QoS specification supplied by the Media Session Handler and uses this complete set of parameters to invoke the PCF according to clause 11.2.x.

-	The set of Service Data Flow description(s) allow the identification and classification by the 5G System of the application traffic involved in a media delivery session. These take the form of an IP packet filter set (as defined in clause 5.7.6 of [23501]) or the Fully-Qualified Domain Name (FQDN) of a Media AS at reference point M4.

NOTE:	It is not defined in this release how a Media AF in an external Data Network selects a specific DNN or S-NSSAI.

The Dynamic Policy Instance resource created as a result of instantiating a Policy Template shall include an MQTT endpoint address that allows the Media Session Handler to subscribe to receive asynchronous notifications from the Media AF concerning Background Data Transfer opportunities available in relation to that Dynamic Policy Instance.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.3.3.2	Create Dynamic Policy Instance resource operation

In order to instantiate a new dynamic policy, the Media Session Handler shall first create a resource for the Dynamic Policy Instance in the Media AF. The Media Session Handler shall use the HTTP POST message for this purpose. The body of the HTTP POST message shall be a Dynamic Policy Instance resource representation that includes a Provisioning Session identifier, the resource identifier of the target Policy Template and a set of Service Data Flow descriptions identifying the application flow(s) to be policed.

If the operation is successful, the Media AF shall create a new Dynamic Policy Instance resource. In this case, the Media AF shall return a 201 (Created) HTTP response message to the Media Session Handler, and the URL of the newly created Dynamic Policy Instance resource, including its resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Dynamic Policy Instance resource (see clause 9.3.3.1), including any properties assigned by the Media AF.

If the Media Session Handler needs to instantiate several dynamic policies, it may invoke this operation as often as needed.

When the Dynamic Policy Instance is successfully instantiated, the Media AF triggers the creation of a corresponding PCC rule in the 5G System according to clause 11.2.x to enforce the required QoS and charging policy on the specified application flow(s). Depending on the selected sdfMethod and populated Service Data Flow objects in the received Dynamic Policy Instance resource representation, the Media AF shall populate a flowDescription object and/or provide an Application Identifier referring to a PFD (Packet Flow Description) object containing the domain name of a Media AS instance.

NOTE:	It is not defined in this release how a Media AF in an external Data Network provides an application identifier.

#### 5.3.3.3	Retrieve Dynamic Policy Instance resource operation

This operation is used by the Media Session Handler to retrieve the current state of an existing Dynamic Policy Instance resource in the Media AF. HTTP GET method shall be used for this purpose, citing the resource identifier of the target Dynamic Policy Instance in the request URL.

If successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the target Dynamic Policy Instance resource (see clause 9.3.3.1) in the response message body.

#### 5.3.3.4	Update Dynamic Policy Instance resource operation

This operation is invoked by the Media Session Handler to entirely replace or modify certain properties of an existing Dynamic Policy resource. All available properties may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose, citing the resource identifier of an existing Dynamic Policy Instance in the request URL. The replacement Dynamic Policy Instance resource representation shall be provided in the body of the HTTP request message.

If all required information is set in the replacement Dynamic Policy Instance, the Media AF shall trigger the appropriate actions towards other Network Functions in the 5G System according to clause 11.2.x to update the associated PCC rule in line with the modified QoS and charging policy.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message that includes a representation of the current state of the target resource in the message body to confirm successful update.

#### 5.3.3.5	Destroy Dynamic Policy Instance resource operation

This operation is invoked by the Media Session Handler to destroy an existing Dynamic Policy Instance resource. theThe HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Dynamic Policy Instance in the request URL. As a result, the Media AF shall trigger the appropriate actions towards other Network Functions in the 5G System according to clause 11.2.x to remove the associated PCC rule and to revert the affected application flow(s) to a default QoS and charging policy.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty message body.

Any subsequent operations citing the resource identifier of a destroyed Dynamic Policy Instance should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

### 5.3.4	Network assistance procedures and operations

#### 5.3.4.1	Procedures

These procedures are used by the Media Session Handler to request Network Assistance from one of the Media AF instances listed in the networkAssistanceConfiguration.serverAddresses property of the Service Access Information resource retrieved using the procedure in clause 5.3.2.3.

1.	The Media Client first creates a Network Assistance Session with its chosen Media AF instance. It provides information that will later be used by the Media AF to request a particular network QoS to be applied by the PCF to one or more application data flows, and to recommend a bit rate to the Media Client. The Media AF assigns a resource identifier to the Network Assistance Session at the point of creation. This procedure is further specified in clause 5.3.4.2.

When a Network Assistance Session is created, the responding Media AF instance may nominate an MQTT endpoint URL in the notificationURL property. The Media Session Handler may subscribe to the MQTT channel provided at this endpoint and receive notifications from the Media AF with an up-to-date bit rate recommendation whenever this changes.

2.	The Network Assistance Session resource may be retrieved by the Media Session Handler using the procedure specified in clause 5.3.4.3.

3.	At any time after the Network Assistance Session resource is created, the 5GMS Client may use the Network Assistance Session resource identifier to explicitly request a bit rate recommendation by invoking a remote procedure call provided for this purpose by the Media AF. This procedure is further specified in clause 5.3.4.4.

4.	Using the Network Assistance Session resource identifier, the Media Client may also request a delivery boost to be provided by the 5G System at any time by invoking a remote procedure call provided for this purpose by the Media AF. This procedure is further specified in clause 5.3.4.5.

5.	The information provided when first creating a Network Assistance Session may be modified subsequently by the Media Session Handler using the session modification operation specified in clause 5.3.4.6.

6.	In order to terminate a Network Assistance Session, the Media Client destroys the Network Assistance Session resource using the procedure specified in clause 5.3.4.7.

Details of the APIs supporting these procedures at reference point M5 are specified in clause 9.4.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.3.4.2	Create Network Assistance Session resource operation

This operation is used by the Media Session Handler to create a Network Assistance Session in the 5GMS AF. The POST HTTP method shall be used for this purpose and the request message body shall be a Network Assistance Session resource representation as specified in clause 9.4.3.1.

1.	The provisioningSessionId property associates the Network Assistance Session with a Provisioning Session.

2.	The Media Session Handler shall populate the Network Assistance Session resource representation in the request with service data flow information and optionally the Policy Template identifier of the network QoS policy currently in force on the media streaming session for which Network Assistance operations are to be performed. (The Media AF subsequently uses this information to execute Network Assistance operations in the 5GC.)

The serviceDataFlowDescriptions property shall contain a set of service data flow templates according to TS 23.503 [33]. Each service data flow template contains one of:

-	a flowDescription object (including 5-tuples, Type of Service, Security Parameter Index, etc.).

-	a domainName.

3.	The requestedQoS property may be provided in the Network Assistance Session resource representation to specify an initial network QoS the Media Session Handler wishes to use for the media streaming session. If the policyTemplateId property is also populated in the Network Assistance Session resource representation, the Media AF shall return a 400 (Bad Request) HTTP response message if the requested network QoS lies outside the limits specified in the referenced Policy Template.

-	If the requestedQoS property is omitted from the Network Assistance Session resource representation but the policyTemplateId is populated, the Media AF shall use the network QoS currently provisioned in the referenced Policy Template as the floor/ceiling for bit rate recommendations and delivery boosts within the scope of the Network Assistance Session.

-	If neither a policyTemplateId nor a requestedQoS are supplied when creating a Network Assistance Session, operations invoked on the Media AF within the scope of the Network Assistance session are constrained only by the policies of the PCF.

Upon successful creation, the Media AF shall return a 201 (Created) response message and the URL of the newly created resource, including its Network Assistance session resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Network Assistance Session resource (see clause 9.4.3.1), including any properties assigned by the Media AF.

The 5GMS Client uses the Network Assistance Session resource identifier (naSessionId) provided by the Media AF to refer all subsequent API calls to the Media AF instance responsible for that Network Assistance Session.

#### 5.3.4.3	Retrieve Network Assistance Session resource operation

This operation is used by the Media Session Handler to retrieve the current state of a Network Assistance Session resource from the Media AF. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Network Assistance Session in the request URL.

If the operation is successful, the Media AF shall return 200 (OK) and shall provide a representation of the requested resource in the HTTP message response body.

#### 5.3.4.4	Bit rate recommendation request operation

This operation is used by the Media Session Handler to request a bit rate recommendation from the Media AF. the HTTP GET method shall be used for this purpose, citing the resource identifier of an existing Network Assistance Session in the request URL along with a sub-resource path indicting the bit rate recommendation operation-.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message and shall provide the recommended bit rate(s) in an HTTP response message body containing an M5QoSSpecification object that is populated as follows:

-	For a downlink media delivery session, the recommended minimum and maximum downlink bit rates shall be indicated in the properties mirBwDlBitRate and marBwDlBitRate respectively. If a unique downlink bit rate is recommended by the Media AF then this value shall be set identically in both of these properties. The Media Session Handler shall ignore the mandatory properties related to uplink media delivery, i.e., mirBwUlBitRate and marBwUlBitRate.

-	For an uplink media delivery session, the recommended minimum and maximum uplink bit rates shall be indicated in the properties mirBwUlBitRate and marBwUlBitRate, respectively. If a unique uplink bit rate is recommended by the Media AF then this value shall be set identically in both of these properties. The Media Session Handler shall ignore the mandatory properties related to downlink media delivery, i.e., mirBwDlBitRate and marBwDlBitRate.

The optional properties minDesBwDlBitRate, minDesBwUlBitRate, desLatency and desLoss shall not be included in the returned M5QoSSpecification object.

#### 5.3.4.5	Delivery boost request operation

This operation is used by the Media Session Handler to request a delivery boost from the Media AF. the HTTP POST method shall be used for this purpose, citing the resource identifier of an existing Network Assistance Session in the request URL along with a sub-resource path indicating the delivery boost operation-.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message and shall provide an OperationSuccessResponse object in the message body indicating whether or not the delivery boost was successfully applied by the Media AF to the application data flow(s) described in the target Network Assistance Session.

#### 5.3.4.6	Update Network Assistance Session resource operation

This operation is used by the Media Session Handler to replace the steaming session parameters in an existing Network Assistance Session resource with new values. The HTTP PUT or PATCH methods shall be used for this purpose, citing the resource identifier of an existing Network Assistance Session in the request URL. Any change to the Policy Template currently in force resulting from instantiation of a Dynamic Policy (see clause 5.3.3) should also be notified to the Media AF using this operation if a Network Assistance session has been created for the media delivery session in question.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message and shall provide a representation of the current state of the resource in the message body to confirm successful update.

#### 5.3.4.7	Destroy Network Assistance Session resource operation

This operation is used by the Media Session Handler to terminate a Network Assistance Session. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Network Assistance Session in the request URL.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty message body.

Any subsequent operations citing the resource identifier of a destroyed Network Assistance Session should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.2.6.

### 5.3.5	Metrics reporting procedures and operations

#### 5.3.5.1	Procedures

These procedures are used by the Media AS at reference point M3 or else by the Metrics Reporting functions of the Media Client and subsequently by the Media Session Handler at reference point M5 to submit a metrics report to one of the Media AF instances listed in the client metrics reporting configuration of the Service Access Information resource previously retrieved using the procedure in clause 5.3.2.3.

When the metrics collection and reporting feature is provisioned for a media delivery session using the operations specified in clause 5.2.10, one or more client metrics reporting configurations, each associated with a provisioned Metrics Reporting Configuration, shall be provided to the Media Session Handler in the Service Access Information. Based on the requirements of TS 26.501 [26501], a given client metrics reporting configuration contains information including:

1.	The subset of metrics from the provisioned metrics scheme to be collected and reported by the Media Client;

2.	The frequency at which these metrics are to be sampled by the Media Client;

3.	The target percentage of media delivery sessions for which reports are to be sent by the Media Session Handler;

4.	The interval at which metrics reports are to be sent by the Media Session Handler if reporting is enabled for a media delivery session; and

5.	The Media AF address(es) to which metrics reports are to be sent.

Editor's Note: The following paragraph seems specific to 5G Media Streaming and could be retained in TS 26.512.

For progressive download and DASH streaming services, the metrics listed in a given client metrics reporting configuration are a subset of those associated with the 3GPP metrics scheme and shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [4]. Metrics related to virtual reality media, as specified in clause 9.3 of TS 26.118 [42] clause 9.3, may also be listed in the client metrics reporting configuration. Metrics related to eMBMS delivery, as specified in clause 9.4.6 of TS 26.346 [51], may also be listed in the client metrics reporting configuration.

Before a downlink media delivery session is started, the Media Session Handler shall check if the Service Access Information includes any Metrics Reporting Configurations. If any such configurations are present, the Media Session Handler shall initiate the metrics reporting procedure for the media delivery session based on these configurations. During the downlink media delivery session, the Media Session Handler shall periodically check if the Metrics Reporting Configurations are added to or removed from the Service Access Information and shall activate or deactivate the metrics reporting procedure as appropriate for the media delivery session in question.

The Service Access Information indicating whether Metrics Reporting is provisioned for media delivery sessions is specified in clause 9.2.3.

Whenever a metrics report is produced for a given client metrics reporting configuration, the Media Session Handler shall reset its reporting interval timer for that configuration to the value of the clientMetrics‌Reporting‌Configurations[].‌reportingInterval property and it shall begin countdown of the timer again. Whenever the Media Session Handler terminates a media delivery session, it shall disable its reporting interval timer for all client metrics reporting configurations.

Details of the APIs supporting these procedures at reference points M3 and M5 are specified in clause 9.5.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.3.5.2	Submit metrics report operation

This operation is used by the Media Session Handler or Media AS to submit a metrics report to the Media AF If several Media AF addresses are listed in the serverAddresses array of the client metrics reporting configuration (see table 9.2.3.1-1), the Media Session Handler shall choose one at random and shall send the metrics report to the selected server endpoint. The HTTP POST method shall be used for this purpose, citing the address of the chosen Media AF in the request URL. The request body shall be formatted according to the metrics scheme indicated in scheme property of the client metrics reporting configuration (see table 9.2.3.1-1), as specified in clause 11.4.3 of TS 26.512 [26512].

A reporting client identifier may be included in the metrics report. If available to the Media Session Handler, its value should be a GPSI value as defined by TS 23.003 [23003]. Otherwise, the reporting client identifier should be represented by a stable and globally unique string.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty body to acknowledge successful processing of the metrics report.

If the target Media AF endpoint is temporarily unavailable, the Media Client should store outstanding metrics reports and reattempt submission when the endpoint later becomes available. Details are left to implementation.

### 5.3.6	Consumption reporting procedures and operations

#### 5.3.6.1	Procedures

These procedures are used by the Media AS at reference point M3 or else by the Consumption Reporting functions of the Media Client and subsequently by the Media Session Handler at reference point M5 to submit a consumption report to one of the Media AF instances listed in the client consumption reporting configuration of the Service Access Information resource previously retrieved using the procedure in clause 5.3.2.3.

When the Consumption Reporting feature is provisioned for a downlink media delivery session using the operations specified in clause 5.2.11, a client consumption reporting configuration shall be provided to the Media Session Handler in the Service Access Information.

Before a downlink media delivery session is started, the Media Session Handler shall check if the Service Access Information includes any Client Consumption Reporting Configuration. If such a configuration is present, the Media Session Handler shall initiate consumption reporting for the downlink media delivery session based on this configuration. During the course of the downlink media delivery session, the Media Session Handler shall periodically check if the Client Consumption Reporting Configuration is added to or removed from the Service Access Information and shall activate or deactivate the consumption reporting procedure as appropriate for the media delivery session in question.

The Service Access Information indicating whether Consumption Reporting is provisioned for a particular downlink media delivery sessions is specified in clause 9.2.3.

When the samplePercentage property in the Client Consumption Reporting Configuration has a value of 100 percent, the Media Session Handler shall activate the consumption reporting procedure. If the samplePercentage value is less than 100 percent, the Media Session Handler shall generate a random number which is uniformly distributed in the range of 0 to 100, and the Media Session Handler shall activate the consumption reporting procedure when the generated random number is of a lower value than the samplePercentage value.

If the consumption reporting procedure is activated, the Media Session Handler shall produce and submit a consumption report to the Media AF using the procedure specified in clause 5.3.6.2 when any of the following conditions are met:

-	At the start of consumption of a downlink media delivery session;

-	At the end of consumption of a downlink streaming session;

-	On determining the need to report ongoing content consumption at periodic intervals determined by the reportingInterval property in the Client Consumption Reporting Configuration.

-	On detecting a location change, if the locationReporting property in the Client Consumption Reporting Configuration is set to True.

-	On detecting a change of access network, if the accessReporting property in the Client Consumption Reporting Configuration is set to True.

Whenever a consumption report is produced, the Media Session Handler shall reset its consumption reporting interval timer to the value of the reportingInterval property of the Client Consumption Reporting Configuration and it shall begin countdown of the timer again. Whenever the Media Session Handler terminates a downlink media delivery session, it shall disable its consumption reporting interval timer.

Details of the APIs supporting these procedures at reference points M3 and M5 are specified in clause 9.6.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.2.5 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.2.6 shall be included in the response message.

#### 5.3.6.2	Submit consumption report operation

This operation is used by the Media Session Handler or Media AS to submit a consumption report to the Media AF If several Media AF addresses are listed in the serverAddresses array of the Client Consumption Reporting Configuration (see table 9.2.3.1-1), the Media Session Handler shall choose one at random and shall send the consumption report to the selected server endpoint. The HTTP POST method shall be used for this purpose, citing the address of the chosen Media AF in the request URL. The request body shall be a ConsumptionReport structure, as specified in clause 9.6.3.1.

A reporting client identifier shall be included in the consumption report. If available to the Media Session Handler, its value should be a GPSI value as defined by TS 23.003 [23003]. Otherwise, the reporting client identifier should be represented by a stable and globally unique string.

If the operation is successful, the Media AF shall return a 200 (OK) HTTP response message with an empty body to acknowledge successful processing of the consumption report.

If the target Media AF endpoint is temporarily unavailable, the Media Client should store outstanding consumption reports and reattempt submission when the endpoint subsequently becomes available	. Details are left to implementation.

## 5.4	UE media session handling (M6, M11) procedures

Editor’s Note: aligned with 4.8 of TS 26.512, but not restricted to downlink streaming.

### 5.4.1	General

This clause specfies the procedures for interactions between the Media-aware Application and the Media Session Handler at reference point M6 and interactions between the Media Access Function and the Media Session Handler at reference point M11. Details are provided in clause 12.

### 5.4.2	Dynamic Policy procedures

At the start of a media delivery session, the Media Session Handler shall determine the external reference and target QoS parameters of the initial Service Operation Point by invoking an appropriate API method on the Media Session Handler at reference point M11. Based on the parameter values supplied, the Media Session Handler shall attempt to instantiate a Dynamic Policy satisfying the Media Access Function's requirements using the procedure specified in clause 5.3.3.2 if the target QoS lies within the bounds of a Policy Template with the corresponding external reference advertised in the Service Access Information for the media delivery session.

The Media Session Handler shall subscribe to receive notifications from the Media Access Function at reference point M11 of changes to the Service Operation Point during the course of the media delivery session. When such a change occurs (e.g., when the Media Access Function selects a different MPEG-DASH Representation), the Media Access Function shall send a notification to the Media Session Handler at reference point M11 citing the external reference and target QoS parameters of the new Service Operation Point. If the QoS of the new Service Operation Point is not satisfied by the currently instantiated Dynamic Policy, the Media Session Handler shall attempt to instantiate a Dynamic Policy satisfying the Media Access Function's requirements using the procedure specified in clause 5.3.3.2 if the target QoS lies within the bounds of a Policy Template with the corresponding external reference advertised in the Service Access Information for the media delivery session.

The Media-aware Application shall subscribe to receive notifications from the Media Session Handler at reference point M6 concerning Background Data Transfer opportunities. When such an opportunity is announced to the Media Session Handler by the Media AF at reference point M5, the Media Session Handler shall send a corresponding notification to the Media-aware Application at reference point M6 that includes an estimate of the opportunity window. If it wishes to avail itself of the Background Data Transfer opportunity, the Media-aware Application shall invoke a suitable API method on the Media Session Handler at reference point M6, providing an estimate of the data volume it intends to transfer over reference point M4. The Media Session Handler shall then attempt to instantiate a Dynamic Policy with Background Data Transfer network characteristics (including the data volume estimate supplied by the Media-aware Application) using the procedure specified in clause 5.3.3.2 if a suitable Policy Template is advertised in the Service Access Information for the media delivery session.

### 5.4.3	Network Assistance procedures

At the start of a media delivery session, the Media Session Handler shall determine an initial bit rate recommendation by invoking an appropriate API method on the Media Session Handler at reference point M11. The Media Session Handler shall invoke the procedure specified in clause 5.3.4.2 to create a new Network Assistance Session and shall return to the Media Access Function the bit rate recommendation information in the response from the Media AF. The Media Access Function may use this information to set an initial bit rate for the media delivery session (e.g., by selecting an initial MPEG-DASH Representation or contribution media encoding bit rate). The Media Access Function may also use this bit rate recommendation to select a different Service Operation Point, resulting in a change to the current Dynamic Policy (see clause 5.4.2).

The Media Session Handler shall subscribe to receive notifications from the Media Access Function at reference point M11 of bit rate recommendation updates. When such an update is received from the Media AF, the Media Session Handler shall send a notification to the Media Access Function at reference point M11 providing the new bit rate recommendation. As a result, the Media Access Function may modify the bit rate of the media delivery session (e.g., by switching to a different MPEG-DASH Representation or by varying the rate controller of a contribution media encoder). The Media Access Function may also use this bit rate recommendation to select a different Service Operation Point, resulting in a change to the current Dynamic Policy (see clause 5.4.2).

During the course of the media delivery session the Media Access Function may request a delivery boost by invoking an appropriate API method on the Media Session Handler at reference point M11.

### 5.4.4	Metrics reporting procedures

When metrics reporting is active for a given media delivery session, the Media Session Handler shall periodically sample the metrics required by the metrics scheme(s) configured per clause 5.3.5.1 by invoking an appropriate API method at reference point M11 on the Media Access Function and shall use these to populate metrics reports.

### 5.4.5	Consumption reporting procedures

Before a streaming session is started, the Media Session Handler shall check if the Service Access Information contains any Consumption reporting configuration, as specified in clause 4.7.3. If such a configuration is present, the Media Session Handler shall initiate consumption reporting based on this configuration for the current streaming session.

When consumption reporting is active for a given media delivery session, the Media Session Handler shall periocally determine the consumption reporting parameters required per clause 5.4.6.1 by invoking an appropriate API method on the Media Access Function at reference point M11 and shall use these to populate consumption reports.

# 6	3GPP Service URL

Editor’s Note: General design of 3GPP Service URLs, which may be factored out into its own TS in future.

# 7	General aspects of APIs

Editor’s Note: port clause 6 from TS 26.512.

## 7.1	Usage of HTTP

### 7.2.1	HTTP protocol version

Implementations of the Media AF shall expose both HTTP/1.1 [HTTP11] and HTTP/2 [HTTP2] endpoints at reference points M1 and M5, including support for the HTTP/2 starting mechanisms specified in section 3 of RFC 7540 [HTTP2]. In both protocol versions, TLS version 1.3 [TLS13] shall be supported and HTTPS interactions should be used on these interfaces in preference to cleartext HTTP.

The Media Application Provider may use any supported HTTP protocol version at reference point M1.

The Media Session Handler may use any supported HTTP protocol version at reference point M5.

All responses from the Media AF that carry a message body shall provide a strong entity tag in the form of an ETag response header and a modification timestamp in the form of a Last-Modified response header.

All endpoints at reference points M1 and M5 shall support the conditional HTTP requests If-None-Match and If-Modified-Since.

## 7.2.2	HTTP resource URIs and paths

The resource URI used in each HTTP request to the API provider shall have the structure defined in subclause 4.4.1 of TS 29.501 [TS29501], i.e.:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}

with the following components:

-	{apiRoot} shall be set as described in [TS29501].

-	{apiName} shall be set as defined by the following clauses.

-	{apiVersion} shall be set to "v2" in this release of the specification.

-	{apiSpecificResourceUriPart} shall be set as described in the following clauses.

### 7.2.3	Usage of HTTP headers

#### 7.2.3.1	General

Standard HTTP headers shall be used in accordance with clause 5.2.2 of TS 29.500 [29500] for both HTTP/1.1 and HTTP/2 messages.

#### 7.2.3.2	User Agent identification

Individual specifications referencing the present document shall specify the User-Agent HTTP request header used by the Media Session Handler to identify itself.

#### 7.2.3.3	Server identification

Individual specifications referencing the present document shall specify the Server HTTP response header used by the Media AF to identify itself.

#### 7.2.3.2	Support for conditional HTTP GET requests

All responses from the Media AF that carry a resource message body shall include:

-	a strong entity tag for the resource, conveyed in an ETag response header,

-	a resource modification timestamp, conveyed in a Last-Modified response header, and

-	a predicted time-to-live period for the resource, conveyed in a Cache-Control: max-age response header.

All API endpoints on the Media AF that expose the HTTP GET method shall support conditional requests using the If-None-Match and If-Modified-Since request headers. API clients should not attempt to revalidate their cached copy of a resource using a conditional GET request before the indicated time-to-live period has elapsed.

#### 7.2.3.5	Support for conditional HTTP POST, PUT, PATCH and DELETE requests

All API endpoints on the Media AF that expose the HTTP POST, PUT, PATCH or DELETE methods shall support conditional requests using the If-Match request header. The API client should supply a strong entity tag in an ETag request header when invoking any of these HTTP methods.

### 7.2.4	HTTP message bodies for API resources

The OpenAPI [OpenAPI300] specification of request and response HTTP messages is contained in annex C.

## 7.2.5	HTTP response codes

Guidelines for error responses to the invocation of APIs of NF services are specified in clause 4.8 of TS 29.501 [29501]. API-specific error responses are specified in the respective clauses of the present document.

### 7.2.6	HTTP error response message bodies

Error messages shall be conveyed to the API client using an HTTP response body of type ProblemDetails, as specified in clause 4.8.2 of TS 29.501 [29501].

## 7.3	Common API data types

### 7.3.1	General

The data types defined in this clause are intended to be used by more than one of the Media Delivery APIs.

### 7.3.2	Simple data types

### 7.3.3	Structured data types

### 7.3.4	Enumerated data types

## 7.4	Explanation of API data model notation

The data models in the following API clauses are specified using the following notational conventions:

1.	Data models are expressed as an unordered list of JSON properties [38] with one property defined in each row of the data model table.

2.	The Data type column defines the type of the property, according to JSON notation [38].

3.	The keyword "Array" in the Data type column indicates that zero or more elements of the data type in brackets are included. The number of elements in the array may additionally be constrained by normative text in the Description column.

4.	The Cardinality column defines whether a property is optional or mandatory. An array with cardinality 0 indicates that the array property is optional in the data structure. An array with cardinality 1 indicates that the property is mandatory in the data structure, even when the array is empty.

5.	The keyword "Object" in the Data type column indicates a structured sub-object of an unnamed type whose properties are defined inline in the indented table rows immediately afterwards. The "Object" type may be combined with the "Array" type.

6.	In the case of data types specifying RESTful resources, the additional Usage column defines the property behaviour for each CRUD Operation as follows:

-	"C" (Create), "R" (Read) and "U" (Update) refers to the CRUD procedure during which the property is present in the resource type. (The Delete operation never takes any input data type.)

-	"RO" signifies a read-only property. Only the API provider function is permitted to modify the property value. The API invoker can only read the value.

-	"RW" signifies a read/write property. The API provider and API invoker may both modify the property value.

7.	An additional read-only property is included at the start of all data models defining resources that are members of a RESTful collection. This property is populated with the unique identifier of the resource within its parent collection, and corresponds to the leaf path element in the RESTful URL of that resource.

# 8	Provisioning (M1) APIs

Editor’s Note: port clause 7 from TS 26.512.

## 8.1	General

## 8.2	Content protocols discovery API

## 8.3	Provisioning Sessions API

## 8.4	Server Certificates provisioning API

## 8.5	Content Preparation Templates provisioning API

## 8.6	Edge Resources provisioning API

## 8.7	Policy Templates provisioning API

## 8.8	Content Hosting provisioning API

## 8.9	Content Publishing provisioning API

## 8.10	Metrics reporting provisioning API

## 8.11	Consumption reporting provisioning API

## 8.12	Event data processing provisioning API

# 9	Network media session handling (M3, M5) APIs

Editor’s Note: port clause 11 from TS 26.512.

## 9.1	General

## 9.2	Service Access Information API

## 9.3	Dynamic Policies API

## 9.4	Network Assistance API

## 9.5	Metrics Reporting API

## 9.6	Consumption Reporting API

# 10	UE media session handling (M6) APIs

## 10.1	General

Editor’s Note: port clause 12.1 from TS 26.512.

## 10.2	Session launch mechanism

### 10.2.1	5G Media Streaming session launch mechanism

#### 10.2.1.1	3GPP Service URL format for 5GMS

Editor’s Note: New Clause.

### 10.2.2	webRTC session launch mechanism

Editor’s Note: New Clause.

## 10.3	Media session handling APIs for downlink media streaming

Editor’s Note: port clause 12.2 from TS 26.512. (Functional model in 12.2.1 should be migrated to TS 26.501.)

## 10.4	Media session handling APIs for uplink media streaming

Editor’s Note: port clause 12.3 from TS 26.512.

## 10.5	Media session handling APIs for webRTC

Editor’s Note: New Clause.

# 11	Usage of existing APIs

## 11.1	Introduction

While the core functionality of 5GMS is specified in terms of the dedicated system interfaces and APIs that impact the UE and the network, specified in clauses 7 to 9, certain features of OPA rely on interfaces and APIs that are either defined in the 5G Core or internal to the UE.

## 11.2	Usage of 5GC service APIs

Editor’s Note: port clause 16 from TS 26.512.

## 11.3	Usage of miscellaneous UE-internal APIs

Editor’s Note: port clause 15 from TS 26.512.

Annex A (informative):
Index of HTTP REST APIs

Editor’s Note: port annex D from TS 26.512.

Annex B (normative):
OpenAPI representation of HTTP REST APIs

Editor’s Note: port annex C from TS 26.512.

Annex C (normative):
Controlled vocabularies of UE data parameters

Editor’s Note: port annex E from TS 26.512.

Annex <F> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-08-24 | SA4#125 | S4-231500 |  |  |  | Initial document skeleton proposal. | 0.0.1 |
| 2023-10-24 | ad hoc post SA4#125 | S4aI230151 |  |  |  | Added assigned TS number to cover page. Ported provisioning and session handling operations to clause 5. | 0.1.0 |