| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev |  | Current version: | 0.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoE metrics reporting procedure for RTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 7th November 2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This pCR proposes the list of QoE metrics that needs to be reported by an RTC UE and the quality metrics reporting protocol and the format. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Update to clause 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

The 5GMS protocols specification TS 26.512 discusses the procedures and APIs for metric measurement, collection and reporting in the 5G system networks for 5GMS services. The TS 26.512 does not address the procedures and APIs for Quality of Experience (QoE) metric measurement, collection and reporting in the 5G system networks for real-time media communication services which are defined in TS 26.506.

The TS 26.234 specification describes the list of QoE metrics used for progressive download and DASH streaming services. The specification also provides the Quality reporting scheme for configuring the UE to send the QoE metrics reports and Quality reporting protocol for reporting the QoE metrics by the UE to the Metrics server.

This contribution proposes the QoE metrics to be reported to the metric collection server by the 5G real-time media communication (5G-RTC) UE or endpoint. This contribution also proposes the QoE metrics reporting protocol for sending the metrics by the 5G-RTC UE to the metrics collection server.

# Changes

First Change

# Control transport interface (RTC-5)

## Metrics reporting

### General

The Metrics Reporting API allows the Media Session Handler to send QoE metrics reports to the RTC AF. The metrics reporting procedure is as defined in clause 11.4.2 of TS 26.512.

A 5G-RTC UE supporting Quality of Experience (QoE) shall report QoE metrics according to the QoE configuration. QoE reporting is optional, but if a RTC UE reports QoE metrics, it shall report all requested metrics.

### Quality of Experience metrics definition

#### Introduction

This clause provides the general QoE metric definitions and measurement framework. A 5G-RTC UE supporting the QoE metrics feature shall support the reporting of the metrics in this clause. The metrics are valid for speech, video and text media, and are calculated for each measurement resolution interval "measureinterval". They are reported to the server according to the measurement reporting interval "reportinginterval" and after the end of the session.

The optional "measureinterval" field, if used, shall define a time over which each metrics value is calculated. The "measureinterval" field splits the session duration into a number of equally sized periods where each period is of the length specified by the "measureinterval" field. The "measureinterval" field is thus defining the time before the calculation of a QoE parameter starts over. If the "measureinterval" field is not present, the metrics resolution shall cover the period specified by the "measurerange" field. If the "measurerange" field is not present the metrics resolution shall be for the whole session duration.

The optional "measurerange" field, if used, shall define the time range in the stream for which the QoE metrics will be reported. There shall be only one range per measurement specification. The range format shall be any of the formats allowed by the media. If the "measurerange" field is not present, the metrics range shall be the whole call duration.

There are two kinds of timestamp defined i.e. real time (wall-clock time) and media time.

#### Corruption duration metric

Corruption duration, M, is the time period from the NPT time of the last good frame (since the NPT time for the first corrupted frame cannot always be determined) before the corruption, to the NPT time of the first subsequent good frame. A corrupted frame may either be an entirely lost frame, or a media frame that has quality degradation and the decoded frame is not the same as in error-free decoding.

A good frame is a completely received frame:

-	where all parts of the image are guaranteed to contain the correct content; or

-	that is a refresh frame, that is, does not reference any previously decoded frames; or

-	which only references previously decoded good frames

Completely received means that all the bits are received and no bit error has occurred.

Corruption duration, M, in milliseconds can be calculated as below:

a)	M can be derived by the client using the codec layer, in which case the codec layer signals the decoding of a good frame to the client. A good frame could also be derived by error tracking methods, but decoding quality evaluation methods shall not be used.

b)	Alternatively, the corruption is considered as ended after N milliseconds with consecutively completely received frames, or when a refresh frame has been completely received, whichever comes first.

The optional configuration parameter N can be set to define the average characteristics of the codec. If N has not been configured it shall default to the length of one measurement interval for video media, and to one frame duration for non-video media.

The N parameter is specified in milliseconds and is used with the "CorruptionDuration" parameter. The value of N may be set by the server.

All the occurred corruption durations within each measurement period are summed and stored in the vector TotalCorruptionDuration. The unit of this metrics is expressed in milliseconds. Within each measurement period the number of individual corruption events are summed up and stored in the vector NumberOfCorruptionEvents.

The syntax for the metric "CurruptionDuration" is as defined in Table 1

Table 1: Corruption duration metric information for Quality Reporting

| Key |  | Type | Description |
| --- | --- | --- | --- |
| CorruptionDuration |  | Object |  |
|  | totalCorruptionDuration | unsignedLongVectorType | An unordered list of all occurred corrupt durations within each measurement period. |
|  | numberOfCorruptionEvents | unsignedLongVectorType | An unordered list of corruption events occurred within each measurement period. Within each measurement period the number of individual corruption events are summed up and stored. |

#### Successive loss of RTP packets

The metric "SuccessiveLoss" indicates the number of RTP packets lost in succession per media channel.

All the number of successively lost RTP packets are summed up within each measurement resolution period of the stream and stored in the vector TotalNumberofSuccessivePacketLoss. The unit of this metric is expressed as an integer equal to or larger than 0. The number of individual successive packet loss events within each measurement resolution period are summed up and stored in the vector NumberOfSuccessiveLossEvents. The number of received packets are also summed up within each measurement resolution period and stored in the vector NumberOfReceivedPackets. These three vectors are reported by the 5G-RTC UE as part of the QoE report.

The syntax for the metric "SuccessiveLoss" is as defined in Table 2.

Table 2: Successive loss of RTP packets metric information for Quality Reporting

| Key |  | Type | Description |
| --- | --- | --- | --- |
| SuccessiveLoss |  | Object |  |
|  | TotalNumberofSuccessivePacketLoss | unsignedLongVectorType | An unordered list of all successively lost RTP packets within each measurement period. |
|  | NumberOfSuccessiveLossEvents | unsignedLongVectorType | The number of individual successive packet loss events within each measurement resolution period are summed up and stored in the vector. Provides an unordered list of successive packet loss events (occurred within each measurement period) measured during a metric reporting period. |
|  | NumberOfReceivedPackets | unsignedLongVectorType | The number of received packets are summed up within each measurement resolution period and stored in the vector. |

#### Frame rate

Frame rate indicates the playback frame rate. The playback frame rate is equal to the number of frames displayed during the measurement resolution period divided by the time duration, in seconds, of the measurement resolution period.

For the Metrics-Name "framerate", the value field indicates the frame rate value. This metric is expressed in frames per second and can be a fractional value. The frame rates for each resolution period are stored in the vector framerate and reported by the 5G-RTC UE as part of the QoE report.

The syntax for the metric "framerate" metric is as defined in Table 3

Table 3: framerate metric for Quality Reporting

| Key | Type | Description |
| --- | --- | --- |
| framerate | doubleVectorType | An unordered list of framerate values reported over a reporting period. The frame rates for each metric resolution period are stored in the vector. |

#### Jitter duration

Jitter happens when the absolute difference between the actual playback time and the expected playback time is larger than Jitterthreshold milliseconds. The expected time of a frame is equal to the actual playback time of the last played frame plus the difference between the NPT time of the frame and the NPT time of the last played frame.

The optional configuration parameter Jitterthreshold can be set to control the amount of allowed jitter. If the parameter has not been set, it defaults to 100 ms. The Jitterthreshold parameter is specified in milliseconds and is used with the "JitterDuration" parameter. The value of Jitterthreshold may be set by the server.

All the jitter durations are summed up within each measurement resolution period and stored in the vector TotalJitterDuration. The unit of this metric is expressed in seconds and can be a fractional value. The number of individual events within the measurement resolution period are summed up and stored in the vector NumberOfJitterEvents. These two vectors are reported by the 5G-RTC UE as part of the QoE report.

The syntax for the metric "JitterDuration" is as defined in Table 4

Table 4: Jitter duration metric information for Quality Reporting

| Key |  | Type | Description |
| --- | --- | --- | --- |
| JitterDuration |  | Object |  |
|  | TotalJitterDuration | doubleVectorType | All the jitter durations are summed up within each measurement resolution period and stored in the vector. |
|  | NumberOfJitterEvents | unsignedLongVectorType | The number of individual events within the measurement resolution period are summed up and stored in the vector. Provides An unordered list of jitter events (occurred within each measurement period) measured during a metric reporting period. |

#### Sync loss duration

Sync loss happens when the absolute difference between value A and value B is larger than SyncThreshold milliseconds. Value A represents the difference between the playback time of the last played frame of the video stream and the playback time of the last played frame of the speech/audio stream. Value B represents the difference between the expected playback time of the last played frame of the video stream and the expected playback time of the last played frame of the speech/audio stream.

The optional configuration parameter syncthreshold can be set to control the amount of allowed sync mismatch. If the parameter has not been set, it defaults to 100 ms. The syncthreshold parameter is specified in milliseconds and is used with the "SynclossDuration" parameter. The value of syncthreshold may be set by the server.

All the sync loss durations are summed up within each measurement resolution period and stored in the vector TotalSyncLossDuration. The unit of this metric is expressed in seconds and can be a fractional value. The number of individual events within the measurement resolution period are summed up and stored in the vector NumberOfSyncLossEvents. These two vectors are reported by the 5G-RTC UE/endpoint as part of the QoE report.

The syntax for the metric "SynclossDuration" is as defined in Table 5

Table 5: Syncloss duration metric information for Quality Reporting

| Key |  | Type | Description |
| --- | --- | --- | --- |
| SynclossDuration |  | Object |  |
|  | TotalSyncLossDuration | doubleVectorType | All the sync loss durations are summed up within each measurement resolution period and stored in the vector. |
|  | NumberOfSyncLossEvents | unsignedLongVectorType | The number of individual sync loss events within the measurement resolution period are summed up and stored in the vector. Provides An unordered list of sync loss events (occurred within each measurement period) measured during a metric reporting period. |

#### Round-trip time

The round-trip time (RTT) consists of the RTP-level round-trip time, plus the additional two-way delay due to buffering and other processing in each RTC UE.

The last RTCP round-trip time value estimated during each measurement resolution period shall be stored in the vector NetworkRTT. The unit of this metrics is expressed in milliseconds.

The two-way additional internal client delay valid at the end of each measurement resolution period shall be stored in the vector InternalRTT. The unit of this metrics is expressed in milliseconds.

The two vectors are reported by the 5G-RTC UE as part of the QoE report.

The syntax for the metric "RoundtripTime" is as defined in Table 6

Table 6: Round-trip time metric information for Quality Reporting

| Key |  | Type | Description |
| --- | --- | --- | --- |
| RoundtripTime |  | Object |  |
|  | NetworkRTT | unsignedLongVectorType | The last RTCP round-trip time value estimated during each measurement resolution period shall be stored in the vector. |
|  | InternalRTT | unsignedLongVectorType | The two-way additional internal client delay valid at the end of each measurement resolution period shall be stored in the vector. |

#### Average codec bitrate

The average codec bitrate is the bitrate used for coding "active" media information during the measurement resolution period.

For speech media the average codec bitrate can be calculated as the number of "active" speech bits received for "active" frames divided by the total time, in seconds, covered by these frames. The total time covered is calculated as the number of "active" frames times the length of each speech frame.

For non-speech media the average codec bitrate is the total number of RTP payload bits received, divided by the length of the measurement resolution period.

The average codec bitrate value for each measurement resolution period shall be stored in the vector AverageCodecBitrate. The unit of this metrics is expressed in kbit/s and can be a fractional value. The vector is reported by the 5G-RTC UE/endpoint as part of the QoE report.

The syntax for the metric "AverageBitrate " is as defined in Table 7

Table 7: Syncloss duration metric information for Quality Reporting

| Key |  | Type | Description |
| --- | --- | --- | --- |
| AverageBitrate |  | Object |  |
|  | AverageCodecBitrate | doubleVectorType | The average codec bitrate value for each measurement resolution period shall be stored in the vector. |

### Quality metrics reporting protocol

#### General

The quality metrics reporting protocol consists of:

- The XML-based report format defined in clause 9.1.3.2.

- The reporting protocol defined in clause 9.1.3.3.

The MIME type of an XML-formatted QoE report shall be "application/3gprtc-qoe-report+xml".

#### Report format

The QoE report is formatted as an XML document that complies with the XML schema in Table 8.

Table 8: QoE Report XML schema

<?xml version="1.0"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2023:RTC:receptionreport" xmlns:sv="urn:3gpp:metadata:2016:PSS:schemaVersion" xmlns="urn:3gpp:metadata:2023:RTC:receptionreport" elementFormDefault="qualified"> <xs:element name="ReceptionReport" type="ReceptionReportType"/> <xs:complexType name="ReceptionReportType"> <xs:choice> <xs:element name="QoeReport" type="QoeReportType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="contentURI" type="xs:anyURI" use="required"/> <xs:attribute name="clientID" type="xs:string" use="optional"/> </xs:complexType> <xs:complexType name="QoeReportType"> <xs:sequence> <xs:element name="QoeMetric" type="QoeMetricType" minOccurs="1" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="periodID" type="xs:string" use="required"/> <xs:attribute name="reportTime" type="xs:dateTime" use="required"/> <xs:attribute name="reportPeriod" type="xs:unsignedInt" use="required"/> <xs:attribute name="mediaid" type="xs:unsignedInt" use="optional"/> <xs:attribute name="qoeReferenceId" type="xs:hexBinary" use="optional"/> <xs:attribute name="recordingSessionId" type="xs:hexBinary" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="QoeMetricType"> <xs:choice> <xs:element name="CorruptionDuration" type="CurruptionDurationType"/> <xs:element name="SuccessiveLoss" type="SuccessiveLossType" /> <xs:element name="framerate" type="doubleVectorType"/> <xs:element name="JitterDuration" type=" JitterDurationType"/> <xs:element name="Syncloss" type=" SynclossType"/> <xs:element name="RoundtripTime" type="RoundtripTimeType"/> <xs:element name="AverageBitrate" type="AverageBitrateType"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="CurruptionDurationType"> <xs:attribute name="totalCorruptionDuration" type="unsignedLongVectorType" use="required"/> <xs:attribute name="numberOfCorruptionEvents" type="unsignedLongVectorType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="SuccessiveLossType"> <xs:attribute name="TotalNumberofSuccessivePacketLoss" type="unsignedLongVectorType" use="required"/> <xs:attribute name="NumberOfSuccessiveLossEvents" type="unsignedLongVectorType" use="required"/> <xs:attribute name="NumberOfReceivedPackets" type="unsignedLongVectorType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="JitterDurationType"> <xs:attribute name="TotalJitterDuration" type="doubleVectorType" use="required"/> <xs:attribute name="NumberOfJitterEvents" type="unsignedLongVectorType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="SynclossDurationType"> <xs:attribute name="TotalSyncLossDuration" type="doubleVectorType" use="required"/> <xs:attribute name="NumberOfSyncLossEvents" type="unsignedLongVectorType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RoundtripTimeType"> <xs:attribute name="NetworkRTT" type="unsignedLongVectorType" use="required"/> <xs:attribute name="InternalRTT" type="unsignedLongVectorType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name=" AverageBitrateType"> <xs:attribute name="AverageCodecBitrate" type="doubleVectorType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="unsignedLongVectorType"> <xs:list itemType="xs:unsignedLong"/> </xs:simpleType> <xs:simpleType name="doubleVectorType"> <xs:list itemType="xs:double"/> </xs:simpleType> <xs:simpleType name="StringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType> <xs:simpleType name="UnsignedIntVectorType"> <xs:list itemType="xs:unsignedInt"/> </xs:simpleType> </xs:schema>

#### Reporting protocol

The metrics reporting protocol is as defined in clause 11.4.3 of TS 26.512. For real-time media communication, clauses 9.1.3.1 and 9.1.3.2 specify the required MIME content type and metrics report format for the 3GPP urn:‌3GPP:‌ns:‌PSS:‌RTC:‌QM1 metrics reporting scheme.

For configuration done via the QMC functionality, the client shall also send QoE reports via the QMC functionality. For OMA-DM configuration, if a specific metrics server has been configured, the client shall send QoE reports using the HTTP (RFC 2616) POST request carrying XML formatted metadata in its body.

An example QoE reporting based on HTTP POST request signalling is as shown in Table 9.

Table 9: QoE Report HTTP POST format

POST http://www.exampleserver.com HTTP/1.1 Host: 192.68.1.1 User-Agent: Mozilla/4.0 (compatible; MSIE 8.0; Windows NT 6.1; Trident/4.0) Content-Type: text/xml; charset=utf-8 Content-Length: 4408

<?xml version="1.0"?> <ReceptionReport contentURI="http://www.example.com/content/content.sdp" clientID="35848574673" xmlns="urn:3gpp:metadata:2023:RTC:receptionreport" xsi:schemaLocation="urn:3gpp:metadata:2023:RTC:receptionreport RTC-QoE-Report.xsd" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"> <QoeReport periodID="Period1" reportTime="2011-02-16T09:00:00" reportPeriod="500"> <QoeMetric> <CorruptionDuration> totalCorruptionDuration="480 0 120" numberOfCorruptionEvents="5 0 2"</CorruptionDuration> </QoeMetric> <QoeMetric> framerate="28.0 30.0 29.8"</framerate> </QoeMetric> <QoeMetric> <JitterDuration> TotalJitterDuration="0 0.346 0" NumberOfJitterEvents="0 1 0"</JitterDuration> </QoeMetric> </QoeReport> <QoeReport periodID="Period2" reportTime="2011-02-16T09:08:20" reportPeriod="500"> <QoeMetric> <CorruptionDuration> totalCorruptionDuration="83 0 0" numberOfCorruptionEvents="1 0 0"</CorruptionDuration> </QoeMetric> <QoeMetric> <JitterDuration> TotalJitterDuration="0 0 0" NumberOfJitterEvents="0 0 0"</JitterDuration> </QoeMetric> </QoeReport> </ReceptionReport>

End of First Change

# Proposal

We propose to agree the proposed change in clause 2 into clause 9 of TS 26.113.