Title:	Update of TR 25.933 chapter 6

Source:	Beijing Xiaomi Mobile Software Co., Ltd, xxx

Document for:	Discussion &Agreement

Agenda Item:	7.8

# Introduction

According to the discussion during the SA4 post-127 telco meeting, this proposal aims to refine the chapter 6 and remove some descriptions about microphone in chapter 4 if the former part is agreed.

# Content

### 6.3.3 Microphone Design for the Tablets

Tablets share a similar structure with mobile phones but are significantly larger in size. This allows for more flexibility in the placement of microphones, with some tablets accommodating up to five microphones.

Small microphone arrays are also a common feature on tablets. However, due to the less common handhold mode in tablets, microphones near the USB port - the closest location to the mouth in handhold mode - are rarely seen. Instead, microphones are typically located near the front and back camera modules and along the edges of the tablet.

| Number of Microphones | Placement of Microphones |
| --- | --- |
| up to 5 microphones | 1-3 on the top of the devices (near the front camera) |
|  | 1 on near the back camera |
|  | 1 on the edge of device |

### 6.3.4 Microphone Design for the Laptops

Due to the clamshell structure of laptop, in current devices, there are more options for microphone placement. Some are positioned on the screen part, while others are located on the keyboard section.  More potential microphone location resulted in larger microphone distance, and moveable microphone location.

Some popular microphone placements on laptop are explained in the following figures:

Figure 6.3.4-1 Laptop microphone placement 1

Figure 6.3.4-2 Laptop microphone placement 2

Figure 6.3.4-3 Laptop microphone placement 3

Figure 6.3.4 Laptop microphone placement 4

### 6.3.5 Microphone Design for the Watches

Modern watch devices are increasingly equipped with microphones and speakers, enabling them to independently establish speech services.

Some up-to-date models often feature two built-in microphones. Typically, one microphone is located on one edge of the watch, while the second is situated on the opposite edge. The spacing between these microphones generally correlates with the size of the watch, ranging from 3-5cm.

## 6.4 Microphone Frequency Response

A significant rise in specific frequencies can often be observed in numerous MEMS microphones, exhibiting a curve like resonance.

The MEMS microphone package includes a port that allows external sound to enter, while the transducer is shielded by a can, which may be perceived as chambers. These components form a Helmholtz resonator, resulting in resonances. Numerous studies in this field have provided various formulas to calculate such resonators.

# Adjustments in chapter4

Since the chapter 6 discuss microphone design, some descriptions about microphone in chapter 4 are proposed to be removed.

## 4.4 Laptops

Compared to tablets, the primary distinction in laptops is their clamshell structure, featuring a hinged screen and an attached keyboard.

Also, most laptops have larger structure size that of mobile phones. The range in length goes from approximately 28 cm to around 41 cm, resulting in a range of about 13 cm. The widths span from about 18.5 cm to approximately 32 cm. The height range extends from a few tenths of a centimetre (around 0.7) to over 2.6 cm.

Figure 4.4-5 Laptop side view

## 4.5 Watches

Many watches nowadays also have calling capabilities. Nowadays, the smartwatches capable of making calls mainly come in two shapes: circular and rectangle.

# Conclusion

## References

- S4aA240015: TR 26.933 Review