| 3GPP TR 26.930 V0.3.0 (2024-01) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on the enhancement for Immersive Real-Time communication for WebRTC (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

1	Scope	11

2	References	12

3	Definitions of terms, symbols and abbreviations	14

3.1	Terms	14

3.2	Symbols	15

3.3	Abbreviations	16

4	Motivations for native WebRTC signalling and assumptions	16

4.1	General	16

4.2.	High-level network model and target interfaces	17

4.3	C-plane Signalling comparison	18

5	Key issues	19

5.1	General	19

5.2	Key Issue #1: Enhancements on RTC architecture	19

-	Study the expected architecture variant for the collaboration scenario 4 and enhancements on the current RTC architecture defined in 3GPP TS 26.506 [12], based on the gap analysis studied in 5). This also includes the clarifications on the focused functions and interfaces in this document.	20

5.3	Key Issue #2: Functional requirements for C-Plane	21

5.4	Key Issue #3: C-Plane signalling protocol	21

5.5	Key Issue #4: Functional requirements for U-plane	21

5.6	Key Issue #5: Functional requirements for service control API	21

5.6.1	General	21

5.6.2	RTC Application Provider	21

5.6.2.1	General	21

5.6.2.2	Service provider	22

5.6.2.3	Content provider	23

Figure 5.6.2.3-1: Content provider-operator connection and functional deployment diagram	24

5.6.3	Functional requirements for service control	29

5.6.3.1	General	29

5.6.3.2	CRUD of RTC ID resource and associated properties	29

5.6.3.3	User connection control using asserted identity	29

5.6.3.4	Media data forwarding control	29

5.6.4	Summary	30

SP performs service control using its own network functions. On the other hand, in the case of CP performing service control, at least three functions described in clause 5.6.3 should be provided through APIs to allow CP's service logic mangers to instruct and the operator's service logic enforcers to perform those functions. In this key issue, the procedures and APIs which enable CP to achieve above functionalities and related processes are to be studied.	30

5.7	Key Issue #6: WSF discovery mechanism	32

5.8	Key Issue #7: Interworking with IMS Network	33

5.9	Key Issue #8: Protocol-level interworking between RTC network and IMS network	33

5.10	Key Issue #9: Tethered cases	33

5.11	Key Issue #10: Security considerations	33

5.12	Key Issue #11: Related groups considerations	34

6	Solutions	34

6.1	General	34

6.2	Solution #1: Enhancements on RTC architecture	34

6.2.1	Solution description	34

6.2.2	Possible enhancements on functional entities and RTC architecture based on WebRTC viewpoint	35

6.2.2.1	Overview	35

6.2.2.2	Functional entities for WebRTC	35

6.2.2.2.1	General	35

6.2.2.2.2	Functional entities defined in WebRTC-related standardized specifications	36

6.2.2.2.2.1	UE (User Equipment)	36

6.2.2.2.2.1.1	General	36

6.2.2.2.2.1.2	Considerations specific to WebRTC endpoint types	36

6.2.2.2.2.2	WSF (WebRTC Signalling Function)	37

6.2.2.2.3	Functional entities widely implemented for WebRTC	38

6.2.2.2.3.1	WMCF (WebRTC Media Centre Function)	38

6.2.2.2.3.2	CSF (Conference Supporting Function)	38

6.2.2.2.4	Functional entities needed for inter-operator services	38

6.2.2.2.4.1	WNSGF (WebRTC NNI Signalling Gateway Function)	38

6.2.2.2.4.2	WNMGF (WebRTC NNI Media Gateway Function)	39

6.2.2.3	Reference Points	39

6.2.3	Interaction between functional entities in the enhanced RTC architecture and 5GC	39

6.2.3.1	Overview	39

6.2.3.2	Mapping of functional entities for interaction with 5GC	40

6.2.3.2.1	General	40

6.2.3.2.2	WSF and AF	40

6.2.3.2.3	WNSGF	40

6.2.3.2.3.1	Overview	40

6.2.3.2.3.2	WNSGF and NEF	40

6.2.3.2.3.3	WNSGF and SEPP	41

6.2.3.2.3.4	New functional entity	41

6.2.3.3	Possible architecture integrated with 5GC	41

6.2.3.4	Mapping to RTC collaboration scenarios	43

6.2.4	Media connection model	43

6.2.4.1	General	43

6.2.4.2	Target use cases from network view	44

6.2.4.3	QoS-enabled end-to-end path	50

6.2.5	IP Addressing	50

6.2.5.1	Overview	50

6.2.5.2	NAT	51

6.2.5.2.1	Overview	51

6.2.5.2.2	NAT Variation	51

6.2.5.2.3	Existing NAT-traversal	52

6.2.5.2.3.1	General	52

6.2.5.2.3.2	STUN	52

6.2.5.2.3.3	TURN	53

6.2.5.2.3.4	HNT	53

6.2.5.2.4	Conclusion of NAT handling	54

6.2.5.3	IP Address and Trustable Subscriber Identifier	54

6.2.5.4	Conclusion of IP Addressing	55

6.2.6	Alignment and gap analysis between the enhanced RTC architecture and the current RTC architecture	55

6.2.6.1	General	55

6.2.6.2	WebRTC endpoint and RTC endpoint on UE	56

6.2.6.3	WSF and (RTC) WSF	56

6.2.6.4	WNSGF and Inter-working Function	56

6.2.6.5	CSF and Application Supporting Web Function	57

6.2.6.6	WMCF and Media Function	57

6.2.6.7	WNMGF and Transport Gateway Function	57

6.2.7	Enhanced RTC Architecture for collaboration scenario 4	57

6.2.8	Proposed enhancements on RTC architecture	59

6.2.8.1	General	59

6.2.8.2	Derivative RTC architecture supporting collaboration scenario 3 and 4	60

6.2.8.3	Enhancements on functionality in RTC AS functional entities	61

6.2.8.3.1	General	61

6.2.8.3.2	User Equipment	61

6.2.8.3.3	WebRTC Signalling Function	62

6.2.8.3.4	Media Function	62

6.2.8.3.5	Application Supporting Web Function	63

6.2.8.3.6	Inter-working Function	63

6.2.8.3.7	Transport Gateway Function	63

6.2.8.4	Enhancements on reference points	64

6.2.8.5	Enhancements on architecture diagrams in 3GPP TS 26.506	64

6.2.9	Solution evaluation	66

6.3	Solution #2: Functional requirements for C-Plane	99

6.3.1	Solution description	99

6.3.2	Functional requirements for C-Plane interface	99

6.3.2.1	General	99

6.3.2.2	Support of WebRTC based RTC services	100

6.3.2.3	Transport of signalling message	100

6.3.2.4	Media session control and management	100

6.3.3	Protocol stack for C-Plane interface	101

6.3.3.1	General	101

6.3.3.2	Base protocol	101

6.3.3.3	Upper layer protocol over WebSocket	102

6.3.3.3.1	SIP	102

6.3.3.3.2	XMPP	102

6.3.3.3.3	Other existing implementations	102

6.3.3.4	Proposed Protocol Stack	102

6.3.4	Solution evaluation	103

6.4	Solution #3: C-Plane signalling protocol	108

6.5	Solution #4: Functional requirements for U-Plane	108

6.5.1	Solution description	108

6.5.2	Functional requirements for U-Plane interface	109

6.5.3	Protocol stack	109

6.5.4	Solution evaluation	110

6.6	Solution #5: Service control API	110

6.6.1	Solution Description	110

6.6.2	Procedures for service control	111

6.6.2.1	General	111

6.6.2.2	CP-oriented procedure	112

6.6.2.2.1	General	112

6.6.2.2.2	RTC ID resource management	112

6.6.2.2.2.1	General	112

6.6.2.2.2.2	Create RTC ID resource	113

6.6.2.2.2.3	Read RTC ID resource	113

6.6.2.2.2.4	Update RTC ID resource	113

6.6.2.2.2.5	Delete RTC ID resource	113

6.6.2.3	Callback procedure	113

6.6.2.3.1	General	113

6.6.2.3.2	Notification of RTC ID resource deletion	114

6.6.2.3.3	Notification of forced RTC ID resource deletion	114

6.6.2.3.4	Notification of RTC ID resource suspended	114

6.6.2.3.5	Notification of RTC ID resource resumed	115

6.6.2.3.6	 Notification of user call in requested	115

6.6.2.3.7	Notification of user call in accepted	115

6.6.2.3.8	Notification of user call in connected	115

6.6.2.3.9	Notification of user call disconnected	115

6.6.2.3.10	Notification of media routing query	115

6.7	Solution #6: WSF Discovery mechanism	116

6.7.1	Solution description	116

6.7.1.1	General	116

6.7.1.2	Analysis on possible mechanisms	116

6.7.2	Common URL based WSF discovery mechanism	117

6.7.2.1	General	117

6.7.2.2	Common URL format	117

6.7.2.3	Common URL based WSF discovery procedure	118

6.7.2.3.1	General	118

6.7.2.3.2	Protocol	118

6.7.2.3.3	Procedure	118

6.7.2.3.4	Definition of the HTTP response body for RESPECT	119

6.7.2.3.5	Common URL based WSF discovery flow example	119

6.7.3	Functional entity supporting WSF discovery function	120

6.7.4	Solution evaluation	120

6.8	Solution #7: Interworking with IMS Network	120

6.9	Solution #8: Protocol-level interworking between RTC network and IMS network	120

6.10	Solution #9: Tethered cases	120

6.11	Solution #10: Security considerations	120

6.11.1	Solution Description	120

6.11.2	Adaptation of the trust domain	121

6.11.3	Network-asserted identity within the trust domain	121

6.11.4	Adaptation of calling number verification using signature verification and attestation information	121

6.12	Solution #11: Related groups considerations	122

6.12.1	Solution description	122

6.12.2	Solution evaluation	123

7	Conclusions and Recommendations	123

7.1	General	123

7.2	Expected normative works in stage-2 aspect	123

Annex A (informative): Use cases	125

A.1	General	125

A.2	VR streaming in first person view	125

A.2.1	General	125

A.2.2	Service flows	125

A.3	VR streaming in third person view	126

A.3.1	General	126

A.3.2	Service flows	126

A.4	Voice communication and media analytics	127

A.4.1	General	127

A.4.2	Service flows	127

A.5	VR streaming over NNI	128

A.5.1	General	128

A.5.2	Service flows	128

A.6	Peer-to-peer communication	129

A.6.1	General	129

A.6.2	Service flows	129

Annex B (informative): Examples	130

Annex C (informative): Change history	131

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document extends immersive Real-time Communication for WebRTC (iRTCW) and introduces a new concept called native WebRTC signalling.

This document includes following aspects:

1.	Analysis of gaps and required enhancements of terminal device and network architectures including additional functional entities (e.g., WebRTC Signalling Server, ICE-STUN Server, IMS Interworking Gateway, NNI Gateway).

2.	Impacts and possible enhancements for the WebRTC-based U-plane components in terms of adaptation, media handling, and cross-layer optimizations over 5G systems.

3.	C-Plane signalling protocol details (e.g., based on JSON) for the common WebRTC-based immersive RTC session management.

4.	Information elements in the C/U-Plane signal (including NNI) to enhance connectivity of media sessions with carrier assistance for WebRTC-based applications (including OTT applications).

5.	Minimal functional capabilities needed to support the enhancements identified in 2, 3 and 4 (including transport, NAT-traversal, and XR conferencing), state transitions, and typical call flows.

6.	Consideration of collaboration formation with other WGs in 3GPP and SDOs including IETF and W3C.

7.	Enhancements for E2E QoS realizations over 5G systems for communications between MNOs and WebRTC clients operating over 5G access or non-5G access (e.g., Wi-Fi) using WebRTC-based transport. This also includes communication between WebRTC clients operating on tethering/tethered devices.

8.	Security aspects and rate adaptation in tethered use cases (including coordination of Uu and non-3GPP access).

One of the main scopes of this document is to study the enhancements of RTC network for supporting collaboration scenario 4 specified in 3GPP TS 26.506 [12]. Collaboration scenario 4 enables inter-operable WebRTC services, which is extended collaboration scenario 3 with functions to support MNO to MNO inter-operability. This collaboration scenario 4 could include the roaming case, but this case is outside the scope of this document.

NOTE:	There is a possibility that collaboration scenario 4 includes the roaming case in the future release work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[7]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[8]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[9]	3GPP TS 24.371: "Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3; Protocol specification".

[10]	3GPP TS 26.113: "Real-Time Media Communication; Protocols and APIs".

[11]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[12]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[13]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[14]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[15]	3GPP TS 29.162: "Interworking between the IM CN subsystem and IP networks".

[16]	3GPP TS 29.165: "Inter-IMS Network to Network Interface (NNI)".

[17]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[18]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[19]	IETF RFC 768: "User Datagram Protocol".

[20]	IETF RFC 791: "Internet Protocol".

[14]	IETF RFC 793: "Transmission Control Protocol".

[21]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[22]	IETF RFC 3324: "Short Term Requirements for Network Asserted Identity".

[23]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[24]	IETF RFC 3489: "STUN – Simple Traversal of User Datagram Protocol (UDP) Through Network Address Translators (NATs)".

[18]	IETF RFC 4566: "SDP: Session Description Protocol".

[25]	IETF RFC 5888: "The Session Description Protocol (SDP) Grouping Framework".

[26]	IETF RFC 6120: "Extensible Messaging and Presence Protocol (XMPP): Core".

[27]	IETF RFC 6455: "The WebSocket Protocol".

[28]	IETF RFC 6598: "IANA-Reserved IPv4 Prefix for Shared Address Space".

[29]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[30]	IETF RFC 7092: "A Taxonomy of Session Initiation Protocol (SIP) Back-to-Back User Agents".

[31]	IETF RFC 7362: "Latching: Hosted NAT Traversal (HNT) for Media in Real-Time Communication".

[32]	IETF RFC 7635: "Session Traversal Utilities for NAT (STUN) Extension for Third-Party Authorization".

[33]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6) Specification".

[34]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[35]	IETF RFC 8225: "PASSporT: Personal Assertion Token".

[36]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[37]	IETF RFC 8285: "A General Mechanism for RTP Header Extensions".

[38]	IETF RFC 8441: "Bootstrapping WebSockets with HTTP/2".

[39]	IETF RFC 8445: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal".

[40]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[41]	IETF RFC 8489: "Session Traversal Utilities for NAT (STUN)".

[42]	IETF RFC 8656: "Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)".

[43]	IETF RFC 8588: "Personal Assertion Token (PaSSporT) Extension for Signature-based Handling of Asserted information using toKENs (SHAKEN)".

[44]	IETF RFC 8825: "Overview: Real-Time Protocols for Browser-Based Applications".

[45]	IETF RFC 8826: "Security Considerations for WebRTC".

[46]	IETF RFC 8827: "WebRTC Security Architecture".

[47]	IETF RFC 8829: "JavaScript Session Establishment Protocol (JSEP)"

[48]	IETF RFC 8835: "Transports for WebRTC".

[49]	IETF RFC 8838: "Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol".

[50]	IETF RFC 8839: "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)".

[51]	IETF RFC 8859: "A Framework for Session Description Protocol (SDP) Attributes When Multiplexing".

[52]	IETF RFC 8864: "Negotiation Data Channels Using the Session Description Protocol (SDP)"

[53]	IETF RFC 8866: "SDP: Session Description Protocol".

[54]	IETF RFC 9110: "HTTP Semantics".

[55]	IETF RFC 9111: "HTTP Caching".

[56]	IETF RFC 9112: "HTTP/1.1".

[57]	IETF RFC 9113: "HTTP/2".

[58]	IETF RFC 9114: "HTTP/3".

[59]	IETF RFC 9143: "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)".

[60]	IETF RFC 9220: "Bootstrapping WebSockets with HTTP/3".

[61]	IETF RFC 9293: "Transmission Control Protocol (TCP)".

[62]	IETF RFC 9457: "Problem Details for HTTP APIs".

[63]	AsyncAPI Initiative "AsyncAPI Specification v3.0.0" https://github.com/asyncapi/spec/releases/tag/v3.0.0

[64]	OpenAPI Initiative "OpenAPI Specification v3.0.0" https://spec.openapis.org/oas/v3.0.0

[65]	W3C Proposed Recommendation, "WebRTC 1.0: Real-time Communication Between Browsers", <https://www.w3.org/TR/webrtc/>.

[45]	IETF RFC 8826: "Security Considerations for WebRTC".

[46]	IETF RFC 8827: "WebRTC Security Architecture".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Service provider (SP): An RTC application provider who provides RTC services with its own network. SP owns resources and functionalities within its network, and SP's network is connected to operator network via NNI.

Content Provider (CP): An RTC application provider who provide RTC services partially using operator's functionalities. CP connects to the operator network via UNI (RTC-4s/4m) as a WebRTC endpoint in order to use the operator's MF and WSF for the service specific content delivery.

Originating RTC network: A network which sends Media session set up request to another network (i.e., other operator network or SP network) over the NNI.

Originating UE: A RESPECT client on the UE which sends Media session set up request over the UNI.

RESPECT client: A RESPECT endpoint which acts as an WebSocket client.

RESPECT endpoint: A UE and RTC AS functionality supporting RESPECT protocol. RTC Aware Application on the UE or CP acting as UE, RTC AS for C-Plane signalling (i.e., WSF and IWF) that comply with RESPECT protocol are RESPECT endpoints. When distinction between UE/CP (acting as UE) and RTC AS is required, RESPECT endpoint (UE) or RESPECT endpoint (AS) is used respectively in this document.

RESPECT server: A RESPECT endpoint which acts as an WebSocket server.

RTC resource: A media which a media session is connected to. Media service such as conference room, media content for distribution and RESPECT endpoint are example of RTC resource.

RTC user: An RTC service user who connects to the RTC service by using RESPECT endpoint.

Service provider (SP): An RTC application provider who provides RTC services with its own network. SP owns resources and functionalities within its network, and SP's network is connected to operator network via NNI.

Terminating RTC network: A network which receives Media session set up request from another network (i.e., other operator network or SP network) over the NNI.

Terminating UE: A RESPECT client on the UE which receives Media session set up request over the UNI.

User Equipment (UE): It indicates the user equipment and servers acting as user equipment such as a content server of a content provider. User equipment includes an RESPECT endpoint.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 29.165 [16] apply:

Calling number verification using signature verification and attestation information

For the purposes of the present document, the following terms and definitions given in IETF RFC 6455 [27] apply:

WebSocket client

WebSocket server

For the purposes of the present document, the following terms and definitions given in IETF RFC 8825 [44] apply:

WebRTC Browser (also called a "WebRTC User Agent" or "WebRTC UA")

WebRTC Endpoint

WebRTC Non-Browser

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Rs-u	Reference Point between a WSF and a UE.

Rs-i	Reference Point between a WSF and another WSF in the same network (DN) or between a WSF and a WNSGF.

Rs-a	Reference Point between a WSF and a CSF.

Rs-n	Reference Point between a WNSGF and another WNSGF in an external network.

Rm-u	Reference Point between a WMCF and a UE.

Rm-i	Reference Point between a WMCF and another WMCF in the same network (DN) or between a WMCF and a WNMGF.

Rm-n	Reference Point between a WNMGF and another WNMGF in an external network.

Mc-i	Reference Point between a WSF and a WMCF.

Mc-r	Reference Point between a WNSGF and a WNMGF.

Rh-u	Reference Point between a CSF and UE. This reference point is used for providing CSF functionalities (e.g., application usage assistance such as downloading an application) to UE.

Rh-n	Reference Point between a CSF and Application service provider. This reference point is used for interaction between CSF and Application service provider for media session set up related interaction.

N5		Reference Point between a WSF and PCF.

RTC-X	Reference Point between a ASWF and application service provider.

RTC-4	Reference Point between an RTC network and a UE.

RTC-4m	Reference Point between a MF and a UE or between as ASWF and a UE..

RTC-4s	Reference Point between a WSF and a UE.

RTC-Y	Reference Point between two different operator's RTC networks or between an operator's RTC network and service provider's network.

RTC-Ym	Reference Point between a TGF and a TGF in another RTC network or a service provider's network.

RTC-Ys	Reference Point between a IWF and a IWF in another RTC network or a service provider's network.

RTC-Z	Reference Point between an RTC network and IMS.

RTC-Zm	Reference Point between a TGF and IMS.

RTC-Zs	Reference Point between a IWF and IMS.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ASWF	Application Supporting Web Function

CP	Content Provider

CSF	Conference Supporting Function

IBCF	Interconnection Border Control Function

IWF	Inter-working Function

MDFC	Media Data Forwarding Control

MF	Media Function

NNI	Network to Network Interface

RESPECT	REaltime&REality media Setup Protocol, Extensible and CompacT

SEPP	Security Edge Protection Proxy

SP	Service Provider

TGF	Transport Gateway Function

UNI	User to Network Interface

WMCF	WebRTC Media Centre Function

WNMGF	WebRTC NNI Media Gateway Function

WNSGF	WebRTC NNI Signalling Gateway Function

WSF	WebRTC Signalling Function

4	Motivations for native WebRTC signalling and assumptions

4.1	General

In 3GPP, the use of WebRTC technologies has been investigated since Release-12 (around 2014). They are a network-based architecture for WebRTC access to IMS specified in Annex U to 3GPP TS 23.228 [3] and its stage 3 has specified in 3GPP TS 24.371 [9]. They define functional entities including WIC (WebRTC IMS Client) and eP-CSCF (P-CSCF enhanced for WebRTC). The eP-CSCF is assumed to be located in the home IMS domain and communicates with other IMS entities using the existing interfaces. For the C-plane signalling between WIC and eP-CSCF, those specifications specify an option to use SIP over WebSocket, whose information model can be used for options other than SIP over WebSocket. However, a lot of real-time communication services are familiar with JSON based light weight signalling protocol which is flexible, extensible, and can be optimized for new XR conversational applications. These characteristics remind us of the original design principle of WebRTC. WebRTC, by its inherent characteristics, does not regulate C-plane signalling and allow a wide range of C-plane signalling. This document looks over this design principle again and investigates a new SIP-decoupled C-plane signalling, called native WebRTC.

Regarding the level of signalling details, 3GPP TS 24.371 [9] specifies a signalling transport mechanism using SIP over WebSocket, but it is not a mandatory mechanism for eP-SCSF. Even though there are other options such as XMPP or other application protocols over WebSocket, a RESTful based interface, etc., 3GPP TS 24.371 [9] does not specify any details of C-plane signalling using other options. Each service provider (e.g., operator) develops its own application by following the guidelines in 3GPP TS 24.371 [9]. Its subscriber downloads the application and connects to the service and other subscribers only within the same service. Detailed C-plane signalling is left open to each operator's design. In contrast, this document identifies a new C-plane signalling in detail (as an interface specification) to the extent that client implementations based on it have enough interoperability. This realizes connectivity to any operators or roaming services for new XR real-time communications. Operators can provide the interface common to them according to well-defined C-plane signalling specifications. Clients can connect to any operators via the interface (see Figure 4.1-1).

Figure 4.1-1:	Two approaches for defining specifications and their application connectivity

4.2.	High-level network model and target interfaces

The new C-Plane signalling protocol (i.e., RESPECT) studied in this document is intended for various media session control on the following interfaces:

-	UNI: The interface between operator network and UE (e.g., smart phone, content server of the content provider).

-	NNI: The interface between the two different operator networks, or that between operator network and service provider network.

A UE and a content provider can set up a media session by using RESPECT for session control on the UNI. A service operator can set up a media session by using RESPECT for session control on the NNI. Figure 4.2-1 shows the high-level network model indicating above interfaces and media sessions established via the functional entities supporting RESPECT (which described in clause 6.2) by using RESPECT.

There are following benefits to using RESPECT.

-	A UE (including the equipment of content provider) which is compliant with the RESPECT can connect to any operator network which supports the RESPECT and set up a media session in the operator network, based on the same signalling requirements.

-	A UE (including the equipment of content provider) which is compliant with the RESPECT can connect to services provided by other operator network or service provider network via NNI, based on the same signalling requirements.

-	Content Providers can set up an operator assisted media session (e.g., media session with QoS) with UEs connected to the Operator Network via the UNI, by connecting to the operator network via the UNI.-	Service providers can set up an operator assisted media session (e.g., media session with QoS) with UEs connected to the operator network via the UNI, by connecting to the operator network via the NNI.

Figure 4.2-1:	High-level network model and interfaces

<Terminology>

User Equipment (UE): It indicates the user equipment and servers acting as user equipment such as a content server of a content provider. User equipment includes an WebRTC endpoint supporting RESPECT.

Operator: Mobile and Fixed network operator who provides telecommunication services.

Service Provider (SP): 3rd party service provider who connects its service to operator network via NNI. OTT service is one of the typical services provided by service provider. Network Operator is excluded from the definition of this terminology in this document.

Content Provider (CP): 3rd party service provider who connects its service to operator network via UNI. Network Operator is excluded from the definition of this terminology in this document.

UNI: User-to-Network Interface. The interface between UE and Network.

NNI: Network-to-Network Interface. The interface between two different Networks.

4.3	C-plane Signalling comparison

The C-plane signalling can be expressed as follows. Now, there are roughly four possible methods, classified in terms of their protocol stacks (see Figure 4.3-1).

Figure 4.3-1:	Comparison of protocol stacks

The first method is MTSI-based, using SIP and SDP. General C-plane signalling requirements for conversational services can be covered by SIP. Interoperability is fine with the existing 5G core network. It is to be treated in IMS-based AR Conversational Services (IBACS).

The second is the method specified in 3GPP TS 24.371 [9]. It enables the WebRTC clients to communicate over an IMS-based core network; only the interfaces for downloading dedicated applications and the signalling path using WebSocket are specified for C-plane signalling. Ordinary implementations adopt SIP-like protocols over WebSocket. In most cases, it is partially SIP-compliant or tightly coupled with SIP to adapt WebRTC clients in IMS domain.

The third method is an alternative to the second method that uses SIP-like protocol over WebSocket. The third method uses another signalling protocol over WebSocket, but SIP-decoupled approaches are investigated. It can be more lightweight, omitting features that is not used in XR conversational. Some constraints on SDP are necessary for interoperability. Non-browser based implementations are also in the scope. This method is the main subject of this document.

The other is a general WebRTC protocol stack that is not specified and left open to the users (i.e., service providers). C-plane may be SIP, XMPP, http, etc. A general WebRTC application uses SDP syntax compliant to RFC 8866 [53] for its internal representation, when setting the local and remote descriptions. C-plane protocol may have its own on-the-wire format for SDP, which can be constructed from SDP and be serialized out to SDP.

5	Key issues

5.1	General

This clause describes the key issues of this document.

5.2	Key Issue #1: Enhancements on RTC architecture

As described in 3GPP TS 26.506 [12], the detailed scenario and the RTC architecture for collaboration scenario 4 is FFS. This key issue identifies the scenarios and the possible enhancements on RTC architecture to realize collaboration scenario 4 in addition to collaboration scenario 3, based on the high-level network model described in clause 4.2.

This key issue includes:

1)	Possible enhancements on functional entities and RTC architecture

-	Study possible enhancements on functional entities and RTC architecture for collaboration scenario 4 in addition to collaboration scenario 3 based on general WebRTC implementation viewpoint.

2)	Interaction with 5GC

-	To realize the QoS control, study the interaction between functional entities in the enhanced RTC architecture and those in 5GC.

3)	Media connection model

-	Study the target use cases (i.e., connection model) of user plane (U-Plane) and considerations of QoS-enabled end-to-end path.

4)	IP addressing

-	Study the considerations on IP addressing related issues and identify the possible additional enhancements of ICE functionality.

5)	Alignment and gap analysis between the enhanced RTC architectures and the current RTC architecture.

-	Study the alignment between the enhanced RTC architecture derived from 1) – 4) and the current RTC architecture defined in 3GPP TS 26.506 [12]. This also includes gap analysis of functionalities between the architectures.

6)	Enhanced architecture for collaboration scenario 4

-	Study the expected architecture variant for the collaboration scenario 4 and enhancements on the current RTC architecture defined in 3GPP TS 26.506 [12], based on the gap analysis studied in 5). This also includes the clarifications on the focused functions and interfaces in this document.

5.3	Key Issue #2: Functional requirements for C-Plane

This key issue identifies the functional requirements for C-Plane based on the architecture proposed in Solution #1 of this document.

This key issue includes:

1.	Functional requirements for C-Plane; and

2.	Protocol Stack for C-Plane interfaces.

5.4	Key Issue #3: C-Plane signalling protocol

This key issue studies the details of C-Plane signalling protocol based on the possible architecture studied in Solution #1, the functional requirements for C-Plane studied in Solution #2 and the other related requirements in the other solutions.

5.5	Key Issue #4: Functional requirements for U-plane

This key issue identifies the functional requirements for U-Plane on the derivative architecture. This key issue includes:

1)	Functional requirements for U-Plane; and

2)	Protocol stack for U-Plane interface.

5.6	Key Issue #5: Functional requirements for service control API

5.6.1	General

This key issue identifies the functional requirements for service control API that is required for the content provider, a form of RTC application provider, to provide an RTC services.

5.6.2	RTC Application Provider

5.6.2.1	General

In RTC architecture defined in 3GPP TS 26.506 [12], third-party application provider that provides WebRTC-based immersive RTC services is referred to as an RTC application provider. Such RTC application providers have several resources and functions to achieve service delivery.

NOTE 1:	Depending on the service delivery form of the RTC application providers (e.g., content provider detailed in clause 5.6.2.3), several resources and functions need to be deployed in the operator network.

RTC application provider is thought to have following resources:

1)	Service specific content: content for RTC services (e.g., back-ground graphical image/video and sound effects in VR conference services)

2)	RTC ID resource: resources reserved in control plane and used for establishing connection to RTC exchange resources. When accessing RTC services, a UE indicates this resource as the destination during media session setup. This resource is identified by a URI called RTC resource ID. RTC ID resource is composed of the RTC resource ID and properties associated with the RTC resource ID.

3)	RTC exchange resource: resources reserved in user plane. For example, individual RTC exchange resource is associated with a VR space or a conference room.

NOTE 2:	RTC ID resources and RTC exchange resources are collectively referred to as RTC resources.

Also, RTC application provider is thought to have following functions:

1)	Service logic function: a function to execute the logic to realize RTC services. Following functionalities are included:

a1)	RTC ID resource manager: a functionality responsible for controlling the creation, update, and deletion of RTC ID resources.

a2)	RTC ID resource handling enforcer: a functionality to perform actual creation, update, and deletion procedures on RTC ID resources as directed by the RTC ID resource manager.

b1)	RTC exchange resource manager: a function responsible for controlling the creation, update, and deletion of RTC exchange resources.

b2)	RTC exchange resource handling enforcer: a functionality to perform actual creation, update, and deletion procedures on RTC exchange resources as directed by the RTC exchange resource manager.

c1)	Connection control manager: a C-plane functionality responsible for determining the acceptance of UE's connection requests targeting a specific RTC ID resource.

c2)	Connection control enforcer: a functionality to perform connection control as directed by the connection control manager.

d1)	Media data forwarding control manager: a U-plane functionality responsible for configuring the rule of forwarding control for individual media or data exchanged in U- plane.

d2)	Media data forwarding control enforcer: a functionality to perform media data forwarding control as directed by the media data forwarding control manager.

NOTE 3:	Functionalities included in service logic function can be classified into the following two types. One is "service logic manager", including a1), b1), c1) and d1). The other is "service logic enforcer", including a2), b2), c2) and d2).

2)	WebRTC endpoint function: a function to terminate WebRTC communication. Following functionalities are included:

a)	C-plane signalling: a functionality to perform C-plane signalling using the RESPECT in this document.

b)	U-plane transport: a functionality to perform U-plane media communication using WebRTC protocol stack. Service specific content is provided through this functionality.

The control achievable through these functionalities above is referred to as "service control" in this document.

An RTC application provider performs as either service provider or content provider.

5.6.2.2	Service provider

An RTC application provider who provides RTC services with its own network is referred to as service provider (SP) in this document. SP owns resources and functionalities described in clause 5.6.2.1 within its network, and SP's network is connected to operator network via NNI. (See Figure 5.6.2.2-1.)

When a connection is initiated from the UE toward the SP, the SP uses the ID assigned and verified by the operator in order to control the UE's connections and provides services considering status of subscription associated with that ID.

NOTE:	When accessing SP's services, the UE initially attempts to connect to the operator's network using the ID assigned by the operator. After successful completion of authentication, the UE can use this ID as a network-assigned ID for establishing media sessions. The operator also treats this ID as a network-asserted ID through its verification. When a media session establishment request is made from the UE toward the SP, the SP will receive this network-asserted UE's ID in the signalling message from the operator's network.

Figure 5.6.2.2-1: Service provider-operator connection and functional deployment diagram

5.6.2.3	Content provider

Contrary to SP, an RTC application provider who provide RTC services partially using operator's functionalities is referred to as content provider (CP) in this document. CP connects to the operator network via UNI (RTC-4s/4m) as a WebRTC endpoint in order to use the operator's MF and WSF for the service specific content delivery. (See Figure 5.6.2.3-1.)

Since operator network accommodates CP via UNI, RTC ID/exchange resources are reserved within the operator's WSF and MF respectively. Therefore, in order to perform dedicated procedures on these RTC resources, RTC ID / RTC exchange resource handling enforcer functionalities needs to be deployed in the operator's WSF and MF. Also, connection control enforcer and media data forwarding control enforcer needs to be deployed in the operator network, as CP depends on C-Plane signalling and U-Plane transport functionalities of the operator network. As a result, CP's service logic managers in service logic function requires APIs for controlling related service logic enforcers deployed in the operator network. Since this functionality is not provided over RTC-4s/4m, it is considered to be provided through a different reference point than RTC-4s/4m. Required functionalities are to be discussed in clause 5.6.3.

NOTE:	The architectural requirements and reference point name for the APIs between CP's service logic manager and operator's service logic enforcers is FFS and will be addressed in the future work.

Also, CP and SP differ in terms of UE authentication by the service logic function. In the interconnection scenario between operator and SP networks, a UE connected to an operator network always uses an ID provided by the operator. In contrast, when RTC application provider provides its RTC service as CP, there are two possible network-asserted IDs that are used for media session setup by UE:

Operator-provided ID: the ID allocated to UE by the operator and managed by the operator. After successful completion of authentication, UE can use this ID as the network-asserted ID when requesting media session setup. The operator network will treat this ID as network-asserted ID and CP will perform connection control and provide service considering status of subscription related to the ID.

CP-provided ID: the ID allocated to UE by CP and managed by CP. The operator queries the CP to verify the authenticity of ID. After successful completion of authentication by CP, UE can use this ID as the network-asserted ID when requesting media session setup. The operator network will treat this ID as network-asserted ID and CP will perform connection control and provide service considering status of subscription related to the ID.

NOTE:	The querying the CP to verify the authenticity of CP-provided ID is performed at WSF in the authentication procedure triggered when receiving the signalling message for authentication from UE. The details are available in clause 5.4 (Key Issue #3: C-Plane signalling protocol).

Figure 5.6.2.3-1: Content provider-operator connection and functional deployment diagram

5.6.3	Functional requirements for service control

5.6.3.1	General

As mentioned in clause 5.6.2.3, CP requires APIs for controlling each service logic enforcer from each service logic manager. It is assumed that the service control instructed by CP and performed by the operator network requires the capability to describe instructions related to the following three functionalities at least:

- CRUD of RTC ID resource and associated properties

- User connection control using asserted identity

- Media data forwarding control

5.6.3.2	CRUD of RTC ID resource and associated properties

As described in clause 5.6.2, CP provides its RTC service by registering RTC ID resources to operator's network. RTC ID resource that serves as the destination for UE and associated properties are to be registered (e.g., notification settings for events related to the RTC ID resources, expiration timing of RTC ID resource, and handling of new connections for controlling graceful shutdown).

In the deployment of CP's functions, RTC ID resource handling enforcers belong to the operator. This means that CP initiates CRUD procedure requests, but the actual handling of RTC ID resources is performed by the operator based on these requests.

As procedures on the RTC ID resources, registering RTC ID resource, update to the properties of registered RTC ID resources, deletion of RTC ID resources, and acquisition of current RTC ID resource status are essential functions instructed by CP and performed by operator network.

5.6.3.3	User connection control using asserted identity

CP needs to be able to instruct connection control as part of functionalities supported in service logic manager when UE indicates an RTC ID resource as its destination and attempts to connect to the corresponding RTC exchange resource. Connection control manager in CP's service logic function can determine the acceptance of connection from its own managed user subscription information and network-asserted ID. There are two possible methods for the WSF performing the functionality of connection control enforcer to process connection control:

- CP registers specific connectable CP-provided or operator-provided IDs in the CRUD procedures described in clause 5.6.3.2.

- Operator network queries CP to determine whether to accept the connection from a UE having a CP-provided or operator-provided ID to an RTC ID resource.

5.6.3.4	Media data forwarding control

In general, RTC application providers determine how individual audio/video media and non-media data from UNI are transmitted or terminated, reflecting service requirements (e.g., user experience, security., etc). Such process is defined as media data forwarding control (MDFC) in this document. Individual audio/video media refers to a single track of audio or video. Also, individual non-media data refers to the data other than audio or video that is transmitted and received over a single data channel. These are collectively referred to as RTC media/data.

MDFC deals with connections of individual RTC media/data to the endpoints of specific UE or service specific content function through MF's input and output.

For example, when it comes to an audio media in a conference, the upstream audio media from a specific UE is duplicated by the MF and sent to all other participants' UEs. On the other hand, when providing services such as audio analysis or recording, it is expected that only the audio media of UE which has consented to information collection by CP will be sent to the audio analysis or recording module. In some cases, MF can simply duplicate and transfer the video media without any processing, while in other cases, it can terminate the video media, perform video processing such as motion detection, and then send the video media as avatar animations. When using the Data Channel for text messages, in an open chat where all participants can see, the chat text is sent to all UEs. However, for the private messages, the text is only sent to specific UEs and not to others.

It is a part of MDFC functionality that optimizing the allocation of internal resource of MF depending on the patterns of RTC media/data duplication and UE connectivity. Examples of use cases with different patterns of RTC media/data duplication and UE connectivity are:

- Conference where audio and video media are connected in a full-mesh manner between participants

- Webinar where only the presenters' audio and video media are delivered to all participants

- Large-scale broadcasting where one presenter's audio and video media are delivered to much larger audience

MDFC mentioned above cannot be described by SDP. Therefore, SP implements the MDFC as an internal logic within the service logic function. In the CP's service delivery, the transfer and termination of RTC media/data are processed by the MF including MDFC enforcer, and CP's MDFC manager is responsible for creating MDFC rules and instructing MDFC enforcer. An API is required for CP to instruct the MDFC enforcer in operator.

5.6.4	Summary

SP performs service control using its own network functions. On the other hand, in the case of CP performing service control, at least three functions described in clause 5.6.3 should be provided through APIs to allow CP's service logic mangers to instruct and the operator's service logic enforcers to perform those functions. In this key issue, the procedures and APIs which enable CP to achieve above functionalities and related processes are to be studied.

5.7	Key Issue #6: WSF discovery mechanism

In collaboration scenario 3 and collaboration scenario 4 of Real-Time Communication (RTC) for WebRTC, RTC application using WebRTC connects to a WebRTC signalling function (WSF) specified in 3GPP TS 26.506 [12] in order to setup media session, where the following steps are expected to be applied at the UE.

1)	Download an RTC application;

2)	launch the RTC application and identify the local WSF in the operator network where the UE attached to; and

3)	connect to the WSF and use the RTC services.

For step 2), the RTC application (i.e., WebRTC endpoint) is expected to connect to a local WSF in an operator's network where the UE attached to. Then the RTC application needs to identify the local WSF in the connected operator's network.

To enable WebRTC endpoint to identify the WSF without specific setting per connected operator's network, it is desirable to standardize a common WSF discovery mechanism for zero configuration. This will make benefits for both user and operator as follows:

-	User perspective:
Users do not need to change the application and/or parameters depends on the connected operator network. Then the user can use the RTC application without having to worry about the connected operator network.

- Operator perspective:
Connection management between WebRTC endpoint and WSF becomes easier, since the RTC application behavior for WSF discovery is standardized and operators are able to control the connected WSF by modification of the operator network settings.

This key issue identifies the WSF discovery mechanism without user manual setting and applicable regardless of the connected operator network.

NOTE:	Step 1) (Downloading an RTC application) is outside the scope of this key issue. Step 3) (Connecting to the WSF and using the RTC services) is studied in Key Issue #4, then step 3) is also outside the scope of this key issue.

5.8	Key Issue #7: Interworking with IMS Network

Editor’s note: Description will be added.

5.9	Key Issue #8: Protocol-level interworking between RTC network and IMS network

Editor’s note: Description will be added.

5.10	Key Issue #9: Tethered cases

Editor’s note: Description will be added.

5.11	Key Issue #10: Security considerations

This key issue addresses the security related considerations specific to real-time media communicaiton by WebRTC-based media session setup.

In IETF RFC 8825 [44] (which gives the WebRTC overview), the following items are described as security considerations:

a)	security of the components,

b)	security of the communication channels, and

c)	security of the partner's identities.

NOTE:	IETF RFC 8826 [45] and IETF RFC 8827 [46] describes further security considerations on real-time communication on the Web.

Regarding a), RTC application is outside the scope of 3GPP TS 26.506 [12] and RTC AF/RTC AS of this specification are defined as located in trusted DN - this means the RTCAF/RTC AS are protected by adequate network domain security. Then this document assumes that the security of components in RTC network is guaranteed.

Regarding b), secure transport protocol is applied for both C-plane and U-plane of RTC network in the Release-18 stage 3 work (i.e., WI: iRTCW). This document also applies the secure transport protocol (i.e., Secure WebSocket for C-plane, SRTP and SCTP for U-plane). Then, the security of the communication channels is regarded as guranteed.

Regarding c), as an operator provided/assisted RTC service, trustable subscriber identification and verification are required to prevent unauthorized use of service and spoofing in the operator's network since a user self-claimed RTC user identity is untrusted. In addition to this, the originating RTC user identity is intended to be used not only for display at the terminating party but also for authorization at the opposite RTC network in the collaboration scenario 4, a solution to deliver the trusted user identity between two different RTC network is required.

Then, this key issue addresses the solution to deliver the trusted RTC user identity in the RTC networks considering the collaboration scenario 4 for the aspect of c).

5.12	Key Issue #11: Related groups considerations

This key issue studies the potential impacts on the specifications of other WGs in 3GPP and organizations including IETF and W3C.

6	Solutions

6.1	General

This clause describes the solutions for key issues in clause 5. Each solution corresponds to a key issue with the same number (e.g., Solution #1 corresponds to Key Issue #1). In subsequent clauses for each solution, solution evaluations are described per solution.

6.2	Solution #1: Enhancements on RTC architecture

6.2.1	Solution description

This solution addresses Key Issue #1.

This clause identifies the enhancements on RTC architecture considering what functionalities, functional entities and reference points are needed for WebRTC-based immersibe RTC services in collaboraion scenario 4. This includes:

1)	Possible enhancements on functional entities and RTC architecture based on WebRTC view point (see clause 6.2.2);

2)	Interaction between fuctional entities in the enhanced RTC architecture and 5GC (see clause 6.2.3);

3)	Media connnection model (see clause 6.2.4);

4)	IP addressing (see clause 6.2.5);

5)	Alignment and gap analysis between the enhanced RTC architecture and the current RTC architecture (see clause 6.2.6); and

6)	Enhanced RTC architecture for collaboration scenario 4.

As a conclusion of 1) to 6), the derivative RTC architecture and enhancements on 3GPP TS 26.506 [12] are proposed as a solution for Key Issue #1 in clause 6.2.8.

6.2.2	Possible enhancements on functional entities and RTC architecture based on WebRTC viewpoint

6.2.2.1	Overview

Figure 6.2.2.1-1 depicts a possible RTC architecture based on the WebRTC viewpoint. It contains the functional entities described in clause 6.2.2.2 and reference points described in clause 6.2.2.3. The names of functional entities and reference points described here are only for discussion of this solution and the alignment of these names with 3GPP TS 26.506 [12] are considered in clause 6.2.7.

Figure 6.2.2.1-1:	Possible RTC architecture from WebRTC viewpoint

WebRTC Signalling Function (WSF) and Conference Supporting Function (CSF) could co-locate in a physical node. WebRTC NNI Signalling Gateway Function (WNSGF) and WebRTC NNI Media Gateway Function (WNMGF) are optional when gateway functions are not needed at the network boundary.

6.2.2.2	Functional entities for WebRTC

6.2.2.2.1	General

This clause enumerates functional entities in terms of 1) WebRTC-related standardized specifications, 2) WebRTC implementations, and 3) providing inter-operator services.

1)	Functional entities that are essential for this document and those already defined in IETF RFCs or 3GPP specifications concerning WebRTC (see clause 6.2.2.2.2).

2)	Functional entities that are not directly defined in WebRTC-related specifications of IETF RFCs or 3GPP specifications but considered to be widely implemented for realizing WebRTC services; they are essential for this document (see clause 6.2.2.2.3).

3)	Functional entities that may be specifically required for inter-operator or third-party collaboration services if modification of signalling and termination of media at network boundaries are needed (see clause 6.2.2.2.4).

6.2.2.2.2	Functional entities defined in WebRTC-related standardized specifications

6.2.2.2.2.1	UE (User Equipment)

6.2.2.2.2.1.1	General

User Equipment (UE) contains a user agent function which is equivalent to "WebRTC Endpoint" as described below. For the purposes of the present document, the following terms and definitions given in IETF RFC 8825 [44] are applied:

WebRTC Endpoint

-	Either a WebRTC browser or a WebRTC non-browser. It conforms to the protocol specification.

WebRTC Browser (also called a "WebRTC User Agent" or "WebRTC UA")

-	Something that conforms to both the protocol specification and the JavaScript API specification (W3C WebRTC 1.0 [65]).

NOTE:	WebRTC browser is also called a "web app" in this document.

WebRTC Non-Browser

-	Something that conforms to the protocol specification but does not claim to implement the JavaScript API. This can also be called a "WebRTC device" or "WebRTC native application".

Both "WebRTC Browser" type endpoint and "WebRTC Non-Browser" type endpoint are supported on the RTC architecture proposed in this document, as same as the current RTC architecture defined in 3GPP TS 26.506 [12]).

6.2.2.2.2.1.2	Considerations specific to WebRTC endpoint types

There are two types of WebRTC Endpoint; one is "WebRTC Browser" type, and the other is "WebRTC Non-Browser" type. This clause shows possible functional model for each type of endpoints on RTC architecture for identifying the specific issues related to the WebRTC endpoint types. If the RTC application provider connects its server (e.g., media server, content server) to a WSF in an operator network without providing WSF functionality (i.e., connect to the operator's WebRTC DN via UNI not NNI), the server is treated as UE (WebRTC endpoint) for the operator's network.

Regarding the "WebRTC Browser" type WebRTC endpoint, a JavaScript application runs on a web browser that has capabilities of JavaScript APIs including WebRTC APIs defined by W3C (see Figure 6.2.2.2.2.1.2-1). According to the concept of WebRTC described in IETF RFC 8829 [47], the procedures and protocols stated in this document are expected to be fully writable only with JavaScript.

Figure 6.2.2.2.2.1.2-1:	"WebRTC Browser" type endpoint

However, in the current situation, most of the OSS (e.g., android, iOS) and the web browsers (e.g., chrome, firefox) do not support/provide the enablers (provided by RTC MSH) for immersive RTC as JavaScript API. Therefore, to provide functionalities for realizing immersive RTC on "WebRTC Browser" type WebRTC endpoint, the mechanisms other than RTC MSH need to be supported. In order to support "WebRTC Browser" type endpoint, the protocols and procedures in this document can be implemented without RTC MSH.

Regarding the "WebRTC Non-Browser" type WebRTC endpoint, an application written in a programming language specific to the UE platform runs on UE using libraries and/or system call handlers. (See Figure 6.2.2.2.2.1.2-2)

Figure 6.2.2.2.2.1.2-2:	"WebRTC Non-Browser" type endpoint

NOTE:	The programming language and programming APIs used to write applications depend on the UE platform. For example, Java and Android API (SDK) will be selected for Android platform UEs, Swift and its libraries will be selected for iOS platform UEs, and C++ and Win64 API will be selected for Windows platform UEs.

The application can be realized in a way other than JavaScript running on a web browser. The application can support the functions provided by RTC MSH since the application can be developed proprietary.

In this document, the solution which realizes the immersive RTC services without using RTC MSH is addressed to support both "WebRTC Browser" type endpoint and "WebRTC Non-Browser" type endpoint.

This document does not state details of the application's implementation; but the network interfaces, which is applicable for both "Browser" and "Non-Browser" type UEs are mainly described.

6.2.2.2.2.2	WSF (WebRTC Signalling Function)

The WebRTC Signalling Function (WSF) is a functional entity that is responsible for WebRTC signalling mechanism including capability exchange and management of media sessions between UEs and the network. This functional entity is described as "Servers" or "Web Server" in clause 3 of IETF RFC 8825 [44]. Each operator or third-party in this document is assumed to have their own WSF(s) in their network.

WSF also provides the following functionalities:

-	Interaction with WMCF for media session (real-time streaming and data channel) control.

-	Interaction with CSF for collaboration with web applications/services.

-	Interaction with 5GC, using Network Support function AF's (NS-AF) functionality.

6.2.2.2.3	Functional entities widely implemented for WebRTC

6.2.2.2.3.1	WMCF (WebRTC Media Centre Function)

The WebRTC Media Centre Function (WMCF) is a functional entity that performs media processing. WMCF terminates media path (including audio/video stream and data channel) and performs media processing (e.g., mixing, selective forwarding, transcoding) which are required for immersive RTC applications. It may also perform decryption and encryption of media packets if DTLS, SRTP, or TLS is used for a transport layer. It also has the function of storing contents (including text or other static material as well as audio and video) and providing them to the UE. For media transport control, the WMCF interacts with WSF.

In the case that the WMCF acts as a simple media relay function, the WMCF simply relays media data packets and supports IP packet connectivity. When UE behaves as ICE Agents defined in IETF RFC 8445 [39] or IETF RFC 8838 [49], WMCF may be either STUN servers defined in IETF RFC 8489 [41] for connectivity check or TURN servers defined in IETF RFC 8656 [42] for relaying media data packets. This functional entity facilitates NAT traversal of UE and the connectivity between UE and other network functions.

This functional entity is generally implemented in WebRTC Multipoint Control Unit (MCU) or Selective Forwarding Unit (SFU).

6.2.2.2.3.2	CSF (Conference Supporting Function)

The Conference Supporting Function (CSF) is a functional entity that provides various functionality to realize WebRTC based RTC services with operator assistance. The CSF is expected to provide the following functionalities:

-	Conference session management, i.e., "CRUD" operation – create, read, update, delete of conference instances.

NOTE 1:	CSF needs to support service control APIs. The details of these APIs are addressed in Key Issue #5 and Solution #5.

-	Providing supplementary files (e.g., icon images of participants, and shared documents) via best-effort transport different from the channels for real-time media.

-	Storage of user subscription data specific to MNO's WebRTC services.

NOTE 2:	In this document, it is assumed that a single user (i.e., identity) and its subscription data (associated with the identity) are assigned, owned, and managed by both operator and RTC application provider independently. The two identities have a link with each other via some technique. User subscription data specific to RTC application provider's services are stored in their networks.

-	Authorization endpoint and token endpoint of OAuth 2.0 described in IETF RFC 6749 [29] for establishing authentication linkage between MNO's ID and RTC application provider's ID.

NOTE 3:	OAuth token will be used to C-Plane authentication at WSF and RTC application providers. STUN/TURN authentication with OAuth token is defined in IETF RFC 7635 [32]. Portal http(s) servers of WebRTC services provide this function in general implementations.

6.2.2.2.4	Functional entities needed for inter-operator services

6.2.2.2.4.1	WNSGF (WebRTC NNI Signalling Gateway Function)

The WebRTC NNI Signalling Gateway Function (WNSGF) is located at the boundary of the RTC networks where different operator's or third-party RTC network connects.

Each operator or third-party has its own WebRTC Signalling Functions (WSF) so that WSFs are connected to each other with border control functions such as security, policy management, charging, etc. WNSGF is inserted into "Signalling Path" in Figure 2 of IETF RFC 8825 [44] and responsible for border control functions and supports session establishment between disparate address realm's networks.

Also, WNSGF is able to support the functionality for interworking between WebRTC based signalling message and SIP message of IMS as a border control function.

NOTE:	The details of interworking with IMS network are addressed in Key Issue #7 and Solution #7.

6.2.2.2.4.2	WNMGF (WebRTC NNI Media Gateway Function)

The WebRTC NNI Media Gateway Function (WNMGF) is a media relay located at the boundary of the RTC networks where different operator's or third-party RTC network connects. WNMGF is responsible for the border control and transport of media data packets between different RTC networks. WNMGF may also transcode media data packets.

Also, WNMGF is able to support the functionality for interworking between WebRTC media and IMS media (e.g., transcoding of codec) as a border control function.

NOTE:	The details of interworking with IMS network are addressed in Key Issue #7 and Solution #7.

6.2.2.3	Reference Points

The reference points shown in Figure 6.2.2.1-1 are enumerated as follows.

Reference points for signalling are called as "control plane" or "C-Plane" in this document. Reference points for media are similarly called as "user plane" or "U-Plane" in this document.

Reference Points for C-Plane:

Rs-u: Reference Point between a WSF and a UE.

Rs-i: Reference Point between a WSF and another WSF in the same network (DN) or between a WSF and a WNSGF.

Rs-a: Reference Point between a WSF and a CSF.

Rs-n: Reference Point between a WNSGF and another WNSGF in an external network.

Reference Points for U-Plane:

Rm-u: Reference Point between a WMCF and a UE.

Rm-i: Reference Point between a WMCF and another WMCF in the same network (DN) or between a WMCF and a WNMGF.

Rm-n: Reference Point between a WNMGF and another WNMGF in an external network.

Reference Points for signalling nodes to control media nodes:

Mc-i: Reference Point between a WSF and a WMCF.

Mc-r: Reference Point between a WNSGF and a WNMGF.

Other Reference Points:

Rh-u: Reference Point between a CSF and UE. This reference point is used for providing CSF functionalities (e.g., application usage assistance such as downloading an application) to UE.

Rh-n: Reference Point between a CSF and RTC application provider. This reference point is used for interaction between CSF and RTC application provider for media session set up related interaction.

Detailed protocol for each reference point will be discussed in the dedicated key issue and solution.

6.2.3	Interaction between functional entities in the enhanced RTC architecture and 5GC

6.2.3.1	Overview

This clause shows a solution for integrating the enhanced RTC architecture based on pure WebRTC with 5GC. In other words, this clause identifies the possible interaction between the functional entities of the enhanced RTC architecture and the functional entities of 5GC.

NOTE:	"pure WebRTC" means the original WebRTC described in IETF work, which basically does not consider domain specific functions or features (e.g., mobile networks).

6.2.3.2	Mapping of functional entities for interaction with 5GC

6.2.3.2.1	General

This clause identifies the mapping of functional entities shown in Figure 6.2.2.1-1 into 5GC functional entities defined in 3GPP TS 23.501 [4].

In this document, the mapping of WSF and AF, and the mapping of WNSGF and 5GC functional entities are considered. Other functional entities (i.e., CSF, WMCF, WNMGF) are not considered since these functional entities are not expected to interact with 5GC.

6.2.3.2.2	WSF and AF

WSF is connected from UE and is expected to process the following:

1)	authenticate a UE.

2)	setup a WebRTC media session required by a UE, which may be in another network.

3)	manage QoS for the media path of a WebRTC session.

Then it is expected that the WSF interacts with functional entities of 5GC and UE to perform 1) and 3) as the following:

1)	WSF can retrieve the identity of a UE from 5GC, then authenticates and authorizes the UE.

3)	WSF can request PCF to enable QoS control for the media path through e.g., N5, N32 (specified in 3GPP TS 23.501 [4]) or CAPIF reference points (specified in 3GPP TS 23.222 [2]).

NOTE:	These processes are close to the processes of IMS functional entities such as P-CSCF and S-CSCF defined in 3GPP TS 23.228 [3]. The process of 1) is similarly performed by S-CSCF and UDM, and 3) is similarly performed by P-CSCF and PCF.

WSF can be mapped into "AF (Application Function)" of 5GC according to the definition of AF in 3GPP TS 23.501 [4] clause 5.2.10 due to the following reasons:

-	WSF interacts with the 3GPP core network to provide services.

-	The interaction between 5GC (e.g., PCF/UDM) and WSF is close to the interaction between 5GC and IMS entities (e.g., P-CSCF) that are AFs.

6.2.3.2.3	WNSGF

6.2.3.2.3.1	Overview

This clause identifies the mapping of WNSGF to a 5GC functional entity. There are a couple of possibilities currently identified. The following two 5GC functional entities can be mapped from WNSGF:

-	NEF (see clause 6.2.3.2.3.2)

-	SEPP (see clause 6.2.3.2.3.3)

As another possibility, it may be appropriate that WNSGF is mapped to a new functional entity (like Interconnection Border Control Function (IBCF) in IMS). The exact mapping of WNSGF is clarified in the alignment with 3GPP TS 26.506 [12] (clause 6.2.6 of this document).

6.2.3.2.3.2	WNSGF and NEF

When WSF is mapped into an AF and if WNSGF is deployed as 5GC functional entity, WNSGF can be mapped into an NEF due to the following reasons:

-	When WSF processes 2) of clause 6.2.3.2.2 and the media session relates to other operator's network, WSF (mapped to an AF) of operator-A is requested to interact with WNSGF on the boundary of operator-B to communicate with WSF (mapped into an AF) in operator-B due to operator-B's policy. In this model, the relationship between WSF (in operator-A) and WNSGF (in operator-B) is close to the relationship between AF and NEF described in clause 6.2.10 of 3GPP TS 23.501 [4].

-	The major function of WNSGF is close to the former three functionalities described in 3GPP TS 23.501 [4] clause 6.2.5.0; WNSGF exposes WSF's WebRTC signalling capability and events. WNSGF interworks with WebRTC C-Plane signalling between Rs-i and Rs-n reference points in terms of security and translation of internal-external information.

When WNSGF is mapped into an NEF, the definition of the NEF may need to be modified as follows:

-	Descriptions for the exposure of WSF's WebRTC signalling capability and the events by WNSGF are added in 3GPP TS 23.501 [4] clause 7.2.8.

-	Descriptions for the event exposure details are added in 3GPP TS 23.502 [5] clause 4.15.3.

-	Descriptions for the capability exposure details are added in 3GPP TS 23.502 [5] clause 5.2.6.

6.2.3.2.3.3	WNSGF and SEPP

Security Edge Protection Proxy (SEPP) is defined in 3GPP TS 33.501 [18] and 3GPP TS 23.501 [4]. The SEPP is an entity sitting at the perimeter of the PLMN for protecting control plane messages, hiding network topology. The SEPP enforces inter-PLMN security on the N32 interface that is a reference point between a SEPP in one PLMN and a SEPP in another PLMN.

If WNSGF is deployed as 5GC functional entity, WNSGF is also located at the perimeter of the PLMN and its function is protecting control plane messages and hiding network topology. The function of WNSGF is close to that of SEPP.

The difference between WNSGF and SEPP is the type of located PLMN. WNSGF is located at the edge of inter-HPLMN. On the other hand, SEPP is expected to be used for N32 that lies between HPLMN and VPLMN.

6.2.3.2.3.4	New functional entity

WNSGF is a border control function over C-Plane signalling path and located at the boundary of the networks where different operators or third-party network connects, as described in clause 6.2.2.2.4.1. Then, WNSGF is not expected to interact with 5GC functional entities and act as the gateway function for SBI.

In this document, the C-Plane signalling messages are expected to be exchanged via a DN over N6 interfaces and WNSGF is located at the DN. Therefore, WNSGF needs to be treated as a new border control function for C-Plane signalling path in WebRTC domain.

6.2.3.3	Possible architecture integrated with 5GC

The functional entities shown in Figure 6.2.2.1-1 can be connected to 5GC as described in Figure 6.2.3.3-1.

Figure 6.2.3.3-1:	Possible architecture integrated with 5GC

WSF (with NS-AF functionality of RTC architecture) is mapped into an AF as the 5GC viewpoint.

WSF (with NS-AF functionality of RTC architecture) is interconnected with PCF via N5 interface. WSF manages QoS of real-time media packets and C-Plane signalling packets via N5 interface. WSF may interact with UDM to authenticate and to authorize the UE.

Both signalling packets and media packets between UE and the network are transmitted via N6 interface. Signalling packets (C-Plane packets) from UE are transmitted to WSF, and real-time media packets (U-Plane packets) from UE are transmitted to WMCF. C-Plane signals may travel to WNSGF via Rs-i, and may travel further to other operator's WNSGF via Rs-n. U-Plane signals may travel to WNMGF via Rm-i, and may travel further to other operator's WNMGF via Rm-n. (see Figure 6.2.3.3-2)

Figure 6.2.3.3-2:	Possible architecture from 5GC viewpoint with data flows of C/U-Planes

6.2.3.4	Mapping to RTC collaboration scenarios

The following table shows the mapping of functional entities in this document into the collaboration scenarios described in 3GPP TS 26.506 [12]. Each box shows the condition (required or not) for MNO. The targets of this document are collaboration scenarios 3 and 4.

Table 6.2.3.4-1:	Functional entities required for each collaboration scenarios

| Functional Entity | Collaboration Scenario 3 |  | Collaboration Scenario 4 |
| --- | --- | --- | --- |
|  | 3A / Service Provider provides WebRTC services and MNO assists the services | 3B / MNO provides WebRTC services only in the MNO's network | MNO's WebRTC service interconnects with other MNO's or Service Provider’s service |
| WSF | Required | Required | Required |
| WMCF | Required | Required | Required |
| CSF | Required | Required | Required |
| WNSGF | N/A (NOTE) | N/A | Required |
| WNMGF | N/A (NOTE) | N/A | Required |
| NOTE: Scenario 3A in this table assumes service provider’s WebRTC functions communicate with WSF and WMCF via UNI-like interface, i.e., WSF and WMCF work as a gateway by themselves. Further Operator-Assistance models may be introduced. |  |  |  |

6.2.4	Media connection model

6.2.4.1	General

In the original WebRTC design, the communication between UEs is thought to be peer-to-peer (P2P). In most of the existing WebRTC implementations, however, the media connection is not P2P. An intermediate server (or servers) between UEs is used. In the multi-party call, the intermediate server which performs media processing is helpful for a UE because, for a UE, decoding all media from other UEs is a heavy load. Direct full-mesh connections among multiple UEs consumes a lot of network resources. Additionally, such an intermediate server is useful even for a one-to-one communication for offloading immersive media processing which needs more computation power than conventional media. This leads to the discussion about split rendering.

This document mainly focuses on the media connection model with intermediate servers.

P2P connection has some benefit for one-to-one communication (i.e., no need for an intermediate server and less server-relayed delay). For that reason, P2P connection is also considered for some special cases.

6.2.4.2	Target use cases from network view

Based on the high-level network model and target interfaces described in clause 4.2 and the enhanced RTC architecture described in clause 6.2.2, a signalling protocol which is called as "RESPECT" in this document supports the following use cases of media session set up from network view.

<Media session set up with RTC resource served in the operator network via UNI>

a.	UE  RTC Resource (served by the same operator)  UE

b.	UE  RTC Resource (served by the same operator)  UE (CP)

<Media session set up with RTC resource via NNI>

c.	UE  RTC Resource (served by the other operator)

d	UE  RTC Resource (served by an SP)

e.	UE (served by the other operator)  RTC Resource  UE (CP)

f.	UE  Transit network (served by the other operator)  RTC Resource (served by an SP)

<Media session set up between UEs>

g.	UE  UE (served by the same operator) without media gateway

h.	UE  UE (served by the other operator) without media gateway

i.	UE  UE (CP) without media gateway

j.	UE (served by the other operator)  UE (CP) without media gateway

The overviews of these use cases are described below.

NOTE 1:	Content provider connected via UNI is treated as UE.

NOTE 2:	CSF is not shown in the figures for simplicity.

- UE  RTC Resource (served by operator)  UE

-	UE establishes a media session destined for an RTC ID resource (e.g., immersive conference room) served by the same operator. Figure 6.2.4.2-1 shows an example that UE_A and UE_B establish media sessions destined for the same RTC ID resource (e.g., immersive conference room) to communicate with each other.

Figure 6.2.4.2-1:	UE  RTC Resource (served by the same operator)  UE

- UE  RTC Resource (served by operator)  UE (CP)

-	UE_A and UE (CP) establish a media session destined for a RTC ID resource served by an operator network which UE_A and UE (CP) connected to.

Figure 6.2.4.2-2:	UE  RTC Resource (served by the same operator)  UE (CP)

- UE  RTC Resource (served by the other operator)

-	UE_A establishes a media session destined for a RTC ID resource (e.g., Immersive conference room) served by the operator that different from the network which the UE_A is connected to. In this scenario, the C-Plane signalling messages and media/data are sent over the NNI. Other UEs can connect to the RTC ID resource as same as use case a.

Figure 6.2.4.2-3:	UE  RTC Resource (served by the other operator)

- UE  RTC Resource (served by an SP)

-	UE_A establishes a media session destined for a RTC ID resource (e.g., Immersive conference room) served by an SP. In this scenario, the C-Plane signalling messages and media/data are sent over the NNI.

Figure 6.2.4.2-4:	UE  RTC Resource (served by an SP)

- UE (served by other operator)  RTC Resource  UE (CP)

-	UE_C in the other operator network than an operator network serving a RTC ID resource (e.g., Immersive conference room) and UE (CP) in the operator network serving the RTC ID resource establish a media session destined for the RTC ID resource. In this scenario, the C-Plane signalling messages and media/media from UE are sent over the NNI.

Figure 6.2.4.2-5:	UE (served by the other operator)  RTC Resource  UE (CP)

- UE  Transit NW (other operator)  RTC Resource (served by an SP)

-	UE_A establishes a media session with a RTC ID resource (e.g., Immersive conference room) served by an SP via transit network served by the another operator. In this scenario, the C-Plane signalling messages and media/data are sent over the two different NNIs.

Figure 6.2.4.2-6:	UE  Transit network (served by the other operator)  RTC Resource (served by an SP)

- UE - UE (served by the same operator) without WMCF

-	UE_A establishes a media session (e.g., voice chat) with UE_B served by the same operator, without using WMCF.

Figure 6.2.4.2-7:	UE - UE (served by the same operator) without media gateway

- UE  UE (served by the other operator) without WMCF

-	UE_A establishes a media session (e.g., voice chat) with UE_C served by the different operator, without using WMCF. In this scenario, the C-Plane signalling messages and media/data are sent over the NNI.

Figure 6.2.4.2-8:	UE  UE (served by the other operator) without media gateway

- UE  UE (CP) without WMCF

-	UE_A establishes a media session with a UE (CP) which is connected to the same operator, without using WMCF.

Figure 6.2.4.2-9:	UE  UE (CP) without media gateway

- UE (connected to the other operator)  UE (CP) without WMCF

-	UE_C establishes a media session with a UE (CP) which is connected to the different operator, without using WMCF. In this scenario, the C-Plane signalling messages and media/data are sent over the NNI.

Figure 6.2.4.2-10:	UE (connected to the other operator)  UE (CP) without media gateway

6.2.4.3	QoS-enabled end-to-end path

In the collaboration scenario where the WebRTC functions in an operator's network assists an external provider (i.e., service provider, content provider, or another operator), setting up a QoS-enabled media path across different networks needs to be considered.

The media path from a UE to the external provider is roughly divided into the following four sections:

Section 1) Between a UE and the UPF (operator's CN section)

Section 2) Between the UPF and the operator's network edge (operator's DN section)

Section 3) Between the operator's network edge and the external provider network edge

Section 4) A network in the external provider

Section 4) is a matter of an external provider and out of scope of this document.

Regarding Section 1), this section includes the operator's core network. In this section, QoS is controlled by the PCF.

Regarding Section 2), operator's DN may have sufficient bandwidth or the other QoS mechanism may be adopted.

Regarding Section 3), this section's QoS control needs a bandwidth guaranteed path (i.e., a dedicated line). On the enhanced RTC architecture described in clause 6.2.2, when the media path is connected to a RTC resource in other operator's network or service provider's network, the media packets to be prioritized are transmitted to WNMGF placed in the operator's network and the WNMGF relays the media to the main media server in the other operator's network or service provider's network via guaranteed path as shown in Figure 6.2.4.3-1 (red-line). If the media path is connected to a WebRTC endpoint function in a content provider's network via WSF and WMCF (which work as a gateway) in the operator's network, this section is treated as UNI, as shown in Figure 6.2.4.3-1 (blue-line).

Figure 6.2.4.3-1:	Sections of end-to-end media path

6.2.5	IP Addressing

6.2.5.1	Overview

IP addressing for UE has some options: assigning IPv4 address only, IPv6 address only, or both.

In the operator deployment, the number of available IPv4 addresses would be insufficient for its subscribers. Generally, operators use IPv4 private address (and ISP shared address defined in IETF RFC 6598 [28]) with network address translation (NAT).

In clause 6.2.5, appropriate IP addressing is identified, discussing NAT-traversal in the WebRTC user plane and network verified ID retrieval.

6.2.5.2	NAT

6.2.5.2.1	Overview

NAT, including port translation as NAPT (Network Address and Port Translation), is a method of mapping an IP address space into another, which is mainly used to translate a private IP address into a global IP address, and vice versa, for communicating with external networks.

Generally, UE can be assigned with an IP address through a PDU session in operator networks. When an IPv4 address is allocated, as mentioned in clause 6.2.5.1, a private IP address or an ISP shared address is used. On the contrary, when an IPv6 address is allocated, a global unicast address is assigned.

NAT is essential for carrier-grade network deployment. Subscribers can be much more than usually available IPv4 global address space, and they are treated by using IPv4 private address and NAT. The same private address can be reused in each different domain behind NAT. Although NAT deployments have a wide variety, NAT is generally installed in a DN (data network) and often put in the middle between the UPF and other functional entities (see Figure 6.2.5.2-1).

On the other hand, IPv6 global unicast addresses basically do not require NAT, except for special security reasons or some transition method between IPv6 and IPv4 domains.

Figure 6.2.5.2-1:	Possible NAT location

6.2.5.2.2	NAT Variation

NAT is classified into some types by its address translation and packet filtering behavior.

The first version of STUN in IETF RFC 3489 [24] defines three types:

-	Full Cone NAT,

-	Restricted NAT (Restricted Cone NAT or Restricted Port Cone NAT), and

-	Symmetric NAT.

Full cone NAT does not limit access to an internal UE from external network entities, which have not communicated with the internal UE. Any external entities can re-use the external IP address and port number mapped to a specific internal UE and can access to it (Figure 6.2.5.2.2-1). Full cone NAT is less restrictive than other NATs. Restricted NAT only permits external entities to access the internal UE if the NAT have received any packets from the internal UE directed to the external UE (Figure 6.2.5.2.2-2). Symmetric NAT uses a different pair of an external IP address and port, which are specific to each external entity and only the external entity can access to the internal UE through the IP address and port pair.

Figure 6.2.5.2.2-1:	 Full Cone NAT behaviour

Figure 6.2.5.2.2-2:	Restricted or Symmetric NAT behaviour

6.2.5.2.3	Existing NAT-traversal

6.2.5.2.3.1	General

An effective NAT-traversal method is different depending on the NAT type described in clause 6.2.5.2.2.

In the original WebRTC design, STUN and TURN are listed, included as ICE, for major NAT-traversal methods. In addition, Hosted NAT Traversal (HNT, described in IETF RFC 7362 [31]) and its similar mechanism are frequently used in real implementations for conversational applications.

6.2.5.2.3.2	STUN

STUN is the method for UE behind the NAT to discover its external IP address observed by external networks. This method supports P2P communications and only works for full-cone NAT.

This document excludes STUN because the main communication model is not P2P but with intermediate servers (as described in clause 6.2.5), and general NATs deployed in operator networks are not limited to full-cone type.

6.2.5.2.3.3	TURN

TURN is the method for UE behind the NAT to communicate with external nodes via an intermediate server. TURN is a protocol for the session management and requires an intermediate server.

Generally, this method is regarded as the last resort for NAT-traversal for UDP-based conversational services. This method does not require the alignment with other control plane signalling, but is equipped as its own user plane connection management mechanism. This method needs additional message exchanges and has a protocol overhead.

The TURN server has its authentication mechanism for UEs and can be used for the purpose of traffic steering for an inter-operator communication scenario detailed in clause 4.2.4.3.

6.2.5.2.3.4	HNT

HNT (Hosted NAT Traversal) is the mechanism that a session border controller (SBC) placed at the edge of networks intermediates the communication between UEs behind NAT.

The problem tackled by HNT is that a UE behind a NAT tries to set up a session with its private address and port number for media, which have no clue to the SBC for the real media which comes later.

Regarding the control plane signalling, the signalling part of the SBC modifies media-related information represented by the private IP address and port number set in the SDP offered by an originating node into a global IP address and a new port number. This modification enables a terminating node to target the accessible IP address and port pair provided by the SBC. In the signalling return path, the SBC also modifies the terminating node's IP address and port number set in the SDP answered by the terminating node into new ones, and forwards it to the originating node. This is to solicit the originating node to send media to the SBC. Once the SBC receives the first media packet from the originating node targeting at the solicitation, the SBC recognizes the real NAT-ed IP address and port pair of the originating node. The SBC captures that information and uses it for relaying packets from the terminating node to the originating node. This is called "latching".

This method is embedded in the control plane signalling and does not require extra message exchange. For that reason, it has no additional protocol overhead. It is a better feature than TURN in the same condition requiring an intermediate server.

Since this document focuses on the connection model with an intermediate server, the NAT issues can be argued differently. Let's assume that all communication services are provided by the intermediate server as a conference. UEs can just join the open channel provided by the server and receives media from the server. UEs can also send their media to the intermediate server and the server mixes the media and distributes to other UEs. In this model, the first join packet from a UE to the NAT and the NAT to the server creates an address mapping at the NAT. The server simply sends packets to the source address of the join packet from the UE.

This mechanism does not need the dedicated protocol and there is no additional protocol overhead for NAT-traversal by sending media to the specific IP address and port pair exposed by WMCF. That points are analogous to HNT (Figure 6.2.5.2.2.4-1).

Figure 6.2.5.2.2.4-1:	HNT like NAT-traversal

6.2.5.2.4	Conclusion of NAT handling

NAT-traversal problems have been discussed and several solutions have been proposed as described above. However, if equipment for NAT-traversal is not required, certainly less server resources would be needed.

In short, it is preferable that only IPv6 global unicast address be assigned to UE and no dedicated NAT-traversal equipment be used. Intermediate servers are used mainly for media processing and for the media relay when there is no direct IP reachability (e.g., across inter-operator connection).

6.2.5.3	IP Address and Trustable Subscriber Identifier

The operator uses subscription identifiers (e.g., GPSI (Generic Public Subscription Identifier) in 5GC) for managing its customer's service subscription and charging. In WebRTC support, the operator needs to check customer's service requests by checking against operator's subscriber database organized with the subscription identifier. An OTT-specific ID and password may be insufficient even in the collaboration scenario with external service providers because they cannot be securely linked with subscriber information in the viewpoint of the operator. The issue is how the MNO deduces (or retrieves) the trustable subscriber identifier from customer's requests, which are carried by IP packets.

Trustable subscriber identifiers in the MNO network are required for certain validity check, since a UE's self-claimed GPSI and source IP address are untrusted.

The EDGEAPP architecture specifies the method how the EAS function block retrieves the GPSI from terminal's source IP address. The AF regarded as an EAS can retrieve the GPSI bound to the UE by Eees_UEIdentifier API in EDGEAPP. This mechanism and its flow contain authentications conducted at the related network functions (i.e., EES and NEF), which enable the EAS to acquire the valid GPSI in the operator network as a trustable subscriber identifier.

Validity of the terminal's source IP address needs consideration. UE's self-claimed IP address, especially presented in an application level, is not trustable. The source IP address presented in an IP header can be relatively trustable when the IP packet is transmitted through a connection with some handshake procedures.

The IP address linkage with a subscriber identifier also has an issue when NAT is deployed. In release 18, the method with which the AF can identify the trustable subscriber identifier (e.g., GPSI) to invoke the 3GPP network service API for the UE (Application client) remains to be investigated in eEDGEAPP. In VoLTE, this linkage with NAT can be achieved with the help of additional operator-specific information (e.g., PDN session related value). In the AF for WebRTC, it depends on which additional information element can be acquired by the AF. There is no clear answer for the ID linkage between the NAT-ed IP address and the subscriber identifier.

Contrarily, the UE IP address without translated by NAT can be linked with GPSI by Eees_UEIdentifier API (though detailed specification is needed).

In terms of ID linkage, using IPv6 global unicast address for UE is reasonable.

Using IPv4 private address will be studied further when NAT-ed ID linkage issue is solved.

6.2.5.4	Conclusion of IP Addressing

In terms of the required server resources for NAT-traversal and unclear retrieval of the trustable subscriber identifier, using IPv6 global unicast address for UE is reasonable. NAT deployments have a wide variety of behaviors and cannot be treated straightforward (refer to clause 6.2.5.2). Using media relay servers that act as either TURN or HNT covers most cases with NAT-traversal. However, there are still issues using IPv4 private address with NAT, such as ID linkage (refer to clause 6.2.5.3). For the sake of simplicity and to concentrate on identifying signalling requirements, this document considers IPv6-only use.

Then the use of ICE Function and the enhancements of ICE function are excluded from the scope of this document.

NOTE 1:	As specified in 3GPP TS 26.506 [12], the use of ICE Function is optional and is not restricted.

NOTE 2:	IPv6-only use is acceptable for future services because IPv6 address allocation to UEs is now widely available among operators. Also, IPv6-only deployment leads to efficient system development and equipment utilization.

6.2.6	Alignment and gap analysis between the enhanced RTC architecture and the current RTC architecture

6.2.6.1	General

This clause identifies the architectural and functional mapping between enhanced RTC architecture described in clause 6.2.2 of this document and the current RTC architecture defined in 3GPP TS 26.506 [12]. Figure 6.2.6.1-1 shows the RTC general architecture specified in 3GPP TS 26.506 [12].

Figure 6.2.6.1-1:	RTC General Architecture

6.2.6.2	WebRTC endpoint and RTC endpoint on UE

WebRTC endpoint on the UE is expected to be mapped to RTC endpoint on the UE on the RTC architecture with the following consideration.

-	An WebRTC endpoint includes signalling related aspects of applications on the UE, however, an RTC endpoint does not include applications on the UE. To support the signalling protocol for media session setup, the signalling related functionality of application needs to be included in the scope of the RTC endpoint. Note that application itself is not included in this scope.

6.2.6.3	WSF and (RTC) WSF

WSF is expected to be mapped to WSF (integrated with NS-AF) on RTC architecture with the following considerations.

-	WSF needs to support the functionality for interaction with Application Supporting Web Function (ASWF) for collaboration with web applications/services.

-	WSF needs to support the functionality for interaction with 5GC, using network Support function (NS-AF) functionality.

-	WSF needs to support the functionality for retrieval of the identity of a UE from 5GC, and authentication of the UE.

Regarding the retrieval of the identity of a UE from 5GC, as described in clause 6.2.5.4, the WSF is not able to retrieve the identity from 5GC in the case that the UE is assigned an IPv4 private address behind NAT in the current release. In that case, the authentication mechanism in commercial use such as SMS OTP (One Time Password) is possibly applicable for enhancement of authentication of the UE.

6.2.6.4	WNSGF and Inter-working Function

Inter-working Function (IWF) is specified in 3GPP TS 26.506 [12] as an inter-working functionality to enable MNO-facilitated WebRTC sessions that involve endpoints across different MNOs (e.g., providing cross-network signalling functionality). This is the expected functionality for WNSGF, since WNSGF is a gateway function for signalling messages between MNOs. Then, WNSGF is expected to be mapped to IWF on RTC architecture.

No gap is found between WNSGF and IWF.

6.2.6.5	CSF and Application Supporting Web Function

CSF is expected to be mapped to ASWF on RTC architecture. Also, the ASWF is expected to support the additional functionalities described in clause 6.2.2.2.3.2 in addition to the current functionality defined in 3GPP TS 26.506 [12].

6.2.6.6	WMCF and Media Function

WMCF is expected to be mapped to Media Function (MF) on RTC architecture. Also, the MF is expected to support the following functionalities.

-	Performing decryption and encryption of media packets if DTLS, SRTP, or TLS is used for a transport layer.

-	Storing contents (including text or other static material as well as audio and video) and providing them to the UE.

6.2.6.7	WNMGF and Transport Gateway Function

WNMGF is expected to be mapped to Transport Gateway Function (TGF) on RTC architecture.

No gap is found between WNMGF and TGF.

6.2.7	Enhanced RTC Architecture for collaboration scenario 4

This clause identifies the enhanced architecture for collaboration scenario 4 specified in 3GPP TS 26.506 [12] based on the consideration in above clauses. Figure 6.2.7-1 shows the derivative RTC architecture for collaboration scenario 4.

Figure 6.2.7-1:	Possible derivative RTC architecture for collaboration scenario 4

NOTE 1:	Other network includes RTC ASs in different MNO and service provider.

NOTE 2:	If RTC AF and RTC AS are controlled by a single operator and located in the same operator network, these functions are trusted. Inter-working Function and Transport Gateway Function act as a border controller function at the boundary of the network.

The following interfaces are expected to be introduced for collaboration scenario 4.

-	RTC-Y: This reference point is for C-Plane signalling and U-Plane media transport between RTC AS (Inter-working Function) and other RTC network or service provider network. This interface is necessary for inter-connect RTC-AS with other RTC network or service provider network to realize collaboration scenario 4. RTC-Y may further be grouped into two sub-interfaces as follows.

i)	RTC-Ys:	This interface is for C-Plane signalling between Inter-working Function and other RTC network or service provider network.

ii)	RTC-Ym:	This interface is for U-Plane media transport between Transport Gateway Function and other RTC network or service provider network.

The following interfaces are expected to be introduced/extended for collaboration scenario 3 and collaboration scenario 4. These interfaces are to enable operator assistance for RTC application providers and UEs, then these interfaces are used not only for inter-MNO scenario (Collaboration scenario 4) but also single MNO assistance scenario (Collaboration scenario 3).

-	RTC-X: This interface is application interface between RTC AS and content provider, a form of RTC application provider. The interface is used for providing RTC AS functionalities via ASWF. (e.g., subscription of RTC resource in RTC-AS.). This interface is necessary for real-time interaction between RTC-AS and content provider for service control.

-	RTC-4m: This interface needs to be extended for providing ASWF functionalities (e.g., application usage assistance such as downloading an application) to UE. This extension is necessary for providing RTC AS functionalities to UE as operator assistance.

The functional entities in Figure 6.2.2.1-1 correspond to the functions defined in 3GPP TS 26.506 [12] as follows:

-	WSF (WebRTC Signalling Function): WebRTC Signalling Function

-	WMCF (WebRTC Media Centre Function): Media Function

-	CSF (Conference Supporting Function): Application Supporting Web Function

-	WNSGF (WebRTC NNI Signalling Gateway Function): Inter-working Function

-	WNMGF (WebRTC NNI Media Gateway Function): Transport Gateway Function

NOTE 3:	As described in 3GPP TS 26.506 [12], the integration/collocation of RTC AF and WebRTC signalling server is possible. Co-located WebRTC signalling server is able to act as a RTC AF which is accessible to 5GC, and replace some of this RTC AF's interfaces and APIs with WebRTC signalling. For example, interfaces and APIs between this RTC AF and UE will be replaced to avoid concurrent/redundant requests from UE.

The reference points in Figure 6.2.2.1-1 correspond to those defined in TS 26.506 [12] as follows:

-	Rs-u: RTC-4s

-	Rs-n: RTC-Ys

-	Rm-u: RTC-4m

-	Rm-n: RTC-Ym

-	Rh-u:	RTC-4m

-	Rh-n:	RTC-X

In the C-Plane signalling aspects, this document focuses on RTC-4 based solutions as shown in Figure 6.2.7-2 to support the collaboration scenario 4 and the case for the application which is not able to use MSH (e.g., Web App).

-	RTC AF functionalities are integrated in WebRTC signalling function, since MSH is not used. Then, MSH related interfaces are omitted in Figure 6.2.7-2.

-	Functions of RTC AF are integrated within WebRTC Signalling Function, then RTC-3 is out of the scope.

-	The use and usage of ICE Function is optional functionality and is not used for non- NAT case. Then the extension of ICE functionality and its usage are outside the scope of this document, since no further extension is not identified in this document.

-	The representation of RTC-4s and RTC-4m are simplified. Web App and Native WebRTC App are expected to use these interfaces as follows:

*	Web App utilizes the web browser's JS API (including WebRTC API) to send/receive signalling message on RTC-4s and media/data on RTC-4m.

*	Native WebRTC App utilizes the SDK provided by the OS of the UE to send/receive signalling message on RTC-4s and media/data on RTC-4m.

Figure 6.2.7-2:	The focused interface of C-Plane signalling protocol

NOTE 4:		RTC-4m is connected to ICE function when TURN server needs to be used. Otherwise, RTC-4m is connected to Media Function (MF) or Application Supporting Web Function (ASWF).

NOTE 5:		The interfaces and the functionalities related to MSH, NS-AF, configuration function and provisioning function are not in the focus.

6.2.8	Proposed enhancements on RTC architecture

6.2.8.1	General

In this clause, the followings are described as proposed enhancements on RTC architecture.

1)	The derivative RTC architecture supporting collaboration scenario 3 and 4 (see clause 6.2.8.2)

2)	Enhancements on functionality in RTC AS functional entities (see clause 6.2.8.3)

3)	Enhancements on reference points (see clause 6.2.8.4)

4)	Enhancements on architecture diagrams in 3GPP TS 26.506 (see clause 6.2.8.5)

6.2.8.2	Derivative RTC architecture supporting collaboration scenario 3 and 4

This clause describes the derivative RTC architecture for collaboration scenario 3 and 4 according to the previous considerations. Figure 6.2.8.2-1 shows the derivative RTC architecture and reference points between RTC AS functions and other entities.

Figure 6.2.8.2-1: Derivative RTC architecture diagram

NOTE 1:	WebRTC endpoint function of content provider connects to RTC AS via RTC-4s/RTC-4m (UNI). For simplicity, this line is snipped in this figure.

NOTE 2:	NAT functionality and ICE functionality can be applied. However, these are snipped in this figure.

The derivative RTC architecture for collaboration scenario 3 and 4 with 5GC interaction viewpoint is shown in Figure 6.2.8.2-2. Network Support function (NS-AF) defined in 3GPP TS 26.506 [12] is integrated in the WSF to interact with 5GC via N5 interface.

Figure 6.2.8.2-2:	Derivative RTC architecture diagram with 5GC interaction viewpoint

6.2.8.3	Enhancements on functionality in RTC AS functional entities

6.2.8.3.1	General

This clause describes the functionalities needed for the functional entities in the derivative RTC architecture, which are identified in this document.

6.2.8.3.2	User Equipment

The User Equipment (UE) contains a user agent function for WebRTC. The user agent function is equivalent to "WebRTC Endpoint", which is either a WebRTC browser or a WebRTC non-browser as defined in IETF RFC 8825 [44]. The definitions of Web Browser and WebRTC Non-Browser in IETF RFC 8825 [44] are given below. WebRTC endpoint is the RTC endpoint supporting signalling related functionality of the application. Application itself is not scope of this document.

WebRTC Browser (also called a "WebRTC User Agent" or "WebRTC UA"): Something that conforms to both the protocol specification and the JavaScript API specification (W3C WebRTC 1.0 [65]).

NOTE 1:	WebRTC browser is also called "web app" in this document.

WebRTC Non-Browser: Something that conforms to the protocol specification but does not claim to implement the JavaScript API. This can also be called a "WebRTC device" or "WebRTC native application".

When a content provider provides a service via UNI, the content provider acts as UE (i.e., WebRTC endpoint). Since this is not considered in the current versions of 3GPP TS 26.506 [12], a certain clarification on "content provider" in 3GPP TS 26.506 [12] is expected.

This document identifies the following functionality needed for the UE. Since these functionalities are not defined in the current versions of 3GPP TS 26.506 [12], the enhancement on the functional definition in 3GPP TS 26.506 [12] is expected.

-	Support of WSF discovery mechanism (NOTE 2)

NOTE 2:	This solution does not address the details of WSF discovery mechanism since this is addressed in Key Issue #6 and Solution #6.

6.2.8.3.3	WebRTC Signalling Function

The WebRTC Signalling Function (WSF) is one of the RTC AS functional entities defined in 3GPP TS 26.506 [12]. The WSF is responsible for WebRTC signalling including capability exchange and management of media sessions between UEs and the RTC network. This functional entity is described as "Servers" or "Web Server" in clause 3 of IETF RFC 8825 [44]. Each operator or third-party in this document is assumed to have their own WSF in their RTC network.

This document identifies the following functionalities needed for the WSF. Since these functionalities are not defined in the current versions of 3GPP TS 26.506 [12], the enhancements on the functional definition in 3GPP TS 26.506 [12] are expected.

-	Interaction with MF for media session control

-	Interaction with ASWF for collaboration with web applications/services.

-	Interaction with 5GC, using network Support function (NS-AF).

-	Authentication/authorization of the UE.

-	Functionalities derived from service control API (i.e., connection control enforcer and RTC ID resource handling enforcer). (NOTE 1)

-	Signing and verification of network-asserted UE's ID. (NOTE 2)

NOTE 1:	This solution does not address the details of service control API since this is addressed in Key Issue #5 and Solution #5.

NOTE 2:	This solution does not address the details of signing and verification of network-asserted UE’s ID since this is addressed in Key Issue #10 and Solution #10.

NOTE 3:	Regarding the retrieval of the identity of a UE from 5GC for authentication of UE, the WSF is not able to retrieve the identity from 5GC in the case that the UE is assigned an IPv4 private address behind NAT in the current release. In that case, the authentication mechanism in commercial use such as SMS OTP (One Time Password) is possibly applicable for enhancement of authentication of the UE.

6.2.8.3.4	Media Function

The Media Function (MF) is one of the RTC-AS functional entity defined in 3GPP TS 26.506 [12]. The MF performs media processing. MF terminates media path (including data channel path) and performs media processing (e.g., mixing, selective forwarding, transcoding) which are required for immersive RTC applications. The MF is able to perform decryption and encryption of media packets if DTLS, SRTP, or TLS is used for a transport layer. The MF has the function of storing contents (including text or other static material as well as audio and video) and providing them to the UE. For Media transport control, the MF is able to interact with WSF.

In cases where an MF performs as a simple media relay function, the MF simply relays media data packets and supports IP packet connectivity. When a UE behave as ICE agents defined in IETF RFC 8445 [39] or IETF RFC 8838 [49], the MF may be either STUN servers defined in IETF RFC 8489 [41] for connectivity check or TURN servers defined in IETF RFC 8656 [42] for relaying media data packets. This functional entity facilitates NAT traversal of UE and the connectivity between UE and other network functions.

This functional entity is generally implemented in WebRTC Multipoint Control Unit (MCU) or Selective Forwarding Unit (SFU).

This document identifies the following functionality needed for the MF. Since the functionality is not defined in the current versions of 3GPP TS 26.506 [12], the enhancement on the functional definition in 3GPP TS 26.506 [12] is expected.

-	Functionalities derived from service control API (i.e., media data forwarding control enforcer and RTC exchange resource handling enforcer for service control).

NOTE:	This solution does not address the details of service control API since this is addressed in Key Issue #5 and Solution #5.

6.2.8.3.5	Application Supporting Web Function

The Application Supporting Web Function (ASWF) is one of the RTC AS functional entities defined in 3GPP TS 26.506 [12]. This document identifies the following functionalities needed for the ASWF. Since these functionalities are not clearly defined in the current versions of 3GPP TS 26.506 [12], the enhancements on the functional definition in 3GPP TS 26.506 [12] are expected.

-	Exposing the service control APIs. (NOTE 1)

-	Storage of user subscription data specific to MNO's WebRTC services. (NOTE 2)

-	Authorization endpoint and token endpoint of OAuth 2.0 described in IETF RFC 6749 [29] for establishing authentication linkage between MNO's ID and RTC application provider's ID. (NOTE 3)

-	Providing supplementary files (e.g., icon images of participants, and shared documents) via best-effort transport different from the channels for real-time media.

-	Providing WSF discovery functionality (NOTE 4).

NOTE 1:	This solution does not address the details of service control APIs since this is addressed in Key Issue #5 and Solution #5.

NOTE 2:	In this document, it is assumed that a single user (i.e., identity) and its subscription data (associated with the identity) are assigned, owned, and managed by both MNO and application provider independently. The two identities have a link with each other via some technique. User subscription data specific to application provider's services are stored in their networks.

NOTE 3:	OAuth token will be used to C-Plane authentication at WSF and RTC application providers. STUN/TURN authentication with OAuth token is defined in IETF RFC 7635 [32]. Portal http(s) servers of WebRTC services provide this function in general implementations.

NOTE 4:	This solution does not address the details of WSF discovery functionality since this is addressed in Key Issue #6 and Solution #6.

6.2.8.3.6	Inter-working Function

The Inter-working Function (IWF) is one of RTC AS functional entity defined in 3GPP TS 26.506 [12]. The IWF is located at the boundary of the RTC network where different operator or third-party network inter-connects.

The IWF is inserted into "Signalling Path" in Figure 2 of IETF RFC 8825 [44] and responsible for border control functions and supports session establishment between disparate address realm's networks. By inserting the IWF into "Signalling Path", each operator or 3rd-party network can securely inter-connect with the other network.

This document identifies the following functionalities needed for the IWF. Since these functionalities are not defined in the current versions of 3GPP TS 26.506 [12], the enhancements on the functional definition in 3GPP TS 26.506 [12] are expected.

-	C-plane signalling protocol interworking between RTC network and IMS network. (NOTE 1)

-	Signing and verification of network-asserted UE's ID. (NOTE 2)

NOTE 1:	This solution does not address the details of interworking with IMS network since this is addressed in Key Issue #7 and Solution #7.

NOTE 2:	This solution does not address the details of signing and verification of network-asserted UE’s ID since this is addressed in Key Issue #10 and Solution #10.

6.2.8.3.7	Transport Gateway Function

The Transport Gateway Function (TGF) is one of RTC AS function entity defined in 3GPP TS 26.506 [12]. The TGF is a media relay located at the boundary of the RTC network where different operator or third-party inter-network connects. The TGF is the function responsible for the border control and transport of media data packets between different networks. The TGF is responsible for the border control and transport of media data packets between different networks.

This document identifies the following functionality needed for the TGF. Since the functionality is not defined in the current versions of 3GPP TS 26.506 [12], the enhancement on the functional definition in 3GPP TS 26.506 [12] is expected.

-	U-Plane protocol interworking between RTC network and IMS network.

NOTE:	This solution does not address the details of interworking with IMS network since this is addressed in Key Issue #7 and Solution #7.

6.2.8.4	Enhancements on reference points

The reference points shown in Figure 6.2.8.2-1 (Derivative RTC architecture diagram) are listed in Table 6.2.8.4-1.

The reference points marked as "No" in the 3rd column of Table 6.2.8.4-1 are expected to be introduced in 3GPP TS 26.506 [12].

Table 6.2.8.4-1: Reference points used for derivative RTC architecture

| Reference point (NOTE) | Descriptions | 3GPP TS 26.506 [12] already define? |
| --- | --- | --- |
| RTC-4 | Reference Point between an RTC network and a UE for C/U-Plane. | Yes |
| RTC-4s | Reference Point between a WSF and a UE for C-Plane signalling. | Yes |
| RTC-4m | Reference Point between a MF and a UE or between an ASWF and a UE for U-Plane. | Yes |
| RTC-X | Reference Point between a ASWF and a content provider network for service control. | No |
| RTC-Y | Reference Point between an RTC network and another network (i.e., other operator network or service provider network) for C/U-Plane. | No |
| RTC-Ys | Reference Point between a IWF and another network (i.e., other operator network or service provider network) for C-Plane signalling. | No |
| RTC-Ym | Reference Point between a TGF and another network (i.e., other operator network or service provider network) for U-Plane. | No |
| RTC-Z | Reference Point between an RTC network and IMS for C/U-Plane. | No |
| RTC-Zs | Reference Point between a IWF and IMS for C-Plane signalling. | No |
| RTC-Zm | Reference Point between a TGF and IMS for U-Plane. | No |
| N5 | Reference Point between a WSF and PCF in 5GC. | Yes |
| NOTE: RTC-X/Y/Z reference points need to be assigned/defined considering the common architecture for 5GMS and RTC. |  |  |

6.2.8.5	Enhancements on architecture diagrams in 3GPP TS 26.506

This clause describes the expected enhancements on architecture diagrams in 3GPP TS 26.506 [12].

The expected enhancements on RTC general architecture are shown in Figure 6.2.8.5-1. RTC-4 reference point is connected to UE rather than WebRTC Framework since the interface is used for C-Plane signalling between application in the UE and RTC AS in addition to U-Plane media (audio/video stream) and data between RTC endpoint and RTC AS. RTC-X reference point, RTC-Y reference point and RTC-Z reference point are newly introduced.

NOTE 1:	RTC-X is applicable between the RTC AS (ASWF) and the content provider in Figure 6.2.8.5-1.

Figure 6.2.8.5-1:	Expected enhancements on RTC General Architecture

Figure 6.2.8.5-2 shows the expected enhancements on derivative RTC architecture for collaboration scenario 3 defined in 3GPP TS 26.506 [12]. RTC-4m reference point is clarified that this interface is used for providing ASWF functionality to UE, and RTC-X reference point is newly introduced to provide the service control API for content provider, a form of RTC application provider, from ASWF.

NOTE 2:	RTC-X is applicable between the RTC AS (ASWF) and the content provider in Figure 6.2.8.5-2.

Figure 6.2.8.5-2:	Expected enhancements on derivative architecture for collaboration scenario 3

Figure 6.2.8.5-3 shows the expected derivative RTC architecture for collaboration scenario 4. Collaboration scenario 4 supports inter-operable WebRTC services. Then collaboration scenario 3 is extended with functions and interfaces to support MNO to MNO inter-operability. RTC-Y (RTC-Ys and RTC-Ym) reference point and RTC-Z (RTC-Zs and RTC-Zm) reference point are introduced to support the inter-connection between MNO's RTC ASs and between RTC AS and IMS.

NOTE 3:	RTC-X is applicable between the RTC AS (ASWF) and the content provider in Figure 6.2.8.5-3.

NOTE 4:	Other RTC network in Figure 6.2.8.5-3 includes other operator's network and service provider's network.

Figure 6.2.8.5-3:	Expected derivative architecture for collaboration scenario 4

6.2.9	Solution evaluation

The proposed architecture in clause 6.2.8 supports the functionalities and capabilities to support immersive RTC services for collaboration scenario 4 (also applicable for collaboration scenario 3) and the architecture is consistent with RTC architecture in 3GPP TS 26.506 [12]. Then it is proposed to;

-	reflect the architectural enhancements on functional entities, reference point described in clause 6.2.8 into the stage 2 specification of RTC (i.e., 3GPP TS 26.506 [12]) and

-	based on the above enhancements, update the architecture diagrams (RTC general architecture, derivative architectures for collaboration scenario 3 and 4.

6.3	Solution #2: Functional requirements for C-Plane

6.3.1	Solution description

This solution addresses Key Issue #2.

This solution identifies the functional requirements for control plane (C-Plane) signalling for WebRTC-based RTC session supporting inter-operator connection (i.e., collaboration scenario 4 in 3GPP TS 26.506 [12]) in addition to collaboration scenario 3 in 3GPP TS 26.506 [12].

Figure 6.3.1-1 shows the C-Plane reference points on the derivative RTC architecture. RTC-4s an RTC-Ys are focussed reference points as described in Solution #1.

Figure 6.3.1-1:	Reference points for C-Plane

To support collaboration scenario 3, RTC-4s needs to be supported. According to 3GPP TS 26.506 [12] clause 4.3.3, RTC-4s supports the exchange of signalling information related to the WebRTC session between two or more WebRTC endpoints using trusted application servers.

To support collaboration scenario 4, RTC-Ys is introduced in the enhanced RTC architecture as a new C-Plane interface for signalling information exchange between two different operator's networks or between an operator network and a service provider network.

6.3.2	Functional requirements for C-Plane interface

6.3.2.1	General

This clause identifies the functional requirements for C-Plane interface to enable WebRTC-based RTC media session supporting collaboration scenario 3 and 4. The requirements are considered based on following aspects:

1.	Support of WebRTC based RTC services (general aspect)

2.	Transport of signalling message

3.	Media session control and management

6.3.2.2	Support of WebRTC based RTC services

This clause identifies the functional requirements on C-Plane interface to support WebRTC based RTC services.

1.	It is required to support any WebRTC application (i.e., it should not be overfit for a specific use case.).

a.	It is required to support any kind of WebRTC endpoints (e.g., web browser).

b.	It is required to allow application specific methods and information elements.

2.	It is required to be web-friendly to support rapid and easy deployment in web environments:

a.	by using web technologies such as HTTP, JSON, etc…

b.	complying with WebRTC standards (e.g., SDP for session description and supporting the exchange of ICE candidates, etc…) defined in IETF and W3C, with an exception for codecs.

3.	It is required to be able to be simple to implement and deploy (e.g., simpler in complexity compared to SIP).

4.	It is required to be able to authenticate and authorise the UE using RTC services.

5.	It is required to protect user privacy and mitigate the linkability and tracking attack caused by unnecessary user information disclosure.

6.3.2.3	Transport of signalling message

This clause identifies the functional requirements on transport of signalling message.

1.	It is required to be web-friendly to support rapid and easy deployment in web environments by using web technologies such as WebSocket, etc…

2.	It is required to support the secure exchange of messages supporting integrity-protection and/or encryption.

3.	It is required to be support connection management mechanisms (e.g., keep alive) for reliable exchange of signalling messages.

6.3.2.4	Media session control and management

This clause identifies the functional requirements on media session control and management.

1.	It is required to support following methods for media session control.

a.	media session set up

b.	media session update

c.	media session disconnection

2.	It is required to support a method for querying information from a connected network. The information includes the service configuration information such as server address.

3.	It is required to be able to set up a media session with any kind of RTC resources (e.g., WebRTC endpoint on the UE, conference, metaverse).

4.	It is required that an WebRTC endpoint is able to set up multiple media sessions simultaneously.

5.	It is required to support incoming call set up (i.e., be able to receive a media session set up request).

6.	It is required to be able to set up a media session with RTC resources in different operator network or RTC application provider network. This requirement is applied for both of the following cases.

a.	The connected network support RTC AS functionalities. (i.e., connected via NNI)

b.	The connected network does not support RTC AS functionalities. (i.e., connected via UNI)

7.	It is required to be familiar with existing web-services to exchange media capabilities. It is also required that WebRTC endpoints can notify own media capabilities to a network, and network can handle the notified media capability appropriately.

8.	It is required to support a mechanism to exchange meta data associated with media session.

9.	It is required to support QoS control of a media session based on the information contained in the signalling message related to the media session.

10.	It is required to be able to negotiate the use of optional features.

11.	It is required to support the mechanisms for reliable media session control. (e.g., error handling).

12.	It is required to be able to identify the RTC service user uniquely. The identity of the user is able to be associated with multiple devices (WebRTC endpoints) belongs to the user.

13.	It is required to be able to enable communicating parties to verify each other's identity, if required by application.

6.3.3	Protocol stack for C-Plane interface

6.3.3.1	General

This clause identifies the appropriate protocol stack for C-plane interfaces, considering the requirements in clause 6.3.2. Especially, the following requirements are considered:

-	It is required to support the secure exchange of messages supporting integrity-protection and/or encryption.

-	It is required to protect user privacy and mitigate the linkability and tracking attack caused by unnecessary user information disclosure.

-	It is required to be web-friendly to support rapid and easy deployment in web environments:

-	by using web technologies such as JSON, WebSockets, etc…

-	complying with WebRTC standards (e.g., SDP for session description and supporting the exchange of ICE candidates) defined in IETF and W3C, with an exception for codecs.

-	It is required to be simple to implement and deploy (e.g., simpler in complexity compared to SIP).

6.3.3.2	Base protocol

HTTP (IETF RFC 9110 [54], IETF RFC 9111 [55], IETF RFC 9112 [56], and IETF RFC 9113 [57]) / HTTPS and WebSocket (IETF RFC 6455 [27]) are available options to transport signalling message between UE and WSF so that connection setup procedure could be invoked by JavaScript API as described in clause 3 of IETF RFC 8825 [44]. Nevertheless, HTTP / HTTPS is less appropriate for two reasons described in clause 1.1 of IETF RFC 6455 [27]:

-	Server load caused by http transactions (based on request-response)

-	A connection has two sessions each for sending and receiving signalling packets

In addition, when a notification from the network to the UE is required, for such as an incoming call, an HTTP(S) connection is originated from the network side, but this case has some problem. Generally, NAT box is placed between UE and network entities, therefore NAT-traversal problem should be resolved. Besides, in terms of security configuration, UEs often deny incoming TCP (IETF RFC 9293 [61]) connections.

WebSocket fulfils the requirement for secure transport of signalling messages since WebRTC supports the secure transport over TLS.

For those reasons mentioned above, only WebSocket over TLS is utilized as the base protocol for transport of signalling messages. WebSocket can solve the three problems, server load, number of sessions and the NAT-traversal.

6.3.3.3	Upper layer protocol over WebSocket

In IETF RFC 8825 [44], upper layer protocols over WebSocket are not specified and are thought to be application specific. In the IETF RFC 8825 [44], SIP (IETF RFC 3261 [21]) and XMPP (IETF RFC 6120 [26]) are listed as candidate upper layer protocols for C-Plane signalling.

6.3.3.3.1	SIP

Utilizing SIP for C-Plane signalling for WebRTC is already described in clause 5 of 3GPP TS 24.371 [9]. One of the main advantages of using SIP is the ease of interworking between WebRTC-aware network and IMS network. On the other hand, disadvantages of using SIP are as follows:

-	UE and network must be able to understand both WebRTC and SIP. SIP is not widely used outside of telephony. If SIP must be used in conjunction with WebRTC, the advantage of WebRTC, friendliness to web-based development environments and developers, is to be spoiled.

-	SIP has a strictly managed communication model as SIP dialog. In principle, the originated signalling is transparently relayed through the network and the terminals manage the dialog with each other. These characteristics are not compatible with the UE-network relation model, which is the scope of this document.

-	SIP specifies methods divided by each signalling characteristic (i.e., INVITE, ACK, BYE, CANCEL, OPTIONS, PRACK, UPDATE, SUBSCRIBE, NOTIFY, REFER, PUBLISH, INFO). Adding control for a new characteristic may need to start from the method definition.

-	Less affinity with cloud environment where HTTP is mainly used. For example, raw values of the IP addresses related to the SIP dialog (consisting of a communication path of SIP trapezoid) are in the protocol header or message body, therefore changing communication elements is difficult once the call session is established.

For the reasons above, more optimal protocol for the upper layer of C-Plane signalling is expected to be identified.

6.3.3.3.2	XMPP

There is no specification using XMPP for the upper layer protocol of the C-Plane signalling in 3GPP and no major commercial implementations of WebRTC either. The reason seems that XMPP can be used on its own and does not need to be combined with other protocols. WebSocket encapsulation of XMPP has little benefit except the case that an application using XMPP is implemented using JavaScript.

For the reasons above, more optimal protocol for the upper layer of C-Plane signalling is expected to be identified.

6.3.3.3.3	Other existing implementations

Among the existing implementations of WebRTC communication services, JSON (IETF RFC 8259[36]) format is mainly used as a format for the upper layer of C-Plane signalling. This is because JSON format is easy to handle in JavaScript. Taking this advantage, this document investigates more optimal protocol using JSON format for the upper layer of C-Plane signalling protocol.

In 3GPP specifications, RESTful APIs (such as service-based interface and Northbound APIs) are often defined using OpenAPI 3 (OpenAPI [64]) and the message-body of the APIs are based on JSON. However, OpenAPI is mainly suitable for RESTful APIs and not suitable for message-driven APIs such as C-Plane signalling over WebSocket. There is another possible API specification for JSON based API. AsyncAPI [63] (managed by Linux Foundation) is a message/event-driven architecture concept and familiar with message-driven API. For this reason, AsyncAPI [63] is used for identifying API schemas in this document.

6.3.3.4	Proposed Protocol Stack

The protocol stack for C-Plane interface is shown in Figure 6.3.3.4-1. As described above, JSON based protocol over WebSocket over TLS is an expected solution for C-plane signalling protocol.

WebSocket can be deployed over several versions of HTTP.

-	WebSocket with HTTP/1.1 is specified in IETF RFC 6455 [27] and used in this document. HTTP/1.1 is not, however, shown in the protocol stack because HTTP/1.1 does not remain after upgrading into WebSocket.

-	WebSocket with HTTP/2 is specified in IETF RFC 8441 [38] and used in this document. HTTP/2 is shown in the protocol stack because HTTP/2 framing remains after a stream in HTTP/2 connection is upgraded into WebSocket.

-	WebSocket with HTTP/3 (IETF RFC 9114 [58]) is specified in IETF RFC 9220 [60] but not used in the current version of this document. The transport protocol used over HTTP/3 needs to be selected in alignment with IETF/W3C discussions.

The sub layers of each protocol are according to the existing specifications.

-	TLS under HTTP/1.1 and HTTP/2 is specified in IETF RFC 8446 [40].

-	TCP under TLS is specified in IETF RFC 9293 [61].

-	IPv4 and IPv6 under TCP are specified in IETF RFC 791 [20] (IPv4) and IETF RFC 8200 [33] (IPv6).

Figure 6.3.3.4-1:	Protocol Stack for C-Plane interface

6.3.4	Solution evaluation

There is no misalignment between the functional requirements proposed in clause 6.3.2 and 6.3.3, and those specified in 3GPP SA4 RTC specifications (i.e., 3GPP TS 26.506 [12] and 3GPP TS 26.113 [10]), therefore it is proposed to develop the C-Plane signalling protocol based on the proposed functional requirements and protocol stack.

6.4	Solution #3: C-Plane signalling protocol

Editor’s note: Description will be added.

6.5	Solution #4: Functional requirements for U-Plane

6.5.1	Solution description

This solution addresses Key Issue #4.

This clause identifies requirements for U-Plane needed for WebRTC-based immersive RTC session management supporting inter-operator connection (i.e., collaboration scenario 4 in TS 26.506 [12]) based on the architecture described in clause 6.2. Figure 6.5.1-1 shows the U-Plane reference points on the derivative architecture. RTC-4m and RTC-Ym are focussed reference points of this document as described in clause 6.2.7.

Figure 6.5.1-1:	Reference points for U-Plane

6.5.2	Functional requirements for U-Plane interface

This clause identifies the functional requirements of RTC-4m and RTC-Ym reference point as U-Plane interface.

The derivative architecture supports collaboration scenario 3 and 4 defined in 3GPP TS 26.506 [12]. The requirements of RTC-4m on the derivative architecture are compliant with the requirements of RTC-4m specified in 3GPP TS 26.506 [12]. RTC-Ym is a new U-plane interface for WebRTC media transport between different operator's network or between an operator and service provider network. On the viewpoint of service interoperability, the requirements of RTC-Ym are required to be same as RTC-4m.

Functional requirements applied to both the RTC-4m and RTC-Ym are to transport:

-	Media data transmitted over RTP;

-	Application data transmitted using data channel; and

-	Media related meta-data transmitted using data channel

NOTE:	As RTC-Ym is the interface between the networks operating by two different operators (or an operator and a service provider) where the different policy/application can be adopted/provided; therefore, a bilateral agreement may be required.

6.5.3	Protocol stack

RTC-4m and RTC-Ym on the derivative architecture are U-Plane interfaces for WebRTC media transport. Then the protocol stack of RTC-4m and RTC-Ym conforms to the protocols specified in RFC 8835 [48]. This protocol stack is also applied for U-Plane interface in 3GPP TS 26.113 [10] which specified "enabler for Immersive Real-time Communication".

Detailed protocol stack for the U-Plane interface studied in this document is defined by selecting the protocol from the protocol stack specified in clause 13.1 of 3GPP TS 26.113 [10] in the corresponding normative work for stage 3 specification.

NOTE:	The specification other than protocols (e.g., codec) is not referred.

6.5.4	Solution evaluation

The U-Plane functional requirements proposed in clause 6.5.2 and the U-Plane protocol stack proposed in clause 6.5.3 are appropriate for WebRTC media transport and aligned with 3GPP SA4 RTC specifications (i.e., 3GPP TS 26.506 [12] and 3GPP TS 26.113 [10]). Therefore, these are proposed as the U-Plane requirements in this document.

6.6	Solution #5: Service control API

6.6.1	Solution Description

This solution addresses Key Issue #5.

In this solution, procedures and APIs required for the following functions described in clause 5.6.3 and additional notification process between the CP and operator network are identified. These procedures and APIs are used for instructions and notifications between the service logic managers and the service logic enforcers.

- CRUD of RTC ID resource and associated properties

- User connection control using asserted identity

- Media data forwarding control (MDFC)

In the extension of the RTC architecture described in clause 6.2 of this document, the ASWF has the functionalities of conference session management and capability exposure to CP. Therefore, these API are provided through RTC-X between ASWF and CP, as illustrated in the Figure 6.6.1-1.

NOTE:	RTC-X is expected to be added to the RTC architecture specified in 3GPP TS 26.506 [12]. However, the RTC architecture shares common elements with the media delivery architecture, as specified in 3GPP TS 26.510 [13]. Therefore, it is necessary to identify possible impacts of RTC-X against the common elements before normative works.

Figure 6.6.1-1: Reference point representation of service control

6.6.2	Procedures for service control

6.6.2.1	General

This clause identifies the procedures for the functions mentioned in clause 5.6.3.

These procedures refer to instructions and notifications performed over RTC-X between the service logic managers in CP and the service logic enforcers in the operator network.

The CRUD of RTC ID Resource are procedures for manipulation of resources. In these procedures, requests are always initiated proactively by the CP's service logic manager, and performed by the operator network's service logic enforcer.

In contrast, user connection control is performed in a callback procedure. The callback procedure is triggered by a new attempt of media session setup establishment from UE in an operator network and the operator network requests instruction from the CP along with the notification.

Regarding MDFC, it can be divided into two procedures. First, static rules associated with VR spaces or conference rooms are instructed from the CP to the operator network. Second, in cases where exceptional rule cannot be handled by pre-configured rules, the operator requests instructions from the CP with a notification through a callback procedure.

Other notifications of state changes in RTC ID resources can be performed as callback procedures, informing about the occurrence of events that are previously subscribed to.

Based on the above, all the procedure for service control in this document categorized into the following two types:

- CP-oriented procedure: procedures refer to the instructions by the CP's service logic manager to the operator network's service logic enforcer.

- Callback procedure: procedures refer to the notifications from an operator network's service logic enforcer to a CP's service logic manager regarding the occurrence of events CP has subscribed to (e.g., new attempt of media session establishment from UE). In the response to callback, CP could instruct the operator network's service logic enforcer to perform a certain process associated with the event.

6.6.2.2	CP-oriented procedure

6.6.2.2.1	General

CP-oriented procedures provide instructions from the CP to the operator network. The procedures are listed in Table 6.6.2.2.1-1.

Table 6.6.2.2.1-1: CP-oriented procedures

| Procedures | Description | Section Defined |
| --- | --- | --- |
| Create RTC ID resource | This procedure is used by CP to register the RTC ID resource and associated properties. | Clause 6.6.3.2 |
| Read RTC ID resource | This procedure is used by CP to retrieve the status of the RTC ID resource and associated properties. |  |
| Update RTC ID resource | This procedure is used by CP to update the RTC ID resource and associated properties. |  |
| Delete RTC ID resource | This procedure is used by CP to delete the RTC ID resource and associated properties. |  |

6.6.2.2.2	RTC ID resource management

6.6.2.2.2.1	General

This procedure group provides the functionality for CP's RTC ID resource manager to instruct RTC ID resource management performed by operator network's RTC ID resource enforcer. It consists of four procedures providing the functionality of CRUD of RTC ID resource and associated properties.

The creation and deletion of VR space / conference room provided by CP are performed as registration and de-registration of RTC ID resources to operator network. An RTC resource ID, that is a property of an RTC ID resource used to identify the RTC ID resource, is represented as a URI. This URI consists of user part and host part (e.g., userinfo@host). The host part represents a CP providing services, and the user part represents individual VR space / conference room in the specific CP's service.

In addition, the associated configurations for individual VR space / conference room are executed by CRUD of properties associated with an RTC ID resource. The information of properties managed by this procedure could include:

- Expiration time: This property is used to set expiration time for a RTC resource ID. In the case of a conference service, it may be necessary to set an expiration time for the meeting. This expiration time is typically set when making a reservation for the meeting. It is commonly used to send notifications to participants at the start time, and to provide information to participants about the remaining time before the end time.

- Acceptance of new connections: This property is used to restrict new connections to a RTC resource ID. Depending on the RTC service, there may be a need to disallow any new connections to an RTC ID resource. For example, after the expiration of a meeting, a new participant can be disallowed to enter the meeting, while participants joined before the expiration of the meeting are allowed to continue communications.

- Permission list: This property is used to restrict session participation to specific UEs assigned certain IDs, enabling private sessions. When flexible connection control is required instead of static settings, the "user connection control using asserted identity" procedure in clause 6.6.2.5 will be used.

- Event subscription: This property is information about the type of subscribed events and the related callback URI(s). These events may include the new connection attempt from a UE to a specific RTC ID resource, and completion of RTC exchange resource after a CP's RTC ID resource deletion request. Since these events are related to RTC ID resources, it is thought to be natural for CP to configure event subscription settings associated with this procedure. In accordance with this event subscription, callback procedures described in clause 6.6.2.3 are performed.

- Media data forwarding control configuration: This property is information about media data forwarding control. The role of the UE in the context of MDFC, labelling of media/data transmitted and received by the UE, and the routing rules for those media/data are configured by associating this property with the RTC ID resource.

6.6.2.2.2.2	Create RTC ID resource

In this procedure, the RTC ID resource manager in the CP instructs the RTC ID resource enforcer in the operator network to register a new RTC ID resource and reflect the configurations based on the associated properties.

The register request from CP use HTTP POST method and may include properties associated with RTC resource ID such as RTC ID resource, the expiration time of RTC exchange resources, the acceptance of new connections, permission lists, event subscriptions and MDFC rules as described in clause 6.6.2.2.2.1.

After successful creation, the operator network will send 201 (Created) response including the created RTC ID resource URI in the Location header of accept response. This URI allows CP to identify the target resources for subsequent Update/Read/Delete procedures.

6.6.2.2.2.3	Read RTC ID resource

In this procedure, the RTC ID resource manager in the CP instructs the RTC ID resource enforcer in the operator network to retrieve the configurations related to the registered RTC ID resource.

The read request from CP uses the HTTP GET method for this purpose.

After successful information retrieval, the operator network will respond with a 200 (OK) response, including the properties of the specified RTC ID resource.

6.6.2.2.2.4	Update RTC ID resource

In this procedure, the RTC ID resource manager in the CP instructs the RTC ID resource enforcer in the operator network to update the configurations related to the registered RTC ID resource.

The update request from CP uses the HTTP PATCH method for this purpose and may include changed properties associated with the RTC ID resource.

After successful RTC ID resource update, the operator network will respond with a 200 (OK) response.

6.6.2.2.2.5	Delete RTC ID resource

In this procedure, the RTC ID resource manager in the CP instructs the RTC ID resource enforcer in the operator network to delete the registered RTC ID resource and related configurations.

CP uses HTTP DELETE method for this purpose.

Upon accepting the request, the operator network will respond with a 202 (Accepted) response. The completion of the RTC ID resource deletion is notified through the callback procedure described in clause 6.6.2.3.2.

6.6.2.3	Callback procedure

6.6.2.3.1	General

Callback procedures provide notifications from operator network to the CP for events occurrence corresponding to subscribed callback triggers. When notifying events, if additional instructions are required, the CP issues instructions in the response of this procedure. For each callback trigger, callback procedures are listed in Table 6.6.2.3.1-1.

The Callback URI registered or updated in RTC ID resource management procedures described in clause 6.6.2.2 is used as the destination of requests. The operator network uses HTTP POST method for this purpose. Upon receiving the notification, the CP responds with a 200 (OK) response.

Common information elements for callback request procedures could include RTC resource ID, callback trigger, and a sequence ID to identify the request.

Table 6.6.2.3.1-1: Callback procedures

| Procedures | Description | Section Defined |
| --- | --- | --- |
| Notification of RTC ID resource deletion | This procedure is used by the OP to notify that the deletion of the RTC ID resource, as requested by the CP, has been completed. | Clause 6.6.3.3 |
| Notification of forced RTC ID resource deletion | This procedure is used by the OP to notify that the RTC ID resource has been forcefully deleted. | Clause 6.6.3.4 |
| Notification of RTC ID resource suspended | This procedure is used by the OP to notify that the RTC ID resource is temporarily unavailable. | Clause 6.6.3.5 |
| Notification of RTC ID resource resumed | This procedure is used by the OP to notify that the suspended RTC ID resource is available again. | Clause 6.6.3.6 |
| Notification of user call in requested | This procedure is used by the OP to notify that a new connection has been requested from the UE. | Clause 6.6.3.7 |
| Notification of user call in accepted | This procedure is used by the OP to notify that a new connection request from the UE has been accepted by operator network. | Clause 6.6.3.8 |
| Notification of user call in connected | This procedure is used by the OP to notify that a media session has been established as a result of a new connection request from the UE. | Clause 6.6.3.9 |
| Notification of user call disconnected | This procedure is used by the OP to notify that the media session of the UE has been disconnected. | Clause 6.6.3.10 |
| Notification of media routing query | This procedure is used by the OP to notify that additional media/data is added during the establishment or update of a UE's media session and the OP requests instructions from the CP. | Clause 6.6.3.11 |

6.6.2.3.2	Notification of RTC ID resource deletion

In this procedure, the RTC ID resource enforcer in the operator network notifies the CP's RTC ID resource manager about the completion of a RTC ID resource including the release of RTC exchange resource associated with the RTC ID resource.

It is expected that even if the "Delete RTC ID resource" request is acceptable for operator network, there may be situations where RTC exchange resources in RTC exchange enforcer (MF) associated with the RTC ID resource cannot be immediately released while UEs are actively participating in conference rooms. This procedure is necessary to indicate the completion of both RTC ID resource and the associated RTC exchange resource deletion after all active sessions have ended and corresponding RTC exchange resources have been released.

6.6.2.3.3	Notification of forced RTC ID resource deletion

In this procedure, the RTC ID resource enforcer in the operator network notifies the CP's RTC ID resource manager that the RTC ID resource has been forcefully deleted.

In cases where the RTC ID resource becomes unavailable independently of CP requests and is not expected to recover in the short term (e.g., due to network failures), this procedure is used for notification. After this notification, it is assumed that the CP cannot perform status retrieval, update, or deletion related to the respective resource.

6.6.2.3.4	Notification of RTC ID resource suspended

In this procedure, the RTC ID resource enforcer in the operator network notifies the CP's RTC ID resource manager that the RTC ID resource is temporarily unavailable.

When this procedure is performed, new connections to the corresponding RTC ID resource are not possible, and existing connections are expected to either be disconnected or in the process of being disconnected. Upon receiving this notification, the CP is only capable of performing the deletion process described in clause 6.6.2.2.2.5 for the RTC ID resource.

6.6.2.3.5	Notification of RTC ID resource resumed

In this procedure, the RTC ID resource enforcer in the operator network notifies the CP's RTC ID resource manager that the RTC ID resource has recovered from the suspended state previously notified in clause 6.6.2.3.4.

For the corresponding RTC ID resource, new connections are possible after recovery, but it is not guaranteed that the state will be the same as it was before entering the suspended state.

6.6.2.3.6		Notification of user call in requested

In this procedure, the connection control enforcer in the operator network notifies the connection control manager in the CP that a UE has attempted a new media session setup. The CP provides instructions regarding the permissibility of this UE's connection in the response of this procedure, offering the functionality of User connection control using asserted identity.

As described in clause 5.6.3.3, the CP performs connection control based on its own user's subscriptions data and its own service requirements when a UE requests the media session setup to the registered RTC ID resource. Therefore, when UE requests a media session setup, the operator queries the CP to determine whether UE's connection is permissible. Since UE's media session setup requests are made through RTC-4s to WSF, the connection control enforcer in WSF handles the connection control.

The information elements for the request in this procedure would additionally include the CP-provided ID indicating the originating UE and media information in the media session setup request. CP can determine the validity of UE's media session setup request based on the CP-provided ID and subscription data managed by CP.

The response in this procedure would involve the acceptance of connection request by the UE. Furthermore, along with the determination of connection permissibility, CP may indicate the role of the UE in the context of MDFC. As a property of the RTC ID resource, it is possible to statically configure which UE roles are associated with specific RTC user IDs. However, if a connection attempt is made with an RTC user ID that has not been pre-configured, the response of this procedure is used to convey information about connection permissibility and the role of UE.

6.6.2.3.7	Notification of user call in accepted

In this procedure, the connection control enforcer in the operator network notifies the CP's connection control manager that a UE has attempted to connect, and the connection request has been accepted. The CP provides instructions regarding the permissibility of this UE's connection in the response of this procedure, offering the functionality of User connection control using asserted identity.

The content of the request-response for this procedure follows clause 6.6.2.3.6, excluding the callback trigger.

6.6.2.3.8	Notification of user call in connected

In this procedure, the connection control enforcer in the operator network notifies the CP's connection control manager that a UE has attempted to connect, and the media session has been established.

6.6.2.3.9	Notification of user call disconnected

In this procedure, the connection control enforcer in the operator network notifies the CP's connection control manager that the established media session has been disconnected.

6.6.2.3.10	Notification of media routing query

In this procedure, the media data forwarding control enforcer in the operator network notifies the CP's media data forwarding control manager that a UE has attempted to connect or update media session, and there is additional media that cannot be handled by pre-configured rules. The CP provides instructions regarding the requested media/data connections in the response of this procedure.

As described in clause 5.6.3.5, the CP controls how individual RTC media/data are connected to the endpoints of specific UE or WebRTC Endpoint Functions in response to a media session setup request or media update for a registered RTC ID resource from UE or WebRTC Endpoint Functions.

In many RTC services, participating UEs can be classified into several groups that share same MDFC patterns based on their roles (e.g., hosts and guests in meetings, broadcaster and audiences in streaming). By describing MDFC patterns as presets for each role, efficient representation of MDFC can be achieved. Additionally, by refining the classification of roles, the MDFC can be attributed to the "copy to all" pattern, such as forwarding the RTC media/data to all terminals except the sender, resembling the common SFU (Selective Forwarding Unit) forwarding process.

Therefore, in MDFC, the preset roles for UE, labelling of media/data, and routing rules are configured as associated properties at the time of RTC ID resource registration. Additionally, the assignment of UE roles during connection is either pre-configured as a property of the RTC ID resource or instructed by the CP during the determination of connection permissibility as described in clause 6.6.2.3.6.

In exceptional cases, such as private one-to-one RTC media/data forwarding, individual queries to the MDFC manager are necessary. When such specific inquiries are required, the MDFC enforcer uses this procedure to query the MDFC manager.

As additional information elements in the request of this procedure, CP-provided ID indicating the originating UE, the role of the UE, the number of UEs with the same role, and media information included in the media session request are expected to be included. The additional information in the response of this procedure conforms to clause 6.6.2.3.6.

6.7	Solution #6: WSF Discovery mechanism

6.7.1	Solution description

6.7.1.1	General

This solution addresses Key Issue #6.

This clause identifies the mechanism which discovers a WSF in the connected operator network without user manual setting, regardless of the connected operator network.

There are following possible mechanisms to find a WSF without user settings.

a)	Media Session Handler (via RTC-5 API) (3GPP TS 26.506 [12])

b)	Edge application enabler (EAS discovery) (3GPP TS 23.558 [7])

c)	PCO in NAS signalling during PDU session set up (3GPP TS 23.501 [4], 3GPP TS 23.548 [6])

d)	DNS resolution

6.7.1.2	Analysis on possible mechanisms

As described in 3GPP TS 26.506 [12] and the clause 6.2 of this document, RTC services need to support both native WebRTC application (i.e., WebRTC non-browser type endpoint) and web application (i.e., WebRTC browser type endpoint).

a) and b) are the mechanisms using application enabler specified in 3GPP. However, in the current situation, most of the Oss (e.g., android, iOS) and the web browsers (e.g., chrome, firefox) do not support these enablers for JavaScript Application as JavaScript API. Then, a) and b) are not suitable for the time being.

c) is the mechanism to get a server information from Protocol Configuration Option (PCO) during PDU session establishment, however, there are same issue as a) and b) to apply this mechanism. Then, c) is also not suitable for the time being.

d) intends to use the local DNS server in the connected operator network to resolve the single specific FQDN into the actual IP address of the server in the connected operator network. This mechanism does not have the limitation mentioned above, then d) is the possible candidate of the WSF discovery mechanism.

Therefore, this solution studies a mechanism which apply d) using a specific URL (which is common among operators) to discovers a WSF in the connected operator network without user manual setting, regardless of the connected operator network.

6.7.2	Common URL based WSF discovery mechanism

6.7.2.1	General

This solution studies the mechanism which discovers the WSF in the connected operator network using a specific URL which is common among operators (names the URL as "common URL" in this document) and local DNS server in the connected operator network.

As the prerequisites of the study on WSF discovery mechanism, the following requirements specified in 3GPP TS 26.113 [10] need to be considered.

1)	The mechanism can identify the signalling protocol used for the RTC session set up, since there are multiple signalling protocols for RTC services (i.e., SWAP, RESPECT).

2)	Secure WebSocket (WSS) connection is applicable between the WebRTC endpoint and the WSF.

Above requirements are not fulfilled, if the common URL indicates only the WSF URL (e.g., the public TLS certificate cannot be prepared.). Then, the URLs for the WSF discovery mechanism are specified as follows.

Common URL:
A specific URL which is common among RTC operator networks and is used to get the WSF URL(s) from the WSF discovery function in the connected operator network. This URI indicates the signalling protocol in addition to WSF URI, which is derived from the above requirement 1).

WSF URL:
Secure WebSocket URI of WSF which is specified in 3GPP TS 26.113 [10]. The hostname of the WSF URL is specific hostname for RTC service and assigned by the operator.

Considering the above, this Solution studies the followings.

1)	Common URL format

2)	Common URL based WSF discovery procedure

6.7.2.2	Common URL format

This clause studies the format of common URL.

Common URL needs to indicate the signalling protocol which expected to be used by application, since multiple signalling protocols (i.e., SWAP and RESPECT) are applicable for RTC session set up as described in clause 6.7.2.1. Then, the common URL need to include "protocolName" which specified in 3GPP TS 26.113 [10]. Therefore, the following format is proposed as common URL.

CommonURL: {commonHostname}/<protocolName>

NOTE 1:	WebSocket URI includes "protocol version". However, "protocol version" is not included in common URL, since the compatibility between versions and its version management depend on the signalling protocol.

For "commonHostname", it seems appropriate that RTC applies the domain name ".3gppservices.org" as 3gpp service, which is defined in 3GPP 5GMS (3GPP TS 26.512 [14]) as default AF's hostname. Then, "commonHostname" in common URL is proposed as following:

{commonHostname}: "rtc.3gppservices.org"

NOTE 2:	As an alternative domain name for (commonHostname), there are IETF RFC 6762 based domain name (e.g., .internal). However, the IETF based solution is not studied since the 3GPP based approach is appropriate for RTC service.

In case of RESPECT protocol studied in this document, the Common URL will be following URL.

-	rtc.3gppservices.org/3gpp-respect

6.7.2.3	Common URL based WSF discovery procedure

6.7.2.3.1	General

This clause studies the procedure at the WebRTC endpoint to discover the WSF in the connected operator network by using common URL.

As described in clause 6.7.2.1, common URL is used to get the WSF URL(s) from WSF discovery function. Then, the UE procedure for discovering and connecting to the WSF is as follows.

i)	Get WSF URL(s) from a WSF discovery function by using common URL

ii)	Connect to a selected WSF from the obtained WSF URL(s) by WSS (secure WebSocket)

NOTE 1:	The operator who provide this WSF discovery mechanism needs to provide DNS server to resolve the commonHostname (i.e., rtc.3gppservices.org) of common URL into the IP address of the WSF discovery function.

NOTE 2:	The method to decide the connecting WSF from the obtained WSF list depends on the application. This solution addresses the procedure for step i). Step ii) is studied in Key Issue #3 and corresponding solution in this document.

6.7.2.3.2	Protocol

There are two possible protocols for getting WSF URL(s) using common URL.

a)	HTTP

b)	WebSocket

For the following reasons, WebSocket connection is too much for getting WSF URL. Then this solution studies HTTP-based procedure for getting the WSF URL.

-	Push notification from discovery function is not required.

-	WSF discovery procedure is expected to be triggered when the RTC application is activated or in case of WSF connection error, then the frequency of execution is low.

NOTE:	Cross-Origin Resource Sharing (CORS) needs to be considered for WSF discovery since the domain of Common URL and WSF URL are different, as described in the prerequisites in clause 6.7.2.1.

6.7.2.3.3	Procedure

The following procedure is proposed as HTTP based WSF discovery procedure.

i)	The RTC application sends a HTTP GET request to the Common URL (the request is sent to WSF discovery function)

ii)	The WSF discovery function sends back an HTTP response as follows, depending on whether the indicated signalling protocol is supported or not in the connected operator network.

a)	200 (OK)
When the connected operator network supports the indicated signalling protocol, the WSF discovery function sends back an HTTP 200 (OK) response. The response body (Content-Type: application/json) includes WSF URLs (WebSocket URI specified in 3GPP TS 26.113 [10]) for all protocol versions which the operator network supports. The response body format is protocol-independent. Example of the response body for RESPECT is shown below:

<Response body for RESPECT>

{

"v1": {

"wsfUrl": ["wss://wsf-1.example.com/3gpp-respect/v1", "wss://wsf-2.example.com/3gpp-respect/v1"]

}

}

b)	404 (Not Found):
When the connected operator network does not support the indicated singalling protocol, the WSF discovery function sends an HTTP 404 (Not Found) response.

6.7.2.3.4	Definition of the HTTP response body for RESPECT

The definition of the HTTP response body for WSF discovery procedure using Common URL for RESPECT.

Table 6.7.2.3.4-1: Information Element in the response body for RESPECT

| IE name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| v1 | v1Info | 1 | This information element is for WSF information for RESPET version 1. |

Table 6.7.2.3.4-2: Data type definition of v1Info

| IE name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| wsfUrl | array(string) | 1 | This information element indicates the WSF URL(s). The format of the WSF URL is required to be the WebSocket URI specified in 3GPP TS 26.113 [10]. e.g., wss://wsf.example.com/3gpp-respect/v1 |

6.7.2.3.5	Common URL based WSF discovery flow example

Following message flow is an example of WSF discovery procedure using Common URL for RESPECT.

1)	HTTP GET Request (RTC application -> WSF discovery function)

GET /3gpp-respect HTTP/1.1

Host: rtc.3gppservices.org

-- other HTTP headers are snipped --

2)	HTTP 200 OK response (WSF discovery function -> RTC application)

HTTP/1.1 200 OK

Content-Type: application/json

-- other HTTP headers are snipped --

{

"v1": {

"wsfUrl": ["wss://wsf.example.com/3gpp-respect/v1"]

}

}

6.7.3	Functional entity supporting WSF discovery function

This clause studies the functional deployment of WSF discovery function in generic RTC architecture specified in 3GPP TS 26.506 [12].

As described in the above clauses, WSF discovery function provides the WSF URL(s) in the connected operator network to RTC applications. The WSF discovery function is required to provide WSF URL(s) to RTC applications even if MSH (RTC-5) is not applicable. Then, it seems better to implement the function in RTC AS. In RTC AS functionalities, Application Supporting Web Function (ASWF) is appropriate function to implement WSF discovery function, since ASWF is the function to support RTC applications as a web server.

6.7.4	Solution evaluation

The proposed WSF discovery mechanism using common URL described in clause 6.7.2 fulfils the purpose of the mechanism (i.e., the WSF discovery mechanism without user manual setting and applicable regardless of the connected operator network) described in the corresponding key issue and consistent with existing 3GPP specification. Then, it is proposed to implement the proposed WSF discovery mechanism as the functionality supported at the ASWF into the stage 2 specification of RTC (i.e., 3GPP TS 26.506 [12]) as an optional mechanism.

6.8	Solution #7: Interworking with IMS Network

Editor’s note: Description will be added.

6.9	Solution #8: Protocol-level interworking between RTC network and IMS network

Editor’s note: Description will be added.

6.10	Solution #9: Tethered cases

Editor’s note: Description will be added.

6.11	Solution #10: Security considerations

6.11.1	Solution Description

This solution addresses Key Issue #10.

As a solution to deliver the trusted user identity in the RTC networks, it is reasonable for the RTC user identity to adopt the concept of "trust domain" defined in IETF RFC 3324 [22] which is constructed by human, since this concept has been widely used in the IMS network to use/provide the trusted user identity for telephony services.

In some case, the solution may need more reliability/strength for the interconnection scenario (i.e., collaboration scenario 4), since the "trust domain" is constructed by human being (e.g., thorough bi-lateral agreement). To address this case, this solution addresses the adaptation of "Calling number verification using signature verification and attestation information" used for the IMS interconnection defined in 3GPP TS 29.165 [16].

In the subsequent clauses, the followings are described as a solution:

-	Adaptation of the trust domain (clause 6.11.2)

-	Network-asserted identity within the trust domain (clause 6.11.3)

-	Adaptation of calling number verification using signature verification and attestation information (clause 6.11.4)

6.11.2	Adaptation of the trust domain

In the RTC network conforms to this document, a member of a trust domain consists of the C-Plane functional entities (i.e., WSF and IWF). C-Plane functional entities that belongs to the other RTC network can be a member of the trust domain only if there is an inter-connection agreements.

All the functional entity in the trust domain needs to comply with a clearly defined specification set and a functional entity in the trust domain needs to know that all the functional entities in the trust domain will behave as defined in the specification set through configuration information. This specification set is termed "Spec(T)" in IETF RFC 3324 [22].

In this document, "Spec(T)" is given per feature or information element(s) in C-Plane signalling message. Also, the concept of the trust domain is intended for the identity related problem, but this document may extend the use of the trust domain for the protection of sensitive information within the operator network.

6.11.3	Network-asserted identity within the trust domain

As a network-asserted identity of an originating RTC user, this document supports the following three types of identifies and these are contained in the "network" object of "oId" object as defined in this document:

-	RTC user ID (URI format),

-	Telephone number (global number digits excluding "+"), and

-	Display name.

These are identities initially derived by a functional entity in an RTC network (i.e., WSF) as a result of an authentication process over C-Plane signalling, and these are not RTC user-provided identities.

This document also provides the privacy mechanism just like IETF RFC 3325 [23], by which an originating UE indicates the privacy to prevent presentation of its identities to a WebRTC endpoint of final destination. Based on this privacy information, a functional entity on the terminating side within a trust domain will handle (forward, delete or anonymize) the originating RTC user's identities.

This feature is a target of trust domain and a "Spec(T)" for this feature is given as follows:

1.	Protocol requirements

-	The protocol specification specific to this feature (handling of "network" and "privacy" object in "oId" object) described in this document needs to be supported.

2.	Authentication requirements

-	RTC users need to be authenticated through authentication procedures using the signalling protocol described in this document. As an authentication mechanism, the signalling protocol in this document applies bearer authentication, basic authentication, and digest authentication.

3.	Security requirements

-	Members within the trust domain needs to be in trusted DN and use the Secure WebSocket for transport of signalling messages.

4.	Scope of trust domain for this feature

-	A trust domain for the network-asserted identity consists of C-Plane functional entities (i.e., WSF and IWF) in an RTC network. In addition to this, C-Plane functional entities that belongs to the other RTC network can be a member of the trust domain only if there is an inter-connection agreements on this feature.

6.11.4	Adaptation of calling number verification using signature verification and attestation information

For the interconnection scenario (i.e., collaboration scenario 4), this document adapts a framework "calling number verification using signature verification and attestation information" used for the IMS interconnection defined in 3GPP TS 29.165 [16] with an extension. In this framework, an originating RTC network sets a signature for an originating RTC user identity into a setup request of C-Plane signalling and then a terminating RTC network within a trust domain validates the signature. This signature is generated at an originating RTC network by constructing "PASSporT (Personal Assertion Token)" JSON object and signing a hash of this JSON object with private key associated with the appropriate credential for the identity. At the terminating RTC network, the received signature is validated by using public key.

While target of verification is a telephone number of an originating user in IMS interconnection, this document extends its use to validating both RTC user ID (URI format) and telephone number.

NOTE:	The mechanism defined in IETF RFC 8224 [34] are used in IMS network, and this mechanism works not only for telephone-number but also URI.

As an extension in this framework, this document also applies the feature to provide the levels of attestation to a terminating RTC network as with IMS network.

This feature is a target of trust domain and a "Spec(T)" for this feature is given as follows:

1.	Protocol requirements

-	The protocol specification specific to this feature (handling of "passport" object in "oId" object) described in this document needs to be supported.

2.	Authentication requirements

-	RTC users need to be authenticated through authentication procedures using the signalling protocol described in this document. As an authentication mechanism, the signalling protocol in this document applies bearer authentication, basic authentication, and digest authentication.

3.	Security requirements

-	Members within the trust domain needs to be in trusted DN and use the Secure WebSocket for transport of signalling messages.

4.	Scope of trust domain for this feature

-	A trust domain for the network-asserted identity consists of C-Plane functional entities (i.e., WSF and IWF) in an RTC network. In addition to this, C-Plane functional entities that belongs to the other RTC network can be a member of the trust domain only if there is an inter-connection agreements on this feature.

6.12	Solution #11: Related groups considerations

6.12.1	Solution description

This solution addresses Key Issue #11.

As for the stage 2 specification:

-	Any necessary considerations for other 3GPP WG's stage 2 specifications are not identified since SA4 is responsible for the network architecture for RTC media services, thus the other 3GPP WG groups do not specify that network architecture.

-	Also, any necessary considerations for other organizations are not identified since the stage 2 descriptions of this document are 3GPP-specific and have no impact on any specifications developed in the other organizations (i.e., IETF and W3C).

As for the stage 3 (e.g., C-Plane signalling protocol, API):

-	The proposals in this document do not conflict with the IETF RFCs (i.e., referred IETF specifications in this document such as IETF RFC 8825 [44]) and W3C WebRTC 1.0 [65], therefore any considerations for other organizations are not needed.

6.12.2	Solution evaluation

There is no impact on the specifications of other WGs in 3GPP and organizations including IETF and W3C.

7	Conclusions and Recommendations

7.1	General

In this clause, the expected normative works during release-19 timeframe are summarized as conclusions of this Study Item (FS_eiRTCW). The expected normative works are summarized per stage-2 and stage-3 aspects, based upon the solution evaluations for enabling collaboration scenario 4 and collaboration scenario 3 in clause 6.

7.2	Expected normative works in stage-2 aspect

The architecture diagram that best represents the architectural solutions in this document is shown in Figure 7.2-1. While Network Support function (NS-AF) is snipped in the figure, this document expects RTC AF to be integrated with WebRTC Signalling Function (WSF) to interact with 5GC via N5 interface, as indicated in 3GPP TS 26.506 [12] clause 4.2.5.

Also, as an extension of collaboration scenario 4, this document addresses the interworking between RTC network and IMS network over NNI (RTC-Z reference point) which conforms to 3GPP TS 29.162 [15].

Figure 7.2-1:	Derivative RTC architecture diagram

For the expected normative works in stage-2 aspect, the works of A) B) are recommended to proceed.

A)	New reference points

-	This document identifies that the following reference points are required to be newly introduced, in addition to the existing reference point defined in 3GPP TS 26.506 [12] clause 4.3. These reference points need to be assigned/defined considering the common architecture for 5GMS and RTC.

1)	RTC-X

*	To enable the immersive RTC service in collaboration with content providers, the interface between a ASWF and a content provider network for service control is needed. Over this reference point, ASWF is expected to provide the service control APIs.

2)	RTC-Y

*	To enable the collaboration scenario 4 supporting interoperability between multiple MNOs, the interface between two different operator's RTC networks or between an operator's RTC network and service provider's network is needed.

*	RTC-Y is divided into two sub-interfaces, RTC-Ys (for C-Plane signalling) and RTC-Ym (U-Plane media/data transport).

3)	RTC-Z

*	To enable the interworking with IMS network as an extension of collaboration scenario 4, the interface between RTC network and IMS network is needed.

*	RTC-Z is also divided into two sub-interfaces, RTC-Zs (for C-Plane signalling) and RTC-Zm (U-Plane media/data transport).

B)	Functional enhancements on the existing entities

-	This document identifies that the functional enhancements on the existing entities shown in Table 7.2-2 are required against the functional definitions in 3GPP TS 26.506 [12] clause 4.2. This table does not show all the enhancements, but major enhancements identified in this document.

-	This document does not identify the new functional entity needed.

Table 7.2-2:	Major expected functional enhancements

| Entity | Expected functional enhancement | Interface | Solution |
| --- | --- | --- | --- |
| UE | Support of WSF discovery mechanism at UE. | RTC4m | Sol#6 |
| WSF | Interaction with MF for media session control. | - | - |
|  | Interaction with ASWF for collaboration with web applications/services. | - | - |
|  | Interaction with 5GC, using network Support function (NS-AF). | N5 | Sol#1 |
|  | Authentication/authorization of the UE. | RTC-4s | Sol#2 |
|  | Support of functionalities needed for service control. - Connection control enforcer - RTC ID resource handling enforcer | RTC-X | Sol#5 |
|  | Signing and verification of network-asserted UE's ID. | RTC-Ys | Sol#10 |
| MF | Support of functionalities needed for service control. - Media data forwarding control enforcer - RTC exchange resource handling enforcer | RTC-X | Sol#5 |
| ASWF | Exposing the service control APIs. | RTC-X | Sol#5 |
|  | Storage of user subscription data specific to MNO's WebRTC services. | - | - |
|  | Providing supplementary files via best-effort transport different from the channels for real-time media. | RTC-4m | Sol#1 |
|  | Providing WSF discovery functionality. | RTC-4m | Sol#6 |
| IWF | C-plane signalling protocol interworking between RTC network and IMS network. | RTC-Zs | Sol#7 |
|  | Signing and verification of network-asserted UE's ID. | RTC-Ys | Sol#10 |
| TGF | U-Plane protocol interworking between RTC network and IMS network. | RTC-Ym | Sol#10 |

Annex A (informative):
Use cases

A.1	General

In this annex, the following typical use cases targeted in this document are described.

- VR streaming in first person view (see clause A.2)

- VR streaming in third person view (see clause A.3)

- Voice communication and media analytics (see clause A.4)

- VR streaming over NNI (see clause A.5)

- Peer-to-peer communication (see clause A.6)

A.2	VR streaming in first person view

A.2.1	General

This use case is an example where a UE connects to an RTC ID resource (i.e., VR space) and receives audio/video media rendered by a content provider based on the user's operations (e.g., scroll, zoom out). In this use case, the user enjoys media streamed by the content provider in first person view.

A.2.2	Service flows

- A user visits a website provided by a content provider and sign-in to an account through authentication. After successful sign-in, the necessary JavaScript files are downloaded to the web browser in the UE and the procedure for establishing a media session with data network (RTC ID resource) is started when the user takes a certain action on the web browser. In this procedure, the content provider assigns the RTC ID resource (URI) which is the destination of media session connection, requests the operator to configure the URI, and informs the UE of the URI.

- The content provider starts to set up a media session with data network using the URI after receiving a notification indicating the attempt of session establishment by the UE from the operator. After establishing the media session between the UE and the data network and between the content provider and the data network, the content provider sends audio and video media to the data network and exchanges the data via datachannel (Figure A.2.2-1). Accordingly, the user enters the personal room in the VR space.

- The UE sends an application-specific data to content provider via the datachannel. Based on this data, the content provider renders media (e.g., displayed video media, BGM).

Figure A.2.2-1: Media/data flows for VR streaming in first person view

A.3	VR streaming in third person view

A.3.1	General

This use case is an example where multiple UEs connect to an RTC ID resource (i.e., VR room) and receive audio/video media rendered by a content provider based on the user's operations (e.g., scroll, zoom out). In this use case, the users enjoy same media streamed by the content provider in third person view.

A.3.2	Service flows

- The content provider establishes the media session with data network for streaming media for the third person view.

- UE_A and UE_B establish the media sessions with the data network according to the flows described in clause A.2.2. After establishing the media sessions, the content provider sends audio and video media to the data network and exchanges the data via datachannel (Figure A.3.2-1). Accordingly, the users enter the VR space.

- Each UEs sends the application-specific data to content provider via the datachannel. Based on this data, the content provider renders media (e.g., displayed video media, BGM). User_A and User_B see the same view (e.g., own avatar's moving and other user's avatar moving) in third person view if the users are in the same square in the VR space.

Figure A.3.2-1: Media/data flows for VR streaming in third person view

A.4	Voice communication and media analytics

A.4.1	General

This use case is an example where multiple users talk each other by connecting RTC ID resource_2 (i.e., conference room) after connecting to RTC ID resource_1 (i.e., VR space) according to the service flows described in clause A.3.2.

In this example, an analytics server exists in the content provider network in order to analyze audio and video media sent from the UEs and reflect the analysis result to media provided by the rendering server. The analytics server sends the analysis result to a rendering server in proprietary manner and based on the analysis result the content provider renders media (e.g., displayed video media, BGM). Audio/video media and data are not exchanged between the analytics server and the rendering server.

A.4.2	Service flows

- The analytics server in the content provider establishes the media session with the data network for analyzing upstreaming media (e.g., user's video and user's voice) from UE(s).

- UE_A and UE_B establish the media sessions with the data network according to the flows described in clause A.3.2. In this procedure, the UE informs the data network not to send video media to anywhere if the user unwanted. After establishing the media sessions, the UEs in the same VR space can talk each other (Figure A.4.2-1).

Figure A.4.2-1: Media/data flows for voice communication and media analytics

A.5	VR streaming over NNI

A.5.1	General

This use case is an example where a UE connects to an RTC ID resource (i.e., VR space) and receives audio/video media rendered by a content provider accommodated by different operator than that UE attached to, based on the user's operations (e.g., scroll, zoom out). In this use case, the user enjoys media streamed by the content provider in first person view.

A.5.2	Service flows

- There is no difference with service flows in clause A.2.1 except that the operator accommodating the content provider differs from that the UE attached to (Figure A.5.2-1).

Figure A.5.2-1: Media/data flows for VR streaming over NNI

A.6	Peer-to-peer communication

A.6.1	General

This use case is an example where multiple users talk each other via the data network. UE_A and UE_B connect to an RTC ID resource (i.e., VR space) according to the service flows described in clause A.2.2.

In this example, UE_A makes a call to UE_B identified by the application specific identity (i.e., RTC user ID), or UE_A makes a call to the IMS-UE identified by the telephone number.

A.6.2	Service flows

- UE_A and UE_B establish the media sessions with the data network according to the flows described in clause A.2.2 for entering a VR space.

- In this VR space, UE_A calls UE_B identified by application specific identity (i.e., RTC user ID) or calls IMS-UE identified by the telephone number and enjoy the voice communication in addition to the VR space (Figure A.6.2-1).

Figure A.6.2-1: Media/data flows for Peer-to-peer communication

Annex B (informative):
Examples

Editor’s note: Description will be added.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-11 | SA4#126 | S4-231727 |  |  |  | initial version | 0.1.0 |
| 2023-11 | SA4#126 | S4-231997 |  |  |  | Scope, References, Definitions of terms, symbols and abbreviations, Motivations for Native WebRTC Signalling and assumptions,Key issue #1, Key issue #2, Key issue #3, Key issue #4, Key issue #5, Key issue #6, Key issue #10, Solution #1, Solution #2, Solution #4, Solution #6 are incorporated. | 0.2.0 |
| 2024-01 | SA4#127 | S4-240XXX |  |  |  | Scope, Key issue #1, Key issue #5, Key issue #6, Key issue #10, Key issue #11, Solution #1, Solution #2, Solution #5, Solution #10, Solution #11, Conclusions and Recommendations, Use cases. | 0.3.0 |