Source:	HEAD acoustics GmbH

Title:	Results with new error profile for JBM performance testing

Document for:	Discussion

# Introduction

In [1], a new error profile for testing JBM performance was proposed for revised clauses 7/8/9/10.13 in TS 26.132 [2], which contains packet duplications and delay bursts between 160 ms (base delay) and 260 ms. The first 37.0 s of the test signal (out of 165 s) are shown jointly with the new profile in Figure 1. To evaluate this new profile, a test series with NN devices was conducted.

Figure 1: Test signal vs error profile

It can be noticed that the range of delay variation ranges is 100 ms, i.e., from 160 ms to 260 ms. For the JBM in the UE, it is expected that this would correspond to a maximum of (around/up to) five additional packets in the jitter buffer.

# Test setup

The devices as shown in Table 1. To void any impact of acoustic paths on the test results, all tests were conducted via digital electrical interface (USB).

Table 1: UEs used in JBM test series

| Title | Release year |
| --- | --- |
| DUT-1 | 2020 |
| DUT-2 | 2015 |
| DUT-3 | 2023 |
| DUT-4 | 2020 |
| DUT-5 | 2015 |

The speech quality and delay-vs-time tests for VoLTE/VoNR were conducted in NB (AMR 12.2 kbit/s), WB (AMR-WB 12.65 kbit/s), and SWB (EVS-SWB 24.4 kbit/s) mode. Due to time constraints, not all devices could be tested at all bandwidths.

# Measurement results

The result overview is shown in Table 2. The following conventions apply:

- MOS-LQO and round-trip delays are reported for different profiles:

  - C0: jitter- and error-free measurement (Condition #0).

  - AVP: proposed error profile ("Artificial VoLTE Profile")

  - AVP-ND: proposed error profile, but without duplicated packets.

- "Δ-MOS-LQO [X]" is calculated versus the result of "C0 – [X]".

For reference, the performance objectives and requirements for error-/jitter-free conditions (C0) from TS 26.131 [3] are shown in Table 3 (including an additional budget BD of 20 ms for electrical interface).

Table 2: Test results

| DUT | Band- width | MOS- LQO C0 | MOS-LQO AVP | MOS-LQO AVP-ND | Δ-MOS AVP | Δ-MOS AVP-ND | Delay C0 | Delay AVP | Delay AVP-ND |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUT-1 | NB | 4.64 | 3.16 | 3.11 | 1.48 | 1.53 | 131.5 | 211.5 | 211.5 |
|  | WB | 4.08 | 3.59 | 3.76 | 0.49 | 0.32 | 171.5 | 241.7 | 241.7 |
|  | SWB | 4.75 | 4.75 | 4.75 | 0.00 | 0.00 | 164.0 | 277.0 | 277.0 |
| DUT-2 | NB | 4.66 | 3.44 | 3.42 | 1.22 | 1.24 | 540.8 | 674.7 | 684.7 |
|  | WB | 4.18 | 3.31 | 4.15 | 0.88 | 0.04 | 531.7 | 645.9 | 682.9 |
|  | SWB | 4.59 | 4.51 | 4.58 | 0.08 | 0.01 | 529.8 | 657.4 | 677.0 |
| DUT-3 | NB | 3.08 | 1.85 | 2.10 | 1.23 | 0.98 | 260.5 | 362.3 | 380.8 |
| DUT-4 | NB | 4.62 | 2.45 | 2.54 | 2.17 | 2.08 | 187.9 | 248.1 | 248.1 |
|  | WB | 4.16 | 2.93 | 2.87 | 1.23 | 1.29 | 197.1 | 277.0 | 277.5 |
|  | SWB | 4.69 | 3.58 | 3.52 | 1.12 | 1.18 | 151.5 | 251.4 | 250.4 |
| DUT-5 | NB | 4.41 | 4.00 | 3.92 | 0.41 | 0.49 | 128.0 | 188.7 | 208.1 |
|  | WB | 4.27 | 4.15 | 4.15 | 0.12 | 0.12 | 151.9 | 231.9 | 231.8 |
|  | SWB | 4.21 | 4.03 | 4.08 | 0.18 | 0.13 | 131.8 | 211.7 | 211.7 |
| DUT-6 | NB | 4.43 | 3.35 | 3.41 | 1.08 | 1.02 | 125.6 | 251.1 | 246.0 |
|  | WB | 4.35 | 4.28 | 4.27 | 0.07 | 0.08 | 151.5 | 257.6 | 270.9 |

Table 3: Existing delay requirements for C0 with electrical/digital interface (BD = 20ms)

| Bandwidth | Performance objective [ms] | Performance requirement [ms] |
| --- | --- | --- |
| NB | 170 | 210 |
| WB | 170 | 210 |
| SWB | 177 | 217 |

NOTE:	DUT-2 shows extremely high delay values and does not pass any of the roundtrip delay requirements for C0. This is an expected behaviour since this device is an early EVS-SWB-capable prototype.

# Conclusion

The results obtained by the test series meets the expectations:

- Independent of the quality loss, obviously all devices increase their jitter buffer size significantly, resulting in a much larger round-trip delay.

- In general, the trade-off between larger delays (increased jitter-buffer size) versus possible lost packets (smaller jitter-buffer size) is handled in different ways by the devices. Most of them do not show much loss in quality but have much higher resulting delays.

- Except for DUT-5, all Δ-MOS-LQO values in NB mode are much higher than for WB or even SWB. This could indicate that JBM of IMS-based calls with AMR (and in some cases, also AMR-WB) were not sufficiently considered by the manufacturer.

- In most cases, Δ-MOS-LQO determined with and without packet duplication is very close (<0.1), which indicates that most devices simply ignore duplicated packets.

- Only DUT-2 in WB mode shows a significant lower performance with duplicated packets. An additional increase of delay (~40 ms) compared to the profile without duplicates can also be observed.

Similar to the corresponding clauses on delay measurements (5.12, 6/7/8.11) in TS 26.131 [3], possible requirements for Δ-MOS and delay (absolute or delta to C0) have to be found for the new profile. The results shown in the present document may be considered for this purpose: Table 4 and Table 5 provide averaged results per bandwidth of Δ-MOS-LQO and Δ-Delay. The latter one is indicated as number of packets, since the increase in delay is caused by the increased size of the jitter buffer in the UE.

Table 4: Δ-MOS and Δ-Delay aggregated per bandwidth (with duplicates)

|  | Δ-MOS AVP |  |  | Δ-Delay AVP (# packets) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | min | mean | max | min | mean | max |
| Bandwidth |  |  |  |  |  |  |
| NB | 0.41 | 1.27 | 2.17 | 3 | 4.7 | 7 |
| WB | 0.07 | 0.56 | 1.23 | 4 | 4.6 | 6 |
| SWB | 0.00 | 0.35 | 1.12 | 4 | 5.3 | 6 |

Table 5: Δ-MOS and Δ-Delay aggregated per bandwidth (without duplicates)

|  | Δ-MOS AVP-ND |  |  | Δ-Delay AVP-ND (# packets) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | min | mean | max | min | mean | max |
| Bandwidth |  |  |  |  |  |  |
| NB | 0.49 | 1.22 | 2.08 | 3 | 5.0 | 7 |
| WB | 0.04 | 0.37 | 1.29 | 4 | 5.2 | 8 |
| SWB | 0.00 | 0.33 | 1.18 | 4 | 5.5 | 7 |

# References

| [1] | 3GPP S4-232019, „Updated proposal for JBM performance testing,“ Orange, Chicago. |
| --- | --- |
| [2] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release-17.1. |
| [3] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release-17.1. |