3GPP TSG-SA4 Meeting #127	S4-240110

# Sophia-Antipolis, France, 29th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | pseudo | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [PROMISE] pCR 26.143 Updates to Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PROMISE |  |  |  |  |  | Date: |  |  | 2024-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capabilities were not fully defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defines encoding and decoding capabilities for different media types |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5, 5.1, 5.2, 5.3, 5.4, 5.5., 5.6, 5.7, 5.8, 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Remove some Editor’s Notes before integration |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Khronos glTF 2.0, glTF™ 2.0 Specification (khronos.org)

[3] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[4]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[5]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[6]	IETF Draft draft-ietf-mimi-content-01: "More Instant Messaging Interoperability (MIMI) message content", 	Rohan Mahy

[7]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[8]	Open Mobile alliance, "MMS Architecture" OMA-AD-MMS-V1_3-20110913-A.

[9]	Open Mobile alliance, "Multimedia Messaging Service Encapsulation Protocol" OMA-TS-MMS_ENC-V1_3-20110913-A.

[10]	GSMA "RCS Universal Profile Service Definition Document", Version 2.6, 19 December 2022

[11]	GSMA PRD RCC.07 version 13.0 - "Rich Communication Suite - Advanced Communications Services and Client Specification" 19 December 2022[12]	IETF RFC 2046: "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types"

[12]	IETF RFC 2046, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types"

[13]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[14]	ISO/IEC 23000-24:2023 Preliminary Draft of: Information technology — Multimedia application format (MPEG-A) — Part 24: Messaging Media Application Format (MeMAF) ".

NOTE: 	A preliminary draft of this standard is available as MDS23345_W03_N1082 here: https://www.mpeg.org/wp-content/uploads/mpeg_meetings/144_Hannover/w23345.zip

[15]	3GPP 23.140: "Multimedia Messaging Service (MMS); Functional Description; Stage 2".

[16]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[17]	"JPEG File Interchange Format", Version 1.02, September 1, 1992.

[18]	"Exchangeable image file format for digital still cameras: EXIF 2.2", Specification by the Japan Electronics and Information Technology Industries Association (JEITA), April 2002, URL: http://www.exif.org/

[19]	CompuServe Incorporated: "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA, 1987.

[20]	Compuserve Incorporated, Columbus, Ohio (1990): "Graphics Interchange Format (Version 89a)".

[21]	IETF RFC 2083: "PNG (Portable Networks Graphics) Specification version 1.0 ", T. Boutell, et. al., March 1997.

[22]	ISO/IEC 23000-22:2019 Information technology — Multimedia application format (MPEG-A) — Part 22: Multi-image application format (MIAF)

[23]	IETF RFC 2045, "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies", November 1996

[24]	ISO/IEC 23008-12:2019 Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 12: Image File Format

[25]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[26]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

[27]	3GPP TS 26.245: "Transparent end-to-end packet switched streaming service (PSS); Timed text format"

[28]	ISO/IEC 14496-30: "Information technology - Coding of audio-visual objects - Part 30: Timed text and other visual overlays in ISO base media file format".

[29]	IETF RFC 2387, "The MIME Multipart/Related Content-type"

[30]	IETF RFC 6381, "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types"

[31]	3GPP TS 26.307, "Presentation Layer for 3GPP Services"

[32]	3GPP TS 26.307, "Multimedia Messaging Service (MMS); Media formats and codecs"

# =====  CHANGE =====

# 5	MMBP Media Capabilities for different Media Types

## 5.1	Introduction

In order to guarantee a minimum support and compatibility between messaging capable terminals, MMBP Generators and MMBP Players in UEs supporting specific media content with associated media types.

The clause defines multimedia messaging body parts (MMBPs) for different media types as well as the associated media types.

According to the introduction in clause 4, MMBPs defined in this clause,

-	may be used as full body parts or sub-parts in message bodies.

-	may either be a single binary octet string, or they may consist of multiple parts. If the latter, the conceptual relationship introduced in clause 4.4 is defined that is mapped to container formats defined in clause 5.2.

Media Types and related capabilities defined in this specification for playback are provided in Table 5.1-1.

Table 5.1-1 Media Types and Capabilities defined in TS 26.143 for playback and decoding

| Media Type in the present TS | Capabilities defined in this specification | Clause | Media Type signalling example |
| --- | --- | --- | --- |
| Multipart MMBPs and Container Formats | 26143_CONTAINER_RFC2046_SINGLE 26143_CONTAINER_RFC2046_MIXED 26143_CONTAINER_RFC2046_ALTERNATIVE 26143_CONTAINER_RFC2046_PARALLEL 26143_CONTAINER_RFC2387_RELATED 26143_CONTAINER_MP4_3GP9 | 5.2.1 | Media type of subtype multipart/mixed multipart/alternative multipart/parallel multipart/related video/mp4, profile="3gp9" |
| Text | 26143_TEXT_PLAIN | 5.3.1 | text/plain |
| Speech | 26143_AUDIO_EVS 26143_AUDIO_AMR-WB 26143_AUDIO_AMR | 5.5.1 | audio/mp4 |
| Audio | 26143_AUDIO_XHE-AAC 26143_AUDIO_EAAC+ | 5.5.1 | audio/mp4 |
| Image | 26143_IMG_JPEG 26143_IMG_HEIC 26143_IMG_GIF 26143_IMG_PNG | 5.4.1 | image/jpeg image/heic, profile="heic,mif1" codecs="hvc1.1.2.L153.B0" image/gif image/png |
| Video | 26143_VIDEO_AVC-HD 26143_VIDEO_AVC-FullHD 26143_VIDEO_HEVC-HD 26143_VIDEO_HEVC-FullHD 26143_VIDEO_HEVC-UHD | 5.6.1 | video/mp4, profile="3gp9" codecs="avc1.640028" video/mp4, profile="3gp9" codecs="avc1.640029" video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L123.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L153.B0" |
| Subtitles and Text | 26143_TT_3GPP 26143_TT_IMSC11 | 5.7.1 | text/mp4, profile="3gp9" codecs="tx3g" application/mp4, profile="3gp9" codecs="stpp.ttml.im2t" |
| 3d scenes and assets | 26143_SCENE_GLTF20 26143_SCENE_GLTF20_AR 26143_SCENE_GLTF20_GLB 26143_SCENE_GLTF20_GLB_AR | 5.8 | model/gltf+json model/gltf-binary |
| Presentation format | 26143_PRESENTATION_HTML5 | 5.9 | text/html |

Media Types and related capabilities defined in this specification for content generation are provided in Table 5.1-2.

Table 5.1-2 Media Types and Capabilities defined in TS 26.143 for generation

| Media Type in the present TS | Capabilities defined in this specification | Clause | Media Type signalling example |
| --- | --- | --- | --- |
| Multipart MMBPs and Container Formats | 26143_CONTAINER_RFC2046_SINGLE_GEN 26143_CONTAINER_RFC2046_MIXED_GEN 26143_CONTAINER_RFC2046_ALTERNATIVE_GEN 26143_CONTAINER_RFC2046_PARALLEL_GEN 26143_CONTAINER_MP4_3GP9_GEN | 5.2.2 | Media type of subtype multipart/mixed multipart/alternative multipart/parallel video/mp4, profile="3gp9" |
| Text | 26143_TEXT_ENC_PLAIN | 5.3.1 | text/plain |
| Speech | 26143_AUDIO_ENC_EVS 26143_AUDIO_ENC_AMR-WB 26143_AUDIO_ENC_AMR | 5.5.1 | audio/mp4 |
| Audio | 26143_AUDIO_ENC_XHE-AAC 26143_AUDIO_ENC_EAAC+ | 5.5.1 | audio/mp4 |
| Image | 26143_IMG_ENC_JPEG | 5.4.1 | image/jpeg |
| Video | 26143_VIDEO_ENC_AVC-HD 26143_VIDEO_ENC_AVC-FullHD 26143_VIDEO_ENC_HEVC-HD 26143_VIDEO_ENC_HEVC-FullHD 26143_VIDEO_ENC_HEVC-UHD | 5.6.2 | video/mp4, profile="3gp9" codecs="avc1.640028" video/mp4, profile="3gp9" codecs="avc1.640029" video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L123.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L153.B0" |
| Text | 26143_TT_ENC_3GPP | 5.7.2 | text/mp4, profile="3gp9" codecs="tx3g" |

# =====  CHANGE =====

## 5.2	Multipart MMBPs and Container Formats

### 5.2.1	Player and Decoding capabilities

The capability 26143_CONTAINER_RFC2046_SINGLE is defined as the capability of processing a body part of with a top-level media type as defined in IETF RFC 2046 [12] with one of the following top-level media types: text, audio, image, video, model, multipart, and application.

The capability 26143_CONTAINER_RFC2046_MIXED is defined as the capability of processing a body part of subtype multipart/mixed as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for mixed MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_MIXED shall be signalled with multipart/mixed as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2046_ALTERNATIVE is defined as the capability of processing a body part of subtype multipart/alternative as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for alternative MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_MIXED shall be signalled with multipart/alternative as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2046_PARALLEL is defined as the capability of processing a body part of subtype multipart/parallel as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_PARALLEL shall be signalled with multipart/parallel as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2387_RELATED is defined as the capability of processing a body part of subtype multipart/related as defined in IETF RFC 2387 [29] further restricted by the processing defined in clause 4.4 for related MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2387_RELATED shall be signalled with multipart/parallel as defined in IETF RFC 2387 [29] with the root MMBP either signalled with the start parameter, or if not present, the root MMBP being the first body part within the Multipart/Related body and the type of the root MMBP signalled with the type parameter.

The capability 26143_CONTAINER_MP4_3GP9 is defined as the capability of processing a body part conforming to a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9' and further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_MP4_3GP9 shall be signalled using a media type as defined IETF RFC 6381 [30] using for example video/mp4, profile="3gp9" or an equivalently compatible media type and shall use the codecs parameter to further provide information about the contained MMBPs.

NOTE: This specification does not define mechanisms for referencing external content. This aspect is for further study.

### 5.2.2	MMBP Content Generator capabilities

The capability 26143_CONTAINER_RFC2046_SINGLE_GEN is defined as the capability of generating a body part of with a top-level media type as defined in IETF RFC 2046 [12] with one of the following top-level media types: text, audio, image, video, and multipart that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_SINGLE.

The capability 26143_CONTAINER_RFC2046_MIXED_GEN is defined as the capability of generating a body part of subtype multipart/mixed as defined in IETF RFC 2046 [12] with media type signalling with multipart/mixed as defined in IETF RFC 2046 [12] that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_MIXED.

The capability 26143_CONTAINER_RFC2046_ALTERNATIVE_GEN is defined as the capability of generating a body part of subtype multipart/alternative as defined in IETF RFC 2046 [12] with media type signalling with multipart/alternative as defined in IETF RFC 2046 [12] that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_ALTERNATIVE.

The capability 26143_CONTAINER_RFC2046_PARALLEL_GEN is defined as the capability of generating a body part of subtype multipart/parallel as defined in IETF RFC 2046 [12] with media type signalling with multipart/parallel as defined in IETF RFC 2046 [12] that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_PARALLEL.

## The capability 26143_CONTAINER_MP4_3GP9_GEN is defined as the capability of generating a body part conforming to a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9' using a media type as defined IETF RFC 6381 [30], for example video/mp4, profile="3gp9" or an equivalently compatible media type and using the codecs parameter to further provide information about the contained MMBPs.

# =====  CHANGE =====

## 5.3	Text

### 5.3.1	Player and Decoding capabilities

The capability 26143_TEXT_PLAIN is defined as the capability of decoding and rendering plain text with any character encoding (charset) that contains a subset of the logical characters in Unicode [2] (e.g. US-ASCII [3], ISO-8859-1 [4], UTF-8 [5], Shift_JIS, etc.). In the context of this specification, the media type for text with this capability 26143_TEXT_PLAIN shall be signalled with text/plain as defined in IETF RFC 2046 [12]. However, the decoding and rendering capability 26143_TEXT_PLAIN

includes that unrecognized subtypes of text shall be treated as subtype plain as long as the MIME implementation knows how to handle the charset.

Interoperability with SMS/MMS text type is according to [15].

### 5.3.2	MMBP Content Generator capabilities

The capability 26143_TEXT_ENC_PLAIN for a content generator is defined as the combination of the following capabilities:

-	the capability to generate plain text with any character encoding (charset) that contains a subset of the logical characters in Unicode [2], such that the file can be played back by a player with the capability 26143_TEXT_PLAIN,

-	the provisioning of media type signalling with the MMBP using text/plain as defined in IETF RFC 2046 [12].

# =====  CHANGE =====

## 5.4	Image

### 5.4.1	Player and Decoding capabilities

The capability 26143_IMG_JPEG is defined as the capability of decoding and rendering images according to ISO/IEC JPEG [8] together

-	with JFIF [16] and the following two modes:

-	baseline DCT, non-differential, Huffman coding, as defined in table B.1, symbol ‘SOF0’ in [17];

-	progressive DCT, non-differential, Huffman coding, as defined in table B.1, symbol ‘SOF2’ [17].

-	with EXIF compressed image file format as defined in [18] and the baseline DCT mode.

In the context of this specification, the media type for images with this capability 26143_IMG_JPEG shall be signalled with image/jpeg as defined in IETF RFC 2046 [12].For JPEG baseline DCT, EXIF compressed image file format should also be supported, as defined in [18].

The capability 26143_IMG_HEIC is defined as the capability of decoding and rendering images conforming to

-	the 'heic' brand as defined in ISO/IEC 23008-12 [24],

-	the 'mif1' brand as defined in ISO/IEC 23000-22:2019 [22], and

-	the contained elementary bitstream conforming to H.265 (HEVC) Main Profile, Main Tier, Level 5.1[25] bitstreams have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

In the context of this specification, the media type for images with this capability 26143_IMG_HEIC shall be signalled with image/heic, profile="heic,mif1" codecs="hvc1.1.2.L153.B0" or an equivalently compatible media type as defined in [22].

The capability 26143_IMG_GIF is defined as the capability of decoding and rendering bitmap graphics conforming either to

GIF87a [19] or to

GIF89a [20]. In the context of this specification, the media type for images with this capability 26143_IMG_GIF shall be signalled with image/gif as defined in IETF RFC 2046 [X].

The capability 26143_IMG_PNG is defined as the capability of decoding and rendering bitmap graphics conforming to PNG [21]. In the context of this specification, the media type for images with this capability 26143_IMG_GIF shall be signalled with image/png.

### 5.4.2	MMBP Content Generator capabilities

The capability 26143_IMG_ENC_JPEG for a content generator is defined as the combination of the following capabilities:

-	the capability to generate an image according to ISO/IEC JPEG [8] that can be played by a player conforming to 26143_IMG_JPEG as defined in clause 5.4.1, i.e. restricted by either

-	using JFIF [16] and one of the following two modes:

-	baseline DCT, non-differential, Huffman coding, as defined in table B.1, symbol ‘SOF0’ in [17], or

-	progressive DCT, non-differential, Huffman coding, as defined in table B.1, symbol ‘SOF2’ [17].

-	using EXIF compressed image file format as defined in [18] and the baseline DCT mode.

-	the provisioning of media type signalling with the MMBP using image/jpeg as defined in IETF RFC 2046 [12].

# =====  CHANGE =====

## 5.5	Speech and Audio

### 5.5.1	Player and Decoding capabilities

The capability 26143_AUDIO_EVS is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the EVS decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.4.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'sevs' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profile="3gp9" codecs="sevs" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR-WB is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR-WB decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.3.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'sawb' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profile="3gp9" codecs="sawb" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'samr' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profile="3gp9" codecs="samr" or an equivalently compatible media type.

The capability 26143_AUDIO_XHE-AAC is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the xHE-AAC stereo decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.4.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'mp4a.40.29',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_XHE-AAC shall be signalled with audio/mp4, profile="3gp9" codecs="mp4a.40.29" or an equivalently compatible media type.

The capability 26143_AUDIO_EAAC+ is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the eAAC+ decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.3.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'mp4a.40.5',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profile="3gp9" codecs="mp4a.40.5" or an equivalently compatible media type.

### 5.5.2	MMBP Content Generator capabilities

The capability 26143_AUDIO_ENC_EVS for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_EVS,

-	the EVS encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.4.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the codec entry 'sevs' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="sevs" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR-WB for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR-WB,

-	the AMR-WB encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.3.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the codec entry 'sawb' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="sawb" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR,

-	the EVS encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the codec entry 'samr' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="samr" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_XHE-AAC for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_XHE-AAC,

-	the xHE-AAC stereo encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.4.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the codec entry 'mp4a.40.29' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="mp4a.40.29" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_EAAC+ for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_EAAC+,

-	the eAAC+ encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.3.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the codec entry 'mp4a.40.5' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="mp4a.40.5" or an equivalently compatible media type.

# =====  CHANGE =====

## 5.6	Video

### 5.6.1	Player and Decoding capabilities

Editor’s note: 16:9 and 9:16 image formats are needed. Resolutions should be aligned with the considerations above, also in terms for video signal characteristics.

The capability 26143_VIDEO_AVC-HD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1 of TS 26.511 [4],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'avc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_AVC-HD shall be signalled with video/mp4, profile="3gp9" codecs="avc1.640028" or an equivalently compatible media type.

The capability 26143_VIDEO_AVC-FullHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AVC-FullHD-Dec decoding capabilities as defined in clause 4.2.1.1 of TS 26.511 [4],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'avc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_AVC-FullHD shall be signalled with video/mp4, profile="3gp9" codecs="avc1.640029" or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-HD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-HD-Dec decoding capabilities as defined in clause 4.2.2.1 of TS 26.511 [4],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-HD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-FullHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-FullHD-Dec decoding capabilities as defined in clause 4.2.2.1 of TS 26.511 [4],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-FullHD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.2.4.L123.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-UHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-UHD-Dec decoding capabilities as defined in clause 4.2.2.1 of TS 26.511 [4],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the codec entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-UHD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.2.4.L153.B0" or an equivalently compatible media type.

Editor’s note: 16:9 and 9:16 image formats are needed. Resolutions should be aligned with the considerations above, also in terms for video signal characteristics.

### 5.6.2	MMBP Content Generator capabilities

Editor’s note: Do we define capabilities that are not used in media profiles?

The capability 26143_VIDEO_ENC_AVC-HD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_AVC-HD,

-	the AVC-HD-Enc encoding capabilities as defined in clause 4.1.2.2 of TS 26.511 [4] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the codec entry 'avc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs=" avc1.640028" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_AVC-FullHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_AVC-FullHD,

-	the AVC-FullHD-Enc encoding capabilities as defined in clause 4.1.2.2 of TS 26.511 [4] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the codec entry 'avc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs=" avc1.640029" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-HD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-HD,

-	the HEVC-HD-Enc encoding capabilities as defined in clause 4.2.2.2 of TS 26.511 [4] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the codec entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-FullHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-FullHD,

-	the HEVC-FullHD-Enc encoding capabilities as defined in clause 4.2.2.2 of TS 26.511 [4] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the codec entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.2.4.L123.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-UHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-UHD,

-	the HEVC-UHD-Enc encoding capabilities as defined in clause 4.2.2.2 of TS 26.511 [4] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the codec entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.2.4.L153.B0" or an equivalently compatible media type.

# =====  CHANGE =====

## 5.7	Timed Text and Subtitles

### 5.7.1	Player and Decoding capabilities

The capability 26143_TT_3GPP is defined as the capability of decoding and rendering a file that

-	includes a track conforming conforms to the sample entry 'tx3g' as defined in TS 26.245 [27] and

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_TT_3GPP shall be signalled with text/mp4, profile="3gp9" codecs="tx3g" or an equivalently compatible media type.

The capability 26143_TT_IMSC11 is defined as the capability of decoding and rendering a file that

-	includes a track conforming conforms to the sample entry 'stpp.ttml.im2t' as defined in ISO/IEC 14496-30 [28] and

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_TT_IMSC11 shall be signalled with application/mp4, profile="3gp9" codecs="stpp.ttml.im2t" or an equivalently compatible media type.

### 5.7.2	MMBP Content Generator capabilities

The capability 26143_ENC_TT_3GPP for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a timed text signal, such that the file can be played back by a player with the capability 26143_TT_3GPP,

-	the capability to generate an ISO BMFF track from the bitstream track that conforms with the requirements of the sample entry 'tx3g' as defined in TS 26.245 [27].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="tx3g" or an equivalently compatible media type.

# =====  CHANGE =====

## 5.8	3D scenes and assets

The capability 26143_SCENE_GLTF20 is defined as the capability of rendering glTF 2.0 scenes as specified in [2] for which all components of the 3D scene are included as multiple parts in a related MMBP as defined in clause 4.4, for which the root MMBP is a glTF2.0 JSON document.

The capability 26143_SCENE_GLTF20_GLB is defined as the capability of rendering glTF 2.0 scenes as specified in [2] for which all components of the 3D scene are either

-	encapsulated in a GLB file, or

-	included as multiple parts in a related MMBP, for which the root MMBP is a GLB file.

The capability 26143_SCENE_GLTF20_AR is defined as the capability of 26143_SCENE_GLTF20 with the addition, that the scene may include MPEG_anchor extension, EXT_lights_image_based and MPEG_lights_texture_based extensions as defined in [3].

The capability 26143_SCENE_GLTF20_GLB_AR is defined as the capability of 26143_SCENE_GLTF20_GLB with the addition, that the scene may include MPEG_anchor extension, EXT_lights_image_based and MPEG_lights_texture_based extensions as defined in [3].

In the context of this specification, the media type for scenes with this capability 26143_SCENE_GLTF20 and 26143_SCENE_GLTF20_GLB_AR shall be signalled with model/gltf+json as defined in [2].

In the context of this specification, the media type for scenes with this capability 26143_SCENE_GLTF20_GLB and 26143_SCENE_GLTF20_GLB_AR shall be signalled with model/gltf+binary as defined in [2].

# =====  CHANGE =====

## 5.9	Presentation Format

The capability 26143_PRESENTATION_HTML5 is defined as the capability of rendering HTML-5 content according to the profile defined TS 26.307 [31] with the following further restrictions:

-	all components of the 3D scene are included as multiple parts in a related MMBP as defined in clause 4.4, for which the root MMBP is an HTML-5 document.

-	the APIs as defined in TS 26.307 [31], clause 5, are further restricted to the Forms API and the Canvas API.

-	the media types may be further restricted.

NOTE: As this specification does not define mechanisms for referencing external content, any network APIs from TS 26.307 [31] are excluded.

In the context of this specification, the media type for scenes with this capability 26143_PRESENTATION_HTML5 shall be signalled with text/html as defined in [2].