3GPP TSG-SA4 Meeting #127	S4-230134

# Sophia-Antipolis, FR,  29 Jan - 02 Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | 0058 | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Capability Exchange |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 1-20-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 7.4 capability exchange is empty. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 6.2.5: The device capability data object to carry the device capabilities - 7.4: How the device uses the capability data object to communicate its capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of discovery of the UE capabilities by the network services and other UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5 (new), 7.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 6.2.5 Device capabilities signalling

The device capabilities shall be signalled using the following data object.

| Name | Cardinality | Description |
| --- | --- | --- |
| deviceCapabilities | 0..1 | Device capabilities including the supported device profile and optionally the details capability |
| deviceType | 0..N | Device profile identifier. The profile identifiers are listed in Annex X. |
| deviceDetailedCapabilities | 0..1 | Device detailed capabilities, including the optional codecs and capabilities that are not defined in deviceType. |
| videoDecoding | 1 | Video decoding capabilities. |
| decoder | 0..N | List of supported decoders |
| id | 1 | name of decoder |
| concurrent_instances | 1 | number of concurrent instances |
| decoderInterface | 1 | Decoder interface capabilities |
| videoEncoding | 1 | Video encoding capabilities. |
| encoder | 1 | List of supported encoders |
| id | 1 | name of encoder |
| concurrent_instances | 1 | number of concurrent instances |
| audioDecoding | 0..N | Audio decoding capabilities. |
| decoder | 1 | List of supported decoders |
| id | 1 | name of decoder |
| concurrent_instances | 1 | number of concurrent instances |
| decoderInterface | 1 | Decoder interface capabilities |
| audioEncoding | 1 | Audio encoding capabilities. |
| encoder | 0..N | List of supported encoders |
| id | 1 | name of encoder |
| concurrent_instances | 1 | number of concurrent instances |
| xrRuntime | 0..1 | Capabilities of the device XR runtime |
| profile | 0..N | the supported profile URI |
| extensions | 0..N | Name of the supported extension |
| sceneManager | 0..1 | Capabilities of the device scene manager |
| profile | 0..N | the supported profile URI |

2nd Change

## 7.4	Capability exchange

The capabilities of the device is captured in the data structure defined in 6.2.5 and is communicated with the network.