3GPPSA4#127	S4-240185

# Sophia-Antipolis, 29 January 2024

Source:	NTT

Title:	[FS_eiRTCW] Pseudo-CR on Solution #5: API and solution evaluation for service control API

Spec:	3GPP TR 26.930

Agenda item:	10.9

Document for:	Agreement

1. Introduction

Regarding TR 26.930, service control procedures are proposed for the KI#5: Functional requirements for service control API. The procedures for Sol#5 are agreed during the official telco 3GPPSA4-e (AH) RTC SWG post 126.

2. Reason for Change

The possible APIs description that is applicable to the agreed service control procedures need to be added to TR 26.930.

3. Proposal

It is proposed to agree on the following changes to 3GPP TR 26.930.

* * * First Change * * * *

6.6.3	Service Control APIs

6.6.3.1	General aspects of service control API

6.6.3.1.1	Usage of HTTP

For service control API, ASWF and CP are expected to support HTTP/2 [XX].

6.6.3.1.2	Content type

The bodies of HTTP request and successful HTTP responses are required to be encoded in JSON format [XX]. The MIME media type that is required to be used within the related Content-Type header field is "application/json", unless described otherwise in the API definition.

When the "Problem Details" JSON object [XX] is used to indicate error details in an HTTP response, the Content-Type is set to "application/problem+json"

6.6.3.1.3	URI structure

6.6.3.1.3.1	Resource URI structure

API URIs for service control APIs are expected to be:

{apiRoot}/<apiVersion>/<apiName>/<apiSpecificResourceUriPart>/

"apiRoot" is configured following clause 4.4.1 of 3GPP TS 29.501 [XX]. "apiVersion" is dependent on APIs but expected to be "v1" in the first version. "apiName" is also dependent on APIs. "apiSpecificSuffixes" can be set depending on resource definitions of each API.

6.6.3.1.3.2	Custom operations URI structure

The custom operation is the operations that is non-idempotent and not a CREATE on a collection as described in Annex C of 3GPP TS 29.501 [XX].

The URI of a custom operation which is associated with a resource is required to have the following structure:

- {apiRoot}/<apiVersion>/<apiName>/<apiSpecificResourceUriPart>/<custOpName>

Custom operations can also be associated with the service instead of a resource. The URI of a custom operation which is not associated with a resource is required to have the following structure:

- {apiRoot}/<apiVersion>/<apiName>/<custOpName>

In the above URI structures, "apiRoot", "apiName", "apiVersion" and "apiSpecificResourceUriPart" are as defined in clause 6.6.3.1.3.1 and "custOpName" represents the name of the custom operation as defined in clause 5.1.3.2 of 3GPP TS 29.501 [XX].

6.6.3.1.4	Error handling

Response bodies for error handling are described in table 6.6.3.1.5-1. The HTTP client is expected to handle the described status codes and 'ProblemDetails' JSON (with the "application/problem+json" set in the Content-Type header) appropriately.

Table 6.6.3.1.4-1: Response bodies supported for responses to all requests

| Response body | Data type | Cardinality | Response Codes | Remarks | Applied Methods |
| --- | --- | --- | --- | --- | --- |
|  | ProblemDetails | 1 | 403 Forbidden | This represents the case when the server is able to understand the request but unable to fulfil the request due to errors. | GET, POST, PATCH, DELETE |
|  | ProblemDetails | 1 | 404 Not Found | The resource URI was incorrect. | GET, POST, PATCH, DELETE |
|  | ProblemDetails | 1 | 500 Internal Server Error | The server encountered an unexpected condition that prevented it from fulfilling the request. | GET, POST, PATCH, DELETE |
|  | ProblemDetails | 1 | 503 Service Unavailable | The server is unable to handle the request. | GET, POST, PATCH, DELETE |

6.6.3.1.5	HTTP headers

This API follows the HTTP Semantics [XX], and HTTP headers are set accordingly.

Table 6.6.3.1.5-1: HTTP headers used in service control API

| Name | Reference | Description |
| --- | --- | --- |
| Content-Type | IETF RFC 9110 [XX] | When setting JSON in the message body of an HTTP response, "application/json" is used and when setting Problem Details "application/problem+json" is used. If the message body is not set, this header is not used. |
| Location | IETF RFC 9110 [XX] | This header is used in 201 (Created) response upon successful RTC ID resource registration. The URI set in this header is used as a path parameter during RTC ID resource state retrieval, state modification, and deletion. |

6.6.3.2	RTC ID resource management API

6.6.3.2.1	API URI

The HTTP requests from CP's RTC ID resource manager have the structure of the request URI as indicated in clause 6.6.3.1.3. However, the following is applied for the base path for this operation:

- <apiVersion>: "v1"

- <apiName>: "rtc-service-control"

6.6.3.2.2	Resources

6.6.3.2.2.1	Overview

This clause describes the structure for the resource URIs and the resources and methods used for the service.

Figure 6.6.3.2.2.1-1 depicts the resource URIs structure for the RTC ID resource management API.

Figure 6.6.3.2.2.1-1: Resource URI structure of the CRUD of RTC ID resource and related configuration operation

Table 6.6.3.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.6.3.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| RTC ID resources | /rtcResourceIds | POST | Create an RTC ID resource |
| Individual RTC ID resource | /rtcResources/{rtcResourceId} | DELETE | Delete the RTC ID resource specified by {rtcResourceId} |
|  |  | PATCH | Update the status of the RTC ID resource specified by {rtcResourceId} |
|  |  | GET | Get the status of the RTC ID resource specified by {rtcResourceId} |

6.6.3.2.2.2	Resource: RTC ID resources

6.6.3.2.2.2.1	Description

RTC ID resources represents the all RTC ID resources registered to RTC ID resource enforcer.

6.6.3.2.2.2.2	Resource Definition

Resource URI : {apiRoot}/<apiVersion>/rtc-service-control/rtcResourceIds

This resource is required to support the resource URI variable defined in table 6.6.3.2.2.2.2-1.

Table 6.6.3.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.6.3.1.3 |

6.6.3.2.2.2.3	Resource Standard Methods

6.6.3.2.2.2.3.1	POST

This method is required to support the URI query parameters described in table 6.6.3.2.2.2.3.1-1.

Table 6.6.3.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.2.2.2.3.1-2 and the response data structure and response codes described in table 6.6.3.2.2.2.3.1-3.

Table 6.6.3.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcResourceRegReq | M | 1 | Information for RTC ID resource to register (e.g., RTC resource ID, callback URI) |

Table 6.6.3.2.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 201 Created | RTC ID resource in the corresponding request is successfully created. The URI of the created resource is required to be returned in the "Location" HTTP header. |

Table 6.6.3.2.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource. |

6.6.3.2.2.3	Resource: Individual RTC ID resource

6.6.3.2.2.3.1	Description

The individual RTC ID resource represents the individual RTC ID resource registered to RTC ID resource enforcer.

6.6.3.2.2.3.2	Resource Definition

Resource URI : {apiRoot}/<apiVersion>/rtc-service-control/rtcResourceIds/{rtcResourceId}

This resource is required to support the resource URI variables defined in table 6.6.3.2.2.3.2-1.

Table 6.6.3.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.6.3.1.3 |
| rtcResourceId | string | String identifying an individual RTC ID resource |

6.6.3.2.2.3.3	Resource Standard Methods

6.6.3.2.2.3.3.1	DELETE

This method is required to support the URI query parameters described in table 6.6.3.2.2.3.3.1-1.

Table 6.6.3.2.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.2.2.3.3.1-2 and the response data structure and response codes described in table 6.6.3.2.2.3.3.1-3.

Table 6.6.3.2.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.2.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 202 Accepted | RTC ID resource deletion is accepted. The completion of resource deletion is expected to be notified by callback operation described in clause 6.6.3.3. |

Table 6.6.3.2.2.3.3.1-4: Headers supported by the 202 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.2.2.3.3.2	PATCH

This method is required to support the URI query parameters described in table 6.6.3.2.2.3.3.2-1.

Table 6.6.3.2.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.2.2.3.3.2-2 and the response data structure and response codes described in table 6.6.3.2.2.3.3.2-3.

Table 6.6.3.2.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcResourceModReq | M | 1 | Update of RTC ID resource |

Table 6.6.3.2.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 200 OK | Update of RTC ID resource is successfully completed. |

Table 6.6.3.2.2.3.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.2.2.3.3.3	GET

This method is required to support the URI query parameters described in table 6.6.3.2.2.3.3.3-1.

Table 6.6.3.2.2.3.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.2.2.3.3.3-2 and the response data structure and response codes described in table 6.6.3.2.2.3.3.3-3.

Table 6.6.3.2.2.3.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.2.2.3.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcResourceStatRes | M | 1 | 200 OK | Requested RTC ID resource is successfully retrieved. |

Table 6.6.3.2.2.3.3.3-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.2.3	Data model

6.6.3.2.3.1		General

This clause describes the application data model supported by the API.

Table 6.6.3.2.3.1-1 depicts the data types defined specifically for the RTC ID resource management API service.

Table 6.6.3.2.3.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| rtcResourceRegReq | Clause 6.6.3.2.3.2.1 | RTC resource create request body |  |
| rtcResourceModReq | Clause 6.6.3.2.3.2.2 | RTC resource update request body |  |
| rtcResourceStatRes | Clause 6.6.3.2.3.2.3 | RTC resource read response body |  |
| callbackInformation | Clause 6.6.3.2.3.2.4 | Event subscription for callbacks |  |
| timeRange | Clause 6.6.3.2.3.2.5 | Time range |  |
| mediaRouting | Clause 6.6.3.2.3.2.6 | MDFC rule |  |
| rtcUserStatus | Clause 6.6.3.2.3.2.7 | Connected RTC user status |  |
| ueRoleElem | Clause 6.6.3.2.3.2.8 | UE Role information |  |
| accessControl | Clause 6.6.3.2.3.2.9 | Access control information |  |
| anonymityControl | Clause 6.6.3.2.3.2.10 | ID anonymity control information |  |
| mc | Clause 6.6.3.2.3.2.11 | Labels for media channel group |  |
| dc | Clause 6.6.3.2.3.2.12 | Labels for data channel group |  |
| mcGroupLabelElem | Clause 6.6.3.2.3.2.13 | Media channel group definition |  |
| mediaDesc | Clause 6.6.3.2.3.2.14 | Media description |  |
| codecElem | Clause 6.6.3.2.3.2.15 | Codec information |  |
| mediaAttributeElem | Clause 6.6.3.2.3.2.16 | Media attribute |  |
| extmapElem | Clause 6.6.3.2.3.2.17 | RTP header extension |  |
| setupControl | Clause 6.6.3.2.3.2.18 | Media negotiation control information |  |
| iniOffer | Clause 6.6.3.2.3.2.19 | Initial offer control information |  |
| iniAnswer | Clause 6.6.3.2.3.2.20 | Initial answer control information |  |
| subOffer | Clause 6.6.3.2.3.2.21 | Subsequent offer control information |  |
| subAnswer | Clause 6.6.3.2.3.2.22 | Subsequent answer control information |  |
| dcGroupLabelElem | Clause 6.6.3.2.3.2.23 | Data channel group definition |  |
| routingRuleElem | Clause 6.6.3.2.3.2.24 | Rule entry of media routing |  |
| routingElem | Clause 6.6.3.2.3.2.25 | Media routing pipeline element |  |

Table 6.6.3.2.3.1-2 describes data types re-used by the RTC ID resource management API service.

Table 6.6.3.2.3.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.2.3.2		Structured data types

6.6.3.2.3.2.1		Type: rtcResourceRegReq

Table 6.6.3.2.3.2.1-1: Definition of type rtcResourceRegReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rtcResourceId | string | M | 1 | RTC resource ID for the RTC ID resource to be created, based on the instruction by the RTC ID resource manager. |  |
| callback | callbackInformation | O | 0..1 | Callback information associated with the RTC ID resource. |  |
| validityPeriod | timeRange | O | 0..1 | The period where the RTC ID resource is available. |  |
| newConnectionEnabled | boolean | O | 0..1 | Set to true indicate new connection can be established. Otherwise set to false. |  |
| mediaRouting | mediaRouting | O | 0..1 | Configuration for media data forwarding control. If not set, all media and data are forwarded to all other endpoints. |  |

6.6.3.2.3.2.2		Type: rtcResourceModReq

Table 6.6.3.2.3.2.2-1: Definition of type rtcResourceModReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callback | callbackInformation | O | 0..1 | Callback information associated with the RTC ID resource. |  |
| validityPeriod | timeRange | O | 0..1 | The period where the RTC ID resource is available. |  |
| newConnectionEnabled | boolean | O | 0..1 | Set to true indicate new connection can be established. Otherwise set to false. |  |
| mediaRouting | mediaRouting | O | 0..1 | Configuration for MDFC. If not set, all media and data are forwarded to all other endpoints. |  |

6.6.3.2.3.2.3		Type: rtcResourceStatRes

Table 6.6.3.2.3.2.3-1: Definition of type rtcResourceStatRes

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callback | callbackInformation | O | 0..1 | Callback information associated with the RTC ID resource. |  |
| validityPeriod | timeRange | O | 0..1 | The period where the RTC ID resource is available. |  |
| newConnectionEnabled | boolean | O | 0..1 | Set to true indicate new connection to the RTC ID resource can be established. Otherwise set to false. |  |
| mediaRouting | mediaRouting | O | 0..1 | Configuration for MDFC. If not set, all media and data are forwarded to all other endpoints. |  |
| rtcUserStatusList | array(rtcUserStatus) | M | 0..N | Status list of connected RTC users. |  |

6.6.3.2.3.2.4		Type: callbackInformation

Table 6.6.3.2.3.2.4-1: Definition of type callbackInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callbackUri | string | M | 1 | Callback URI assigned to the RTC ID resource. |  |
| triggerList | array(string) | M | 0..N | Subscribed event triggers. Callback triggers are described in Table 6.6.3.2.3.2.4-2. |  |
| bundleDuration | uint32 | O | 0..1 | Set the duration in seconds for bundling callbacks. If set to zero in update request, the callback bundling is released. |  |

Table 6.6.3.2.3.2.4-2: Definition of callback trigger

| Callback trigger name | Description |
| --- | --- |
| "call.in.requested" | Callback is triggered when a new media session setup request to the related RTC ID resource is attempted. |
| "call.in.accepted" | Callback is triggered when a new media session setup request to the related RTC ID resource is accepted. |
| "call.in.connected" | Callback is triggered when a new media session setup request to the related RTC ID resource is connected and all media routings are completed. |
| "call.out.accepted" | Callback is triggered when a new instruction from CP that initiate call out session from the related RTC ID resource is accepted. |
| "call.out.connected" | Callback is triggered when a new instruction from CP that initiate call out session from the related RTC ID resource is completed. |
| "call.disconnected" | Callback is triggered when an existing media session is disconnected. |
| "call.media.requested" | Callback is triggered when operator network request CP's instruction regarding routing rules for added media. |
| "resource.deleted" | Callback is triggered when RTC ID resource is deleted in response to a resource delete request. |
| "resource.deletedForced" | Callback is triggered when RTC ID resource is deleted without a resource delete request. (e.g., network failure) |
| "resource.suspended" | Callback is triggered when RTC ID resource is temporarily unavailable. |
| "resource.resumed" | Callback is triggered when RTC ID resource is recovered from suspended state. |

6.6.3.2.3.2.5		Type: timeRange

Table 6.6.3.2.3.2.5-1: Definition of type timeRange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| startTime | string | M | 1 | String formatted time that indicates the absolute start time of the time range. |  |
| endTime | string | M | 1 | String formatted time that indicates the absolute end time of the time range. |  |

6.6.3.2.3.2.6		Type: mediaRouting

Table 6.6.3.2.3.2.6-1: Definition of type mediaRouting

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueRole | array(ueRoleElem) | M | 0..N | UE's role presets for MDFC. |  |
| mcGroupLabel | array(mcGroupLabelElem) | O | 0..N | Label definition for media channel group label. |  |
| dcGroupLabel | array(dsGroupLabelElem) | O | 0..N | Label definition for data channel group label. |  |
| routingRule | array(routingRuleElem) | O | 0..N | Routing rules for individual media and data. If not set, all media is forwarded to all other endpoints. |  |
| routingMode | string | O | 0..1 | If "callback" is set, all media routing is controlled by CP's instruction in callback. If "auto" is set, the operator network performs media routing control and callback for requesting CP's instruction will not be triggered. If not set, "auto" is applied. |  |

6.6.3.2.3.2.7		Type: rtcUserStatus

Table 6.6.3.2.3.2.7-1: Definition of type rtcUserStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rtcUserId | string | M | 1 | RTC User ID for UE or WebRTC Endpoint Function. |  |
| roleName | string | M | 1 | Role name associated with the RTC User ID. This role is related to the routing rules for media data forwarding control. |  |

6.6.3.2.3.2.8		Type: ueRoleElem

Table 6.6.3.2.3.2.8-1: Definition of type ueRole

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| roleName | string | M | 1 | Role name associated with UEs or WebRTC Endpoint Functions. |  |
| numMax | integer | O | 0..1 | Maximum number of UEs that have the role represented by "roleName" and are connected to the RTC ID resource. If not set, maximum number is unlimited. |  |
| accessControl | accessControl | O | 0..1 | Information for connection control. If not set, all connections are permitted unconditionally. |  |
| anonimityControl | anonimityControl | O | 0..1 | Information for anonymity control of connecting UE. |  |
| mc | array(mc) | O | 0..N | Information list of media channels related to the role. |  |
| dc | array(ds) | O | 0..N | Information list of data streams related to the role. |  |

6.6.3.2.3.2.9		Type: accessControl

Table 6.6.3.2.3.2.9-1: Definition of type accessControl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| allow | array(string) | O | 0..N | Originating RTC user ID list for UEs that are unconditionally permitted to connect. If set "#any", connection requests with/without any originating RTC user ID are permitted. If set "anonymous", connection requests without originating RTC user ID are permitted. |  |
| deny | array(string) | O | 0..N | Originating RTC user ID list for UEs that are unconditionally prohibited to connect. If set "#any", connection requests with/without any originating RTC user ID are prohibited. If set "anonymous", connection requests without originating RTC user ID are prohibited. |  |
| callback | array(string) | O | 0..N | Originating RTC user ID list for UEs that are dynamically permitted to connect based on CP's instruction with "call.in.requested" callback. "#any" and "anonymous" are treated as in the case of "allow". |  |

6.6.3.2.3.2.10		Type: anonymityControl

Table 6.6.3.2.3.2.10-1: Definition of type anonymityControl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oIdAllow | array(string) | O | 0..N | Originating RTC user ID list for UEs that unconditionally disclose their ID in signalling and API. If set "#any", connection requests with any originating RTC user ID is disclosed. |  |
| oIdDeny | array(string) | O | 0..N | Originating RTC user ID list for UEs that unconditionally conceal their ID in signalling and API. If set "#any", connection requests with any originating RTC user ID is concealed. |  |
| oIdCallback | array(string) | O | 0..N | Originating RTC user ID list for UEs that dynamically disclose their ID in signalling and API based on CP's instruction with "call.in.requested" callback. "#any" are treated as in the case of "oIdAllow". |  |

6.6.3.2.3.2.11		Type: mc

Table 6.6.3.2.3.2.11-1: Definition of type mc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mcGroupLabel | string | M | 1 | Label name specifying the media channel group to which the UE's media belongs. |  |

6.6.3.2.3.2.12		Type: dc

Table 6.6.3.2.3.2.12-1: Definition of type dc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dcGroupLabel | string | M | 1 | Label name specifying the data channel group to which the UE's data channel belongs. |  |

6.6.3.2.3.2.13		Type: mcGroupLabelElem

Table 6.6.3.2.3.2.13-1: Definition of type mcGroupLabelElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mcGroupName | string | M | 1 | Name of media channel group. It is required to be unique in a specific RTC ID resource. |  |
| numMax | uint32 | O | 0..1 | Maximum number of media channels that have the label indicated in mc. If not set, the number is unlimited |  |
| numMinPerUe | uint32 | O | 0..1 | Minimum number of media channels that have the label indicated in mc and belong to a specific UE. If not set, zero is applied. |  |
| numMaxPerUe | uint32 | O | 0..1 | Maximum number of media channels that have the label indicated in mc and belong to a specific UE. If not set, the number is unlimited. |  |
| mediaDesc | mediaDesc | M | 1 | Media description for the media channel group. |  |
| setupControl | setupControl | M | 1 | Information for media setup control. |  |

6.6.3.2.3.2.14		Type: mediaDesc

Table 6.6.3.2.3.2.14-1: Definition of type mediaDesc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| media | string | M | 1 | Media type. "audio" or "video" is set. |  |
| codec | array(codecElem) | M | 1..N | Information for supported codec for this media channel. |  |
| mediaAttribute | array(mediaAttributeElem) | O | 0..N | Information for media attribute. |  |
| direction | string | M | 1 | Media direction. If "upstream" is set, the media is uni-directional from network to UE (recvonly in network's viewpoint). If "downstream" is set, the media is uni-directional from UE to network (sendonly in network's viewpoint). If "bidirectional" is set, the media is bi-directional (equal to sendrecv). If "inactive" is set, media forwarding is temporarily unavailable. |  |

6.6.3.2.3.2.15		Type: codecElem

Table 6.6.3.2.3.2.15-1: Definition of type codecElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| encodingName | string | M | 1 | String value following "encoding-name" specified in IETF RFC 8866 [XX]. If any codec is allowed, only one codecElem is set and encodingName is required to be "#any". If specific codecs are set, network's initial offer include all listed codecs and. When UE send offer, codecs set in this value are allowed. |  |
| clockRate | uint32 | O | 0..1 | String value following "clock-rate" specified in IETF RFC 8866 [XX]. |  |
| encodingParams | uint32 | O | 0..1 | String value following "encoding-params" specified in IETF RFC 8866 [XX]. If specific value is set to this key, clockRate is required to be set specific value. An offer from network has this encoding-params and UE's offer with different encoding-params will be denied. If this key is not set, network's offer has no encoding-params and UE's offer with encoding-params will be denied. |  |
| formatSpecificValue | string | O | 0..1 | String value following "format-specific-value" specified in IETF RFC 8866 [XX]. An offer from network has a=fmtp line based on this value. If this key is not set, network's offer does not have a=fmtp line. Regardless of this key, a=fmtp line in offers from UE are not checked. |  |

6.6.3.2.3.2.16		Type: mediaAttributeElem

Table 6.6.3.2.3.2.16-1: Definition of type mediaAttributeElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ptime | uint32 | O | 0..1 | Number value following "ptime" specified in IETF RFC 8866 [XX]. This key is set only if media type is set "audio". An offer from UE that has different ptime value will be denied. An offer from network has a=ptime line based on this value. If this key is not set, a=ptime line in UE's offer will be not checked and derived a=ptime value will be used in network's offer. |  |
| extmap | array(extmapElem) | O | 0..N | Information for RTP extension. |  |

6.6.3.2.3.2.17		Type: extmapElem

Table 6.6.3.2.3.2.17-1: Definition of type extmapElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extensionName | string | M | 1 | String value following "extensionname" specified in IETF RFC 8285 [XX]. If any RTP extension headers are allowed, only one extmapElem is set to "extmap" and extensionName is required to be set "#any". An offer from network has a=rtpmap line based on this value. An offer from UE that has a=rtpmap line that is not included in "extmap" will be denied. If this key is not set, an offer from network has no a=rtpmap line. |  |

6.6.3.2.3.2.18		Type: setupControl

Table 6.6.3.2.3.2.18-1: Definition of type setupControl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| iniOffer | iniOffer | O | 0..1 | Information for negotiation control for initial offer. |  |
| iniAnswer | iniAnswer | O | 0..1 | Information for negotiation control for initial answer. |  |
| subOffer | subOffer | O | 0..1 | Information for negotiation control for subsequent offer. |  |
| subAnswer | subAnswer | O | 0..1 | Information for negotiation control for subsequent answer. |  |
| NOTE: If each attribute is not set, default rule is applied. |  |  |  |  |  |

6.6.3.2.3.2.19		Type: iniOffer

Table 6.6.3.2.3.2.19-1: Definition of type iniOffer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| direction | string | O | 0..1 | If "network" is set, initial offer is always sent from network. If "ue" is set initial offer is always sent from UE. If "both" is set, initial offer can be sent from UE or network. If not set, "network" is applied. |  |
| onMsetupReq | string | O | 0..1 | If "required" is set, initial offer is required to be set in msetup request. If "optional" is set, initial offer can be set in msetup request. If "none" is set, initial offer is not allowed to be set in msetup request. If not set, "optional" is applied. |  |
| oaTimeout | uint32 | O | 0..1 | Limited time in seconds for receiving the initial answer in response to the initial offer until timeout. If zero is set, the time is unlimited. If a timeout occurs, the media session will be disconnected. If not set, 180 is applied. |  |

6.6.3.2.3.2.20		Type: iniAnswer

Table 6.6.3.2.3.2.20-1: Definition of type iniAnswer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| onMsetupRes | string | O | 0..1 | If "required" is set, initial answer is required to be set in msetup response. If "optional" is set, initial answer can be set in msetup response. If "none" is set, initial answer is not allowed to be set in msetup response. If not set, "optional" is applied. |  |
| oaTimeout | uint32 | O | 0..1 | Limited time in seconds for receiving the initial answer in response to the initial offer until timeout. If zero is set, the time is unlimited. If a timeout occurs, the media session will be disconnected. If not set, 180 is applied. |  |

6.6.3.2.3.2.21		Type: subOffer

Table 6.6.3.2.3.2.21-1: Definition of type subOffer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| direction | string | O | 0..1 | If "network" is set, subsequent offer is only sent from network. If "ue" is set subsequent offer is only sent from UE. If "both" is set, subsequent offer can be sent from UE or network. If "none" is set, subsequent offer is not allowed. If not set, "network" is applied. |  |
| oaTimeout | uint32 | O | 0..1 | Limited time in seconds for receiving the subsequent answer in response to the subsequent offer until timeout. If zero is set, the time is unlimited. If a timeout occurs, the media session will be disconnected. If not set, 180 is applied. |  |

6.6.3.2.3.2.22		Type: subAnswer

Table 6.6.3.2.3.2.22-1: Definition of type subAnswer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oaTimeout | uint32 | O | 0..1 | Limited time in seconds for receiving the subsequent answer in response to the subsequent offer until timeout. If zero is set, the time is unlimited. If a timeout occurs, the media session will be disconnected. If not set, 180 is applied. |  |

6.6.3.2.3.2.23		Type: dcGroupLabelElem

Table 6.6.3.2.3.2.23-1: Definition of type dcGroupLabelElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dsGroupName | string | M | 1 | Name of data channel group. It is required to be unique in a specific RTC ID resource. |  |
| numMax | uint32 | O | 0..1 | Maximum number of data channels that have the label indicated in ds. If not set, the number is unlimited |  |
| numMinPerUe | uint32 | O | 0..1 | Minimum number of data channels that have the label indicated in ds and belong to a specific UE. If not set, zero is applied. |  |
| numMaxPerUe | uint32 | O | 0..1 | Maximum number of data channels that have the label indicated in ds and belong to a specific UE. If not set, the number is unlimited. |  |
| subprotocol | string | O | 0..1 | String value following "subprotocol" parameter specified in IETF RFC 8864 [XX]. |  |
| ordered | boolean | O | 0..1 | String value following "ordered" parameter specified in IETF RFC 8864 [XX]. If not set, "true" is applied. |  |
| maxretr | uint32 | O | 0..1 | String value following "max-retr" parameter specified in IETF RFC 8864 [XX]. If not set, reliable transmission is performed. |  |
| maxtime | uint32 | O | 0..1 | String value following "max-time" parameter specified in IETF RFC 8864 [XX]. If not set, reliable transmission is performed. |  |
| priority | uint16 | O | 0..1 | String value following "priority" parameter specified in IETF RFC 8864 [XX]. If not set, 256 is applied. |  |

6.6.3.2.3.2.24		Type: routingRuleElem

Table 6.6.3.2.3.2.24-1: Definition of type routingRuleElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| routingName | string | M | 1 | Name of the routing rule. |  |
| routing | array(routingElem) | M | 2..N | Description of media/data routing pipeline. First element indicates input stage. Last element indicates output stage. Other middle element(s) indicate several media/data processes. |  |

6.6.3.2.3.2.25		Type: routingElem

Table 6.6.3.2.3.2.25-1: Definition of type routingElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| element | array(string) | M | 1..N | Elements of media/data pipeline. If the routingElem is first element, media/data group label, routingName of another routingRuleElem or processer name is set. If the routingElem is last element, media/data group label or routingName of another routingRuleElem is set. If the routingElem is not first or last one, only processor name can be set. |  |
| NOTE: Processor names are listed in Table 6.6.3.2.3.2.y-2. |  |  |  |  |  |

Table 6.6.3.2.3.2.25-2: Definition of processor

| Processor name | Description |
| --- | --- |
| std.mc.dup | Media process performs media duplication. Single input media (e.g., audio, video) is duplicated and sent to multiple output. Previous stage (media group or routingRule) is required to have only one output media. If the next stage is routingRule, only one media channel is output. If the next stage is group label, one media channel is output to each UE. |
| std.mc.cast | Media process performs media duplication in large scale (e.g., more than hundreds output). (NOTE 1) |
| std.mc.forward | Media process performs forwarding. Input media are duplicated and forwarded to outputs. Previous stage can be one or more group labels or routingRules with multiple media channels. If "loopback=true" parameter is set, input media is also sent to source UE. If parameter is not set, "loopback=false" is applied. |
| std.mc.audio.mix | Media process performs audio mixing. Multiple input audio are processed and output single mixed audio. |
| std.dc.dup | Data stream duplication. The rule for input and output follows "std.mc.dup". |
| std.dc.forward | Data stream forwarding. The rule for input and output follows "std.mc.forward". |
| std.dc.mix | Data stream mixing (e.g., data channel stream multiplexed with label). The rule for input and output follows "std.mc.audio.mix". |
| NOTE 1: In cases, large scale media duplication has different internal implementation from ordinary one (e.g., multi-stage duplication). Therefore, "std.mc.dup" and "std.md.cast" are listed as different processor. NOTE 2: Processors are expected to have internal attribute (e.g., allowed input/output group label, expected number of media/data) |  |

6.6.3.2.3.3		Simple data types and enumerations

6.6.3.2.3.3.1	Simple data types

The simple data types defined in table 6.6.3.2.3.2.1-1 shall be supported.

Table 6.6.3.2.3.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.2.4	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.3	Notification of RTC ID resource deletion

6.6.3.3.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's RTC ID resource enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.3.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.3.3	Notification operation

6.6.3.3.3.1	Description

The Callback operation is used by the RTC ID resource enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that RTC ID resource deletion, requested by CP, has been completed.

6.6.3.3.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.3.3.2-1.

Table 6.6.3.3.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.3.3.2-2 and the response data structure and response codes described in table 6.6.3.3.3.2-3.

Table 6.6.3.3.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of deleted RTC ID resource |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.3.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | Completion of RTC ID resource deletion is acknowledged. |

Table 6.6.3.3.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.3.4	Data Model

6.6.3.3.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.3.4.1-1 depicts the data types used for Notification of RTC ID resource deletion API. Those data types are shared among following callback operations. As described in the "Applicability", optional keys are not used in this API.

Table 6.6.3.3.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Callback request body |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Callback response body |  |
| mediaControlElem | Clause 6.6.3.3.4.2.3 | Information for media control |  |

Table 6.6.3.3.4.1-2 describes data types re-used by the API service.

Table 6.6.3.3.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| mediaInfo | Clause 6.4.5.5.4.3.19 | Re-used in rtcCallbackReq and mediaControlElem object. |  |

6.6.3.3.4.2	Structured data types

6.6.3.3.4.2.1	Type: rtcCallbackReq

Table 6.6.3.3.4.2.1-1: Definition of type rtcCallbackReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trigger | string | M | 1 | Callback trigger described in Table 6.6.3.2.3.2.4-2. |  |
| rtcId | string | M | 1 | The RTC resource ID that triggered the event for the callback request. |  |
| seqId | uint32 | M | 1 | The event sequence number initiated by the corresponding RTC resource ID, starting from zero. |  |
| oId | string | O | 0..1 | Originating ID of a new media session setup attempt. RTC user ID or RTC resource ID is expected to be set. | "call.in.requested" "call.in.accepted" "call.in.connected" |
| mediaInfo | mediaInfo | O | 0..1 | MediaInfo object included in msetup request. | "call.in.requested" "call.in.accepted" "call.in.connected" |
| participantId | string | O | 0..1 | Participant ID assigned by WSF to UE. | "call.in.accepted" "call.in.connected" "call.disconnected" |
| mediaControl | array(mediaControlElem) | O | 0..N | Information for mediaControl. | "call.media.requested" |
| cleared | boolean | O | 0..1 | Set to true if UE connection status is cleared. | "resource.resumed" |

6.6.3.3.4.2.2	Type: rtcCallbackRes

Table 6.6.3.3.4.2.2-1: Definition of type rtcCallbackRes

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| success | boolean | M | 1 | Set to true if corresponding request is successfully processed. |  |
| ueRole | string | O | 0..1 | Indicate UE's role based on media routing properties described in clause 6.6.3.2.3.2.6. | "call.in.requested" "call.in.accepted" "call.in.connected" |
| mediaControl | array(mediaControlElem) | O | 0..N | Information for media control. | "call.in.accepted" "call.in.connected" "call.disconnected" "call.media.requested" |

6.6.3.3.4.2.3	Type: mediaControlElem

Table 6.6.3.3.4.2.3-1: Definition of type mediaControlElem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| participantId | string | M | 1 | Participant ID assigned by WSF to UE. |  |
| mediaInfo | mediaInfo | M | 1 | MediaInfo object included in msetup request of the UE. |  |

6.6.3.3.4.3	Simple data types and enumerations

6.6.3.3.4.3.1	Simple data types

Table 6.6.3.3.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.3.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.4	Notification of forced RTC ID resource deletion

6.6.3.4.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's RTC ID resource enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.4.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.4.3	Notification operation

6.6.3.4.3.1	Description

The Callback operation is used by the RTC ID resource enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that RTC ID resource has been deleted independently of the CP's request.

6.6.3.4.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.4.3.2-1.

Table 6.6.3.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.4.3.2-2 and the response data structure and response codes described in table 6.6.3.4.3.2-3.

Table 6.6.3.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of deleted RTC ID resource |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | RTC ID resource deletion is acknowledged. |

Table 6.6.3.4.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.4.4	Data Model

6.6.3.4.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.4.4.1-1 depicts the data types specifically used for Notification of forced RTC ID resource deletion API.

Table 6.6.3.4.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.4.4.1-2 describes data types re-used by the API service.

Table 6.6.3.4.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. Optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. Optional keys are not used. |  |

6.6.3.4.4.2	Structured data types

None.

6.6.3.4.4.3	Simple data types and enumerations

6.6.3.4.4.3.1	Simple data types

Table 6.6.3.4.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.4.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.5	Notification of RTC ID resource suspended

6.6.3.5.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's RTC ID resource enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.5.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.5.3	Notification operation

6.6.3.5.3.1	Description

The Callback operation is used by the RTC ID resource enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that RTC ID resource has been temporarily unavailable.

6.6.3.5.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.5.3.2-1.

Table 6.6.3.5.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.5.3.2-2 and the response data structure and response codes described in table 6.6.3.5.3.2-3.

Table 6.6.3.5.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of suspended RTC ID resource |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.5.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | The state transition of RTC ID resource to the suspended state is acknowledged. |

Table 6.6.3.5.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.5.4	Data Model

6.6.3.5.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.5.4.1-1 depicts the data types specifically used for Notification of RTC ID resource suspended API.

Table 6.6.3.5.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.5.4.1-2 describes data types re-used by the API service.

Table 6.6.3.5.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. Optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. Optional keys are not used. |  |

6.6.3.5.4.2	Structured data types

None.

6.6.3.5.4.3	Simple data types and enumerations

6.6.3.5.4.3.1	Simple data types

Table 6.6.3.5.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.5.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.6	Notification of RTC ID resource resumed

6.6.3.6.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's RTC ID resource enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.6.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.6.3	Notification operation

6.6.3.6.3.1	Description

The Callback operation is used by the RTC ID resource enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that RTC ID resource has been recovered from suspended state.

6.6.3.6.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.6.3.2-1.

Table 6.6.3.6.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.6.3.2-2 and the response data structure and response codes described in table 6.6.3.6.3.2-3.

Table 6.6.3.6.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of resumed RTC ID resource |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.6.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | The state transition of RTC ID resource to the resumed is acknowledged. |

Table 6.6.3.6.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.6.4	Data Model

6.6.3.6.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.6.4.1-1 depicts the data types specifically used for Notification of RTC ID resource resumed API.

Table 6.6.3.6.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.6.4.1-2 describes data types re-used by the API service.

Table 6.6.3.6.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. "cleared" key can be set. Other optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. Optional keys are not used. |  |

6.6.3.6.4.2	Structured data types

None.

6.6.3.6.4.3	Simple data types and enumerations

6.6.3.6.4.3.1	Simple data types

Table 6.6.3.6.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.6.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.7	Notification of user call in requested

6.6.3.7.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's connection control enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.7.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.7.3	Notification operation

6.6.3.7.3.1	Description

The Callback operation is used by the connection control enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that a new attempt of media session setup has been requested by the UE.

6.6.3.7.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.7.3.2-1.

Table 6.6.3.7.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.7.3.2-2 and the response data structure and response codes described in table 6.6.3.7.3.2-3.

Table 6.6.3.7.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of the new attempt of media session setup by UE |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.7.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | The new attempt of media session setup by UE is acknowledged and CP provides the instruction including connection control and role assignment. |

Table 6.6.3.7.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.7.4	Data Model

6.6.3.7.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.7.4.1-1 depicts the data types specifically used for Notification of user call in requested API.

Table 6.6.3.7.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.7.4.1-2 describes data types re-used by the API service.

Table 6.6.3.7.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| mediaInfo | Clause 6.4.5.5.4.3.19 | Re-used in rtcCallbackReq. |  |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. Keys "oId" and "mediaInfo" are required to be set. Other optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. "ueRole" key can be set. Other optional keys are not used. |  |

6.6.3.7.4.2	Structured data types

None.

6.6.3.7.4.3	Simple data types and enumerations

6.6.3.7.4.3.1	Simple data types

Table 6.6.3.7.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.7.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.8	Notification of user call in accepted

6.6.3.8.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's connection control enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.8.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.8.3	Notification operation

6.6.3.8.3.1	Description

The Callback operation is used by the connection control enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that a new attempt of media session setup of UE has been accepted by the operator network.

6.6.3.8.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.8.3.2-1.

Table 6.6.3.8.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.8.3.2-2 and the response data structure and response codes described in table 6.6.3.8.3.2-3.

Table 6.6.3.8.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of the accepted attempt of media session setup by UE |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.8.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | The accept of media session setup by UE is acknowledged and CP provides the instruction including connection control and role assignment. |

Table 6.6.3.8.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.8.4	Data Model

6.6.3.8.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.8.4.1-1 depicts the data types specifically used for Notification of user call in accepted API.

Table 6.6.3.8.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.8.4.1-2 describes data types re-used by the API service.

Table 6.6.3.8.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| mediaInfo | Clause 6.4.5.5.4.3.19 | Re-used in rtcCallbackReq. |  |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. Keys "oId", "mediaInfo" and "participantId" are required to be set. Other optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. "ueRole" and "mediaControl" key can be set. |  |
| mediaControlElem | Clause 6.6.3.3.4.2.3 | Re-used in rtcCallbackRes. |  |

6.6.3.8.4.2	Structured data types

None.

6.6.3.8.4.3	Simple data types and enumerations

6.6.3.8.4.3.1	Simple data types

Table 6.6.3.8.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.8.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.9	Notification of user call in connected

6.6.3.9.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's connection control enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.9.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.9.3	Notification operation

6.6.3.9.3.1	Description

The Callback operation is used by the connection control enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that a new media session has been established and media have been connected.

6.6.3.9.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.9.3.2-1.

Table 6.6.3.9.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.9.3.2-2 and the response data structure and response codes described in table 6.6.3.9.3.2-3.

Table 6.6.3.9.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of the established media session |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.9.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | The connected media session is acknowledged and CP provides the instruction including role assignment and media routing control. |

Table 6.6.3.9.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.9.4	Data Model

6.6.3.9.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.9.4.1-1 depicts the data types specifically used for Notification of user call in connected API.

Table 6.6.3.9.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.9.4.1-2 describes data types re-used by the API service.

Table 6.6.3.9.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| mediaInfo | Clause 6.4.5.5.4.3.19 | Re-used in rtcCallbackReq. |  |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. Keys "oId", "mediaInfo" and "participantId" are required to be set. Other optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. "ueRole" and "mediaControl" key can be set. |  |
| mediaControlElem | Clause 6.6.3.3.4.2.3 | Re-used in rtcCallbackRes. |  |

6.6.3.9.4.2	Structured data types

None.

6.6.3.9.4.3	Simple data types and enumerations

6.6.3.9.4.3.1	Simple data types

Table 6.6.3.9.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.9.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.10	Notification of user call in connected

6.6.3.10.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's connection control enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.10.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.10.3	Notification operation

6.6.3.10.3.1	Description

The Callback operation is used by the connection control enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that an established media session has been disconnected.

6.6.3.10.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.10.3.2-1.

Table 6.6.3.10.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.10.3.2-2 and the response data structure and response codes described in table 6.6.3.10.3.2-3.

Table 6.6.3.10.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of the disconnected media session |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.10.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | The media session disconnection is acknowledged and CP provides the instruction for media routing control. |

Table 6.6.3.10.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.10.4	Data Model

6.6.3.10.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.10.4.1-1 depicts the data types specifically used for Notification of user call disconnected API.

Table 6.6.3.10.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.10.4.1-2 describes data types re-used by the API service.

Table 6.6.3.10.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| mediaInfo | Clause 6.4.5.5.4.3.19 | Re-used in mediaControlElem. |  |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. "participantId" is required to be set. Other optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. "mediaControl" key can be set. |  |
| mediaControlElem | Clause 6.6.3.3.4.2.3 | Re-used in rtcCallbackRes. |  |

6.6.3.10.4.2	Structured data types

None.

6.6.3.10.4.3	Simple data types and enumerations

6.6.3.10.4.3.1	Simple data types

Table 6.6.3.10.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.10.5	Error Handling

General error responses are defined in clause 6.6.3.1.5.

6.6.3.11	Notification of media routing query

6.6.3.11.1	API URI

The operation is a notification operation for subscribed events, and HTTP requests from the OP's MDFC enforcer are made to the Callback URI set in the RTC ID resource management operations in clause 6.6.3.2.

6.6.3.11.2	Resources

The operation is a notification operation for subscribed events, and CP does not have structured resources.

6.6.3.11.3	Notification operation

6.6.3.11.3.1	Description

The Callback operation is used by the MDFC enforcer to notify the CP, which has subscribed to events related to registered RTC ID resources, about the occurrence of events. This operation is used to notify events that media are added or removed in the existing media session.

6.6.3.11.3.2	Notification operation definition

The POST method is required to be used for event notification and the URI is to be the one set in the RTC ID resource registration.

Callback URI: {callbackURI}

This method is required to support the URI query parameters described in table 6.6.3.11.3.2-1.

Table 6.6.3.11.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method is required to support the request data structures described in table 6.6.3.11.3.2-2 and the response data structure and response codes described in table 6.6.3.11.3.2-3.

Table 6.6.3.11.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| rtcCallbackReq | M | 1..N | Callback request including the information of added or removed media |
| NOTE: Multiple callback operations can be sent simultaneously and bundled in a request. |  |  |  |

Table 6.6.3.11.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| rtcCallbackRes | M | 1 | 200 OK | Addition or removal of media is acknowledged and CP provides the instruction of media routing control. |

Table 6.6.3.11.3.2-4: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

6.6.3.11.4	Data Model

6.6.3.11.4.1	General

This clause describes the application data model supported by the API.

Table 6.6.3.11.4.1-1 depicts the data types specifically used for Notification of media routing query API.

Table 6.6.3.11.4.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.6.3.11.4.1-2 describes data types re-used by the API service.

Table 6.6.3.11.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| mediaInfo | Clause 6.4.5.5.4.3.19 | Re-used in mediaControlElem. |  |
| rtcCallbackReq | Clause 6.6.3.3.4.2.1 | Re-used for rtcCallbackReq. "mediaControl" key is required to be set. Other optional keys are not used. |  |
| rtcCallbackRes | Clause 6.6.3.3.4.2.2 | Re-used for rtcCallbackRes. "mediaControl" key is required to be set. |  |
| mediaControlElem | Clause 6.6.3.3.4.2.3 | Re-used in rtcCallback Req and rtcCallbackRes. |  |

6.6.3.11.4.2	Structured data types

None.

6.6.3.11.4.3	Simple data types and enumerations

6.6.3.11.4.3.1	Simple data types

Table 6.6.3.11.4.3.1-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.6.3.11.5	Error Handling

General error responses are defined in clause 6.6.3.1.5

6.6.4	Solution evaluation

The proposed service control procedures and APIs fulfil the purpose that CP's service logic managers instruct operator's service logic enforcers to achieve the co-operated services (with flexible resource management and elaborate media routing control) described in corresponding key issue. Then, it is proposed to implement the procedures and APIs in the stage 3 specification of RTC on the basis of the proposals above.

* * * End of Changes * * * *