3GPP TSG-SA WG4 Meeting #127	    S4-240298

Sophia-Antipolis, France, 29 January - 2 February 2024 	revision of S4-240229

Source:	Fraunhofer IIS

Title:	Updated Proposal for IVAS Characterization Tests

Document for:	Agreement

Agenda Item:	7.5

Changes compared to S4-240229:

- Add proposed test configurations in Annex A (MUSHRA), B (P.SUPPL800)

- Proposed to change P.SUPPL800 test on ISM from 3-4 ISM  1-4 ISM; see also Annex ‎B.4)

Introduction

A proposal on the test plan for IVAS characterization testing was done by the source in [1] and subjective test results were presented in [2]. The source would like to address the feedback received during the Audio SWG calls and present a revised proposal for the characterization testing.

In the view of the source, the characterization phase has to offer a significant additional value compared to the IVAS selection tests to demonstrate the performance of the new codec, especially against solutions currently used in the field. By purely repeating conditions from the selection tests just with a different code-base or by pure comparison of floating- against fixed-point code-bases, this is not considered to provide attractive additional information which help to deploy the codec in the field.

To overcome these shortcomings, it is again proposed for the characterization tests to provide additional information, to especially answer the following questions:

- How does IVAS compare against current service quality, i.e. state-of-the-art monaural codecs (EVS)?

- How is the rate-distortion curve of IVAS, and how does it possibly interact with state-of-the-art monaural codecs (EVS)?

During the Audio SWG calls, concerns were raised, whether the proposed tests can show such results with the underlying P.SUPPL800 test methodology. The source takes these concerns serious and contributes an updated proposal, where higher emphasis is put on BS.1534 (MUSHRA) testing.

Compared to P.SUPPL800, the MUSHRA test methodology has a number of advantages:

- In MUSHRA, the subject can switch at will between the reference signal and any of the systems under test, allowing for a more precise evaluation of spatial and other impairments.

- MUSHRA recommends experienced listeners, which are typically more skilled judging different types of degradations, e.g. degradations in spatial or signal domain

The updated proposal consists of two parts:

- A larger series of low- and high-rate MUSHRA tests

- A smaller series of P.SUPPL800 tests

By means of the MUSHRA tests, a reliable comparison of IVAS against few EVS operation points will be feasible. The P.SUPPL800 tests back up this data with additional EVS and IVAS operation points and include a comparison of IVAS floating- and fixed-point code bases. By include both test methodologies in the IVAS characterization tests, also an assessment of these test methodologies for immersive audio content will be feasible.

According to [3], at least the following test capacity seems to be available:

19 x P.SUPPL800

30 x BS.1534, up to 12 with LS rendering

1 x ACR

1 x room acoustics testing

Updated Test Proposal

MUSHRA

The source proposes to set up the MUSHRA tests as follows:

- 12x high bitrate MUSHRA tests, allowing for an extensive evaluation of most supported IVAS formats

- 6x low bitrate MUSHRA tests, allowing for an additional evaluation of commonly used IVAS formats

It is proposed, to use the IVAS fixed-point code for the proposed MUSHRA tests. In case the fixed-point code is not available in time, also floating-point code could be used.

### High Bitrate MUSHRA Tests

It is proposed to set up the high bitrate MUSHRA tests using the following conditions:

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | LP 3.5 kHz | - | - |
| c03 | EVS | 1x 64.0 | Off |
| c04 | EVS | 1x128.0 | Off |
| c05 | IVAS | 64.0 | Off |
| c06 | IVAS | 96.0 | Off |
| c07 | IVAS | 128.0 | Off |
| c08 | IVAS | 256.0 | Off |

The tests are foreseen for the following formats:

- Stereo (see Anex ‎A.2)

- FOA (see Annex ‎A.4)

- HOA3 (se Annex ‎A.5)

- 5.1, 7.1 (both formats in same test, binaural rendering) (see Annex ‎A.7)

- 5.1+2, 5.1+4 (both formats in same test, binaural rendering) (see Annex ‎A.8)

- 7.1+4 (see Annex ‎A.9)

- ISM (1-2 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering) (see Annex ‎A.10)

- ISM (3-4 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering) (see Annex ‎A.12)

- MASA 1TC (see Annex ‎A.14)

- MASA 2TC (see Annex ‎A.15)

- OSBA (1-4 objects in same test) (see Annex ‎A.17)

- OMASA (1-4 objects in same test) (see Annex ‎A.18)

### Low Bitrate MUSHRA Tests:

It is proposed to set up the low bitrate MUSHRA tests using the following conditions:

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | LP 3.5 kHz | - | - |
| c03 | EVS | 1x 16.4 | Off |
| c04 | EVS | 1x 32.0 | Off |
| c05 | IVAS | 16.4 | Off |
| c06 | IVAS | 24.4 | Off |
| c07 | IVAS | 32.0 | Off |
| c08 | IVAS | 48.0 | Off |

The tests are foreseen for the following formats:

- Stereo (see ‎Annex A.1)

- FOA (see Annex ‎A.3)

- 5.1 (see Annex ‎A.6)

- ISM (3-4 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering); test starting at 24.4 kbit/s, possibly adding 64 kbit/s (see Annex ‎A.11)

- MASA 1TC (see Annex ‎A.13)

- OSBA (1-4 objects in same test) (see Annex ‎A.16)

P.800

Additionally, the source proposes to complement the MUSHRA tests by six P.SUPPL800 tests. It is proposed to set up the tests using the following conditions (nearly identical to the proposal in [1]):

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx |  |  |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

The tests are foreseen for the following formats:

- Stereo (see Annex ‎B.1)

- FOA (see Annex ‎B.2)

- 5.1 (see Annex ‎B.3)

- ISM (1-4 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering) (see Annex ‎B.4)

- MASA 1TC (see Annex ‎B.5)

- OSBA (1-4 objects in same test) (see Annex ‎B.6)

The general assumptions for these tests are:

- The internal IVAS renderer is used, rendering to the playout configuration; no head rotation is to be used for these tests.

- P.SUPPL800 tests are carried out via headphones; for these, the decoder output format is set to binaural

- The mono conditions are generated using the IVAS renderer as a preprocessing stage.

- It is proposed to use mixed material for both, P.SUPPL800 and MUSHRA, i.e. a combination of clean-speech, noisy-speech and mixed/music.

As can be seen, the proposed formats are identical as for the low bitrate MUSHRA tests incl. an overlap in operation points, allowing also for an assessment of both test methodologies.

Conclusion

This contribution provides an updated proposal for the characterization tests. It is proposed, to carry out:

- 18 MUSHRA tests and

- 6 P.SUPPL800 tests

out of all available tests for characterization as proposed above. The remaining available test capacity could be filled with other tests.

It is proposed to agree on this contribution, and include the tests as proposed in 2) in IVAS-8b.

References:

[1] S4aA230124 - Proposal for IVAS Characterization Tests

[2] S4aA230137 - IVAS Characterization Tests: Experimental Results for Proposed Tests

[3] S4aA230127 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, Version v.0.1.2

Proposed MUSHRA Test Configurations for Inclusion in IVAS-8b

Experiment BS1534-1a: Stereo

Conditions (BS1534-1a Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with stereo audio input at 16.4 kbps, 24.4 kbps, 32 kbps, 48 kbps DTX off at 0% FER |
| Codec references |  |
| Codec references | Mono EVS 16.4 kbps, 32 kbps DTX off at 0% FER Mono signal generated with [IVAS Pre-renderer or EVS compatible Mono DMX] |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-1b: Stereo

Conditions (BS1534-1b Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with stereo audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with [IVAS Pre-renderer or EVS compatible Mono DMX] |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-2a: FOA

Conditions (BS1534-2a Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with FOA audio input at 16.4 kbps, 24.4 kbps, 32 kbps, 48 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 16.4 kbps, 32 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-2b: FOA

Conditions (BS1534-2b Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with FOA audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-3: HOA3

Conditions (BS1534-3 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with HOA3 audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | [High-quality headphone for diotic presentation, in accordance with clause XXX or High-quality loudspeaker: 7.1+4 overhead speaker setup with the configuration following IVAS-8a] |
| Listening Environment | No room noise |

Experiment BS1534-4: Multichannel 5.1

Conditions (BS1534-4 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with multichannel 5.1 audio input at 16.4 kbps, 24.4 kbps, 32 kbps, 48 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 16.4 kbps, 32 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | [High-quality headphone for diotic presentation, in accordance with clause XXX or High-quality loudspeaker: 5.1] |
| Listening Environment | No room noise |

Experiment BS1534-5: Multi-channel 5.1, 7.1

Conditions (BS1534-5 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with multichannel 5.1, 7.1 audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-6: Multi-channel 5.1+2, 5.1+4

Conditions (BS1534-6 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with multichannel 5.1+2, 5.1+4 audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-7: Multi-channel 7.1+4

Conditions (BS1534-7 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with multichannel 7.1+4 audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | [High-quality headphone for diotic presentation, in accordance with clause XXX or High-quality loudspeaker: 7.1+4 overhead speaker setup with the configuration following IVAS-8a] |
| Listening Environment | No room noise |

Experiment BS1534-8: ISM 1-2

Conditions (BS1534-8 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with ISM 1-2 audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-9a: ISM 3-4

Conditions (BS1534-9a Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with ISM 3-4 audio input at 24.4 kbps, 32 kbps, 48 kbps, 64 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 24.4 kbps, 48 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-9b: ISM 3-4

Conditions (BS1534-9b Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with ISM 3-4 audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-10a: MASA (1TC)

Conditions (BS1534-10a Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with MASA (1TC) audio input at 16.4 kbps, 24.4 kbps, 32 kbps, 48 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 16.4 kbps, 32 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-10b: MASA (1TC)

Conditions (BS1534-10b Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with MASA (1TC) audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-11: MASA (2TC)

Conditions (BS1534-11 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with MASA (2TC) audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-12a: OSBA (1-4 obj.)

Conditions (BS1534-12a Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with OSBA (1-4 obj.) audio input at 16.4 kbps, 24.4 kbps, 32 kbps, 48 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 16.4 kbps, 32 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-12b: OSBA (1-4 obj.)

Conditions (BS1534-12b Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with OSBA (1-4 obj.) audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Experiment BS1534-13: OMASA (1-4 obj.)

Conditions (BS1534-1a Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with OMASA (1-4 obj.) audio input at 64 kbps, 96 kbps, 128 kbps, 256 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | Mono EVS 64 kbps, 128 kbps DTX off at 0% FER Mono signal generated with IVAS Pre-renderer |
| Other references |  |
| Reference | Direct signal, Nominal input level |
| Hidden Reference | Direct signal, Nominal input level |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

Proposed P.SUPPL800 Test Configurations for Inclusion in IVAS-8b

Experiment P800-1: Stereo

Experiment setup

Tables E.1.1 to E.1.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.1.1: Conditions for Experiment P800-1

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| Codec references |  |
| Codec references Bitrates | EVS - Mono signal generated with IVAS Pre-renderer 13.2, 16.4, 24.4, 32, 48, 64, 96, 128 kbps |
| Input level DTX | -26 LKFS DTX off |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| Other references |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [27] and impulse responses provided by MC. Cat. 5-6: Pre-produced content |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause XX |
| Listening System | Headphones, in accordance with clause XX |
| Listening Environment | No room noise, in accordance with clause XX |
| Languages | TBD |

Table E.1.2: Preliminaries for Experiment P800-1

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c02 |  | Mono | - | - |
| 3 | c07 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c10 |  | ESDRU α = xx | - | - |
| 6 | c11 |  | EVS | 13.2 | No errors |
| 7 | c04 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c18 |  | EVS | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table E.1.3: Test conditions for Experiment P800-1,
stereo speech and mixed music under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx |  |  |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Content type categories and scene definitions

Clean and noisy speech categories

| Category | Room | Reverb | Microphone Setup | Background | SNR [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | Low level idle noise | 45 | 1 | Max available up to SWB | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | echoic | A-B (150cm) | Low level idle noise | 45 | -1 | max available up to SWB | 5-11 1-6 3-7 5-8 9-7 10-9 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | echoic | Binaural | office | 15 | 1 | max available up to SWB | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | car | car | A-B Cardioid pair20 cm | car | 15 | -1 | Max available up to SWB | Driver-Passenger BackRight-Driver Driver-BackCenter BackLeft-Driver BackRight-BackLeft BackCenter-BackRight | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Mixed music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | music |

Experiment P800-2: FOA

Experiment setup

Tables E.2.1 to E.2.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.2.1: Conditions for Experiment P800-2

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| Codec references |  |
| Codec references Bitrates | EVS - Mono signal generated with IVAS Pre-renderer 13.2, 16.4, 24.4, 32, 48, 64, 96, 128 kbps |
| Input level DTX | -26 LKFS DTX off |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| Other references |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (FOA) ambient noise mixing. Cat. 5-6: Pre-produced content |
| Binaural renderer | FOA to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause XX |
| Listening System | Headphones, in accordance with clause XX |
| Listening Environment | No room noise, in accordance with clause XX |

Table E.2.2: Preliminaries for Experiment P800-2

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c02 |  | Mono | - | - |
| 3 | c07 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c10 |  | ESDRU α = xx | - | - |
| 6 | C11 |  | EVS | 13.2 | No errors |
| 7 | c04 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c18 |  | EVS | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table E.2.3: Test conditions for Experiment P800-2,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx |  |  |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Content type categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_FOA | room_1_cleanbg_FOA | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_FOA | room_[1/4]_cleanbg_FOA | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Mixed music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-3: MC 5-1

Experiment setup

Tables E.3.1 to E.3.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.3.1: Conditions for Experiment P800-3

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| Codec references |  |
| Codec references Bitrates | EVS - Mono signal generated with IVAS Pre-renderer 13.2, 16.4, 24.4, 32, 48, 64, 96, 128 kbps |
| Input level DTX | -26 LKFS DTX off |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| Other references |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective to various talker positions relative to a capture point and spatial (FOA) ambient noise mixing, converted to 5.1 using IVAS Pre-renderer. Cat. 5-6: Pre-produced native 5.1 content |
| Binaural renderer | MC to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Generic audio content as described in clause XX |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause XX |
| Listening System | Headphones, in accordance with clause XX |
| Listening Environment | No room noise, in accordance with clause XX |

Table E.3.2: Preliminaries for Experiment P800-3

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c02 |  | Mono | - | - |
| 3 | c07 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c10 |  | ESDRU α = xx | - | - |
| 6 | c11 |  | EVS | 13.2 | No errors |
| 7 | c04 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c18 |  | EVS | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table E.3.3: Test conditions for Experiment P800-3,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx |  |  |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Content type categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_FOA | room_1_cleanbg_FOA | 45 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_FOA | room_[1/4]_cleanbg_FOA | 45 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Mixed music categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | music |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a.  Background name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-4: 1-4 Objects

Experiment setup

Tables E.4.1 to E.4.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.4.1: Conditions for Experiment P800-4

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| Codec references |  |
| Codec references Bitrates | EVS - Mono signal generated with IVAS Pre-renderer 13.2, 16.4, 24.4, 32, 48, 64, 96, 128 kbps |
| Input level DTX | -26 LKFS DTX off |
| Input frequency mask | HP50 |
| Noise | No noise |
| Error Conditions | 0% |
| Other references |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: | Cat. 1-2: Defined scenes, 1 ISM Cat. 3-4: Defined scenes, 2 ISMs Cat. 5-6: Pre-produced content, 3-4 ISMs |
| Binaural renderer | ISM to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause XX |
| Listening System | Headphones, in accordance with clause XX |
| Listening Environment | No room noise, in accordance with clause XX |

Table E.4.2: Preliminaries for Experiment P800-4

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c02 |  | Mono | - | - |
| 3 | c07 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c10 |  | ESDRU α = xx | - | - |
| 6 | c11 |  | EVS | 13.2 | No errors |
| 7 | c04 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c18 |  | EVS | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table E.4.3: Test conditions for Experiment P800-4,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx |  |  |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Content type categories and scene definitions

Scene definitions categories 1-2

A leading and trailing silence is present for each sample, in accordance with IVAS-7a. The metadata corresponds to the whole duration of the samples. This means that for moving objects, only a part of the trajectory corresponds to active speech. The following scenes are used:

Talker sitting at a table (elevation 0°), at different azimuths.

Standing talker (elevation 35°), at different azimuths.

Smaller talker (child) walking around a table in the positive sense (counterclockwise), elevation 0°. Azimuth varies continuously for the sentence pair.

Adult talker walking around a table in the negative sense (clockwise), elevation 35°. Azimuth varies continuously for the sentence pair.

Elevation displacement: Elevation varies continuously for the sentence pair . Azimuth is constant for a sentence pair, but different for each sentence pair.

Azimuth and elevation displacement: Azimuth and elevation vary continuously.

Each of the sentences uttered by a certain talker is encoded using different scene. To balance the test, in addition to listeners of each panel listening to all talkers (Categories), all scenes are also covered in each panel. Allocation of scenes to each panel is given in the Table E 4.4

Scene definitions categories 3-4

The listening database consists of artificially created spatial audio samples from monophonic clean speech recordings where always 1 female and 1 male talker are combined in conversation-like scenarios following the Scene descriptions below.

A leading and trailing silence is present for each artificially created spatial audio sample, in accordance with IVAS-7a. The metadata corresponds to the whole duration of the sample. This means that for moving objects, only a part of the trajectory corresponds to active speech.

In one half of the samples, the 2nd talker’s utterance follows the 1st talker’s utterance simulating natural conversation. The gap between the utterances is set to 1 s. In the other half of the samples, the situation is similar, but the utterances partially overlap. The targeted overlap is also 1 s. Non-overlapping sentence pairs are used for Scenes a., c., and e. as described below. Overlapping sentence pairs are used for Scenes b., d., and f. The following scenes are used:

- Two talkers sitting at a table (elevation 0°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair.

Two standing talkers (elevation 35°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair.

One talker sitting at a table (elevation 0°), second talker standing beside the table (elevation 45°). Non-overlapping utterances.

One talker sitting at a table (elevation 0°), second talker walking around the table (elevation 45°). The azimuth of the 2nd talker varies continually, positive sense is counter clockwise.

Two talkers walking side-by-side around the table (elevation 45°). The azimuth is the same for both talkers and varies continually.

Two talkers walking around the table in opposite directions (elevation 30°), starting at the same position. Azimuths of both talkers vary continually.

The following table lists the test Categories corresponding to different talker pairs. Each of the sentence pairs uttered by a certain talker pair is associated to different scenes. To balance the test, listeners of each panel listen to all talker pairs, and all scenes are covered in each panel.

Table E.4.4: Allocation of scenes for each talker pair (category cat 1 – cat 4) and listening panel (P1-P6)

| Category | Talker initial elevation | Elevation change(1 | Talker initial azimuth | Azimuth change(2 | Panel |
| --- | --- | --- | --- | --- | --- |
| cat 1: M1 | 0° 35° 0° 35° -90° 35° | static static static static 0.3°/ frame -0.2°/ frame | 0° 180° 120° 180° 120° 0° | static static 1°/ frame -1°/ frame static 0.5°/ frame | P1 P2 P3 P4 P5 P6 |
| cat 2: F1 | 35° 0° 35° -90° 35° 0° | static static static 0.3°/ frame -0.2°/ frame static | 120° 60° 120° 60° 300° 300° | static 1°/ frame -1°/ frame static 0.5°/ frame static | P1 P2 P3 P4 P5 P6 |

| Category | Overtalk [s] (1 | 1st talker elevation | 2nd talker elevation | 1st talker initial azimuth | 1st talker azimuth change(2 | 2nd talker initial azimuth | 2nd talker azimuth change(2 | Panel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 3: M2 + F2 | 1 -1 1 -1 1 -1 | 35° 0° 0° 45° 30° 0° | 35° 45° 45° 45° 30° 0° | 20° 30° 250° 290° 180° 10° | static static static -1°/ frame 1°/ frame static | 170° 230° 340° 290° 180° 110° | static static -1°/ frame -1°/ frame -1°/ frame static | P1 P2 P3 P4 P5 P6 |
| cat 4: M3 + F3 | -1 1 -1 1 -1 1 | 0° 0° 45° 30° 0° 35° | 45° 45° 45° 30° 0° 35° | 40° 300° 180° 240° 20° 30° | static static 1°/ frame 1°/ frame static static | 290° 290° 180° 240° 170° 230° | static -1°/ frame 1°/ frame 1°/ frame static static | P1 P2 P3 P4 P5 P6 |

Mixed music categories

| Category | Type |
| --- | --- |
| cat 5 | Speech + effects (3-4 objects) |
| cat 6 | Speech + music or music only (3-4 objects) |

Notes:

(1 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(2 The positive sense for azimuth is counterclockwise

Experiment P800-5: MASA 1 TC

Experiment setup

Tables E.5.1 to E.5.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.5.1: Conditions for Experiment P800-5

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| Codec references |  |
| Codec references Bitrates | EVS - Mono signal generated with IVAS Pre-renderer 13.2, 16.4, 24.4, 32, 48, 64, 96, 128 kbps |
| Input level DTX | -26 LKFS DTX off |
| Input frequency mask | HP50 |
| Noise | No noise for cat 1,2,5,6, 15dB for cat 3,4 |
| Error Conditions | 0% |
| Other references |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Model-based generation according to convolution of raw mono clean speech sentences with FOA Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA background. MASA format generation from FOA according to MASA analysis [28]. |
| Binaural renderer | IVAS MASA internal binaural rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.4 |
| Listening System | Headphones, in accordance with clause 4.6 |
| Listening Environment | No room noise, in accordance with clause 4.6 |

Table E.5.2: Preliminaries for Experiment P800-5

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c02 |  | Mono | - | - |
| 3 | c07 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c10 |  | ESDRU α = xx | - | - |
| 6 | c11 |  | EVS | 13.2 | No errors |
| 7 | c04 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c18 |  | EVS | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table E.5.3: Test conditions for Experiment P800-5,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx |  |  |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Content type categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_MASA | room_1_cleanbg_MASA | 45 | 1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_4_MASA | room_4_cleanbg_MASA | 45 | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[1/2]_MASA | [park_1_bg_MASA / nature_1_bg_MASA / event_1_bg_MASA / street_[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_MASA | [cafeteria_1_bg_MASA / mall_1_bg_MASA/ office[1/2]_bg_MASA] | 15 | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

| Category | Type |
| --- | --- |
| cat 5 | mixed content |
| cat 6 | Generic audio |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

Experiment P800-6: OSBA (1-4 objects)

Experiment setup

Tables E.6.1 to E.6.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.6.1: Conditions for Experiment P800-6

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 46, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | HP50 |
| Noise | 15 dB for cat 1,2,3,4, tbd for cat 5,6 |
| Error Conditions | 0% |
| Codec references |  |
| Codec references Bitrates | EVS - Mono signal generated with IVAS Pre-renderer 13.2, 16.4, 24.4, 32, 48, 64, 96, 128 kbps |
| Input level DTX | -26 LKFS DTX off |
| Input frequency mask | HP50 |
| Noise | 15 dB for cat 1,2,3,4, tbd for cat 5,6 |
| Error Conditions | 0% |
| Other references |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q= xx, xx, xx, xx dB α = xx, xx, xx |
| Input frequency mask | HP50 |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-2: Defined scenes, 1 ISM + FOA [or/and HOA3 if available] background Cat. 3-4: Defined scenes, 2 ISMs + FOA [or/and HOA3 if available] background Cat. 5-6: Pre-produced content, 3-4 ISMs + FOA [or/and HOA3 if available] background |
| Binaural renderer | OSBA to binaural internal rendering |
| Audio sampling frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments (with or without background) and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause XX |
| Listening System | Headphones, in accordance with clause XX |
| Listening Environment | No room noise, in accordance with clause XX |

Table E.6.2: Preliminaries for Experiment P800-6

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | IVAS FL | 13.2 | No errors |
| 2 | c02 |  | Mono | - | - |
| 3 | c07 |  | ESDRU α = xx | - | - |
| 4 | c20 |  | IVAS FL | 16.4 | No errors |
| 5 | c10 |  | ESDRU α = xx | - | - |
| 6 | c11 |  | EVS | 13.2 | No errors |
| 7 | c04 |  | MNRU Q=xx dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c18 |  | EVS | 128 | No errors |
| 10 | c07 |  | ESDRU α = xx | - | - |
| 11 | c05 |  | MNRU Q=xx dB | - | - |
| 12 | c24 |  | IVAS FL | 64 | No errors |

Table E.6.3: Test conditions for Experiment P800-6,
clean speech under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx |  |  |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Content type categories and scene definitions

ISM positions for categories cat 1-4 as defined respectively in Table E.4.4.

SBA scenes as follows:

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_[X]_FOA | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | 15 | 1 | Max |  | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[X]_FOA | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | 15 | -1 | Max |  | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Mixed music categories

| Category | Type |
| --- | --- |
| cat 5 | 3-object + General audio background |
| cat 6 | 4-objects + General audio background |

Notes:

(1 Editor’s note: The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Editor’s note: Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a.  Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 Editor’s note: The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.