Source:	Nokia Corporation

Title:	Characterization of IVAS codec

Document for:	Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], discuss specifics of IVAS Codec characterization testing.

In this document the source proposes test material and conditions for ACR (Absolute Category Rating) experiment covering both legacy mono conditions and all IVAS spatial operation modes. It is not possible to cover all bitrates and IVAS operation modes, but ACR test methodology allows significant amount operation modes to be tested.

# 2. Overview

In the current work in progress IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], one ACR test has been reserved. The source proposes specific test material and conditions that can be used to characterize IVAS codec with ACR test methodology. Since the subjective quality varies from low bitrate mono AMR to uncompressed high order ambisonics conditions it is proposed to use extended ACR5 scale for quality evaluation.

# 3. Discussion

Some observations and considerations are made relating, e.g., to size of experiment. In particular, the P.SUPPL800 [3] DCR experiments can have 36 test conditions. ACR test in principle can have the double amount of conditions, i.e. 72 conditions, for the same amount of listenable audio, since there is no reference sample. However, for the listener there is still double number of votes to be casted, causing significant mental load. Thus, the number of conditions should be somewhat lower than 72 conditions. In this document the proposal is to have 66 ACR conditions.

- Currently one ACR listening test is proposed for IVAS characterization.

  - The assumption of this proposal is that this test will conducted within Nokia listening facilities.

  - It would be highly beneficial to conduct this test in two different listening laboratories with two different languages.

- It is proposed to test all conditions with either IVAS internal binaural renderer or with IVAS_rend functionality for Direct references. For mono conditions diotic playback is used.

- Fixed point version of the IVAS codec should be used, if available.

- It is proposed to test all codecs up to the highest available bitrate or up to a bitrate, where quality saturation is expected.

- No FER, nor DTX conditions are tested.

- No floating point to fixed point interoperation is tested.

- Listeners can be both experienced or naïve. It is clear that experienced listeners will provide more consistent results (e.g. separation of mono and spatial conditions), but even with naïve listeners we have had consistent results, when instructed properly and having enough preliminaries.

- Test samples

  - Since high order ambisonics and 2Dir MASA modes are included to the test conditions the source material at minimum should be HOA2

  - There are six sample categories. It is proposed to reuse some suitable signal categories from the IVAS selection testing. I.e. Clean speech and noisy speech samples, which have HOA2 material available.

  - In addition to the script generated test material, additional real recorded speech material is used for the listening evaluation.

  - This additional material must be either Eigenmike or HO3 material. In case of Eigenmike material, it can be converted to HOA3 for operation modes needing ambisonics input. MASA can use Eigenmike signal directly (with masaAnalyser).

  - Length of the test samples should be around 6- 8 seconds. Too short or too varying length samples cause surprises to listeners. I.e. listeners are not mentally ready for casting a vote and playback has already ended.

- Enough coverage of the IVAS operation points should be achieved

  - Most common EVS deployed bitrates 13.2 and 24.4 kbps should be well covered

  - High bitrates up to quality saturation should be tested.

  - Six bitrates for each operation mode are reserved

  - FOA and 1TC MASA operation bitrates are slightly lower than HOA and 2TC operation bitrates

- Table 1 shows the coverage of the proposed bitrates and operation points.

Table 1 Bitrate distribution for different codec conditions. Most common service deployed EVS bitrates high-lighted.

| bitrate | EVS-WB | EVS | IVAS STEREO | IVAS FOA | IVAS HOA | IVAS MASA 1TC | IVAS MASA 2TC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7,2 | X |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 9,6 | X |  |  |  |  |  |  |
| 13,2 | X | X | X | X |  | X |  |
| 16,4 | X |  |  |  | X |  | X |
| 24,4 | X | X | X | X | X | X | X |
| 32 |  | X |  | X |  | X |  |
| 48 | X |  | X |  | X |  | X |
| 64 |  | X |  | X |  | X |  |
| 80 |  |  | X |  | X |  | X |
| 96 |  | X |  | X |  | X |  |
| 128 |  | X | X |  | X |  | X |
| 160 |  |  |  | X |  | X |  |
| 192 |  |  | X |  |  |  |  |
| 256 |  |  |  |  | X |  | X |
| 384 |  |  |  |  |  |  |  |
| 512 |  |  |  |  |  |  |  |

# 4. Extended ACR5 voting scale

Nokia has conducted listening tests for a long time with 9-scale ACR test methodology. Recently, Nokia reported some comparison results made with ACR5, ACR9 and extACR5 voting scales for ITU-T P.SCALE question [2]. The P.SCALE work is still on-going in ITU-T. However, the ITU-T submitted results and further listening tests done within Nokia indicate clear benefit of using more that 5-scale ACR, especially when there are multiple quality dimensions in the same ACR listening test.

In the proposed IVAS characterization listening test, there are multiple quality dimensions. At least following can be easily separated and mentioned:

•	Bandwidth loss (NB, WB, FB, low bitrate effects on bandwidth)

•	Signal distortion (generated MNRU noise or low bitrate codec introduced signal distortions)

•	Spatiality degradation (generated ESDRU spatial distortion, mono audio, low bitrate introduced spatial issues)

It is easier to find proper vote for them, if more than 5 grades are available for vote casting. For example, if every quality dimension is perfect, just vote ~5 (4.5, 5 or 5.5) (Direct or high bitrate spatial audio). If one quality dimension is clearly lacking: vote at maximum ~4 (WB spatial audio (lacking bandwidth) or Fullband mono (lacking spatiality)). If two quality dimensions are lacking: vote at maximum ~3 (WB mono (lacking both in bandwidth and spatial dimension), or low bitrate EVS-SWB (lacking spatial dimension and has clear signal distortion)). If all quality dimensions have significant issues: vote at maximum ~2 (Low bitrate mono AMR-WB (lacking in bandwidth, has distortion and spatiality loss). Sounds just bad vote ~1 (low bitrate mono AMR).

We note that there exists a lot of listening test results and data performed with standard ITU-T P.800 test methodology and ACR5 grading scale. Thus, the new scale is compatible with the old results and it is straight forward to mentally map the results to the standard P.800 ACR scale. Thus, a new scale (see Table 2) is proposed to be used. We call this an extended ACR5 scale. It is quite similar to ITU-T P.910 Annex B.2 11-scale, with the change of numbering from 0 - 10 to 0.5 – 5.5 with 0.5 step size. It should be noted that listeners did not see the decimal values. The user interface screen shot can be seen in Figure 1.

Table 2 Extended ACR5 voting scale

| 5.5 |  |
| --- | --- |
| 5 | Excellent |
| 4.5 |  |
| 4 | Good |
| 3.5 |  |
| 3 | Fair |
| 2.5 |  |
| 2 | Poor |
| 1.5 |  |
| 1 | Bad |
| 0.5 |  |

Figure 1 Example of using extended ACR 5 voting scale in the listening test software.

# 4.1 Preliminary listening test results

Nokia has performed a shortened version of the proposed extended ACR5 listening test. The main difference is reduced number of conditions and slightly different sample categories: 4 Eigenmike recordings, 2 HOA2 conditions from the IVAS selection testing [4]. First part of the listening test result was conducted with expert listeners. Attached are results with N=8 expert listeners in Figure 2.

Figure 2 Results of preliminary ext ACR IVAS characterization listening test with N=8 expert listeners.

Figure 3 Naïve listeners N=8 results

Figure 4 Both naïve and expert listeners combined N=16

As can be seen from the listening test results shown in Figure 2. Expert listeners easily distinguish between mono and spatial audio. Different 3GPP audio codec generations are also easily distinguishable from each other. Bitrate scaling is very clear. In the preliminary listening test the maximum test bitrate was set to 128 kbit/s, which is not enough to saturate the results. In the proposed listening test, the maximum bitrate is increased to 256 kbit/s for some operation modes.

Naïve listeners results shown in Figure 3 show also clear difference between different codecs and bitrates. The difference between spatial audio and mono is significantly less clear than with expert listeners.

The combined results shown in Figure 4 show the middle ground of the above results.

# 5. Proposal for specific test conditions for the ACR test

It is proposed for to have following conditions for the IVAS characterization listening test using extended ACR5 voting scale.

Table 3 Overview of test conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 6 | IVAS stereo 13.2, 24.4, 48, 80, 128, and 192 kbps |
|  | 6 | IVAS FOA 13.2, 24.4, 32, 64, 96 and 160 kbps |
|  | 6 | IVAS HOA 16.4, 24.4, 48, 80, 128, 256 kbps |
|  | 6 | IVAS 1TC MASA 13.2, 24.4, 32, 64, 96 and 160 kbps |
|  | 6 | IVAS 2TC MASA 16.4, 24.4, 48, 80, 128, 256 kbps |
| Codec references | 4 | AMR (narrow band) operated at 4.75, 7,95, 10,2 and 12,2 kbps |
|  | 6 | AMR-WB operated at 6.6, 8.85, 12.65,15.85, 19.85 and 23.85 kbps |
|  | 6 | EVS-WB 7.2, 9.6, 13.2, 16.4, 24.4 and 48 kbps |
|  | 6 | EVS-SWB/FB 13.2, 24.4, 32, 64, 96 and 128 kbps |
| Other references |  |  |
| Directs | 8 | NB mono, WB mono, FB mono, stereo, FOA, HOA, 1TC MASA, 2 TC MASA |
| P.50 MNRU (applied to MASA transport streams) | 3 | Q = 30, 25, and 20 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 3 | α = 0.8, 0.5, and 0.2 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 2 | 3 categories using model-based generation according to processing scripts. 3 categories using true recordings in Eigenmike or HOA3 format. |
| Binaural rendering | 1 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend). Diotic playback for mono conditions. Stereo conditions use stereo in and out. |
| Audio sampling frequency / bandwidth | 4 | 48 kHz / maximum available audio bandwidth (NB, WB, SWB, FB) |
| Content types / categories | 6 | Scenes as described in Table 4 |
| Number of talkers | 3 | In each sample there may be 2, 3, or 4 different talkers |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) measured from each listenable sample (post-scaling). |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve and experienced listeners (about equal amount) |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | ACR with instructions according to [P.SCALE] |
| Languages | [2] | Finnish / [tbd another test conducted with other language is seen beneficial] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

Table 4 Sample Categories

| Category | Format | Overall Level [dB] | Background signal type | Background Level | Overtalk [s] | Talker positions |
| --- | --- | --- | --- | --- | --- | --- |
| cat 1 | HOA2 | -26 | Clean speech | 45 dB SNR Idlenoise | No overtalk | As in selection P800-8 catX |
| cat 2 | HOA2 | -26 | Indoors | 15 db SNR | No overtalk | As in selection P800-9 catY (Office?) |
| cat 3 | HOA2 | -26 | Outdoors | 15 db SNR | No overtalk | As in selection P800-9 catZ (Street?) |
| cat 4 | Eigenmike / HOA3 | -26 | Clean speech (real recording) | Not available | Overtalk | Frontal area 4 talkers (indoors) |
| cat 5 | Eigenmike / HOA3 | -26 | Outdoors (real recording) | Not available | No overtalk | 2/3 talkers (outdoors) |
| cat 6 | Eigenmike / HOA3 | -26 | Talkers with BG music (real recording) | Not available | Speech over music | 2 talkers with BT speaker music (outdoors) |

Table 5 Test conditions for extended ACR5 IVAS characterization

| Label | Condition | Bitrate / MNRU / ESDRU | Notes |
| --- | --- | --- | --- |
| C01 | Reference HOA |  | With IVAS_rend BINAURAL |
| C02 | Reference FOA |  | With IVAS_rend BINAURAL |
| C03 | Reference 2TC MASA |  | With IVAS_rend BINAURAL |
| C04 | Reference 1TC MASA |  | With IVAS_rend BINAURAL |
| C05 | Reference stereo |  | Side point cardioids (HOA) or MASA 2TC transports (Eigen) |
| C06 | Mono FB |  | W channel (HOA) or MASA 1TC transport (Eigen) |
| C07 | Mono WB |  | Upto 8 kHz, 50 Hz high-pass |
| C08 | Mono NB |  | Upto 4 kHz, 50 Hz high-pass |
| C09 | MNRU | Q= 30 dB |  |
| C10 | MNRU | Q= 25 dB |  |
| C11 | MNRU | Q= 20 dB |  |
| C12 | ESDRU | α=0,8 |  |
| C13 | ESDRU | α=0,5 |  |
| C14 | ESDRU | α=0,2 |  |
| C15 | AMR | 4,75 | Mono is either: |
| C16 | AMR | 7,95 | W channel for HOA samples or |
| C17 | AMR | 10,2 | MASA 1TC transport for Eigenmike samples |
| C18 | AMR | 12,2 | 50Hz high pass filtering |
| C19 | AMR-WB | 6,6 | Mono is either: |
| C20 | AMR-WB | 8,85 | W channel for HOA samples or |
| C21 | AMR-WB | 12,65 | MASA 1TC transport for Eigenmike samples |
| C22 | AMR-WB | 15,85 | 50Hz high pass filtering |
| C23 | AMR-WB | 19,85 |  |
| C24 | AMR-WB | 23,85 |  |
| C25 | EVS-WB MONO | 7,2 | Mono is either: |
| C26 | EVS-WB MONO | 9,6 | W channel for HOA samples or |
| C27 | EVS-WB MONO | 13,2 | MASA 1TC transport for Eigenmike samples |
| C28 | EVS-WB MONO | 16,4 | 16 kHz input, 48 kHz decoding |
| C29 | EVS-WB MONO | 24,4 |  |
| C30 | EVS-WB MONO | 48 |  |
| C31 | EVS-SWB MONO | 13,2 | Mono is either: |
| C32 | EVS-FB MONO | 24,4 | W channel for HOA samples or |
| C33 | EVS-FB MONO | 32 | MASA 1TC transport for Eigenmike samples |
| C34 | EVS-FB MONO | 64 |  |
| C35 | EVS-FB MONO | 96 |  |
| C36 | EVS-FB MONO | 128 |  |
| C37 | IVAS STEREO | 13,2 | Stereo is either: |
| C38 | IVAS STEREO | 24,4 | Side pointing cardioids for HOA samples or |
| C39 | IVAS STEREO | 48 | MASA 2TC for Eigenmike samples |
| C40 | IVAS STEREO | 80 |  |
| C41 | IVAS STEREO | 128 |  |
| C42 | IVAS STEREO | 192 |  |
| C43 | IVAS FOA | 13,2 | FOA truncated from HOA2 or HOA3 input |
| C44 | IVAS FOA | 24,4 |  |
| C45 | IVAS FOA | 32 |  |
| C46 | IVAS FOA | 64 |  |
| C47 | IVAS FOA | 96 |  |
| C48 | IVAS FOA | 160 |  |
| C49 | IVAS HOA | 16,4 | Input is HOA2 for script generated content |
| C50 | IVAS HOA | 24,4 | HOA3 for Eigenmike originating real recordings. |
| C51 | IVAS HOA | 48 | Conversion from Eigenmike to HOA3 done outside scripts |
| C52 | IVAS HOA | 80 | with method [tbd] |
| C53 | IVAS HOA | 128 |  |
| C54 | IVAS HOA | 256 |  |
| C55 | IVAS 1TC MASA | 13,2 | All MASA content is generated with masaAnalyzer |
| C56 | IVAS 1TC MASA | 24,4 | HOA2 mode for script generated content |
| C57 | IVAS 1TC MASA | 32 | Eigen mode for Eigenmike content |
| C58 | IVAS 1TC MASA | 64 |  |
| C59 | IVAS 1TC MASA | 96 |  |
| C60 | IVAS 1TC MASA | 160 |  |
| C61 | IVAS 2TC MASA | 16,4 | All MASA content is generated with masaAnalyzer |
| C62 | IVAS 2TC MASA | 24,4 | HOA2 mode for script generated content |
| C63 | IVAS 2TC MASA | 48 | Eigen mode for Eigenmike content |
| C64 | IVAS 2TC MASA | 80 |  |
| C65 | IVAS 2TC MASA | 128 |  |
| C66 | IVAS 2TC MASA | 256 |  |

# 6. Summary

We have presented some IVAS characterization test plan specifics and options for agreement in clauses 4 and 5 of this input. The specific proposal covers extended scale ACR5 characterization experiment for IVAS operation.

# References

[1]	S4aA230127-IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase, Version v.0.1.2

[2]	ITU-T T22-SG12-C-0033, “Evaluation of extended ACR voting scales for subjective speech and audio quality testing”, Q7/SG12, Geneve, 7-17 June 2022

[3]	ITU-T P.800 – Use cases, P Suppl. 29 (01/2023)

[4]	S4aA-230086: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1.1.0