Source:	NTT and Orange

Title:	Characterization test plan of stereo downmix for EVS

Document for:	Discussion and Agreement

Agenda Item:	7.5

Introduction

This contribution provides a test plan for the characterization test of stereo downmix for interoperable EVS.

For interoperable EVS mono coding, only two modes of bit rates (13.2 and 24.4 kbps) are selected since these rates of EVS are widely deployed in the market. We assume six clean, six noisy, and six music items for each panel. Tables 4 and 5 show categories of input materials, and they are the same as those for the P800-1 for clean speech and P800-2 for noisy speech in the selection phase listening test [3]. For music items, we assume a variety of signals and no preselection for finding critical items.

Table 1: Factors and conditions for Experiment P800-dmx

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 2 | 13.2, 24.4 kbps with DTX off at 0% FER |
| DTX | 0 | DTX off |
| Input Level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 1 | 0%, |
| Codec references |  |  |
| Codec references | 5 | IVAS stereo (stereo output) EVS dual mono + static dmx, static dmx + EVS IVAS stereo 24.4 kbps mono output + EVS |
| Operating modes | 2 | 13.2, 24.4 kbps with DTX off at 0% FER |
| DTX | 0 | DTX off |
| Input Level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 1 | 0% |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=16, 22 28 dB α = 0.1, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool |
| Audio sampling frequency/bandwidth | 1 | 32 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and three female) |
| Number of categories | 6 | 6 Different environments and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | Six panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | １ | [tbd] |

Table 2: Preliminaries for Experiment P800-dmx

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c11 | clean | Static dmx + EVS | 13.2 | No errors |
| 2 | c02 | noisy | MNRU Q=28 dB | - | - |
| 3 | c05 | music | ESDRU α = 0.7 | - | - |
| 4 | c08 | music | IVAS stereo | 24.4 | No errors |
| 5 | c07 | clean | IVAS stereo | 13.2 | No errors |
| 6 | c14 | noisy | Stereo dmx +EVS | 13.2 | No errors |
| 7 | c12 | noisy | Static dmx+EVS | 24.4 | No errors |
| 8 | c01 | clean | Reference | - | - |
| 9 | c09 | clean | Dual EVS + static dmx | 2x7.2 | No errors |
| 10 | c04 | music | MNRU Q=16 dB | - | - |
| 11 | c14 | noisy | IVAS stereo monoout+ EVS | 24.4 | No errors |
| 12 | c16 | music | Stereo dmx + EVS | 24.4 | No errors |

Table 3: Test conditions for Experiment P800-dmx,
clean speech, noisy speech, and music

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=28 dB | - | - |  |
| c03 | MNRU Q=22 dB | - | - |  |
| c04 | MNRU Q=16 dB | - | - |  |
| c05 | ESDRU α=0.7 | - | - |  |
| c06 | ESDRU α=0.1 | - | - |  |
| c07 | IVAS stereo | 13.2 | No errors |  |
| c08 | IVAS stereo | 24.4 | No errors |  |
| c09 | Dual mono EVS@7.2 kbps + static dmx | 2x7.2 | No errors |  |
| c10 | Dual mono EVS@13.2 kbps + static dmx | 2x13.2 | No errors |  |
| c11 | Static dmx + EVS @13.2 kbps | 13.2 | No errors |  |
| c12 | Static dmx + EVS @24.4 kbps | 24.4 | No errors |  |
| c13 | IVAS stereo 24 kbps mono-out+ EVS @13.2 kbps | 13.2 | No errors |  |
| c14 | IVAS stereo 24 kbps mono-out+ EVS @24.4 kbps | 24.4 | No errors |  |
| c15 | CuT Stereo dmx + EVS | 13.2 | No errors | NWT c09 |
| c16 | CuT Stereo dmx + EVS | 24.4 | No errors | NWT c10 |

- Table 4: Content type categories and scene definitions (Exp P800-dmx: clean speech)

| Category |  | Room(1 | Reverb | Microphone Setup | Level [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 |  | small | anechoic | A-B (100cm) | -26 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 |  | large | anechoic | A-B (150cm) | -26 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 |  | small | anechoic | M-S | -26 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 |  | small | echoic | A-B (100cm) | -26 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 |  | large | echoic | A-B (150cm) | -26 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 |  | large | echoic | Binaural | -26 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

- Table 5: Content type categories and scene definitions (Exp P800-dmx: noisy speech)

| Category | Room(1 | Reverb | Microphone Setup | Level [dB] | Background | SNR In dB | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | -26 | car | 15 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | anechoic | A-B (150cm) | -26 | office | 15 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | anechoic | M-S | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | small | echoic | A-B (100cm) | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | large | echoic | A-B (150cm) | -26 | office | 15 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | large | echoic | Binaural | -26 | car | 15 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences.

(2 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(3 The talker positions are part of the scene definition of the different categories. They should be chosen in a random way from the available set of SRIRs for the used room, making sure that there is a good coverage of different possible positions. Different random selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7b.

(4 All sentences by the 6 talkers shall be unique.

Summary

Sources have presented an IVAS characterization test plan for stereo downmix. The specific proposal covers the P.SUPPL800 DCR experiment for IVAS.

- References

[1]	S4aA230127-IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase, Version v.0.1.2

[2]	ITU-T P.800 – Use cases, P Suppl. 29 (01/2023)

[3]	S4aA-230086: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1