3GPP TSG-SA4 Meeting #127	 S4-240342

# Sophia-Antipolis, France, 29 January 2024 – 2 February 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.132 | CR | 0110 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of jitter buffer management behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange, HEAD acoutics GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUET |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification of tests on the performance of jitter buffers in MTSI is currently a basic reporting of performance statistics with some preliminary profiles. In the field (e.g. drive tests), speech quality in MTSI-based services was found to depend on UE implementations of JBM, especially when jitter conditions are not those from clean or good channel conditions. This CR updates clauses 7.13, 8.18, 9.13, 10.13 ("Test conditions") of TS 26.132 for jitter buffer management to define requirement with one new profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of test method to assess performance of jitter buffer management. Removal of characterization test method, use of existing delay test method. Inclusion of one new profile with related description in Annex F, replacement of provisional profiles defined for JBM characterization. Editorial clarification in Annex E. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | JBM performance not tested with impairments that simulate realistic behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.13, 7.13.0, 7.13.1, 7.13.2, 7.13.3 (new), 8.13, 8.13.0, 8.13.1, 8.13.2, 8.13.3 (new), 9.13, 9.13.0, 9.13.1, 9.13.2, 9.13.3 (new), 10.13, 10.13.0, 10.13.1, 10.13.2, 10.13.3 (new), Annex F.1, Annex F.2, Annex F.3 (new), attachment (jbm_characterisation_profiles.zip replaced by dly_artificial_volte_profile.txt) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 26.131 CR 0089 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.13	Jitter buffer management behaviour (handset, headset and electrical interface UE)

### 7.13.0	General

For speech-only with LTE, NR or WLAN access, a jitter buffer is used in receiving to handle the variation in packet receiver timing. To minimize the additional latency introduced by the jitter buffer, adaptation is used to minimize delay while preventing packet losses due to packet delivery timing variations. See clause 8 of TS 26.114 [39] for the definition of jitter buffer and minimum performance requirements on JBM.

### The test method in clause 7.13.3 is used to assess the requirement on JBM performance.

### 7.13.1	Void

### 7.13.2	Void

### 7.13.3	Delay and speech quality in conditions with packet arrival time variations (handset, headset, electrical interface UE)

The test method is the same as in clause 7.10.4.

The test profiles are defined in Annex F.

## 8.13	Jitter buffer management behaviour (handset, headset and electrical interface UE)

### 8.13.0	General

For speech-only with LTE, NR or WLAN access, a jitter buffer is used in receiving to handle the variation in packet receiver timing. To minimize the additional latency introduced by the jitter buffer, adaptation is used to minimize delay while preventing packet losses due to packet delivery timing variations. See clause 8 of TS 26.114 [39] for the definition of jitter buffer and minimum performance requirements on JBM.

### The test method in clause 8.13.3 is used to assess the requirement on JBM performance.

### 8.13.1	Void

### 8.13.2	Void

### 8.13.3	Delay and speech quality in conditions with packet arrival time variations (handset, headset, electrical interface UE)

The test method is the same as in clause 8.10.4.

The test profiles are defined in Annex F.

## 9.13	Jitter buffer management behaviour (handset, headset and electrical interface UE)

### 9.13.0	General

For MTSI-based speech-only with LTE, NR or WLAN access, a jitter buffer is used in receiving to handle the variation in packet receiver timing. To minimize the additional latency introduced by the jitter buffer, adaptation is used to minimize delay while preventing packet losses due to packet delivery timing variations. See clause 8 of TS 26.114 [39] for the definition of jitter buffer and minimum performance requirements on JBM.

The test method in clause 9.13.3 is used to assess the requirement on JBM performance.

### 9.13.1	Void

### 9.13.2	Void

### 9.13.3	Delay and speech quality in conditions with packet arrival time variations (handset, headset, electrical interface UE)

The test method is the same as in clause 9.10.4.

The test profiles are defined in Annex F.

## 10.13	Jitter buffer management behaviour (handset, headset and electrical interface UE)

The test method is the same as in super-wideband (see clause 9.13, observing the test signal properties for fullband described in clause 5.4).

### 10.13.0	General

The test method is the same as in super-wideband (see clause 9.13, observing the test signal properties for fullband described in clause 5.4).

The same considerations as described in clause 9.13.0 apply for fullband mode. The test methods are the same as in super-wideband (see clause 9.13, observing the test signal properties for fullband described in clause 5.4).

### 10.13.1	Void

### 10.13.2	Void

### 10.13.3	Delay and speech quality in conditions with packet arrival time variations (handset, headset, electrical interface UE)

The test method is the same as in clause 10.10.4.

The test profiles are defined in Annex F.

Annex F (normative):
Packet delay and loss profiles for jitter buffer management behaviour

# F.1	General

This Annex provides packet delay and loss profiles to be inserted on the IP packets in the test system when testing the UE delay and speech quality for speech-only with LTE, WLAN, or NR access in jitter and error conditions, to evaluate the jitter buffer management behaviour. The profiles are based on the assumption that one IP packet corresponds to 20ms of the speech sequence. In order to preserve the synchronization of the elements in the profile and the timing of the speech sequence (e.g. the RTP time stamp) when the speech codec does not produce one packet every 20ms, e.g. during speech codec DTX operation, the profiles needs to be sub-sampled in accordance with the rate of production of the packets containg the speech frames.

# F.2	Packet delay and loss profiles

Table F.1: Packet delay profiles.

| Delay and Loss profile | Packet loss (%) | Description |
| --- | --- | --- |
| dly_artificial_volte_profile | 0 | See Annex F.3 |
| NOTE: The profile is constructed based on considerations from realistic packet impairments observed on LTE. It does not include any packet loss for better disentanglement between JBM and PLC performance. |  |  |

Since the model simulates the packet transmission end-to-end, the delay profiles include a packet delay component that according to the definition of the UE delay in 3GPP TS 26.131 [1] is not attributed to the UE delay. This delay shall be included in the test equipment delay when deriving the UE delay from the delay measurements based on the cross-correlation of the injected and measured signals. Values for the compensation attributed to the packet delay profiles are shown in Table F.2.

Table F.2: Packet delay profiles compensation values.

| Delay and Loss profile | Compensation value |
| --- | --- |
| dly_artificial_volte_profile | 160 ms |

# F.3	Methodology to construct a profile based on impairments observed in live LTE operation

The following methodology has been used to construct the profile dly_artificial_volte_profile:

Step 0: identification of most sensitive active speech packets

Condition 0 from delay tests is recorded to obtain a reference PCAP file. A single packet loss is applied on each individual active speech packet of the PCAP file and the impaired PCAP file is decoded; the MOS-LQO score is measured for each packet, and the most sensitivity active speech packets are identified with respect to packet loss at a sentence level.

Step 1: set a fixed delay applied to all packets of 160 ms

Step 2: add jitter bursts

Apply an extra delay of 100 ms to 2.81% among most sensitive active speech packets in each sentence; when this extra delay of 100 ms is applied to packet N, the subsequent packets N+1, N+2, N+3 and N+4 are also delayed by respectively 81, 62, 43 and 24 ms to avoid adding packet order impairments

NOTE: There are 4 double sentences repeated 5 times, there are 4 impaired packets in each sentence, except one sentence with 5 impaired packets because this sentence is longer than other sentences. This results in 165 impaired packets.

Step 3: add packet inversions

Select 2.81% among most sensitive active speech packets in each sentence (outside the set of packets impaired in step 2) and add an extra delay of 21 ms to simulate an inversion; if packet N is selected to be inverted, packet N+1 is not impaired.

Step 4: add duplications

Select 2.81% among most sensitive active speech packets in each sentence (outside the set of packets impaired in steps 2 and 3) and add a duplicate with an extra delay of 1 ms

Replace jbm_characterisation_profiles.zip (including current profiles) by new zip including new profile “dly_artificial_volte_profile.dly”

- End of changes