3GPP TSG-SA4 Meeting #127	S4-240420

# Sophia-Antipolis, France, 29th Jan 2024 - 2nd Feb 2024			revision of S4-240360

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | pseudo | rev | 2 | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [PROMISE] pCR 26.143 Definition of Profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PROMISE |  |  |  |  |  | Date: |  |  | 2024-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Messaging Profiles were not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defines a Messaging Profile baseline for Player Defines a Messaging Profile baseline for Generator Adds registration Annex |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Profiles and registration not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6, 6.2 (new), 6.3 (new), Annex A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Remove some Editor’s Notes before integration |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-240111; [PROMISE] pCR 26.143 Definition of Profiles; Qualcomm Tech. Netherlands B.V; Thomas Stockhammer. E-mail Discussion: none Revisions: none Presenter: Thomas Stockhammer Online Discussion: - Frederic: To be clear, this spec doesn’t apply to MMS. - Stephane: EVS was “shall”. - Frederic: Yes, it should be “shall” for EVS. There are also mistakes in the others on the decoding side. This should be aligned with 26.511. - Dimitri: I believe this has some “shall” for GLTF. We dont agree to that - Thomas: I am just implementing the agreements. - Waqar: DOnt understand the text. Can you explain - “If presentations are supported”. What does it mean? - Frederic: If you have messaging client that supports rendering, then it is so and so. - Waqar: Are we talking hardware and software? I am assuming we are not talking third party plugins. This - “3D scenes and assets are supported” - needs to be better phrased - Waqar: We do not agree to have “shall” in 2nd change. Okay with having “should” for first two. However, no for HTML5 Decision: 3 paragraphs on 3D and HTML5 to be removed and the URL to be corrected. Revised. The revision will be agreed without presentation. S4-240111 is revised to S4-240360. S4-240360; [PROMISE] pCR 26.143 Definition of Profiles; Qualcomm Tech. Netherlands B.V; Thomas Stockhammer. E-mail Discussion: none Revisions: none Presenter: Thomas Stockhammer Online Discussion: - Frederic: There is a proposition to put a shoul dinstead of shall. - Chris: We should put “for” before media type ‘model’ - Waqar: Maybe we could indicate in clause 6.2.3 “It is recommended that clients supporting such capabilities determine if media can be safely processed prior to any processing and rendering.” under the 2 capabilities. - Thomas: I am fine. It could be better to put this sentence in another spec. Decision: Revised. S4-240360 is revised to S4-240420. S4-240420; [PROMISE] pCR 26.143 Definition of Profiles; Qualcomm Tech. Netherlands B.V; Thomas Stockhammer. Decision: Agreed without presentation. S4-240420 is agreed. |

# =====  CHANGE =====

# 2	References

Add at the end

[X]	IETF RFC 2077, "The Model Primary Content Type for Multipurpose Internet Mail Extensions"

[Y]	3GPP TS 26.119, "Media Capabilities for Augmented Reality"

# =====  CHANGE =====

# 6	Messaging Media Profiles

## 6.1	Overview

This specification defines several messaging media profiles that apply for the player or the content generator.

# =====  CHANGE =====

## 6.2	Baseline MMBP Player Profile

### 6.2.1	Overview

The baseline MMBP Player profile is aligned with TS 26.140 [32].

The container format is based on IETF RFC 2045 [23] as the format for the MMBPs. Offering of alternative content is the container is permitted. In addition, the profile permits to encapsulate encapsulation of real-time video into the 3GP file format using the baseline profile. The container does not support external bodies, i.e. the MMBP is expected to be delivered as a single message. For details on the container format requirements, refer to clause 6.2.2.

The media types address basic text, audio/speech, images, video, text/subtitle, 3D scenes including AR as well as simple HTML-5 presentations.

Conten conforming to the baseline MMBP player may include media types that are not explicitly supported by the media capabilities as defined in clause 6.2.3. Receivers shall ignore non-recognized media types. However, based on the container requirements, ignoring media types may results in specific processing requirements, for example pick an alternative, or ignore the entire MMBP.

Content generated to be compatible for playback on players for this profile as well as players expose their capabilities should use the URN identifier "urn:3GPP:26143:18:baseline-mmbp-player".

### 6.2.2	Container Format

The following capabilities for the container format as defined in clause 5.2.1 shall be supported:

-	26143_CONTAINER_RFC2046_SINGLE

-	26143_CONTAINER_RFC2046_MIXED

-	26143_CONTAINER_RFC2046_ALTERNATIVE

-	26143_CONTAINER_RFC2387_RELATED

-	26143_CONTAINER_MP4_3GP9

The following capabilities for the container format as defined in clause 5.2 should be supported:

-	26143_CONTAINER_RFC2046_PARALLEL

In addition, multipart/parallel shall not be present in an MMBP on the same level if a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9'is present and contains more than on track.

### 6.2.3	Media Types

The capability 26143_TEXT_PLAIN as defined in clause 5.3 shall be supported.

If still images are supported,

-	the 26143_IMG_JPEG capability as defined in clause 5.4.1 shall be supported,

-	the 26143_IMG_HEIC capability as defined in clause 5.4.1 should be supported.

If bitmap graphics are supported,

-	the 26143_IMG_GIF capability as defined in clause 5.4.1 should be supported.

-	the 26143_IMG_PNG capability as defined in clause 5.4.1 should be supported.

If the reception of audio or speech is supported, then the following applies:

-	the 26143_AUDIO_EVS capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_AMR-WB capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_XHE-AAC capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_AMR capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_EAAC+ capability as defined in clause 5.5.1 should be supported.

If the reception of video is supported, then the following applies:

-	the 26143_VIDEO_AVC-HD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-HD capability as defined in clause 5.6.1 should be supported.

If the reception of HD-HDR video is supported, then the following applies:

-	the 26143_VIDEO_AVC-FullHD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-FullHD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-UHD capability as defined in clause 5.6.1 should be supported.

If timed text is supported,

-	the 26143_TT_3GPP capability as defined in clause 5.7.1 shall be supported.

-	the 26143_TT_IMSC11 capability as defined in clause 5.7.1 should be supported.

If a processor for media type 'model' as defined in RFC2077 [X] is supported (i.e. a processor for 3D scenes and objects), then a processor for the media subtype 'model/gltf' should be supported. If a processor for the media subtype 'model/gltf' is supported,

-

the 26143_SCENE_GLTF20 capability and the 26143_SCENE_GLTF20_GLB capability as defined in clause 5.8 shall be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2.

-	and if the device is a device type as defined in TS 26.119 [Y], clause 10, the 26143_SCENE_GLTF20_AR and the 26143_SCENE_GLTF20_GLB_AR capability as defined in clause 5.8 shall be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2.

If a processor for the media type text/html is supported, the 26143_PRESENTATION_HTML5 capability as defined in clause 5.9 should be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2. The media formats shall be restricted to the capabilities defined in this clause.

# =====  CHANGE =====

## 6.3	Baseline MMBP Generator Profile

### 6.3.1	Overview

The baseline MMBP Generator profile is aligned with TS 26.140 [32].

It primarily addresses the ability for users to generate content, for example on a mobile device.

The container format is based on IETF RFC 2045 [23] as the format for the MMBPs. Offering of alternative content is the container is permitted. In addition, the profile permits to encapsulate encapsulation of real-time video into the 3GP file format using the baseline profile. The container does not support external bodies, i.e. the MMBP is expected to be delivered as a single message. For details on the container format requirements, refer to clause 6.3.2.

The media types supported by this profile are basic text, audio/speech, images, video, and text.

Content conforming to the baseline MMBP generator profile may include media types that are not explicitly supported by the media capabilities as defined in clause 6.3.3. Receivers are expected to ignore non-recognized media types. However, based on the container requirements, ignoring media types may results in specific processing requirements, for example pick an alternative, or ignore the entire MMBP.

Additional packaging requirements and recommendations are provided in clause 6.3.4.

Content generated to be compatible for this profile as well as generators expose their capabilities should use the URN identifier "urn:3GPP:26143:18:baseline-mmbp-generator".

### 6.3.2	Container Format

The following capabilities for the container format as defined in clause 5.2.2 shall be supported:

-	26143_CONTAINER_RFC2046_SINGLE_GEN

-	26143_CONTAINER_RFC2046_MIXED_GEN

-	26143_CONTAINER_MP4_3GP9_GEN

The following capabilities for the container format as defined in clause 5.2 should be supported:

-	26143_CONTAINER_RFC2046_PARALLEL_GEN

-	26143_CONTAINER_RFC2046_ALTERNATIVE_GEN

### 6.3.3	Media Types

If the transmission of images is supported, then the following applies:

-	the 26143_IMAGE_ENC_JPEG capability as defined in clause 5.4.2 shall be supported.

If the transmission of speech is supported, then the following applies:

-	the 26143_AUDIO_ENC_EVS capability as defined in clause 5.5.2 shall be supported.

-	the 26143_AUDIO_ENC_AMR-WB capability as defined in clause 5.5.2 should be supported.

-	the 26143_AUDIO_ENC_AMR capability as defined in clause 5.5.2 may be supported.

If the transmission of audio is supported, then the following applies:

-	the 26143_AUDIO_ENC_EAAC+ capability as defined in clause 5.5.2 shall be supported.

-	the 26143_AUDIO_ENC_XHE-AAC capability as defined in clause 5.5.2 should be supported.

If the transmission of video is supported, then the following applies:

-	the 26143_VIDEO_ENC_HEVC-FullHD capability as defined in clause 5.6.2 shall be supported.

If the transmission of timed text is supported, then the following applies:

-	the 26143_TT_ENC_3GPP capability as defined in clause 5.7.2 shall be supported.

### 6.3.4	Packaging Requirements and Recommendations

For content generated by this baseline generator, multipart/parallel shall not be present in an MMBP on the same level if a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9'is present and contains more than on track.

NOTE: 	This avoids the necessity to playback multiple tracks included in a single file with tracks provided to the multipart MMBP in the parallel structure.

When generating a speech message, the MMBP Generator shall at least include an EVS Content together with potential alternatives.

# When generating an Audio message, the MMBP Generator shall at least include an eAAC+ stereo Content together with potential alternatives.

# =====  CHANGE =====

Annex A (informative):
Registration Information

# A.1	3GPP Registered URIs

The clause documents the registered URIs in this specification following the process in https://www.3gpp.org/3gpp-groups/core-network-terminals-ct/ct-wg1/uniform-resource-identifier-uri-list,

Table A-1 lists all registered URN values as well as

-	a brief description of its functionality;

-	a reference to the specification or other publicly available document (if any) containing the definition;

-	the name and email address of the person making the application; and

-	any supplementary information considered necessary to support the application.

Table A-1: 3GPP Registered URNs

| URN | Description | Reference | Contact | Remarks |
| --- | --- | --- | --- | --- |
| urn:3GPP: 26143:18:baseline-mmbp-player | Media Messaging Baseline MMBP Player Profile | TS 26.143, clause 6.2.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP: 26143:18:baseline-mmbp-generator | Media Messaging Baseline MMBP Generator Profile | TS 26.143, clause 6.3.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |