| 3GPP TS 26.565 V1.1.0 (2024-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Split Rendering Media Service Enabler; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	General	8

4.1	Overview	8

4.2	Typical Use Cases	8

5	Reference Architecture and Procedures	8

5.1	Reference Architecture	8

5.1.1	Introduction	8

5.1.2	Client Architecture	9

5.1.3	End-to-End Architecture	9

5.1.5	User Plane Architecture	10

5.2	Procedures and Call Flows	11

5.2.1	Call flow for Split Rendering instance discovery	11

5.2.1.1	Call flow for edge server and split rendering session setup	11

5.2.1.2	Client-driven procedures and call flows	11

5.2.2	Call flow for Split Rendering session setup	12

6	Prerequisites	14

6.1	Requirements on 5G System	14

6.2	Requirements on Device APIs and Functionality	14

7	Network Support	14

7.1	Overview	14

7.2	Provisioning	14

7.3	Dynamic Policy and Network Assistance	14

7.4	Edge Resources	15

7.5	Metrics and Consumption Reporting	15

8	Split Rendering User Plane8.1 General	15

8.2	Split Rendering Signalling Protocols	16

8.3	Split Rendering Formats for Media and Metadata	17

8.3.1	General	17

8.3.2	Metadata Formats	17

8.3.2.1	General	17

8.3.2.2	Pose Format	17

8.3.2.3	Action Format	19

8.3.3	Metadata Data Channel Message Format	20

8.4	Split Rendering Formats for Session Setup and Negotiation	20

8.4.1	General	20

8.4.2	Split Rendering Configuration Format	21

8.4.2.1	Introduction	21

8.4.2.2	Split Rendering Configuration Format	21

8.4.3	Output Format Description	23

8.5	Split Rendering Transport Protocols	23

9	Split Rendering Client	23

9.1	Functionality	23

9.2	Client API	24

9.3	Split Rendering Metrics	24

9.3.1	General	24

9.3.2	QoE Metrics Formats	25

9.3.2.1	Timing Information Format	25

9.3.2.2	Latency metrics	25

10	Security and Privacy Aspects	27

10.1	Security	27

10.2	Privacy	27

Annex A (informative):	Implementation Guidelines	28

A.1	Guidelines for Application Developers	28

A.2	Guidelines for Split Rendering MSE Implementers	28

A.2.1	Guidelines for implementers of the Split Rendering Server	28

A.3	Conformance Testing	28

Annex B (normative):	IDL Definition of Client API	29

Annex C (normative):	Split Rendering Profiles	30

C.1	Pixel Streaming Profile	30

C.1	Introduction	30

C.2	2D Pixel Streaming Profile	30

C.2.1	Introduction	30

C.2.2	SRC Capabilities	30

C.2.2.1	Overview	30

C.2.2.2	Media Capabilities	30

C.2.2.3	Metadata Formats	30

C.2.3	SRS Capabilities	31

C.2.3.1	Overview	31

C.2.3.2	Video encoding	31

C.2.3.3	Audio and Speech encoding	31

C.2.3.4	Video decoding	31

C.2.3.5	Audio and Speech decoding	31

C.2.3.6	Metadata Formats	31

C.3	3D Pixel Streaming Profile	31

C.3.1	Introduction	31

C.3.2	SRC Capabilities	31

C.3.2.1	Overview	31

C.3.2.2	Media Capabilities	32

C.3.2.3	Metadata Formats	32

C.3.3	SRS Capabilities	32

C.3.3.1	Overview	32

C.3.3.2	Video encoding	32

C.3.3.3	Audio and Speech encoding	32

C.3.3.4	Video decoding	32

C.3.3.5	Audio and Speech decoding	32

C.3.3.6	Metadata Formats	32

C.4	Description of the Rendering Format for Pixel Streaming Profiles	33

C.4.1	General	33

C.4.2	3D Pixel Streaming Profile-specific glTF Extension	33

Annex X (informative):	Change history	37

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This specification defines a media service enabler for split rendering in the 5G system.

# 1	Scope

The present document defines a Media Service Enabler for Split Rendering according to the guidelines of TR26.857 [1]. The Split Rendering MSE covers functionality on the UE and on the Media AS. It also defines an API that is exposed to application developers on the UE to start and manage split rendering sessions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 26.857: "5G Media Service Enablers".

[2]		ISO/IEC 12113:2022, Information technology, Runtime 3D asset delivery format, Khronos glTF 2.0

[3]		ISO/IEC 23090-14: Information technology — Coded representation of immersive media — Part 14: Scene Description for MPEG Media.

[4]	3GPP TS 26.119, Media Capabilities for Augmented Reality

[5]	3GPP TS 26.506, 5G Real-time Media Communication Architecture (Stage 2)

[6]	3GPP TS 26.113, Real-Time Media Communication; Protocols and APIs

[7]	3GPP TS 26.512, 5G Media Streaming (5GMS); Protocols

[8]	3GPP TS 26.522, 5G Real-time Media Transport Protocol Configurations

[9]	3GPP TS 26.510, Media Delivery: interactions and APIs for provisioning and media session handling

[10]	Khronos, The OpenXR API, https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html

[11]	W3C, WebXR Device API, WebXR Device API (immersive-web.github.io)

[12]		Khronos, WebGL Specification 1.0, WebGL Specification (khronos.org)

[13]		W3C, Web Audio API, Web Audio API (w3.org)

[14]	3GPP TS23.501, System architecture for the 5G System (5GS).

[15]	3GPP TS23.503, 5G; Policy and charging control framework for the 5G System (5GS).

[16]	3GPP TS26.857, 5G Media Service Enablers.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AF		Application Function

AS		Application Server

MAF	Media Access Function

MSH		Media Session Handler

5G-RTC		5G Real-Time Communication

SR 		Split Rendering

SRC		Split Rendering Client

SRS		Split Rendering Server

UE		User Equipment

# 4	General

## 4.1	Overview

The Split Rendering Media Service Enabler collects a set of 5G media functions to build a media service enabler that targets application developers, network operators, and application service providers, to enable the realization of split rendered applications.

The interfaces, formats, protocols, and APIs are either referenced or defined in this specification. This will allow for interoperability between multiple vendor implementations.

This specification targets primarily XR applications. However, it is not limited to XR applications and may be used for rendering for 2D displays.

## 4.2	Typical Use Cases

A typical use case for the split rendering MSE is immersive gaming. In this use case, the UE benefits from invoking split rendering by avoiding the download of the game to the phone and getting high quality graphics from edge rendering.

Another use case that can benefit from split rendering is immersive communication, where users gather in a shared space and interact with each other and with the environment. Users may be represented by sophisticated Avatars and as the number of users increases the rendering will become more complex.

# 5	Reference Architecture and Procedures

## 5.1	Reference Architecture

### 5.1.1	Introduction

In this clause, different variants of the reference architecture for the split rendering MSE are defined, each representing a different perspective and level of details.

The following functions are introduced:

- Split-Rendering Client (SRC): This function is responsible to acquire the UE media capabilities and negotiates with the RTC AS to agree on the split-rendering process at the RTC AS.

- Split-Rendering Server (SRS): This function is responsible for negotiation of SR session with SRC, monitoring the server’s edge resource usage, and managing/running the split rendering process.

- Application Function (AF): responsible for provisioning, QoS allocation, and edge resource discovery.

- Application Service Provider: The application provider that offers the service.

- Application: The application running on UE

- Media Session Handler (MSH): is the entity on UE that is responsible for the control plane communication with the AF.

### 5.1.2	Client Architecture

The client architectural breakdown is based on the client architecture in TS 26.119 [4] clause 5.1. The figure depicting the client architecture is replicated here as Figure 5.1.2-1 for convenience.

Figure 5.1.2-1 - XR Baseline terminal architecture

The split rendering client consists of the following components:

- The Media Access Functions: allow for fetching and processing of the pre-rendered media in preparation of final display. The MAF is also responsible for the carriage of any metadata or local media to the split rendering server.

- The scene manager and thin Presentation Engine: is responsible for the negotiation of the split rendering session and the parsing of the description of the rendered media as provided by the SR server. It is also responsible for setting up and managing the XR session with the XR runtime.

- The XR source management is responsible for gathering timed metadata such as pose and action information and sending it to the SR server.

### 5.1.3	End-to-End Architecture

Figure 5.1-3 – Split management architecture

As shown in Figure 5.1.3:

1. The 5G Application Providers (AP) provisions the split-rendering through RTC-1.

2. In the use cases in which the AP is involved in the media delivery, the RTC-2 interface is used for this purpose.

3.The communication between AF and SRS is through RTC-3.  This interface is out of the scope of this document. This interface may for instance include the EDGE-3 interface.

4. The signaling as well as the media delivery between SRC and SRS is though RTC-4.

5. The AF may provide the split-rendering information to the Media Session Handler defined by RTC-5, defined in  TS26.506 [5].

6. SRC in the UE discovers the application through RTC-6 and handles the XR runtime.

7. The SRC discovers the client media capabilities through the RTC-7 interface. This interface is out of the scope of this document.

8. The 5G Application and AP interact through RTC-8-8. This interface is out of the scope of this document.

### 5.1.5	User Plane Architecture

Figure 5.1.5-1 depicts the user plane architecture for split rendering.

Figure 5.1.5-1 – User Plane Architecture for Split management architecture

The SR interfaces are considered to be specializations of their parent RTC interfaces as defined in TS26.506.

In the context of split rendering, the SR-4 interface is further classified as SR-4s and SR-4m sub-interfaces. The SR-4s interface covers all user-plane signaling, including WebRTC and ICE signaling. The SR-4m serves for media and metadata exchange between the split rendering client and the split rendering server.

## 5.2	Procedures and Call Flows

### 5.2.1	Call flow for Split Rendering instance discovery

#### 5.2.1.1	Call flow for edge server and split rendering session setup

Figure 5.2.1.1-1 demonstrates a general call flow for split-rendering.

Figure 5.2.1-1: High-level call flow for split-rendering

Steps:

- In this optional step, the Application Provider requests and sets up the edge server(s) used for the split-rendering as described in TS 26.506 clauses 6.1 or 6.2. The Application provider may use any other method to allocation edge servers, or leave it to the MNO to set up appropriate edge servers to run the split-rendering process.

- The Application Provider provisions the split-rendering session using SR-1 and SR-3, as defined in call flow of clauses 5.2.1.1. If the edge servers were provisioned in step 1, the edge servers ids are provided in this session to employ them for split-rendering.

NOTE: In the case of the client-driven edge management (TS 26.501 8.1), only the client-driven split-rendering (5.2.1.1) is applicable.

- The split-rendering session is set up according to clause 5.2.2.

#### 5.2.1.2	Client-driven procedures and call flows

Figure 5.2.1.2-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1.2-1: High-level call flow for initiating a split

Steps:

- The Application Service Provider requests the SRF the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The Application requests a split of the client media functions from the SRC.

- The SRC inquires the Media Session Handler about the client’s media capabilities.

- The SRC and SRS negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRS starts the split rendering process.

- The SRC provides the session information via the RTC-6 interface and requests the application of dynamic policy and subscription to network assistance.

- The SRC establishes the WebRTC session.

9. The SRC informs the application that the split-rendering on edge is running.

10. The SRC sends uplink metadata, such as pose and action information.

11. The SRS sends the rendered media to the SRC.

### 5.2.2	Call flow for Split Rendering session setup

The split rendering operation can be described by the as depicted in the call flow in the Figure 5.2.2-1.

Figure 5.2.2- 1 High-level call flow for split rendering session setup and operation

The steps are:

- The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose.

- In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering server.

- The split rendering server uses that information to render the frame.

- The rendered frame is encoded and streamed down to the MAF.

# 6	Prerequisites

## 6.1	General

The below section provides guidance on the pre-requisites on the 5G system and device APIs in order to host and run a split rendering session.

## 6.2	Pre-requisites on 5G System

Pre-requisites document what is expected to be available either from the 5G System (i.e. certain functionalities of the 5G System) or from implementation (for example functions available on the device). These pre-requisites may be considered to be part of the specification (as reference to an external specification), but it is important to identify them separately in order to clearly demarcate the boundaries of the split rendering MSE with respect to other functions. Pre-requisites include, but are not limited to:

- The split rendering session is expected to operate in the 5G system as specified in [14] that supports functionalities and procedures to configure the quality of service (QoS) and charging information and functionality, which may be used by split rendering sessions.

- The split rendering session is expected to operate within the 5G system as specified in [14], which supports functionalities and procedures to configure policy and charging control (PCC) information as specified in [15]. Split rendering sessions may benefit from these procedures.

- The split rendering server functionality may operate within the 5G system as specified in |14], which supports the discovery of and access to edge resources. The edge functionality may be used to deploy split rendering server functionality.

## 6.2	Pre-requisites on Device APIs and Functionality

The following assumptions for the split rendering client are made:

- The SRC may have access to an XR runtime through a well-defined API such as the OpenXR [10] or WebXR [11] APIs.

- The SRC has access to 3D graphics library, such as. WebGL [12], and to an audio rendering engine, such as WebAudio [13].

- The split rendering server functionality should be within the 5G system specified as per [14] that supports the Application Provider with a set of functions that can be easily accessed in the same way that device functions are accessed today, namely through well-defined device APIs. The Application Provider can also use regular IP connectivity to operate its application.

# 7	Network Support

## 7.1	Overview

The Split Rendering MSE stands to benefit from the several procedures that the network offers. These include but are not limited to:

- Dynamic QoS and charging

- Edge resources

- Consumption and metrics reporting

- Configuration Information

In this clause, the control plane procedures that are relevant for split rendering and their usage are described.

## Provisioning

An Application Provider that wishes to offer applications using split rendering shall use the procedures defined in TS 26.113 [6] clause 6 and data models defined in TS 26.510 [9] clause 8 to create a Provisioning Session with the RTC AF.

The ProvisioningSessionType shall be set to “BIDIRECTIONAL”.

The aspId shall be configured and shall be a unique identifier for the Application Service Provider that offers split rendering.

The externalApplicationId shall be a URN that uniquely identifies the application and shall be terminated by the sub-string “+3gpp-sr”. An examples is as follows: “urn:com:example:game+3gpp-sr”.

## 7.3	Dynamic Policy and Network Assistance

Dynamic policy and network assistance may be provisioned by the Application Provider with the RTC AF. The allowed dynamic policies for the split rendering sessions of the application provider are communicated to the MSH in the client using the Configuration procedure.

Upon the creation of a new split rendering session and upon eligibility, the MSH shall use the Dynamic Policy API to request the allocation of network resources and charging policy to the session based on the information in the corresponding Provisioning session.

A policy template that is provisioned for split rendering should be associated with the split rendering configuration.

The MSH And the WebRTC Signaling Server shall support the dynamic policy API as defined in TS 26.113 [6].

The Application Provider may provision support for PDU Set marking. The SRS shall support the PDU Set marking and should support the End of Burst marking for the RTP streams that are generated by the Split Rendering Server.

## 7.4	Edge Resources

A split rendering application may use the procedures defined in TS26.512 [7] clause 7.10 to define an edge resource configuration to be used for split-rendering sessions. In this case:

- The eligibilityCriteria shall be present and shall have appRequest set to true.

- The easRequirements shall indicate “SR” as the easType and shall include “3gpp-sr” among the easFeatures. The serviceKpi shall be present and indicate the SRS processing and networking capabilities and requirements.

## 7.5	Metrics and Consumption Reporting

The Application Provider may use the Provisioning procedure to configure the collection of split rendering metrics and logging of consumption statistics. When present, this information shall be passed to the MSH using the Configuration procedure.

The SRC shall collect and report the data for a split rendering session that matches the criteria for metrics and/or consumption reporting as indicated by the configuration procedure.

# 8	Split Rendering User Plane8.1	General

The user plane for split rendering covers all traffic between the SRC and SRS, or the SRC and any other RTC AS. The common formats for split rendering are defined in this clause. Split rendering profiles may define additional user plane formats.

This clause illustrates the protocol stack for the User plane transport related to the signalling as well as the media delivery between SRC and SRS though RTC-4.

Figure 8.1-1 Split rendering protocol Stack

## 8.2	Split Rendering Signalling Protocols

Both SRC and SRS shall support the SWAP protocol as defined in TS 26.113 [6] clause 12.2.

The SWAP protocol allows for the definition of application-specific messages.

The following application-specific messages shall be supported for split rendering:

- The configuration message carries the split rendering configuration information from the SRC to the SRS. It shall be identified by the type “urn:3gpp:sr-mse:sr-configuration” and the object shall be formatted according to clause 8.4.2.2.

- The rendering description message carries the description of the split rendered media from the SRS to SRC. It shall be identified by the type “urn:3gpp:sr-mse:sr-description” and the object shall be formatted according to clause 8.4.3. The rendering description message provides the semantics of the media that is delivered over WebRTC from the SRS to SRC.

The SWAP message exchange for the establishment of a split rendering session is depicted by the following call flow diagram:

Pre-requisites:

- The SRC has discovered the identifier of the SRS that it will use for its split rendering session.

- The SRC has retrieved the address of the SWAP server as part of the configuration.

The stpes are as follows:

- The SRC sends the configuration message as an application-specific SWAP message to the SWAP server. It provides the identifier of the target SRS as a matching criteria.

- The SWAP server uses the provided matching criteria to locate the SRS.

- The SWAP server forwards the configuration message to the target SRS.

- The SWAP server confirms the successful forwarding of the message to the SRC

- The SRS processes the SR configuration message. It may for instance verify application and resource availablity, launch the application, configure its rendering, and create a rendering description.

- The SRS sends the rendering description message as an application-specific SWAP message to the SWAP server.

- The SWAP server forwards the message to the SRC.

- The SWAP server acknowledges the successful forwarding of the message to the SRS.

- The SRC processes the rendering description and identifies the required data channel and media sessions.

- SRC sends a connect message with the SDP offer to the SRS. The offer reflects the negotiated media and data channel streams.

- The SWAP server acknowledges the forwarding of the message to the SRS

- The SRS replies with an accept message that includes the SDP answer. The SDP answer reflects the information that was provided in the split rendering description.

- The SWAP server acknowledges the forwarding of the message to the SRC

## 8.3	Split Rendering Formats for Media and Metadata

### 8.3.1	General

This clause defines media and metadata formats that are common to one or more split rendering profiles.

### 8.3.2	Metadata Formats

#### 8.3.2.1	General

Both SRC and SRS shall support the usage of the WebRTC data channel for the exchange of split rendering metadata. The WebRTC data channel shall declare “3gpp-sr” as the data channel sub-protocol. The message content format depends on the type of the message. The data channel sub-protocol is defined in clause 8.3.3.

Message types shall be unique identifiers in the URN format. This clause defines a set of message types and their formats. The messages are derived from the OpenXR API to ensure smooth operation with AR devices that support OpenXR. In case other XR APIs are used, mapping the message payload to the appropriate XR API structures shall be performed by the split rendering client.

Editor’s Note: This following sections will potentially reference the corresponding formats in TS 26.119 [4].

#### 8.3.2.2	Pose Format

#### The pose format that is used by all split rendering profiles defined by this specification shall comply with the format defined in TS 26.119 [4] clause 6.2.2. The pose information shall be carried as part of the data channel messaging mechanism defined in clause 8.3.3 and shall be provided in JSON format. The message type shall be “urn:3gpp:split-rendering:v1:pose”.

#### 8.3.2.3	Action Format

The action information format that is used by all split rendering profiles defined by this specification shall comply with the format defined in TS 26.119 [4] clause 6.2.3. The action information shall be carried as part of the data channel messaging mechanism defined in clause 8.3.3 and shall be provided in JSON format. The message type shall be “urn:3gpp:split-rendering:v1:action”.

### Metadata Data Channel Message Format

For the carriage of metadata defined in clause 8.2, such as pose and action information, the SRS and SRC shall use the WebRTC data channel. The data channel sub-protocol shall be identified as “3gpp-sr-metadata”, which shall be included in the dcmap attribute of the SDP.

The transmission order for the data channel shall be set to in-order and the transmission reliability shall be set to reliable.

The split rendering metadata message format shall be set to text-based and the messages shall be UTF-8 encoded JSON messages.

A data channel message may carry one or more split rendering messages as defined in Table 8.2.3-1.

Table 8.2.3-1 Split Rendering Metadata Messages Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| messages | Array(Message) | 1..n | A list of split rendering metadata messages. Each message shall be formatted according to the Message data type as defined in Table |

Each split rendering message shall follow the format specified in Table 8.2.3-2.

Table 8.2.3-2 Split Rendering Metadata Message Data Type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | An unique identifier of the message in the scope of the data channel session. |
| Type | string | 1..1 | A urn that identifies the message type. |
| Message | object | 1..1 | The message content depends on the message type. |
| sendingAtTime (ref. T1’) | number | 0..1 | The time when the split rendering metadata message is transmitted from the split rendering client to the split rendering server. |

## 8.4	Split Rendering Formats for Session Setup and Negotiation

### 8.4.1	General

In Figure 5.2.1-1 and 5.2.1-2, in step 5, the negotiation between the SRC and SRS for the split-rendering configuration takes place. The detailed call flow for such a negotiation between the SRC and the SRS may vary. Depending upon the split rendering profile, the negotiation between the SRC and the SRS may be straight forward or go back and forth.

In the simplest case, the SRC provides SRS the capabilities of the device and if SRS can accommodate the split-rendering processing that addresses the device’s needs and capabilities, it confirms by providing a description of the output format. In such scheme, the SRS is responsible to make the decision and no back-and-forth negotiation occurs.

### 8.4.2	Split Rendering Configuration Format

#### 8.4.2.1	Introduction

The Split Rendering client establishes an XR session locally based on the device configuration and user selection. The SR client defines the view configuration (e.g. mono or stereo views), the projection format (such as projection, equirectangular, quad, or cubemap), the swap chain image configuration, etc.

In addition, XR space and action configurations are negotiated between the SR client and server. This includes defining common XR spaces and defining and selecting actions and action sets.

The format is extensible to support the exchange of additional/future configuration information.

#### 8.4.2.2	Split Rendering Configuration Format

The session configuration information shall be in JSON format. It shall have the following format:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| renderingFlags | Array(SR_CONFIG_FLAGS) | 0..1 | Provides a set of flags to activate/deactivate selected rendering functions. The defined SR_CONFIG_FLAGS are: - FLAG_ALPHA_BLENDING - FLAG_DEPTH_COMPOSITION - FLAG_EYE_GAZE_TRACKING |
| splitRenderingProfile | array(URI) | 0..1 | A list of supported split-rendering profile identifiers on the UE. The profile identifiers are listed in Annex C for each profile. |
| deviceCapabilities | Object | 0..1 | Device capabilities as defined in TS 26.119 [4], clause 6.2.5. |
| spaceConfiguration | Object | 0..1 | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |
| referenceSpaces | Array | 0..1 | An array of reference spaces and their identifiers. |
| Id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| refSpace | enum | 1..1 | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE. |
| actionSpaces | Array | 0..1 | An array of action spaces that need to be defined by the split rendering client in the XR session. |
| Id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| actionId | number | 1..1 | Provides the unique identifier of the action. |
| subactionPath | string | 1..1 | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |
| initialPose | Pose | 0..1 | Provides the initial pose of the new XR space’s origin. |
| viewConfiguration | Object | 0..1 | Conveys the view configuration that is configured for the XR session. |
| Type | Enum | 1..1 | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |
| Width | number | 1..1 | The recommended width of the swapchain image. |
| Height | number | 1..1 | The recommended height of the swapchain image. |
| compositionLayer | string | 1..1 | An identifier of the selected composition layer. |
| minPoseInterval | number | 0..1 | The minimum time interval between two consecutive pose information instances sent to the network, in milliseconds. |
| Fovs | Array | 0..1 | An array that provides a list of the field of views (FoV) associated with each view. |
| Fov | Object | 1..n | Indicates the four sides of the field of view used for the projection of the corresponding XR view. The number of views n is determined by the type enum of the viewConfiguration. Both the viewPoses in the Pose Format and the fovs arrays shall be ordered in a consistent way (i.e., a same index can be used to retrieve the view pose and the related FoV information). |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |
| environmentBlendMode | enum | 1..1 | The type indicates the environment blend mode configuration. Defined values are OPAQUE, ADDITIVE and ALPHA_BLEND. Other values may be added. |
| actionConfiguration | Array | 0..1 | This contains a list of the actions that are to be defined by the SR client. |
| Action | Object | 1..n | A definition of a single action object. |
| Id | number | 1..1 | A unique identifier of the action. |
| actionType | enum | 1..1 | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |
| subactionPaths | string | 1..n | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |
| extraConfigurations | Object | 0..1 | A placeholder for addition configuration information. |

### Output Format Description

The output format description depends on the split rendering profile and is defined by the split rendering profile in Annex C.

## Split Rendering Transport Protocols

Split Rendering shall use WebRTC for the real-time transport of the rendered media. The RTP restrictions for WebRTC as specified in RFC8834 shall apply. The usage of the WebRTC data channel shall be in accordance with RFC8831.

Editor’s Note: applicable guidelines for the usage of the PDU Set Marking are pending completion of TS 26.522 [8].

# 9	Split Rendering Client

## 9.1	Functionality

The Split Rendering Client (SRC) is a function that runs on the UE to provide split rendering functionality to applications. The SRC is designed to be offered as an SDK to application developers. The SRC abstracts the details of the split rendering operation and provides a simple to use API to the application to facilitate the usage of split rendering.

The SRC performs the following functions:

- Creates and manages the XR session,

- Discovers and selects a split rendering server (SRS) in the network,

- Establishes a split rendering session with the SRS,

- Communicates the necessary information about the session to the MSH/AF to benefit from dynamic policy, network assistance, consumption reporting, etc.

- Operates the rendering loop on the UE.

## 9.2	Client API

As described in clause 5.1.3, the SRC exposes an API over RTC-7 interface to the application. The SRC defines the following interface:

| Method | Parameters |  | State after Success | Description |
| --- | --- | --- | --- | --- |
|  | in | out |  |  |
| SplitRenderer() | - appId - aspId? - settings? | - session handle | STATE_READY | Creates a SplitRenderer object, which can subsequently be used to connect to an SRS and perform split rendering. |
| connect() | - settings? - criteria? | - connection handler | STATE_CONNECTED | Instructs the SRC to discover and connect to an SRS. |
| request() | - media configuration | -confirmation, notifications | N/A | Used by the application to request the application of dynamic policy to the split rendering session. The SRC will also request the MSH to subscribe and relay any notifications about network assistance associated with this session. |
| disconnect() | - reason? |  | STATE_DISCONNECTED | Terminates the connection to the SRS. |
| getMetrics() | - metrics | - metrics report | N/A | Retrieves a set of metric reports for the split rendering session that describe the quality of experience of the session. |

The application is able to subscribe to events related to the split rendering session by setting the corresponding event handler. The supported events are:

- State change: the state of the SR session has changed

- Error: an error has occurred during the split rendering session. The error is not severe enough to cause a state change to the STATE_ERROR state.

- Quality change: the SRC has observed a change in the quality of the split rendering session. This may involve one or more SR metrics.

The Settings object shall contain the following information:

- Information about the desired rendering, e.g. choose to render on 2D device or on one of the available connected XR devices.

The criteria object may contain the following information:

- Requirements for latency and bitrate that are different from the ones in the provisioning,

- KPIs for the SRS instance, such as its graphics capabilities or current load.

## 9.3	Split Rendering Metrics

### 9.3.1	General

This clause defines a set of metrics that are relevant to the operation of a split rendering session.

### 9.3.2	QoE Metrics Formats

#### 9.3.2.1	Timing Information Format

The timing information associated with the rendered frame is transmitted in the RTCP report block formats. This timing information is listed in the Table 8.3.Y.3-1.

The SRS may use the “QoE timing information” RTCP Extended Reports messages to transmit the timing information required for measuring the QoE metrics to an SRC. The RTCP report block format for transmitting the QoE timing information is specified in TS 26.522 [8]. SDP signalling required for negotiating the transmission of QoE metrics between the UE and the SRS is documented in TS 26.522 [8].

The latency metrics that use the timing information defined in Table 9.3.2-1 are detailed in the section 9.3.2.2.

Table 9.3.2-1: Timing information in the RTCP block formats.

| Name | Description |
| --- | --- |
| estimatedAtTime (ref. T1) | This wall clock time is defined in TS 26.119 [4] - Pose Format. This time is sent from the split rendering client. This time is then received by the split rendering server and sent back to the split rendering client with the associated media frame. |
| sendingAtTime (ref. T1’) | This time is defined in Table 8.2.3-2 - Split Rendering Metadata Message Data Type This time is sent from the split rendering client. This time is then received by the split rendering server and sent back to the split rendering client with the associated media frame. |
| startToRenderAtTime (ref. T3) | The time when the renderer in the Split Rendering Server starts to render the associated media frame. |
| sceneUpdateTime (ref. T6) | The time when the Scene Manager starts to update the 3D scene graph according to the viewer pose and the user actions. |
| serverTransmitTime (ref. T5) | The time when the encoded rendered frame is transmitted from the split rendering server to the split rendering client. |

#### 9.3.2.2	Latency metrics

To enable good XR experiences, it is relevant to monitor latencies such as the motion-to-photon and the pose-to-render-to-photon.

Beyond the sense of presence and immersiveness, the age of the content and user interaction delay are of the uttermost importance for immersive and non-immersive interactive experiences, i.e., experiences for which the user interaction with the scene impacts the content of scene (such as online gaming).

Table 9.3.2-1 provides timing information collected to compute the latency metrics at the split rendering client or split rendering server endpoint.

Table 9.3.2-1: Timing information for latency metrics

| Source endpoint | Timing information | Definition |
| --- | --- | --- |
| Split Rendering Client | estimatedAtTime (ref. T1) | This time is expressed in wall clock time (refer to Table 9.3.2-1). |
| Split Rendering Client | lastChangeTime | The time the user action is made. It corresponds to the lastChangeTime field defined in the action format in Table 9.3.2-1. This time is expressed in XR system time clock. |
| Split Rendering Server | sceneUpdateTime (ref. T6) | This time is a NTP timestamp format (refer to Table 9.3.2-1). |
| Split Rendering Server | startToRenderAtTime (ref. T3) | This time is expressed in wall clock time (refer to Table 9.3.2-1). |
| Split Rendering Client | actualDisplayTime (ref. T2.actual) | The actual display time of the rendered frame in the swapchain. The estimation of the actual display time is available through the XR runtime. |
| Split Rendering Client | sendingAtTime (ref. T1’) | This time is expressed in wall clock time (refer to Table 9.3.2-1). |
| Split Rendering Server | serverTransmitTime (ref. T5) | This time is expressed in wall clock time (refer to Table 9.3.2-1). |
| Split Rendering Client | receptionTime | The time when the data is received by the split rendering client. |

The latency metrics are specified in Table 9.3.2-2. Latency calculation formulas are defined using the timing information defined in Table 9.3.2-1.

Table 9.3.2-2: Latency metrics

| Latency metric | Description |
| --- | --- |
| poseToRenderToPhoton | The time duration, in units of milliseconds, between the time to provide the pose information from the XR runtime to the renderer (the renderer uses this pose to generate the rendered frame) and the display time of the rendered frame. It can be computed as follows: actualDisplayTime – estimatedAtTime |
| renderToPhoton | The time duration, in units of milliseconds, between the start of the rendering by the Presentation Engine and the display time of the rendered frame. It can be computed as follows: actualDisplayTime – startToRenderAtTime (NOTE 1) |
| roundtripInteractionDelay | The time duration, in units of milliseconds, between the time a user action is initiated and the time the action is presented to the user. It can be computed as follows: actualDisplayTime – lastChangeTime |
| userInteractionDelay | The time duration, in units of milliseconds, between the time a user action is initiated and the time the action is taken into account by the content creation engine in the scene manager. It can be computed as follows: sceneUpdateTime – lastChangeTime (NOTE 1) |
| ageOfContent | The time duration, in units of milliseconds, between the time the content is created in the scene by the Scene Manager and the time it is presented to the user. It can be computed as follows: actualDisplayTime – sceneUpdateTime (NOTE 1) |
| sceneUpdateDelay | The time duration, in units of milliseconds, spent by the Scene Manager to update the scene graph. It can be computed as follows: startToRenderAtTime – sceneUpdateTime |
| metadataDelay | The time duration, in units of milliseconds, between the time the split rendering metadata message is sent from the split rendering client and the time the split rendering server start to render using that metadata. It can be computed as follows: startToRenderAtTime – sendingAtTime |
| dataFrameDelay | The time duration, in units of milliseconds, spent to transmit the media rendered frame from the split rendering server to the split rendering client. It can be computed as follows: receptionTime – serverTransmitTime |
| NOTE 1: for the latency metrics computation, the timing information mentioned above need to be converted to a single time format (e.g., Wall clock time). |  |

# 10	Security and Privacy Aspects

## 10.1	Security

Signaling for session establishment and exchange of application-specific messages shall use a secure transport channel based on WebSockets as defined in TS 26.113 [6].

Media transport shall be secured by the usage of WebRTC.

## 10.2	Privacy

Users of the split rendering MSE shall be aware that the application data and traffic are fully accessible to the SRS. The SRC shall ensure that the SRS used is trusted by the application provider, for example through the validation of the SRS’s X.509 certificates.

Annex A (informative):
Implementation Guidelines

## A.1	Guidelines for Application Developers

## A.2	Guidelines for Split Rendering MSE Implementers

### A.2.1	Guidelines for implementers of the Split Rendering Server

If the use of eye gaze tracking is activated, the SRS may use this confidence information to perform foveated rendering. In foveated rendering, areas of the picture are encoded with a higher SNR quality than other areas, to match the user’s current gaze.

With gaze predictions, the SRS should create an importance map for the picture based on the confidence values associated with the gaze predictions. Additionally, the SRS may also use other information to produce the importance map, such as content Regions of Interest. A low confidence score may indicate that the estimation on the device is not adequate. In this case the server can try to re-estimate the pose prediction prior to rendering.

This importance map is passed to the encoder to properly allocate bits for the encoding of the picture.

## A.3	Conformance Testing

Annex B (normative):
IDL Definition of Client API

The Split Rendering Client API is defined using the IDL syntax (according to ISO/IEC 19516) as follows:

interface SplitRenderer { readonly attribute SRState state; attribute EventHandler onstatechange; attribute EventHandler onerror; attribute EventHandler onqualitychange; void SplitRenderer(in string application_id, in string aspId, in map settings); void connect(in map settings, in List criteria); void disconnect(string reason); Metrics getMetrics(sequence<string> metrics); };

Annex C (normative):
Split Rendering Profiles

## C.1	Pixel Streaming Profile

### C.1.1	Introduction

This Annex defines split rendering profiles to define requirements for SRC and SRS for different scenarios. At this stage the following two profiles are defined:

- 2D Pixel Streaming Profile in clause C.2 to support split rendering to 2D screens, devices of type 3 in TS 26.119 [4].

- 3D Pixel Streaming Profile in clause C.3 to support split rendering to devices of type 1, 2, and 4 in TS 26.119 [4].

### C.1.2	2D Pixel Streaming Profile

#### C.1.2.1	Introduction

This profile defines required capabilities for UE-based SRC functionalities as network-side SRS capabilities to support split rendering to 2D screens.

#### C.1.2.2	SRC Capabilities

##### C.1.2.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

##### C.1.2.2.2	Media Capabilities

The SRC shall support the media capabilities of a device type 3 as defined in TS 26.119 [4], clause 10.4.

##### C.1.2.2.3	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose predication shall be formatted according to clause 8.2.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.2.2.3.

#### C.1.2.3	SRS Capabilities

##### C.1.2.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the SRC are shared with the SRC prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study.

##### C.1.2.3.2	Video encoding

Editor’s Note: Video Encoding capabilities is for further study to match the SRC capabilities.

##### C.1.2.3.3	Audio and Speech encoding

Editor’s Note: Audio and Speech Encoding capabilities is for further study to match the SRC capabilities.

##### C.1.2.3.4	Video decoding

Editor’s Note: Video Decoding Capabilities are for further study

##### C.1.2.3.5	Audio and Speech decoding

Editor’s Note: Audio and Video Decoding Capabilities are for further study

##### C.1.2.3.6	Metadata Formats

Editor’s Note: Metadata capabilities are for further study to match the SRC capabilities.

#### C.1.2.4 	Profile identifier

The type urn:3gpp:sr-mse:src:profile:2dpixelstreaming shall be included in splitRenderingProfile parameter when the SRC signals SRS the Split Rendering Configuration [8.4.2.2].

### C.1.3	3D Pixel Streaming Profile

#### C.1.3.1	Introduction

This profile defines required capabilities for UE-based SRC functionalities as network-side SRS capabilities to support MeCAR devices.

#### C.1.3.2	SRC Capabilities

##### C.1.3.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

##### C.1.3.2.2	Media Capabilities

The SRC shall support the media capabilities of a device type 1 as defined in TS 26.119 [4], clause 10.2.

If the device is a device type 2 as defined in TS 26.119 [4], clause 10.4, it shall also support the media capabilities of a device type 2 as defined in TS 26.119 [4], clause 10.3.

If the device is a device type 4 as defined in TS 26.119 [4], clause 10.5, it shall also support the media capabilities of a device type 2 as defined in TS 26.119 [4], clause 10.5.

##### C.1.3.2.3	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose predication shall be formatted according to clause 8.2.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.2.2.3.

#### C.1.3.3	SRS Capabilities

##### C.1.3.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the SRC are shared with the SRC prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study.

##### C.1.3.3.2	Video encoding

Editor’s Note: Video Encoding capabilities is for further study to match the SRC capabilities.

##### C.1.3.3.3	Audio and Speech encoding

Editor’s Note: Audio and Speech Encoding capabilities is for further study to match the SRC capabilities.

##### C.1.3.3.4	Video decoding

Editor’s Note: Video Decoding Capabilities are for further study

##### C.1.3.3.5	Audio and Speech decoding

Editor’s Note: Audio and Speech Decoding Capabilities are for further study

##### C.1.3.3.6	Metadata Formats

Editor’s Note: Metadata capabilities are for further study to match the SRC capabilities.

#### C.1.3.4 	Profile identifier

The type urn:3gpp:sr-mse:src:profile:3dpixelstreaming shall be included in splitRenderingProfile parameter when the SRC signals SRS the Split Rendering Configuration [8.4.2.2].

### C.1.4	Description of the Rendering Format for Pixel Streaming Profiles

#### C.1.4.1	General

In response to the Split Rendering Configuration message, the SRS shall reply with a description of the rendering format.

The rendering format description shall be a compliant glTF 2.0 [2] file. The file may include references to the buffer streams that contain the components of the rendered media.

Both SRS and SRC shall comply with the SD-Rendering-Ext1 capability as defined in TS26.119 [4].

In addition, both SRS and SRC shall support for referencing WebRTC RTP streams and data channels as described in [3].

An SRC that complies with the 3D Pixel Streaming profile shall support the 3GPP_node_prerendered extension as defined in C.4.2.

#### C.1.4.2	3D Pixel Streaming Profile-specific glTF Extension

The 3GPP_node_prerendered extension is an extension at the node level to describe that the corresponding node is accessible as a prerendered content. The 3GPP_node_prerendered extension should be associated with the root node of the scene. It constitutes an alternative representation of the node and all its children. As such, if present, if the client decides to use the pre-rendered representation, it shall completely ignore the mesh description of the node and its children nodes.

The 3GPP_node_prerendered supports multiple 2D video textures and audio sources that correspond to the rendered views and audio content.

The semantics of the 3GPP_node_prerendered are provided by the following table:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| visual | Object | O | N/A | An object that describes the rendered visual components of the content. |
| audio | Object | O | N/A | An object that describes the rendered audio components of the content. |

The description of the visual object is provided in the following table:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| visual_configuration | enum | O | VIEW_STEREO | An indication of the view configuration for the pre-rendered media. It can either be VIEW_MONO or VIEW_MONO. |
| Views | array(Object) | M |  | An array that describes the views of the prerendered content. |
| eye_visibility | enum | M |  | The visibility of the current view. This can take one of the following values: “EYE_LEFT”, EYE_RIGHT”, “EYE_BOTH”, or “EYE_NONE”. EYE_NONE is used for depth and transparency components. |
| composition_layers | array(number) | M |  | An array of accessors identifiers that each corresponds to a composition layer of the parent view. |
| composition_layer_type | array(enum) | M |  | For each of the composition layers of the parent view, this indicates the type of that composition layer. The values should be provided in the same order as the composition_layers. The allowed values are: “COMPOSITION_LAYER_PROJECTION”, “COMPOSITION_LAYER_QUAD”, “COMPOSITION_LAYER_EQUIRECTANGULAR”, “COMPOSITION_LAYER_CUBEMAP”, “COMPOSITION_LAYER_DEPTH”, and “COMPOSITION_LAYER_OCCUPANCY”. |

The description of the audio object in the prerendered media extension is provided in the following table:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enum | O | AUDIO_STEREO | describes the format of the prerendered audio content. The type can take one of the following values: “AUDIO_MONO”, “AUDIO_STEREO”, and “AUDIO_HOA”. |
| Components | array(number) | M |  | provides a list of the accessors that point to the media streams associated with rendered audio content. |

The JSON scheme for the 3GPP_node_prerendered is as follows:

{ "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered", "type" : "object", "description": "glTF extension to described pre-rendered content", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "visual": { "$ref": "3GPP_node_rendered.visual.schema.json", "description": "visual streamed buffers" }, "audio": { "$ref": "3GPP_node_rendered.audio.schema.json", "description": "audio streamed buffers" }, "extensions": {}, "extras": {} }, "required": ["visual"] } { "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered.visual", "type" : "object", "description": "Object representing the visual rendered media", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "view_configuration": { "type": "string", "description": "the view configuration used for the session", "gltf_detailedDescription": "the view configuration used for the session", "enum": ["VIEW_MONO", "VIEW_STEREO"] }, "views": { "type": "array", "description": "array of layer view objects", "gltf_detailedDescription": "", "items": { "$ref": "3GPP_node_rendered.visual.view.schema.json" }, "minItems": 1 }, "extensions": {}, "extras": {} }, "required": ["views"] } { "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered.visual.view", "type" : "object", "description": "A representation of a rendered view", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "eye_visibility": { "type": "string", "description": "the visibility of the current view", "enum": ["EYE_LEFT", "EYE_RIGHT", "EYE_BOTH", "EYE_NONE"] }, "composition_layers": { "type": "array", "description": "array of timed accessors that carry the streamed buffers for each composition layer of the view", "items": { "type": "integer" }, "minItems": 1 }, "composition_layer_type": { "type": "array", "items": { "type": "string", "description": "the type of composition layer in the array of composition layers with the same array index", "gltf_detailedDescription": "the type of composition layer in the array of composition layers with the same array index", "enum": ["COMPOSITION_LAYER_PROJECTION", "COMPOSITION_LAYER_QUAD", "COMPOSITION_LAYER_EQUIRECTANGULAR", "COMPOSITION_LAYER_CUBEMAP", "COMPOSITION_LAYER_DEPTH", "COMPOSITION_LAYER_OCCUPANCY"] }, "minItems": 1 }, "extensions": {}, "extras": {} }, "required": ["views"] } { "$schema" : "http://json-schema.org/draft-07/schema", "title" : "3GPP_node_rendered.audio", "type" : "object", "description": "Object representing the audio rendered media", "allOf": [ { "$ref": "glTFProperty.schema.json"} ], "properties" : { "type": { "type": "string", "description": "the type of the rendered audio", "gltf_detailedDescription": "the type of the rendered audio", "enum": ["AUDIO_MONO", "AUDIO_STEREO", "AUDIO_HOA"], "default": "AUDIO_STEREO" }, "components": { "type": "array", "description": "array of timed accessors to audio component buffers", "items": { "type": "integer" }, "minItems": 1 }, "extensions": {}, "extras": {} }, "required": ["components"] }

### C.1.5	Profile Restrictions and Requirements

All Pixel Streaming profile are expected to be relocation intolerant and if using the 5G edge procedure shall set he easRelocationRequirements to “RELOCATION_INTOLERANT” in the tolerance field.

When the 2D Pixel Streaming profile is used, a policy template and a dynamic policy request may include the following QoS specifications, one for each of the components of the downlink streams:

- 1  QoS specification corresponding to the mono view.

- 1 QoS specification corresponding to one depth buffer stream associated with the mono view.

- 1 QoS specification corresponding to an occupancy/transparency buffer stream associated with the mono view.

- 1 QoS specification corresponding to an audio stream.

When the 3D Pixel Streaming profile is used, a policy template and a dynamic policy request may include the following QoS specifications, one for each of the components of the downlink streams:

- 2  QoS specifications corresponding to for left and right eye buffer streams.

- 2 QoS specifications corresponding to one depth buffer stream associated with the left and/or the right views.

- 2 QoS specification corresponding to an occupancy/transparency buffer stream associated with the left and/or the right views.

- 1 QoS specification corresponding to an audio stream.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04-2023 | 123-e | S4-230726 |  |  |  | Improvements and Corrections to edge and dynamic policy procedures in SR |  |
| 05-2023 | 124 | S4-121075 |  |  |  | General updates to TS26.565 |  |
| 05-2023 | 124 | S4-121004 |  |  |  | SR Rendering API |  |
| 05-2023 | 124 | S4-231005 |  |  |  | Pixel Streaming Media Profile |  |
| 05-2023 | 124 | S4-231003 |  |  |  | pCR on signaling for SR session control |  |
| 05-2023 | 124 | S4-230925 |  |  |  | On SR configuration API and view configuration |  |
| 08-2023 | 125 | S4-231449 |  |  |  | [SR_MSE] Transport protocols |  |
| 08-2023 | 125 | S4-231518 |  |  |  | [SR_MSE] Rendering optimization |  |
| 08-2023 | 125 | S4-231432 |  |  |  | [SR_MSE] Updates to Media Capabilities |  |
| 08-2023 | 125 | S4-231324 |  |  |  | Split rendering Metrics |  |
| 08-2023 | 125 | S4-231434 |  |  |  | Editorial corrections on SR MSE architectures |  |
| 11-2023 | 126 | S4-231909 |  |  |  | Editor’s updates | 0.7.0 |
| 11-2023 | 126 | S4-231911 |  |  |  | Added pose interval to configuration | 0.7.0 |
| 11-2023 | 126 | S4-231912 |  |  |  | Added signaling of SR profile in configuration | 0.7.0 |
| 11-2023 | 126 | S4-231914 |  |  |  | Clarified session setup and configuration | 0.7.0 |
| 11-2023 | 126 | S4-231796 |  |  |  | Added protocol stack | 0.7.0 |
| 11-2023 | 126 | S4-232007 |  |  |  | Timing information in QoE metrics | 0.7.0 |
| 11-2023 | 126 | S4-231800 |  |  |  | Made fov optional in pose format | 0.7.0 |
| 11-2023 | 126 | S4-231802 |  |  |  | Defined output signaling format for pixel streaming | 0.7.0 |
| 11-2023 | 126 | S4-232011 |  |  |  | Updated media profiles for pixel streaming profile | 0.7.1 |
| 12-2023 | SA#102 | SP-231306 |  |  |  | Version 1.0.0 created by MCC | 1.0.0 |
| 02-2024 | 127 | S4-240404 |  |  |  | TS cleanup | 1.1.0 |
| 02-2024 | 127 | S4-240135 |  |  |  | Profile identifiers | 1.1.0 |
| 02-2024 | 127 | S4-240400 |  |  |  | Pre-requisites for Split Rendering | 1.1.0 |
| 02-2024 | 127 | S4-240405 |  |  |  | Device Type | 1.1.0 |
| 02-2024 | 127 | S4-240198 |  |  |  | Editorial corrections | 1.1.0 |
| 02-2024 | 127 | S4-240422 |  |  |  | QoE metrics timing information format | 1.1.0 |