| 3GPP TS 26.249 V0.1.2 (2024-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group SA; Immersive Audio for Split Rendering Scenarios; Detailed Algorithmic Description of Split Rendering Functions (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	General description of split renderer	7

4.1	Introduction	7

4.2	ISAR system overview	7

5	ISAR baseline	8

5.1	Split pre-rendering	8

5.1.1	Clauses referenced to [4]	8

5.1.2	Interfaces for split pre-rendering	8

5.1.3	Interface requirements	8

5.2	Intermediate split renderer format	9

5.2.1	Clauses referenced to [4]	9

5.3	Split post-rendering	10

5.3.1	Clauses referenced to [4]	10

5.3.2	Interfaces for split post-rendering	10

5.3.3	Interface requirements	10

6	Additional features	10

6.1	Introduction	10

Annex A (informative): ISAR Application Programming Interfaces	11

ISAR pre-renderer API	11

ISAR post-renderer API	17

Annex B (normative): RTP Payload Format and SDP Parameters	21

Annex C (normative): ISAR Reference Source Code	21

Annex D (normative): Test Vectors	21

Annex E (informative): Change history	21

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

An essential architectural characteristic of XR clients is the reliance on a functional split between a set of composite pre-renderers that are implemented as parts of a presentation engine and a set of post-rendering operations implemented on an End Device prior to final output. Split rendering may be a necessity if the End Device is power constrained or limited in computational power. However, split rendering is not precluded from other End Devices that do not have such constraints. A discussion of relevant split rendering scenarios is provided in TR 26.865 [1], together with general design guidelines for immersive audio split rendering systems and specific design constraints and performance requirements for split rendering solutions for the 3GPP IVAS codec [1]. The latter are the basis for the split rendering feature of the IVAS codec. This TS presents ISAR split rendering solutions in a detailed algorithmic description, applicable even for other coding systems and renderers, whereby the split rendering solutions of the IVAS codec constitute a baseline set of the provided split rendering solutions

.

# 1	Scope

The present document is a detailed algorithmic description of Split Rendering functions (ISAR) addressing Immersive Audio for Split Rendering Scenarios and that are applicable to a broad range of immersive audio coding systems and renderers. Functional solutions are described on an algorithmic level. Annexes of this document specify APIs, RTP payload format and SDP parameters as well as source code and test vectors.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 26.865: "Immersive Audio for Split Rendering Scenarios; Requirements ".

[3]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[4]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services (IVAS); Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions".

[5]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)".

[6]	3GPP TS 26.252: "Codec for Immersive Voice and Audio Services (IVAS); Test sequences".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	General description of split renderer

## 4.1	Introduction

The main part of the present document is a detailed algorithmic description of functions for Split Rendering of immersive audio. It comprises

- Intermediate pre-rendered audio representation,

- Encoder, bitstream and decoder for the intermediate representation,

- Post-rendering of the decoded intermediate representation to provide binaural audio output with and without head-tracker input and post-rendering control metadata.

Along with the intermediate pre-rendered audio representation, functional requirements for pre-renderer operations are provided, which, if met, enable a Presentation Engine to connect to an ISAR compliant ISAR decoder and post-renderer. Interfaces are described allowing an immersive audio decoder/renderer in a Presentation Engine to connect to the ISAR pre-renderer.

The post-renderer procedures of this document are mandatory for implementation all User Equipment (UE)s claiming ISAR compliant post-rendering capabilities.

## 4.2	ISAR system overview

This clause provides a generic ISAR systems overview based on the example of the ISAR compliant IVAS split rendering feature, which define the baseline ISAR system illustrated in Figure 4.2-1.

The immersive audio rendering process is split between a capable device or network node (by the Presentation Engine relying on IVAS decoding and rendering) and a less capable device with limited computing and memory resources and motion-sensing for head-tracked binaural output. ISAR split rendering consists of the following core components:

- Pre-renderer & encoder with

  - Pose correction metadata computation and metadata encoder

  - Binaural audio encoder (transport codec encoder)

- ISAR decoder & post-renderer with

  - Pose correction metadata decoder

  - Binaural audio decoder (transport codec decoder)

  - Pose corrective post renderer

The metadata-based pose correction scheme allows adjusting in a lightweight process a binaural audio signal originally rendered for a first pose according to a second pose. In split rendering context, the first pose is the potentially outdated lightweight-device pose available at the pre-renderer while the second pose is the current and accurate pose of the lightweight-device. The metadata is calculated at the capable device or network node based on additional binaural renditions at probing poses different from the first pose. For increasing degrees-of-freedom (DOF) an increasing number of additional binaural renditions at different probing poses is required. The metadata is transmitted to the lightweight device along with the coded binaural audio signal rendered for the first pose.

The pose correction metadata computation is done in CLDFB domain. Thus, unless the immersive audio decoder/renderer already operates in that domain, a conversion of the pre-rendered immersive audio signal and the additional binaural renditions to that domain is required. The binaural audio signal rendered to the first pose is encoded using one of the two codecs, LCLD or LC3plus, whereby the former of these codecs operates in CLDFB domain. The two codecs have complementary properties giving implementors the freedom to make individual trade-offs between complexity, memory, latency, and rate-distortion performance and to implement a design that is optimized for a given immersive audio service and hardware configuration. It is also possible to use another transport codec for the binaural audio signal.

ISAR split rendering can operate at various DOFs, ranging from 0-DOF (no pose correction) to 3-DOF (pose correction on the three rotational axes yaw, pitch, roll) at bit rates from 256 kbps (0-DOF) to 384 - 768 kbps (3-DOF).

Figure 4.2-1: ISAR Split Rendering baseline system

# 5	ISAR baseline

## 5.1	Overview

ISAR baseline functionality is largely defined by the algorithmic description of the IVAS codec split rendering feature in TS 26.253 [4]. In the following, an overview of the functional components of the ISAR baseline is provided along the corresponding description or with a reference in what clause of TS 26.253 they are described.

## 5.2	Split pre-rendering

### 5.2.1	Functional components/topics specified TS 26.253

The following list displays where a given functional component or topic is described in TS 26.253 [4]. The righthand side specifies the respective clause in [4].

Overview	 7.6.2.1

Complex Low-delay Filter Bank (CLDFB) analysis	 6.2.5.1

### 5.2.2	Interfaces for split pre-rendering

Split pre-rendering (or ISAR pre-rendering) API is described in detailed in Annex A.  The interface for split pre-renderer takes in the user head pose and generates N probing head poses, where N >= 0 and depends on Degree of Freedom (DoF) and rotation axis. Then, the renderer of the immersive audio decoder/renderer system (see Figure 4.2-1) is called N+1 times (one call per head pose) to generate N+1 binaural signals. The split pre-rendering interface takes in the N+1 binaural signals and generates split rendering bitstream. The interface provides the support to take the binaural signals in either time domain or CLDFB domain.

### 5.2.3	Interface requirements

Split rendering supports sampling frequency of 48 kHz only. The supported frame sizes and bitrates are described in IVAS specification TS 26.253 [4] clause 7.6.2.2 and clause 7.6.2.3.

## 5.3	Intermediate split renderer format

### 5.3.1	Functional components/topics specified TS 26.253

### The following list displays where a given functional component or topic is described in TS 26.253 [4]. The righthand side specifies the respective clause in [4].

Supported Split Rendering bitrates	 7.6.2.2/3

Intermediate split renderer metadata format	 7.6.3

Overview	 7.6.3.1

Metadata computation, quantization and coding	 7.6.3.2

Metadata computation for deviations about Yaw axis	 7.6.3.2.1

Quantization and coding of Yaw metadata	 7.6.3.2.2

Metadata computation for deviations about Pitch axis	 7.6.3.2.3

Quantization and coding of Pitch metadata	 7.6.3.2.4

Metadata computation for deviations about Roll axis	 7.6.3.2.5

Quantization and coding of Roll metadata	 7.6.3.2.6

Common split rendering metadata quantization and coding strategies	 7.6.3.3

Intermediate split renderer metadata decoder	 7.6.3.4

Intermediate split renderer metadata loss concealment	 7.6.3.5

LCLD coded intermediate split renderer binaural audio format	 7.6.4

LCLD codec overview	 7.6.4.1

LCLD encoder		 7.6.4.2

Overview		 7.6.4.2.1

Perceptual Banding	 7.6.4.2.2

Joint Channel Coding	 7.6.4.2.3

Overview	 7.6.4.2.3.1

Bitstream Syntax	 7.6.4.2.3.2

Parameter Computation and Quantization	 7.6.4.2.3.3

Joint Coding Type Decision	 7.6.4.2.3.4

Entropy Coding of Parameters	 7.6.4.2.3.5

Temporal Grouping	 7.6.4.2.4

RMS Envelope	 7.6.4.2.5

RMS Envelope Calculation	 7.6.4.2.5.1

Normalizing the CLDFB Coefficients with the RMS Envelope	 7.6.4.2.5.2

RMS Envelope Transmission	 7.6.4.2.5.3

Perceptual Model	 7.6.4.2.6

Linear Prediction	 7.6.4.2.7

Overview	 7.6.4.2.7.1

Bitstream Syntax	 7.6.4.2.7.2

Prediction for 20ms frames	 7.6.4.2.7.3

Prediction for frames shorter than 20ms	 7.6.4.2.7.4

Prediction Signalling	 7.6.4.2.7.5

Quantization of Prediction Parameters	 7.6.4.2.7.6

Estimation of Prediction Parameters	 7.6.4.2.7.7

Bit Allocation	 7.6.4.2.8

Quantization of the Normalized CLDFB Coefficients	 7.6.4.2.9

Overview	 7.6.4.2.9.1

Differential Coding of the Normalized CLDFB Coefficients	 7.6.4.2.9.2

Quantization of Normalized CLDFB coefficients and Prediction Residuals	 7.6.4.2.9.3

Huffman Coding of Quantized Normalized CLDFB coefficients and

Quantized Prediction Residuals	 7.6.4.2.9.4

LCLD decoder	 7.6.4.3

Overview	 7.6.4.3.1

Decoding Group Information	 7.6.4.3.2

Decoding RMS Envelope Information	 7.6.4.3.3

Perceptual Model	 7.6.4.3.4

Bit Allocation	 7.6.4.3.5

Normalized CLDFB Coefficient and Prediction Residual Huffman Decoding and

Inverse Quantization	 7.6.4.3.6

Inverse Prediction	 7.6.4.3.7

Overview	 7.6.4.3.7.1

Status Tracking after Frame Loss	 7.6.4.3.7.2

Inverse RMS Envelope Normalization	 7.6.4.3.8

Inverse Joint Stereo Processing	 7.6.4.3.9

LCLD packet loss concealment	 7.6.4.4

General	 7.6.4.4.1

Synthesis model	 7.6.4.4.2

Analysis and parameter estimation	 7.6.4.4.3

Tonality determination	 7.6.4.4.4

Sinusoidal extension	 7.6.4.4.5

Predictive extension	 7.6.4.4.6

Cross-fade	 7.6.4.4.7

Burst-loss handling	 7.6.4.4.8

LC3plus coded intermediate split renderer binaural audio format	 7.6.5

Introduction (Informative)	 7.6.5.1

Overview	 7.6.5.2

Encoder	 7.6.5.3

Decoder	 7.6.5.4

Frame Structure	 7.6.5.5

Packet Loss Concealment	 7.6.5.6

LC3 interoperable mode	 7.6.5.7

Bit allocation for Split rendering	 7.6.7

## 5.4	Split post-rendering

### 5.4.1	Functional components/topics specified TS 26.253

The following list displays where a given functional component or topic is described in TS 26.253 [4]. The righthand side specifies the respective clause in [4].

Overview		 7.6.6.1

Post rendering with pose correction	 7.6.6.2

Metadata decoding	 7.6.6.2.1

Metadata interpolation or extrapolation	 7.6.6.2.2

Matrix mixing	 7.6.6.2.3

Post rendering in 0 DOF mode	 7.6.6.3

Complex Low-delay Filter Bank (CLDFB) synthesis	 5.3.7

### 5.4.2	Interfaces for split post-rendering

Split post-rendering (or ISAR post-rendering) API is described in detailed in Annex A.  The interface for split post-renderer takes in the ISAR bitstream and decodes it to generate reference binaural signal and pose correction metadata. The interface also takes in the latest user head pose and performs pose correction on reference binaural signal to generate output binaural signal in time domain.

### 5.4.3	Interface requirements

Split rendering supports sampling frequency of 48 KHz only. The post renderer supports output frame sizes of 5, 10 and 20ms. The output frame size of post renderer needs to be less than or equal to the frame size of pre-renderer.

Annex A (informative):
ISAR Application Programming Interfaces

## A.1	Overview

ISAR API is available in lib_isar\lib_isar_pre_rend.h and lib_isar\lib_isar_post_rend.h header files. These header files are present in source code as part of the IVAS codec floating-point code specification [5]. The tables below provide a detailed description of ISAR pre-renderer and post renderer API functions.

## A.2	ISAR pre-renderer API

Table 1 provides an overview of ISAR pre-renderer API.

Table 1: ISAR pre-renderer API functions

| Function names | Description of function | Function arguments | Description of arguments |
| --- | --- | --- | --- |
| ISAR_PRE_REND_open() | Allocates and initializes the buffers/handles inside SPLIT_REND_WRAPPER | SPLIT_REND_WRAPPER *hSplitBinRend | - Input/output parameter. - The wrapper handle hSplitBinRend should be allocated prior to calling this function. - hSplitBinRend->multiBinPoseData should be set as per ISAR_PRE_REND_GetMultiBinPoseData() prior to calling this function - ISAR pre-renderer handle described in detail in Table 3 |
|  |  | ISAR_SPLIT_REND_CONFIG_DATA *pSplitRendConfig | - Input/output parameter - The handle pSplitRendConfig should be allocated prior to calling this function. - ISAR pre-renderer config handle described in detail in Table 2 |
|  |  | const int32_t output_Fs | Input parameter. Output sampling rate |
|  |  | const int16_t cldfb_in_flag | Input parameter. 1: if multi-binaural signal input is in CLDFB domain only 0: Otherwise |
|  |  | const int16_t pcm_out_flag | Input parameter. 1: if output binaural signal is in PCM format (BINAURAL_SPLIT_PCM config) 0: Otherwise |
|  |  | const int16_t num_subframes | Input parameter. Number of 5ms subframes to process. Supported values: 1/2/4: For 0-DOF cases 4: For non-zero DOF cases |
|  |  | const int16_t mixed_td_cldfb_flag | Input parameter. 1: if multi-binaural signal input is in both CLDFB and time domain 0: Otherwise |
| ISAR_PRE_REND_GetMultiBinPoseData() | - Get the Pose correction metadata configuration - Should be called during runtime if rot_axis changes | const ISAR_SPLIT_REND_CONFIG_DATA *pSplit_rend_config | Input parameter. ISAR pre-renderer config handle described in detail in Table 2 |
|  |  | MULTI_BIN_REND_POSE_DATA *pMultiBinPoseData | Output parameter. ISAR pre-renderer pose config data handle described in detail in Table 4 |
|  |  | const ISAR_SPLIT_REND_ROT_AXIS rot_axis | Input parameter. ISAR rotation axis described in detail in Table 4 |
| ISAR_PRE_REND_MultiBinToSplitBinaural() | Process call to generate ISAR pre-renderer output on frame-by frame basis from the multi-binaural signal | SPLIT_REND_WRAPPER *hSplitBin | - Input/output parameter - ISAR pre-renderer handle |
|  |  | const IVAS_QUATERNION headPosition | - Input parameter - Quaternion for head rotation |
|  |  | const int32_t SplitRendBitRate | - Input parameter - Split rendering bitrate |
|  |  | ISAR_SPLIT_REND_CODEC splitCodec | - Input parameter - ISAR transport codec described in detail in Table 2 - Should be set as per ISAR_SPLIT_REND_CONFIG_DATA output of ISAR_PRE_REND_open() |
|  |  | int16_t codec_frame_size_ms | - Input parameter - Codec frame size if ISAR transport codec - Should be set as per ISAR_SPLIT_REND_CONFIG_DATA output of ISAR_PRE_REND_open() |
|  |  | ISAR_SPLIT_REND_BITS_HANDLE pBits | - Handle pBits should be allocated and initialized prior to calling function - Output parameter - ISAR bitstream handle described in detail in Table 5 |
|  |  | float Cldfb_In_BinReal[][][] | - Input parameter - CLDFB real input buffer |
|  |  | float Cldfb_In_BinImag[][][] | - Input parameter - CLDFB imaginary input buffer |
|  |  | const int16_t max_bands | - Input parameter - Number of bands in CLDFB input |
|  |  | float *output[] | - Input/Output parameter - PCM input/output buffer |
|  |  | const int16_t low_res_pre_rend_rot | Reserved flag. Should always be set to 1 |
|  |  | const int16_t cldfb_in_flag | Input parameter. 1: if multi-binaural signal input is in CLDFB domain only 0: Otherwise |
|  |  | const int16_t pcm_out_flag | Input parameter. 1: if output binaural signal is in PCM format (BINAURAL_SPLIT_PCM config) 0: Otherwise |
|  |  | const int16_t ro_md_flag | Input parameter. 1: if the HRTFs used in the generation on multi-binaural signal can lead to ITD difference above 1.6 kHz 0: Otherwise |
| ISAR_PRE_REND_close() | Deallocates the ISAR handles/memory | SPLIT_REND_WRAPPER *hSplitBinRend | - Input/Output parameter - ISAR pre-renderer handle |
|  |  | IVAS_REND_AudioBuffer *pSplitRendEncBuffer | reserved buffer field. Can be set to NULL |

Table 2: ISAR_SPLIT_REND_CONFIG_DATA description

| ISAR_SPLIT_REND_CONFIG_DATA fields | description |
| --- | --- |
| int32_t splitRendBitRate | Split rendering bitrate |
| int16_t hq_mode | - High quality mode for 3 DOF - Supported values: 0 or 1 NOTE: Adds complexity at pre-renderer |
| int16_t dof | - Degree of Freedom - Supported values: 0 to 3 |
| int16_t codec_delay_ms | look ahead delay of the external transport codec that is used to code BIN signal output of pre-renderer in BINAURAL_SPLIT_PCM mode |
| int16_t codec_frame_size_ms | Frame size of the ISAR pre-renderer transport codec |
| ISAR_SPLIT_REND_POSE_CORRECTION_MODE poseCorrectionMode; | ISAR_SPLIT_REND_POSE_CORRECTION_MODE_NONE: No pose correction or 0-DOF ISAR_SPLIT_REND_POSE_CORRECTION_MODE_CLDFB: Pose correction OR non-zero DOF |
| ISAR_SPLIT_REND_CODEC codec | - ISAR transport codec - Supported values: 0 to 3 - 0: LCLD - 1: LC3plus - 2: Default (internally set to LC3plus for time domain input, LCLD otherwise) - 3: NONE (BINAURAL SPLIT PCM mode) |
| ISAR_SPLIT_REND_RENDERER_SELECTION rendererSelection; | Debug field. Default value: ISAR_SPLIT_REND_RENDERER_SELECTION_DEFAULT |

Table 3: SPLIT_REND_WRAPPER description

| SPLIT_REND_WRAPPER fields | description |
| --- | --- |
| MULTI_BIN_REND_POSE_DATA multiBinPoseData | ISAR pre-renderer pose config data handle described in detail in Table 4 |
| ISAR_BIN_HR_SPLIT_PRE_REND_HANDLE hBinHrSplitPreRend | ISAR pre-renderer pose correction metadata handle |
| ISAR_BIN_HR_SPLIT_LCLD_ENC_HANDLE hSplitBinLCLDEnc | LCLD encoder handle |
| CLDFB_HANDLES_WRAPPER_HANDLE hCldfbHandles | CLDFB handles |
| ISAR_LC3PLUS_ENC_HANDLE hLc3plusEnc | LC3plus encoder handle |
| float *lc3plusDelayBuffers | LC3plus delay buffer |
| int32_t lc3plusDelaySamples | LC3plus delay in samples |

Table 4: MULTI_BIN_REND_POSE_DATA description

| MULTI_BIN_REND_POSE_DATA fields | description |
| --- | --- |
| int16_t num_poses; | Number of poses (including reference pose) for Metadata computation OR number of binaural signals in multi-binaural input to ISAR pre-renderer |
| float relative_head_poses[MAX_HEAD_ROT_POSES][3]; | - Head poses (relative to reference head pose) along yaw, pitch and roll axis - Binaural signals corresponding probing poses (poses other than reference head pose) in the multi-binaural input signal are computed based on this field. |
| int16_t dof; | Same as described in ISAR_SPLIT_REND_CONFIG_DATA |
| int16_t hq_mode; | Same as described in ISAR_SPLIT_REND_CONFIG_DATA |
| ISAR_SPLIT_REND_ROT_AXIS rot_axis; | - Rotation axis along which pose correction is needed - Supported values for 1 DOF (0, 1, 2, 3). 0: Default or YAW, 1: YAW, 2: Pitch, 3: Roll - Supported values for 2 DOF (0, 4, 5, 6). 0: Default or YAW+PITCH, 4: YAW+PITCH, 5: YAW+ROLL, 6: PITCH+ROLL - Supported values for 3 DOF (0 or Default) |
| ISAR_SPLIT_REND_POSE_CORRECTION_MODE poseCorrectionMode | Same as described in ISAR_SPLIT_REND_CONFIG_DATA |

Table 5 ISAR_SPLIT_REND_BITS_HANDLE description

| ISAR_SPLIT_REND_BITS_HANDLE fields | Description |
| --- | --- |
| uint8_t *bits_buf | Bitstream buffer (to be allocated by caller of ISAR pre-renderer) |
| int32_t buf_len | Length of bits_buf buffer in bytes |
| int32_t bits_written | Number of bits written to bits_buf buffer |
| int32_t bits_read | Number of bits read from bits_buf buffer |
| int16_t codec_frame_size_ms | Transport Codec frame size as described in Table 2 |
| ISAR_SPLIT_REND_CODEC codec | Transport Codec as described in Table 2 |
| ISAR_SPLIT_REND_POSE_CORRECTION_MODE pose_correction | Pose correction mode as described in Table 2 |

## A.3	ISAR post-renderer API

Table 6 provides an overview of ISAR pre-renderer API.

Table 6: ISAR post-renderer API functions

| Function names | Description of functions | Function arguments | Description of parameters |
| --- | --- | --- | --- |
| ISAR_POST_REND_open() | Allocates and initializes ISAR handle phIsarRend | ISAR_POST_REND_HANDLE *phIsarRend | ISAR handle |
|  |  | const int32_t outputSampleRate | Output sampling rate |
|  |  | const IVAS_AUDIO_CONFIG outConfig | Output configuration (Only BINAURAL output config supported) |
|  |  | const bool asHrtfBinary | Reserved/Unused parameter |
|  |  | const int16_t nonDiegeticPan | Reserved/Unused parameter |
|  |  | const float nonDiegeticPanGain | Reserved/Unused parameter |
|  |  | const int16_t num_subframes | Number of 5ms subframes in the output. This specifies output frame size. Supported framesizes are 5, 10 and 20ms |
| ISAR_POST_REND_InitConfig() | Initializes hIvasRend->splitRenderConfig | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | const IVAS_AUDIO_CONFIG outAudioConfig | Output configuration (Only BINAURAL output config supported) |
| ISAR_REND_SetSplitRendBitstreamHeader() | Sets one time configuration. To be set during SDP negotiation | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | const ISAR_SPLIT_REND_CODEC codec | ISAR transport codec |
|  |  | const ISAR_SPLIT_REND_POSE_CORRECTION_MODE poseCorrection | ISAR pose correction method |
|  |  | const int16_t codec_frame_size_ms | ISAR transport codec frame size |
| ISAR_POST_REND_NumOutChannels() | Get number of output channels | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | int16_t *numOutChannels | Number of output channels |
| ISAR_POST_REND_AddInput () | Add input stream for processing | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | const IVAS_AUDIO_CONFIG inConfig | Input configuration (Only BINAURAL_SPLIT_CODED and BINAURAL_SPLIT_PCM input configs supported) |
|  |  | ISAR_POST_REND_InputId *inputId | Input ID |
| ISAR_POST_REND_GetInputNumChannels() | Get number of input channels | ISAR_POST_REND_CONST_HANDLE hIsarRend | ISAR handle |
|  |  | const ISAR_POST_REND_InputId inputId | Input ID |
|  |  | int16_t *numChannels | number of input channels |
| ISAR_POST_REND_GetDelay() | Get ISAR delay (this includes both pre-renderer and post renderer delay) | ISAR_POST_REND_CONST_HANDLE hIsarRend | ISAR handle |
|  |  | int16_t *nSamples | Delay in number of samples |
|  |  | int32_t *timeScale | Time scale of the delay, equal to renderer output sampling rate |
| ISAR_POST_REND_FeedInputAudio() | Feed the audio PCM part of the input. Used in BINAURAL_SPLIT_PCM mode | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | const ISAR_POST_REND_InputId inputId | Input ID |
|  |  | const ISAR_POST_REND_ReadOnlyAudioBuffer inputAudio | Input audio buffer |
| ISAR_POST_REND_FeedSplitBinauralBitstream() | Feed ISAR bitstream | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | const ISAR_POST_REND_InputId inputId | Input ID |
|  |  | ISAR_POST_REND_BitstreamBuffer *hBits | Input bitstream buffer |
| ISAR_POST_REND_GetSplitBinauralSamples() | Get ISAR output (BINAURAL pcm samples) | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | IVAS_REND_AudioBuffer outAudio | Output audio buffer |
|  |  | bool* needNewFrame | Flag to indicate the caller if a new bitstream frame is to be fed |
| ISAR_POST_REND_SetHeadRotation() | Set head pose for each 5ms subframe | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | const IVAS_QUATERNION headRot | Head pose in quaternions |
|  |  | const IVAS_VECTOR3 Pos | Position vector(reserved/unused field) |
|  |  | const ISAR_SPLIT_REND_ROT_AXIS rot_axis | Rotation axis (reserved/unused field) |
|  |  | const int16_t sf_idx | Subframe index |
| ISAR_POST_REND_SetSplitRendBFI() | Set Bad frame index (BFI) flag if the frame is lost | ISAR_POST_REND_HANDLE hIsarRend | ISAR handle |
|  |  | const int16_t bfi | BFI flag |
| ISAR_POST_REND_Close() | Free ISAR memory and deallocate ISAR handle hIsarRend | ISAR_POST_REND_HANDLE* hIsarRend | ISAR handle |

Annex B (normative):
RTP Payload Format and SDP Parameters

The definition of the RTP payload format and SDP parameters for the ISAR feature of IVAS as part of the Annex A of TS 26.253 [4] is FFS.

A system agnostic RTP payload format and SDP parameters for the transport of the ISAR intermediate format and associated metadata is FFS.

Annex C (normative):
ISAR Reference Source Code

ISAR reference source code is available both as fixed-point code and as floating-point code.

The intermediate split renderer format encoder in floating-point code is defined in source code as part of the IVAS codec floating-point code specification [5] and can be built to an IVAS independent object library which functions are accessable by a pre-renderer through the corresponding APIs (see Annex A).

The post renderer including intermediate split renderer format decoder in floating-point code is defined as part of the IVAS codec floating-point code specification [5]. The post renderer reference source code can be built to an IVAS independent stand-alone executable.

While IVAS fixed-point code is not yet specified, ISAR fixed-point code is provided as a software patch to the IVAS floating-point code [5]; the software patch is provided as electronic attachment to this specification. Applying the patch allows running the ISAR split rendering feature in fixed-point code within the IVAS floating-point software framework.

Note: For future 3GPP releases that are expected to provide a IVAS fixed-point code specification, the ISAR fixed point-code will be provide as part of the IVAS fixed-point code specification. In that case, the electronic attachment of this specifications with the patch to the IVAS floating-point code will become obsolete and shall not be used.

Annex D (normative):
Test Vectors

Test vectors for the ISAR reference source code are defined as part of the IVAS test vector specification [6].

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA4#127-bis-e | S4-240716 | - | - | - | Presented to 3GPP SA4 Audio SWG | 0.0.1 |
| 2024-04 | SA4#127-bis-e | S4-240809 | - | - | - | Presented to 3GPP SA4 plenary | 0.1.0 |
| 2024-05 | SA4#128 | S4-241051 |  |  |  | Rapporteur’s updates: Clause 5, Annexes A-D | 0.1.1 |
| 2024-05 | SA4#128 | S4-241183 |  |  |  | Rapporteur’s updates after initial presentation to audio SWG | 0.1.2 |