Source:	HUAWEI

Title:	[FS_AVATAR] 2D Avatar description update

Spec:	FS_AVATAR PD 0.3.1

Agenda item:	9.8

Document for:	Discussion and agreement

1. Introduction

In last SA4#127-bis-e meeting, further clarification of 2D avatar was introduced by S4-240821 and accepted as in the latest PD v0.3.1. During the previous discussion, it was suggested that further information should be provided for better understanding the standardization scope of 2D avatar.

This contribution is to provide the following updates of the 2D avatar descriptions:

1) Refine the 2D avatar related terms (e.g. distinguishing reference image for the DNN model generation process and the base image/video for the avatar animation process)

2) Update the 2D avatar animation description to make it clearer.

2. Reason for Change

The contribution proposes to update the 2D avatar descriptions to address the above issues.

3. Proposal

It is proposed to agree on the following changes to the FS_AVATAR PD 0.3.1.

* * * First Change * * * *

3.5.3	2D Base Avatar generation

The 2D Base Avatar includes the following components:

-	DNN model: It is a Deep Neural Network model used to generate the animated target avatar through the inference process driven by the user’s input such as audio, video or text [18][19]. The DNN model can be optionally stored in the Avatar Storage and downloaded for avatar animation.

-	Base image/video: It is a 2D image/video in order to specify the target avatar character based on the DNN model. It is stored together with the DNN model in the Avatar Storage and will be downloaded for avatar animation (see details in 3.5.4). It can be generated by AIGC tools offline. The base video can help to improve the performance of the avatar animation compared to the base image.

The process of 2D Avatar DNN model generation is to train a DNN model through a supervised learning algorithm. The result model is the DNN model for a 2D Avatar. The training process is depicted in Figure 3.5.3-1.

Figure 3.5.3-1: 2D Avatar DNN model generation process (training)

The training process of a 2D Avatar DNN model takes three inputs as listed in Table 3.5.3-1:

-	Reference image: provides the baseline reference of the 2D Avatar representation in the form of a picture.

-	Ground truth: It is the target output video of the 2D Avatar to be trained against. The reference image should have the same character as in the ground truth video but not exactly the same appearance at the same time. For example, the lip shape or the facial expression can be different between the reference image and the ground truth at a certain time point.

-	Training input: It can be video, audio or text depending on the target use cases (i.e., which type media is going to be used for avatar animation at the inference stage).

The relationship between the reference image, training input, and the ground truth are the following:

- If the training input is audio, the ground truth is the video where the character in the reference image is talking as recorded in the audio [9].

- If the training input is video, the ground truth is the video where the character in the reference image makes the same action [10]. (In this case, the character in the training input is usually different from the ones in the reference image and the ground truth video for the use case of “face swap”.)

- If the training input is text, the ground truth is the video where the character in the reference image is talking the text content [11].

A typical DNN used for the 2D Avatar has the encoder-decoder architecture. The encoder part is used to map the input to the embedding space, and the decoder part is utilized to map the embeddings to the image space so as to generate the target 2D avatar video. A pre-processing function may be introduced to perform cropping, resizing, and normalization on the reference image and the training input data before injecting them into the DNN. It can be implemented inside or outside of the DNN. The loss function defines a measurement of the difference between the generated video and the ground truth video. The output of the loss function is used to perform the backpropagation to update the DNN parameters.

Upon the training process completion, the converged DNN results as the DNN model of the 2D Avatar. The DNN model can be stored in an exchangeable format such as .pth (PyTorch [12]), .onnx (ONNX [13]),  .trt (TensorRT [14]) or mindir (MindSpore [15]) files. In order to support the interoperability of the DNN model between UE and the network, the metadata of the model information (such as model identifier, input/output size, format/framework & version number, processing capabilities) as defined in clause 6.6.2 of [20] may be used.

Considering different levels of generalization capabilities, 2D Avatar DNN models can be categorized in two groups:

-	Robust model: It provides good generalization capability that can support the animation of different target 2D avatar characters (designated by different base images/videos).

-	Personalized model: It is good at the animation of a particular target 2D avatar character (designated by the base image/video of the same avatar character as in the reference image) but not others. A personalized model may be a fine-tuned model from a robust model.

The cost of training a robust model is generally higher than that of a personalized model, since the former takes much more input training data and training time for different avatar characters. Based on the state-of-the-art implementations and typical hardware conditions, a robust model could be trained within a day, while a personalized model could be trained less than one hour.

Table 3.5.3-1: Training input data for 2D Avatar DNN model generation

| Training input data | Description | Requirements |
| --- | --- | --- |
| Reference image | The baseline reference of the 2D avatar character to be trained. | The character in the reference image is the same as the one in the ground truth, while the character’s actions (e.g. mouth movements or facial expressions) are generally different at the same point in time. |
| Training input | The media content (video, audio, or text) to drive the animation of the 2D avatar character out of the DNN. | The actions in the training input are consistent with the ones in the ground truth video (e.g. the syllables of speaking match the mouth shape). |
| Ground truth | The training target of the 2D avatar. | The character in the ground truth video is consistent with the reference image, and the character’s action is consistent with the one in the training input. |

3.5.4	2D Avatar animation

The process of 2D Avatar animation is to generate the 2D video representation of the avatar through the inference process of the DNNmodel driven by the input video, audio or text from the user.

The inference process is depicted in Figure 3.5.4-1.

Figure 3.5.4-1: 2D Avatar animation process (inference)

For different use cases of avatar animation, the inference process may take different input data combinations as listed in Table 3.5.4-1:

-	Base image/video: It is used to specify the target avatar character to be driven. If a robust model is used as the DNN model, the base image/video can be an avatar character different from the reference image used in the DNN model generation phase. If a personalized model is used as the DNN model, the base image/video should use the same avatar character as the reference image.

-	Driving input: It may be the audio or video stream coming from the user during a real-time call. It may also be a text stream converted from the audio signals depending on the input requirement of the DNN model.

As described in clause 3.5.3, the DNNmodel may be implemented as a robust model or a personalized model. The robust model can be shared for general users and don’t need to be downloaded for animation every time. Instead, different base images/videos should be downloaded in order to support the animation of user specific avatar representations. In the case of the personalized model being used, different DNN models and the corresponding base images/videos will be downloaded for the avatar animation of different users.

The output of the DNNmodel inference is essentially the animated 2D Avatar, which can be further encapsulated, streamed and played with any existing video codecs (e.g. H.264/H.265) and deliver protocols (e.g. RTP).

Table 3.5.4-1: Possible input data combinations for 2D Avatar animation

| DNN Model | Driving input | Base image/video | Notes |
| --- | --- | --- | --- |
| Robust model | Video | Base image | The base image/video can use a different avatar character from the reference image used in the DNN model generation process. |
|  | Audio | Base image/video |  |
|  | Text | Base image/video |  |
| Personalized model | Video | Base image | The base image/video should use the same avatar character as in the reference image used in the DNN model generation process. |
|  | Audio | Base image/video |  |
|  | Text | Base image/video |  |

The following embedded video clip gives an exemplary demo of the 2D avatar animation driven by audio.

Figure 3.5.4-2: 2D Avatar animation example

* * * Second Change * * * *

8		References

[1]	Ekman, P., & Friesen, W.V. (1978). Facial action coding system: a technique for the measurement 	of facial movement.

[2]	Ekman, P., Friesen, W. V., & Hager, J. C. (2002). Facial Action Coding System.

[3]	G. A. Abrantes and F. Pereira, "MPEG-4 facial animation technology: survey, implementation,  and results," in IEEE Transactions on Circuits and Systems for Video Technology, vol. 9, no. 2,  pp. 290-305, March 1999, doi: 10.1109/76.752096.

[4]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5]	3GPP TR 22.856: “Feasibility Study on Localized Mobile Metaverse Services”

[6]	ISO/IEC 23090-14:2023, Information Technology – Coded Representation of Immersive Media – Part 14: MPEG Scene Description.

[7]	 ITU-T F.748.14 (03/2022), "Requirements and evaluation methods of non-interactive 2D real-person digital human application system".

[8] 	ITU-T F.748.15 (03/2022), "Framework and metrics for digital human application system".

[9] 	Zhou Y, Han X, Shechtman E, et al. Makelttalk: speaker-aware talking-head animation[J]. ACM Transactions On Graphics (TOG), 2020, 39(6): 1-15.

[10] 	Zhao J, Zhang H. Thin-plate spline motion model for image animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2022: 3657-3666.

[11] 	Liu, Jinglin, et al. "Parallel and High-Fidelity Text-to-Lip Generation." Proceedings of the AAAI Conference on Artificial Intelligence. Vol. 36. No. 2. 2022.

[12]	PyTorch, https://pytorch.org/

[13]	ONNX, https://onnx.ai/

[14]	TensorRT, https://developer.nvidia.com/tensorrt

[15] 	mindspore, https://www.mindspore.cn

[16] 	https://www.theverge.com/2018/11/8/18074806/ai-news-anchor-china-xinhua-digital-composite

[17] 	https://www.tiktok.com/@chriscanbefunny/video/6902602432811961605

[18]	Yilong Liu, et al., “Video-Audio Driven Real-Time Facial Animation”, https://www.cs.sjtu.edu.cn/~shengbin/course/SE/Video-Audio%20Driven%20Real-Time%20Facial%20Animation.pdf

[19]	Diana T. Mosa, et al, “A real-time Arabic avatar for deaf–mute community using attention mechanism”, https://link.springer.com/article/10.1007/s00521-023-08858-6

[20]	3GPP TR 26.927 v0.7.0: “Study on Artificial Intelligence and Machine Learning in 5G”.

* * * End of Changes * * * *