3GPP TSG SA WG4#129-e	S4-241479

online, Teams, 19 - 23 August 2024

Agenda item: 	6

Source: 	Qualcomm Incorporated

Title: 	[VOPS, FS_ACAPI] 3GPP Codecs as Web Codecs

Document for	Agreement

# Introduction

During SA#103 the new work item on “Video Operating Points - Harmonization and Stereo MV-HEVC” was approved in SP-240060. The objectives of this work are to:

- Harmonize and include as needed all the SA4 video operating points, such as Video profiles Operation Points, Video Operation Points, video encode and decode capabilities etc., which are currently scattered in various SA4 specifications (e.g. TS 26.116, TS 26.118, TS 26.119, TS 26.143, and TS 26.511), into a new specification that will be home to all such video operating points and upgrade HEVC-based levels based on industry practices.

- Define the MV-HEVC capability in this new specification.

- Then add and harmonize stereoscopic MV-HEVC (potentially with auxiliary information, e.g. alpha channels) encode/decode operating points, capabilities, streaming (e.g. CMAF, DASH) and transport aspects for:

  - 5G-media streaming profiles, codecs, and formats (TS 26.511)

  - Media capabilities for AR devices (TS 26.119)

  - Video messaging media profiles (TS 26.143)

- Perform the above work in coordination with related SDOs and industrial fora such as MPEG, DASH-IF, CTA-WAVE, and IETF, and by referencing the related specifications, e.g. the Common Media Application Format (CMAF) and the ISO base media file format (ISOBMFF), among others.

In addition, during SA#103 the new study item on “Study on Audio Codec APIs” was approved in SP-240480. The objectives of this work are to:

1)	Identify relevant interfaces on hardware/firmware platforms to the (supported) relevant codecs defined in TS 26.114 and TS 26.117 and other common pre- and postprocessing blocks, including capture front end and renderer, and characterize signal (including control signal and metadata) flows on these interfaces.

2)	Identify potential gaps in the 3GPP and W3C specifications to enable and simplify the use of the codecs in TS 26.114 and TS 26.117 in WebCodecs and WebRTC

3)	Identify common APIs for the relevant codecs defined in TS 26.114 and TS 26.117 and other common pre- and postprocessing blocks, including capture front end and renderer, that are useable to expose the identified interfaces.

4)	Provide recommendations for interfaces and adapters to WebRTC and WebCodec.

5)	Provide recommendations for potential normative work in 3GPP and/or other organizations, for example W3C and IETF.

This document addresses some thoughts on how to more beneficially support 3GPP video and audio codecs in web codecs environments.

# What are Web Codecs?

## 2.1	Introduction

The WebCodecs API (https://www.w3.org/TR/webcodecs/) is a W3C working draft that is currently under development. It specifies a powerful web API that provides developers with low-level access to the individual frames of a video stream and chunks of audio. It is particularly useful for web applications that require full control over the way media is processed, such as video or audio editors, and video conferencing applications.

The WebCodecs API provides access to codecs that are already in the browser, eliminating the need for additional software codecs and leveraging the existing hardware acceleration on the device. It gives access to raw video frames, chunks of audio data, image decoders, audio and video encoders, and decoders.

The WebCodecs API uses an asynchronous processing model. Each instance of an encoder or decoder maintains an internal, independent processing queue. Methods named configure(), encode(), decode(), and flush() operate asynchronously by appending control messages to the end of the queue, while methods named reset() and close() synchronously abort all pending work and purge the processing queue.

The WebCodecs API provides several interfaces:

- AudioDecoder: Decodes EncodedAudioChunk objects.

- VideoDecoder: Decodes EncodedVideoChunk objects.

- AudioEncoder: Encodes AudioData objects.

- VideoEncoder: Encodes VideoFrame objects.

- EncodedAudioChunk: Represents codec-specific encoded audio bytes.

- EncodedVideoChunk: Represents codec-specific encoded video bytes.

- AudioData: Represents unencoded audio data.

- VideoFrame: Represents a frame of unencoded video data.

- VideoColorSpace: Represents the color space of a video frame.

- ImageDecoder: Unpacks and decodes image data, giving access to the sequence of frames in an animated image.

- ImageTrackList: Represents the list of tracks available in the image.

- ImageTrack: Represents an individual image track.

The following table provides a simple example code for the usage of WebCodecs to demonstrate the functionality of the WebCodecs API:

// Create a new VideoDecoder and configure it const init = { output: handleFrame, error: (e) => { console.log(e.message); }, }; const config = { codec: "hevc", codedWidth: 1280, codedHeight: 720 }; let decoder = new VideoDecoder(init); decoder.configure(config); // Create a new VideoEncoder and configure it let encoder = new VideoEncoder({ output: (chunk) => { const buffer = new ArrayBuffer(chunk.byteLength); chunk.copyTo(buffer); chunks.push(buffer); }, error: (e) => console.error(e.message) }); encoder.configure({ codec: 'hevc', width: 1280, height: 720, bitrate: 2000000, framerate: 25 }); // Encode every image as a frame track.requestFrame().then((frame) => { encoder.encode(frame, {keyFrame: true}); frame.close(); }); // Create a video from it encoder.flush().then(() => { const blob = new Blob(chunks, {type: 'video/webm; codecs=vp8'}); const url = URL.createObjectURL(blob); decoder.decode(new EncodedVideoChunk({ type: 'key', timestamp: 0, data: blob })); }); }) .catch((error) => { console.error("Error: ", error); });

A full example can be found under https://bouazizi.dev/webcodecs/

# Video Related Interfaces

## Video Decoder Interface

The video decoder interface is defined as an API that allows to apply three main methods

- Configure: Enqueues a control message to configure the video decoder for decoding chunks as described by config.

- Decode: Enqueues a control message to decode the given chunk.

- IsConfigSupported: Returns a promise indicating whether the provided config is supported by the User Agent.

[Exposed=(Window,DedicatedWorker), SecureContext]

interface VideoDecoder : EventTarget {

constructor(VideoDecoderInit init);

readonly attribute CodecState state;

readonly attribute unsigned long decodeQueueSize;

attribute EventHandler ondequeue;

undefined configure(VideoDecoderConfig config);

undefined decode(EncodedVideoChunk chunk);

Promise<undefined> flush();

undefined reset();

undefined close();

static Promise<VideoDecoderSupport> isConfigSupported(VideoDecoderConfig config);

};

dictionary VideoDecoderInit {

required VideoFrameOutputCallback output;

required WebCodecsErrorCallback error;

};

callback VideoFrameOutputCallback = undefined(VideoFrame output);

The configuration of the codec is here

dictionary VideoDecoderConfig {

required DOMString codec;

AllowSharedBufferSource description;

[EnforceRange] unsigned long codedWidth;

[EnforceRange] unsigned long codedHeight;

[EnforceRange] unsigned long displayAspectWidth;

[EnforceRange] unsigned long displayAspectHeight;

VideoColorSpaceInit colorSpace;

HardwareAcceleration hardwareAcceleration = "no-preference";

boolean optimizeForLatency;

};

A codec string describes a given codec format to be used for encoding or decoding.

A valid codec string must meet the following conditions.

- Is valid per the relevant codec specification (see examples below).

- It describes a single codec.

- It is unambiguous about codec profile, level, and constraint bits for codecs that define these concepts.

NOTE: In other media specifications, codec strings historically accompanied a MIME type as the "codecs=" parameter (isTypeSupported(), canPlayType()) [RFC6381]. In this specification, encoded media is not containerized; hence, only the value of the codecs parameter is accepted.

NOTE: Encoders for codecs that define level and constraint bits have flexibility around these parameters, but won’t produce bitstreams that have a higher level or are less constrained than requested.

The format and semantics for codec strings are defined by codec registrations listed in the [WEBCODECS-CODEC-REGISTRY]. A compliant implementation may support any combination of codec registrations or none at all.

## Video Encoder Interface

Similar as for the decoder, an API for the encoder is defined

[Exposed=(Window,DedicatedWorker), SecureContext]

interface VideoEncoder : EventTarget {

constructor(VideoEncoderInit init);

readonly attribute CodecState state;

readonly attribute unsigned long encodeQueueSize;

attribute EventHandler ondequeue;

undefined configure(VideoEncoderConfig config);

undefined encode(VideoFrame frame, optional VideoEncoderEncodeOptions options = {});

Promise<undefined> flush();

undefined reset();

undefined close();

static Promise<VideoEncoderSupport> isConfigSupported(VideoEncoderConfig config);

};

dictionary VideoEncoderInit {

required EncodedVideoChunkOutputCallback output;

required WebCodecsErrorCallback error;

};

callback EncodedVideoChunkOutputCallback =

undefined (EncodedVideoChunk chunk,

optional EncodedVideoChunkMetadata metadata = {});

The configuration for the encoder is provided here

dictionary VideoEncoderConfig {

required DOMString codec;

[EnforceRange] required unsigned long width;

[EnforceRange] required unsigned long height;

[EnforceRange] unsigned long displayWidth;

[EnforceRange] unsigned long displayHeight;

[EnforceRange] unsigned long long bitrate;

double framerate;

HardwareAcceleration hardwareAcceleration = "no-preference";

AlphaOption alpha = "discard";

DOMString scalabilityMode;

VideoEncoderBitrateMode bitrateMode = "variable";

LatencyMode latencyMode = "quality";

DOMString contentHint;

};

# Audio Interfaces

## 4.1	Audio Decoder Interface

The audio decoder interface is defined as an API that allows to apply three main methods

- Configure: Enqueues a control message to configure the audio decoder for decoding chunks as described by config.

- Decode: Enqueues a control message to decode the given chunk.

- IsConfigSupported: Returns a promise indicating whether the provided config is supported by the User Agent.

[Exposed=(Window,DedicatedWorker), SecureContext]

interface AudioDecoder : EventTarget {

constructor(AudioDecoderInit init);

readonly attribute CodecState state;

readonly attribute unsigned long decodeQueueSize;

attribute EventHandler ondequeue;

undefined configure(AudioDecoderConfig config);

undefined decode(EncodedAudioChunk chunk);

Promise<undefined> flush();

undefined reset();

undefined close();

static Promise<AudioDecoderSupport> isConfigSupported(AudioDecoderConfig config);

};

dictionary AudioDecoderInit {

required AudioDataOutputCallback output;

required WebCodecsErrorCallback error;

};

callback AudioDataOutputCallback = undefined(AudioData output);

The configuration for the decoder is provided here

dictionary AudioDecoderConfig {

required DOMString codec;

[EnforceRange] required unsigned long sampleRate;

[EnforceRange] required unsigned long numberOfChannels;

BufferSource description;

};

## 4.2	Audio Encoder Interface

The audio encoder interface is defined as an API that allows to apply three main methods

- Configure: Enqueues a control message to configure the audio encoder for encoding audio data as described by config.

- encode: Enqueues a control message to encode the given data.

- IsConfigSupported: Returns a promise indicating whether the provided config is supported by the User Agent.

[Exposed=(Window,DedicatedWorker), SecureContext]

interface AudioEncoder : EventTarget {

constructor(AudioEncoderInit init);

readonly attribute CodecState state;

readonly attribute unsigned long encodeQueueSize;

attribute EventHandler ondequeue;

undefined configure(AudioEncoderConfig config);

undefined encode(AudioData data);

Promise<undefined> flush();

undefined reset();

undefined close();

static Promise<AudioEncoderSupport> isConfigSupported(AudioEncoderConfig config);

};

dictionary AudioEncoderInit {

required EncodedAudioChunkOutputCallback output;

required WebCodecsErrorCallback error;

};

callback EncodedAudioChunkOutputCallback =

undefined (EncodedAudioChunk output,

optional EncodedAudioChunkMetadata metadata = {});

The configuration for the encoder is provided here

dictionary AudioEncoderConfig {

required DOMString codec;

[EnforceRange] required unsigned long sampleRate;

[EnforceRange] required unsigned long numberOfChannels;

[EnforceRange] unsigned long long bitrate;

BitrateMode bitrateMode = "variable";

};

# Codec Registration Procedure

The codec registration procedure for new codecs is defined by W3C in https://www.w3.org/TR/webcodecs-codec-registry/.

The registration requirements are request:

- Codec String and specification

- The codec string has certain requirements

- Each registration must include

  - Recognized codec strings

  - EncodedAudioChunk or EncodedVideoChunk internal data

  - AudioDecoderConfig or VideoDecoderConfig description bytes

  - Expectations for EncodedAudioChunk or EncodedVideoChunk

- Registration may include description of extensions to VideoEncoderConfig or AudioEncoderConfig dictionaries

- Candidate entries must be announced by filing an issue in the WebCodecs GitHub issue tracker so they can be discussed and evaluated for compliance before being added to the registry.

The audio codec registry includes flac, mp3, AAC, opus, etc., but no 3GPP codec.

As an example for AAC, go here: https://www.w3.org/TR/webcodecs-aac-codec-registration/

The video codec registry includes AV1, H.264/AVC, H.265/HEVC, VP8, VP9

As an example HEVC codec registrations, please go here: https://www.w3.org/TR/webcodecs-hevc-codec-registration/

# Media Service Enablers – see TR 26.857

## 6.1	General Concepts

In TR 26.857, the concept of 5G Media Service Enablers is introduced.

The basic concept of the Media Service Enabler is to support third-party applications to make use of advanced functionalities provided by the 5G System, combined with additional well-defined client and network functionalities for media services: an MSE enables improved media services.

In implementations and deployments, such packaged functions are typically referred to as a Software Development Kit (SDK) and they are usable by applications through well-defined APIs. A few potential properties of a Media Service Enabler are provided:

-	A set of functions that may be used to deploy applications that can make simple use of 5G System functionalities.

-	A set of robust features and functionalities which reduce the complexity of developing applications.

-	Functions to leverage system and radio optimizations as well as features defined in 5G System (5G Core Network and 5G NR).

-	Usability of the set of functions by well-defined and well-documented device APIs.

-	Provision of network interfaces to connect to the 5G System.

-	A testable set of functions. Testing and conformance may be addressed outside 3GPP, for example by a Market Representation Partner (MRP) such as 5G-MAG or by an industry forum.

-	Guidelines and examples to make use of the set of functionalities provided by an MSE.

A general initial idea on how to define Media Service Enablers is documented below:

-	Combine functions defined in 3GPP (for example a codec) and/or reference technologies defined outside 3GPP, for example in MPEG or Khronos, and provide relevant subsets and profiles of these.

-	Include mandatory, recommended and optional functions.

-	Define signaling and capability negotiation for all functions.

-	Specify requirements for client and network functions, as needed.

-	Include relevant functions such as QoE metrics and KPIs.

Providing a Media Service Enabler in this form has several benefits:

-	The Application Provider has a set of functions that can be easily accessed in the same way that device functions are accessed today, namely through well-defined device APIs. The Application Provider can also use regular IP connectivity to operate its application.

-	For the MSE developer, the focus is on providing a well-defined set of functions that are exposed to the application through MSE-1 and MSE-2 on the network side, and via MSE-6 on the UE device side.

-	The MSE developer may provide the MSE Application Function and Application Server as well as the MSE Client. In this case, the primary interoperability aspects are at reference points MSE-1 and MSE-6.

-	In another case, the network functions for MSE may be provided by a 5G System operator. In this case the MSE Client and MSE AF are expected to also implement the functions and interoperability defined at reference points MSE-4 and MSE-5.

## 6.2	MSE Client specification

In TR 26.857, an MSE client specification is provided

9.1	Overview of the MSE API Calls

9.2	Functional description

-	Uses reference pre-requisites, user plane functionality, control plane, and client API.

-	Defines states of the MSE client in relation to the application. Examples for state are IDLE, REGISTERED, ACTIVE, etc. State changes may occur through or by information received through MSE-6 the network interface.

-	A set of client-internal reference parameters that are changed based on either configuration or API calls through MSE-6 or by information received through the network interface MSE-4 or MSE-5.

-	Metrics, data and KPI collections, for example to be provided to analytics servers.

9.3 MSE Client API methods and parameters

-	Different methods that allow the application to communicate with the MSE client. For each method, the following information is provided:

-	A high-level description of the method.

-	An example call flow.

-	The parameters that are exchanged as part of the API call.

-	The usage of the API by the application.

-	The MSE Client actions, including pre and post conditions.

-	Configuration

-	Capabilities

This API typically includes functionalities such as configurations, settings, notifications, events, data and status query as well as functional methods. As an example, the API may provide the ability to query metrics and KPIs,

-	Optional capability discovery: discovery of the capabilities supported by an implementation including the additional configuration parameters specific to that MSE implementation

-	Specification using a well-defined language, for example C or IDL,

## 6.3	Beyond the MSE Specification – guidelines, tests and reference implementations

Beyond the MSE specification, and as indicated in clause 6.3, the following aspects are considered in the annexes for the specification template:

-	Guidelines for application developers.

-	Guidelines for MSE implementers and reference implementations.

-	Device API instantiations.

-	Conformance Test Suite.

Such efforts are not necessarily suitable for 3GPP working processes. Hence, collaboration with other organizations, such 3GPP market representation partners (MRPs) or open-source projects may be considered. The annexes indicated above may initially contain only considerations that can be used by third parties in order to develop their own implementations, guidelines, test frameworks and reference implementations.

As an example, the development of a reference implementation of MSE Client and network functions can support developers and Application Providers to quickly gain access to newly defined functionalities. This is, for example, shown in figure 6.4-1 for which reference implementations of the MSE are used as part of a reference, demonstration or production application. In this case, the reference implementation makes use of existing device functions and 5G System functions. As an example, the 5G-MAG reference tools https://www.5g-mag.com/reference-tools provide an approach to developing such reference implementations.

Figure 6.4-1: MSE Reference Implementation

As another example to support the specification development, a conformance test suite may be developed in order to test the 3GPP-defined APIs and conformance for correct implementation. A framework for this is provided in figure 6.4-2.

Figure 6.4-2: Test/Conformance Framework for MSE Client Implementation

In this case, a test framework is developed in order to test the functionality of the MSE Client implementation. If all tests are passed, the MSE Client may be considered conformant to the specification. Such an approach may be even extended to create an adopter program, i.e. providing a process that allows an MSE implementation to officially claim support of the MSE specification by having verified that the all tests have been passed.

While 3GPP is not in a position to mandate such a conformance regime, it is highly recommended to consider the potential benefits of supporting third parties in developing suitable test and conformance programs.

## 6.4	Relation to VOPS and FS_ACAPI work

3GPP codecs can be viewed as media service enablers.

Key aspects are the ability to

- Use the encoder and decoder independently of a specific service

- Allow to use and configure the encoder and decoder through APIs

- Consider the ability to create unit tests

These aspects should be taken into account when defining codecs, decoding and encoding capabilities.

# Interface and Conformance View

A first view of interfaces and conformance is provided in the below diagram.

# Proposal

It is proposed to consider in context of the VOPS work item and FS_ACAPI study item:

- To register relevant 3GPP audio codecs as web codecs including

  - Definition of codecs strings

  - Definition of audio chunks

  - Definition of audio decoder config

  - Definition of extensions of the audio encoder config (bitrates, modes, error resilience)

  - Definition of extensions of the audio decoder config

- To support registration of relevant 3GPP video codecs (including AVC, HEVC, MV-HEVC) together with MPEG as web codecs including

  - Definition of codecs strings

  - Definition of video chunks

  - Definition of video decoder config

  - Definition of extensions of the video encoder config

  - Definition of extensions of the video decoder config

- Consider to document codecs with MSE principles and support testing of codecs.

It is proposed that in VOPS, an informative Annex is created and in FS_ACAPI the TR includes such information. However, as pull requests to the registry are needed, it is proposed to create a registry as part of 3GPP/ETSI githubs.

The registration process of codecs should be initiated officially by 3GPP through an LS at appropriate time.