3GPP SA4 #129-e 	S4-241497

Online, 19-23 August 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] Update on the PDU Set Size information correction by indicating the remaining PDU Set Size in RTP header extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 08/19/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This revise solution #7: PDU Set Size information correction by indicating the remaining PDU Set Size in RTP header extension The change makes the solution compatable with the semantics of the remaining PSSize (rPSSize) field in Rel-18 for the first PDU of a PDU Set. That is, the revised rPSSize and the PSSize defined in Rel-18 will give the same value and has the same meaning for the first PDU in a PDU Set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The remaining PSSize now includs the size of the PDU that carries the PSSize field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of backward compatability with Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 6.x	Solution #x: PDU Set Size information correction by indicating the remaining PDU Set Size in RTP header extension

### 6.7.1	Key Issue mapping

This maps to Key Issue #1.

### 6.7.2	Description

According to the current TS26.522, all PDUs of a PDU Set carry the same information in the PDU Set Size (PSSize) field in the RTP header extension for PDU Set marking. Repeating the same information in general is a waste of resource.

We propose to reuse the PSSize field, giving it a new interpretation or a new name, to indicate the remaining PDU Set Size (rPSSize) which includes the size of the PDU carrying the PSSize field, i.e., how many bytes the PDU Set has  from this PDU to the last PDU of the PDU Set. For the first PDU of a PDU Set, the rPSSize and the PSSize fields indicate the same value and have the same meaning. As an example, if the PSSize is 4000 bytes consisting of 4 PDUs with 1000 bytes each. The rPSSize field of the first PDU in the PDU Set will indicate 4000 bytes, the same as the PSSize does. The rPSSize field of the second PDU in the PDU Set will indicate 3000 byes instead of 4000 bytes that would be indicated by PSSize.

This proposal allows a router to compare the indicated size (i.e., the size from the perspective of the application sender) of a PDU (by taking the difference in the rPSSize between two adjacent PDUs) and the observed size of the PDU and derive the PSSize error due to network operations such as NAT46/64 that alter the PSSize. Using the same example, we assume that there is NAT46 in the network unknown to the packet source. The router can derive the indicated size of the first PDU (PDU Sequence Number or PSN=0) by taking the difference between the rPSsize carried in the 1st PDU (PSN=0) and the rPssize carried in the 2nd PDU, as shown in Figure 6.7.2-1. The difference will be 4000 – 3000 = 1000 bytes. On the other hand, the router observes tha the 1st PDU has an actual size of 1020 bytes. Then, the router knows that the network has added 20 bytes to the packet, and it needs to add 20 bytes for each PDU (this means that the number of PDUs in the PDU Set (NPDS needs to be signaled in the PDUs) in the PDU Set to get the actual PSSize.

Figure 6.7.2-1 Deriving the indicated size of the 1st PDU by taking the difference in the rPSSize between the 1st PDU and the 2nd PDU

One may argue that in the event of out-of-order delivery, with the current specification TS 26.522, if every PDU carries the PSSize, the first received PDU (whose PSN may not be equal to 0) will provide the PSSize information needed by a router. This is not necessary. First, for low-latency applications, a reasonable design should not lead to severe out-of-roder delivery.

Second, if every PDU carries its respective rPSSize, the router can use the PSN filed in the RTP header extension together with the rPSSize to estimate the PSSize. As more packets arrive, the estimate will get more accurate.

Third, even if the packets arrive at a router out-of-order, it may not have a problem. To see this, consider two cases. Case (1) all packets arrive in an ideal burst (i.e., all PDUs arrive within a time slot or a transmission time interval (TTI) equal 1ms): in this case, the router performs the PSSize correction described earlier, but in a very short duration of an ideal burst, offering even more lead time for scheduling. Case (2) all PDUs are evenly distributed in time until the next PDU Set: in this case, not being able to get the PSSize in the first time slot does not necessarily prevent scheduling the PDUs arriving in the first time slot. Under rare conditions (e.g., the first arrived PDU has the largest PSN among all PDUs of the PDU Set), the rPSSize obtained in the first time slot is less than the total size of the PDUs in the first time slot, the unscheduled PDUs can still be scheduled in the next time slot. What really matters is to timely deliver the PDU Set as a whole, an observation that motivated the notion of Nominal PDU Set Delay Budget (NPSDB) (see clause 6.20 of [6]). As long as the last few PDUs are scheduled on time, which is guaranteed, the timely delivery of the whole PDU set is not affected.

To implement this solution, we can replace the PSSize in the RTP header extension for PDU Set marking with rPSSize, updating the semantics of the field without changing the format.

Alternatively, without changing the name, the PSSize field can be re-interpreted as the remaining PSSize during session setup.

Pros: compared to other solutions, it doesn’t incur additional signaling in the user plane or the control plane beyond what is needed for supporting the PSSize in the current TS 26.522.

Cons: it assumes that the intermediate routers (e.g., UPF, gNB) use the rPSsize value to correct the PSSize, but when a router serves a large number of traffic flows

-	such computation may not be scalable,

-	the router needs to maintain a state variable (to store the rPSSize in the most recently received PDU) in the memory,

-	it needs the inclusion of the optional Number of PDUs in a PDU Set (NPDS) field in the RTP header extension for PDU Set marking,

-	this method cannot correct the PSSize when the first PDU is received, and it needs to wait for at least another PDU before it can correct the PSSize

-	this solution would not work with current Stage 2 work when the first PDU of the PDU set is not delivered first (i.e., in order) and requires stage 2 update to address this issue.

-	it also requires changing the semantics of PSSize as defined in TS 26.522

# * * * * End of 1st  change * * * *