Title:		          Proposed Updates to the Draft TR 26.933

Source:	Panasonic Holdings Corporation

Document for:	Discussion & Agreement

Agenda item:	7.6

Introduction

The source has reviewed Draft TR 26.933[1] and identified potential gaps in some of the clauses for completion of the TR. This contribution proposes updates to address these gaps and provides suggestions for revising the clauses in the TR.

Proposed Updates to TR 26.933

Table of contents of TR 26.933 is proposed to be updated as follows.

1. Scope

2. References

3. Definitions of terms, symbols and abbreviations

4. Size and structure evolution of UEs [Change in title]

5. Microphones used in audio capture

6. Audio Capture format [New clause split from the current clause 6]

7. Microphone design for UEs [New clause split from the current clause 6]

8. Signal Processing

9. Example audio capture processing solutions

10. Conclusion and Recommendations

4 Size and structure evolution of UEs [Renamed the title]

Clause 4 is titled 'Factors of Different UE Categories Related to Audio Capture,' but the corresponding subclause limits the discussion to size and structure of the UE. It might be more appropriate to rename Clause 4 to 'Size and structure evolution of UEs.’ or with any other suitable name.

4.8 Conclusion [New Addition]

Due to different structures and sizes in each UE category, possible microphone design (placement, distance between microphones) will be different. Such different microphone designs will be discussed in clause 7.

5 Microphones used in audio capture [Re-arrange subclauses]

5.1 Introduction

5.2 Transducer Type

5.3 Directional Microphone

5.4 Binaural acoustic simulation

5.5 Conclusion

Note:

- Merge sub-clause 5.2.5 Summary of transducer text to 5.5.

- Remove 5.6 (Other components used in audio capture) in the TR as it is not related to clause 5 microphones used in audio capture.

5.5 Conclusion [New addition]

Following advancements in microphone devices, it grants more possibilities for UEs integrated with the miniature microphones to capture audio signals, specifically, the digital MEMS microphone includes preamps, ADC, and clocks, which makes it can directly output digital signals, so audio capture has become significantly more convenient. This innovation is particularly beneficial for small-sized devices according to its size and consistent performance. These miniature microphones can also be used to generate immersive audio in combination with signal processing.

6 Audio capture format [Split clause 6]

Text in clause 6.1 stereo capture, 6.2 spatial audio capture focused mostly about audio capture format, so move the 6.1 and 6.2 under a new clause titled “6. Audio Capture Format”.

Text in 6.3 Microphone design for the terminals discusses the microphone design for different UE types, so 6.3 can be renumbered to a new clause “7. Microphone Design for UEs”.

6.3 Summary [New addition]

Principles, configurations, and audio quality factors for achieving immersive capture were discussed. Stereo capture involves recording two audio signals to create a sound image with spaciousness, direction. Spatial capture uses at least 2 microphones to accurately reproduce audio scenes with spatial cues, such as direction and elevation, for immersive listening.

7.6 Summary

MEMS microphones are the default choice for the UEs discussed in the above sub-clauses. Each UE category typically includes a minimum of two microphones, enabling multi-channel capture, which may offer improved audio quality, better spatial awareness, enhanced noise suppression, and sound localization compared to a mono microphone. However, to achieve optimal sound quality with a multi-microphone setup, several factors should be considered, such as microphone placement, type, characteristics, user interaction and handling, etc.

Some of the UE designs has narrow spacing between microphones, which may result in poor spatial characteristics and may not provide the best sound quality for multi-channel capture. For UE categories such as smartphones, users can hold and operate their devices in various ways, which can obstruct microphones and affect audio quality, stereo imaging, or spatial awareness. Additionally, variations in hardware quality across different devices and UEs can also lead to inconsistent audio performance, even within the devices of the same UE brand.

To fully benefit from 3GPP IVAS (Immersive Voice and Audio Services) codec for the UEs of various categories, recommendations per UE category such as spacing between microphones, type, characteristics, etc., will be needed without restricting the manufacturer’s flexibility in UE hardware design.

Note: Above summary was agreed at the 3GPP SA4-128-adHoc (SA4aA240045)

8.5 Conclusion [New addition]

The raw microphone signals are used for generating the expected format audio signals using MASP to get satisfied audio signal quality, some necessary signal processing modules such as AEC and possible NS are needed for enhancing the quality of the raw microphone signals.

10. Conclusions and Recommendations

Different UE terminals and their corresponding designs discussed in the above clause are grouped into following categories:

Category A: Mobile, Tablets, Laptops, AR Glasses, Headphones

Category B: Watches

Category C: Car.

The market need for immersive voice communication (more than mono) for Category B terminal is unknown. if Category B is used as a peripheral device, it can be considered a part of Category A. Additionally, the internal acoustics of Category C are entirely different from those of Categories A and B. Sufficient study on Category C was not captured in this TR, so it can be deferred for future study.

Modern Category A devices (Mobile, Tablets, Laptops, AR Glasses, Headphones) have multi-microphone setups, but they still do not support immersive voice and audio capturing for communication. Given that the 3GPP IVAS codec is available for market adoption, the need for supporting Immersive voice and audio capturing for IVAS must be identified. The following points attempt to identify what may be needed for IVAS input from the raw microphone signals captured by Category A devices.

IVAS supports multiple audio formats, such as stereo, multi-channel, Independent Streams with Metadata (ISM), Scene-based Audio (SBA), Metadata Assisted Spatial Audio (MASA), and Binaural. Given that Category A devices are mostly used in real-world environments, raw multi-channel microphone capture signals may not be suitable as input to the IVAS codec. As such, following are identified to missing in the Category A devices.

Before supplying the raw multi-microphone captured signal, support for equalization filter design may be needed to achieve balanced frequency response characteristics across all microphones.

As Category A device orientation can be changed during capturing, enhancement such as orientation adjustments may be needed.

Support for converting multi-channel raw input to binaural and/or stereo, especially for headphone and/or stereo loudspeaker consumption.

When a binaural microphone is used for capturing, support may be needed for binaural to stereo conversion (for loudspeaker consumption at the receiver side)

Support for multi-channel AEC, noise reduction and beamforming to improve the captured signal quality.

Conversion of raw multi-channel input to MASA supported format for IVAS MASA voice communication

As the Category A devices are available in the different price range, at least support for stereo capturing may be recommended.

End–to–End Latency and respective latencies for enhancement modules needs to be identified and defined.

Note:

1. Latency is a critical element but was not studied in the DaCED study phase.

2. Premium headphones often come with built-in processing capabilities, such as active noise cancellation (ANC), and other audio enhancement features. These features are processed directly within the headphones, which can reduce the need for additional processing in UE terminals. On the other hand, many other headphones, particularly those at lower price points, may not support or process these enhancements independently. As a result, the UE terminal may still need to perform some audio processing tasks to ensure that the audio quality meets certain requirements. So, a minimum quality may need to be defined for ensuring suitable captured signals.

3. UEs should be able to negotiate the best appropriate format depending on the capturing and rendering capabilities, and network environment characteristics. Support for signal negotiation needs to be defined.

Conclusion

This document offers suggestions for improving the TR 26.933 [1] and proposes updates to the respective clauses. The source requests agreement and adoption of the proposed clauses.

Note: Source has ported the proposals to the draft TR, for convenience. PF in the attachment

References

- TR 26.933 v1.0.1: Study on Diverse Audio Capturing system