3GPP TSG-SA4 Meeting #129-e	S4-241572

# Online, , 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0019 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New clause 5.X QUIC-based Media Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FS_AMD includes a new topic about opportunities with QUIC for segmented streaming (topic “m)”). After updating the clause on HTTP/3 (clause 5.4) of TR 26.804 with up-to-date information since some parts are common to the new topic, we introduce a new section with QUIC-specific media delivery aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - Add clause X.X QUIC-based Media Delivery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1

## 5.X	QUIC-based media delivery

### 5.X.1	Description

#### 5.X.1.1	General

QUIC is a network protocol built on top of UDP [32]. It is widely available and its impact on media streaming requires further study. In this Section we study QUIC-specific media streaming, not necessarily based on HTTP/3 that is studied in Section 5.4.

A QUIC server can establish a connection with a client, and within this connection transport data through multiple streams. A QUIC connection is established faster than a TCP one, therefore reducing initialization time. Additionally, the streams within a connection are independent to each other and as such there is no head-of-line blocking issue. As a result, when retransmissions occur (e.g. in a lossy connection) a stream is not affected by losses occurring to another stream. Finally, QUIC supports prioritization capabilities on a stream level.

Note: Further content to be provided

### 5.X.2	Collaboration Scenarios

Note: Further content to be provided - in TDoc S4-241571

### 5.X.3	Architecture Mapping

#### 5.X.3.1	General

Mapping against 4.1.2 Generalized Media Delivery architecture of TS26.501 [15] is to be provided.

Note: Further content to be provided

### 5.X.4	High-Level Call Flow

#### 5.X.4.1	General

Note: Further content to be provided

### 5.X.5	Gap Analysis and Requirements

#### 5.X.5.1	Introduction

Even though QUIC solves many issues compared to TCP (and HTTP/3 comparing to HTTP2/ respectively), there are some open issues and shortcomings when it comes to media content delivery

#### 5.X.5.2	QUIC API

An application cannot directly access the QUIC API – since it is a transport layer protocol. Typically access to QUIC is achieved by the usage of HTTP/3 or WebTransport. For that reason, the features available to the application are the features supported by the HTTP/3 or WebTransport APIs. This approach facilitates application development and integration of new features with the tradeoff of limiting management of the connections and streams.

#### 5.X.5.3	Stream prioritization

Even though QUIC supports prioritization on a stream level, the way this prioritization is applied is not standardized and it is left to the implementors. Additionally, it is handled by the sender, and as such neither the network, nor the client are aware of the applied mechanism.

#### 5.X.5.4	Connections and Streams management

Since most applications use QUIC because they are using HTTP/3 the mechanism that the connections and the streams are handled are opaque and the application does not have control over them. The alternative to HTTP/3, i.e. using WebTransport does not provide more flexibility either.

Note: Further content to be provided

### 5.X.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

### 5.X.7	Summary and Conclusions

The study of this Key Issue has explored the ways in which QUIC can be deployed to support the 5G Media Streaming architecture, and the potential open issues arising from this deployment.

Note: Further content to be provided

END OF CHANGES