3GPP TSG-SA4 Meeting #129-e	S4-241616

Electronic Meeting, 19th Aug – 23rd Aug 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0021 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Multi-Access Media Delivery – Gap on dynamic policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Document S4-241252 was endorsed during SA4#128 meeting with initial draft on multi-access media delivery. The study of this topic is ongoing. This contribution discusses existing policy management aspects supported in 5G Media Streaming, and proposes a gap when multi-access media delivery is chosen for the media streaming session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a gap analysis and requirement for multi-access media delivery study based on existing dynamic policy support |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One of the study topics will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TS 26.501 and TS 26.510 specify the stage-2 and stage-3 procedures and data model definitions for dynamic policy feature in 5G Media Streaming. Specified as part of high-level procedure for service provisioning in clause 5.3.2 of TS 26.501, when the dynamic policy feature is offered and selected, the 5GMSd Application Provider specifies a set of policies which can be invoked for the unicast downlink streaming session. The data model for PolicyTemplate resource in specified in clause 8.7.3 of TS 26.510. The UE becomes aware of the selected policies in the form of a list of valid Policy Template Ids. When the Media Session Handler intends to activate dynamic policy for the media session, the MSH sends a Dynamic Policy API request to the 5GMSd AF. As specified in clause 5.7.3 of TS 26.501, the request includes at least the Provisioning Session identifier, the Service Data Flow Description(s) and the Policy Template identifier to be applied to the described transport session. The details of the Dynamic Policy API are specified in clause 9.3 of TS 26.510, and the data model for DynamicPolicy resource is specified in clause 9.3.3 of TS 26.510. The “applicationFlowBindings” property in the DynamicPolicy data model specify the bindings between application flows at reference point M4 managed within the scope of the Dynamic Policy Instance and their network QoS requirements. This property includes three sub-properties that allow for specification of application flows to which the dynamic policy QoS specification is to be applied: - componentIdentifier that references a particular service component in the Policy Template - applicationFlowDescription which provides a specification of an application flow to be used by the 5G Core for application traffic identification purposes - qosSpefication that provides network QoS requirements for the application flow(s) described by applicationFlowDescription The applicationFlowDescription type is specified in clause 7.3.3.2 of TS 26.510 which includes the following information: - filterMethod: Of type SdfMethod (details in clause 7.3.4.2 of TS 26.510), provides details about how the Service Data Flow is described - packetFilter: Of type IPPacketFilterSet (details in clause 7.3.3.1 of TS 26.510), provides details about different properties to describe a flow - domainName: Description of application flow in terms of FQDN of Media AS - mediaType: Type of media carried by this application flow - mediaTransportParameters: Of type ProtocolDescription (specified in clause 5.5.4.13 of TS 29.571), describes set of media transport protocol parameters to be used by the 5G Core for the purpose of PDU Set identification and/or end of data burst detection on this application flow. In the context of Multi-access using ATSSS architecture specified in TS 23.501, if the Media Streaming session is conveyed over a Multi-Access PDU Session, the application flow between the Media Access Client and the Media AS (5.g., 5GMS AS) may use two different access networks. Clause 5.32.6 of TS 23.501 specifies the supported steering functionalities in an example UE model with ATSSS. For ATSSS procedures, the UE may be provided with 5 different IP addresses – one IP address/prefix for the Multi-Access PDU session (allocated regardless of type of steering functionality), two IP addresses/prefixes called the “MPTCP link specific multipath” addresses (if UE and network agree on using MPTCP steering functionality), and two IP addresses/prefixes called the “MPQUIC link-specific multipath” addresses (if UE and network agree on using MPQUIC steering functionality). The MPTCP link specific multipath addresses and the MPQUIC link specific multipath addresses may not be routable via N6. When UE and network agree to use a Mult-Access PDU Session during a Media Streaming session, it is unclear how the dynamic policy feature specified in TS 26.501 and TS 26.510 work. It is unclear - If M4 application flows are carried over two access networks, what does activate dynamic policy with QoS requirements mean – whether one, or more, or all access paths are to be applied with requested QoS? - Is it feasible to request application of QoS for a subset of access paths over a specific access network? - Are any enhancements to the applicationFlowDescription type necessary to support identification of M4 application flows over specific access networks? To study the above questions, propose to add the following to the gap analysis and requirements clause 5.15.5: - Whether and how dynamic policy procedures and data model definitions specified in TS 26.501 and TS 26.510 are to be enhanced to support M4 application flows in a Multi-Access PDU session. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-241252: Endorsed CR on initial draft for multi-access media delivery |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

## 5.15	Multi-access media delivery

### 5.15.1	Description

#### 5.15.1.1	Introduction

Media streaming applications traditionally obtain content from a single source over a single path within a network. This imposes several limitations:

1.	Performance is constrained to that of the source and path chosen. Any limits on network bandwidth and latency between the client and that source are directly translated to the client’s achievable Quality of Service (QoS) and Quality of Experience (QoE).

2	Disruptions or degraded performance caused by the source in use or on any of the network links between the client and source can lead to poor user experience, often in the form of lower playback quality, rebuffering, or complete playback failure.

This Key Issue considers integration of different technologies into the 5G Media Streaming System that addresses these, and similar, issues by allowing media streaming applications to efficiently access content across multiple access networks. Different client implementations may then beneficially use the content on these multiple access networks either serially or concurrently, potentially guided by the service or network provider. Study of integration of different technologies into the 5G Media Streaming System is of relevance to address content provisioning, content hosting, impacts on user plane reference points M2 and M4, and on media session handling at reference point M5 as well as potential benefits in terms of quality and resource usage.

Challenges that multi-access architectures aim to address may include:

1.	Disruptions to QoS and QoE resulting from degraded performance or loss of availability of one or more network interfaces/access networks. An example is disruption such as significant delays and loss of throughput caused during the process of switching from one access network to another as transport layer connections are migrated to new endpoint addresses on a different access network, or are destroyed and need to be re-established.

2.	Inability to efficiently utilise multiple network interfaces/access networks concurrently to achieve a target QoS or QoE. An example is the inability of a UE to effectively utilise its connection with a secondary, reliable but high-cost, 5G access network in support of the primary, unreliable but inexpensive, access network using Wi-Fi.

#### 5.15.1.1	Multi-access using more than one USIM

A UE contains at most one USIM. Multiple UEs may be combined in a single device to form a composite terminal that is able to access more than one access network.

Editor's Note: Common scenario in media production where 5G modem units provide multiple SIM card slots intended for concurrent use. (Smartphone UEs with multiple slots aren't typically able to use more than one at the same time.) This Key Issue should also study what (if any) changes to the 5GMS System are needed to take advantage of this. Unlikely to be transparent to the 5GMS Client, requiring the use of multipath transport protocols, or applications specifically written to work with multiple paths.

#### 5.15.1.2	Multi-access using ATSSS

Clause 5.32 of TS 23.501 [23501] describes ATSSS (Access Traffic Steering, Switching, and Splitting) an optional feature supported by the UE and 5G Core network for multi-access. Some of the key principles this feature defines that are relevant for our study are:

1.	The ATSSS feature enables a Multi-Access PDU Connectivity Service allowing for the exchange of PDUs between the UE and a Data Network by simultaneously using one 3GPP access network and one non-3GPP access network via two independent N3/N9 tunnels between a PDU Session Anchor UPF (PSA UPF) and the RAN/AN.

NOTE 1:	The limits on the number and type of access network refer to Release 18 and may differ in subsequent releases.

2.	The Multi-Access PDU Connectivity Service is facilitated by a Multi-Access PDU (MA PDU) Session that may have User Plane resources on two access networks. In the context of the generalised media delivery architecture specified in TS 26.501 [15]:

-	If conveyed over an MA PDU Session, the application flow between the Media Session Handler and the Media AF (e.g., 5GMS AF) at reference point M5 may use two different access networks.

-	If conveyed over an MA PDU Session, the application flow between the Media Access Client (e.g., Media Player or Media Streamer) and the Media AS (e.g., 5GMS AS) at reference point M4 may use two different access networks.

3.	The UE is supplied with policy rules ("ATSSS rules") by the network for deciding how to distribute uplink traffic across multiple access networks. Similarly, the UPF anchor is supplied with policy rules ("N4 rules") by the network for deciding how to distribute downlink traffic across the two N3/N9 tunnels and the two access networks. The network entity configuring ATSSS rules and N4 rules is the SMF. The SMF may map PCC rules from the PCF to create these ATSSS and N4 rules.

4.	The UE indicates its support for ATSSS (steering functionalities and steering modes) in the PDU Session Establishment Request that is sent to request a new MA PDU Session.

5.	If the UE requests a network slice instance, the same S-NSSAI is allowed to span both access networks.

NOTE 2:	Support for  QoS when PDUs are conveyed over a PDU Session belonging to a network slice that spans non-3GPP access network is unknown.

6.	For QoS support, the same 5G QoS model used for conventional PDU Sessions also applies to MA PDU Sessions, i.e. QoS Flow is the finest granularity of QoS differentiation. However, QoS Flow is access-agnostic: the same network QoS applies to each of the different access network comprising the MA PDU Session, i.e. the same QoS is available across two different paths in different access networks. The network (SMF) may provide QoS rules to the UE via one access network that are used for both the 3GPP access network and non-3GPP access network.

-	In the context of the generalised media delivery architecture, application flows at reference point M5 and/or M4 using a MA PDU Session may have similar network QoS as when they are transmitted via the 3GPP access network alone.

NOTE 3:	Support for PDU Session QoS when PDUs are conveyed over a non-3GPP access network is unknown.

7.	The network may provide Measurement Assistance Information to the UE and/or UPF to assist them in determining which measurements (packet round-trip time measurements, packet loss rate measurements) are to be performed before deciding how to distribute traffic across the two access networks.

8.	The ATSSS rules provided to the UE by the network contain information about the type of steering functionality to be used to distribute traffic across multiple access networks. Steering functionality is the functionality that can steer, switch, and split traffic across multiple access networks. From clause 5.32.8 of TS 23.501 [23501], the supported steering functionalities include:

-	Higher-layer MPTCP (Multipath TCP) functionality – The UPF provides MPTCP proxy functionality. Corresponding MPTCP functionality in the UE may communicate with the MPTCP proxy in the UPF to distribute and aggregate traffic across multiple access networks.

-	Higher-layer MPQUIC (Multipath-enabled QUIC) functionality – The UPF provides MPQUIC proxy functionality. The corresponding MPQUIC functionality in the UE may communicate with the MPQUIC proxy in the UPF to distribute and aggregate traffic across multiple access networks.

-	ATSSS-LL (ATSSS Low-Layer) functionality – The UPF allows steering, switching, and splitting of traffic across two access networks based on information from the IP layer and below.

9.	The ATSSS rules provided to the UE by the network indicate which steering mode is to be applied to matching traffic for each Service Data Flow (SDF). The steering mode determines how the matching traffic is to be distributed across 3GPP and non-3GPP access networks. Supported steering modes in Release 18 include:

-	Active-Standby: Used to steer matching SDF packets onto one access network (the "Active access") when this is available, and onto another (the "Standby access") when the Active access is unavailable.

-	Smallest Delay: Matching SDF packets are steered to the access network with smallest packet round-trip time.

-	Load-Balancing: Used to split the delivery of SDF packets between both the access networks if both of them are available.

-	Priority-based: Used to steer SDF packets onto an access network with a higher priority.

-	Redundant: Used to duplicate SDF packets on both access networks if both of them are available.

To support the operation of media delivery services specified in TS 26.501 [26501], TS 26.506 [26506], and TS 26.502 [26502] with multi-access, there is a need to first document clear potential issues to split, steer, and switch the M4 application flows of the above media delivery services based on methods specified in ATSSS architecture.

#### 5.15.1.3	Non-ATSSS Multi-access

UE connected to multiple access networks (whether they be a 3GPP and non-3GPP network, multiple disjoint 3GPP networks, etc.) inherently have the capability to deploy and utilize multi-access techniques without the need of ATSSS assuming support for the techniques and/or protocols are available at the application server. In these cases, the UE may establish multiple connections with the application server over different access networks such that media delivered by the application server is routed back over the appropriate access network.

### 5.15.2	Collaboration scenarios

#### 5.15.2.1	Multi-access media delivery in the 5G Media Streaming System

In this scenario, the 5GMSd Client is connected (either directly or via functions within the UE) to multiple data, or access, networks (e.g., an unmanaged Wi-Fi network and the 5G network). The client requests adaptive media streaming content from one or more 5GMSd Application Servers. The Client may choose to switch between access networks or use multiple simultaneously, as described in clause 5.x2. This allows the client to distribute network load across access networks, optimise costs, as well as improve QoS.

The client’s Media Session Handler discovers the URL of the Application Server either from a Media Entry Point provided by the 5GMSd Application Function (AF) at reference point M5d, or through a separate piece of metadata obtained from the 5GMSd Application Provider via reference point M8d.

Figure 5.x.2.1-1 shows the client communicating with a single Application Server through different data networks. Neither data network has direct communication with its peers. The 5GMSd AS communicates (minimally) with the Application Provider at reference point M2 and with the 5GMSd AF (not depicted) via reference point M3d. In some scenarios, the 5GMSd Client and 5GMSd AS may use lower-layer functionality and/or functions for multi-access delivery. In these cases, a single reference point M4 may be split among multiple access networks such that the 5GMSd Client and 5GMSd AS are unaware of the use of multi-access delivery.

Figure 5.x.2.1-1: Multi-access media delivery within 5G Media Streaming system

##### 5.15.2.2	Multi-Access media delivery using ATSSS

Figure 5.X.2.2-1 shows the collaboration scenario for multi-access media delivery using ATSSS. In this scenario, the multi-access delivery is supported by ATSSS functionalities in UE and the UPF. The type of ATSSS functionalities supported in the UE and UPF in this release are described in clause 5.X.1.1 of the present document. The ATSSS functionalities in the UE and UPF in the MNO network are responsible for steering, switching, and splitting of M4 application flows as summarised in clause 5.X.1.1 of the present document. The 5GMSd Client and the 5GMSd AS may be unaware of multi-access media delivery.

Figure 5.x.2.2-1: Multi-access media delivery using ATSSS Low-Layer Steering Functionality

### 5.15.3	Architecture mapping

Editor’s Note:	Based on existing architectures, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.15.4	High-level call flow

Editor’s Note:	Map the key topics to basic functions and develop high-level call flows.

### 5.15.5	Gap analysis and requirements

The following potential open issues are identified:

- Document potential open issues to split, switch, and steer M4 application flows based on methods specified in ATSSS architecture.

- Whether and how dynamic policy procedures and data model definitions specified in TS 26.501 [26501] and TS 26.510 [26510] are to be enhanced to support M4 application flows in a Multi-Access PDU session.

### 5.15.6	Candidate solutions

Editor’s Note:	Candidate solutions for identified key issue.

### 5.15.7	Summary and conclusions

# * * * * End of changes * * * *