3GPP TSG SA WG4  Meeting #129	S4-241538

# Online August 19 2024- August 23 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.822 | CR | - | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP] updates on text on KI #4 AL-FEC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP |  |  |  |  |  | Date: |  |  | 7-8-2024 |
| Category: |  |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update key issue text to add end to end and user experience aspects, minor correction on the text. Move the text from clause 5 to clause 6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to KI description, move technology parts to clause 6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clear base for evaluation regarding KI #4 and KI #3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 6.Y (new clause) 6.Z (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## CHANGE 1

## 5.4	Key Issue #4: AL-FEC awareness for PDU Set handling

### 5.4.1	Description

The application layer FEC mechanisms are widely used to improve packet transmission robustness in the presence of packet losses without going through packet retransmissions that can introduce delays   that violate real-time constraints. In the draft TR 23.700-70 [6] of FS_XRM_Ph2, Key Issue #1 is proposed to study the enhancement of PDU Set handling including Application-Layer Forward Error Correction (AL-FEC) encoded PDU Sets and specifically:

whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced.

To provide some background information, the basic idea is to expose AL-FEC related information to the NG-RAN via the control plane or user plane. The AL-FEC related information could be redundancy information or marking to differentiate among source and repair PDUs of a PDU Set etc. Based on the AL-FEC awareness, the NG-RAN may optimize the PDU Set delivery over the air interface accordingly (e.g., by discarding redundant PDUs of AL-FEC encoded PDU Sets). In the context of this cross-layer design, it is important to understand how to expose the AL-FEC information to the communication network (UPF, RAN) to enable intelligent resource allocation. Furthermore, there are intricate interactions to consider between the application and the network. In particular, the dropping extra PDUs of a PDU Set encoded with AL-FEC in a network, if any, may send a false signal to the application on the packet loss rate and the congestion level in the network. This may lead to undesired adaptation from the application such as increased redundancy ratio and reduced sending rate that can negatively impact end-to-end performance and user experience. It is thus important to understand the interactions between the application and the network in the case of AL-FEC and intentional packet dropping by the network and the impact on the media performance.

Therefore, it’s proposed to study:

-	benefits of AL-FEC awareness for PDU Set handling given application and network interactions in the context of 3GPP, if any;

-	whether and how to assist the 5GS to get aware of the AL-FEC;

-	for AL-FEC awareness for PDU Set handling, how to avoid/minimize the impact to the application layer, if any.

-	Benefits to the user experience and application end-end performance when enabling AL-FEC awareness

Editor’s Note: Collaboration with SA2/RAN2 is needed.

Editor’s Note: This Key Issue is based on the study on the Key Issue #3: Enhancements for application-layer FEC support.

### 6.Y	Solution #Y Analysis of AL-FEC awareness in 3GPP

#### 6.Y.1 Key Issue Mapping

This solution addresses key issue #4 on AL-FEC awareness for PDU Set handling and Key Issue #3: Enhancements for application-layer FEC support.

#### 6.Y.2 Description

#### 6.Y.2.1	Common attributes of AL-FEC deployments

AL-FEC codecs, as the schemes documented in Solution #5, are used in ensuring low latency media delivery for networks with bursty losses and RTT delays comparable or higher than the jitter buffer delay constraints of an application. AL-FEC is not an exclusive packet loss mechanism and may be used interchangeably with retransmissions (e.g., as documented in Solution #8 in section 6.8 as media source bit rate adaptation or other flow control mechanisms. AL-FEC encoding is in fact utilized in practice in supplement to RTCP NACK indication and the AL-FEC redundancy level is usually dynamically adapted [25], [26] to network conditions.

The advantage of AL-FEC over other schemes is that error recovery is proactively realizedby redundant packets and a decoder is able to recover from any packet losses without any additional transport-related delays (e.g., as for retransmissions-based mechanisms). However, the cost to pay for using AL-FEC is additional bandwidth utilization. A dynamic AL-FEC controller (or media optimizer [26]) ensures usually in practice an optimal trade-off between AL-FEC bandwidth utilization and QoS is achieved by balancing the bandwidth split between the source media content and the AL-FEC redundancy added. Figure 5.4.2.1-1 outlines potential operation points for an AL-FEC encoded media stream (e.g., a video stream) as adapted by a dynamic AL-FEC controller based on network conditions. Operation point (1) illustrates an unprotected media stream at 30 Mbps, operation point (2) illustrates an AL-FEC protected media stream with a high AL-FEC redundancy rate of 100% (i.e., equal bandwidth split between media and repair packets), and operation point (3) illustrates an AL-FEC protected media stream with a low AL-FEC redundancy rate of 25%.

Figure 5.4.2.1-1: Example operation points of AL-FEC encoded media

The network metrics and statistics that influence the operation of AL-FEC controller are usually diverse and may involve at least [23], [24], [25], [26]:

-	available bandwidth estimation/information

-	packet loss statistics

-	packet loss feedback

-	RTT delay estimation/information.

Congestion events impact therefore the operation of the AL-FEC dynamic behavior. At a high-level in low packet loss conditions the AL-FEC redundancy rate is reduced considerably (e.g., operation point (3) in Figure 5.4.2.1-1), or even eliminated (e.g., operation point (1) in Figure 5.4.2.1-1) as per application configuration and preferences. On the other hand, for some higher packet loss values, given that the link has necessary bandwidth, the AL-FEC redundancy rate is increased to provide more redundancy and protection against network losses. However, in case congestion events persist and available bandwidth degrades the AL-FEC redundancy rate is usually backed-off and the media source rate is adapted to a lower source rate to account for the lower bandwidth available.

The high-level control loop detailed above implies frequent network conditions monitoring (e.g., every second, [26]) and corresponding reactive AL-FEC redundancy rate and media source rate adaptation. This achieves an elastic and robust transport mechanism for low latency media delivery even over bursty lossy networks.

NOTE 1:	Typical bursty losses are usually comparable to the network conditions monitoring times (e.g., couple of seconds resolution) and may affect multiple frames in a row.

#### 6.Y.2.2	End-to-end transport perspective

Figure 5.4.2.2-1 illustrates the end-to-end perspective of AL-FEC encoded media streams over a 3GPP network. The Application Server (AS) is situated into a data network with potentially no QoS guarantees. It serves DL AL-FEC encoded XR media traffic to a UE connected to a 3GPP network. The DL traffic is transported over the data network to the UPF (ingested at reference point N6), then over the core network to the NG-RAN (ingested at reference point N3) and finally reaches the UE over the UE air-interface. Optionally, in case the UE is not the XR endpoint, it may relay over a tethered connection the media content to an XR tethered device.

Figure 5.4.2.2-1: End-to-end transport path for AL-FEC encoded PDU Sets.

The following points are worth remarking if AL-FEC awareness is enabled for NG-RAN to actively discard obsolete PDUs out of AL-FEC encoded PDU Sets:

-	the AL-FEC obsolete PDU discarding at NG-RAN may be perceived by applications as congestion events, unless applications are fully aware of the NG-RAN behaviour (e.g., based on configuration, e.g., QoS configuration, or other feedback mechanisms);

-	a fixed operation point at a static AL-FEC redundancy rate is not advisable from an information rate optimization perspective and media delivery standpoint since the AL-FEC redundancy will inefficiently utilize available bandwidth in detriment of the media source, as described above;

-	a dynamic AL-FEC behavior is preferrable;

NOTE 2:	Dynamic AL-FEC awareness signalling should be supported in supplement to PDU Set awareness, yet how to support it is FFS and may further involve SA2 and RAN2 coordination.

-	the AL-FEC obsolete PDUs discarding at RAN will in effect reduce the 5GS operating bandwidth over the Uu interface to the media source rate;

-	the AL-FEC encoding may protect against bursty packet losses over any of the data network, core network and air-interface link segments, yet the core network and air-interface link segments are part of the QoS flow architecture ensuring QoS guarantees (e.g., PSDB, PSER, or alternatively, PDB, PER);

-	it seems that AL-FEC is mostly effective against the data network link segment, or alternatively, any link segment in the path to N6 that is out of the scope of 3GPP QoS flow architecture;

NOTE 3:	How the AL-FEC encoding will impact the QoS configuration if NG-RAN discarding of obsolete AL-FEC PDUs is enabled is FFS.

-	the AL-FEC obsolete PDU discarding at NG-RAN cannot protect against losses on tethered links, if present;

The analysis and remarks above implicitly assume NG-RAN feasibility (e.g., fast determination of PDUs ACK/NACK in various RLC modes to enable obsolete AL-FEC encoded PDU discard) and net benefits (e.g., capacity gains/bandwidth savings vs. added complexity) in discarding obsolete AL-FEC PDUs. However, the latter assumptions should be further verified in coordination with RAN2.

### 6.Z	Solution #Z Real-time Communication Congestion Control Algorithms and AL-FEC

#### 6.Z.1 Key Issue Mapping

This solution addresses key issue #4 on AL-FEC awareness for PDU Set handling and Key Issue #3: Enhancements for application-layer FEC support.

#### 6.Z.2 Description

#### 6.Z.2.1	general

This clause introduces the congestion control algorithms for real-time communication, including those currently implemented in WebRTC [28] and two algorithms described in two IETF RFCs [29] [30].

Observations are made on these algorithms and AL-FEC.

#### 6.Z.2.2	Google Congestion Control (GCC)

Google Congestion Control (GCC) is one of the two congestion control algorithms supported in the WebRTC implementation. The algorithm determines a sending bit rate, which limits the total bit rate for RTP packets, RTCP packets and RTP retransmissions (if any) and drives the bit rate adaptation of the media codecs. It uses packet losses and delays as signals of network congestion in adjusting the sending rate. The packet loss feedback is based on RTCP reports. The delays are one-way delays measured by the “absolute send time” RTP header extension as described in TS 26.522, and the change in the one-way delay is computed to detect network congestion.

GCC has not been standardized. Although the informational IETF document [27] describes GCC, the description is outdated. In what follows, we present GCC based on the current WebRTC implementation [28].

GCC sets the sending rate to the lesser of a loss-based bandwidth estimation and a delay-based bandwidth estimation. That is, the sending rate Rt  at time t is set to:

Rt=min⁡(RLtk,RDtn)

where RLtk is the loss-based bandwidth estimation made at time tk when the lastest RTCP loss report (the kth) is received, and RDtn is delay-based bandwidth estimation made at time tn when the lastest one-way delay measurement is taken or reported.

Delay-based bandwidth estimation:

This is described in aimd_rate_control.cc, based on the detection of overuse. The one-way delay

RDtn=βRDtn-1,                                in 'decrease' state RDtn-1,                                           in 'hold' state RDtn-1+ (tn-tn-1)δ,      in 'increase' state

where δ is a constant equal to 1200 bytes/second/RTT, β<1 is a backoff factor. The state ‘decrease’, ‘hold’ and ‘increase’ are based on the slope of the change in the one-way delay as a function of time. The change in the one-way delay from packet group i-1 to packet group i is defined as

di= ti-Ti-(t(i-1)- T(i-1))

where ti is the arrival time of packet group i, and Ti is the departuture time of packet group i, ti-1 is the arrival time of packet group i-1, and Ti-1 is the departuture time of packet group i-1. The slope (termed trendline in the WebRTC source code) is estimated and compared with thresholds to determine the state.

Loss-based bandwidth estimation:

There are three versions of loss-based bandwidth estimation algorithms.

Version 0 (static thresholds):

The loss-based bandwidth estimation is calculated as:

RLtk=RLtk-11-0.5ptk,                        ptk>0.1 RLtk-1,                                       0.02≤ptk≤0.11.08RLtk-1+1000 bits/sec,           ptk<0.02

where tk-1 is the time when the previous RTCP loss report is received, ptk is the loss (fration of loss) received in the most recent RTCP loss report, and RLtk-1 is the loss-based bandwidth estimation at the time when the previous RTCP loss report is received.

Version 1 (dynamic thresholds):

This version is similar to version 0, except that it uses (1) dynamic thresholds are used, and (2) the factor for increasing the estimation is adaptive to RTT, instead of being a constant 1.08 in Version 1. The loss-based bandwidth estimation is:

RLtk=0.99RAtk,                                             pdectk>γdec αRLtk-1+1000 bits/sec,               pinctk<γinc RLtk-1,                                                   otherwise

where γdec and γinc are dynamic thresholds, γdec=(4 kpbsRLtk-1)1/2, γinc=(0.5 kpbsRLtk-1)1/2

pdectk is the probability for decreasing the bandwidth estimation, equal to the minimum of the average loss probability and the most recent loss probability,

pinctk is the probability for increasing the bandwidth estimation, equal to the average loss probability pavgtk if pinctk-1<pavgtk and equal to pinctk-1+(1-exp⁡(∆tw-1))(pavgtk-pinctk-1),

where  ∆t is time elaspsed, w is a time window,

0.99RA is the bit rate of the acknowledged transmissions,

α=1.02+0.061-minmax(minτ, τmax,τmin )-τmin τmax-τmin , 1

where τ is the RTT, τmin is a pre-configured minimum RTT and τmax is a pre-configured maximum RTT. With this dynamic threshold α, the increase in loss-based bandwidth estimation will be slower when the RTT gets higher.

Version 2 (maximum likelihood):

To calculate the loss-limited bandwidth, in a nutshell, the sender chooses a tuple of an inherent loss probability Pinherent_loss (loss probability induced by the channel error rather than network congestion) and a loss limited bandwidth  RL that maximizes the following objective function which is based on the logarithmic maximum likelihood of observing the number of packet losses:

U=i=0K-1wi(NllogPl(i)+Nrlog(1-Pl(i))+B(Nl+Nr))

where:

Kis the number of observations,

w=0.9 is a weight,

Nl is the number of bytes lost,

Nr is the number of bytes received,

Pli=Pinherent_loss+(1-Pinherent_loss)(Rti-RL)/Rti,

where Rti is the sending rate at the time ti of the ith observation,

B is a high bandwidth bais (that depends on RL),

i=0 is the index of the most recent observation and Kis the index of the oldest observation.

#### 6.Z.2.3	PCC

Performance-oriented Congestion Control (PCC) is the other congestion control algorithm supported in the current WebRTC implementation [28]. The sender adjusts the sending rate and observe the performance metrics including the delay and packet loss, and pick the sending rate that maximizes a utility function:

Ux,dτdt, L=xt-bxmdτdt-cxmL

where x is the sending rate, dτdt is the gradient of RTT τ, L is the loss rate, m=1 or 2, and t, b, c are coefficients.

#### 6.Z.2.4	NADA

Network-Assisted Dynamic Adaptation (NADA) is specified in RFC 8698 [29]. This algorithm considers delay, packet loss, and ECN marking as signals of network congestion. Furthermore, it converts packet loss and ECN marking to some equivalent penalty in terms of delay and forms an aggregate congestion signal. Specifically, the receiver calculates

xcurr=d+Dmark(pmarkpmark_ref )2+Dloss(plossploss_ref )2

where d is the equivalent delay after non-linear warping,

pmark is the estimated packet ECN marking ratio, pmark_ref is the reference packet ECN marking ratio,

Dmark is the reference delay penalty for ECN marking when packet marking is at pmark_ref,

ploss is the estimated packet loss ratio, ploss_ref is the reference packet loss ratio,

Dloss is the reference delay penalty for packet loss when packet loss is at ploss_ref.

The receiver decides whether the sender is to be in the accelerated ramp-up mode (rate update mode rmode=0) or in the gradual update mode (rmode=1). The sender should be in the accelerated ramp-up mode if there are no recent packet losses in an observation window (of 500ms) and there is no build-up of queueing delay. Otherwise, the sender should be in the gradual update mode.

The receiver also calculates the receiving bitrate brecv.

The receiver sends xcurr, rmode, and brecv to the sender.

The sender performs multiplicative increase in the accelerated ramp-up mode using the RTT and brecv, and performs additive decrease in the gradual update mode using xcurr.

#### 6.Z.2.5	SCReAMv2

Self-Clocked Rate Adaptation for Multimedia 2 (SCReAMv2) [30] uses packet losses, delay, and ECN marking as signals of network congestion.

A congestion window size limits the sending rate and is adjusted based on:

-	Packet losses: A packet loss causes the congestion window size to decrease, but not as much as a packet loss does in the case of TCP Reno

-	Queueing delay: The congestion window size decreases linearly when the average queueing delay exceeds a threshold.

-	ECN marking: When a classic ECN marking is received, the congestion window size decreases by a factor. When L4S ECN marking is received, the congestion window size decreases in proportion to the fraction of packets that are L4S ECN marked.

#### 6.Z.2.6	Summary of congestion control algorithms

The interactions between the application and the network are through signaling manifested by packet losses, queueing delay and ECN marking, as summarized in the table below:

Table 5.4.3.5-1: Comparison of congestion control algorithms

| Congestion Control Algorithm | React to packet losses? | React to ECN marking? | React to queueing delay? |
| --- | --- | --- | --- |
| GCC (with loss based bandwidth estimation v0) | No, if the packet loss rate is between 2% and 10%; yes otherwise. | No | Yes |
| GCC (with loss based bandwidth estimation v1) | Yes | No | Yes |
| GCC (with loss based bandwidth estimation v2) | Yes | No | Yes |
| PCC | Yes | No | Yes |
| NADA | Yes | Yes | Yes |
| SCReAMv2 | Yes | Yes | Yes |

Observation 1: All congestion control algorithms for real-time communication in Table 5.43.5-1 use queuing delay (among other metrics) as a signal of network congestion.

Observation 2: Although all congestion control algorithms for real-time communication in Table 5.4.3.5-1 use packet losses as a signal of network congestion, one algorithm is not sensitive to packet losses when the packet loss rate is within the range [2%, 10%].

Observation 3: Two of the six congestion control algorithms for real-time communication in Table 5.4.3.5-1 support ECN marking.

Although WebRTC currently implements only GCC (with three versions) and PCC, it does not prevent one from adding other congestion control algorithms such as NADA and SCReAMv2. Since these algorithms support ECN marking, we have the following observation.

Observation 4: It is possible to add ECN marking support to WebRTC for congestion control. Updates of RFCs is required to ensure interoperability.

#### 6.Z.2.7	Packet loss rate calculation for AL-FEC

When AL-FEC is used, the source packets and the repair packets may be sent in a single RTP stream (identified by an SSRC). This is the case for ULPFEC in the WebRTC implementation [28].

Alternatively, he source packets and the repair packets may be sent in different RTP streams (identified by different SSRC’s) within the same RTP session. This is the case for FlexFEC in the WebRTC implementation [28]. The packet loss rate is calculated individually. The packet loss rates are then combined to form a single packet loss rate as an input to the congestion control algorithms.

Observation 5: For AL-FEC, the packet loss rate on the source packets and that on the repair packets can be calculated separately. RFC 5109 [13](Clause 12) give congestion considerations. However, there are no considerations to handle repair losses differently than source packet losses.

Although RFC 5109 [13] recommended that the source packets and the repair packets may be sent in different sessions (identified by different IP 5-tuples), we are unaware of any commercial implementation of such scheme.

Note that RFC 8085 (UDP usage Guidelines) recommend that an "application SHOULD perform congestion control over all UDP traffic it sends to a destination, independently from how it generates this traffic".

Recommendation: SA4 to study how to handle the packet losses for congestion control when the application traffic is encoded with AL-FEC.