3GPP TSG-SA WG4 Meeting #130						S4-241964

USA, Orlando, 18 – 22 November 2024

Source:	HEAD acoustics GmbH

Title:	Test methods for UE headtracking and binaural rendering

Agenda item:	7.8

Document for:	Discussion

# Introduction

One important aspect in immersive audio applications (including conversational and non-conversational services) is the usage of headtracking whenever possible and supported by the UE. Adapting the binaural rendering to the user’s head orientation improves perceived externalization and immersion. A low latency between head movement and adapted sound is crucial for the quality of experience.

Test methods (for 3GPP TS 26.260 [1]) and performance requirements/objectives (for 3GPP TS 26.261 [2]) for the assessment of binaural rendering in receive direction are in the scope of the new work item ATIAS_Ph2 [3], and explicitly include headtracking functionality and motion-to-sound latency as performance measure.

The present document presents some initial possible approaches and suggestions to address these objectives from the work item description.

# Motion-to-sound latency

## Overview

Headtracking improves the perceived externalization and immersion by adapting binaural rendering to head orientation. As depicted in Figure 1, Motion-to-Sound (M2S) latency tM2S refers to the time difference between the time instant of movement of a listener's head t0 and the time instant t1 when the interaural cues present in the headphone signal are adapted. Perceptual thresholds for M2S latencies found in literature are in the order of about 60…100 ms [4]. Lower values can be considered acceptable for a seamless and immersive listening experience.

Figure 1: Motion-to-sound latency - definition

## Existing test methods

Clause 4.2.3 of 3GPP TS 26.260  [1] ("Motion to Sound Latency in Dynamic Binaural Rendering Systems") already addresses a lab method to determine motion-to-sound latency. However, the method requires developer access to the rendering system (two parallel rendering chains, manipulation of headtracking data) and does not consider acoustic signal capture with a HATS. The method is thus not feasible for the "blackbox testing" approach, which applies for IVAS-based UEs according to clause 5 of TS 26.260 [1].

## Proposed testing approach

For this reason, it is proposed to define new test methods that acoustically capture binaural signals rendered and played back by an RCV-headset UE. The HATS is physically rotated and time instant t0 is measured when the HATS passes 0° orientation. Then, t1 is measured at which the interaural cues present in the binaural signal correspond to 0° orientation.

At an orientation of 0°, the monaural cues are monotonic when the angle changes and a sign change occurs. Evaluating M2S latency at zero degree is hence beneficial and robust to cope with different renderers and HRTF sets. However, with typical rotation speeds of the human head in the range of roughly 100°/s for a medium-fast head rotation, even slight deviations of the interaural cues can lead to significant measurement deviations when determining t1. Therefore, calibration is required to precisely identify any deviation of the interaural cues from zero, caused e.g., by asymmetrical HRTF sets, left/right level imbalances of the UE (D/A, headphone drivers, ...) as well as fitting of earpieces and artificial ears. The general approach should consist of the following stages:

Calibration: measure interaural features for 0° UE orientation (fitting, sensor, rendering, UE hardware)

Measurement: rotate UE from left to right and/or vice versa, record binaural signal, determine time t0 of passing 0°; repeat multiple times

Analysis: compare interaural features of recorded binaural signal to calibration data to determine t1 at which binaural signal corresponds to 0° orientation; latency is determined via tM2S=t1-t0

## Outlook

The source is currently investigating the proposed test procedure on M2S latency in an internal feasibility study. The HATS is rotated from +90° (left) to -90° (left) and vice versa several times with a velocity of 108°/s (corresponds to medium-fast head rotation [5]). The test signal is a sine tone at 2 kHz and the interaural level difference (ILD) is evaluated. Figure 2 shows exemplary measurement results of a device under test:

Figure 2: Exemplary results of an M2S latency measurement

Explanations:

-	The dotted line shows the reference ILD for 0° obtained in the calibration phase.

-	The times at which the current ILD crosses the reference ILD at 0° are marked in red.

-	As discussed before, the ILD shows monotonic behavior for angles close to 0°, which facilitates evaluation.

-	The times at which the HATS physically passed 0° are shown with blue vertical lines.

-	The resulting M2S latency can be determined by measuring the distance between the blue and red vertical lines. One can see that the exemplary device under test far exceeds any perceptual threshold from literature.

The internal study shows very promising results despite the difficulty of the problem. More details will be presented in a next meeting or telco.

## Related measures

### Sensor drift

Besides motion-to-sound-latency, another important feature of head-tracked binaural rendering systems is a low drift of the orientation sensor. Typically, UEs are accelerometer/gyroscope-based and may exhibit a sensor drift that may impact the quality of experience.

In addition to motion-to-sound latency, ATIAS_Ph2 should also address a measurement to assess sensor drift. A specific test method is TBD.

### Nullification

The "trajectory nullification" approach was initially proposed in S4-231709 for the processing of listening test conditions for the IVAS characterization phase to test the (default) binaural IVAS renderer. The test method is particularly suitable for testing whether a head tracker follows a desired trajectory and whether the rendering for this trajectory is implemented correctly. In general, the approach works as follows:

-	A certain sound source is spatialized such that it follows a predefined trajectory.

-	The inverse trajectory is then applied physically to the UE under test (in most cases, RCV-headset UE).

-	If binaural rendering and headtracking work as expected, they cancel out each other and the resulting binaural signal exhibits constant binaural cues as if it was constantly rendered to the 0° direction.

As already mentioned for the M2S latency test, the interaural cues for the 0° direction and deviations from it can be analyzed particularly well, even when varying the HRTF set used. A particular challenge with this test is that the M2S latency must be known or previously measured, as the M2S latency must be compensated by temporally adjusting the trajectory of the test signal.

In contrast to the M2S measurement described above, the trajectory nullification method tests the head tracking and rendering for a continuous angle range. Moreover, angular deviations/non-linear behavior and other effects of the orientation sensor such as overshoot can be tested. Thus, ATIAS_Ph2 may also address such a measurement to assess the overall performance of headtracking.

# Conclusion

The present document provided an overview of relevant test methods regarding headtracking and binaural rendering, which are proposed to be considered in ATIAS_Ph2.

Based on the outcome of the discussion, the source will work further on the discussed approaches and prepare some results of an ongoing (and promising) pilot study for the next meeting or telco.

# References

| [1] | 3GPP TS 26.260, "Objective test methodologies for the evaluation of immersive audio systems," Release-18. |
| --- | --- |
| [2] | 3GPP TS 26.261, "Terminal audio quality performance requirements for immersive audio services," Release-18. |
| [3] | 3GPP SP-241314, "WID on Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2," HEAD acoustics GmbH, Qualcomm Incorporated, Nokia Corporation, Dolby Sweden AB, Orange, Xiaomi, Melbourne, 2024. |
| [4] | D. S. Brungart, B. D. Simpson and A. J. Kordik, "The detectability of headtracker latency in virtual audio displays," in International Conference on Auditory Display (ICAD), Limerick, Ireland, 2005. |
| [5] | C. Rappin, J. Palacino, P. Rueff, L. Feichter and M. Paquier, "Latency detection threshold of head-tracking for different head rotation speeds in binaural rendering," in Forum Acusticum 2023, Torino, Italy, 2023. |