3GPP TSG-SA WG4 Meeting #130	S4-242226

# USA, Orlando, 18 – 22 November 2024

Source:	Nokia, Philips, Interdigital, Deutsche Telekom, Fraunhofer HHI, KDDI, Samsung, Sony, Huawei

Title:	Pseudo-CR on Streaming of professionally produced Volumetric Video with single asset containing people – Scenario sections 5, 6 and 8

Spec:	3GPP TR <26.956 Version 0.1.1>

Agenda item:	9.8

Document for:	Decision

1. Introduction

This pCR adds elements for the scenario “Streaming of professionally produced Volumetric Video with single asset containing people – Scenario sections 5, 6 and 8” to the scenario clause 7.X in the TR.

2. Reason for Change

PD 0.0.4 (S4-241714) includes 5 scenarios including scenario “Streaming of professionally produced Volumetric Video with single asset containing people”, which relies on dense point clouds as representation format. TR 0.1.0 includes already dense point clouds as a representation format and therefore it is proposed to include the corresponding scenario. This document addresses source format properties in section 5, encoding and decoding constraints in section 6 and interoperability considerations for the application in section 8 of the scenario template.

3. Conclusions

Without the scenario description the motivation of the dense point cloud representation format may not be well understood, which makes the FS_Beyond2D study less comprehensive.

4. Proposal

It is proposed to agree the following changes to 3GPP TS <26.956  version 0.1.1>.

* * * First Change * * * *

<Proposed change in revision marks>

## 7.x	Scenario x: Streaming of professionally produced Volumetric Video with single asset containing people

### 7.x.1	Scenario name

Streaming of professionally produced Volumetric Video with single asset containing people.

### 7.x.5	Source format properties

This defines a clear range of the considered and relevant source formats, including the signal properties, but also the characteristics of the content. As an example, the texture and depth format properties of the source may be used which include spatial resolution, chroma format, chroma subsampling, aspect ratios, frame rates, color space formats, transfer characteristics, bit depth, viewpoints, other signal properties.

As source format for the scenario the representation format dense dynamic point clouds is considered. Section 4.3.3 describes the dense dynamic point cloud format and the present section provides more detail in direct relation with the scenario.

The following table includes signal properties that are typically used in the near/mid term to represent people or objects.

| Source format properties | Volumetric Video with single asset |
| --- | --- |
| Number of points /Spatial Resolution | Up to 2 million points per frame |
| Chroma format | RGB |
| Chroma subsampling | Not Applicable |
| Picture aspect ratio | Not Applicable |
| Frame rates | 25, 30 Hz |
| Bit depth | 8 and 10 |
| Colour space formats | RGB 444 nonlinear, BT.709 |
| Transfer characteristics | BT.709 |
| Viewpoints | All assets can be viewed from all directions and different distances |

Table X1  Signal properties for dense point cloud format

For highly dynamic sports sequences higher frame rates such as 50 or 60 fps may be useful and this is left for the future.

In the following the quality of the dense point cloud representation format for representing people is further investigated.

#### 7.x.5.1 Conversion and quantization

If volumetric video source sequences are delivered in production quality, then bandwidth for consumer delivery may be too high and a conversion/quantization is necessary to reduce the number of points and use the fixed-point format. The result of that conversion is that the sequence is in a bounding box of 10, 11 or 12 bit which is named in the following as Vox10, Vox11 and Vox12.

- Vox 12 creates a quality that is close to the quality that comes out of the production system for most provided sequences. Emotional facial expressions are clearly visible and tissue structure of cloths is visible. See figures Y2 and Y5 below.

- Vox 11 creates a quality that allows viewing the sequence from a wider distance and it allows to zoom closer. Emotional facial expressions and patterns on cloths is visible. See figures Y3 and Y6 below.

- Vox 10 creates a quality allowing to view the sequence from a wider distance, but getting too close is less recommended. See figures Y4 and Y7 below.

#### 7.x.5.2 Impact of rendering

It is referred to section 4.3.3.3 that covers rendering and display systems. Rendering is typically not covered by standards and allows manufacturers to differentiate. The referred section discusses hole filling, but also techniques such as local meshing could be applied.

#### 7.x.5.3 Impact of the background

To avoid interference between the background and the test sequence, it is recommended to use a neutral background colour for evaluation. If a VR or AR background is used instead of a neutral background, potential artefacts in the sequence are less visible, as the human eye concentrates on the overall picture and not only on the sequence.

This means also that in a real service where a background is used, potential artefacts in a sequence are less visible.

Figure Y1 Impact of background on perceived quality (content courtesy RenderPeople [R1])

#### 7.x.5.4 Visual quality examples sequences in dense dynamic point cloud format

In the following the impact of number of points in a frame and type of renderer on the visual quality of the dense point cloud representation format is demonstrated. In this section still pictures are used to judge the preserved detail. In a volumetric video sequence some artefacts in the snapshots may not be visible.

It can be observed that sequences represented as dense point clouds of around 1M points/frame allow to watch a sequence with a person in AR from a wider distance (e.g. from 3m*) and point clouds of around 2M points/frame allow to get closer (e.g to around 1.5m distance) at good quality for the target scenario. In the latter emotional facial expressions and tissue structure of cloths is visible. More points per frame improve the details, but this may not be required for the target scenario. But if a scenario would require it, a high-end volumetric video production system is able to capture details from e.g. skin or finer details of tissue and it can be represented with the point cloud representation format.

*A typical demonstration scenario would be to use e.g. a smartphone or tablet running a volumetric video application showing a real person of e.g. 3m distance on the screen captured by the camera and rendering at the same time a second person rendered from a point cloud next to the first person.

In the following the sequence Volucap_T003_ThomasScenic-03 from Volucap [R2] is used.

##### 7.x.5.4.1	Thomas near with representative renderer (splat blend mode) and neutral background

Figure Y2 Vox 12 with 9.5M points per frame (content courtesy by Volucap [R2])

Figure Y3 Vox 11 with 2.3M points per frame (content courtesy by Volucap [R2])

Figure Y4 Vox 10 with 600K points per frame (content courtesy by Volucap [R2])

##### 7.x.5.4.1	Thomas near with representative renderer (cube mode) and neutral background

Figure Y5 Vox 12 with 9.5M points per frame (content courtesy by Volucap [R2])

Figure Y6 Vox 11 with 2.3M points per frame (content courtesy by Volucap [R2])

Figure Y7 Vox 10 with 600K points per frame (content courtesy by Volucap [R2])

### 7.x.6	Encoding and decoding constraints and settings

References to external performance data that can be added, for example other SDOs, public documents and so on.

The following table provides an overview of encoding and decoding constraints for V-PCC with H.265/HEVC as codec for the Volumetric Video with single asset streaming scenario. Contribution aspects are not considered in this table.

| Encoding and Decoding Constraints | V-PCC with H.265/HEVC |
| --- | --- |
| Relevant Codec and Codec Profile/Levels | H.265/HEVC Main 10 Profile Level 4.1, 5.1 Metadata stream parsing |
| Random access frequency | 1 seconds |
| Bit rates and quality configuration | Fixed QP Geometry: [32;28;24;20;16] Fixed QP Texture: [42;37;32;27;22] bitrates [1;50 Mbps] |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Covering a range of relevant bitrates and qualities |
| Latency requirements and specific encoding settings | No specific latency requirement |
| Encoding complexity context | Cloud-based encoding, offline encoding |
| Required decoding capabilities | 3 decoder instantiations of H.265/HEVC Main 10 Profile Level 4.1, 5.1 for (occupancy, geometry and color) One synchronized metadata bitstream (Atlas) |

Table X2  Encoding and decoding constraints

### 7.x.8	Interoperability Considerations for the application

a. Streaming with DASH/HLS/CMAF/QUIC

MPEG-DASH is used with ISO/IEC 23090-10 Carriage of visual volumetric video-based coding data – 1st edition.[R3]

b. RTP based delivery

RTP is not proposed for this scenario.

* * * Next Change * * * *

<Proposed change in revision marks>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[R1] Renderpeople, https://renderpeople.com/

[R2] Volucap, https://volucap.com/

[R3] ISO/IEC 23090-10 Carriage of visual volumetric video-based coding data – 1st edition

* * * Next Change * * * *

<Proposed change in revision marks>

* * * End of Changes * * * *