Source:	Qualcomm Inc.

Title:	IMS_TELEP: Codec Considerations for IMS-Based Telepresence

Document for:	Discussion and Approval

Agenda Item:	14.3

- Background

In the MTSI conference call on August 4, the possibility of reconsidering the legacy codec requirements in MTSI in the context of IMS-based telepresence was suggested. An option is that high quality codec requirements may be mandated on TP UEs and that some of the existing legacy codec requirements on MTSI UEs may therefore not need to be enforced on TP UEs.

The view was also expressed that SA1 TS 22.228 requires backwards compatibility of TP UEs with MTSI UEs and therefore by definition TP UEs would have to inherit or fulfil all of the codec requirements on MTSI UEs, such that TP UEs can interoperate with MTSI UEs when needed.

It was felt that this aspect needed more discussion and there might be alternative approaches for guaranteeing interoperability of TP UEs with MTSI UEs.

In the above context, TS 22.228 specifies in Section 7.10.2.2:

A user participating in a Telepresence session shall be able to communicate with others using different types of UEs in the same Telepresence session (e.g. mobile phone, laptop/pc, conference room with a different number of cameras and displays).

This service requirement from SA1 implies handling a large variety of systems at the same time which is possible through the gateway. In the SA1 examples, mobile phones, globally thinking, may use NB, WB, SWB bandwidths in various codecs, e.g. FR, EFR, AMR, EVRC-NW etc. A laptop may be equipped by several codecs with the possibility of support for Skype codecs, for example. A conference room using existing telepresence system typically supports super-wideband and/or fullband codecs.

- User Experience in Current Telepresence Systems

Currently existing telepresence systems offer a greatly improved user experience over regular video conferencing. Indeed, telepresence is a premium service with the goal of feeling like being in the same room for the session participants.

This is illustrated on Figure 1 which shows a typical telepresence system that is available commercially these days.

Fig. 1: Typical telepresence system (commercially available)

Telepresence clearly goes beyond the grade of standard telephony such as PSTN or MMTel (MM Telephony over IMS). In 3GPP, MMTel is capable of offering regular conferencing already now and the user experience in telepresence will go beyond that.

From this perspective, it is clear that the regular audio quality in telepresence must be better than in standard telephony and hence it is worth considering how to distinguish between the two in terms of codecs. In case of lack of sufficient differentiation, there is a risk that 3GPP telepresence is not successful and other solutions (e.g. standard telephony based conferencing) is used instead.

- Proposal

Our proposal is to clearly distinguish between telepresence and standard videoconferencing in terms of user experience determined by, among others, codec audio bandwidth.

To manifest that in terms of codec audio bandwidth, narrowband is obviously not any more the right common platform that every telepresence system should use as a default. Super-wideband and/or fullband are more suitable choices as even currently existing telepresence products use them.

On this aspect, we propose to require that if a legacy (e.g. narrowband) terminal joins a TP session through a media gateway, the TP session shall maintain the actual high quality (super-wideband / fullband) and it shall not fall back to narrowband just because of one legacy user joining.

3GPP telepresence systems are proposed to support super-wideband and/or fullband codec(s) as a default and offer lower bandwidth codecs on recommended or optional basis.

A related specification text is included below:

TP clients in terminals offering speech communication shall support super-wideband or fullband audio bandwidth through:

-	EVS codec (3GPP TS 26.441 [121], 3GPP TS 26.445 [125], 3GPP TS 26.442 [122] and 3GPP TS 26.443 [123] as described below including functions for backwards compatibility with AMR-WB (3GPP TS 26.446 [126]) and discontinuous transmission (3GPP TS 26.450 [130]).

TP clients in terminals offering speech communication should support narrowband or wideband audio bandwidth through:

-	AMR speech codec (3GPP TS 26.071 [11], 3GPP TS 26.090 [12], 3GPP TS 26.073 [13] and 3GPP TS 26.104 [14]) including all 8 modes and source controlled rate operation ‎3GPP TS 26.093 [15]. The TP client in terminal shall be capable of operating with any subset of these 8 codec modes.

-	AMR-WB codec (3GPP TS 26.171 ‎‎[17], 3GPP TS 26.190 ‎[18], 3GPP TS 26.173 ‎[19] and 3GPP TS 26.204 [20]) including all 9 modes and source controlled rate operation ‎3GPP TS 26.193 [21]. The TP client in terminal shall be capable of operating with any subset of these 9 codec modes. When the EVS codec is supported, the EVS AMR-WB IO mode may serve as an alternative implementation of AMR-WB.