Source:	ORANGE

Title:	On SWB/FB frequency response testing and requirements

## Document for:	Discussion, Approval

## Agenda Item:	8.4

## Introduction

The ART_LTE-SUPER WI in 3GPP Rel-12 updated acoustic test specifications to include super-wideband/fullband (SWB/FB) test cases. Several test cases were left undefined, in particular frequency responses. In TS 26.131 there is provision for defining SWB/FB frequency masks defined in 1/3 octave but values are in brackets, i.e. they do not apply; there is some text requiring to pass not only SWB/FB masks in 1/3 octave resolution, but also to pass existing WB masks defined for wideband (WB) terminals.

Some proposals have been made this year to resume the discussion on SWB/FB frequency masks in April [1] and at this meeting some Tdocs address the same topic [2,3].

## General considerations on 1/3 vs 1/12 octave bands

We recall that technical analysis of differences between sub-octaves was provided for instance in [5,6]. Our view is that the 1/3 octave precision is in general not sufficient to characterize a UE frequency response in modes like handset and headset and other bodies (ITU-T, ETSI) have defined SWB masks in 1/12 octave bands. The compromise to define a SWB mask in 1/3 octave bands and a WB mask in 1/12 octave bands is what could be achieved in 3GPP Rel-12.

While in the WB range from 100 to 6300 Hz going from 1/12 to 1/3 octave bands means going from 76 to 19 values, for the high frequencies above 6.3 kHz we move from 16 values to 4 values in 1/3 octave bands to characterize a (linear) range >8000 Hz.

Table 1: Center frequency for each 1/3 octave and 1/12 octave for super-wideband measurement (here ISO 3 R.10 and R.40 are used [4])

| 100 Hz | 95 Hz | 315 Hz | 300 Hz | 1000 Hz | 950 Hz | 3150 Hz | 3000 Hz | 10000 Hz | 9500 Hz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 Hz |  | 315 Hz |  | 1000 Hz |  | 3150 Hz |  | 10000 Hz |
|  | 106 Hz |  | 335 Hz |  | 1060 Hz |  | 3350 Hz |  | 10600 Hz |
|  | 112 Hz |  | 355 Hz |  | 1120 Hz |  | 3550 Hz |  | 11200 Hz |
| 125 Hz | 118 Hz | 400 Hz | 375 Hz | 1250 Hz | 1180 Hz | 4000 Hz | 3750 Hz | 125000 Hz | 11800 Hz |
|  | 125 Hz |  | 400 Hz |  | 1250 Hz |  | 4000 Hz |  | 12500 Hz |
|  | 132 Hz |  | 425 Hz |  | 1320 Hz |  | 4250 Hz |  | 13200 Hz |
|  | 140 Hz |  | 450 Hz |  | 1400 Hz |  | 4500 Hz |  | 14000 Hz |
| 160 Hz | 150 Hz | 500 Hz | 475 Hz | 1600 Hz | 1500 Hz | 5000 Hz | 4750 Hz | 16000 Hz | 15000 Hz |
|  | 160 Hz |  | 500 Hz |  | 1600 Hz |  | 5000 Hz |  | 16000 Hz |
|  | 170 Hz |  | 530 Hz |  | 1700 Hz |  | 5300 Hz |  | 17000 Hz |
|  | 180 Hz |  | 560 Hz |  | 1800 Hz |  | 5600 Hz |  | 18000 Hz |
| 200 Hz | 190 Hz | 630 Hz | 600 Hz | 2000 Hz | 1900 Hz | 6300 Hz | 6000 Hz |  |  |
|  | 200 Hz |  | 630 Hz |  | 2000 Hz |  | 6300 Hz |  |  |
|  | 212 Hz |  | 670 Hz |  | 2120 Hz |  | 6700 Hz |  |  |
|  | 224 Hz |  | 710 Hz |  | 2240 Hz |  | 7100 Hz |  |  |
| 250 Hz | 236 Hz | 800 Hz | 750 Hz | 2500 Hz | 2360 Hz | 8000 Hz | 7500 Hz |  |  |
|  | 250 Hz |  | 800 Hz |  | 2500 Hz |  | 8000 Hz |  |  |
|  | 265 Hz |  | 850 Hz |  | 2650 Hz |  | 8500 Hz |  |  |
|  | 280 Hz |  | 900 Hz |  | 2800 Hz |  | 9000 Hz |  |  |

In terms of bandwidth, the 1/3 octave band division is obtained as follows (note that ISO 3 R.20 is used here instead of the theoretical division):

{100Hz} = [90Hz, 112Hz]

{125Hz} = [112Hz, 140Hz]

{160Hz} = [140Hz, 180Hz]

{200Hz} = [180Hz, 224Hz]

{250Hz} = [224Hz, 280Hz]

{315Hz} = [280Hz, 355Hz]

{400Hz} = [355Hz, 450Hz]

{500Hz} = [450Hz, 560Hz]

{630Hz} = [560Hz, 710Hz]

{800Hz} = [710Hz, 900Hz]

{1000Hz} = [900Hz, 1120Hz]

{1250Hz} = [1120Hz, 1400Hz]

{1600Hz} = [1400Hz, 1800Hz]

{2000Hz} = [1800Hz, 2240Hz]

{2500Hz} = [2240Hz, 2800Hz]

{3150Hz} = [2800Hz, 3550Hz]

{4000Hz} = [3550Hz, 4500Hz]

{5000Hz} = [4500Hz, 5600Hz]

{6300Hz} = [5600Hz, 7100Hz]

{8000Hz} = [7100Hz, 9000Hz]

{10000Hz} = [9000Hz, 11200Hz]

{12500Hz} = [11200Hz, 14000Hz]

{16000Hz} = [14000Hz, 18000Hz]

More importantly, the move from 1/12 to 1/3 octave bands implies smoothing / compression of the dynamic range. See Annex A of the present contribution for more details. Due to the non-linear smoothing process it's difficult to convert a mask defined in 1/12 octave bands to 1/3 bands, however it is useful to inspect real frequency curves in the two domains [5] to observe that one should not expect the same margin between low and high mask limits in both domains.

## Derivation of SWB mask from subjective tests

Orange and Qualcomm contributed in Rel-12 with P.800 DCR test results to derive masks in SWB/FB [6,7]. Based on subjective test results, there were proposals from Orange for the SWB receive side in 1/12 bands and then also in 1/3 octave bands [6]. We repeat in Table 2 the main proposal for 1/3 octave bands from [6] for the SWB receive mask.

Table 2: SWB receive mask (handset and headset) proposed by Orange in [6].

| Frequency (Hz) | Upper limit 8 ± 2 N | Lower limit 8 ± 2 N |
| --- | --- | --- |
| 100 | 9 |  |
| 120 | 9 | -3 |
| 160 | 9 | 2 |
| 6 000 | 9 | 2 |
| 12 000 | 9 | -1 |
| 14 000 | 9 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

The proposal from Qualcomm can be found in [7] for 13.2 SWB and 24.4 FB modes of EVS.

## Derivation of SWB mask from measured responses

Another approach to derive (validate) a mask is to measure a real device and compare its frequency response with a specific mask. As EVS is not yet available in commercial phones, one approach can be to use a softphone with EVS in OTT mode (Over the Top) .

The source has measured several commercial phones in OTT mode (VoIP with EVS) and also some commercial headsets typically used in VoIP / call centers. We have in general the same observation as in [2] that many devices are not yet capable of handling a frequency range up to 14-16 kHz. Actually many devices seem to limit the actual send / receive masks to WB (sometimes even NB) possibly due to hard-coded limitations in resampling and/or VQE; in some cases the frequency range extends to 10-12 kHz and most of the time equalization seems necessary. However, this observation does not mean that the SWB mask envisioned in 3GPP should be limited to e.g. 10 kHz. There is here a chicken-egg problem: as long as a SWB mask is not defined, it will be hard to expect phone with compliant SWB frequency responses; however, the fact that there is almost no SWB-capable device available yet does not mean 3GPP should not define a SWB extending up to 14 kHz.

At least there are few commercial phones that could pass a mask extending up to 14 kHz. For instance in sending one commercial phone in OTT mode has a sending response that can fit in to Table 3.

Table 3: SWB sending mask (handset and headset).

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 3 |  |
| 200 | 3 | -3 |
| 5000 | 3 | -3 |
| 12500 | 3 | -5 |
| 16000 | 3 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

## Other observations

Several observations can be made when deciding on the required SWB mask:

- As pointed out in [7], there can be interoperability problems if the combined sending and receive masks allow for a too large overall tolerance. This problem already exists in WB and will be even more critical in SWB. For instance if a tilt in sending with device A and the same tilt is present in receiving with device B, the overall (end to end) response can be very titled which can be very detrimental to quality.

- The basic principle for setting frequency response requirements in SWB/FB is to secure that there is differentiation (enhancement) of SWB/FB compared with WB. If values have to be agreed for 1/3 octave bands in SWB/FB, the masks in WB used as a 'safefy net' have to be revised to ensure sufficiently good quality already in the WB region. Currently the safety net only refers to exiting WB mask which is felt insufficient.

## Proposal

Based on the above considerations, we propose the SWB masks in sending and received masks in Tables 3 and 2.

We also propose to revise the text in TS 26.131:

A UE operating in super-wideband mode shall pass the super-wideband requirements (i.e. when measured according to in 1/3rd octaves) as specified here, and also pass the wideband sensitivity/frequency characteristics requirements in the wideband range using the wideband measurement (i.e. measured in 1/12th octaves).

to:

A UE operating in super-wideband mode shall pass the super-wideband requirements (i.e. when measured according to in 1/3rd octaves) as specified here, and also pass the wideband sensitivity/frequency characteristics performance objectives in the wideband range using the wideband measurement (i.e. measured in 1/12th octaves).

with performance objectives ensuring a response differentiated from the required WB characteristics. A WB mask similar to [10] could serve this purpose.

In short, our view is that the SWB frequency mask should guarantee some progress with respect to wideband telephony. We propose to ensure some differentiation compared to HD voice but also to minimize end to end quality issues with combined sending and receiving frequency characteristics in SWB.

## References

[1] Tdoc S4-150393, Proposal for send and receive frequency responses for SWB, Source: Samsung Electronics Co., Ltd

[2] Tdoc S4-150987, Apple SWB mask proposal, Source: Apple Italia S.R.L.

[3] Tdoc S4-151016, Compromise proposal for send and receive frequency responses for SWB, Source: Samsung Electronics Co., Ltd

[4] Tdoc S4-150211, LS reply on the reference of frequency band definition in terminal acoustic testing (To: ITU-T SG12), Source: SA4

[5] Tdoc S4-110484, Comments on sub-octaves bands for terminal frequency response, Source: ORANGE SA

[6] Tdoc S4-141262, On frequency responses for SWB acoustics, Source: ORANGE

[7] Tdoc S4-141209, SWB frequency masks for EVS, Source: Qualcomm, Incorporated, DELTA

[8] Tdoc S4-AHQ092, Proposal for frequency masks in SWB, Source: ORANGE

[9] Tdoc S4-150559, EVS – Objective Codec Evaluation - including results of codec release version 12.1.0, Source: HEAD acoustics GmbH

[10] Tdoc S4-110196, Proposal for Receive handset/headset WB frequency mask (Sec. 6.4.2 of TS 26.131), Source: ORANGE SA

## Annex A: comparison between 1/12 to 1/3 octave resolution

We recall here basic observations.

A dip of -20 dB in 1/12 octave will become -2,21 dB in 1/3 octave.

A peak of +20 dB in 1/12 octave will become about 10 dB in 1/3 octave.

More examples with real frequency responses (in WB) can found in [5].

In general one can expect that a given frequency response should fit in a tighter mask in 1/3 octave bands. Moreover the level of equalization to compensate for dips is highly reduced when going to 1/3 octave bands, the same applies for peaks to a smaller extent.