# Minimum Technical Requirements for use of the HD Voice Logo with Headset Electrical Interface issued by GSMA (Annex G)

# Version 2.0

# 07 November 2016

This is a Non-binding Permanent Reference Document of the GSMA

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is confidential to the Association and is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2016 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Antitrust Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

Annex G	Minimum Requirements of the Headset Socket and Jack Interface	3

Recommended PIN Assignment	3

G.1	Minimum Requirements for Mobile Terminals in Communication mode for the usage of the ‘HD Voice’ logo	4

G.2	Minimum Requirements for Headsets / Headphones in Communication Mode for the usage of ‘HD Voice” logo	6

G.3	Minimum Requirements for Headsets / Headphones in Multimedia Mode for the usage of ‘HD Voice” logo	7

G.4	Minimum Functional Requirements for Headsets / Headphones for the usage of ‘HD Voice” logo	7

Minimum Requirements of the Headset Electrical Interface

This Annex is applicable to Mobile Terminals that do not include a companion headset in the sales item box.

NOTE: The tolerance for the sensitivity levels in G1.3 and G2.3 are not aligned with the 3GPP headset mode tolerances and are under investigation.

The tests in this Annex shall be conducted over a Headset Electrical Interface compliant to Section 6 of ITU-T P.381. For Mobile Terminals that do not include an analog Headset Electrical Interface compliant to Section 6 of ITU-T P.381, an Adapter shall be provided by the Mobile Terminal vendor.The Adapter shall provide a Headset Electrical Interface compliant to Section 6 of ITU-T P.381.

## Recommended PIN Assignment

The Mobile Terminal, or Mobile Terminal in combination with Adapter,  shall support headset functionality compliant to ITU-T P.381 Section 6.2.1 or Section 6.2.2 Pin Assignment.

Minimum Requirements for Mobile Terminals in Communication mode for the usage of the ‘HD Voice’ logo

Test Setup for Terminal Communication Mode measurements

Test Setup for Terminal Communication Mode measurements in ITU-T P.381 per section 7.1.

The Test Cases do NOT assume a Headset is connected to the Mobile Terminal, but the Mobile Terminal is connected to the Test System via the Mobile Terminal Headset Electrical Interface, monitoring the electrical input and output of the Mobile Terminal Headset Electrical Interface. No Acoustical interface such as a Head and Torso Simulator is needed.

Test Signals and Test Signal Levels

Test Signals and Test Signal Levels are per ITU-T P.381 Section 7.1.1.2.

Communication Mode – Delay

For Future Study (FFS)

Communication Mode – Level

Communication Mode – Level in Send

The test method is in accordance to ITU-T P.381 Section 7.1.3.2. The requirement is as follows:

The sending level shall be –16dBm0 ±6 dB when inserting the sending signal at the nominal level as described in ITU-T P.381 Section 7.1.1.2.

Communication Mode – Level in Receive

The test method is in accordance to ITU-T P.381 Section 7.1.4.2. The requirement is as follows:

The receiving level shall be -30dBV ±6 dB at maximum volume setting when inserting the receiving signal at nominal level as described in ITU-T P.381 Section 7.1.1.2.

Communication Mode – Frequency Response

Communication Mode – Frequency Response Sending Side

The test method is in accordance to ITU-T P.381 Section 7.1.7.2. The requirement is as follows:

The measured frequency response shall be within the limits as defined in ITU-T P.381 Section 7.1.7; Table 7-2 for narrowband and Table 7-3 for wideband.

Communication Mode – Frequency Response Receiving Side

The test method is in accordance to ITU-T P.381 Section 7.1.8.2. The requirement is as follows:

The measured frequency response shall be within the limits as defined in ITU-T P.381 Section 7.1.8; Table 7-4 for narrowband and Table 7-5 for wideband.

Communication Mode – Sidetone

Communication Mode – Sidetone Loss

The test method is in accordance to ITU-T P.381 Section 7.1.9.2. The requirement is as follows:

The talker sidetone masking rating (STMR) (electrical sidetone) shall be ≥ 20 dB and should be ≤ 35 dB for the nominal setting of the volume control. For all other positions of the volume control, the STMR shall be ≥ 10 dB.

Communication Mode – Sidetone Delay

The test method is in accordance to ITU-T P.381 Section 7.1.10.2. The requirement is as follows:

The maximum sidetone round-trip delay shall not exceed 5 ms in wideband, and should not exceed 5 ms in narrowband. The measured result is only applicable where the level of the electrical sidetone is sufficiently high to be measured.

Communication Mode – Noise

Communication Mode – Noise in Sending Side

The test method is in accordance to ITU-T P.381 Section 7.1.11.2. The requirement is as follows:

The Signal-to-Noise Ratio (SNR) shall be ≥ 30 dB. Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB.

Communication Mode – Noise in Receiving Side

The test method is in accordance to ITU-T P.381 Section 7.1.12.2. The requirement is as follows:

The Signal-to Noise Ratio (SNR) shall be ≥ 30 dB. Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB.

Communication Mode – Distortion

Communication Mode – Distortion in Sending Side

The test method is in accordance to ITU-T P.381 Section 7.1.13.2. The requirement is as follows:

The ratio of signal to harmonic distortion shall be above the mask defined in ITU-T P.381 Section 7.1.13.1; Table 7-6.

Communication Mode – Distortion in Receiving Side

The test method is in accordance to ITU-T P.381 Section 7.1.14.2. The requirement is as follows:

The ratio of signal to harmonic distortion shall be above the mask defined in ITU-T P.381 Section 7.1.14.1; Table 7-7.

Noise Cancellation Test in Send

FFS

Communication Mode – One-way Speech Quality

Communication Mode – One-way Speech Quality in Sending Side

The test method is in accordance to ITU-T P.381 Section 7.1.16.2. The requirement is as follows:

FFS

Communication Mode – One-way Speech Quality in Receiving Side

The test method is in accordance to ITU-T P.381 Section 7.1.17.2. The requirement is as follows:

FFS

Communication Mode – Terminal coupling loss TCLw

The test method is in accordance to ITU-T P.381 Section 7.1.18.2. The requirement is as follows:

The TCLw provided by the headset signal processing in conjunction with typical echo paths, as described in Figure 7-2 of ITU-T P.381, shall be ≥ 46 dB at the volume control setting according to clause 7.1.1.2 of ITU-T P.381. The TCLw shall be also ≥ 46 dB at maximum setting of volume control.

Minimum Requirements for Headsets / Headphones in Communication Mode for the usage of ‘HD Voice” logo

Test Setup for Headset Communication Mode measurements

Test Setup for Headset Communication Mode measurements in ITU-T P.381 Section 8.1. The input and output characteristics of the test system for connecting the headset are as per ITU-T P.381 Section 8.1.1.1.

Test Signals and Test Signal Levels

Test Signals and Test Signal Levels are as per ITU-T P.381 Section 8.1.1.2.

Positioning of HATS, and fitting the Headsets to HATS

Guidelines laid out in ITU-T P.381 Section 8.1.1.3 and 8.1.1.4 shall be adhered to when setting up the tests.

Headset Communication Mode – Sensitivity

Headset Communication Mode – Sensitivity Sending side

The test method is in accordance to ITU-T P.381 Section 8.1.2.2.The requirement is as follows:

The sending sensitivity shall be –55dBV/Pa ±6 dB when inserting the sending signal at the nominal level as described in ITU-T P.381 Section 8.1.1.2.

Headset Communication Mode – Sensitivity Receiving Side

The requirement and test setup is defined in Section G4.2.

Headset Communication Mode – Frequency Response

Headset Communication Mode – Frequency Response Sending Side

The test method is in accordance to ITU-T P.381 Section 8.1.3.2. The requirement is as follows:

The measured frequency response shall be within the limits defined in ITU-T P.381 Table 18.

Headset Communication Mode – Frequency Response Receiving Side

The test method is in accordance to ITU-T P.381 Section 8.1.4.2. The requirement is as follows:

The measured frequency response shall be within the limits defined in Table 8-1 of ITU-T P.381.

Headset Communication Mode – Noise

The test method is in accordance to ITU-T P.381 Section 8.1.5.2. The requirement is as follows:

The measured frequency response shall be within the limits defined in Table 8-2 of ITU-T P.381.

Headset Communication Mode – Noise in Sending Side

The test method is in accordance to ITU-T P.381 Section 8.1.6.2. The requirement is as follows:

The idle noise in sending direction shall be < −90 dBV(A).

Headset Communication Mode – Distortion

Headset Communication Mode – Distortion in Sending Side

The test method is in accordance to ITU-T P.381 Section 8.1.7.2. The requirement is as follows:

The ratio of signal to harmonic distortion in sending side shall be above the mask shown in Table 8-3 of ITU-T P.381.

Headset Communication Mode – Coupling loss HTCLw

The test method is in accordance to ITU-T P.381 Section 8.1.8.2. The requirement is as follows:

The headset terminal coupling loss weighted (HTCLw) provided by the headset shall be ≥ 40 dB.

Minimum Requirements for Headsets / Headphones in Multimedia Mode for the usage of ‘HD Voice” logo

Test Setup for Headset Multimedia Playback Mode measurements

Test Setup for Headset Multimedia Playback Mode measurements in ITU-T P.381 Section 8.2.

Test Signals and Test Signal Levels

Test Signals and Test Signal Levels are as per ITU-T P.381 Section 8.2.1.2.

Positioning of HATS, and fitting the Headsets to HATS

Guidelines laid out in ITU-T P.381 Section 8.2.1.3 and 8.2.1.4 shall be adhered to when setting up the tests.

Multimedia Playback Mode – Sensitivity

The test method is in accordance to ITU-T P.381 Section 8.2.2.2. The requirement is as follows:

The wideband characteristic voltage shall be greater than or equal to 75 mV and less than or equal to 300 mV when measured according to EN 50332.

Minimum Functional Requirements for Headsets / Headphones for the usage of ‘HD Voice” logo

3.5 mm diameter plug connector with four poles

The plug connector with four poles shall conform to requirements per ITU-T P.381 Appendix I.3

G4.2 3.5 mm diameter socket connector with four poles

The socket connector with four poles shall conform to requirements per ITU-T P.381 Appendix I.4

Template for reporting test results

FFS

# Document Management

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 1.0 | 14 August 2015 | Inclusion of minimum network & terminal requirements for the use of HD Voice Logo Headset Jack and Socket Interface | TSG | Andre Schevciw, (Qualcomm, Incorporated) |
| 2.0 (DRAFT) | 07 November 2016 | Updates to reflect latest developments in ITU-T P.381 | TSG | Andre Schevciw, (Qualcomm, Incorporated) |

# Other Information

| Type | Description |
| --- | --- |
| Document Owner | GSMA Terminal Steering Group; GSMA IREG Group |
| Editor / Company | Andre Schevciw (Qualcomm) |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at membership@gsma.com.

Your comments or suggestions & questions are always welcome.