TSG SA4#96 meeting	Tdoc S4-171147

13-17 November 2017, Albuquerque, New Mexico, USA	Agenda Item: 6.2

3GPP TSG-SA WG6 Meeting #19	S6-171485

Dubrovnik, Croatia, 9th – 13th October 2017

Title:	Reply LS on QCI values for MC Video

Response to:	LS (S2-174856, S6-171142) on QCI values for MC Video from SA2

Release:	Rel-14

Work Item:	TEI14

Source:	3GPP TSG SA WG6

To:	SA WG2

Cc:	SA WG4, CT WG1

Contact Person:

#### Name:	Cuili Ge

Tel. Number:	+86 18710098373

E-mail Address:	gecuili at huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

SA6 wishes to thank SA2 for the LS (S2-174856, S6-171142) on QCI values for MC Video.

SA6 has discussed the QCI value for MC Video service based on the questions from SA2. SA6 provides answers to the questions as below:

Q1: SA WG2 is seeking SA WG6 guidance on whether the current set of standardized QCI assignments, as specified in Table 6.1.7 of TS 23.203, are sufficient for supporting MC Video service.

A1: SA6 believes that a new QCI is needed for MC Video media plane. The control signalling for MC Video can use the existing QCI=69 defined for Mission Critical delay sensitive signalling (e.g., MCPTT signalling).

Q2: If SA WG6 perceives the need for new QCI value(s) for MC Video, SA WG2 requests guidance in how the desired QoS characteristics are different from existing standardized QCI values (e.g., relative to the current QoS characteristics for QCI=2, Conversational Video).

In the context of Question Q2, particular consideration should be given related to any proposed QCI Priority Level assignment(s) for MC Video, relative to the existing QCI Priority Level values assigned to currently-standardized QCIs in TS 23.203.

A2: Regarding the MC Video media plane, it is proposed a new QCI to be defined as shown in the table below. The media plane control signalling can utilize the new QCI to be defined or QCI=69.

Q3: SA WG2 would welcome guidance from SA WG6 and CT WG1 concerning relative QCI Priority Level assignments for MC Video versus other existing assignments.  Of particular interest are potential impacts of assigning QCI Priority Level values for MC Video that would grant higher priority to handling of MC Video packets than for IMS signaling.

A3: Refer to the answer to Q2. No impacts on IMS signalling. The priority level for MC Video media plane is recommended to be lower than IMS signalling.

In summary, SA6 concludes that a new QCI is needed for MC Video media plane, and the following characteristics are suggested for consideration.

| QCI | Resource Type | Priority Level | Packet Delay Budget | Packet Error Loss | Example Services |
| --- | --- | --- | --- | --- | --- |
| X | GBR | Between 1 and 2 | Between 75ms and 150ms | 10-3 | Mission Critical Video user plane |

2. Actions:

To SA WG2 group.

ACTION: 	SA6 asks SA2 to take the above into consideration

3. Date of Next SA6 Meetings:

SA6#20 27th November – 1st December 2017	Reno, US

SA6#21 22nd January – 26th January 2018	Gothenburg, SE