TSG SA4#96 meeting	Tdoc S4-171150

13-17 November 2017, Albuquerque, New Mexico, USA	Agenda Item: 6.2

# 3GPP TSG CT WG 1 Meeting 106	TDoc C1-174038

Kochi (India), 23-27 October 2017

Title:	LS on default values for 5GS QoS averaging windows

Release:	Release 15

Work Item:	CT aspects on 5G System - Phase 1 (5GS_Ph1-CT)

Source:	3GPP TSG CT WG1

To:	3GPP TSG SA WG4

Cc:	3GPP TSG SA WG2

Contact person:	Lena Chaponniere

lguellec@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

At SA2#122bis (August 2017), SA2 added the following requirement in 5GS stage 2 TS 23.501 subclause 5.7.2 regarding measurement of the Session-AMBR:

The Session-AMBR is measured over an averaging window with a standardized value.

SA2 also added the "Averaging window" to the list of the 5G QoS characteristics in TS 23.501 subclause 5.7.3, and specified that:

The Averaging window is defined only for GBR QoS flows. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. (R)AN, UPF, UE). The averaging window is signalled with 5QIs, and if it is not received a standardized default value will apply (for standardized 5QIs the value in the QoS characteristics Table 5.7.4-1 applies).

CT1 noted that SA4 TS 26.114 subclause 7.5.5.1 contains the following requirement regarding the sliding window to be used for bit rate control at the MTSI client:

The method to control the encoding bitrate and/or the transmission of packets is left for the discretion of the implementation. However, the average bandwidth of transmitted packets shall be calculated over a sliding window that is no longer than T seconds. Different media types may use different window lengths. The default window length T is 2 seconds if nothing else is specified below.

Based on the above, CT1 tentatively decided to set both the standardized value of the averaging window for Session-AMBR measurement, and the standardized default value of the averaging window for GFBR and MFBR calculation in case of non-standardized 5QI, to 2 seconds.

CT1 kindly asks SA4 to confirm whether these values are appropriate.

.

# 2	Actions

To SA4

ACTION: 	3GPP TSG CT WG1 kindly asks SA4 to confirm that:

- setting the standardized value of the averaging window for Session-AMBR measurement to 2 seconds is appropriate;

- Setting the standardized default value of the averaging window for GFBR and MFBR calculation for non-standardized 5Qis to 2 seconds is appropriate.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 107	27 November – 1 December 2017	Reno, USA

TSG CT WG1 Meeting 108	22 - 26 January 2018		Gothenburg, Sweden