3GPP TSG-SA4 Meeting #96 	S4-171342

# Albuquerque, NM, USA, 13 – 17 November 2017	revision of S4-171234

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.918 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 26.918 on Encoding First-Order Ambisonics with HE-AAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CODVRA |  |  |  |  |  |  | Date: |  | 2017-11-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of study carried out in the course of the FS_CODVRA feasibility study. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of study description and results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR would not contain the study assessing what quality level can be obtained by encoding FOA content with existing 3GPP eAAC+ audio codec.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

(…)

[64]	Purnhagen H., Hirvonen T., Villemoes L., Samuelsson J., Klejsa J., "Immersive Audio Delivery Using Joint Object Coding" AES 140th Convention 2016 June 4-7, Paris, France.

[65]	3GPP TS 26.401 V14.0.0 (2017-03) – General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description (Release 14)

### 6.1.4	Test of the ISO/IEC 23008-3 MPEG-H 3D Audio scene based coding scheme

The MPEG-H verification test report [36] provides details on four listening tests that were conducted to assess the performance of the Low Complexity (LC) Profile of MPEG-H 3D Audio.

### 6.1.5	Report of one test on encoding First-Order Ambisonics with 3GPP enhanced AAC+ with loudspeakers and with non-individualized HRTF and non-equalized headphones

#### 6.1.5.1	Introduction

This contribution presents an experiment that was conducted to analyze the performance of the Enhanced aacPlus [65] codec used by 3GPP services when encoding First-Order Ambisonics audio representationsconverted from a 7.1.4 channel representation.

Specifically, this experiment uses the ITU-R BS.1534-3 [22] methodology to validate that dual stereo eAAC+ streams at reduced bit-rates is a viable transmission format for 4-channel B-format audio to be rendered over both a specific loudspeaker configuration (7.1.4) and binaural (headphone) endpoints.

The binaural testing methodology introduces distortions and localization errors associated with:

1) 	the limited speaker layout channel count in the renderer,

2) 	the differences between the single HRTF used and the assessor's individual HRTF and,

3) 	distortions in the headphone frequency response. Especially in the practically relevant case when using non-individualized HRTF and non-equalized headphones, spectral colouring distortions and more or less severe localization errors are frequently observed. Note that the testing methodology does not include any measure of absolute spatial accuracy. The test conditions were simply compared to the reference condition and all used the same non-individualised HRTF, with no indication of the intended spatial position.

The testing methodology over loudspeakers does not present the limitations listed in 2) and 3).

#### 6.1.5.2	Test Method

In this experiment, a coding solution was tested at various bit-rates with 10 different test signals. The test signals were sourced from 7.1.4 channel-based content and converted to a First-Order Ambisonics representation, coded and then converted back to 7.1.4, through the process described in Clause 6.1.5.3. Given this process, the tests only assess the degradation caused by the encoding of the Ambisonics representation but not potential degradations caused by the format conversion itself.

The eAAC+ codec was used to encode stereo pairs at different configurations.

The test material used covers a range of audio content used to provide immersive experiences. The items were created from real recordings, as well as synthetic sounds and were authored in a post-production/mixing stage.

The test items were of the following type of content:

- amaze2: ambience, nature sounds

- audiosphere2: music, ambience

- bailando2: music, speech

- entity1: ambience

- leap1,2: ambience, nature sounds, speech

- santeria2: music, ambience, nature sounds

- silent6,7: music, ambience, nature sounds

- unfold1: ambience, synth-glitch hits

Testing was conducted in a critical, noise insulated (NC20) listening room with acoustic wall treatment.

Speaker-based listening and headphone-based listening tests were conducted.

For speaker presentation, all test material was rendered to a 7.1.4 speaker layout.

Loudspeaker delivery used Revel Salon 2 floor and Gem 2 ceiling speakers with Paradigm subwoofers. More specifically 5 inch single-cone full-range drivers were employed in a cabinet that is custom made for the listening room, to be as small as practicable possible. The speakers were placed at a distance of approximately 2.0m from the listener.

For headphone listening the material was subsequently binauralized. Headphones used were Sennheiser HD-600s with a Grace amplifier. No individualized HRTF or headphone equalization were used in this test, which may limit the generality of the results. No head-tracking was used in this test.

The participants were all members of Dolby Laboratories Inc. and experienced in audio quality evaluation. Post-screening of assessors was per ITU-R BS.1534-3 [22] section 4.1.2 and all assessors passed post-screening.

The listening panel contained 7 assessors for the loudspeaker listening test and 8 for headphone listening test. All listeners listened to all test content.

Participants were asked to consider all perceptual differences, including spatial characteristics, between the configurations under test and the reference signal when scoring the basic audio quality. They reported these measures of degradation on a 100 point ‘MUSHRA’ scale in accordance with the ITU-R BS.1534-3 [22] methodology, with standardized verbal anchors (100-80 is ‘excellent’, 60-80 ‘good’, 40-60 ‘fair’, 20-40 ‘poor’ and 0-20 ‘bad’).

#### 6.1.5.3	Processing First-Order Ambisonics for Stereo eAAC+ Encoding

For the experiment, the original 7.1.4 signals were firstly down-mixed to First-Order Ambisonics representation. This was performed using a 4x12 matrix operation, with each speaker feed being panned to B format according to its direction of arrival, with no Near-Field-Compensation. In this operation, the LFE channel was ignored. The First-Order Ambisonics (FOA) signals were defined in terms of the standard Schmidt-Normalized ACN channel format (this is also often referred to as AmbiX format). As found based on theoretical considerations and confirmed by informal experiments with the content used in this test and further content, the four channels of the FOA signals were found less suitable for direct encoding/decoding via two stereo instances of eAAC+ at low bit-rates, due to the occurrence of out-of-phase components between each of the channel pairs.

Accordingly, the FOA signals were prepared for eAAC+ encoding by first remixing them into A-Format – in the form of 4 cardioid virtual-microphone signals. These 4 A-Format signals are defined in Table 6.5 as follows:

Table 6.5: Definition of A-Format signals

| Signal Name | Orientation | Meaning |
| --- | --- | --- |
| FL | Azimuth: +54.7, Elevation 0 | Front left facing cardioid |
| FR | Azimuth: 54.7, Elevation 0 | Front right facing cardioid |
| BU | Azimuth: 180, Elevation +54.7 | Back upward facing cardioid |
| BD | Azimuth: 180, Elevation 54.7 | Back downward facing cardioid |

The four cardioid signals of A-Format are created from the FOA signals via a linear mixing matrix:

M=1216011212-16011212016-112120-16-112

The mixing process is applied as follows:

FLFRBUBD=1216011212-16011212016-112120-16-112×ASN3D,00+ASN3D,11-ASN3D,10+ASN3D,11+

The overall process is shown in the following diagrams. Encoding is performed by mixing the FOA components into A-Format, splitting the A-Format signals into two stereo pairs, and encoding the stereo pairs with an eAAC+ encoder:

Decoding is performed by the reverse process:

wherein the M-1 matrix is defined as:

M-1=1212121232-32000032-323232-32-32

and hence, the FOA signals are recovered by the following matrix operation:

ASN3D,00+ASN3D,11-ASN3D,10+ASN3D,11+=1212121232-32000032-323232-32-32×FLFRBUBD

Finally, the FOA signal went through an up-mix stage back to 7.1.4 representation, using a sub-band approach and direct panning to all of the 7.1.4 speaker channels.

Note: The AACfronteAAC+front signal may also be utilized as a stereo version of the original sound-field, being very similar to a standard “crossed cardioid” stereo recording (typical crossed-cardioid recordings make use of a pair of cardioid microphones at 45, whereas the FL and FR signals represent cardioids at 54.7). This stereo pair is suitable for listening on standard stereo playback devices.

Figure 6.3 visualizes the audio processing stages as described above.

Figure 6.3: Audio processing stages

Table 6.6 summarizes the tested configurations.

Table 6.6: Configurations under test

| Item | Description | Configuration |
| --- | --- | --- |
| 1 | FOA reference | 7.1.4->(downmix)->B-A-BFormat->(upmix)->7.1.4 |
| 2 | 7.0 LP Anchor | 7.1.4->(downmix)->B-A-BFormat->(upmix)->7.1.4->(lowpass7.0k)->7.1.4 |
| 3 | 3.5 LP Anchor | 7.1.4->(downmix)->B-A-BFormat->(upmix)->7.1.4->(lowpass3.5k)->7.1.4 |
| 4 | eAAC+:2x96kbps | 7.1.4->(downmix)->B-AFormat->(eAAC+:2x96kbps)->A-BFormat->(upmix)->7.1.4 |
| 5 | eAAC+:2x64kbps | 7.1.4->(downmix)->B-AFormat->(eAAC+:2x64kbps)->A-BFormat->(upmix)->7.1.4 |
| 6 | eAAC+:2x48kbps | 7.1.4->(downmix)->B-AFormat->(eAAC+:2x48kbps)->A-BFormat->(upmix)->7.1.4 |
| 7 | eAAC+:2x32kbps | 7.1.4->(downmix)->B-AFormat->(eAAC+:2x32kbps)->A-BFormat->(upmix)->7.1.4 |

#### 6.1.5.4	Results

Figure 6.4 visualizes the absolute scores per test item and mean scores for all items. Table 6.7 below further summarizes the results across all test items. Figure 6.5 and Table 6.8 describe results for binaural delivery.

Figure 6.4: B-format extension using eAAC+, 7.1.4 loudspeaker configuration

Table 6.6: Loudspeaker scores

| Condition | Amaze 2 | Audiosphere 2 | Bailando 2 | Entity 1 | leap 1 | leap 2 | santeria 2 | silent 6 | silent 7 | unfold 1 | all items |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOA reference | 100 | 93 | 99 | 98 | 99 | 95 | 100 | 98 | 100 | 99 | 98 |
| 3.5 LP Anchor | 28 | 26 | 24 | 27 | 28 | 27 | 27 | 25 | 26 | 28 | 27 |
| 7.0 LP Anchor | 39 | 45 | 40 | 44 | 43 | 41 | 42 | 46 | 44 | 43 | 43 |
| eAAC+:2x96kbps | 88 | 93 | 93 | 93 | 95 | 93 | 92 | 93 | 86 | 95 | 92 |
| eAAC+:2x64kbps | 81 | 88 | 90 | 89 | 88 | 83 | 83 | 85 | 83 | 85 | 85 |
| eAAC+:2x48kbps | 72 | 80 | 83 | 74 | 82 | 79 | 82 | 81 | 79 | 77 | 79 |
| eAAC+:2x32kbps | 65 | 77 | 74 | 75 | 75 | 80 | 73 | 72 | 72 | 77 | 74 |

Figure 6.5: B-format extension to eAAC+, binaural rendering

Table 6.7: Binaural scores

| Condition | Amaze 2 | Audiosphere 2 | Bailando 2 | Entity 1 | leap 1 | leap 2 | santeria 2 | silent 6 | silent 7 | unfold 1 | all items |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOA reference | 100 | 100 | 99 | 100 | 100 | 100 | 99 | 100 | 100 | 100 | 100 |
| 3.5 LP Anchor | 18 | 21 | 20 | 20 | 23 | 23 | 19 | 22 | 19 | 22 | 21 |
| 7.0 LP Anchor | 33 | 44 | 36 | 38 | 42 | 39 | 35 | 40 | 36 | 39 | 38 |
| eAAC+:2x96kbps | 88 | 94 | 93 | 92 | 88 | 89 | 93 | 91 | 91 | 91 | 91 |
| eAAC+:2x64kbps | 79 | 88 | 85 | 86 | 84 | 84 | 87 | 87 | 84 | 82 | 85 |
| eAAC+:2x48kbps | 69 | 78 | 83 | 69 | 77 | 68 | 75 | 77 | 69 | 74 | 74 |
| eAAC+:2x32kbps | 59 | 72 | 71 | 70 | 73 | 70 | 74 | 70 | 62 | 67 | 69 |

#### 6.1.5.5	Conclusions

In this study, a workflow is tested in which a First-Order Ambisonics content stream is represented through 2 stereo eAAC+ streams. The workflow involves matrixing pre- and post-processing operations, from B to A-format and the reverse. Sending and receiving ends thus need to be aware of the chosen configuration with two eAAC+ instances and the specific conversions between A- and B-Formats. In the experiments, no precautions for potential overload after the format conversions were made.

The observed increase in quality over bitrate for the given test material is roughly linear for both endpoints, giving an impression of expected quality/bitrate trade-offs. Mean scores for the headphone endpoint are overall slightly lower than loudspeaker, but the shift is consistently very small (<5 MUSHRA points).

Together, these data suggest that dual stereo eAAC+ streams can reliably carry 7.1.4 channel audio encoded as B-format audio, after conversion to A-format, with no unexpected encoding artifacts inherent to B-format, which would have affected the MUSHRA rating scores. However, a direct carriage of B-format was not formally evaluated and therefore no conclusions related to A-format vs. B-format can be drawn. As the study is limited to 3GPP eAAC+ no conclusions can be drawn on the suitability or necessary configurations of other 3GPP codecs for the coding of First-Order Ambisonics content.