# 3GPP TSG SA WG5 (Telecom Management) Meeting #100	S5-152151

# 13-17 April 2015 Dubrovnik, Croatia	revision of S5-140abc

# Source:	Alcatel-Lucent

# Title:	pCR 32.422 Changes related to Deactivation of Logged MBSFN MDT

# Document for:	Discussion and approval

Agenda Item:	6.4.4

# 1	Decision/action requested

Discussion and approval

# 2	References

[1] 3GPP TS 36.300 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); overall description; Stage 2

[2] 3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description, Stage 2".

# 3	Rationale

Introduce MBSFN MDT related changes in TS 32.422.

# 4	Detailed proposal

First modification

### 4.1.3	Management deactivation

::::::

#### 4.1.3.8	Deactivation mechanisms at UE for MDT

The UE shall silently discard a logged MDT or Logged MBSFN MDT trace session when logging duration expires and shall indicate the availability of logged measurement results to the network next time it enters connected mode.

Second modification

#### 4.1.4.9	EPC deactivation mechanisms for MDT

When the MME receives a Trace Session Deactivation request for an MDT Trace Session of a UE, it shall act according to the following.

In case of an immediate MDT trace session and the UE being in connected mode, the MME shall send trace session deactivation toward the eNodeB. The eNodeB shall deactivate the corresponding MDT RRC measurements in the UE and shall discard the given trace session context.

In case of an immediate MDT trace session and the UE being in idle mode, the MME shall silently discard the stored trace session context.

Note: 	Signaling based deactivation does not apply for logged MDT or Logged MBSFN MDT trace sessions, the logged MDT and Logged MBSFN MDT trace session terminates when logging duration expires.

#### 4.1.4.10	Deactivation mechanisms at UE for MDT

The UE shall discard a logged MDT or Logged MBSFN MDT trace session when logging duration expires and shall indicate the availability of logged measurement results to the network next time it enters connected mode.