# 3GPP TSG SA WG5 (Telecom Management) Meeting #116	S5-176045

# 27 November - 1 December 2017, Reno,US	revision of S5A-17xabc

# Source:	OMA

# Title:	LS to 3GPP SA5 on improvements on AVP definition for CPM Charging

# Document for:	Information, Discussion

Agenda Item:	7.1

Liaison Statement

| Title: | LS to 3GPP SA5 on improvements on AVP definition for CPM Charging | Public Confidential LS |
| --- | --- | --- |
| Date: | 03 November 2017 |  |
| To: | 3GPP SA5 |  |
| Source: | OMA ARChitecture Working Group |  |
| Send Replies to: | OMA-LIAISON@mail.openmobilealliance.org |  |
| Contact(s): | Gerald Görmer, Nokia, gerald.goermer@nokia-bell-labs.com |  |
| Attachments: | n/a |  |

Overview

OMA CPM group enabled in CPM version 2.2, CPM Charging with an additional parameter which should be used for CPM Charging.

One of the existing parameters for the identification for the CPM Server function is the CPM Server Role which has been extended from being a participating function (0) or controlling function (1) to in addition being an interworking function (2) or interworking selection function (3). Because of the mapping of this set of identifications for the ‘Role of Node’ to the existing PoC-Server-Role AVP,, an enhancement of this 3GPP defined AVP is proposed. Such an enhancement of the exiting original reference will additionally provide the backward compatibility for existing implementations.

Proposal

OMA ARC has reviewed the list of parameters for CPM Charging specification and has identified the need for the proposed  enhancement on the specific AVP for the OMA Data Definition specification [1].

3GPP SA5 is invited to modify the existing AVP in support of the OMA Charging Data element as described above and referenced in 3GPP TS 32.299 [2] for the following section.

### 7.2.145	PoC-Server-Role AVP

The PoC-Server-Role AVP (AVP code 883) is of type Enumerated and specifies the role of the PoC server. The identifier can be one of the following:

0	Participating PoC Server

1	Controlling PoC Server

for which the following change is proposed.

### 7.2.145	PoC-Server-Role AVP

The PoC-Server-Role AVP (AVP code 883) is of type Enumerated and specifies the functional role of the PoC server or other server. The identifier can be one of the following:

0	Participating function

1	Controlling function

2	Interworking function

3	Interworking selection function

Requested Action(s)

3GPP SA5 is kindly requested to update the existing AVP for the OMA Charging Data element as described in section 2 of this liaison statement.

Conclusion

OMA ARC WG would like to thank 3GPP SA5 for the alignment of the corresponding specification.

References

[1]	OMA-DDS-Charging_Data-V1_0_1

[2]	3GPP TS 32.299 Diameter charging application