# 3GPP TSG SA WG5 (Telecom Management) Meeting #117	S5-181150

# 29 January - 2 February 2018, Rome, Italy	revision of S5-18xabc

# Source:	ETRI, KT

# Title:	Proposed Update on Principles of Background and Concepts section

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

The group is asked to discuss and approve the contribution.

2	References

[a]	TS 28.530 v0.3.0: “Management of network slicing in mobile networks; Concepts, use cases and requirements”.

3	Rationale

This pCR proposes to update principles of the 5G network slicing management framework by separating slice lifecycle management functions from slice instance management functions.

4	Detailed proposal

It is proposed to make the following changes to TR 28.530.

1st modified section

4.2	Principles

The 5G network slicing management framework is built upon the following principles:

1.	Standardized interfaces between the newly defined network slice lifecycle management functions and the newly defined network slice instance management functions.

2.	Standardized interfaces between the newly defined network slice instance management functions (within the network slicing management function set), as well as in between the network slicing management function set and a given network function, or its network function related management functions.

3.	Multi-vendor interaction in between the newly defined network slice lifecycle and instance management functions (within the network slicing management function set), as well as in between the network slicing management function set and the network function(s) or its network function related management functions.

4.	A simple network slicing set of management functions to simplify the management of network function(s) from the slicing management point-of-view.

5.	A network slicing management function set capable to support various Network Operator deployment options to support diverse use cases, and a set of generic functions to all kinds of network function and the network function provider.

Figure 4.2.1 illustrates the basic principle of standardized interfaces for network slicing management related capabilities.

NOTE:	Network function may have relevant network function level management functions, illustrated in Figure 4.2.1 by using “management function”.

Network slicing management function(s) set

Network function management function(s)

Standard interface

Figure 4.2.1: Standardized interface(s) between network slice lifecycle & instance management functions set and network function or its network function related management functions.

End of modified sections