# 3GPP TSG SA WG5 (Telecom Management) Meeting #119	S5-183209

# 14-18 May 2018, La Jolla (US)

# Source:	China Mobile

# Title:	pCR 28.552 Add PRB usage distribution measurements

# Document for:	Approval

Agenda Item:	6.4.5

1	Decision/action requested

It is asked to discuss and approve this contribution.

2	References

[1]	3GPP TS 28.552 Management and orchestration of networks and network slicing; NR and NG-RAN performance measurements and assurance v0.2.0

[2]	3GPP TS 32.404 Performance Management (PM); Performance measurements; Definitions and template v14.1.0

[3]	3GPP TS 32.425 Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network v15.0.0

[4]           3GPP TS 36.314 Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 - Measurements v15.0.0

3	Rationale

The proposed radio resource utilization related measurements (Distribution of Total PRB Usage) can enable the operator to monitor the resource utilization status of the radio network.

4	Detailed proposal

Start of Modified Section

5	Performance measurements and assurance data for gNB

5.2	Performance measurements and assurance data for gNB-DU

5.2.x	Radio resource utilization related measurements

5.2.x.y	Distribution of DL Total PRB Usage

a) This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the downlink in different ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the downlink may experience high load in certain short times (e.g. in a second) and recover to normal very quickly.

b) CC

c) Each measurement sample is obtained as: Mn=M1nPn*100, where  Mn  is total PRB usage at sample n for DL, which is a percentage of PRBs used, averaged during time period tn with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for DL traffic transmission shall be included; Pn is the total number of PRBs available for DL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period T.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is provided by the operator.

d) A set of integers. Each representing the (integer) number of samples with a DL total PRB percentage usage in the range represented by that bin.

e) RRU.PrbTotDlDist.BinX, which indicates the distribution of DL PRB Usage for all traffic.

f) GNBDUFunction. NRCellDU (ENGNBDUFunction.NRCellDU for NR option 3)

g) Valid for packet switched traffic

h) NR and NG-RAN

5.2.x.z	Distribution of UL Total PRB Usage

a) This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the uplink in different usage ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the uplink may experience high load in certain short times (e.g. in a second) and recover to normal very quickly.

b) CC

c) Each measurement sample is obtained as: Mn=M1nPn*100, where  Mn  is total PRB usage at sample n for UL, which is a percentage of PRBs used, averaged during time period tn with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for UL traffic transmission shall be included; Pn is the total number of PRBs available for UL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period T.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is provided by the operator.

d) A set of integers, each representing the (integer) number of samples with a UL PRB percentage usage in the range represented by that bin.

e) RRU.PrbTotUlDist.BinX, which indicates the distribution of UL PRB Usage for all traffic.

f) GNBDUFunction. NRCellDU (ENGNBDUFunction.NRCellDU for NR option 3)

g) Valid for packet switched traffic

h) NR and NG-RAN

End of Modified Section