3GPP TSG-SA5 Meeting #122	S5-187080

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0008 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Invocation result at http ok |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2018-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Since there is no ok for the Invocation Result Code this cannot be sent at successful invocation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing that mandatory for Invocation Result in the unsuccessful case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sending an error in case of success can lead to miss interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.290 CR 0021, TS 32.291 CR 0014 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G data connectivity converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Result Code | OC | Described in TS 32.290 [57] |
| Failed parameter | OC | Described in TS 32.290 [57] |
| Failure Handling | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | Described in TS 32.290 [57] |
| Multiple Unit information | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | M | Described in TS 32.290 [57] |
| UPF ID | OC | This field holds the UPF identifier used for quota granted per UPF by CHF |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |
| Final Unit Indication | OC | Described in TS 32.290 [57] |
| Time Quota Threshold | OC | Described in TS 32.290 [57] |
| Volume Quota Threshold | OC | Described in TS 32.290 [57] |
| Unit Quota Threshold | OC | Described in TS 32.290 [57] |
| Quota Holding Time | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Quota Consumption Time | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in clause 6.2. |
| Roaming QBC information | OM | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is not applicable to FBC. |

End of changes