# 3GPP TSG SA WG5 (Telecom Management) Meeting #123	S5-191266

# 21-25 January 2019, Montreal, Canada	revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0055 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PDCP data volume measurements for EE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE_5G |  |  |  |  |  | Date: |  |  | 2019-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 5GS, PDCP data volume related measurements are useful measurements which represent the real data traffic for each interface towards a GNBDuFunction. Monitoring the PDCP data volume for a gNB node, could provide operators with informationon that is helpful for operators to perform energy efficiency evaluation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PDCP data volume related measurements for EE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The volume of PDCP data for EE can’t be monitored by the operators. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.Y (new) 5.1.1.Y.1 (new), 5.1.1.Y.1.1 (new), 5.1.1.Y.1.2 (new), A.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

#### 5.1.1.Y	PDCP data volume measurements

#### 5.1.1.Y.1  	PDCP SDU data volume per interface

#### 5.1.1.Y.1.1 	DL PDCP SDU Data Volume

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered from GNB-CU-UP to GNB-DU (F1-U interface), to External gNB-CU-UP (Xn-U interface) and to External eNB (X2-U interface). The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per Interface (F1-U, Xn-U, X2-U).

b)	CC

c)	This measurement is obtained by counting the number of DL PDCP SDU bits sent to GNB-DU (F1-U interface), sent to External gNB-CU-UP (Xn-U interface) and sent to External eNB (X2-U interface). The measurement is performed in GNB-CU-UP per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per interface (F1-U, Xn-U, X2-U).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels per interface plus the number of S-NSSAIs per interface.

e)	The measurement names have the form:

- DRB.F1uPdcpSduVolumeDl.QoS and DRB.F1uPdcpSduVolumeDl.SNSSAI (F1-U interface measurements)

- DRB.XnuPdcpSduVolumeDl.QoS and DRB.XnuPdcpSduVolumeDl.SNSSAI (Xn-U interface measurements)

- DRB.X2uPdcpSduVolumeDl.QoS and DRB.X2uPdcpSduVolumeDl.SNSSAI (X2-U interface measurements)

where QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	EP_F1U (F1-U interface), EP_XnU (Xn-U interface), EP_X2U (X2-U interface)

g)	Valid for packet switched traffic

h)	5GS

- One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

#### 5.1.1.Y.1.2 	UL PDCP SDU Data Volume

a)  This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered to GNB-CU-UP from GNB-DU (F1-U interface), from External gNB-CU-UP (Xn-U interface) and from External eNB (X2-U interface). The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per Interface (F1-U, Xn-U, X2-U).

b)	CC

c)	This measurement is obtained by counting the number of UL PDCP SDU bits entering the GNB-CU-UP from GNB-DU (F1-U interface), from External gNB-CU-UP (Xn-U interface) and from External eNB (X2-U interface). The measurement is performed in GNB-CU-UP per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per Interface (F1-U, Xn-U, X2-U).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels per interface plus the number of S-NSSAIs per interface.

e)	The measurement names have the form:

- DRB.F1uPdcpSduVolumeUl.QoS and DRB.F1uPdcpSduVolumeUl.SNSSAI (F1-U interface measurements)

- DRB.XnuPdcpSduVolumeUl.QoS and DRB.XnuPdcpSduVolumeUl.SNSSAI (Xn-U interface measurements)

- DRB.X2uPdcpSduVolumeUl.QoS and DRB.X2uPdcpSduVolumeUl.SNSSAI (X2-U interface measurements)

where QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	EP_F1U (F1-U interface), EP_XnU (Xn-U interface), EP_X2U (X2-U interface)

g)	Valid for packet switched traffic

h)	5GS

- One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

Next modified section

# A.y	Monitoring of PDCP data volume per interface

In 5GS, PDCP data volume is a useful measurement which represents the real data traffic towards each GNBDUFunction (F1-U interface), each External gNB-CU-UP (Xn-U interface) and each External eNB (X2-U interface). The monitoring of the PDCP data volume could provide operators with traffic information and is a useful measure in performance assurance within integrity area (user plane connection quality) and in energy efficiency evaluation.

End of modifications