3GPP TS 28.541 V16.2.0 (2019-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Management and orchestration;

5G Network Resource Model (NRM);

Stage 2 and stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Management, resource,model,orchestration,5G

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

Introduction	14

1	Scope	15

2	References	15

3	Definitions and abbreviations	17

3.1	Definitions	17

3.2	Abbreviations	17

4	Information model definitions for NR NRM	18

4.1	Imported and associated information	18

4.1.1	Imported information entities and local labels	18

4.1.2	Associated information entities and local labels	18

4.2	Class diagram	18

4.2.1	Class diagram for gNB and en-gNB	18

4.2.1.1	Relationships	18

4.2.1.2	Inheritance	21

4.3	Class definitions	22

4.3.1	GNBDUFunction	22

4.3.1.1	Definition	22

4.3.1.2	Attributes	23

4.3.1.3	Attribute constraints	23

4.3.1.4	Notifications	23

4.3.2	GNBCUCPFunction	23

4.3.2.1	Definition	23

4.3.2.2	Attributes	23

4.3.2.3	Attribute constraints	24

4.3.2.4	Notifications	24

4.3.3	GNBCUUPFunction	24

4.3.3.1	Definition	24

4.3.3.2	Attributes	24

4.3.3.3	Attribute constraints	24

4.3.3.4	Notifications	25

4.3.4	NRCellCU	25

4.3.4.1	Definition	25

4.3.4.2	Attributes	25

4.3.4.3	Attribute constraints	25

4.3.4.4	Notifications	25

4.3.5	NRCellDU	25

4.3.5.1	Definition	25

4.3.5.2	Attributes	26

4.3.5.3	Attribute constraints	27

4.3.5.4	Notifications	27

4.3.6	NRSectorCarrier	27

4.3.6.1	Definition	27

4.3.6.2	Attributes	27

4.3.6.3	Attribute constraints	28

4.3.6.4	Notifications	28

4.3.7	BWP	28

4.3.7.1	Definition	28

4.3.7.2	Attributes	28

4.3.7.3	Attribute constraints	28

4.3.7.4	Notifications	28

4.3.8	EP_E1	29

4.3.8.1	Definition	29

4.3.8.2	Attributes	29

4.3.8.3	Attribute constraints	29

4.3.8.4	Notifications	29

4.3.9	EP_XnU	29

4.3.9.1	Definition	29

4.3.9.2	Attributes	29

4.3.9.3	Attribute constraints	29

4.3.9.4	Notifications	29

4.3.10	EP_NgC	29

4.3.10.1	Definition	29

4.3.10.2	Attributes	30

4.3.10.3	Attribute constraints	30

4.3.10.4	Notifications	30

4.3.11	EP_NgU	30

4.3.11.1	Definition	30

4.3.11.2	Attributes	30

4.3.11.3	Attribute constraints	30

4.3.11.4	Notifications	30

4.3.12	EP_F1C	30

4.3.12.1	Definition	30

4.3.12.2	Attributes	31

4.3.12.3	Attribute constraints	31

4.3.12.4	Notifications	31

4.3.13	EP_F1U	31

4.3.13.1	Definition	31

4.3.13.2	Attributes	31

4.3.13.3	Attribute constraints	31

4.3.13.4	Notifications	31

4.3.14	EP_S1U	31

4.3.14.1	Definition	31

4.3.14.2	Attributes	32

4.3.14.3	Attribute constraints	32

4.3.14.4	Notifications	32

4.3.15	EP_X2C	32

4.3.15.1	Definition	32

4.3.15.2	Attributes	32

4.3.15.3	Attribute constraints	32

4.3.15.4	Notifications	32

4.3.16	EP_X2U	32

4.3.16.1	Definition	32

4.3.16.2	Attributes	32

4.3.16.3	Attribute constraints	33

4.3.16.4	Notifications	33

4.3.17	EP_XnC	33

4.3.17.1	Definition	33

4.3.17.2	Attributes	33

4.3.17.3	Attribute constraints	33

4.3.17.4	Notifications	33

4.3.18	ExternalGNBCUCPFunction	33

4.3.18.1	Definition	33

4.3.18.2	Attributes	33

4.3.18.3	Attribute constraints	33

4.3.18.4	Notifications	34

4.3.19	ExternalGNBCUUPFunction	34

4.3.19.1	Definition	34

4.3.19.2	Attributes	34

4.3.19.3	Attribute constraints	34

4.3.19.4	Notifications	34

4.3.20	ExternalGNBDUFunction	34

4.3.20.1	Definition	34

4.3.20.2	Attributes	34

4.3.20.3	Attribute constraints	34

4.3.20.4	Notifications	34

4.3.21	ExternalUPFFunction	35

4.3.21.1	Definition	35

4.3.21.2	Attributes	35

4.3.21.3	Attribute constraints	35

4.3.21.4	Notifications	35

4.3.22	ExternalAMFFunction	35

4.3.22.1	Definition	35

4.3.22.2	Attributes	35

4.3.22.3	Attribute constraints	35

4.3.22.4	Notifications	35

4.3.23	ExternalMFs <<ProxyClass>>	35

4.3.23.1	Definition	35

4.3.23.2	Attributes	36

4.3.23.3	Attribute constraints	36

4.3.23.4	Notifications	36

4.3.24	ENBFunction <<ProxyClass>>	36

4.3.24.1	Definition	36

4.3.24.2	Attributes	36

4.3.24.3	Attribute constraints	36

4.3.24.4	Notifications	36

4.3.25	GNBCUCPFunction <<ProxyClass>>	36

4.3.25.1	Definition	36

4.3.25.2	Attributes	36

4.3.25.3	Attribute constraints	36

4.3.25.4	Notifications	36

4.3.26	GNBCUUPFunction <<ProxyClass>>	36

4.3.26.1	Definition	36

4.3.26.2	Attributes	37

4.3.26.3	Attribute constraints	37

4.3.26.4	Notifications	37

4.3.27	GNBDUFunction <<ProxyClass>>	37

4.3.27.1	Definition	37

4.3.27.2	Attributes	37

4.3.27.3	Attribute constraints	37

4.3.27.4	Notifications	37

4.3.28	ServingGWFFunction <<ProxyClass>>	37

4.3.28.1	Definition	37

4.3.28.2	Attributes	37

4.3.28.3	Attribute constraints	37

4.3.28.4	Notifications	37

4.3.29	UPFFunction <<ProxyClass>>	37

4.3.29.1	Definition	37

4.3.29.2	Attributes	38

4.3.29.3	Attribute constraints	38

4.3.29.4	Notifications	38

4.3.30	AMFFunction <<ProxyClass>>	38

4.3.30.1	Definition	38

4.3.30.2	Attributes	38

4.3.30.3	Attribute constraints	38

4.3.30.4	Notifications	38

4.3.31	Void	38

4.3.32	NRCellRelation	38

4.3.32.1	Definition	38

4.3.32.2	Attributes	38

4.3.32.3	Attribute constraints	38

4.3.32.4	Notifications	38

4.3.33	NRFreqRelation	39

4.3.33.1	Definition	39

4.3.33.2	Attributes	39

4.3.33.3	Attribute constraints	39

4.3.33.4	Notifications	39

4.3.34	EutranFreqRelation	39

4.3.34.1	Definition	39

4.3.34.2	Attributes	39

4.3.34.3	Attribute constraints	39

4.3.34.4	Notifications	39

4.3.35	ExternalNRCellCU	39

4.3.35.1	Definition	39

4.3.35.2	Attributes	40

4.3.35.3	Attribute constraints	40

4.3.35.4	Notifications	40

4.3.36	RRMPolicyRatio2 <<dataType>>	40

4.3.36.1	Definition	40

4.3.36.2	Attributes	40

4.3.36.3	Attribute constraints	40

4.3.36.4	Notifications	40

4.3.37	S-NSSAI <<dataType>>	40

4.3.37.1	Definition	40

4.3.37.2	Attributes	41

4.3.37.3	Attribute constraints	41

4.3.37.4	Notifications	41

4.4	Attribute definitions	41

4.4.1	Attribute properties	41

4.5	Common notifications	48

5	Information Model definitions for 5GC NRM	49

5.1 	Imported information entities and local labels	49

5.2 	Class diagram	49

5.2.1	Class diagram of 5GC NFs	49

5.2.1.1	Relationships	49

5.2.1.2	Inheritance	55

5.2.2	Class diagram of AMF Region/AMF Set	56

5.2.2.1	Relationships	56

5.2.2.2	Inheritance	57

5.3	Class definitions	57

5.3.1	AMFFunction	57

5.3.1.1	Definition	57

5.3.1.2	Attributes	57

5.3.1.3	Attribute constraints	58

5.3.1.4	Notifications	58

5.3.2	SMFFunction	58

5.3.2.1	Definition	58

5.3.2.2	Attributes	58

5.3.2.3	Attribute constraints	58

5.3.2.4	Notifications	58

5.3.3	UPFFunction	58

5.3.3.1	Definition	58

5.3.3.2	Attributes	58

5.3.3.3	Attribute constraints	59

5.3.3.4	Notifications	59

5.3.4	N3IWFFunction	59

5.3.4.1	Definition	59

5.3.4.2	Attributes	59

5.3.4.3	Attribute constraints	59

5.3.4.4	Notifications	59

5.3.5	PCFFunction	59

5.3.5.1	Definition	59

5.3.5.2	Attributes	59

5.3.5.3	Attribute constraints	60

5.3.5.4	Notifications	60

5.3.6	AUSFFunction	60

5.3.6.1	Definition	60

5.3.6.2	Attributes	60

5.3.6.3	Attribute constraints	60

5.3.6.4	Notifications	60

5.3.7	UDMFunction	60

5.3.7.1	Definition	60

5.3.7.2	Attributes	60

5.3.5.3	Attribute constraints	61

5.3.5.4	Notifications	61

5.3.8	UDRFunction	61

5.3.8.1	Definition	61

5.3.8.2	Attributes	61

5.3.8.3	Attribute constraints	61

5.3.8.4	Notifications	61

5.3.9	UDSFFunction	61

5.3.9.1	Definition	61

5.3.9.2	Attributes	61

5.3.9.3	Attribute constraints	62

5.3.9.4	Notifications	62

5.3.10	NRFFunction	62

5.3.10.1	Definition	62

5.3.10.2	Attributes	62

5.3.10.3	Attribute constraints	62

5.3.10.4	Notifications	62

5.3.11	NSSFFunction	62

5.3.11.1	Definition	62

5.3.11.2	Attributes	62

5.3.11.3	Attribute constraints	63

5.3.11.4	Notifications	63

5.3.12	AFFunction	63

5.3.12.1	Definition	63

5.3.13	DNFunction	63

5.3.13.1	Definition	63

5.3.14	SMSFFunction	63

5.3.14.1	Definition	63

5.3.14.2	Attributes	63

5.3.14.3	Attribute constraints	63

5.3.14.4	Notifications	63

5.3.15	LMFFunction	64

5.3.15.1	Definition	64

5.3.15.2	Attributes	64

5.3.15.3	Attribute constraints	64

5.3.15.4	Notifications	64

5.3.16	NGEIRFunction	64

5.3.16.1	Definition	64

5.3.16.2	Attributes	64

5.3.16.3	Attribute constraints	64

5.3.16.4	Notifications	64

5.3.17	SEPPFunction	64

5.3.17.1	Definition	64

5.3.17.2	Attributes	65

5.3.17.3	Attribute constraints	65

5.3.17.4	Notifications	65

5.3.18	NWDAFFunction	65

5.3.18.1	Definition	65

5.3.18.2	Attributes	65

5.3.18.3	Attribute constraints	65

5.3.18.4	Notifications	65

5.3.19	EP_N2	65

5.3.19.1	Definition	65

5.3.19.2	Attributes	65

5.3.19.3	Attribute constraints	65

5.3.19.4	Notifications	66

5.3.20	EP_N3	66

5.3.20.1	Definition	66

5.3.20.2	Attributes	66

5.3.20.3	Attribute constraints	66

5.3.20.4	Notifications	66

5.3.21	EP_N4	66

5.3.21.1	Definition	66

5.3.21.2	Attributes	66

5.3.21.3	Attribute constraints	66

5.3.21.4	Notifications	66

5.3.22	EP_N5	66

5.3.22.1	Definition	66

5.3.22.2	Attributes	66

5.3.22.3	Attribute constraints	67

5.3.22.4	Notifications	67

5.3.23	EP_N6	67

5.3.23.1	Definition	67

5.3.23.2	Attributes	67

5.3.23.3	Attribute constraints	67

5.3.23.4	Notifications	67

5.3.24	EP_N7	67

5.3.24.1	Definition	67

5.3.24.2	Attributes	67

5.3.24.3	Attribute constraints	67

5.3.24.4	Notifications	67

5.3.25	EP_N8	68

5.3.25.1	Definition	68

5.3.25.2	Attributes	68

5.3.25.3	Attribute constraints	68

5.3.25.4	Notifications	68

5.3.26	EP_N9	68

5.3.26.1	Definition	68

5.3.26.2	Attributes	68

5.3.26.3	Attribute constraints	68

5.3.26.4	Notifications	68

5.3.27	EP_N10	68

5.3.27.1	Definition	68

5.3.27.2	Attributes	68

5.3.27.3	Attribute constraints	69

5.3.27.4	Notifications	69

5.3.28	EP_N11	69

5.3.28.1	Definition	69

5.3.28.2	Attributes	69

5.3.28.3	Attribute constraints	69

5.3.28.4	Notifications	69

5.3.29	EP_N12	69

5.3.29.1	Definition	69

5.3.29.2	Attributes	69

5.3.29.3	Attribute constraints	69

5.3.29.4	Notifications	69

5.3.30	EP_N13	70

5.3.30.1	Definition	70

5.3.30.2	Attributes	70

5.3.30.3	Attribute constraints	70

5.3.30.4	Notifications	70

5.3.31	EP_N14	70

5.3.31.1	Definition	70

5.3.31.2	Attributes	70

5.3.31.3	Attribute constraints	70

5.3.31.4	Notifications	70

5.3.32	EP_N15	70

5.3.32.1	Definition	70

5.3.32.2	Attributes	70

5.3.32.3	Attribute constraints	71

5.3.32.4	Notifications	71

5.3.33	EP_N16	71

5.3.33.1	Definition	71

5.3.33.2	Attributes	71

5.3.33.3	Attribute constraints	71

5.3.33.4	Notifications	71

5.3.34	EP_N17	71

5.3.34.1	Definition	71

5.3.34.2	Attributes	71

5.3.34.3	Attribute constraints	71

5.3.34.4	Notifications	71

5.3.35	EP_N20	72

5.3.35.1	Definition	72

5.3.35.2	Attributes	72

5.3.35.3	Attribute constraints	72

5.3.35.4	Notifications	72

5.3.36	EP_N21	72

5.3.36.1	Definition	72

5.3.36.2	Attributes	72

5.3.36.3	Attribute constraints	72

5.3.36.4	Notifications	72

5.3.37	EP_N22	72

5.3.37.1	Definition	72

5.3.37.2	Attributes	72

5.3.37.3	Attribute constraints	73

5.3.37.4	Notifications	73

5.3.38	EP_N26	73

5.3.38.1	Definition	73

5.3.38.2	Attributes	73

5.3.38.3	Attribute constraints	73

5.3.38.4	Notifications	73

5.3.39	Void	73

5.3.40	EP_SBI_IPX	73

5.3.40.1	Definition	73

5.3.40.2	Attributes	73

5.3.40.3	Attribute constraints	73

5.3.40.4	Notifications	74

5.3.41	EP_S5C	74

5.3.41.1	Definition	74

5.3.41.2	Attributes	74

5.3.41.3	Attribute constraints	74

5.3.41.4	Notifications	74

5.3.42	EP_S5U	74

5.3.42.1	Definition	74

5.3.42.2	Attributes	74

5.3.42.3	Attribute constraints	74

5.3.42.4	Notifications	74

5.3.43	EP_Rx	74

5.3.43.1	Definition	74

5.3.43.2	Attributes	75

5.3.43.3	Attribute constraints	75

5.3.43.4	Notifications	75

5.3.44	EP_MAP_SMSC	75

5.3.44.1	Definition	75

5.3.44.2	Attributes	75

5.3.44.3	Attribute constraints	75

5.3.44.4	Notifications	75

5.3.45	EP_NLS	75

5.3.45.1	Definition	75

5.3.45.2	Attributes	75

5.3.45.3	Attribute constraints	75

5.3.45.4	Notifications	76

5.3.46	EP_NLG	76

5.3.46.1	Definition	76

5.3.46.2	Attributes	76

5.3.46.3	Attribute constraints	76

5.3.46.4	Notifications	76

5.3.47	EP_N27	76

5.3.47.1	Definition	76

5.3.47.2	Attributes	76

5.3.47.3	Attribute constraints	76

5.3.47.4	Notifications	76

5.3.48	EP_N31	76

5.3.48.1	Definition	76

5.3.48.2	Attributes	77

5.3.48.3	Attribute constraints	77

5.3.48.4	Notifications	77

5.3.49	ExternalNRFFunction	77

5.3.49.1	Definition	77

5.3.49.2	Attributes	77

5.3.49.3	Attribute constraints	77

5.3.49.4	Notifications	77

5.3.50	ExternalNSSFFunction	77

5.3.50.1	Definition	77

5.3.50.2	Attributes	77

5.3.50.3	Attribute constraints	78

5.3.50.4	Notifications	78

5.3.51	AMFSet	78

5.3.51.1	Definition	78

5.3.51.2	Attributes	78

5.3.51.3	Attribute constraints	78

5.3.51.4	Notifications	78

5.3.52	AMFRegion	78

5.3.52.1	Definition	78

5.3.52.2	Attributes	78

5.3.52.3	Attribute constraints	79

5.3.52.4	Notifications	79

5.3.53	ExternalAMFFunction	79

5.3.53.1	Definition	79

5.3.53.2	Attributes	79

5.3.53.3	Attribute constraints	79

5.3.53.4	Notifications	79

5.3.54	ManagedNFProfile <<dataType>>	79

5.3.54.1	Definition	79

5.3.54.2	Attributes	79

5.3.54.3	Attribute constraints	80

5.3.54.4	Notifications	80

5.3.55	HostAddr <<choice>>	80

5.3.55.1	Definition	80

5.3.56	NFInfo <<choice>>	80

5.3.56.1	Definition	80

5.3.57	UdmInfo <<dataType>>	81

5.3.57.1	Definition	81

5.3.57.2	Attributes	81

5.3.57.3	Attribute constraints	81

5.3.57.4	Notifications	81

5.3.58	AusfInfo <<dataType>>	81

5.3.58.1	Definition	81

5.3.58.2	Attributes	81

5.3.58.3	Attribute constraints	81

5.3.58.4	Notifications	81

5.3.59	UpfInfo <<dataType>>	81

5.3.59.1	Definition	81

5.3.59.2	Attributes	81

5.3.59.3	Attribute constraints	81

5.3.59.4	Notifications	82

5.3.60	AmfInfo <<dataType>>	82

5.3.60.1	Definition	82

5.3.60.2	Attributes	82

5.3.60.3	Attribute constraints	82

5.3.60.4	Notifications	82

5.3.61	Udrinfo <<dataType>>	82

5.3.61.1	Definition	82

5.3.61.2	Attributes	82

5.3.61.3	Attribute constraints	82

5.3.61.4	Notifications	82

5.4	Attribute definitions	82

5.4.1	Attribute properties	82

5.5	Common notifications	85

6	Information model definitions for network slice NRM	86

6.1	Imported information entities and local labels	86

6.2	Class diagram	86

6.2.1	Relationships	86

6.2.2	Inheritance	87

6.3	Class definitions	87

6.3.1	NetworkSlice	87

6.3.1.1	Definition	87

6.3.1.2	Attributes	87

6.3.1.3	Attribute constraints	87

6.3.1.4	Notifications	87

6.3.2	NetworkSliceSubnet	87

6.3.2.1	Definition	87

6.3.2.2	Attributes	88

6.3.2.3	Attribute constraints	88

6.3.2.4	Notifications	88

6.3.3	ServiceProfile <<dataType>>	88

6.3.3.1	Definition	88

6.3.3.2	Attributes	88

6.3.3.3	Attribute constraints	88

6.3.3.4	Notifications	88

6.3.4	SliceProfile <<dataType>>	89

6.3.4.1	Definition	89

6.3.4.2	Attributes	89

6.3.4.3	Attribute constraints	89

6.3.4.4	Notifications	89

6.3.5	NsInfo <<dataType>>	89

6.3.5.1	Definition	89

6.3.5.2	Attributes	89

6.3.5.3	Attribute constraints	89

6.3.5.4	Notifications	89

6.4	Attribute definition	90

6.4.1	Attribute properties	90

6.5	Common notifications	93

7	Solution Set (SS)	93

Annex A (normative):	Cell state handling	95

A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"	95

A.2	Combined state diagram for gNB cell	95

Annex B (normative):	 NSI and NSSI state handling	99

B.1 	NSI state handling	99

B.2	State handling of NSSI	100

Annex C (normative):	XML definitions for NR NRM	103

C.1	General	103

C.2	Architectural features	103

C.3	Mapping	103

C.3.1	General mapping	103

C.3.2	Information Object Class (IOC) mapping	103

C.4	Solution Set definitions	103

C.4.1	XML definition structure	103

C.4.2	Graphical representation	103

C.4.3	XML schema "nRNrm.xsd"	104

Annex D (normative):	JSON definitions for NR NRM	119

D.1	General	119

D.2	Architectural features	119

D.3	Mapping	119

D.4	Solution Set (SS) definitions	119

D.4.1	JSON definition structure	119

D.4.2	Graphical representation	119

D.4.3	JSON schema "nrNrm.json"	119

Annex E (normative):	YANG definitions for NR NRM	141

E.1	General	141

E.2	Void	141

E.3	Void	141

E.4	Void	141

E.5	Modules	141

E.6	Graphical representation	176

E.7	Mount information	177

Annex F (normative):	XML definitions for 5GC NRM	178

F.1	General	178

F.2	Architectural features	178

F.3	Mapping	178

F.3.1	General mapping	178

F.3.2	Information Object Class (IOC) mapping	178

F.4	Solution Set definitions	178

F.4.1	XML definition structure	178

F.4.2	Graphical representation	178

F.4.3	XML schema "ngcNrm.xsd"	179

Annex G (normative):	JSON definitions for 5GC NRM	200

G.1	General	200

G.2	Architectural features	200

G.3	Mapping	200

G.4	Solution Set definitions	200

G.4.1	JSON definition structure	200

G.4.2	Graphical representation	200

G.4.3	JSON schema "ngcNrm.json"	200

Annex H (normative):	YANG definitions for 5GC	220

H.1	General	220

H.2	Void	221

H.3	Void	221

H.4	Void	221

H.5	Modules	221

H.6	Graphical representation	267

H.7	Mount information	301

Annex I (normative):	XML definitions for network slice	303

I.1	General	303

I.2	Architectural features	303

I.3	Mapping	303

I.3.1	General mapping	303

I.3.2	Information Object Class (IOC) mapping	303

I.4	Solution Set (SS) definitions	303

I.4.1	XML definition structure	303

I.4.2	Graphical representation	303

I.4.3	XML schema "sliceNrm.xsd"	304

Annex J (normative):	JSON definitions for network slice	307

J.1	General	307

J.2	Architectural features	307

J.3	Mapping	307

J.4	Solution Set (SS) definitions	307

J.4.1	JSON definition structure	307

J.4.2	Graphical representation	307

J.4.3	JSON schema "sliceNrm.json"	307

Annex K :	Void	312

Annex L (informative):	 Change history	313

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.540:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 1.

TS 28.541:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3.

# 1	Scope

The present document specifies the Information Model and Solution Set for the Network Resource Model (NRM) definitions of NR, NG-RAN, 5G Core Network (5GC) and network slice, to fulfil the requirements identified in 3GPP TS 28.540 [10].

The Information Model defines the semantics and behaviour of information object class attributes and relations visible on the management interfaces in a protocol and technology neutral way. And Solution Set defines one or more solution set(s) with specific protocol(s) according to the Information Model definitions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GR NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV) Release 2; Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[44]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[45]	IETF RFC 8528: "YANG Schema Mount".

[46]	IETF RFC 8340: "YANG Tree Diagrams".

[47]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[48]	3GPP TS 38.463: "NG-RAN; E1 application protocol (E1AP)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1], 3GPP TS 28.540 [10] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 28.540 [10].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.501 [2], 3GPP TS 38.401 [4], 3GPP TS 28.540 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] , 3GPP TS 23.501 [2], 3GPP TS 38.401 [4] and 3GPP TS 28.540 [10].

BWP	Bandwidth part

CM	Configuration Management

DN	Distinguished Name

IOC	Information Object Class

JSON	JavaScript Object Notation

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NS	Network Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

NSSI	Network Slice Subnet Instance

PNF	Physical Network Function

SBA	Service Based Architecture

SS	Solution Set

TN	Transport Network

VNF	Virtualised Network Function

# 4	Information model definitions for NR NRM

## 4.1	Imported and associated information

### 4.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.662 [11], IOC, SectorEquipmentFunction | SectorEquipmentFunction |
| TS 28.658 [19], IOC, ExternalENBFunction | ExternalENBFunction |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.658 [19], IOC, EUtranCellFDD | EUTranCellFDD |
| TS 28.658 [19], IOC, EUtranCellTDD | EUTranCellTDD |
| TS 28.658 [19], dataType, PLMNId | PLMNId |

### 4.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |

## 4.2	Class diagram

### 4.2.1	Class diagram for gNB and en-gNB

#### 4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see 3GPP TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below.

-	Non-split NG-RAN deployment senario, represents the gNB defined in TS 38.401[4].

-	2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1.

-	3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier and <<IOC>>BWP for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

Figure 4.2.1.1-5: View on frequencies and external entities

#### 4.2.1.2	Inheritance

Figure 4.2.1.2-1: Inheritance Hierarchy

## 4.3	Class definitions

### 4.3.1	GNBDUFunction

#### 4.3.1.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBCUUPFunction IOC provide the management of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split and 3-split NG-RAN architecture, tThis IOC provides the management representations of the logical function gNB-DU of gNB or en-gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

#### 4.3.1.2	Attributes

The GNBDUFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBDUId | M | T | T | F | T |
| gNBDUName | O | T | T | F | T |
| gNBId | M | T | F | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.1.3	Attribute constraints

None.

#### 4.3.1.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.2	GNBCUCPFunction

#### 4.3.2.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC provide management representation of the gNB-CU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN deployment scenario, tThis IOC provides management representation ofs the logical function gNB-CU-CP of gNB and en-gNB defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C <<IOC>>EP_F1U. | <<IOC>>EP_XnC, <<IOC>>EP_NgC. |
| en-gNB | <<IOC>>EP_X2C, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_X2C, <<IOC>>EP_F1C. | <<IOC>>EP_X2C. |

#### 4.3.2.2	Attributes

The GNBCUCPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| gNBCUName | O | T | T | F | T |
| pLMNId | M | T | T | T | T |

#### 4.3.2.3	Attribute constraints

None.

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.3	GNBCUUPFunction

#### 4.3.3.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC provide management representation of gNB-CU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN deployment scenario, tThis IOC provides management representation of s the logical function gNB-CU-UP of gNB or en-gNB defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U. | <<IOC>>EP_XnU, <<IOC>>EP_NgU. |
| en-gNB | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U. | <<IOC>>EP_X2U, <<IOC>>EP_S1U. |

#### 4.3.3.2	Attributes

The GNBCUUPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBCUUPId | M | T | F | T | T |
| pLMNIdList | M | T | T | F | T |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.3.3	Attribute constraints

None.

#### 4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.4	NRCellCU

#### 4.3.4.1	Definition

This IOC represents the part of NR cell information that is responsible for the management of inter-cell mobility and neighbour relations via ANR.

#### 4.3.4.2	Attributes

The NRCellCU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNIdList | M | T | T (Note) | F | T |
| sNSSAIList | CM | T | T | F | T |
| rRMPolicyType | CM | T | T | F | T |
| rRMPolicyNSSIId | CM | T | T | F | T |
| rRMPolicyRatio | CM | T | T | F | T |
| rRMPolicyRatio2List | CM | T | T | F | T |
| rRMPolicy | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| Note: Whether the attribute "pLMNIdList" can be writable depends on the implementation. |  |  |  |  |  |

Note 1:	 Void.

Note 2: 	Void.

#### 4.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |
| rRMPolicyRatio2List Support Qualifier | Condition: Network slicing feature is supported and rRMPolicyType is 2. |
| rRMPolicyType Support Qualifier | Condition: RRM policy for network slicing feature is supported. |
| rRMPolicyNSSIId Support Qualifier | Condition: RRM policy for network slicing feature is supported and rRMPolicyType is 1. |
| rRMPolicyRatio Support Qualifier | Condition: RRM policy for network slicing feature is supported and rRMPolicyType is 1. |
| rRMPolicy Support Qualifier | Condition: RRM policy for network slicing feature is supported and rRMPolicyType is 0. |

#### 4.3.4.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.5	NRCellDU

#### 4.3.5.1	Definition

This IOC represents the part of NR cell information that describes s the specific resources instances.

An NR cell transmits SS/PBCH block and always requires downlink transmission at a certain carrier frequency with a certain channel bandwidth. Transmission may be performed from multiple sector-carriers using different transmission points, and these may be configured with different carrier frequencies and channel bandwidths, as long as they are aligned to the cell's downlink resource grids as defined in subclause 4.4 in TS 38.211 [32]. The values of arfcnDL and bSChannelBwDL attributes define the resource grids which each sector-carrier needs to be aligned to. See subclauses 5.3 and 5.4.2 of TS 38.104 for definitions of BS channel bandwidth and NR-ARFCN, respectively.

An NR cell requires an uplink in order to provide initial access. In case of TDD, the values of arfcnUL and bSChannelBwUL have to always be set to the same values as for the corresponding DL attributes. For both FDD and TDD, the arfcnUL and bSChannelBwUL define uplink resource grids to which each sector-carrier needs to align to.

An NR cell can in addition be configured with a supplementary uplink, which has its own arfcnSUL and bSChannelBwSUL, which define resource grids for supplementary uplink sector-carriers.

Each of downlink, uplink and supplementary uplink (if configured) need an initial bandwidth part (BWP), which defines resources to be used by UEs during and immediately after initial access. Additional BWPs can be either configured or calculated by gNB internally and be applied to UEs dynamically by gNB based on e.g. UE capability and bandwidth need of each UE.

NOTE: The S-NSSAI(s) in the S-NSSAIList are common to all PLMNs listed in the pLMNIdList attribute for the NRCellDU.

#### 4.3.5.2	Attributes

The NRCellDU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| operationalState | M | T | F | F | T (see Note 2) |
| administrativeState | M | T | T | F | T (see Note 2) |
| cellState | M | T | F | F | T (see Note 2) |
| pLMNIdList | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| nRPCI | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| arfcnDL | M | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| arfcnSUL | CM | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| ssbFrequency | M | T | T | F | T |
| ssbPeriodicity | M | T | T | F | T |
| ssbSubCarrierSpacing | M | T | T | F | T |
| ssbOffset | M | T | T | F | T |
| ssbDuration | M | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| bSChannelBwSUL | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRSectorCarrier | M | T | T | F | T |
| bWP | M | T | T | F | T |
| Note 1: No state propagation is implied. Note 2: The attribute value change is conveyed by the notifyStateChange notification. |  |  |  |  |  |

#### 4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |
| arfcnUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| arfcnSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| bSChannelBwUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| bSChannelBwSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| nrTAC Support Qualifier | Condition: 5G Standalone solution is supported. |

#### 4.3.5.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.6	NRSectorCarrier

#### 4.3.6.1	Definition

This <<IOC>>NRSectorCarrier represents the resources of each transmission point included in the cell. These in general have different physical locations (of the antennae), and possibly different frequencies or bandwidths. The UE is not directly aware of which NRSectorCarrier resources the network uses for its connection.

An NR sector-carrier can have downlink, uplink or both as specified by txDirection. Attributes related to unavailable direction (DL or UL) shall not be set.

Additional NRSectorCarriers not directly associated to one cell only can also be configured.

If a value of arfcnDL, arfcnUL, bSChannelBwDL or bSChannelBwUL can be derived unambiguously from the referring cell, then that attribute needs not be present. That will not be possible if the NRSectorCarrier is used for supplementary uplink, if it is not directly associated to a cell, or if the sector-carrier uses only a part of the cell's channel bandwidth. Thus, at least in those cases the applicable attributes have to be present and their values need to be set.

#### 4.3.6.2	Attributes

The NRSectorCarrier IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| txDirection | M | T | T | F | T |
| configuredMaxTxPower | CM | T | T | F | T |
| arfcnDL | CM | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| bSChannelBwDL | CM | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| sectorEquipmentFunction | M | T | T | F | T |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| configuredMaxTxPower | Condition: The sector-carrier has a downlink. |
| arfcnDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of arfcnDL. |
| arfcnUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of arfcnUL. |
| bSChannelBwDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of bSChannelBwDL. |
| bSChannelBwUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of bSChannelBwUL. |

#### 4.3.6.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.7	BWP

#### 4.3.7.1	Definition

This IOC represents a bandwidth part (BWP) defined in 3GPP TS 38.211 [32], subclause 4.4.5. A bandwidth part is related to downlink, uplink or supplementary uplink resource grids, and is defined by its subcarrier spacing (SCS), cyclic prefix and location and size related to the common resource grid for the applicable SCS.

A BWP can be either an initial BWP used for initial access, or other ("regular") BWP configured for relevant UEs that support the BWP's characteristics.

#### 4.3.7.2	Attributes

The BWP IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| bwpContext | M | T | T | F | T |
| isInitialBwp | M | T | T | F | T |
| subCarrierSpacing | M | T | T | F | T |
| cyclicPrefix | M | T | T | F | T |
| startRB | M | T | T | F | T |
| numberOfRBs | M | T | T | F | T |

#### 4.3.7.3	Attribute constraints

None.

#### 4.3.7.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.8	EP_E1

#### 4.3.8.1	Definition

This IOC represents the local end point of the logical link, supporting E1 interface between gNB-CU-CP and gNB-CU-UP. The E1 interface is defined in 3GPP TS 38.401 [4].

#### 4.3.8.2	Attributes

The EP_E1 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.8.3	Attribute constraints

None.

#### 4.3.8.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.9	EP_XnU

#### 4.3.9.1	Definition

This IOC represents the one end-point of a logical link supporting the Xn user plane (Xn-U) interface. The Xn-U interface provides non-guaranteed delivery of user plane PDUs between two NG-RAN nodes. The user plane PDUs are carried on GTP-U/UDP/IP/Data link layer/Physical layer stack. See subclause 7.2 of 3GPP TS 38.420 [6].

#### 4.3.9.2	Attributes

The EP_XnU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.9.3	Attribute constraints

None.

#### 4.3.9.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.10	EP_NgC

#### 4.3.10.1	Definition

This IOC represents the local end point of the control plane interface (NG-C) between the gNB and NG-Core entity. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol).

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.10.2	Attributes

The EP_NgC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.10.3	Attribute constraints

None.

#### 4.3.10.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.11	EP_NgU

#### 4.3.11.1	Definition

This IOC represents the local end point of the NG user plane (NG-U) interface between the gNB and the UPGW. The interface provides non-guaranteed delivery of user plane PDUs between the gNB and the UPGW. GTP-U is baseline for this interface.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.11.2	Attributes

The EP_NgU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.11.3	Attribute constraints

None.

#### 4.3.11.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.12	EP_F1C

#### 4.3.12.1	Definition

This IOC represents the local end point of the control plane interface (F1-C) between the DU and CU or CU-CP. The transport network layer is based on IP transport with the SCTP on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol). See subclause 7.1 of 3GPP TS 38.470 [7].

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.12.2	Attributes

The EP_F1C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.12.3	Attribute constraints

None.

#### 4.3.12.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.13	EP_F1U

#### 4.3.13.1	Definition

This IOC represents the local end point of the user plane interface (F1-U) between the DU and CU or CU-UP. The transport network layer is based on IP transport, with the UDP and GTP-U on top of IP.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.13.2	Attributes

The EP_F1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.13.3	Attribute constraints

None.

#### 4.3.13.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.14	EP_S1U

#### 4.3.14.1	Definition

This IOC represents the local end point of the logical link, supporting S1-U interface towards a S-GW node. The S1-U interface is defined in 3GPP TS 36.410 [14].

#### 4.3.14.2	Attributes

The EP_S1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.14.3	Attribute constraints

None.

#### 4.3.14.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.15	EP_X2C

#### 4.3.15.1	Definition

This IOC represents the local end point of the logical link, supporting X2-C application protocols used in EN-DC, to a neighbour eNB or en-gNB node, which is defined in 3GPP TS 36.423 [15]. EN-DC is defined in 3GPP TS 37.340 [9].

#### 4.3.15.2	Attributes

The EP_X2C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.15.3	Attribute constraints

None.

#### 4.3.15.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.16	EP_X2U

#### 4.3.16.1	Definition

This IOC represents the local end-point of a logical link supporting the X2 user plane (X2-U) interface used in EN-DC, which is defined in 3GPP TS 36.425 [16].

#### 4.3.16.2	Attributes

The EP_X2U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.16.3	Attribute constraints

None.

#### 4.3.16.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.17	EP_XnC

#### 4.3.17.1	Definition

This IOC represents the local gNB node end point of the logical link, supporting Xn Application protocols, to a neighbour gNB node. The Xn Application PDUs are carried over SCTP/IP/Data link layer/Physical layer stack. See subclause 7 of 3GPP TS 38.420 [6].

#### 4.3.17.2	Attributes

The EP_X2C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.17.3	Attribute constraints

None

#### 4.3.17.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.18	ExternalGNBCUCPFunction

#### 4.3.18.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUCPFunction managed by another management function. For more information about GNBCUCPFunction, see subclause 4.3.2.

#### 4.3.18.2	Attributes

The ExternalGNBCUCPFunction includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.18.3	Attribute constraints

None.

#### 4.3.18.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.19	ExternalGNBCUUPFunction

#### 4.3.19.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUUPFunction managed by another management function. For more information about GNBCUUPFunction, see subclause 4.3.3.

#### 4.3.19.2	Attributes

The ExternalGNBCUUPFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.19.3	Attribute constraints

None.

#### 4.3.19.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.20	ExternalGNBDUFunction

#### 4.3.20.1	Definition

This IOC represents the properties, known by the management function, of a GNBDUFunction managed by another management function. For more information about GNBDUFunction, see subclause 4.3.1.

#### 4.3.20.2	Attributes

The ExternalGNBDUFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | T | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.20.3	Attribute constraints

None.

#### 4.3.20.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.21	ExternalUPFFunction

#### 4.3.21.1	Definition

This IOC represents the properties, known by the management function, of a UPFFunction managed by another management function. For more information about UPFFunction, see subclause 5.3.3.

#### 4.3.21.2	Attributes

The ExternalUPFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.21.3	Attribute constraints

None.

#### 4.3.21.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.22	ExternalAMFFunction

#### 4.3.22.1	Definition

This IOC represents the properties, known by the management function, of an AMFFunction managed by another management function. For more information about AMFFunction, see subclause 5.3.

#### 4.3.22.2	Attributes

The ExternalAMFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.22.3	Attribute constraints

None

#### 4.3.22.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.23	ExternalMFs <<ProxyClass>>

#### 4.3.23.1	Definition

This represents the following IOCs:

<<IOC>>ExternalAMFFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ExternalGNBDUFunction, <<IOC>>ExternalServingGWFunction, <<IOC>>ExternalUPFFunction.

#### 4.3.23.2	Attributes

See respective IOCs.

#### 4.3.23.3	Attribute constraints

See respective IOCs.

#### 4.3.23.4	Notifications

See respective IOCs.

### 4.3.24	ENBFunction <<ProxyClass>>

#### 4.3.24.1	Definition

This IOC represents an <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.2	Attributes

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.3	Attribute constraints

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBPFunction.

#### 4.3.24.4	Notifications

See respective IOCs.

### 4.3.25	GNBCUCPFunction <<ProxyClass>>

#### 4.3.25.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.3	Attribute constraints

See respective IOCs.

#### 4.3.25.4	Notifications

See respective IOCs.

### 4.3.26	GNBCUUPFunction <<ProxyClass>>

#### 4.3.26.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.4	Notifications

See respective IOCs.

### 4.3.27	GNBDUFunction <<ProxyClass>>

#### 4.3.27.1	Definition

This IOC represents an <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.2	Attributes

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.3	Attribute constraints

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.4	Notifications

See respective IOCs.

### 4.3.28	ServingGWFFunction <<ProxyClass>>

#### 4.3.28.1	Definition

This IOC represents an <<IOC>>ServingGWFFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.2	Attributes

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.3	Attribute constraints

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.4	Notifications

See respective IOCs.

### 4.3.29	UPFFunction <<ProxyClass>>

#### 4.3.29.1	Definition

This IOC represents an <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.2	Attributes

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.3	Attribute constraints

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.4	Notifications

See respective IOCs.

### 4.3.30	AMFFunction <<ProxyClass>>

#### 4.3.30.1	Definition

This IOC represents an <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.2	Attributes

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.3	Attribute constraints

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.4	Notifications

See respective IOCs.

### 4.3.31	Void

### 4.3.32	NRCellRelation

#### 4.3.32.1	Definition

This IOC represents a neighbour cell relation from a source cell to a target cell, where the target cell is an NRCellCU or ExternalNRCellCU instance.

Neighbour cell relations are unidirectional.

#### 4.3.32.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Id | M | T | T | T | F |
| nRTCI | O | T | T | F | T |
| adjacentCell | M | T | T | F | T |

#### 4.3.32.3	Attribute constraints

#### 4.3.32.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.33	NRFreqRelation

#### 4.3.33.1	Definition

This IOC represents a neighbour frequency relation for one or more NRCellRelations.

#### 4.3.33.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.33.3	Attribute constraints

#### 4.3.33.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.34	EutranFreqRelation

#### 4.3.34.1	Definition

This IOC represents a neighbour frequency relation for one or more EutranCellRelations.

#### 4.3.34.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.34.3	Attribute constraints

#### 4.3.34.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.35	ExternalNRCellCU

#### 4.3.35.1	Definition

This abstract IOC represents the properties of an NRCellCU controlled by another Management Service Provider. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by Management Service Provider. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

#### 4.3.35.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| nRPCI | M | T | T | F | T |
| arfcnDL | M | T | T | F | T |

#### 4.3.35.3	Attribute constraints

None.

#### 4.3.35.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.36	RRMPolicyRatio2 <<dataType>>

#### 4.3.36.1	Definition

This data type represents the properties of RRMPolicyRatio2. The RRM policy setting the ratios for the split of the Radio resources between the supported S-NSSAILists, in average over time. RRMPolicyRatio2List is a list and each item of the list has seven attributes. The list of the seven attributes is data type RRMPolicyRatio2. Four of them specify the percentage of radio resources to be allocated to the corresponding sNSSAIList. The attribute rRMPolicyMaxRatio defines the maximum resource limitation for the sNSSAIList. The attribute rRMPolicyMarginMaxRatio defines the safety margin that allows for maximum float limit use with other sNSSAIs borrowing free resources while keeping safety margin to ensure that resources are available for quota specific sNSSAIList. The attribute rRMPolicyMinRatio defines the minimum resource limitation for the sNSSAIList. The attribute rRMPolicyMarginMinRatio defines the safety margin that allows for minimum float limit use with other sNSSAIs borrowing free resources while keeping safety margin to ensure that resources are available for quota specific sNSSAIList.

#### 4.3.36.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | M | T | F | T | F |
| sNSSAIList | M | T | T | F | T |
| quotaType | M | T | T | F | T |
| rRMPolicyMaxRatio | M | T | T | F | T |
| rRMPolicyMarginMaxRatio | O | T | T | F | T |
| rRMPolicyMinRatio | M | T | T | F | T |
| rRMPolicyMarginMinRatio | O | T | T | F | T |

#### 4.3.36.3	Attribute constraints

None

#### 4.3.36.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.37	S-NSSAI <<dataType>>

#### 4.3.37.1	Definition

This data type represents an S-NSSAI. An NSSAI is a set of supported S-NSSAI(s), an S-NSSAI is comprised of an SST (Slice/Service type) and an optional SD (Slice Differentiator) field, (See TS 23.003 [13]).

#### 4.3.37.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sST | M | T | T | F | T |
| sD | O | T | T | F | T |

#### 4.3.37.3	Attribute constraints

None

#### 4.3.37.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: "LOCKED", "SHUTTING DOWN", "UNLOCKED". The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Annex A for Relation between the "Pre-operation state of the gNB-DU Cell" and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked isNullable: False |
| operationalState | It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). allowedValues: "ENABLED", "DISABLED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellState | It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active). The Inactive and Active definitions are in accordance with TS 38.401 [4]: "Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs; Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs." "allowedValues: "IDLE", "INACTIVE", "ACTIVE". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for downlink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnSUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for supplementary uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwDL | BS Channel BW in MHz. for downlink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | BS Channel BW in MHz.for uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwSUL | BS Channel BW in MHz.for supplementary uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxPower | This is the maximum possible for all downlink channels, used simultaneously in a cell, added together. allowedValues:TBD | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cyclicPrefix | Cyclic prefix as defined in TS 38.211 [32], subclause 4.2. allowedValues: "NORMAL", "EXTENDED". | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id. (See IEEE 802.1Q [39]), | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBId | It identifies a gNB within a PLMN. The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells. See "gNB Identifier (gNB ID)" of subclause 8.2 of TS 38.300 [3]). See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 0..4294967295 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdLength | This indicates the number of bits for encoding the gNB ID. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 22 .. 32. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUId | It uniquely identifies the DU at least within a gNB-CU. See 'gNB-DU ID' in subclause 9.3.1.9 of 3GPP TS 38.473 [8]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUUPId | It uniquely identifies the gNB-CU-UP at least within a gNB-CU-CP. See 'gNB-CU-UP ID' in subclause 9.3.1.15 of 3GPP TS 38.463 [48]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUName | It identifies the Central Entity of a NR node, see subclause 9.2.1.4 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUName | It identifies the Distributed Entity of a NR node, see subclause 9.2.1.5 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellLocalId | It identifies a NR cell of a gNB. It, together with the gNB Identifier (using gNBId of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction), identifies a NR cell within a PLMN. This is the NR Cell Identity (NCI). See subclause 8.2 of TS 38.300 [3]), The NCI can be constructed by encoding the gNB Identifier using gNBId (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction) and cellLocalId where the gNB Identifier field is of length specified by gNBIdLength (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction). See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. The NR Cell Global identifier (NCGI) is constructed from the PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of the cell. See relation between NCI and NCGI subclause 8.2 of TS 38.300 [3]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nRPCI | This holds the Physical Cell Identity (PCI) of the NR cell. allowedValues: See 3GPP TS 36.211 subclause 6.11 for legal values of pci. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTAC | This holds the identity of the common Tracking Area Code for the PLMNs. allowedValues: a) It is the TAC or Extended-TAC. b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation). c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 [13] and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473 [8]. | type: Bitstring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUCPFunction.pLMNId | It specifies the PLMN identifier to be used as part of the global RAN node identity. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUUPFunction.pLMNIdList | This is a list of PLMN identifiers. It defines from which set of PLMNs an UE must have as its serving PLMN to be allowed to use the GNB-CU-UP. allowedValues: Not applicable. | type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NRCellCU.pLMNIdList | It defines which PLMNs that can be served by the NR cell. allowedValues: Not applicable. | type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NRCellDU.pLMNIdList | It defines which PLMNs that can be served by the NR cell. The first entry of the list is the PLMN used to construct the nCGI for the NR cell. allowedValues: Not applicable. | type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ExternalNRCellCU.pLMNIdList | It defines which PLMNs that are assumed to be served by the NR Cell in another gNB-CU-CP. This list is either updated by the managed element itself (e.g. due to ANR, signalling over Xn etc) or by consumer over the standard interface. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| sNSSAIList | It represents the list of S-NSSAI the managed object is supporting. The S-NSSAI is defined in 3GPP TS 23.003 [13]. allowedValues: See 3GPP TS 23.003 [13] | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This attribute specifies the Slice/Service type (SST) of the network slice. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sD | This attribute specifies the Slice Differentiator (SD), which is optional information that complements the slice/service type(s) to differentiate amongst multiple Network Slices. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyType | Type of the RRM policy. The value 0 denotes use of the rRMPolicy. The value 1 denotes use of the rRMPolicyNSSIId, rRMPolicyRatio The value 2 denotes use of the rRMPolicyRatio2. allowedValues: 0 : 65535. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rRMPolicyNSSIId | The list of S-NSSAIs for which a rRMPolicyRatio value is specified allowedValues: Not applicable. | type: DN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyRatio | The RRM policy setting the ratio for the split of the Radio resources between the supported S-NSSAI lists A S-NSSAI list is defined in rRMPolicyNSSIId. rRMPolicyRatio is the list of target percentage values assigned to the corresponding rRMPolicyNSSIId values. Every value specifies the percentage of PRBs to be allocated to the corresponding S-NSSAIs, in average over time. The sum of the values shall be less or equal 100. allowedValues: 0 : 100 See NOTE 3 and NOTE 4. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyRatio2List | The attribute specifies a list of RRMPolicyRatio2 which is defined as a datatype. The attribute is used to set the ratios for the split of the Radio resources between the sNSSAILists for radio resources (e.g. RRC connected users, PDCP resource, etc.) in average time (see NOTE 2 and NOTE 3). The sum of the values included in the item of rRMPolicyRatio2 shall be less or equal 100 (see NOTE 4). allowedValues: Not applicable. | type: RRMPolicyRatio2 multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| groupId | The attribute identifies one sNSSAIList group inside NRCellCU. The rRMPolicyRatio2 is configured for each group. The value of the groupId is unique inside one NRCellCU instance | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| quotaType | The attribute indicates the type of the quota which allows to allocate resources as strictly usable for defined slice(s) ( “strict quota”) or allows that resources to be used by other slice(s) when defined slice(s) do not need them ( “float quota”). allowedValues: "STRICT", "FLOAT". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rRMPolicyMaxRatio | The RRM policy setting the maximum percentage of radio resources to be allocated to the corresponding S-NSSAIList. This quota can be strict or float quota. Strict quota means resources are not allowed for other sNSSAIs even when they are not used by the defined sNSSAIList. Float quota resources can be used by other sNSSAIs when the defined sNSSAIList do not need them. Value 0 indicates that there is no maximum limit. allowedValues: 0 : 100 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMarginMaxRatio | Maximum quota margin ratio is applicable when maximum quota policy ratio is of type “float quota”. It defines the resource quota within maximum quota to reserve buffers for new resource requirements for the specified S-NSSAIList. With the margin ratio, unused resources of the maximum resource quota can be allocated to other S-NSSAIs when the free resources are more than resource amount indicated by the margin. The margin resource quota can only be used for the specific S-NSSAIList. Value 0 indicates that no margin is used. allowedValues: 0 : 100 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMinRatio | The RRM policy setting the minimum percentage of radio resources to be allocated to the corresponding S-NSSAIList. This quota can be strict or float quota. Strict quota means resources are not allowed for other sNSSAIs even when they are not used by the defined sNSSAIList. Float quota resources can be used by other sNSSAIs when the defined sNSSAIList do not need them. Value 0 indicates that there is no minimum limit. allowedValues: 0 : 100 NOTE: The averaging time interval is implementation dependent. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMarginMinRatio | Minimum quota margin ratio is applicable when minimum quota policy ratio is of type “float quota”. It defines the resource quota within minimum quota to reserve buffers for new resource requirements for the specified S-NSSAIList. With the margin ratio, unused resources of the minimum resource quota can be allocated to other S-NSSAIs when the free resources are more than resource amount indicated by the margin. The margin resource quota can only be used for the specific S-NSSAIList. Value 0 indicates that no margin is used. allowedValues: 0 : 100 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicy | It represents RRM policy which includes guidance for split of radio resources between the S-NSSAIs that the cell supports in case when the rRMPolicyType is absent or equal to 0. The RRM policy is implementation dependent. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| subcarrierSpacing | Subcarrier spacing configuration for a BWP. See subclause 5 in TS 38.104 [12]. AllowedValues: [15, 30, 60, 120] depending on the frequency range FR1 or FR2. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| txDirection | Indicates if the transmission direction is downlink (DL), uplink (UL) or both downlink and uplink (DL and UL). allowedValues: "DL", "UL", "DL and UL" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bwpContext | It identifies whether the object is used for downlink, uplink or supplementary uplink. allowedValues: "DL", "UL", "SUL" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isInitialBwp | It identifies whether the object is used for initial or other BWP. allowedValues: "INITIAL", "OTHER" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startRB | Offset in common resource blocks to common resource block 0 for the applicable subcarrier spacing for a BWP. This corresponds to N_BWP_start, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 0 to N_grid_size – 1, where N_grid_size equals the number of resource blocks for the BS channel bandwidth, given the subcarrier spacing of the BWP. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| numberOfRBs | Number of physical resource blocks for a BWP. This corresponds to N_BWP_size, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 1 to N_grid_size – startRB of the BWP. Se startRB for definition of N_grid_size. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTCI | This is the Target NR Cell Identifier. It consists of NR Cell Identifier (NCI) and Physical Cell Identifier of the target NR cell (nRPCI). The NRRelation.nRTCI identifies the target cell from the perspective of the NRCell, the name-containing instance of the subject NRCellCU instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| adjacentCell | This attribute contains the DN of an AdjacentNRCell (NRCellCU or ExternalNRCellCU) or AdjacentEutranCell (EutranCellFDD or EutranCellTDD or ExternalEutranCellFDD or ExternalEutranCellTDD) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ssbFrequency | Indicates cell defining SSB frequency domain position Frequency of the cell defining SSB transmission. The frequency provided in this attribute identifies the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block. The frequency must be positioned on the NR global frequency raster, as defined in TS 38.101 [42] subclause 5.4.2. and within bSChannelBwDL. allowedValues: 0..3279165 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbPeriodicity | Indicates cell defined SSB periodicity in number of subframes (ms). The SSB periodicity in msec is used for the rate matching purpose. allowedValues: 5, 10, 20, 40, 80, 160. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbSubCarrierSpacing | Subcarrier spacing of SSB. See subclause 7.4.3.1 of TS 38.211 [32]. allowedValues: Only the values 15kHz or 30 kHz (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbOffset | Indicates cell defining SSB time domain position. Defined as the offset of the measurement window, in number of subframes (ms), in which to receive SS/PBCH blocks, where allowed values depend on the ssbPeriodicity. allowedValues: ssbPeriodicity5 ms 0..4, ssbPeriodicity10 ms 0..9, ssbPeriodicity20 ms 0..19, ssbPeriodicity40 ms 0..39, ssbPeriodicity80 ms 0..79, ssbPeriodicity160 ms 0..159. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbDuration | Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (ms) (see 38.213 [41], subclause 4.1. allowedValues: 1, 2, 3, 4, 5. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: Void NOTE 2: The radio resource can be signaling resources (e.g. RRC connected users) or user plane resources (e.g. PDCP). The detail resource and how to map the ratio to exact number of resources is implementation dependant. NOTE 3: The averaging time interval is implementation dependent. NOTE 4: How to calculate the sum of the ratio is implementation dependent. |  |  |

## 4.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

# 5	Information Model definitions for 5GC NRM

## 5.1 	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.702 [20], IOC, SmsIwmscFunction | SmsIwmscFunction |
| TS 28.702 [20], IOC, SmsGmscFunction | SmsGmscFunction |
| TS 28.702 [20], IOC, GmlcFunction | GmlcFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |

## 5.2 	Class diagram

### 5.2.1	Class diagram of 5GC NFs

#### 5.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of 5GC NFs definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.1.1-1 shows the 5GC NF NRM containment/naming relationship.

Figure 5.2.1.1-1: 5GC NRM containment/naming relationship

The Figure 5.2.1.1-2 shows the transport view of AMF NRM.

Figure 5.2.1.1-2: Transport view of AMF NRM

The Figure 5.2.1.1-3 shows the transport view of SMF NRM.

Figure 5.2.1.1-3: Transport view of SMF NRM

The Figure 5.2.1.1-4 shows the transport view of UPF NRM.

Figure 5.2.1.1-4: Transport view of UPF NRM

The Figure 5.2.1.1-5 shows the transport view of N3IWF NRM.

Figure 5.2.1.1-5: Transport view of N3IWF NRM

The Figure 5.2.1.1-6 shows the transport view of PCF NRM.

Figure 5.2.1.1-6: Transport view of PCF NRM

The Figure 5.2.1.1-7 shows the transport view of AUSF NRM.

Figure 5.2.1.1-7: Transport view of AUSF NRM

The Figure 5.2.1.1-8 shows the transport view of UDM NRM.

Figure 5.2.1.1-8: Transport view of UDM NRM

The Figure 5.2.1.1-12 shows the transport view of NRF NRM.

Figure 5.2.1.1-12: Transport view of NRF NRM

The Figure 5.2.1.1-13 shows the transport view of NSSF NRM.

Figure 5.2.1.1-13: Transport view of NSSF NRM

The Figure 5.2.1.1-14 shows the transport view of SMSF NRM.

Figure 5.2.1.1-14: Transport view of SMSF NRM

The Figure 5.2.1.1-15 shows the transport view of 5G location service related NRM.

Figure 5.2.1.1-15: Transport view of LMF NRM

The Figure 5.2.1.1-16 shows the transport view of 5G-EIR NRM.

Figure 5.2.1.1-16: Transport view of 5G-EIR NRM

The Figure 5.2.1.1-17 shows the transport view of SEPP NRM.

Figure 5.2.1.1-17: Transport view of SEPP NRM

#### 5.2.1.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.1.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM.

Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

Figure 5.2.1.2-2 shows the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM.

Figure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

### 5.2.2	Class diagram of AMF Region/AMF Set

#### 5.2.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of AMF Region/AMF Set definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.2.1-1 shows the AMF Region/AMF Set NRM containment/naming relationship.

Figure 5.2.2.1-1: AMF Region/AMF Set NRM

#### 5.2.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.2.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the AMF Region/AMF Set NRM.

Figure 5.2.2.2-1: AMF Region/AMF Set Inheritance

## 5.3	Class definitions

### 5.3.1	AMFFunction

#### 5.3.1.1	Definition

This IOC represents the AMF functionality in 5GC. For more information about the AMF, see 3GPP TS 23.501 [2].

#### 5.3.1.2	Attributes

The AMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | F | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.1.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.2	SMFFunction

#### 5.3.2.1	Definition

This IOC represents the SMF function in 5GC. For more information about the SMF, see 3GPP TS 23.501 [2].

#### 5.3.2.2	Attributes

The SMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.2.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.3	UPFFunction

#### 5.3.3.1	Definition

This IOC represents the UPF function in 5GC. For more information about the UPF, see 3GPP TS 23.501 [2].

#### 5.3.3.2	Attributes

The UPFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList CM Support Qualifier | The condition is "network slicing feature is supported". |

#### 5.3.3.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.4	N3IWFFunction

#### 5.3.4.1	Definition

This IOC represents the N3IWF function which is used to enable non-3GPP access networks connected to the 5GC. For more information about the N3IWF, see 3GPP TS 23.501 [2].

#### 5.3.4.2	Attributes

The N3IWFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |

#### 5.3.4.3	Attribute constraints

None.

#### 5.3.4.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.5	PCFFunction

#### 5.3.5.1	Definition

This IOC represents the PCF function in 5GC. For more information about the PCF, see 3GPP TS 23.501 [2].

#### 5.3.5.2	Attributes

The PCFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.6	AUSFFunction

#### 5.3.6.1	Definition

This IOC represents the AUSF function in 5GC. For more information about the AUSF, see 3GPP TS 23.501 [2].

#### 5.3.6.2	Attributes

The AUSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.6.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.7	UDMFunction

#### 5.3.7.1	Definition

This IOC represents the UDM function in 5GC. For more information about the UDM, see 3GPP TS 23.501 [2].

#### 5.3.7.2	Attributes

The UDMFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.8	UDRFunction

#### 5.3.8.1	Definition

This IOC represents the UDR function in 5GC. For more information about the UDR, see 3GPP TS 23.501 [2].

#### 5.3.8.2	Attributes

The UDRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.8.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.8.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.9	UDSFFunction

#### 5.3.9.1	Definition

This IOC represents the UDSF function which can be interacted with any other 5GC NF defined in 3GPP TS 23.501 [2]. For more information about the UDSF, see 3GPP TS 23.501 [2].

#### 5.3.9.2	Attributes

The UDSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.9.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.10	NRFFunction

#### 5.3.10.1	Definition

This IOC represents the NRF function in 5GC. For more information about the NRF, see 3GPP TS 23.501 [2].

#### 5.3.10.2	Attributes

The NRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| nFProfileList | CM | T | T | F | T |
| nSIIdList | O | T | T | F | T |

#### 5.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |
| nfProfileList Support Qualifier | Condition: NF profile is registered and deregistered by management system. |
| nSIIdList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.10.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.11	NSSFFunction

#### 5.3.11.1	Definition

This IOC represents the NSSF function in 5GC. For more information about the NSSF, see 3GPP TS 23.501 [2].

#### 5.3.11.2	Attributes

The NSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | M | T | T | F | T |
| nSIIdList | O | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.11.3	Attribute constraints

None.

#### 5.3.11.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.12	AFFunction

#### 5.3.12.1	Definition

This IOC is defined for only purpose to describe the IOCs representing its interaction interface with 5GC (i.e. EP_Rx and EP_N5). It has no any attributes defined.

### 5.3.13	DNFunction

#### 5.3.13.1	Definition

This IOC is defined for only purpose to describe the IOCs representing Data Network (DN) interaction interface with 5GC (i.e. EP_N6). It has no any attributes defined.

### 5.3.14	SMSFFunction

#### 5.3.14.1	Definition

This IOC represents the SMSF function defined in 3GPP TS 23.501 [2].

#### 5.3.14.2	Attributes

The SMSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.14.3	Attribute constraints

None.

#### 5.3.14.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.15	LMFFunction

#### 5.3.15.1	Definition

This IOC represents the LMF function defined in 3GPP TS 23.501 [2].

#### 5.3.15.2	Attributes

The LMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.15.3	Attribute constraints

None.

#### 5.3.15.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.16	NGEIRFunction

#### 5.3.16.1	Definition

This IOC represents the 5G-EIR function in 5GC. For more information about the 5G-EIR, see 3GPP TS 23.501 [2].

#### 5.3.16.2	Attributes

The NGEIRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.16.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.16.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.17	SEPPFunction

#### 5.3.17.1	Definition

This IOC represents the SEPP function which support message filtering and policing on inter-PLMN control plane interface. For more information about the SEPP, see 3GPP TS 23.501 [2].

#### 5.3.17.2	Attributes

The SEPPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |

#### 5.3.17.3	Attribute constraints

None.

#### 5.3.17.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.18	NWDAFFunction

#### 5.3.18.1	Definition

This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see 3GPP TS 23.501 [2].

#### 5.3.18.2	Attributes

The NWDAFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.18.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.18.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.19	EP_N2

#### 5.3.19.1	Definition

This IOC represents the N2 interface between (R)AN and AMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.19.2	Attributes

The EP_N2 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.19.3	Attribute constraints

None.

#### 5.3.19.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.20	EP_N3

#### 5.3.20.1	Definition

This IOC represents the N3 interface between (R)AN and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.20.2	Attributes

The EP_N3 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.20.3	Attribute constraints

None.

#### 5.3.20.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.21	EP_N4

#### 5.3.21.1	Definition

This IOC represents the N4 interface between SMF and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.21.2	Attributes

The EP_N4 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.21.3	Attribute constraints

None.

#### 5.3.21.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.22	EP_N5

#### 5.3.22.1	Definition

This IOC represents the N5 interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.22.2	Attributes

The EP_N5 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.22.3	Attribute constraints

None.

#### 5.3.22.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.23	EP_N6

#### 5.3.23.1	Definition

This IOC represents the N6 interface between UPF and DN, which is defined in 3GPP TS 23.501 [2].

#### 5.3.23.2	Attributes

The EP_N6 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.23.3	Attribute constraints

None.

#### 5.3.23.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.24	EP_N7

#### 5.3.24.1	Definition

This IOC represents the N7 interface between SMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.24.2	Attributes

The EP_N7 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.24.3	Attribute constraints

None.

#### 5.3.24.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.25	EP_N8

#### 5.3.25.1	Definition

This IOC represents the N8 interface between AMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.25.2	Attributes

The EP_N8 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.25.3	Attribute constraints

None.

#### 5.3.25.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.26	EP_N9

#### 5.3.26.1	Definition

This IOC represents the N7 interface between two UPFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.26.2	Attributes

The EP_N9 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.26.3	Attribute constraints

None.

#### 5.3.26.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.27	EP_N10

#### 5.3.27.1	Definition

This IOC represents the N10 interface between SMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.27.2	Attributes

The EP_N10 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.27.3	Attribute constraints

None.

#### 5.3.27.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.28	EP_N11

#### 5.3.28.1	Definition

This IOC represents the N11 interface between AMF and SMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.28.2	Attributes

The EP_N11 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.28.3	Attribute constraints

None.

#### 5.3.28.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.29	EP_N12

#### 5.3.29.1	Definition

This IOC represents the N12 interface between AMF and AUSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.29.2	Attributes

The EP_N12 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.29.3	Attribute constraints

None.

#### 5.3.29.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.30	EP_N13

#### 5.3.30.1	Definition

This IOC represents the N13 interface between AUSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.30.2	Attributes

The EP_N13 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.30.3	Attribute constraints

None.

#### 5.3.30.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.31	EP_N14

#### 5.3.31.1	Definition

This IOC represents the N14 interface between two AMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.31.2	Attributes

The EP_N14 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.31.3	Attribute constraints

None.

#### 5.3.31.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.32	EP_N15

#### 5.3.32.1	Definition

This IOC represents the N15 interface between AMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.32.2	Attributes

The EP_N15 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.32.3	Attribute constraints

None.

#### 5.3.32.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.33	EP_N16

#### 5.3.33.1	Definition

This IOC represents the N16 interface between two SMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.33.2	Attributes

The EP_N16 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.33.3	Attribute constraints

None.

#### 5.3.33.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.34	EP_N17

#### 5.3.34.1	Definition

This IOC represents the N17 interface between AMF and 5G-EIR, which is defined in 3GPP TS 23.501 [2].

#### 5.3.34.2	Attributes

The EP_N17 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.34.3	Attribute constraints

None.

#### 5.3.34.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.35	EP_N20

#### 5.3.35.1	Definition

This IOC represents the N20 interface between AMF and SMSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.35.2	Attributes

The EP_N20 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.35.3	Attribute constraints

None.

#### 5.3.35.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.36	EP_N21

#### 5.3.36.1	Definition

This IOC represents the N21 interface between SMSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.36.2	Attributes

The EP_N21 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.36.3	Attribute constraints

None.

#### 5.3.36.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.37	EP_N22

#### 5.3.37.1	Definition

This IOC represents the N22 interface between AMF and NSSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.37.2	Attributes

The EP_N22 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.37.3	Attribute constraints

None.

#### 5.3.37.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.38	EP_N26

#### 5.3.38.1	Definition

This IOC represents the N26 interface between AMF and MME, which is defined in 3GPP TS 23.501 [2].

#### 5.3.38.2	Attributes

The EP_N26 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.38.3	Attribute constraints

None.

#### 5.3.38.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.39	Void

#### .2	Attributes

#### 5.3.40	EP_SBI_IPX

#### 5.3.40.1	Definition

This IOC represents the inter-connection interface from any 5GC control plane NF to SEPP, see detail in 3GPP TS 23.501 [2].

#### 5.3.40.2	Attributes

The EP_SBI_IPX IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| sBIserviceList | O | T | T | F | T |

#### 5.3.40.3	Attribute constraints

None.

#### 5.3.40.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.41	EP_S5C

#### 5.3.41.1	Definition

This IOC represents the S5-C interface between SGW and SMF/PGW-C, which is defined in 3GPP TS 23.501 [2].

#### 5.3.41.2	Attributes

The EP_S5C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.41.3	Attribute constraints

None.

#### 5.3.41.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.42	EP_S5U

#### 5.3.42.1	Definition

This IOC represents the S5-U interface between SGW and UPF/PGW-U, which is defined in 3GPP TS 23.501 [2].

#### 5.3.42.2	Attributes

The EP_S5U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.42.3	Attribute constraints

None.

#### 5.3.42.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.43	EP_Rx

#### 5.3.43.1	Definition

This IOC represents the Rx interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.43.2	Attributes

The EP_Rx IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.43.3	Attribute constraints

None.

#### 5.3.43.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.44	EP_MAP_SMSC

#### 5.3.44.1	Definition

This IOC represents the MAP interface between SMSF and MSC-IWMSC/GMSC, which is defined in 3GPP TS 23.040 [22].

#### 5.3.44.2	Attributes

The EP_MAP_SMSC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.44.3	Attribute constraints

None.

#### 5.3.44.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.45	EP_NLS

#### 5.3.45.1	Definition

This IOC represents the NLs interface between AMF and LMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.45.2	Attributes

The EP_NLS IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.45.3	Attribute constraints

None.

#### 5.3.45.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.46	EP_NLG

#### 5.3.46.1	Definition

This IOC represents the NLg interface between AMF and GMLC, which is defined in 3GPP TS 23.501 [2].

#### 5.3.46.2	Attributes

The EP_NLG IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.46.3	Attribute constraints

None.

#### 5.3.46.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.47	EP_N27

#### 5.3.47.1	Definition

This IOC represents an end point of N27 interface between vNRF and hNRF, which is defined in 3GPP TS 29.510 [10].

#### 5.3.47.2	Attributes

The EP_N27 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.47.3	Attribute constraints

None.

#### 5.3.47.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.48	EP_N31

#### 5.3.48.1	Definition

This IOC represents an end point of N31 interface between vNSSF and hNSSF, which is defined in 3GPP TS 29.531 [11].

#### 5.3.48.2	Attributes

The EP_N31 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.48.3	Attribute constraints

None.

#### 5.3.48.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.49	ExternalNRFFunction

#### 5.3.49.1	Definition

This IOC represents external NRF function controlled by another management domain. For more information about the NRF, see 3GPP TS 23.501 [2].

#### 5.3.49.2	Attributes

The ExternalNRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.49.3	Attribute constraints

None.

#### 5.3.49.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.50	ExternalNSSFFunction

#### 5.3.50.1	Definition

This IOC represents external NSSF function controlled by another management domain. For more information about the NSSF, see 3GPP TS 23.501 [2].

#### 5.3.50.2	Attributes

The ExternalNSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.50.3	Attribute constraints

None.

#### 5.3.50.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.51	AMFSet

#### 5.3.51.1	Definition

This IOC represents the AMF Set which consists of some AMFs that serve a given area and Network Slice. For more information about the AMF Set, see 3GPP TS 23.501 [2].

#### 5.3.51.2	Attributes

The AMFSet IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFSetId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFRegion | M | T | T | F | T |
| aMFSetMemberList | M | T | T | F | T |

#### 5.3.51.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.51.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.52	AMFRegion

#### 5.3.52.1	Definition

This IOC represents the AMF Region which consists one or multiple AMF Sets. For more information about the AMF Region, see 3GPP TS 23.501 [2].

#### 5.3.52.2	Attributes

The AMFRegion IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFRegionId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFSet | M | T | T | F | T |

#### 5.3.52.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.52.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.53	ExternalAMFFunction

#### 5.3.53.1	Definition

This IOC represents an external AMF functionality used in EN-DC. For more information about the AMF, see 3GPP TS 23.501 [2].

#### 5.3.53.2	Attributes

The ExternalAMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |

#### 5.3.53.3	Attribute constraints

None.

#### 5.3.53.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.54	ManagedNFProfile <<dataType>>

#### 5.3.54.1	Definition

This data type represents a Profile definition of a Managed NF (See TS 23.501 [22]).

#### 5.3.54.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfInstanceID | M | T | F | T | F |
| nfType | M | T | F | F | F |
| hostAddr | M | T | T | F | T |
| authzInfo | O | T | T | F | T |
| location | O | T | T | F | T |
| capacity | O | T | T | F | T |
| nFInfo | M | T | T | F | T |

#### 5.3.54.3	Attribute constraints

None.

#### 5.3.54.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.55	HostAddr <<choice>>

#### 5.3.55.1	Definition

This <<choice>> stereotype represents one of a set of data types as shown in Figure 5.3.55.1-1: HostAddr <<choice>> for data types.

Figure 5.3.55.1-1: HostAddr <<choice>> for data types

NOTE: The IpAddress can be IPv4 address (See RFC 791 [24]) or IPv6 address (See RFC 2373 [25]). Refer TS 23.003 [5] for Fqdn.

### 5.3.56	NFInfo <<choice>>

#### 5.3.56.1	Definition

This <<choice>> stereotype represents one of a set of data types as shown in Figure 5.3.56.1-1: NFInfo <<choice>> for data types.

Figure 5.3.56.1-1: NFInfo choice for data types

NOTE: The AmfInfo <<dataType>> is chosed for AFMFunction, the UdrInfo <<dataType>> is chosed for UDRFunction, the UdmInfo <<dataType>> is chosed for UDMFunction, the AusfInfo<<dataType>> is chosed for AUSFFunction, the UpfInfo <<dataType>> is chosed for UPFFunction

### 5.3.57	UdmInfo <<dataType>>

#### 5.3.57.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.57.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFSrvGroupId | M | T | F | F | F |

#### 5.3.57.3	Attribute constraints

None

#### 5.3.57.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.58	AusfInfo <<dataType>>

#### 5.3.58.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.58.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFSrvGroupId | M | T | F | T | F |

#### 5.3.58.3	Attribute constraints

None.

#### 5.3.58.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.59	UpfInfo <<dataType>>

#### 5.3.59.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.59.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| smfServingAreas | O | T | T | F | T |

#### 5.3.59.3	Attribute constraints

None.

#### 5.3.59.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.60	AmfInfo <<dataType>>

#### 5.3.60.1	Definition

This data type represents AMF specific data in NFProfile definition (See TS 23.501 [22]).

#### 5.3.60.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priority | O | T | T | F | T |

#### 5.3.60.3	Attribute constraints

None.

#### 5.3.60.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.61	Udrinfo <<dataType>>

#### 5.3.61.1	Definition

This data type represents UDR specific data in NFProfile definition (See TS 23.501 [22]).

#### 5.3.61.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedDataSetIds | O | T | T | F | T |
| nFSrvGroupId | O | T | T | F | T |

#### 5.3.61.3	Attribute constraints

None.

#### 5.3.61.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name |  | Documentation and Allowed Values |  | Properties |
| --- | --- | --- | --- | --- |
| aMFIdentifier |  | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetId |  | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetMemberList |  | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A |  | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| aMFRegionId |  | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). |  | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |  |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| nfProfileList |  | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. |  | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nSIIdList |  | It is a set of NSI Id. The NSI ID is defined in subclause 6.1.6.2.8 of 3GPP TS 29.531 [24]. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList |  | See subclause 4.4.1. |  |  |
| sBIFQDN |  | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sBIServiceList |  | It is used to indicate the all supported NF services registered on service-based interface. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nRTACList |  | It is the list of Tracking Area Codes (either legacy TAC or extended TAC). allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| managedNFProfile |  | This parameter defines profile for managed NF (See TS 23.501 [22]). allowedValues: N/A |  | type: ManagedNFProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nfInstanceID |  | This parameter defines unique identity of the NF Instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [44] allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| nfType |  | This parameter defines type of Network Function allowedValues: See TS 23.501[22] for NF types |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fqdn |  | This parameter defines FQDN of the Network Function (See TS 23.003 [5]) allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| ipAddress |  | This parameter defines IP Address of the Network Function. It can be IPv4 address (See RFC 791 [24]) or IPv6 address (See RFC 2373 [25]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| authzInfo |  | This parameter defines NF Specific Service authorization information. It shall include the NF type (s) and NF realms/origins allowed to consume NF Service(s) of NF Service Producer (See TS 23.501[22]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |
| locality |  | The parameter defines information about the location of the NF instance (e.g. geographic location, data center) defined by operator (See TS 29.510[23]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |
| capacity |  | This parameter defines static capacity information in the range of 0-65535, expressed as a weight relative to other NF instances of the same type; if capacity is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]) allowedValues: 0-65535 |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nFInfo |  | This parameter includes NF specific data in Managed NF profile allowedValues: N/A |  | type: NFInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| hostAddr |  | This parameter defines host address of a NF allowedValues: N/A |  | type: HostAddr multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| priority |  | This parameter defines Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection; lower values indicate a higher priority. If priority is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]). allowedValues: 0-65535 |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| supportedDataSetIds |  | This parameter defines list of supported data sets in the UDR instance (See TS 29.510[23]). allowedValues: "SUBSCRIPTION", "POLICY", EXPOSURE", "APPLICATION" |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |
| nFSrvGroupId |  | This parameter defines identity of the group that is served by the NF instance (See TS 29.510[23]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| smfServingAreas |  | This parameter defines the SMF service area(s) the UPF can serve (See TS 29.510[23]). allowedValues: N/A |  | type: String multiplicity: 1..* isOrdered: F isUnique: True defaultValue: None isNullable: False |

## 5.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

# 6	Information model definitions for network slice NRM

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

NOTE 1:	The <<OpenModelClass>> NetworkService and <<OpenModelClass>> VNF are defined in [40].

NOTE 2:	The target Network Service (NS) instance represents a group of VNFs and PNFs that are supporting the source network slice subnet instance.

### 6.2.2	Inheritance

Figure 6.2.2-1: Network slice inheritance relationship

## 6.3	Class definitions

### 6.3.1	NetworkSlice

#### 6.3.1.1	Definition

This IOC represents the properties of a network slice instance in a 5G network. For more information about the network slice instance, see 3GPP TS 28.531 [26].

#### 6.3.1.2	Attributes

The NetworkSlice IOC includes attributes inherited from SubNetwork IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| serviceProfileList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nSSIId | M | T | F | F | T |

#### 6.3.1.3	Attribute constraints

None.

#### 6.3.1.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.2	NetworkSliceSubnet

#### 6.3.2.1	Definition

This IOC represents the properties of a network slice subnet instance in a 5G network. For more information about the network slice subnet instance, see 3GPP TS 28.531 [26].

#### 6.3.2.2	Attributes

The NetworkSliceSubnet IOC includes attributes inherited from SubNetwork IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mFIdList | M | T | F | F | T |
| constituentNSSIIdList | M | T | F | F | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| nsInfo | CM | T | F | F | T |
| sliceProfileList | M | T | T | F | T |

#### 6.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nsInfo Support Qualifier | Condition: It shall be supported if the NSS instance is realized in the virtualized environment. Otherwise this attribute shall be absent. |

#### 6.3.2.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.3	ServiceProfile <<dataType>>

#### 6.3.3.1	Definition

This  data type represents the properties of network slice related requirement that should be supported by the network slice instance in 5G network.

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | M | T | T | F | T |

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.4	SliceProfile <<dataType>>

#### 6.3.4.1	Definition

This  data type represents the properties of network slice subnet related requirement that should be supported by the network slice subnet instance in a 5G network.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.5	NsInfo <<dataType>>

#### 6.3.5.1	Definition

This data type represents the properties of network service information (See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]) corresponding to the network slice subnet instance.

#### 6.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nSInstanceId | M | T | F | F | T |
| nsName | O | T | F | F | T |
| description | O | T | F | F | T |

#### 6.3.5.3	Attribute constraints

None.

#### 6.3.5.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 6.4	Attribute definition

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| availability | This parameter specifies the availability requirement for an network slice instance, expressed as a percentage. | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| nSSIId | It is the DN of the MOI for the NSSI associated with the network slice instance. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| serviceProfileId | A unique identifier of property of network slice related requirement should be supported by the network slice instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sliceProfileId | A unique identifier of the property of network slice subnet related requirement should be supported by the network slice subnet instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| constituentNSSIIdList | It is a list of DN of MOI(s) for the constituent NSSI associated with the network slice subnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| mFIdList | It is a list of DN of the MOI(s) for the NF instances associated with the network slice subnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| operationalState | It indicates the operational state of the network slice instance or the network slice subnet instance. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the network slice instance or the network slice subnet instance. It describes the permission to use or prohibition against using the instance, imposed through the OAM services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nsInfo | This attribute contains the NsInfo of the NS instance corresponding to the network slice subnet instance. The NsInfo is described in clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: NsInfo multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nSInstanceId | This attribute specifies the identifier of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nsName | This attribute specifies the name of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| description | This attribute specifies the description of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| perfReq | This parameter specifies the requirements to the NSI in terms of the scenarios defined in the TS 22.261 [28], such as Experienced data rate, Area traffic capacity (density) information of UE density. It is a structure containing the following elements: - list of perfRequirements Depending on the sST value, the list of perfRequirements will be - list of eMBBPerfReq or - list of uRLLCPerfReq or - list of mIoTPerfReq NOTE: the list of mIoTPerfReq is not addressed in the present document. allowedValues: - list of eMBBPerfReq is a list of entries where an entry identifies the performance requirements to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. An entry has the following attributes: expDataRateDL (Integer), expDataRateUL (Integer), areaTrafficCapDL (Integer), areaTrafficCapUL (Integer), userDensity (Integer), activityFactor (Integer), uESpeed (Integer), coverage (String) (see Table 7.1-1 of TS 22.261 [28]). - list of uRLLCPerfReq is a list of entries where an entry identifies the performance requirements to the NSI in terms of the scenarios defined in the clause 7.2.2 of TS 22.261 [28]. An entry has the following attributes: e2eLatency (Integer), jitter (Integer), survivalTime (Integer), cSAvailability (Float), reliability (Float), expDataRate (Integer), payloadSize (String), trafficDensity (Integer), connDensity (Integer), serviceAreaDimension (String) (see clause 7.2.2 of TS 22.261 [28]). NOTE: Limitation on attribute values in instances of ServiceProfile is not addressed in the present document. | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList | This parameter specifies the S-NSSAI list to be supported by the new NSI to be created or the existing NSI to be re-used. sNSSAList is defined in subclause 4.4.1 |  |
| maxNumberofUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| coverageAreaTAList | An attribute specifies a list of TrackingAreas where the NSI can be selected. allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| latency | An attribute specifies the packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice instance. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uEMobilityLevel | An attribute specifies the mobility level of UE accessing the network slice instance. See 6.2.1 of TS 22.261 [28]. allowedValues: stationary, nomadic, restricted mobility, fully mobility. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice instance may be shared with another network slice instance(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| sliceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice subnet instance may be shared with another network slice subnet instance(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| serviceProfileList | An attribute specifies a list of ServiceProfile (see clause 6.3.3) supported by the network slice instance | type: ServiceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sliceProfileList | An attribute specifies a list of SliceProfile (see clause 6.3.4) supported by the network slice subnet instance | type: SliceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This parameter specifies the slice/service type for a ServiceProfile.. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

## 6.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

# 7	Solution Set (SS)

The present document defines the following NRM Solution Set definitions for NR and NG-RAN:

-	XML based 3GPP NR and NG-RAN NRM Solution Set (Annex C).

-	JSON based 3GPP NR and NG-RAN NRM Solution Set (Annex D).

-	YANG based 3GPP NR and NG-RAN NRM Solution Set (Annex E).

The present document defines the following NRM Solution Set definitions for 5GC:

-	XML based 3GPP 5GC NRM Solution Set (Annex F).

-	JSON based 3GPP 5GC NRM Solution Set (Annex G).

-	YANG based 3GPP 5GC NRM Solution Set (Annex H).

The present document defines the following NRM Solution Set definitions for network slice and network slice subnet:

-	XML based 3GPP Network Slice NRM Solution Set (Annex I).

-	JSON based 3GPP Network Slice NRM Solution Set (Annex J).

-	YANG based 3GPP Network Slice NRM Solution Set (Annex K).

Annex A (normative):
Cell state handling

# A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"

The administrative state indicates the permission to use or prohibition against using the cell, imposed through the OAM services. The administrative state has three values: "LOCKED", "SHUTTING DOWN" or "UNLOCKED"

The meanings of these values are defined in ITU-T Recommendation X.731 [18].

The relation between the administrative state and the "Pre-operation state of the gNB-DU Cell" is defined in subclause 8.5 of TS 38.401 [4]. See below an extract from subclause 8.5 of TS 38.401 [4] on the F1 startup and cell activation.

If the operationalState is "ENABLED" (i.e. the resource is physically installed and working) and if the administrativeState is "UNLOCKED", the step "0: Pre-operational state" will exit and the step "1: F1 Setup Request" will be executed."

8.5	F1 Startup and cells activation

This function allows to setup the F1 interface between a gNB-DU and a gNB-CU and it allows to activate the gNB-DU cells.

Figure 8.5-1: F1 startup and cell activation

# A.2	Combined state diagram for gNB cell

This is the Combined state diagram for gNB cell.

Figure A.2-1: Combined gNB cell state diagram

The gNB-DU maintains cell states. The following table is the gNB cell state transition table.

In 3-split and 2-split deployment scenarios, the interactions between gNB-CU and gNB-DU are standardized. The interactions specified under the column "The state transition events and actions" of "The gNB Cell state transition table" below shall be present for the state transition.

In the non-split deployment scenarios, the interactions between gNB-CU and gNB-DU are not standardized. The interactions between gNB-CU and gNB-DU specified under the column "The state transition events and actions" of "The gNB Cell state transition table" can be replaced by other means that is not standardized.

Table A.2-1: The gNB Cell state transition table

| Transition number | The state transition event and actions |
| --- | --- |
| 1 | Event: Receive request to unlock. Action: None. |
| 2 | Event: Receive request to lock. Action: None. |
| 2a | Event: Receive request to lock Action: Send to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. |
| 3 | Event: When the required cell resource is physically installed and working. Action: none. |
| 4 | Event: When the required cell resource is not physically installed or is not working. Action: Send to gNB-CU the "gNB-DU Configuration update message" with cell to delete. |
| 4a | Event: When the required cell resource is physically uninstalled or is not working. Action: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 5 | Event: Receive from gNB-CU the "F1 Setup Response message" (identifying the cell to be activated). The cell is activated successfully. Actions: Do nothing or send gNB-CU the "gNB-DU Configuration Update message" with Cell stated as active' ----- or ----- Event: Receive from gNB-CU the "gNB-CU Configuration Update message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message"). Actions: The cell is activated successfully. Send to gNB-CU the "gNB-CU Configuration Update Response" to confirm the cell is in active state. ----- or ----- Event: Receive from gNB-CU the "gNB-DU Configuration Update Acknowledge message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message") and the cell is activated successfully Actions: Do nothing. |
| 6 | Event: Receive from gNB-CU the "gNB-CU Configuration Update message" and responds with gNB-CU Configuration Update Acknowledge messages. Actions: Respond with gNB-CU Configuration Update Acknowledge messages. ----- or ----- Event: Event: DU experiences an internal failure and decided to place the cell into inactive state. Actions: Send to gNB-CU the "gNB-DU Cell status Update message" |
| 7 | Event: Send to gNB-CU the "F1 Setup request" (identifying the cell that is configured and ready to be activated). Actions: none. ----- or ----- Send to gNB-CU the "gNB-DU Configuration Update message" with the served cell to add. Actions: none. |
| 8 | Event: Sends to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. Receive response from gNB-CU the "gNB-DU Configuration Update Acknowledge message". Actions: None. |
| 9 | Event: Receive request to shut down. Actions: None. |
| 10 | Event: Last user quit. Actions: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 11 | Event: When a cell is created and is configured. Actions: None |
| 12 | Event: When a cell is deleted. Action: None. |

Annex B (normative): 
NSI and NSSI state handling

# B.1 	NSI state handling

An NSI is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time, the management system needs to know the state of an NSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state, operational state and usage state of systems in general.

Figure B.1: Combined NSI state diagram

In an NSI deployment scenario, the interactions between communication service management function, network slice management function and network slice subnet management function are standardized. The interactions specified under the column "The state transition events and actions" of "NSI state transition table" below shall be present for the state transition.

Figure B.2: NSI state diagram with state transition triggers

Table B.1: The NSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | NSMF responds positively to the "Create NSI request" message, the NSI is created and the state is set to Locked |
| 1 | NSMF responds positively to the "Activate NSI request" message (identifying the NSI to be activated). ----- or ------ CM operation to set administrative state to Unlocked. |
| 1a | CM Operation to set administrative state to Unlocked |
| 2 | The last user of the NSI stops using the NSI |
| 2a | CM Operation to set administrative state to Shutting down |
| 3 | When the NSI and its constituents are installed and working NSMF receives positive response to the "Allocate NSSI" message (applicable to the NSI to be enabled). |
| 4 | When the NSI or its constituents are not installed or not working NSMF receives positive response to the "Deallocate NSSI" message (applicable to the NSI to be disabled) |
| 5 | NSMF responds positively to the "Deallocate NSI request" message, the NSI is deleted and the state is set to NULL |

# B.2	State handling of NSSI

An NSSI is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time the management system needs to know the state of an NSSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state, operational state and usage state of systems in general.

Figure B.2.1: Combined NSSI state diagram

In an NSSI deployment scenario, the interactions between CSMF, NSMF and NSSMF are standardized. The interactions specified under the column "The state transition events and actions" of "NSSI state transition table" below shall be present for the state transition.

Figure B.2.2: NSSI state diagram with state transition triggers

Table B.2.1: The NSSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | NSSMF responds positively to the "Create NSSI request" message, the NSSI is created and the state is set to Locked |
| 1 | NSSMF responds positively to the "Activate NSSI request" message (identifying the NSSI to be activated). ----- or ------ CM operation to set administrative state to Unlocked. |
| 1a | CM Operation to set administrative state to Unlocked |
| 2 | The last user of the NSSI stops using the NSSI |
| 2a | CM Operation to set administrative state to Shutting down |
| 3 | When the NSSI constituents are installed and working NSSMF receives positive response to the "Create NSSI constituent" message (applicable to the NSSI to be enabled). |
| 4 | When the NSSI constituents are not installed or not working NSSMF receive positive response to the "Delete NSSI constituent" message (applicable to the NSSI to be disabled) |
| 5 | NSSMF responds positively to the "Delete NSSI request" message, the NSSI is deleted and the state is set to NULL. |

Annex C (normative):
XML definitions for NR NRM

# C.1	General

This annex contains the XML definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM Information Model definitions specified in clause 4.

# C.2	Architectural features

The overall architectural feature of NR NRM information model is specified in clause 4, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations

# C.3	Mapping

## C.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## C.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# C.4	Solution Set definitions

## C.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [33].

The present document defines the NRM-specific XML schema nrNrm.xsd for the NR NRM Information Model defined in clause 4.

XML schema nrNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [33].

## C.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## C.4.3	XML schema "nRNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 NR Network Resource Model

XML schema definition

nrNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:epc="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:sp="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"/>

<simpleType name="GnbId">

<restriction base="unsignedLong">

<maxInclusive value="4294967295"/>

</restriction>

</simpleType>

<simpleType name="GnbIdLength">

<restriction base="integer">

<minLength value="22"/>

<maxLength value="32"/>

</restriction>

</simpleType>

<simpleType name="Nci">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

</restriction>

</simpleType>

<simpleType name="Pci">

<restriction base="unsignedShort">

<maxInclusive value="503"/>

<!-- Minimum value is 0, maximum value is 3x167+2=503 -->

</restriction>

</simpleType>

<simpleType name="NrTac">

<restriction base="unsignedLong">

<maxInclusive value="16777215"/>

<!--5G TAC is 3-octets length -->

</restriction>

</simpleType>

<simpleType name="GnbDuId">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

<!-- Minimum value is 0, maximum value is 2^36-1=68719476735 -->

</restriction>

</simpleType>

<simpleType name="GnbName">

<restriction base="string">

<minLength value="1"/>

<maxLength value="150"/>

</restriction>

</simpleType>

<simpleType name="CyclicPrefix">

<restriction base="integer">

<enumeration value="15"/>

<enumeration value="30"/>

<enumeration value="60"/>

<enumeration value="120"/>

</restriction>

</simpleType>

<simpleType name="QuotaType">

<restriction base="string">

<enumeration value="STRICT"/>

<enumeration value="FLOAT"/>

</restriction>

</simpleType>

<simpleType name="CellState">

<restriction base="string">

<enumeration value="IDLE"/>

<enumeration value="INACTIVE"/>

<enumeration value="ACTIVE"/>

</restriction>

</simpleType>

<simpleType name="BwpContext">

<restriction base="string">

<enumeration value="DL"/>

<enumeration value="UL"/>

<enumeration value="SUL"/>

</restriction>

</simpleType>

<simpleType name="IsInitialBwp">

<restriction base="string">

<enumeration value="INITIAL"/>

<enumeration value="OTHER"/>

</restriction>

</simpleType>

<simpleType name="qOffsetRangeList">

<restriction base="string">

<enumeration value="dB-24"/>

<enumeration value="dB-22"/>

<enumeration value="dB-20"/>

<enumeration value="dB-18"/>

<enumeration value="dB-16"/>

<enumeration value="dB-14"/>

<enumeration value="dB-12"/>

<enumeration value="dB-10"/>

<enumeration value="dB-8"/>

<enumeration value="dB-6"/>

<enumeration value="dB-5"/>

<enumeration value="dB-4"/>

<enumeration value="dB-3"/>

<enumeration value="dB-2"/>

<enumeration value="dB-1"/>

<enumeration value="dB0"/>

<enumeration value="dB1"/>

<enumeration value="dB2"/>

<enumeration value="dB3"/>

<enumeration value="dB4"/>

<enumeration value="dB5"/>

<enumeration value="dB6"/>

<enumeration value="dB8"/>

<enumeration value="dB10"/>

<enumeration value="dB12"/>

<enumeration value="dB14"/>

<enumeration value="dB16"/>

<enumeration value="dB18"/>

<enumeration value="dB20"/>

<enumeration value="dB22"/>

<enumeration value="dB24"/>

</restriction>

</simpleType>

<simpleType name="cellReselectionPriority">

<restriction base="unsignedLong">

<minInclusive value="0"/>

<maxInclusive value="16"/>

<!--Value 0 means lowest priority-->

</restriction>

</simpleType>

<simpleType name="cellReselectionSubPriority">

<restriction base="unsignedLong">

<minInclusive value="0"/>

<maxInclusive value="16"/>

<!--Value 0 means lowest priority-->

</restriction>

</simpleType>

<simpleType name="PMaxRangeType">

<restriction base="short">

<minInclusive value="-30"/>

<maxInclusive value="33"/>

</restriction>

</simpleType>

<simpleType name="qOffsetFreq">

<restriction base="short">

<minInclusive value="-24"/>

<maxInclusive value="24"/>

</restriction>

</simpleType>

<simpleType name="qQualMin">

<restriction base="integer">

<minInclusive value="-34"/>

<maxInclusive value="0"/>

</restriction>

</simpleType>

<simpleType name="qRxLevMin">

<restriction base="integer">

<minInclusive value="-140"/>

<maxInclusive value="-44"/>

</restriction>

</simpleType>

<simpleType name="Thresxhighp">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="62"/>

</restriction>

</simpleType>

<simpleType name="Threshxhighq">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="31"/>

</restriction>

</simpleType>

<simpleType name="Threshxlowp">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="62"/>

</restriction>

</simpleType>

<simpleType name="Threshxlowq">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="62"/>

</restriction>

</simpleType>

<simpleType name="Treselectionnr">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="7"/>

</restriction>

</simpleType>

<simpleType name="Treselectionnrsfhigh">

<restriction base="string">

<enumeration value="25"/>

<enumeration value="50"/>

<enumeration value="75"/>

<enumeration value="100"/>

</restriction>

</simpleType>

<simpleType name="Treselectionnrsfmedium">

<restriction base="string">

<enumeration value="25"/>

<enumeration value="50"/>

<enumeration value="75"/>

<enumeration value="100"/>

</restriction>

</simpleType>

<simpleType name="Absolutefrequencyssb">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="3279165"/>

</restriction>

</simpleType>

<simpleType name="Ssbsubcarrierspacing">

<restriction base="string">

<enumeration value="15"/>

<enumeration value="30"/>

<enumeration value="120"/>

<enumeration value="240"/>

</restriction>

</simpleType>

<simpleType name="Multifrequencybandlistnr">

<restriction base="integer">

<minInclusive value="1"/>

<maxInclusive value="256"/>

</restriction>

</simpleType>

<complexType name="LocalEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

<element name="vlanId" type="integer"/>

</sequence>

</complexType>

<complexType name="RemoteEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

</sequence>

</complexType>

<complexType name="blackListEntry">

<sequence minOccurs="0" maxOccurs="1007" >

<element name="pci" type="en:Pci" maxOccurs="504"/>

</sequence>

</complexType>

<complexType name="blackListEntryIdleMode">

<sequence minOccurs="0" maxOccurs="1007" >

<element name="pci" type="en:Pci" maxOccurs="504"/>

</sequence>

</complexType>

<complexType name="RRMPolicyRation2">

<sequence>

<element name="groupId" type="integer"/>

<element name="sNSSAIList" type="ngc:SnssaiList"/>

<element name="quotaType" type="QuotaType"/>

<element name="rRMPolicyMaxRation" type="integer"/>

<element name="rRMPolicyMarginMaxRation" type="integer"/>

<element name="rRMPolicyMinRation" type="integer"/>

<element name="rRMPolicyMarginMinRation" type="integer"/>

</sequence>

</complexType>

<complexType name="PLMNIdList">

<sequence>

<element name="pLMNId" type="en:PLMNId" maxOccurs="6"/>

<!-- The first pLMNId of the pLMNIdList is primary PLMN id -->

</sequence>

</complexType>

<complexType name="cellIndividualOffset">

<sequence>

<element name="rsrpOffsetSSB" type="qOffsetRangeList"/>

<element name="rsrqOffsetSSB" type="qOffsetRangeList"/>

<element name="sinrOffsetSSB" type="qOffsetRangeList"/>

<element name="rsrpOffsetCSI-RS" type="qOffsetRangeList"/>

<element name="rsrqOffsetCSI-RS" type="qOffsetRangeList"/>

<element name="sinrOffsetCSI-RS" type="qOffsetRangeList"/>

</sequence>

</complexType>

<element name="GNBDUFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="gnbId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbDUId" type="nn:GnbDuId"/>

<element name="gnbDuName" type="nn:GnbName" minOccurs="0"/>

<element name="pLMNId" type="en:PLMNId"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellDU"/>

<element ref="nn:BWP"/>

<element ref="nn:NRSectorCarrier"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_F1U"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUCPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="gnbId" type="nn:GnbId" />

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbCuName" type=" nn:GnbName" minOccurs="0"/>

<element name="pLMNId" type="en:PLMNId" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellCU"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_E1"/>

<element ref="nn:EP_XnC"/>

<element ref="nn:EP_X2C"/>

<element ref="nn:EP_NgC"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUUPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="gNBId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:EP_E1"/>

<element ref="nn:EP_F1U"/>

<element ref="nn:EP_XnU"/>

<element ref="nn:EP_NgU"/>

<element ref="nn:EP_X2U"/>

<element ref="nn:EP_S1U"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellCU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="nCGI" type="nn:Ncgi"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="rRMPolicyType" type="integer" minOccurs="0"/>

<element name="rRMPolicyNSSIId" type="xn:dn" minOccurs="0"/>

<element name="rRMPolicyRatio" type="integer" minOccurs="0"/>

<element name="rRMPolicy" type="string" minOccurs="0"/>

<element name="rRMPolicyRatio2" type="RRMPolicyRation2" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="nRCellRelation"/>

<element ref="nRFreqRelation"/>

<element ref="eUtranCellRelation"/>

<element ref="eUtranFreqRelation"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellDU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="nCGI" type="nn:Ncgi" minOccurs="0"/>

<element name="operationalState" type="sm:operationalStateType" minOccurs="0"/>

<element name="administrativeState" type="sm:administrativeStateType" minOccurs="0"/>

<element name="cellState" type="nn:CellState"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="nRpci" type="nn:Pci" />

<element name="nRTac" type="nn:NrTac" />

<element name="arfcnDL" type="integer"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="arfcnSUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwSUL" type="integer" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

<element name="nRSectorCarrierRef" type="xn:dn" minOccurs="0"/>

<element name="bWPRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRSectorCarrier">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="txDirection" type="nn:TxDirection"/>

<element name="configuredMaxTxPower" type="integer"/>

<element name="arfcnDL" type="integer" minOccurs="0"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer" minOccurs="0"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

<element name="sectorEquipmentFunctionRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="BWP">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="bwpContext" type="nn:BwpContext"/>

<element name="isInitialBwp" type="nn:IsInitialBwp"/>

<element name="subCarrierSpacing" type="integer"/>

<element name="cyclicPrefix" type="nn:CyclicPrefix"/>

<element name="startRB" type="integer"/>

<element name="numberOfRBs" type="integer"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_E1">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LoacalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellRelation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="nRTCI" type="nn:Nrtci"/>

<element name="cellIndividualOffset" type="en:CellIndividualOffset"/>

<element name="nRFreqRelationRef" type="xn:dn" minOccurs="0"/>

<element name="adjacentNRCellRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFreqRelation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="offsetMO" type="en:qOffsetRangeList"/>

<element name="blackListEntry" type="en:blackListEntry" minOccurs="0"/>

<element name="blackListEntryIdleMode" type="en:blackListEntryIdleMode" minOccurs="0"/>

<element name="cellReselectionPriority" type="en:cellReselectionPriority"/>

<element name="cellReselectionSubPriority" type="en:cellReselectionSubPriority"/>

<element name="pMax" type="en:PMaxRangeType" minOccurs="0"/>

<element name="qOffserFreq" type="nn:qOffserFreq" minOccurs="0"/>

<element name="qQualMin" type="en:qQualMin" minOccurs="0"/>

<element name="qRxLevMin" type="en:qRxLevMin" minOccurs="0"/>

<element name="threshXHighP" type="en:threshxhighp" minOccurs="0"/>

<element name="threshXHighQ" type="en:threshxhighq" minOccurs="0"/>

<element name="threshXLowP" type="en:threshxlowp" minOccurs="0"/>

<element name="threshXLowQ" type="en:threshxlowp" minOccurs="0"/>

<element name="tReselectionNr" type="nn:Treselectionnr" minOccurs="0"/>

<element name="tReselectionNRSfHigh" type="nn:Treselectionnrsfhigh" minOccurs="0"/>

<element name="tReselectionNRSfMedium" type="nn:Treselectionnrsfmedium" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalNRCellCU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="nCGI" type="nn:Ncgi"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRPCI" type="nn:Nrpci" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalGNBCUCPFunction" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass ">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="gnbId" type="nn:GnbId" />

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="pLMNId" type="en:PLMNIdList" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="RRMPolicyRatio2">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="groupId" type="integer"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="quotaType" type="nn:quotaType"/>

<element name="rRMPolicyMaxRatio" type="integer" minOccurs="0"/>

<element name="rRMPolicyMarginMaxRatio" type="integer" minOccurs="0"/>

<element name="rRMPolicyMinRatio" type="integer" minOccurs="0"/>

<element name="rRMPolicyMarginMinRatio" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFrequency" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="absoluteFrequencySSB" type="nn:Absolutefrequencyssb" minOccurs="0"/>

<element name="sSBSubCarrierSpacing" type="nn:Ssbsubcarrierspacing" minOccurs="0"/>

<element name="multiFrequencyBandListNR" type="nn:MultifrequencyBandlistnr" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex D (normative):
JSON definitions for NR NRM

# D.1	General

This annex contains the JSON definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# D.2	Architectural features

# D.3	Mapping

Refer to TS 32.160 [47].

## are listed in the following table.

are listed in the following table.

D.4	Solution Set (SS) definitions

## D.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## D.4.2	Graphical representation

None.

## D.4.3	JSON schema "nrNrm.json"

{

"openapi": "3.0.1",

"info": {

"title": "3GPP NR NRM",

"version": "16.1.0",

"description": "OAS 3.0.1 specification compatible schema for 3GPP NR NRM"

},

"paths": {},

"components": {

"schemas": {

"GnbId": {

"type": "string"

},

"GnbIdLength": {

"type": "integer",

"minimum": 22,

"maximum": 32

},

"GnbName": {

"type": "string",

"maxLength": 150

},

"GnbDuId": {

"type": "number",

"minimum": 0,

"maximum": 68719476735

},

"GnbCuUpId": {

"type": "number",

"minimum": 0,

"maximum": 68719476735

},

"NCi": {

"type": "object",

"properties": {

"plmnId": {

"$ref": "#/components/schemas/PlmnId"

},

"nCI": {

"$ref": "#/components/schemas/NrCellId"

}

}

},

"SnssaiList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/Snssai"

}

},

"RrmPolicy": {

"type": "string"

},

"NrPci": {

"type": "integer",

"maximum": 503

},

"NrTac": {

"type": "integer",

"maximum": 16777215

},

"NrCellId": {

"type": "integer",

"maximum": 68719476735

},

"Sst": {

"type": "integer",

"maximum": 255

},

"Snssai": {

"type": "object",

"properties": {

"sst": {

"$ref": "#/components/schemas/Sst"

},

"sd": {

"type": "string"

}

}

},

"CellState": {

"type": "string",

"enum": [

"IDLE",

"INACTIVE",

"ACTIVE"

]

},

"CyclicPrefix": {

"type": "string",

"enum": [

"15",

"30",

"60",

"120"

]

},

"TxDirection": {

"type": "string",

"enum": [

"DL",

"UL",

"DL and UL"

]

},

"BwpContext": {

"type": "string",

"enum": [

"DL",

"UL",

"SUL"

]

},

"IsInitialBwp": {

"type": "string",

"enum": [

"INITIAL",

"OTHER",

"SUL"

]

},

"QuotaType": {

"type": "string",

"enum": [

"STRICT",

"FLOAT"

]

},

"RrmPolicyRatio2": {

"type": "object",

"properties": {

"groupId": {

"type": "integer"

},

"sNSSAIList": {

"$ref": "#/components/schemas/SnssaiList"

},

"quotaType": {

"$ref": "#/components/schemas/QuotaType"

},

"rRMPolicyMaxRation": {

"type": "integer"

},

"rRMPolicyMarginMaxRation": {

"type": "integer"

},

"rRMPolicyMinRation": {

"type": "integer"

},

"rRMPolicyMarginMinRation": {

"type": "integer"

}

}

},

"Mnc": {

"type": "string",

"pattern": "[0-9]{3}|[0-9]{2}"

},

"PlmnId": {

"type": "object",

"properties": {

"mcc": {

"$ref": "genericNrm.json#/components/schemas/Mcc"

},

"mnc": {

"$ref": "#/components/schemas/Mnc"

}

}

},

"PlmnIdList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/PlmnId"

}

},

"LocalAddress": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv4Addr"

},

"ipv6Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Addr"

},

"vlanId": {

"type": "integer",

"minimum": 0,

"maximum": 4096

},

"port": {

"type": "integer",

"minimum": 0,

"maximum": 65535

}

}

},

"RemoteAddress": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv4Addr"

},

"ipv6Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Addr"

}

}

},

"CellIndividualOffset": {

"type": "object",

"properties": {

"rsrpOffsetSSB": {

"type": "integer"

},

"rsrqOffsetSSB": {

"type": "integer"

},

"sinrOffsetSSB": {

"type": "integer"

},

"rsrpOffsetCSI-RS": {

"type": "integer"

},

"rsrqOffsetCSI-RS": {

"type": "integer"

},

"sinrOffsetCSI-RS": {

"type": "integer"

}

}

},

"QOffsetRange": {

"type": "integer",

"enum": [

-24,

-22,

-20,

-18,

-16,

-14,

-12,

-10,

-8,

-6,

-5,

-4,

-3,

-2,

-1,

0,

24,

22,

20,

18,

16,

14,

12,

10,

8,

6,

5,

4,

3,

2,

1

]

},

"QOffsetRangeList": {

"type": "object",

"properties": {

"rsrpOffsetSSB": {

"$ref": "#/components/schemas/QOffsetRange"

},

"rsrqOffsetSSB": {

"$ref": "#/components/schemas/QOffsetRange"

},

"sinrOffsetSSB": {

"$ref": "#/components/schemas/QOffsetRange"

},

"rsrpOffsetCSI-RS": {

"$ref": "#/components/schemas/QOffsetRange"

},

"rsrqOffsetCSI-RS": {

"$ref": "#/components/schemas/QOffsetRange"

},

"sinrOffsetCSI-RS": {

"$ref": "#/components/schemas/QOffsetRange"

}

}

},

"QOffsetFreq": {

"type": "number"

},

"TReselectionNRSf": {

"type": "integer",

"enum": [

25,

50,

75,

100

]

},

"SsbPeriodicity": {

"type": "integer",

"enum": [

5,

10,

20,

40,

80,

160

]

},

"SsbDuration": {

"type": "integer",

"enum": [

1,

2,

3,

4,

5

]

},

"SsbSubCarrierSpacing": {

"type": "integer",

"enum": [

15,

30,

120,

240

]

},

"GnbDuFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbDuId": {

"$ref": "#/components/schemas/GnbDuId"

},

"gnbDuName": {

"$ref": "#/components/schemas/GnbName"

},

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_F1C": {

"$ref": "#/components/schemas/EP_F1C"

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

},

"NrCellDu": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrCellDu"

}

},

"NrSectorCarrier": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrSectorCarrier"

}

},

"Bwp": {

"type": "array",

"items": {

"$ref": "#/components/schemas/Bwp"

}

}

}

}

]

},

"GnbCuCpFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

},

"gnbCuName": {

"$ref": "#/components/schemas/GnbName"

},

"plmnId": {

"$ref": "#/components/schemas/PlmnId"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_F1C": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1C"

}

},

"EP_E1": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_E1"

}

},

"EP_XnC": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_XnC"

}

},

"EP_X2C": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_X2C"

}

},

"EP_NgC": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_NgC"

}

},

"NrCellCu": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrCellCu"

}

}

}

}

]

},

"GnbCuUpFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

},

"gnbCuUpId": {

"$ref": "#/components/schemas/GnbCuUpId"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_E1": {

"$ref": "#/components/schemas/EP_E1"

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

},

"EP_XnU": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_XnU"

}

},

"EP_NgU": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_NgU"

}

},

"EP_X2U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_X2U"

}

},

"EP_S1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_S1U"

}

}

}

}

]

},

"NrCellCu": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"cellLocalId": {

"type": "integer"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

},

"snssaiList": {

"$ref": "#/components/schemas/SnssaiList"

},

"rrmPolicyType": {

"type": "integer"

},

"rrmPolicyNSSIId": {

"$ref": "genericNrm.json#/components/schemas/Dn"

},

"rrmPolicyRatio": {

"type": "integer"

},

"rrmPolicy": {

"$ref": "#/components/schemas/RrmPolicy"

},

"rrmPolicyRatio2": {

"$ref": "#/components/schemas/RrmPolicyRatio2"

},

"nRFrequencyRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"NRCellRelation": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NRCellRelation"

}

},

"NRFreqRelation": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NRFreqRelation"

}

},

"EUtranCellRelation": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EUtranCellRelation"

}

},

"EUtranFreqRelation": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EUtranFreqRelation"

}

}

}

}

]

},

"NrCellDu": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"administrativeState": {

"$ref": "genericNrm.json#/components/schemas/AdministrativeState"

},

"operationalState": {

"$ref": "genericNrm.json#/components/schemas/OperationalState"

},

"cellLocalId": {

"type": "integer"

},

"cellState": {

"$ref": "#/components/schemas/CellState"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

},

"snssaiList": {

"$ref": "#/components/schemas/SnssaiList"

},

"nrPci": {

"$ref": "#/components/schemas/NrPci"

},

"nrTac": {

"$ref": "#/components/schemas/NrTac"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"arfcnSUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

"bSChannelBwSUL": {

"type": "integer"

},

"ssbFrequency": {

"type": "integer",

"minimum": 0,

"maximum": 3279165

},

"ssbPeriodicity": {

"$ref": "#/components/schemas/SsbPeriodicity"

},

"ssbSubCarrierSpacing": {

"$ref": "#/components/schemas/SsbSubCarrierSpacing"

},

"ssbOffset": {

"type": "integer",

"minimum": 0,

"maximum": 159

},

"ssbDuration": {

"$ref": "#/components/schemas/SsbDuration"

},

"nrSectorCarrierRef": {

"type": "array",

"items": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

},

"bwpRef": {

"type": "array",

"items": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

},

"nRFrequencyRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"NrSectorCarrier": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"txDirection": {

"$ref": "#/components/schemas/TxDirection"

},

"configuredMaxTxPower": {

"type": "integer"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

"sectorEquipmentFunctionRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"Bwp": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"bwpContext": {

"$ref": "#/components/schemas/BwpContext"

},

"isInitialBwp": {

"$ref": "#/components/schemas/IsInitialBwp"

},

"subCarrierSpacing": {

"type": "integer"

},

"cyclicPrefix": {

"$ref": "#/components/schemas/CyclicPrefix"

},

"startRB": {

"type": "integer"

},

"numberOfRBs": {

"type": "integer"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"ExternalGnbDuFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_F1C": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1C"

}

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

}

}

}

]

},

"ExternalGnbCuCpFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

},

"plmnId": {

"$ref": "#/components/schemas/PlmnId"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"ExternalNrCellCu": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalNrCellCu"

}

},

"EP_F1C": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1C"

}

},

"EP_E1": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_E1"

}

},

"EP_XnC": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_XnC"

}

}

}

}

]

},

"ExternalGnbCuUpFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"gnbId": {

"$ref": "#/components/schemas/GnbId"

},

"gnbIdLength": {

"$ref": "#/components/schemas/GnbIdLength"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_E1": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_E1"

}

},

"EP_F1U": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_F1U"

}

},

"EP_XnU": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_XnU"

}

}

}

}

]

},

"ExternalAmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_NgC": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_NgC"

}

}

}

}

]

},

"ExternalUpfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_NgU": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EP_NgU"

}

}

}

}

]

},

"ExternalNrCellCu": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"cellLocalId": {

"type": "integer"

},

"nrPci": {

"$ref": "#/components/schemas/NrPci"

},

"plmnIdList": {

"$ref": "#/components/schemas/PlmnIdList"

},

"nRFrequencyRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"NRCellRelation": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"nRTCI": {

"type": "integer"

},

"cellIndividualOffset": {

"$ref": "#/components/schemas/CellIndividualOffset"

},

"adjacentNRCellRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

},

"nRFrequencyRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"NRFreqRelation": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"offsetMO": {

"$ref": "#/components/schemas/QOffsetRangeList"

},

"blackListEntry": {

"type": "array",

"items": {

"type": "integer",

"minimum": 0,

"maximum": 1007

}

},

"blackListEntryIdleMode": {

"type": "integer"

},

"cellReselectionPriority": {

"type": "integer"

},

"cellReselectionSubPriority": {

"type": "number",

"minimum": 0.2,

"maximum": 0.8,

"multipleOf": 0.2

},

"pMax": {

"type": "integer",

"minimum": -30,

"maximum": 33

},

"qOffsetFreq": {

"$ref": "#/components/schemas/QOffsetFreq"

},

"qQualMin": {

"type": "number"

},

"qRxLevMin": {

"type": "integer",

"minimum": -140,

"maximum": -44

},

"threshXHighP": {

"type": "integer",

"minimum": 0,

"maximum": 62

},

"threshXHighQ": {

"type": "integer",

"minimum": 0,

"maximum": 31

},

"threshXLowP": {

"type": "integer",

"minimum": 0,

"maximum": 62

},

"threshXLowQ": {

"type": "integer",

"minimum": 0,

"maximum": 31

},

"tReselectionNr": {

"type": "integer",

"minimum": 0,

"maximum": 7

},

"tReselectionNRSfHigh": {

"$ref": "#/components/schemas/TReselectionNRSf"

},

"tReselectionNRSfMedium": {

"$ref": "#/components/schemas/TReselectionNRSf"

},

"nRFrequencyRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"NRFrequency": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"absoluteFrequencySSB": {

"type": "integer",

"minimum": 0,

"maximum": 3279165

},

"ssbSubCarrierSpacing": {

"$ref": "#/components/schemas/SsbSubCarrierSpacing"

},

"multiFrequencyBandListNR": {

"type": "integer",

"minimum": 1,

"maximum": 256

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"ExternalENBFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"eNBId": {

"type": "integer"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"ExternalEUTranCell": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalEUTranCell"

}

}

}

}

]

},

"ExternalEUTranCell": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"EUtranFrequencyRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"EUtranCellRelation": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"adjacentEUtranCellRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"EUtranFreqRelation": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"eUTranFrequencyRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"EUtranFrequency": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

}

]

},

"ManagedElement-Single": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedElement-Attributes"

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedElement-ContainingObjects"

},

{

"type": "object",

"properties": {

"GnbDuFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/GnbDuFunction"

}

},

"GnbCuCpFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/GnbCuCpFunction"

}

},

"GnbCuUpFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/GnbCuUpFunction"

}

}

}

}

]

},

"ManagedElement-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ManagedElement-Single"

}

},

"SubNetwork-Single": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-ContainingObjects"

},

{

"type": "object",

"properties": {

"SubNetwork": {

"$ref": "#/components/schemas/SubNetwork-Multiple"

},

"ManagedElement": {

"$ref": "#/components/schemas/ManagedElement-Multiple"

},

"ExternalGnbCuCpFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalGnbCuCpFunction"

}

},

"ExternalENBFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalENBFunction"

}

},

"NRFrequency": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NRFrequency"

}

},

"EUtranFrequency": {

"type": "array",

"items": {

"$ref": "#/components/schemas/EUtranFrequency"

}

}

}

}

]

},

"SubNetwork-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SubNetwork-Single"

}

},

"EP_RP": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/components/schemas/LocalAddress"

},

"remoteAddress": {

"$ref": "#/components/schemas/RemoteAddress"

}

}

}

}

}

]

},

"EP_E1": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_XnC": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_XnU": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_NgC": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_NgU": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_F1C": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_F1U": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_S1U": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_X2C": {

"$ref": "#/components/schemas/EP_RP"

},

"EP_X2U": {

"$ref": "#/components/schemas/EP_RP"

}

}

}

}

Annex E (normative):
YANG definitions for NR NRM

# E.1	General

This annex contains the YANG definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# E.2	Void

# E.3	Void

## nformation model to SS equivalent definitions are listed in the following table.

# E.4	Void

# E.5	Modules

This is the list of YANG modules for NR and NG-RAN NRM.

_3gpp-nr-nrm-bwp.yang

_3gpp-nr-nrm-ep.yang

_3gpp-nr-nrm-eutrancellrelation.yang

_3gpp-nr-nrm-eutranetwork.yang

_3gpp-nr-nrm-eutranfreqrelation.yang

_3gpp-nr-nrm-eutranfrequency.yang

_3gpp-nr-nrm-externalamffunction.yang

_3gpp-nr-nrm-externalenbfunction.yang

_3gpp-nr-nrm-externaleutrancell.yang

_3gpp-nr-nrm-externalgnbcucpfunction.yang

_3gpp-nr-nrm-externalgnbcuupfunction.yang

_3gpp-nr-nrm-externalgnbdufunction.yang

_3gpp-nr-nrm-externalnrcellcu.yang

_3gpp-nr-nrm-externalservinggwfunction.yang

_3gpp-nr-nrm-externalupffunction.yang

_3gpp-nr-nrm-gnbcucpfunction.yang

_3gpp-nr-nrm-gnbcuupfunction.yang

_3gpp-nr-nrm-gnbdufunction.yang

_3gpp-nr-nrm-nrcellcu.yang

_3gpp-nr-nrm-nrcelldu.yang

_3gpp-nr-nrm-nrcellrelation.yang

_3gpp-nr-nrm-nrfreqrelation.yang

_3gpp-nr-nrm-nrfrequency.yang

_3gpp-nr-nrm-nrnetwork.yang

_3gpp-nr-nrm-nrsectorcarrier.yang

This is the YANG modules for NR and NG-RAN NRM.

module _3gpp-nr-nrm-bwp {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-bwp";

prefix "bwp3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the BWP Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

typedef CyclicPrefix {

type enumeration {

enum NORMAL;

enum EXTENDED;

}

}

typedef BwpContext {

type enumeration {

enum DL;

enum UL;

enum SUL;

}

}

typedef IsInitialBwp {

type enumeration {

enum INITIAL;

enum OTHER;

}

}

grouping BWPGrp {

description "Represents the BWP IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf bwpContext {

description "Identifies whether the object is used for downlink, uplink

or supplementary uplink.";

mandatory true;

type BwpContext;

}

leaf isInitialBwp {

description "Identifies whether the object is used for initial or other

BWP.";

mandatory true;

type IsInitialBwp;

}

leaf subCarrierSpacing {

description "Subcarrier spacing configuration for a BWP.";

reference "3GPP TS 38.104";

mandatory true;

type uint32 { range "5 | 30 | 60 | 120"; }

units kHz;

}

leaf cyclicPrefix {

description "Cyclic prefix, which may be normal or extended.";

reference "3GPP TS 38.211";

mandatory true;

type CyclicPrefix;

}

leaf startRB {

description "Offset in common resource blocks to common resource block 0

for the applicable subcarrier spacing for a BWP.";

reference "N_BWP_start in 3GPP TS 38.211";

mandatory true;

type uint32;

}

leaf numberOfRBs {

description "Number of physical resource blocks for a BWP.";

reference "N_BWP_size in 3GPP TS 38.211";

mandatory true;

type uint32;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list BWP {

description "Represents a bandwidth part (BWP).";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses BWPGrp;

}

}

}

}

module _3gpp-nr-nrm-ep {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-ep";

prefix "ep3gpp";

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-gnbcuupfunction { prefix gnbcuup3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NR related endpoint

Information Object Classes (IOCs) that are part of the NR Network

Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping EP_E1Grp {

description "Represents the EP_E1 IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

uses eprp3gpp:EP_Common;

}

grouping EP_F1CGrp {

description "Represents the EP_F1C IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_F1UGrp {

description "Represents the EP_F1U IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_XnCGrp {

description "Represents the EP_XnC IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_XnUGrp {

description "Represents the EP_XnU IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_NgCGrp {

description "Represents the EP_NgC IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_NgUGrp {

description "Represents the EP_NgU IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_X2CGrp {

description "Represents the EP_X2C IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.423";

uses eprp3gpp:EP_Common;

}

grouping EP_X2UGrp {

description "Represents the EP_X2U IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.425";

uses eprp3gpp:EP_Common;

}

grouping EP_S1UGrp {

description "Represents the EP_S1U IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.410";

uses eprp3gpp:EP_Common;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

list EP_F1C {

description "Represents the local end point of the control plane

interface (F1-C) between the DU and CU or CU-CP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1CGrp;

}

}

list EP_NgC {

description "Represents the local end point of the control plane

interface (NG-C) between the gNB and NG-Core entity.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgCGrp;

}

}

list EP_XnC {

description "Represents the local gNB node end point of the logical

link, supporting Xn application protocols, to a neighbour gNB node.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnCGrp;

}

}

list EP_X2C {

description "Represents the local end point of the logical link,

supporting X2-C application protocols used in EN-DC, to a neighbour

eNB or en-gNB node.";

reference "3GPP TS 28.541, 3GPP TS 36.423";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2CGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbcuup3gpp:GNBCUUPFunction" {

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

list EP_F1U {

description "Represents the local end point of the user plane

interface (F1-U) between the DU and CU or CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1UGrp;

}

}

list EP_NgU {

description "Represents the local end point of the NG user plane

(NG-U) interface between the gNB and the UPGW.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgUGrp;

}

}

list EP_XnU {

description "Represents the one end-point of a logical link supporting

the Xn user plane (Xn-U) interface. The Xn-U interface provides

non-guaranteed delivery of user plane PDUs between two NG-RAN nodes.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnUGrp;

}

}

list EP_X2U {

description "Represents the local end-point of a logical link supporting

the X2 user plane (X2-U) interface used in EN-DC.";

reference "3GPP TS 28.541, 3GPP TS 36.425";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2UGrp;

}

}

list EP_S1U {

description "Represents the local end point of the logical link,

supporting S1-U interface towards a S-GW node.";

reference "3GPP TS 28.541, 3GPP TS 36.410";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S1UGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list EP_F1C {

description "Represents the local end point of the control plane

interface (F1-C) between the DU and CU or CU-CP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1CGrp;

}

}

list EP_F1U {

description "Represents the local end point of the user plane

interface (F1-U) between the DU and CU or CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1UGrp;

}

}

}

}

module _3gpp-nr-nrm-eutrancellrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutrancellrelation";

prefix "eutrancellrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranCellRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

typedef ActionAllowed {

type enumeration {

enum YES;

enum NO;

}

}

typedef EnergySavingCoverage {

type enumeration {

enum YES;

enum NO;

enum PARTIAL;

}

}

grouping EUtranCellRelationGrp {

description "Represents the EUtranCellRelation IOC.";

reference "3GPP TS 28.541, EUtranRelation in 3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf tCI {

description "Target Cell Identifier. Consists of E-UTRAN Cell Global

Identifier (ECGI) and Physical Cell Identifier (PCI) of the target

cell. Identifies the target cell from the perspective of the parent

cell instance.";

mandatory true;

type uint64;

}

leaf isRemoveAllowed {

description "Indicates if the subject EUtranCellRelation can be removed

(deleted) or not. If YES, the subject EUtranCellRelation instance can

be removed (deleted). If NO, the subject EUtranCellRelation instance

shall not be removed (deleted) by any entity but an IRPManager.";

mandatory true;

type ActionAllowed;

}

leaf isHOAllowed {

description "Indicates if handover is allowed or prohibited. If YES,

handover is allowed from source cell to target cell. Source cell is

represented by the parent cell instance. Target cell is the adjacent

cell referenced by this EUtranCellRelation instance. If NO, handover

shall not be allowed.";

mandatory true;

type ActionAllowed;

}

leaf isICICInformationSendAllowed {

description "Indicates if ICIC (Inter Cell Interference Coordination)

load information message sending is allowed or prohibited. If YES,

ICIC load information message sending is allowed from source cell to

target cell. Source cell is represented by the parent cell instance.

Target cell is the adjacent cell referenced by this EUtranCellRelation

instance. If NO, ICIC load information message sending shall not be

allowed.";

reference "3GPP TS 36.423";

mandatory true;

type ActionAllowed;

}

leaf isLBAllowed {

description "Indicates if load balancing is allowed or prohibited from

source cell to target cell. If YES, load balancing is allowed from

source cell to target cell. Source cell is represented by the parent

cell instance. Target cell is the adjacent cell referenced by this

EUtranCellRelation instance. If NO, load balancing shall be prohibited

from source cell to target cell.";

mandatory true;

type ActionAllowed;

}

leaf isESCoveredBy {

description "Indicates whether the adjacent cell according to this

planning provides no, partial or full coverage for the parent cell

instance. Adjacent cells with this attribute equal to YES are

recommended to be considered as candidate cells to take over the

coverage when the original cell is about to be transferred to energy

saving state. The entirety of adjacent cells with this property equal

to PARTIAL are recommended to be considered as entirety of candidate

cells to take over the coverage when the original cell is about to be

transferred to energy saving state.";

mandatory true;

type EnergySavingCoverage;

}

leaf qOffset {

description "Offset applicable to a specific neighbouring cell used for

evaluating the cell as a candidate for cell re-selection. Corresponds

to parameter q-OffsetCell broadcast in SIB4 for intra-frequency cells

and in SIB5 for inter-frequency cells. Used for Mobility Robustness

Optimization.";

reference "3GPP TS 36.331";

mandatory true;

type types3gpp:QOffsetRange;

}

leaf cellIndividualOffset {

description "Offset applicable to a neighbouring cell. It is used for

evaluating the neighbouring cell for handover in connected mode. Used

by the HandOver parameter Optimization (HOO) function or Load

Balancing Optimization (LBO) function.";

reference "3GPP TS 36.331";

config false;

type types3gpp:QOffsetRange;

}

leaf adjacentCell {

description "Reference to an EUtranCellFDD/TDD or

ExternalEUtranCellFDD/TDD instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list EUtranCellRelation {

description "Represents a relation between an NR cell and an E-UTRAN cell.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranCellRelationGrp;

}

}

}

}

module _3gpp-nr-nrm-eutranetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranetwork";

prefix "eutranet3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtraNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

feature ExternalsUnderEUtraNetwork {

description "Classes representing external entities like EUtranFrequency,

ExternalENBFunction are contained under a EUtraNetwork list/class.";

}

grouping EUtraNetworkGrp {

description "Represents the EUtraNetwork IOC.";

reference "3GPP TS 28.541";

uses subnet3gpp:SubNetworkGrp;

}

list EUtraNetwork {

description "A subnetwork containing gNB external E-UTRAN entities.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtraNetworkGrp;

leaf-list parents {

description "Reference to all containg EUtraNetwork instances

in strict order from the root EUtraNetwork down to the immediate

parent EUtraNetwork.

If EUtraNetworks form a containment hierarchy this is

modeled using references between the child EUtraNetwork and the parent

EUtraNetworks.

This reference MUST NOT be present for the top level EUtraNetwork and

MUST be present for other EUtraNetworks.";

type leafref {

path "../../../EUtraNetwork/id";

}

}

leaf-list containedChildren{

description "Reference to all directly contained EUtraNetwork instances.

If EUtraNetworks form a containment hierarchy this is

modeled using references between the child EUtraNetwork and the parent

EUtraNetwork.";

type leafref {

path "../../../EUtraNetwork/id";

}

}

}

}

}

module _3gpp-nr-nrm-eutranfreqrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfreqrelation";

prefix "eutranfreqrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranFreqRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping EUtranFreqRelationGrp {

description "Represents the EUtranFreqRelation IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellIndividualOffset {

description "Offset applicable to a neighbouring cell. Used for

evaluating the neighbouring cell for handover in connected mode.

Used by the HandOver parameter Optimization (HOO) function or

Load Balancing Optimization (LBO) function.";

reference "cellIndividualOffset in MeasObjectEUTRA in 3GPP TS 38.331";

default 0;

type types3gpp:QOffsetRange;

}

leaf-list blackListEntry {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in E-UTRAN measurements.";

reference "3GPP TS 38.331";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf-list blackListEntryIdleMode {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in SIB4 and SIB5.";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf cellReselectionPriority {

description "The absolute priority of the carrier frequency used by the

cell reselection procedure. Value 0 means lowest priority. The value

must not already used by other RAT, i.e. equal priorities between RATs

are not supported. The UE behaviour when no value is entered is

specified in subclause 5.2.4.1 of 3GPP TS 38.304.";

reference "CellReselectionPriority in 3GPP TS 38.331, priority in

3GPP TS 38.304";

mandatory true;

type int32 { range "0..7"; }

}

leaf cellReselectionSubPriority {

description "Indicates a fractional value to be added to the value of

cellReselectionPriority to obtain the absolute priority of the

concerned carrier frequency for E-UTRA and NR.";

reference "3GPP TS 38.331";

type uint8 { range "2 | 4 | 6 | 8"; }

units "0.1";

}

leaf pMax {

description "Used for calculation of the parameter Pcompensation

(defined in 3GPP TS 38.304), at cell reselection to a cell.";

reference "PEMAX in 3GPP TS 38.101";

mandatory true;

type int32 { range "-30..33"; }

units dBm;

}

leaf qOffsetFreq {

description "The frequency specific offset applied when evaluating

candidates for cell reselection.";

type int32;

default 0;

}

leaf qQualMin {

description "Indicates the minimum required quality level in the cell.

Value 0 means that it is not sent and UE applies in such case the

(default) value of negative infinity for Qqualmin. Sent in SIB3 or

SIB5.";

reference "qQualMin in TS 38.304";

mandatory true;

type int32 { range "-34..-3 | 0"; }

units dB;

}

leaf qRxLevMin {

description "Indicates the required minimum received Reference Symbol

Received Power (RSRP) level in the (E-UTRA) frequency for cell

reselection. Broadcast in SIB3 or SIB5, depending on whether the

related frequency is intra- or inter-frequency. Resolution is 2.";

reference "Qrxlevmin in 3GPP TS 38.304";

mandatory true;

type int32 { range "-140..-44"; }

units dBm;

}

leaf threshXHighP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, HighP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXHighQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, HighQ in 3GPP TS 38.304";

mandatory true;

type int32 { range 0..31; }

units dB;

}

leaf threshXLowP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, LowP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXLowQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, LowQ in 3GPP TS 38.304";

mandatory false;

type int32 { range "0..31"; }

units dB;

}

leaf tReselectionEutra {

description "Cell reselection timer for intra frequency E-UTRA cell

reselection. May be used for Mobility Robustness Optimization.";

reference "t-ReselectionEUTRA in 3GPP TS 36.331 and in 3GPP TS 23.207";

mandatory true;

type uint8 { range "0..7"; }

units s;

}

leaf tReselectionEutraSfHigh {

description "The attribute tReselectionEutra (parameter TreselectionEUTRA

in 3GPP TS 38.304) multiplied with this scaling factor if the UE is in

high mobility state.";

reference "Speed dependent ScalingFactor for TreselectionEUTRA for high

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf tReselectionEutraSfMedium {

description "The attribute tReselectionEutra (parameter TreselectionEUTRA

in 3GPP TS 38.304) multiplied with this scaling factor if the UE is in

medium mobility state.";

reference "Speed dependent ScalingFactor for TreselectionEUTRA for medium

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf eUtranFrequencyRef {

description "Reference to a corresponding EUtranFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list EUtranFreqRelation {

description "Represents a frequency relation between an NR cell and an

E-UTRAN cell.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFreqRelationGrp;

}

}

}

}

module _3gpp-nr-nrm-eutranfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfrequency";

prefix "eutraneteutranfreq3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranFrequency Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX

3GPP TS 28.658 V15.X.XX";

}

grouping EUtranFrequencyGrp {

description "Represents the EUtranFrequency IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf earfcnDL {

description "Specifies the channel number for the central DL frequency.";

reference "3GPP TS 36.101";

mandatory true;

type uint32 { range "0..262143"; }

}

leaf-list multiBandInfoListEutra {

description "List of additional frequency bands the frequency belongs to.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping EUtranFrequencyWrapper {

list EUtranFrequency {

description "Represents certain E-UTRAN frequency properties.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFrequencyGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses EUtranFrequencyWrapper ;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses EUtranFrequencyWrapper ;

}

}

module _3gpp-nr-nrm-externalamffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalamffunction";

prefix "extamf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalAMFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalAMFFunctionGrp {

description "Represents the ExternalAMFFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

}

grouping ExternalAMFFunctionWrapper {

list ExternalAMFFunction {

description "Represents the properties, known by the management

function, of a AMFFunction managed by another management

function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalAMFFunctionGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalAMFFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalAMFFunctionWrapper;

}

}module _3gpp-nr-nrm-externalenbfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalenbfunction";

prefix "extenb3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalENBFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX

3GPP TS 28.658 V15.X.XX";

}

grouping ExternalENBFunctionGrp {

description "Represets the ExternalENBFunction IOC.";

reference "3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "3GPP TS 36.413, 3GPP TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalENBFunctionWrapper {

list ExternalENBFunction {

description "Represents an external eNB functionality.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalENBFunctionGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalENBFunctionWrapper;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalENBFunctionWrapper;

}

}module _3gpp-nr-nrm-externaleutrancell {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externaleutrancell";

prefix "exteutrancell3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-nr-nrm-externalenbfunction { prefix extenb3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalEUtranCellFDD and

ExternalEUtranCellTDD Information Object Classes (IOCs) that are part

of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX

3GPP TS 28.658 V15.X.XX";

}

grouping ExternalEUtranGenericCellGrp {

description "Represents the ExternalEUtranGenericCell IOC.";

reference "3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf pci {

description "The Physical Cell Identity (PCI) of the cell (for

NM-Centralized, EM-Centralized and Distributed PCI assignment cases).

In the case of NM-Centralized PCI assignment, see 3GPP TS 36.300.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..503"; }

}

list plmnIdList {

description "List of unique identities for PLMNs. A cell can broadcast

up to 6 PLMN IDs. This is to support the case that one cell can be

used by up to 6 operator's core networks. The PLMN(s) included in this

list will use the same single tracking area code (TAC) and the same

Cell Identity (cellLocalId) for sharing the radio access network

resources. One member of plmnIdList is the primary PLMN ID. A PLMN ID

included in this list cannot be included in the cellAccessInfoList.

The PLMN ID is composed of a Mobile Country Code (MCC) and a Mobile

Network Code (MNC).";

reference "3GPP TS 36.300, 3GPP TS 36.331, 3GPP TS 23.003";

key "mcc mnc";

min-elements 1;

max-elements 6;

uses types3gpp:PLMNId;

}

leaf cellLocalId {

description "Unambiguously identifies a cell within an eNodeB.";

reference "NCI defined in 3GPP TS 38.300";

type int32 {range "0..255"; }

}

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "3GPP TS 36.413, 3GPP TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalEUtranCellFDDGrp {

description "Represents the ExternalEUtranCellFDD IOC.";

reference "3GPP TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcnDL {

description "The channel number for the central DL frequency.";

reference "3GPP TS 36.101";

mandatory true;

type int32 { range "0..17999 | 46590..262143"; }

}

leaf earfcnUL {

description "The channel number for the central UL frequency. Value 0

means that the UL channel number is N/A for the DL-only bands.";

reference "3GPP TS 36.101";

mandatory true;

type int32 { range "0 | 18000..35999 | 46590..262143"; }

}

}

grouping ExternalEUtranCellTDDGrp {

description "Represents the ExternalEUtranCellTDD IOC.";

reference "3GPP TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcn {

description "The frequency number for the central frequency.";

reference "3GPP TS 36.104";

mandatory true;

type int32 { range "36000..262143"; }

}

}

grouping ExternalEUtranCellFDDWrapper {

list ExternalEUtranCellFDD {

description "Represents the common properties of external E-UTRAN FDD

cell provided by eNB or NG-RAN FDD cell provided by ng-eNB.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellFDDGrp;

}

}

}

grouping ExternalEUtranCellTDDWrapper {

list ExternalEUtranCellTDD {

description "Represents the common properties of external E-UTRAN cell

TDD provided by eNB or NG-RAN TDD cell provided by ng-eNB.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellTDDGrp;

}

}

}

augment "/subnet3gpp:SubNetwork/extenb3gpp:ExternalENBFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork;

uses ExternalEUtranCellFDDWrapper;

}

augment "/eutranet3gpp:EUtraNetwork/extenb3gpp:ExternalENBFunction" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalEUtranCellFDDWrapper;

}

augment "/subnet3gpp:SubNetwork/extenb3gpp:ExternalENBFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork;

uses ExternalEUtranCellTDDWrapper;

}

augment "/eutranet3gpp:EUtraNetwork/extenb3gpp:ExternalENBFunction" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalEUtranCellTDDWrapper;

}

}module _3gpp-nr-nrm-externalgnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcucpfunction";

prefix "extgnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBCUCPFunction

Information Object Class (IOC), that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalGNBCUCPFunctionGrp {

description "Represets the ExternalGNBCUCPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in 3GPP TS 38.300, Global gNB ID

in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNId {

description "Specifies the PLMN identifier to be used as part of the

global RAN node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

grouping ExternalGNBCUCPFunctionWrapper {

list ExternalGNBCUCPFunction {

description "Represents the properties, known by the management function,

of a GNBCUCPFunction managed by another management function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUCPFunctionGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBCUCPFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBCUCPFunctionWrapper;

}

}

module _3gpp-nr-nrm-externalgnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcuupfunction";

prefix "extgnbcuup3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBCUUPFunction

Information Object Class (IOC), that is part of the NR Network

Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalGNBCUUPFunctionGrp {

description "Represets the ExternalGNBCUUPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in 3GPP TS 38.300, Global gNB ID

in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

}

grouping ExternalGNBCUUPFunctionWrapper {

list ExternalGNBCUUPFunction {

description "Represents the properties, known by the management function,

of a GNBCUUPFunction managed by another management function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUUPFunctionGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBCUUPFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBCUUPFunctionWrapper;

}

}

module _3gpp-nr-nrm-externalgnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbdufunction";

prefix "extgnbdu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBDUFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalGNBDUFunctionGrp {

description "Represets the ExternalGNBDUFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in 3GPP TS 38.300, Global gNB ID

in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNId {

description "Specifies the PLMN identifier to be used as part of the

global RAN node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

grouping ExternalGNBDUFunctionWrapper {

list ExternalGNBDUFunction {

description "Represents the properties, known by the management function,

of a GNBDUFunction managed by another management function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBDUFunctionGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBDUFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBDUFunctionWrapper;

}

}

module _3gpp-nr-nrm-externalnrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalnrcellcu";

prefix "extnrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-externalgnbcucpfunction { prefix extgnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalNRCellCU Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalNRCellCUGrp {

description "Represents the ExternalNRCellCU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with corresponding

gNB ID it forms the NR Cell Identifier (NCI).";

reference "NCI in 3GPP TS 38.300";

mandatory true;

type int32 {range "0..16383"; }

}

leaf nRPCI {

description "The Physical Cell Identity (PCI) of the NR cell.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..1007"; }

}

list pLMNIdList {

description "Defines which PLMNs that are assumed to be served by the

NR cell in another gNB CU-CP. This list is either updated by the

managed element itself (e.g. due to ANR, signalling over Xn, etc.) or

by consumer over the standard interface.";

key "mcc mnc";

min-elements 1;

max-elements 12;

uses types3gpp:PLMNId;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

grouping ExternalNRCellCUWrapper {

list ExternalNRCellCU {

description "Represents the properties of an NRCellCU controlled by

another Management Service Provider.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNRCellCUGrp;

}

}

}

augment "/subnet3gpp:SubNetwork/extgnbcucp3gpp:ExternalGNBCUCPFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalNRCellCUWrapper;

}

augment "/nrnet3gpp:NRNetwork/extgnbcucp3gpp:ExternalGNBCUCPFunction" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalNRCellCUWrapper;

}

}

module _3gpp-nr-nrm-externalservinggwfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalservinggwfunction";

prefix "extservgw3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalServingGWFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalServingGWFunctionGrp {

description "Represents the ExternalServingGWFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalServingGWFunctionWrapper {

list ExternalServingGWFunction {

description "Represents the properties, known by the management

function, of a ServingGWFunction managed by another management

function.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalServingGWFunctionGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalServingGWFunctionWrapper;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalServingGWFunctionWrapper;

}

}

module _3gpp-nr-nrm-externalupffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalupffunction";

prefix "extupf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalUPFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalUPFFunctionGrp {

description "Represents the ExternalUPFFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalUPFFunctionWrapper {

list ExternalUPFFunction {

description "Represents the properties, known by the management

function, of a UPFFunction managed by another management

function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalUPFFunctionGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalUPFFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalUPFFunctionWrapper;

}

}

module _3gpp-nr-nrm-gnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcucpfunction";

prefix "gnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the GNBCUCPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping GNBCUCPFunctionGrp {

description "Represents the GNBCUCPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

leaf gNBCUName {

description "Identifies the Central Unit of an gNB.";

reference "3GPP TS 38.473";

mandatory true;

type string { length "1..150"; }

}

list pLMNId {

description "The PLMN identifier to be used as part of the global RAN

node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUCPFunction {

description "Represents the logical function CU-CP of gNB and en-gNB.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUCPFunctionGrp;

}

}

}

}

module _3gpp-nr-nrm-gnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcuupfunction";

prefix "gnbcuup3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the GNBCUUPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping GNBCUUPFunctionGrp {

description "Represents the GNBCUUPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNIdList {

description "A list of PLMN identifiers. Defines from which set of PLMNs

an UE must have as its serving PLMN to be allowed to use the

gNB CU-UP.";

key "mcc mnc";

min-elements 1;

max-elements 12;

uses types3gpp:PLMNId;

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUUPFunction {

description "Represents the logical function CU-UP of gNB or en-gNB.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUUPFunctionGrp;

}

}

}

}

module _3gpp-nr-nrm-gnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbdufunction";

prefix "gnbdu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the GNBDUFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping GNBDUFunctionGrp {

description "Represents the GNBDUFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

leaf gNBDUId {

description "Uniquely identifies the DU at least within a gNB.";

reference "3GPP TS 38.473";

mandatory true;

type int64 { range "0..68719476735"; }

}

leaf gNBDUName {

description "Identifies the Distributed Unit of an NR node";

reference "3GPP TS 38.473";

mandatory true;

type string { length "1..150"; }

}

}

augment "/me3gpp:ManagedElement" {

list GNBDUFunction {

description "Represents the logical function DU of gNB or en-gNB.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GNBDUFunctionGrp;

}

}

}

}

module _3gpp-nr-nrm-nrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcellcu";

prefix "nrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRCellCU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

typedef quotaType {

type enumeration {

enum STRICT;

enum FLOAT;

}

}

grouping RRMPolicyRatio1 {

description "Represents properties of RRMPolicyRatio1.";

leaf groupId {

description "Uniquely identifies one sNSSAIList group inside an

NRCellCU instance.";

type uint32;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is supporting. NSSAI is

a set of supported S-NSSAIs. An S-NSSAI is comprised of an

SST (Slice/Service type) and an optional SD (Slice Differentiator)

field.";

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

leaf rRRMPolicyRatio {

description "Percentage of PRBs to be allocated to the corresponding

S-NSSAIs, in average over time. The sum of the values for

rRMPolicyRatio described in rRMPolicyRatio1List shall be less or

equal to 100.";

type uint8 { range "0..100"; }

units %;

}

}

grouping RRMPolicyRatio2 {

description "Represents properties of RRMPolicyRatio2. The RRM policy

setting the ratios for the split of the radio resources between the

supported S-NSSAI lists, in average over time.";

leaf groupId {

description "Uniquely identifies one sNSSAIList group inside an

NRCellCU instance.";

type uint32;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is supporting. NSSAI

is a set of supported S-NSSAI(s). An S-NSSAI is comprised of an

SST (Slice/Service type) and an optional SD (Slice Differentiator)

field.";

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

leaf quotaType {

description "The type of the quota which allows to allocate resources as

strictly usable for defined slice(s) (strict quota) or allows that

resources to be used by other slice(s) when defined slice(s) do not

need them (float quota).";

type quotaType;

}

leaf rRMPolicyMaxRatio {

description "The RRM policy setting the maximum percentage of radio

resources to be allocated to the corresponding S-NSSAI list. This

quota can be strict or float quota. Strict quota means resources are

not allowed for other sNSSAIs even when they are not used by the

defined sNSSAIList. Float quota resources can be used by other sNSSAIs

when the defined sNSSAIList do not need them. Value 0 indicates that

there is no maximum limit.";

type uint8 { range "0..100"; }

units %;

}

leaf rRMPolicyMarginMaxRatio {

description "Maximum quota margin ratio is applicable when maximum quota

policy ratio is of type “float quota”. It defines the resource quota

within maximum quota to reserve buffers for new resource requirements

for the specified S-NSSAI list. With the margin ratio, unused resources

of the maximum resource quota can be allocated to other S-NSSAIs when

the free resources are more than resource amount indicated by the

margin. The margin resource quota can only be used for the specific

S-NSSAI list. Value 0 indicates that no margin is used.";

type uint8 { range "0..100"; }

units %;

}

leaf rRMPolicyMinRatio {

description "The RRM policy setting the minimum percentage of radio

resources to be allocated to the corresponding S-NSSAI list. This

quota can be strict or float quota. Strict quota means resources are

not allowed for other sNSSAIs even when they are not used by the

defined sNSSAIList. Float quota resources can be used by other sNSSAIs

when the defined sNSSAIList do not need them. Value 0 indicates that

there is no minimum limit.";

type uint8 { range "0..100"; }

units %;

}

leaf rRMPolicyMarginMinRatio {

description "Minimum quota margin ratio is applicable when minimum quota

policy ratio is of type “float quota”. It defines the resource quota

within minimum quota to reserve buffers for new resource requirements

for the specified S-NSSAI list. With the margin ratio, unused resources

of the minimum resource quota can be allocated to other S-NSSAIs when

the free resources are more than resource amount indicated by the

margin. The margin resource quota can only be used for the specific

S-NSSAI list. Value 0 indicates that no margin is used. Value 0

indicates that there is no minimum limit.";

type uint8 { range "0..100"; }

units %;

}

}

grouping NRCellCUGrp {

description "Represents the NRCellCU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with corresponding

gNB ID it forms the NR Cell Identifier (NCI).";

mandatory true;

type int32 { range "0..16383"; }

}

list pLMNIdList {

description "Defines which PLMNs that can be served by the NR cell.";

// Note: Whether this attribute can be writable depends on the implementation.

key "mcc mnc";

min-elements 1;

max-elements 12;

uses types3gpp:PLMNId;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the cell is capable of supporting. An

S-NSSAI is comprised of an SST (Slice/Service Type) and an optional

SD (Slice Differentiator) field.";

reference "3GPP TS 23.003";

min-elements 0;

type types3gpp:SNssai;

}

leaf rRMPolicyType {

description "Type of RRM policy. The value 0 denotes use of the

rRMPolicy. The value 1 denotes use of the rRMPolicyRatio1List.

The value 2 denotes use of the rRMPolicyRatio2.";

mandatory true;

type uint16 { range "0..65535"; }

}

leaf rRMPolicy {

description "RRM policy which includes guidance for split of radio

resources between multiple slices the cell supports.";

mandatory true;

type string;

}

list rRMPolicyRatio1List {

description "List of RRMPolicyRatio1. Used for setting the ratio for the

split of the radio resources (i.e. PRBs) between the supported S-NSSAI

lists.";

key groupId;

min-elements 1;

uses RRMPolicyRatio1;

}

list rRMPolicyRatio2 {

description "List of RRMPolicyRatio2. Used for setting the ratio for the

split of the radio resources between the S-NSSAI lists for radio

resources (e.g. RRC connected users, PDCP resource, etc.), in average

time.";

key groupId;

min-elements 1;

uses RRMPolicyRatio2;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

config false;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

list NRCellCU {

description "Represents the information required by CU that is

responsible for the management of inter-cell mobility and neighbour

relations via ANR.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellCUGrp;

}

}

}

}

module _3gpp-nr-nrm-nrcelldu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcelldu";

prefix "nrcelldu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRCellDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NRCellDUGrp {

description "Represents the NRCellDU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with the

corresponding gNB identifier in forms the NR Cell Identity (NCI).";

reference "NCI in 3GPP TS 38.300";

mandatory true;

type int32 { range "0..16383"; }

}

leaf operationalState  {

description "Operational state of the NRCellDU instance. Indicates

whether the resource is installed and partially or fully operable

(ENABLED) or the resource is not installed or not operable

(DISABLED).";

config false;

type types3gpp:OperationalState;

}

leaf administrativeState  {

description "Administrative state of the NRCellDU. Indicates the

permission to use or prohibition against using the cell, imposed

through the OAM services.";

mandatory true;

type types3gpp:AdministrativeState;

}

leaf cellState  {

description "Cell state of the NRCellDU instance. Indicates whether the

cell is not currently in use (IDLE), or currently in use but not

configured to carry traffic (INACTIVE), or currently in use and is

configured to carry traffic (ACTIVE).";

config false;

type types3gpp:CellState;

}

list pLMNIdList {

description "Defines which PLMNs that can be served by the NR cell. The

first entry of the list is the PLMN used to construct the nCGI for the

NR cell.";

key "mcc mnc";

min-elements 1;

max-elements 12;

uses types3gpp:PLMNId;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the NR cell is supporting. NSSAI is a set

of supported S-NSSAI(s), and an S-NSSAI is comprised of a SST

(Slice/Service type) and an optional SD (Slice Differentiator) field.";

reference "3GPP TS 23.003";

min-elements 0;

type types3gpp:SNssai;

}

leaf nRPCI {

description "The Physical Cell Identity (PCI) of the NR cell.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..1007"; }

}

leaf nRTAC {

description "The common 5GS Tracking Area Code for the PLMNs.";

reference "3GPP TS 23.003, 3GPP TS 38.473";

mandatory true;

type types3gpp:Tac;

}

leaf arfcnDL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

downlink.";

reference "3GPP TS 38.104";

mandatory true;

type int32;

}

leaf arfcnUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

uplink.";

reference "3GPP TS 38.104";

mandatory true;

type int32;

}

leaf arfcnSUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

supplementary uplink.";

reference "3GPP TS 38.104";

mandatory true;

type int32;

}

leaf bSChannelBwDL {

description "Base station channel bandwidth for downlink.";

reference "3GPP TS 38.104";

mandatory true;

type int32;

units MHz;

}

leaf bSChannelBwUL {

description "Base station channel bandwidth for uplink.";

reference "3GPP TS 38.104";

mandatory false;

type int32;

units MHz;

}

leaf bSChannelBwSUL {

description "Base station channel bandwidth for supplementary uplink.";

reference "3GPP TS 38.104";

mandatory false;

type int32;

units MHz;

}

leaf ssbFrequency {

description "Indicates cell defining SSB frequency domain position.

Frequency (in terms of NR-ARFCN) of the cell defining SSB transmission.

The frequency identifies the position of resource element RE=#0

(subcarrier #0) of resource block RB#10 of the SS block. The frequency

must be positioned on the NR global frequency raster, as defined in

3GPP TS 38.101, and within bSChannelBwDL.";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf ssbPeriodicity {

description "Indicates cell defined SSB periodicity. The SSB periodicity

is used for the rate matching purpose.";

mandatory true;

type int32 { range "5 | 10 | 20 | 40 | 80 | 160"; }

units "subframes (ms)";

}

leaf ssbSubCarrierSpacing {

description "Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz

(< 6 GHz), 120 kHz or 240 kHz (> 6 GHz) are applicable.";

reference "3GPP TS 38.211";

mandatory true;

type int32 { range "15 | 30 | 120 | 240"; }

units kHz;

}

leaf ssbOffset {

description "Indicates cell defining SSB time domain position. Defined

as the offset of the measurement window, in which to receive SS/PBCH

blocks, where allowed values depend on the ssbPeriodicity

(ssbOffset < ssbPeriodicity).";

mandatory true;

type int32 { range "0..159"; }

units "subframes (ms)";

}

leaf ssbDuration {

description "Duration of the measurement window in which to receive

SS/PBCH blocks.";

reference "3GPP TS 38.213";

mandatory true;

type int32 { range "1..5"; }

units "subframes (ms)";

}

leaf-list nRSectorCarrierRef {

description "Reference to corresponding NRSectorCarrier instance.";

min-elements 1;

type types3gpp:DistinguishedName;

}

leaf-list bWPRef {

description "Reference to corresponding BWP instance.";

min-elements 0;

type types3gpp:DistinguishedName;

}

leaf-list nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

min-elements 0;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list NRCellDU {

description "Represents the information of a cell known by DU.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellDUGrp;

}

}

}

}

module _3gpp-nr-nrm-nrcellrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcellrelation";

prefix "nrcellrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRCellRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NRCellRelationGrp {

description "Represents the NRCellRelation IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf nRTCI {

description "Target NR Cell Identifier. It consists of NR Cell

Identifier (NCI) and Physical Cell Identifier of the target NR cell

(nRPCI).";

type uint64;

}

container cellIndividualOffset {

description "A set of offset values for the neighbour cell. Used when

UE is in connected mode. Defined for rsrpOffsetSSB, rsrqOffsetSSB,

sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and

sinrOffsetCSI-RS.";

reference "cellIndividualOffset in MeasObjectNR in 3GPP TS 38.331";

leaf rsrpOffsetSsb {

description "Offset value of rsrpOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetSsb{

description "Offset value of rsrqOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetSsb {

description "Offset value of sinrOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrpOffsetCsiRs{

description "Offset value of rsrpOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetCsiRs {

description "Offset value of rsrqOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetCsiRs {

description "Offset value of sinrOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

}

leaf nRFreqRelationRef {

description "Reference to a corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

leaf adjacentNRCellRef {

description "Reference to an adjacent NR cell (NRCellCU or

ExternalNRCellCU).";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list NRCellRelation {

description "Represents a neighbour cell relation from a source cell

to a target cell, where the target cell is an NRCellCU or

ExternalNRCellCU instance.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellRelationGrp;

}

}

}

}

module _3gpp-nr-nrm-nrfreqrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrfreqrelation";

prefix "nrfreqrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRFreqRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NRFreqRelationGrp {

description "Represents the NRFreqRelation IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

container offsetMO {

description "A set of offset values applicable to all measured cells

with reference signal(s) indicated in corresponding MeasObjectNR. It

is used to indicate a cell, beam or measurement object specific offset

to be applied when evaluating candidates for cell re-selection or when

evaluating triggering conditions for measurement reporting. It is

defined for rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB,

rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS.";

reference "offsetMO in MeasObjectNR in 3GPP TS 38.331";

leaf rsrpOffsetSsb {

description "Offset value of rsrpOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetSsb {

description "Offset value of rsrqOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetSsb {

description "Offset value of sinrOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrpOffsetCsiRs {

description "Offset value of rsrpOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetCsiRs {

description "Offset value of rsrqOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetCsiRs {

description "Offset value of sinrOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

}

leaf-list blackListEntry {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in NR measurements.";

reference "3GPP TS 38.331";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf-list blackListEntryIdleMode {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in SIB4 and SIB5.";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf cellReselectionPriority {

description "The absolute priority of the carrier frequency used by the

cell reselection procedure. Value 0 means lowest priority. The value

must not already used by other RAT, i.e. equal priorities between RATs

are not supported. The UE behaviour when no value is entered is

specified in subclause 5.2.4.1 of 3GPP TS 38.304.";

reference "CellReselectionPriority in 3GPP TS 38.331, priority in

3GPP TS 38.304";

type uint32;

default 0;

}

leaf cellReselectionSubPriority {

description "Indicates a fractional value to be added to the value of

cellReselectionPriority to obtain the absolute priority of the

concerned carrier frequency for E-UTRA and NR.";

reference "3GPP TS 38.331";

type uint8 { range "2 | 4 | 6 | 8"; }

units "0.1";

}

leaf pMax {

description "Used for calculation of the parameter Pcompensation

(defined in 3GPP TS 38.304), at cell reselection to a cell.";

reference "PEMAX in 3GPP TS 38.101";

mandatory false;

type int32 { range "-30..33"; }

units dBm;

}

leaf qOffsetFreq {

description "The frequency specific offset applied when evaluating

candidates for cell reselection.";

mandatory false;

type types3gpp:QOffsetRange;

default 0;

}

leaf qQualMin {

description "Indicates the minimum required quality level in the cell.

Value 0 means that it is not sent and UE applies in such case the

(default) value of negative infinity for Qqualmin. Sent in SIB3 or

SIB5.";

reference "3GPP TS 38.304";

type int32 { range "-34..-3 | 0"; }

units dB;

default 0;

}

leaf qRxLevMin {

description "Indicates the required minimum received Reference Symbol

Received Power (RSRP) level in the NR frequency for cell reselection.

Broadcast in SIB3 or SIB5, depending on whether the related frequency

is intra- or inter-frequency. Resolution is 2.";

reference "3GPP TS 38.304";

mandatory true;

type int32 { range "-140..-44"; }

units dBm;

}

leaf threshXHighP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, HighP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXHighQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, HighQ in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf threshXLowP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, LowP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXLowQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, LowQ in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf tReselectionNR {

description "Cell reselection timer for NR.";

reference "TreselectionRAT for NR in 3GPP TS 38.331";

mandatory true;

type int32 { range "0..7"; }

units s;

}

leaf tReselectionNRSfHigh {

description "The attribute tReselectionNr (parameter TreselectionNR in

3GPP TS 38.304) is multiplied with this scaling factor if the UE is

in high mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for high

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf tReselectionNRSfMedium {

description "The attribute tReselectionNr (parameter TreselectionNR in

3GPP TS 38.304) multiplied with this scaling factor if the UE is in

medium mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for medium

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf nRFrequencyRef {

description "Reference to a corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list NRFreqRelation {

description "Together with the target NRFrequency, it represents the

frequency properties applicable to the referencing NRFreqRelation.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFreqRelationGrp;

}

}

}

}

module _3gpp-nr-nrm-nrfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-nrfrequency";

prefix "nrfreq3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRFrequency Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NRFrequencyGrp {

description "Represents the NRFrequency IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf absoluteFrequencySSB {

description "The absolute frequency applicable for a downlink NR carrier

frequency associated with the SSB, in terms of NR-ARFCN.";

mandatory true;

type uint32 { range "0.. 3279165"; }

}

leaf sSBSubCarrierSpacing {

description "Sub-carrier spacing of the SSB.";

mandatory true;

type uint8 { range "15 | 30 | 60 | 120"; }

units "kHz";

}

leaf-list multiFrequencyBandListNR {

description "List of additional frequency bands the frequency belongs to.

The list is automatically set by the gNB.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping NRFrequencyWrapper {

list NRFrequency {

description "Represents certain NR frequency properties.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFrequencyGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses NRFrequencyWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses NRFrequencyWrapper;

}

}

module _3gpp-nr-nrm-nrnetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork";

prefix "nrnet3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

feature ExternalsUnderNRNetwork {

description "Classes representing external entities like NRFrequency,

ExternalGNBCUCPFunction, ExternalGNBDUFunction

are contained under a NRNetwork list/class.";

}

grouping NRNetworkGrp {

description "Represents the NRNetwork IOC.";

reference "3GPP TS 28.541";

uses subnet3gpp:SubNetworkGrp;

}

list NRNetwork {

description "A subnetwork containing gNB external NR entities.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRNetworkGrp;

}

}

}

module _3gpp-nr-nrm-nrsectorcarrier {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-nrsectorcarrier";

prefix "nrsectcarr3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRSectorCarrier Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NRSectorCarrierGrp {

description "Represents the NRSectorCarrier IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf txDirection {

description "Indicates if the transmission direction is downlink,

uplink, or both downlink and uplink.";

mandatory true;

type types3gpp:TxDirection;

}

leaf configuredMaxTxPower {

description "Maximum possible transmisssion power for all downlink

channels, used simultaneously in a sector-carrier, added together.";

mandatory true;

type int32;

units mW;

}

leaf arfcnDL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN)

for downlink.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf arfcnUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN)

for uplink.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf bSChannelBwDL {

description "Base station channel bandwitdth for downlink.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 |

90 | 100"; }

units MHz;

}

leaf bSChannelBwUL {

description "Base station channel bandwitdth for uplink.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 |

90 | 100"; }

units MHz;

}

leaf sectorEquipmentFunctionRef {

description "Reference to corresponding SectorEquipmentFunction

instance.";

reference "3GPP TS 23.622";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list NRSectorCarrier {

description "Represents the resources of each transmission point

included in the cell.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRSectorCarrierGrp;

}

}

}

}

# E.6	Graphical representation

See the H.6 for the YANG module tree diagram for NR and NG-RAN NRM.

E.7	Mount information

At the mountpoint "children-of-SubNetwork" in the YANG module _3gpp-common-subnetwork, the following YANG modules may be mounted if the class ManagedElement and the underlying hierarchy is contained under a SubNetwork.

See [45] that describes the mechanism that adds the schema trees defined by a set of YANG modules onto a mount point defined in the schema tree in another YANG module.

_3gpp-common-ep-rp.yang

_3gpp-common-managed-element.yang

_3gpp-common-managed-function.yang

_3gpp-common-measurements.yang

_3gpp-common-subnetwork.yang

_3gpp-common-top.yang

_3gpp-common-yang-extensions.yang

_3gpp-common-yang-types.yang

_3gpp-nr-nrm-bwp.yang

_3gpp-nr-nrm-ep.yang

_3gpp-nr-nrm-eutrancellrelation.yang

_3gpp-nr-nrm-gnbcucpfunction.yang

_3gpp-nr-nrm-gnbcuupfunction.yang

_3gpp-nr-nrm-gnbdufunction.yang

_3gpp-nr-nrm-nrcellcu.yang

_3gpp-nr-nrm-nrcelldu.yang

_3gpp-nr-nrm-nrcellrelation.yang

_3gpp-nr-nrm-nrfreqrelation.yang

_3gpp-nr-nrm-nrfrequency.yang

_3gpp-nr-nrm-nrnetwork.yang

_3gpp-nr-nrm-nrsectorcarrier.yang

ietf-inet-types.yang

ietf-yang-types@2013-07-15.yang

If the above files are mounted the yang files described in clause H.7 shall also be mounted.

Annex F (normative):
XML definitions for 5GC NRM

# F.1	General

This annex contains the XML definitions for the 5GC NRM specified in clause 5, in accordance with 5G NRM Information Model definitions specified in clause 4.

# F.2	Architectural features

The overall architectural feature of 5GC NRM information model is specified in clause 4, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations.

# F.3	Mapping

## F.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## F.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# F.4	Solution Set definitions

## F.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [33].

The present document defines the NRM-specific XML schema ngcNrm.xsd for the 5GC NRM Information Model defined in clause 4.

XML schema ngcNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [33].

## F.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## F.4.3	XML schema "ngcNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 5GC Network Resource Model

XML schema definition

ngcNrm.xsd

-->

<schema

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"     xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<!--NGC NRM IM class associated XML elements -->

<complexType name="aMFIdentifier">

<sequence>

<element name="amfRegionId" type="ngc:AmfRegionId"/>

<element name="amfSetId" type="ngc:AmfSetId"/>

<element name="amfPointer" type="ngc:AmfPointer"/>

</sequence>

</complexType>

<simpleType name="AmfRegionId">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- The AMF Region ID is 8-bitslength, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfSetId">

<restriction base="integer">

<maxInclusive value="1023"/>

<!-- The AMF Region ID is 10-bits length, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfPointer">

<restriction base="integer">

<maxInclusive value="63"/>

<!-- The AMF Pointer is 6-bits length, defined in 23.003 -->

</restriction>

</simpleType>

<complexType name="NrTACList">

<sequence>

<element name="tac" type="nn:NrTac" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="NFProfileList">

<sequence>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="NfProfile">

<sequence>

<element name="nfInstanceId" type="string"/>

<!-- nfInstanceId is uuid of NF instance -->

<element name="nfType" type="ngc:NfType"/>

<element name="nfStatus" type="ngc:NfStatus"/>

<element name="plmn" type="en:PLMNId"/>

<element name="nfType" type="ngc:NfType"/>

<element name="sNssais" type="ngc: SnssaiList"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="fqdn" type="string"/>

<element name="ipv4Addresses" type="string"/>

<element name="ipv6Addresses" type="string"/>

<element name="ipv6Prefixes" type="string"/>

<element name="capacity" type="string"/>

<element name="udrInfo" type="ngc:UdrInfo"/>

<element name="amfInfo" type="ngc:AmfInfo"/>

<element name="smfInfo" type="ngc:SmfInfo"/>

<element name="upfInfo" type="ngc:UpfInfo"/>

<element name="nfServices" type="ngc:NfServices"/>

</sequence>

</complexType>

<complexType name="NfServices">

<sequence>

<element name="serviceInstanceId" type="string"/>

<element name="serviceName" type="string"/>

<element name="version" type="string"/>

<element name="schema" type="string"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="ipEndPoints" type="ngc:IpEndpoints"/>

<element name="apiPrefix" type="string"/>

<element name="defaultNotificationSubscriptions" type="ngc:DefaultNotificationSubscriptions"/>

<element name="allowedPlmns" type="en:PLMNIdList"/>

<element name="allowedNfTypes" type="ngc:NFTypeList"/>

<element name="allowedNssais" type="ngc:Nssai"/>

<element name="capacity" type="string"/>

<element name="supportedFeatures" type="string"/>

</sequence>

</complexType>

<simpleType name="NfType">

<restriction base="string">

<!-- NF name is defined in TS 23.501 -->

<enumeration value="NRF"/>

<enumeration value="UDM"/>

<enumeration value="AMF"/>

<enumeration value="SMF"/>

<enumeration value="AUSF"/>

<enumeration value="NEF"/>

<enumeration value="PCF"/>

<enumeration value="SMSF"/>

<enumeration value="NSSF"/>

<enumeration value="UDR"/>

<enumeration value="LMF"/>

<enumeration value="GMLC"/>

<enumeration value="5GEIR"/>

<enumeration value="SEPP"/>

<enumeration value="UPF"/>

<enumeration value="N3IWF"/>

<enumeration value="AF"/>

<enumeration value="UDSF"/>

<enumeration value="DN"/>

</restriction>

</simpleType>

<complexType name="NFTypeList">

<sequence>

<element name="NFType" type="ngc:NfType"/>

</sequence>

</complexType>

<complexType name="LocalEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

<element name="vlanId" type="integer"/>

</sequence>

</complexType>

<complexType name="RemoteEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

</sequence>

</complexType>

<complexType name="UdrInfo">

<sequence>

<element name="supiRange" type="ngc:SupiRange"/>

</sequence>

</complexType>

<complexType name="SupiRange">

<sequence>

<element name="start" type="string"/>

<element name="end" type="string"/>

<element name="pattern" type="string"/>

</sequence>

</complexType>

<complexType name="AmfInfo">

<sequence>

<element name="amfSetId" type="ngc:AmfSetId"/>

</sequence>

</complexType>

<complexType name="SmfInfo">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="UpfInfo">

<sequence>

<element name="snssaiUpfInfo" type="ngc:SnssaiUpfInfo"/>

</sequence>

</complexType>

<complexType name="SnssaiUpfInfo">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

<element name="dnnUpfInfoList" type="ngc:DnnUpfInfoList"/>

</sequence>

</complexType>

<complexType name="DnnUpfInfoList">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="DefaultNotificationSubscription">

<sequence>

<element name="notificationType" type="ngc:NotificationType"/>

<element name="callbackUri" type="string"/>

<element name="n1MessageClass" type="string"/>

<element name="n2InformationClass" type="string"/>

</sequence>

</complexType>

<simpleType name="NotificationType">

<restriction base="string">

<enumeration value="N1_MESSAGES"/>

<enumeration value="N2_INFORMATION"/>

<enumeration value="LOCATION_NOTIFICATION"/>

</restriction>

</simpleType>

<simpleType name="TransportProtocol">

<restriction base="string">

<enumeration value="TCP"/>

</restriction>

</simpleType>

<simpleType name="NfStatus">

<restriction base="string">

<enumeration value="REGISTERED"/>

<enumeration value="SUSPENDED"/>

</restriction>

</simpleType>

<complexType name="NfRegistrationData">

<sequence>

<element name="heartBeatTimer" type="integer"/>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="NSILdList">

<sequence>

<element name="nSIId" type="string"/>

<!-- NSI Id is defined in TS 29.531 -->

</sequence>

</complexType>

<complexType name="SnssaiList">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

</sequence>

</complexType>

<complexType name="SNssai">

<sequence>

<element name="sst" type="ngc:Sst" minOccurs="0"/>

<element name="sd" type="ngc:Sd"/>

</sequence>

</complexType>

<simpleType name="Sst">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- SST is 1-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<simpleType name="Sd">

<restriction base="integer">

<maxInclusive value="65535"/>

<!-- SST is 2-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<complexType name="SBIServiceList">

<sequence>

<element name="nfServices" type="ngc:NfServices"/>

</sequence>

</complexType>

<simpleType name="WeightFactor">

<restriction base="integer">

</restriction>

</simpleType>

<element name="AMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="aMFIdentifier" type="ngc:aMFIdentifier"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="weightFactor" type="ngc:WeightFactor"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="aMFSet" type="xn:dn" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N14"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N17"/>

<element ref="ngc:EP_N22"/>

<element ref="ngc:EP_N26"/>

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_NLS"/>

<element ref="ngc:EP_NLG"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_S5C"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UPFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N3"/>

<element ref="ngc:EP_N9"/>

<element ref="ngc:EP_S5U"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="ngc:EP_N6"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="N3IWFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N3"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="PCFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList" />

<element name="sBIFqdn" type="string" />

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_N5"/>

<element ref="ngc:EP_Rx"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="AUSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N13"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDMFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N13"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="nSIIdList" type="ngc:NSIIdList" minOccurs="0"/>

<element name="nFProfileList" type="ngc:NFProfileList" minOccurs="0"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NSSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="nSIIdList" type="ngc:NSIIdList"/>

<element name="snssaiList" type="ngc: SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="ngc:EP_N31"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMSFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_N21"/>

<element ref="ngc:EP_MAP_SMSC"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="LMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_NLS"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NGEIRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N17"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SEPPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N32"/>

<element ref="ngc:EP_SBI_IPX"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NWDAFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N2">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N3">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N4">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N5">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N6">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N7">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N8">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N9">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngcnn:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N10">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngcnn:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemote" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N11">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemote" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N12">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N13">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N14">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N15">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N16">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpLocal" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N17">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N20">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpLocal" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N21">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:Local" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N22">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N26">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N27">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N31">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N32">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_SBI_X">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_SBI_IPX">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_Rx">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_MAP_SMSC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLS">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpEndPointRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLG">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nnngc:IpEndPointLocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nnngc:IpRemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex G (normative):
JSON definitions for 5GC NRM

# G.1	General

This annex contains the JSON definitions for the 5GC NRM, in accordance with 5GC NRM information model definitions specified in clause 5.

# G.2	Architectural features

The overall architectural feature of 5GC NRM information model is specified in clause 5, this annex specifies features that are specific to the Schema definitions.

# G.3	Mapping

Refer to TS 32.160 [47].

## SS equivalent definitions are listed in the following table.

the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

# G.4	Solution Set definitions

## G.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## G.4.2	Graphical representation

None.

## G.4.3	JSON schema "ngcNrm.json"

{

"openapi": "3.0.1",

"info": {

"title": "3GPP 5GC NRM",

"version": "15.3.0",

"description": "OAS 3.0.1 specification compatible schema for 3GPP 5GC NRM"

},

"paths": {},

"components": {

"schemas": {

"AmfIdentifier": {

"type": "object",

"description": "AmfIdentifier comprise of amfRegionId, amfSetId and amfPointer",

"properties": {

"amfRegionId": {

"$ref": "#/components/schemas/AmfRegionId"

},

"amfSetId": {

"$ref": "#/components/schemas/AmfSetId"

},

"amfPointer": {

"$ref": "#/components/schemas/AmfPointer"

}

}

},

"AmfRegionId": {

"type": "integer",

"description": "AmfRegionId is defined in TS 23.003",

"maximum": 255

},

"AmfSetId": {

"type": "string",

"description": "AmfSetId is defined in TS 23.003",

"maximum": 1023

},

"AmfPointer": {

"type": "integer",

"description": "AmfPointer is defined in TS 23.003",

"maximum": 63

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv4Addr"

},

"ipv6Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Addr"

},

"ipv6Prefix": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Prefix"

},

"transport": {

"$ref": "genericNrm.json#/components/schemas/TransportProtocol"

},

"port": {

"type": "integer"

}

}

},

"NFProfileList": {

"type": "array",

"description": "List of NF profile",

"items": {

"$ref": "#/components/schemas/NFProfile"

}

},

"NFProfile": {

"type": "object",

"description": "NF profile stored in NRF, defined in TS 29.510",

"properties": {

"nFInstanceId": {

"type": "string",

"description": "uuid of NF instance"

},

"nFType": {

"$ref": "genericNrm.json#/components/schemas/NFType"

},

"nFStatus": {

"$ref": "#/components/schemas/NFStatus"

},

"plmn": {

"$ref": "nrNrm.json#/components/schemas/PlmnId"

},

"sNssais": {

"$ref": "nrNrm.json#/components/schemas/Snssai"

},

"fqdn": {

"$ref": "genericNrm.json#/components/schemas/Fqdn"

},

"interPlmnFqdn": {

"$ref": "genericNrm.json#/components/schemas/Fqdn"

},

"nfServices": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NFService"

}

}

}

},

"NFService": {

"type": "object",

"description": "NF Service is defined in TS 29.510",

"properties": {

"serviceInstanceId": {

"type": "string"

},

"serviceName": {

"type": "string"

},

"version": {

"type": "string"

},

"schema": {

"type": "string"

},

"fqdn": {

"$ref": "genericNrm.json#/components/schemas/Fqdn"

},

"interPlmnFqdn": {

"$ref": "genericNrm.json#/components/schemas/Fqdn"

},

"ipEndPoints": {

"type": "array",

"items": {

"$ref": "#/components/schemas/IpEndPoint"

}

},

"apiPrfix": {

"type": "string"

},

"allowedPlmns": {

"$ref": "nrNrm.json#/components/schemas/PlmnId"

},

"allowedNfTypes": {

"type": "array",

"items": {

"$ref": "genericNrm.json#/components/schemas/NFType"

}

},

"allowedNssais": {

"type": "array",

"items": {

"$ref": "nrNrm.json#/components/schemas/Snssai"

}

}

}

},

"NFStatus": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"REGISTERED",

"SUSPENDED"

]

},

"NFRegistrationData": {

"type": "object",

"properties": {

"heartBeatTimer": {

"type": "integer"

},

"nfProfile": {

"$ref": "#/components/schemas/NFProfile"

}

}

},

"NSIIdList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NSIId"

}

},

"NSIId": {

"type": "string",

"description": "NSI Id is defined in TS 29.531"

},

"NrCellId": {

"type": "string"

},

"SBIFqdn": {

"$ref": "genericNrm.json#/components/schemas/Fqdn"

},

"SBIServiceList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NFService"

}

},

"TACList": {

"type": "array",

"items": {

"$ref": "nrNrm.json#/components/schemas/NrTac"

}

},

"WeightFactor": {

"type": "integer"

},

"UdmInfo": {

"type": "object",

"properties": {

"nFSrvGroupId": {

"type": "string"

}

}

},

"AusfInfo": {

"type": "object",

"properties": {

"nFSrvGroupId": {

"type": "string"

}

}

},

"UpfInfo": {

"type": "object",

"properties": {

"smfServingAreas": {

"type": "string"

}

}

},

"AmfInfo": {

"type": "object",

"properties": {

"priority": {

"type": "integer"

}

}

},

"SupportedDataSetId": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"SUBSCRIPTION",

"POLICY",

"EXPOSURE",

"APPLICATION"

]

},

"Udrinfo": {

"type": "object",

"properties": {

"supportedDataSetIds": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SupportedDataSetId"

}

},

"nFSrvGroupId": {

"type": "string"

}

}

},

"NFInfo": {

"oneOf": [

{

"$ref": "#/components/schemas/UdmInfo"

},

{

"$ref": "#/components/schemas/AusfInfo"

},

{

"$ref": "#/components/schemas/UpfInfo"

},

{

"$ref": "#/components/schemas/AmfInfo"

},

{

"$ref": "#/components/schemas/Udrinfo"

}

]

},

"ManagedNFProfile": {

"type": "object",

"properties": {

"nfInstanceID": {

"type": "string"

},

"nfType": {

"$ref": "genericNrm.json#/components/schemas/NFType"

},

"authzInfo": {

"type": "string"

},

"hostAddr": {

"$ref": "genericNrm.json#/components/schemas/HostAddr"

},

"locality": {

"type": "string"

},

"nFInfo": {

"$ref": "#/components/schemas/NFInfo"

},

"capacity": {

"type": "integer"

}

}

},

"AmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"amfIdentifier": {

"$ref": "#/components/schemas/AmfIdentifier"

},

"sBIFqdn": {

"type": "string"

},

"weightFactor": {

"$ref": "#/components/schemas/WeightFactor"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"amfSet": {

"$ref": "genericNrm.json#/components/schemas/Dn"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N2": {

"$ref": "#/components/schemas/EP_N2"

},

"EP_N8": {

"$ref": "#/components/schemas/EP_N8"

},

"EP_N11": {

"$ref": "#/components/schemas/EP_N11"

},

"EP_N12": {

"$ref": "#/components/schemas/EP_N12"

},

"EP_N14": {

"$ref": "#/components/schemas/EP_N14"

},

"EP_N15": {

"$ref": "#/components/schemas/EP_N15"

},

"EP_N17": {

"$ref": "#/components/schemas/EP_N17"

},

"EP_N20": {

"$ref": "#/components/schemas/EP_N20"

},

"EP_N22": {

"$ref": "#/components/schemas/EP_N22"

},

"EP_N26": {

"$ref": "#/components/schemas/EP_N26"

},

"EP_NLS": {

"$ref": "#/components/schemas/EP_NLS"

},

"EP_NLG": {

"$ref": "#/components/schemas/EP_NLG"

}

}

}

]

},

"AmfSet": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"amfSetId": {

"$ref": "#/components/schemas/AmfSetId"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

}

]

},

"AmfRegion": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"amfRegionId": {

"$ref": "#/components/schemas/AmfRegionId"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

}

]

},

"SmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N4": {

"$ref": "#/components/schemas/EP_N4"

},

"EP_N7": {

"$ref": "#/components/schemas/EP_N7"

},

"EP_N10": {

"$ref": "#/components/schemas/EP_N10"

},

"EP_N11": {

"$ref": "#/components/schemas/EP_N11"

},

"EP_N16": {

"$ref": "#/components/schemas/EP_N16"

},

"EP_S5C": {

"$ref": "#/components/schemas/EP_S5C"

}

}

}

]

},

"UpfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N3": {

"$ref": "#/components/schemas/EP_N3"

},

"EP_N4": {

"$ref": "#/components/schemas/EP_N4"

},

"EP_N6": {

"$ref": "#/components/schemas/EP_N6"

},

"EP_N9": {

"$ref": "#/components/schemas/EP_N9"

},

"EP_S5U": {

"$ref": "#/components/schemas/EP_S5U"

}

}

}

]

},

"N3iwfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N3": {

"$ref": "#/components/schemas/EP_N3"

},

"EP_N4": {

"$ref": "#/components/schemas/EP_N4"

}

}

}

]

},

"PcfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N5": {

"$ref": "#/components/schemas/EP_N5"

},

"EP_N7": {

"$ref": "#/components/schemas/EP_N7"

},

"EP_N15": {

"$ref": "#/components/schemas/EP_N15"

},

"EP_N16": {

"$ref": "#/components/schemas/EP_N16"

},

"EP_Rx": {

"$ref": "#/components/schemas/EP_Rx"

}

}

}

]

},

"AusfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N12": {

"$ref": "#/components/schemas/EP_N12"

},

"EP_N13": {

"$ref": "#/components/schemas/EP_N13"

}

}

}

]

},

"UdmFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N8": {

"$ref": "#/components/schemas/EP_N8"

},

"EP_N10": {

"$ref": "#/components/schemas/EP_N10"

},

"EP_N13": {

"$ref": "#/components/schemas/EP_N13"

}

}

}

]

},

"UdrFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

}

]

},

"UdsfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

}

]

},

"NrfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/components/schemas/NSIIdList"

},

"nFProfileList": {

"$ref": "#/components/schemas/NFProfileList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N27": {

"$ref": "#/components/schemas/EP_N27"

}

}

}

]

},

"NssfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/components/schemas/NSIIdList"

},

"nFProfileList": {

"$ref": "#/components/schemas/NFProfileList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N22": {

"$ref": "#/components/schemas/EP_N22"

},

"EP_N31": {

"$ref": "#/components/schemas/EP_N31"

}

}

}

]

},

"SmsfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N20": {

"$ref": "#/components/schemas/EP_N20"

},

"EP_N21": {

"$ref": "#/components/schemas/EP_N21"

},

"EP_MAP_SMSC": {

"$ref": "#/components/schemas/EP_MAP_SMSC"

}

}

}

]

},

"LmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_NLS": {

"$ref": "#/components/schemas/EP_NLS"

}

}

}

]

},

"NgeirFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N17": {

"$ref": "#/components/schemas/EP_N17"

}

}

}

]

},

"SeppFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N32": {

"$ref": "#/components/schemas/EP_N32"

}

}

}

]

},

"NwdafFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"managedNFProfile": {

"$ref": "#/components/schemas/ManagedNFProfile"

}

}

}

]

}

}

}

]

},

"ExternalAmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"amfIdentifier": {

"$ref": "#/components/schemas/AmfIdentifier"

}

}

}

]

}

}

}

]

},

"ExternalNrfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

}

]

},

"ExternalNssfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

}

]

},

"ManagedElement-Single": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedElement-Attributes"

},

{

"type": "object",

"properties": {}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedElement-ContainingObjects"

},

{

"type": "object",

"properties": {

"AmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AmfFunction"

}

},

"SmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SmfFunction"

}

},

"UpfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UpfFunction"

}

},

"N3iwfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/N3iwfFunction"

}

},

"PcfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/PcfFunction"

}

},

"AusfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AusfFunction"

}

},

"UdmFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UdmFunction"

}

},

"UdrFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UdrFunction"

}

},

"UdsfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UdsfFunction"

}

},

"NrfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrfFunction"

}

},

"NssfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NssfFunction"

}

},

"SmsfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SmsfFunction"

}

},

"LmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/LmfFunction"

}

},

"NgeirFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NgeirFunction"

}

},

"SeppFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SeppFunction"

}

},

"NwdafFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NwdafFunction"

}

}

}

}

]

},

"ManagedElement-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ManagedElement-Single"

}

},

"SubNetwork-Single": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

},

{

"type": "object",

"properties": {}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-ContainingObjects"

},

{

"type": "object",

"properties": {

"SubNetwork": {

"$ref": "#/components/schemas/SubNetwork-Multiple"

},

"ManagedElement": {

"$ref": "#/components/schemas/ManagedElement-Multiple"

},

"ExternalAmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalAmfFunction"

}

},

"ExternalNrfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalNrfFunction"

}

},

"ExternalNssfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalNssfFunction"

}

},

"AmfSet": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AmfSet"

}

},

"AmfRegion": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AmfRegion"

}

}

}

}

]

},

"SubNetwork-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SubNetwork-Single"

}

},

"EP_N2": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N3": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N4": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N5": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N6": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N7": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N8": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N9": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N10": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N11": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N12": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N13": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N14": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N15": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N16": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N17": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N20": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N21": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N22": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N26": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N27": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N31": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N32": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_SBI_X": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_SBI_IPX": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_S5C": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_S5U": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_Rx": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_MAP_SMSC": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_NLS": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_NLG": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

}

}

}

}

Annex H (normative):
YANG definitions for 5GC

# H.1	General

This annex contains the YANG definitions for the 5GC NRM, in accordance with 5GC information model definitions specified in clause 4.

# H.2	Void

# H.3	Void

# listed in the following table.

## S equivalent definitions are listed in the following table.

H.4	Void

# H.5	Modules

This is the list of YANG modules for 5GC NRM.

_3gpp-5gc-nrm-affunction.yang

_3gpp-5gc-nrm-amffunction.yang

_3gpp-5gc-nrm-amfregion.yang

_3gpp-5gc-nrm-amfset.yang

_3gpp-5gc-nrm-ausffunction.yang

_3gpp-5gc-nrm-dnfunction.yang

_3gpp-5gc-nrm-ep.yang

_3gpp-5gc-nrm-externalamffunction.yang

_3gpp-5gc-nrm-externalnrffunction.yang

_3gpp-5gc-nrm-externalnssffunction.yang

_3gpp-5gc-nrm-lmffunction.yang

_3gpp-5gc-nrm-n3iwffunction.yang

_3gpp-5gc-nrm-nfprofile.yang

_3gpp-5gc-nrm-nfservice.yang

_3gpp-5gc-nrm-ngeirfunction.yang

_3gpp-5gc-nrm-nrffunction.yang

_3gpp-5gc-nrm-nssffunction.yang

_3gpp-5gc-nrm-nwdaffunction.yang

_3gpp-5gc-nrm-pcffunction.yang

_3gpp-5gc-nrm-seppfunction.yang

_3gpp-5gc-nrm-smffunction.yang

_3gpp-5gc-nrm-smsffunction.yang

_3gpp-5gc-nrm-udmfunction.yang

_3gpp-5gc-nrm-udrfunction.yang

_3gpp-5gc-nrm-udsffunction.yang

_3gpp-5gc-nrm-upffunction.yang

This is the YANG modules for 5GC NRM.

module _3gpp-5gc-nrm-affunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-affunction;

prefix af3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC is defined only to describe the IOCs representing

its interaction interface with 5GC (i.e. EP_Rx and EP_N5).

It has no attributes defined.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping AFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

list AFFunction {

description "5G Core AF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-amffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amffunction;

prefix amf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

description "AMFFunction derived from basic ManagedFunction.";

revision 2019-05-31 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

revision 2018-08-07 {

description "Initial revision";

}

grouping AMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

min-elements 1;

config false;

type string;

}

leaf weightFactor {

description "Set according to the capacity of local node (AMF) relative to

other nodes in the same type, to achieve load balancing among a set of

same type of network functions.";

type uint16;

}

leaf-list sNSSAIList {

min-elements 0;  // conditionally mandatory if network slicing feature is supported

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list AMFFunction {

description "5G Core AMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-amfregion {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfregion;

prefix amfr3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents the AMF Region which consists one or multiple AMF Sets.";

revision 2019-06-11 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping AMFRegionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

leaf aMFRegionId {

description "Represents the AMF Region ID, which identifies the region.";

mandatory true;

type types3gpp:AmfRegionId;

}

leaf-list aMFSet {

description "The AMFSet that the AFMRegion is associated with.";

min-elements 1;

type instance-identifier;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFRegion {

description "5G Core AMFRegion IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFRegionGrp;

}

}

}

}module _3gpp-5gc-nrm-amfset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfset;

prefix amfset3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents the AMF Set which consists of some AMFs that serve a given area and Network Slice.";

revision 2019-06-11 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping AMFSetGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

leaf aMFRegion {

description "The AMFRegion that the AFMSet is associated with.";

type instance-identifier;

}

leaf-list aMFSetMemberList {

description "List of DNs of AMFFunction instances of the AMFSet.";

min-elements 1;

max-elements 1;

type types3gpp:DistinguishedName;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFSet {

description "5G Core AMFSet IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFSetGrp;

}

}

}

}module _3gpp-5gc-nrm-ausffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ausffunction;

prefix ausf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the AUSF function in 5GC. For more information about the AUSF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-22 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping AUSFFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list AUSFFunction {

description "5G Core AUSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AUSFFuntionGrp;

}

}

}

}module _3gpp-5gc-nrm-dnfunction {

yang-version 1.1;

namespace urn:3gpp:sa5_3gpp-5gc-nrm-dnfunction;

prefix dn3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC is defined only to describe the IOCs representing

Data Network (DN) interaction interface with 5GC (i.e. EP_N6).

It has no attributes defined.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping DNFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

list DNFunction {

description "5G Core DN Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses DNFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-ep {

yang-version 1.1;

namespace "urn:3gpp:tsg:sa5:nrm:_3gpp-5gc-nrm-ep";

prefix "cep3gpp";

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-affunction { prefix af3gpp; }

import _3gpp-5gc-nrm-amffunction { prefix amf3gpp; }

import _3gpp-5gc-nrm-ausffunction { prefix ausf3gpp; }

import _3gpp-5gc-nrm-dnfunction { prefix dn3gpp; }

import _3gpp-5gc-nrm-lmffunction { prefix lmf3gpp; }

import _3gpp-5gc-nrm-n3iwffunction { prefix n3iwf3gpp; }

import _3gpp-5gc-nrm-ngeirfunction { prefix ngeir3gpp; }

import _3gpp-5gc-nrm-nrffunction { prefix nrf3gpp; }

import _3gpp-5gc-nrm-nssffunction { prefix nssf3gpp; }

import _3gpp-5gc-nrm-nwdaffunction { prefix nwdaf3gpp; }

import _3gpp-5gc-nrm-pcffunction { prefix pcf3gpp; }

import _3gpp-5gc-nrm-seppfunction { prefix sepp3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-5gc-nrm-smsffunction { prefix smsf3gpp; }

import _3gpp-5gc-nrm-udmfunction { prefix udm3gpp; }

import _3gpp-5gc-nrm-udrfunction { prefix udr3gpp; }

import _3gpp-5gc-nrm-udsffunction { prefix udsf3gpp; }

import _3gpp-5gc-nrm-upffunction { prefix upf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the 5GC related endpoint

Information Object Classes (IOCs) that are part of the 5G Core

Network Resource Model.";

reference "3GPP TS 28.541";

revision 2019-05-31 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

revision 2018-07-31 {

description "Initial revision";

}

grouping EP_N2Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N3Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N4Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N5Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N6Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N7Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N8Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N9Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N10Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N11Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N12Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N13Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N14Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N15Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N16Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N17Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N20Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N21Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N22Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N26Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N27Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N31Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N32Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_S5CGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_S5UGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_RxGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_MAP_SMSCGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_NLSGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_NLGGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_SBI_XGrp {

uses eprp3gpp:EP_Common;

leaf-list sBIService {

min-elements 1;

config false;

type string;

}

}

grouping EP_SBI_IPXGrp {

uses eprp3gpp:EP_Common;

leaf-list sBIService {

min-elements 1;

config false;

type string;

}

}

augment "/me3gpp:ManagedElement/af3gpp:AFFunction" {

list EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

list EP_Rx {

description "Represents the EP_Rx IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_RxGrp;

}

}

}

augment "/me3gpp:ManagedElement/amf3gpp:AMFFunction" {

list EP_N2 {

description "Represents the EP_N2 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N2Grp;

}

}

list EP_N8 {

description "Represents the EP_N8 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N8Grp;

}

}

list EP_N11 {

description "Represents the EP_N11 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N11Grp;

}

}

list EP_N12 {

description "Represents the EP_N12 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N12Grp;

}

}

list EP_N14 {

description "Represents the EP_N14 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N14Grp;

}

}

list EP_N15 {

description "Represents the EP_N15 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N15Grp;

}

}

list EP_N17 {

description "Represents the EP_N17 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N17Grp;

}

}

list EP_N20 {

description "Represents the EP_N20 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N20Grp;

}

}

list EP_N22 {

description "Represents the EP_N22 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N22Grp;

}

}

list EP_N26 {

description "Represents the EP_N26 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N26Grp;

}

}

list EP_NLS {

description "Represents the EP_NLS IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLSGrp;

}

}

list EP_NLG {

description "Represents the EP_NLG IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLGGrp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/ausf3gpp:AUSFFunction" {

list EP_N12 {

description "Represents the EP_N12 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N12Grp;

}

}

list EP_N13 {

description "Represents the EP_N13 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N13Grp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/dn3gpp:DNFunction" {

list EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

}

augment "/me3gpp:ManagedElement/lmf3gpp:LMFFunction" {

list EP_NLS {

description "Represents the EP_NLS IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLSGrp;

}

}

}

augment "/me3gpp:ManagedElement/n3iwf3gpp:N3IWFFunction" {

list EP_N2 {

description "Represents the EP_N2 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N2Grp;

}

}

list EP_N3 {

description "Represents the EP_N3 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N3Grp;

}

}

}

augment "/me3gpp:ManagedElement/ngeir3gpp:NGEIRFunction" {

list EP_N17 {

description "Represents the EP_N17 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N17Grp;

}

}

}

augment "/me3gpp:ManagedElement/nrf3gpp:NRFFunction" {

list EP_N27 {

description "Represents the EP_N27 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N26Grp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/nssf3gpp:NSSFFunction" {

list EP_N22 {

description "Represents the EP_N22 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N22Grp;

}

}

list EP_N31 {

description "Represents the EP_N31 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N31Grp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/nwdaf3gpp:NWDAFFunction" {

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/pcf3gpp:PCFFunction" {

list EP_N5 {

description "Represents the EP_N5 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N5Grp;

}

}

list EP_N7 {

description "Represents the EP_N7 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N7Grp;

}

}

list EP_N15 {

description "Represents the EP_N15 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N15Grp;

}

}

list EP_N16 {

description "Represents the EP_N16 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N16Grp;

}

}

list EP_Rx {

description "Represents the EP_Rx IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_RxGrp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/sepp3gpp:SEPPFunction" {

list EP_N32 {

description "Represents the EP_N32 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N32Grp;

}

}

list EP_SBI_IPX {

description "Used for the SEPP services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_IPXGrp;

}

}

}

augment "/me3gpp:ManagedElement/smsf3gpp:SMSFFunction" {

list EP_N20 {

description "Represents the EP_20 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N20Grp;

}

}

list EP_N21 {

description "Represents the EP_N21 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N21Grp;

}

}

list EP_MAP_SMSC {

description "Represents the EP_MAP_SMSC IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_MAP_SMSCGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list EP_N4 {

description "Represents the EP_N4 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N4Grp;

}

}

list EP_N7 {

description "Represents the EP_N7 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N7Grp;

}

}

list EP_N10 {

description "Represents the EP_N10 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N10Grp;

}

}

list EP_N11 {

description "Represents the EP_N11 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N11Grp;

}

}

list EP_N16 {

description "Represents the EP_N16 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N16Grp;

}

}

list EP_S5C {

description "Represents the EP_S5C IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S5CGrp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/udm3gpp:UDMFunction" {

list EP_N8 {

description "Represents the EP_N8 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N8Grp;

}

}

list EP_N10 {

description "Represents the EP_N10 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N10Grp;

}

}

list EP_N13 {

description "Represents the EP_N13 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N13Grp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/udr3gpp:UDRFunction" {

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/udsf3gpp:UDSFFunction" {

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

}

augment "/me3gpp:ManagedElement/upf3gpp:UPFFunction" {

list EP_N4 {

description "Represents the EP_N4 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N4Grp;

}

}

list EP_N3 {

description "Represents the EP_N3 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N3Grp;

}

}

list EP_N9 {

description "Represents the EP_N9 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N9Grp;

}

}

list EP_S5U {

description "Represents the EP_S5U IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S5UGrp;

}

}

list EP_SBI_X {

description "Used for the services consumed by the contol plane function.

Far entity and remoteAddress are required for this endpoint.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_SBI_XGrp;

}

}

list EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

}

}module _3gpp-5gc-nrm-externalnrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-externalnrffunction;

prefix extnrf3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents external NRF function controlled by another management domain.";

revision 2019-06-11 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalNRFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/subnet3gpp:SubNetwork" {

list ExternalNRFFunction {

description "5G Core External NRF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNRFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-externalnssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-externalnssffunction;

prefix extnssf3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents external NSSF function controlled by another management domain.";

revision 2019-06-11 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping ExternalNSSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/subnet3gpp:SubNetwork" {

list ExternalNSSFFunction {

description "5G Core External NSSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNSSFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-lmffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-lmffunction;

prefix lmf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the LMF function defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping LMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/me3gpp:ManagedElement" {

list LMFFunction {

description "5G Core LMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses LMFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-n3iwffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-n3iwffunction;

prefix n3iwf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the N3IWF function which is used to enable non-3GPP

access networks connected to the 5GC. For more information about the N3IWF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-22 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping N3IWFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/me3gpp:ManagedElement" {

list N3IWFFunction {

description "5G Core N3IWF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses N3IWFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-nfprofile {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfprofile;

prefix nfp3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-5gc-nrm-nfservice { prefix nfs3gpp; }

organization "3gpp SA5";

description "NF profile class.";

reference "3GPP TS 29.510";

revision 2019-06-17 {

description "initial revision";

reference "Based on

3GPP TS 29.510 V15.X.XX";

}

grouping NFProfileGrp {

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

mandatory true;

type string;

}

leaf nfType {

description "Type of Network Function.";

mandatory true;

type types3gpp:NfType;

}

leaf nfStatus {

description "Status of the NF Instance.";

mandatory true;

type NFStatus;

}

leaf heartBeatTimer {

description "Time in seconds expected between 2 consecutive heart-beat messages from

an NF Instance to the NRF. It may be included in the registration request.

When present in the request it shall contain the heartbeat time proposed by the NF service consumer.";

//conditional support

type uint16;

}

list plmnList {

description "PLMN(s) of the Network Function.

This IE shall be present if this information is available for the NF.

If not provided, PLMN ID(s) of the PLMN of the NRF are assumed for the NF.";

//conditional support

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssais { //is the key unique

description "S-NSSAIs of the Network Function. If not provided, the NF can serve any S-NSSAI.

When present this IE represents the list of S-NSSAIs supported in all the PLMNs listed in the plmnList IE.";

min-elements 1;

//optional support

key "sst sd";

uses Snssai;

}

list perPlmnSnssaiList {

description "This IE may be included when the list of S-NSSAIs supported by the NF for each PLMN it is supporting is different.

When present, this IE shall include the S-NSSAIs supported by the Network Function

for each PLMN supported by the Network Function. When present, this IE shall override sNssais IE.";

min-elements 1;

//optional support

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses PlmnSnssai;

}

leaf-list nsiList {

description "NSI identities of the Network Function.

If not provided, the NF can serve any NSI.";

//optional support

min-elements 1;

type string;

}

leaf fqdn {

description "FQDN of the Network Function. For AMF, the FQDN registered with the NRF

shall be that of the AMF Name.";

//conditional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF needs to be discoverable by other NFs in a different PLMN,

then an FQDN that is used for inter-PLMN routing is specified.";

//conditional support

type inet:domain-name;

}

leaf-list ipv4Addresses {

description "IPv4 address(es) of the Network Function.";

min-elements 1;

//conditional support

type inet:ipv4-address;

}

leaf-list ipv6Addresses {

description "IPv6 address(es) of the Network Function.";

min-elements 1;

//conditional support

type inet:ipv6-address;

}

list allowedPlmns {

description "PLMNs allowed to access the NF instance.

If not provided, any PLMN is allowed to access the NF.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the NF instance.

If not provided, any NF type is allowed to access the NF.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to access the NF instance.

If not provided, any NF domain is allowed to access the NF.";

min-elements 1;

//optional support

type string;

}

list allowedNssais { //is the key unique

description "S-NSSAI of the allowed slices to access the NF instance.

If not provided, any slice is allowed to access the NF.";

min-elements 1;

//optional support

key "sst sd";

uses Snssai;

}

leaf priority {

description "Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection;

lower values indicate a higher priority. If priority is also present in the nfServiceList parameters,

those will have precedence over this value. The NRF may overwrite the received priority value when exposing

an NFProfile with the Nnrf_NFDiscovery service.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535, expressed as a weight

relative to other NF instances of the same type; if capacity is also present

in the nfServiceList parameters, those will have precedence over this value.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates the current load percentage of the NF.";

//optional support

type types3gpp:Load;

}

leaf locality {

description "Operator defined information about the location of the NF instance (e.g. geographic location, data center).";

//optional support

type string;

}

grouping udrInfo {

//optional support

leaf groupId {

description "Identity of the UDR group that is served by the UDR instance.

If not provided, the UDR instance does not pertain to any UDR group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

leaf-list supportedDataSets {

description "List of supported data sets in the UDR instance.

If not provided, the UDR supports all data sets.";

min-elements 1;

//optional support

type DataSetId;

}

}

grouping udmInfo {

//optional support

leaf groupId {

description "Identity of the UDM group that is served by the UDM instance.

If not provided, the UDM instance does not pertain to any UDM group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows to route network signalling with SUCI

to the UDM instance. If not provided, the UDM can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

//optional support

min-elements 1;

type string;

}

}

grouping ausfInfo {

//optional support

leaf groupId {

description "Identity of the AUSF group. If not provided, the AUSF instance does not pertain to any AUSF group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by the AUSF instance. If not provided, the AUSF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows to route network signalling with SUCI

to the AUSF instance. If not provided, the AUSF can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

//optional support

min-elements 1;

type string;

}

}

grouping amfInfo {

//optional support

leaf amfRegionId {

description "AMF region identifier";

type string;

}

leaf amfSetId {

description "AMF set identifier";

type string;

}

list guamiList {

description "List of supported GUAMIs.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

min-elements 1;

uses Guami;

}

list taiList {

description "The list of TAIs the AMF can serve. It may contain the non-3GPP access TAI.

The absence of this attribute and the taiRangeList attribute indicate that

the AMF can be selected for any TAI in the serving network.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Tai;

}

list taiRangeList {

description "The range of TAIs the AMF can serve. The absence of this attribute and the taiList

attribute indicate that the AMF can be selected for any TAI in the serving network.";

//optional support

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses TaiRange;

}

list backupInfoAmfFailure {

description "List of GUAMIs for which the AMF acts as a backup for AMF failure.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Guami;

}

list backupInfoAmfRemoval {

description "List of GUAMIs for which the AMF acts as a backup for planned AMF removal.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Guami;

}

list n2InterfaceAmfInfo {

description "N2 interface information of the AMF. This information needs not be sent in NF Discovery responses.

It may be used by the NRF to update the DNS for AMF discovery by the 5G Access Network.";

//optional support

max-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses N2InterfaceAmfInfo;

}

}

grouping smfInfo {

//optional support

list sNssaiSmfInfoList {

description "List of parameters supported by the SMF per S-NSSAI.";

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses sNssaiSmfInfoItem;

}

list taiList {

description "The list of TAIs the SMF can serve. It may contain the non-3GPP access TAI.

The absence of this attribute and the taiRangeList attribute indicate that

the SMF can be selected for any TAI in the serving network.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Tai;

}

list taiRangeList {

description "The range of TAIs the SMF can serve. The absence of this attribute and the taiList

attribute indicate that the SMF can be selected for any TAI in the serving network.";

//optional support

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses TaiRange;

}

leaf pgwFqdn {

description "The FQDN of the PGW if the SMF is a combined SMF/PGW-C.";

//optional support

type inet:domain-name;

}

leaf-list accessType {

description "If included, this IE shall contain the access type (3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF.

If not included, it shall be assumed the both access types are supported.";

//conditional support

min-elements 1;

max-elements 2;

type AccessType;

}

}

grouping upfInfo {

//optional support

list sNssaiUpfInfoList {

description "List of parameters supported by the UPF per S-NSSAI.";

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses SnssaiUpfInfoItem;

}

leaf-list smfServingArea {

description "The SMF service area(s) the UPF can serve.

If not provided, the UPF can serve any SMF service area.";

//optional support

min-elements 1;

type string;

}

list interfaceUpfInfo {

description "List of User Plane interfaces configured on the UPF. When this IE is provided in the NF Discovery response,

the NF Service Consumer (e.g. SMF) may use this information for UPF selection.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses InterfaceUpfInfoItem;

}

leaf iwkEpsInd {

description "Indicates whether interworking with EPS is supported by the UPF.

true: Supported

false (default): Not Supported";

//optional support

type boolean;

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by the UPF. The absence of this attribute indicates that the UPF can be selected

for any PDU session type.";

//optional support

min-elements 1;

type PduSessionType;

}

}

grouping pcfInfo {

//optional support

leaf-list dnnList {

description "DNNs supported by the PCF.

If not provided, the PCF can serve any DNN.";

//optional support

min-elements 1;

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by the PCF instance. If not provided, the PCF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

leaf rxDiamHost {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter host of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

//conditional support

type string;

}

leaf rxDiamRealm {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter realm of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

//conditional support

type string;

}

}

grouping bsfInfo {

//optional support

list ipv4AddressRanges {

description "List of ranges of IPv4 addresses handled by BSF.

If not provided, the BSF can serve any IPv4 address.";

//optional support

key "start end";

uses types3gpp:Ipv4AddressRange;

}

leaf-list dnnList {

description "List of DNNs handled by the BSF

If not provided, the BSF can serve any DNN.";

//optional support

min-elements 1;

type string;

}

leaf-list ipDomainList {

description "List of IPv4 address domains, as described in subclause 6.2 of 3GPP TS 29.513, handled by the BSF.

If not provided, the BSF can serve any IP domain.";

//optional support

min-elements 1;

type string;

}

list ipv6PrefixRanges {

description "List of ranges of IPv6 prefixes handled by the BSF.

If not provided, the BSF can serve any IPv6 prefix.";

//optional support

key "start end";

uses types3gpp:Ipv6PrefixRange;

}

}

grouping chfInfo {

//optional support

list supiRangeList {

description "List of ranges of SUPIs that can be served by the CHF instance. If not provided, the CHF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRangeList {

description "List of ranges of GPSI that can be served by the CHF instance. If not provided, the CHF can serve any GPSI.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list plmnRangeList {

description "List of ranges of PLMNs (including the PLMN IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping nrfInfoGrp {

//optional support

list servedUdrInfo {

description "This attribute contains all the udrInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udrInfo;

}

list servedUdmInfo {

description "This attribute contains all the udmInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udmInfo;

}

list servedAusfInfo {

description "This attribute contains all the ausfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses ausfInfo;

}

list servedAmfInfo {

description "This attribute contains all the amfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses amfInfo;

}

list servedSmfInfo {

description "This attribute contains all the smfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses smfInfo;

}

list servedUpfInfo {

description "This attribute contains all the upfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses upfInfo;

}

list servedPcfInfo {

description "This attribute contains all the pcfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses pcfInfo;

}

list servedBsfInfo {

description "This attribute contains all the bsfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses bsfInfo;

}

list servedChfInfo {

description "This attribute contains all the bsfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses chfInfo;

}

}

list nrfInfo {

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

max-elements 1;

uses nrfInfoGrp;

}

leaf customInfo {

description "Specific data for custom Network Functions.";

type string;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

leaf nfServicePersistence {

description "If present, and set to true, it indicates that the different service instances of a same NF Service in this NF instance,

supporting a same API version, are capable to persist their resource state in shared storage and therefore these resources

are available after a new NF service instance supporting the same API version is selected by a NF Service Consumer (see 3GPP TS 23.527).

Otherwise, it indicates that the NF Service Instances of a same NF Service are not capable to share resource state inside the NF Instance.";

//optional support

type boolean;

}

list nfServices {

description "List of NF Service Instances. It shall include the services produced by the NF that can be discovered by other NFs.";

key serviceInstanceID;

//optional support

min-elements 1;

uses nfs3gpp:NFServiceGrp;

}

leaf nfProfileChangesSupportInd {

description "NF Profile Changes Support Indicator. This IE may be present in the NFRegister or NFUpdate (NF Profile Complete Replacement) request and shall be absent in the response.

true: the NF Service Consumer supports receiving NF Profile Changes in the response.

false (default): the NF Service Consumer does not support receiving NF Profile Changes in the response.";

//optional support

type boolean;

}

leaf nfProfileChangesInd {

description "NF Profile Changes Indicator. This IE shall be absent in the request to the NRF and may be included by the NRF in NFRegister or NFUpdate (NF Profile Complete Replacement) response.

true: the NF Profile contains NF Profile changes.

false (default): complete NF Profile.";

//optional support

type boolean;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

}

typedef NFStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

}

}

typedef DataSetId {

type enumeration {

enum SUBSCRIPTION;

enum POLICY;

enum EXPOSURE;

enum APPLICATION;

}

}

grouping SupiRange {

leaf start {

description "First value identifying the start of a SUPI range. To be used when the range of SUPI's can be represented as a numeric range (e.g., IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of a SUPI range. To be used when the range of SUPI's can be represented as a numeric range (e.g. IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of SUPI's belonging to this range.

A SUPI value is considered part of the range if and only if the SUPI string fully matches the regular expression.";

type string;

}

}

grouping IdentityRange {

leaf start {

description "First value identifying the start of an identity range. To be used when the range of identities can be represented as a numeric range (e.g., MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of an identity range. To be used when the range of identities can be represented as a numeric range (e.g. MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of identities belonging to this range.

An identity value is considered part of the range if and only if the identity string fully matches the regular expression.";

type string;

}

}

grouping TacRange {

leaf start {

description "First value identifying the start of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g., TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6}$)';

}

}

leaf end {

description "Last value identifying the end of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g. TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$';

}

}

leaf pattern {

description "Pattern representing the set of TAC's belonging to this range.";

type string;

}

}

grouping SnssaiUpfInfoItem {

list sNssai { //is the key unique

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

list dnnUpfInfoList {

description "List of parameters supported by the UPF per DNN.";

min-elements 1;

key dnn;

uses DnnUpfInfoItem;

}

}

grouping DnnUpfInfoItem {

leaf dnn {

description "String representing a Data Network.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported by the UPF for this DNN.

The absence of this attribute indicates that the UPF can be selected for this DNN for any DNAI.";

min-elements 1;

type string; //dnai is the type but its only a string with desc: DNAI (Data network access identifier), is this needed as its own typedef or string is ok

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by the UPF for a specific DNN.";

min-elements 1;

type PduSessionType;

}

}

grouping Snssai {

leaf sst {

description "Unsigned integer, within the range 0 to 255, representing the Slice/Service Type.

It indicates the expected Network Slice behaviour in terms of features and services.";

mandatory true;

type uint32;

}

leaf sd {

description "3-octet string, representing the Slice Differentiator, in hexadecimal representation.";

//optional

type string {

pattern '^[A-Fa-f0-9]{6}$';

}

}

reference "3GPP TS 29.571";

}

typedef PduSessionType {

type enumeration {

enum IPV4;

enum IPV6;

enum IPV4V6;

enum UNSTRUCTURED;

enum ETHERNET;

}

}

grouping Guami {

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list amfId {

description "AMF Identity.";

min-elements 1;

max-elements 1;

key "amfRegionId amfSetId amfPointer";

uses types3gpp:AmfIdentifier;

}

}

grouping Tai {

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf tac { type types3gpp:Tac; }

}

grouping InterfaceUpfInfoItem {

leaf interfaceType {

description "User Plane interface type.";

mandatory true;

type UPInterfaceType;

}

////At least one of the addressing parameters (ipv4address, ipv6adress or endpointFqdn) shall be included in the InterfaceUpfInfoItem.

choice address {

case ipv4EndpointAddresses {

leaf-list ipv4EndpointAddresses {

description "Available endpoint IPv4 address(es) of the User Plane interface.";

//conditional support

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddresses {

leaf-list ipv6EndpointAddresses {

description "Available endpoint IPv6 address(es) of the User Plane interface.";

//conditional support

min-elements 1;

type inet:ipv6-address;

}

}

case endpointFqdn {

leaf endpointFqdn {

description "FQDN of available endpoint of the User Plane interface.";

//conditional support

type inet:domain-name;

}

}

}

leaf networkInstance {

description "Network Instance associated to the User Plane interface.";

//optional support

type string;

}

}

typedef UPInterfaceType {

type enumeration {

enum N3;

enum N6;

enum N9;

}

}

grouping TaiRange {

list plmnId {

description "PLMN ID related to the TacRange.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list tacRangeList { //is this key unique

description "The range of the TACs.";

min-elements 1;

key "start end";

uses TacRange;

}

}

typedef AccessType {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

}

grouping N2InterfaceAmfInfo {

//At least one of the addressing parameters (ipv4address or ipv6adress) shall be included.

choice address {

case ipv4EndpointAddress {

leaf-list ipv4EndpointAddress {

description "Available AMF endpoint IPv4 address(es) for N2.";

//conditional support

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddress {

leaf-list ipv6EndpointAddress {

description "Available AMF endpoint IPv6 address(es) for N2.";

//conditional support

min-elements 1;

type inet:ipv6-address;

}

}

}

leaf amfName {

description "AMF name.";

type string;

}

}

grouping sNssaiSmfInfoItem {

list sNssai { //is the key unique

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

list dnnSmfInfoList { //is the key unique

description "List of parameters supported by the SMF per DNN.";

min-elements 1;

key dnn;

uses DnnSmfInfoItem;

}

}

grouping DnnSmfInfoItem {

leaf dnn {

description "Supported DNN.";

mandatory true;

type string;

}

}

grouping PlmnSnssai {

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s) is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssaiList { //is the key unique

description "The specific list of S-NSSAIs supported by the given PLMN.";

min-elements 1;

key "sst sd";

uses Snssai;

}

}

}module _3gpp-5gc-nrm-nfservice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfservice;

prefix nfs3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

organization "3gpp SA5";

description "NF service class.";

reference "3GPP TS 29.510";

revision 2019-06-17 {

description "initial revision";

reference "Based on

3GPP TS 29.510 V15.X.XX";

}

grouping NFServiceGrp {

leaf serviceInstanceID {

description "Unique ID of the service instance within a given NF Instance.";

mandatory true;

type string;

}

leaf serviceName {

description "Name of the service instance (e.g. 'nudm-sdm').";

mandatory true;

type ServiceName;

}

list versions { //check in review if key is ok (unique)

description "API versions supported by the NF Service and if available, the corresponding retirement date of the NF Service.";

min-elements 1;

key "apiVersionInUri apiFullVersion";

uses NFServiceVersion;

}

leaf scheme {

description "URI scheme (e.g. 'http', 'https').";

mandatory true;

type UriScheme;

}

leaf nfServiceStatus {

description "Status of the NF Service Instance.";

mandatory true;

type NFServiceStatus;

}

leaf fqdn {

description "FQDN of the NF Service Instance.";

//optional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF service needs to be discoverable by other NFs in a different PLMN,

then an FQDN that is used for inter PLMN routing.";

//optional support

type inet:domain-name;

}

list ipEndPoints {

description "IP address(es) and port information of the Network Function (including IPv4 and/or IPv6 address)

where the service is listening for incoming service requests.";

//optional support

key idx;

leaf idx {

type string;

}

min-elements 1;

uses ipEndPoint;

}

leaf apiPrefix {

description "Optional path segment(s) used to construct the {apiRoot} variable of the different API URIs.";

//optional support

type string;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

list allowedPlmns {

description "PLMNs allowed to access the service instance.

The absence of this attribute indicates that any PLMN is allowed to access the service instance.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the service instance.

The absence of this attribute indicates that any NF type is allowed to access the service instance.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to access the service instance.";

//optional support

min-elements 1;

type string;

}

leaf-list allowedNssais {

description "S-NSSAI of the allowed slices to access the service instance.

The absence of this attribute indicates that any slice is allowed to access the service instance.";

min-elements 1;

//optional support

type types3gpp:SNssai;

}

leaf priority {

description "Priority (relative to other services of the same type) in the range of 0-65535,

to be used for NF Service selection; lower values indicate a higher priority.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535, expressed as a weight relative to other services of the same type.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates the current load percentage of the NF Service.";

//optional support

type types3gpp:Load;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

list chfServiceInfo { //is the key unique

description "Specific data for a CHF service instance.";

//optional support

max-elements 1;

key "primaryChfServiceInstance secondaryChfServiceInstance";

uses ChfServiceInfo;

}

leaf supportedFeatures {

description "Supported Features of the NF Service instance.";

//optional support

type SupportedFeatures;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type TransportProtocol;

}

leaf port {

type uint16;

}

}

typedef TransportProtocol {

type enumeration {

enum TCP;

enum STCP;

enum UDP;

}

}

grouping NFServiceVersion {

leaf apiVersionInUri {

mandatory true;

type string;

}

leaf apiFullVersion {

mandatory true;

type string;

}

leaf expiry {

//optional to support

type yang:date-and-time;

}

}

typedef ServiceName {

type enumeration {

enum NNRF_NFM;

enum NNRF_DISC;

enum NUDM_SDM;

enum NUDM_UECM;

enum NUDM_UEAU;

enum NUDM_EE;

enum NUDM_PP;

enum NAMF_COMM;

enum NAMF_EVTS;

enum NAMF_MT;

enum NAMF_LOC;

enum NSMF_PDUSESSION;

enum NSMF_EVENT-EXPOSURE;

enum NAUSF_AUTH;

enum NAUSF_SORPROTECTION;

enum NNEF_PFDMANAGEMENT;

enum NPCF_AM-POLICY-CONTROL;

enum NPCF_SMPOLICYCONTROL;

enum NPCF_POLICYAUTHORIZATION;

enum NPCF_BDTPOLICYCONTROL;

enum NPCF_EVENTEXPOSURE;

enum NPCF_UE_POLICY_CONTROL;

enum NSMSF_SMS;

enum NNSSF_NSSELECTION;

enum NNSSF_NSSAIAVAILABILITY;

enum NUDR_DR;

enum NLMF_LOC;

enum N5G_EIR_EIC;

enum NBSF_MANAGEMENT;

enum NCHF_SPENDINGLIMITCONTROL;

enum NCHF_CONVERGEDCHARGING;

enum NNWDAF_EVENTSSUBSCRIPTION;

enum NNWDAF_ANALYTICSINFO;

}

}

typedef UriScheme {

type enumeration {

enum HTTP;

enum HTTPS;

}

}

typedef NFServiceStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

enum UNDISCOVERABLE;

}

}

grouping ChfServiceInfo {

leaf primaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a secondary CHF instance of another primary CHF service instance.";

//conditional to support

type string;

}

leaf secondaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a primary CHF instance of another secondary CHF service instance.";

//conditional to support

type string;

}

}

}module _3gpp-5gc-nrm-ngeirfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ngeirfunction;

prefix ngeir3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the 5G-EIR function in 5GC. For more information about the 5G-EIR, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NGEIRFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list NGEIRFunction {

description "5G Core NGEIR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NGEIRFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-nrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nrffunction;

prefix nrf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the NRF function in 5GC.

For more information about the NRF, see 3GPP TS 23.501 [2].";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NRFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list nSIIdListWrap {

description "Set of NSI Ids. The NSI ID represents the Network Slice Instance Identifier.";

//optional support

type types3gpp:NsiId;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

list nFProfileList {

description "Set of NFProfile(s) to be registered in the NRF instance.";

//optional support

key nfInstanceID;

uses nfp3gpp:NFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NRFFunction {

description "5G Core NRF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-nssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nssffunction;

prefix nssf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the NSSF function in 5GC. For more information about the NSSF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NSSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

leaf-list nSIIdListWrap {

description "Set of NSI Ids. The NSI ID represents the Network Slice Instance Identifier.";

//optional support

type types3gpp:NsiId;

}

}

augment "/me3gpp:ManagedElement" {

list NSSFFunction {

description "5G Core NSSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NSSFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-nwdaffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nwdaffunction;

prefix nwdaf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping NWDAFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list NWDAFFunction {

description "5G Core NWDAF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NWDAFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-pcffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-pcffunction;

prefix pcf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the PCF function in 5GC. For more information about the PCF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-22 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping PCFFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list PCFFunction {

description "5G Core PCF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses PCFFuntionGrp;

}

}

}

}module _3gpp-5gc-nrm-seppfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-seppfunction;

prefix sepp3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the SEPP function which support message filtering

and policing on inter-PLMN control plane interface. For more information about the SEPP, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping SEPPFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/me3gpp:ManagedElement" {

list SEPPFunction {

description "5G Core SEPP Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SEPPFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-smffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smffunction;

prefix smf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

description "SMFFunction derived from basic ManagedFunction.";

revision 2019-05-31 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

revision 2018-08-07 {

description "Initial revision";

}

grouping SMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

min-elements 1;

config false;

type string;

}

leaf-list sNSSAIList {

min-elements 0;

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list SMFFunction {

description "5G Core SMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-smsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smsffunction;

prefix smsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the SMSF function defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping SMSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

}

augment "/me3gpp:ManagedElement" {

list SMSFFunction {

description "5G Core SMSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMSFFunctionGrp;

}

}

}

}module _3gpp-5gc-nrm-udmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udmfunction;

prefix udm3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the UDM function in 5GC. For more information about the UDM, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-22 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping UDMFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list UDMFunction {

description "5G Core UDM Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDMFuntionGrp;

}

}

}

}module _3gpp-5gc-nrm-udrfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udrfunction;

prefix udr3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the UDR function in 5GC. For more information about the UDR, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-22 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping UDRFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list UDRFunction {

description "5G Core UDR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDRFuntionGrp;

}

}

}

}module _3gpp-5gc-nrm-udsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udsffunction;

prefix udsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the UDSF function which can be interacted with any other 5GC NF defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-05-22 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping UDSFFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sBIServiceList {

description "All supported NF services registered on the service-based interface.";

config false;

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list UDSFFunction {

description "5G Core UDSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDSFFuntionGrp;

}

}

}

}module _3gpp-5gc-nrm-upffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-upffunction;

prefix upf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

description "UPFFunction derived from basic ManagedFunction.";

revision 2019-05-31 {

description "Ericsson refactoring.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

revision 2018-08-07 {

description "Initial revision";

}

grouping UPFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "A list of PLMN identifiers (Mobile Country Code and Mobile Network Code).";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

min-elements 0;

reference "3GPP TS 23.003";

type types3gpp:SNssai;

}

}

augment "/me3gpp:ManagedElement" {

list UPFFunction {

description "5G Core UPF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UPFFunctionGrp;

}

}

}

}

# H.6	Graphical representation

This is the YANG module tree diagram for 5GC, NR and NG-RAN NRM. Its syntax is defined in [46].

module: _3gpp-common-managed-element

+--rw ManagedElement* [id]

+--rw id                             string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--ro locationName?               string

|  +--ro managedBy*                  types3gpp:DistinguishedName

|  +--ro managedElementTypeList*     string

|  +--ro supportedMeasurementsGPs* [measurementType] {MeasurementsUnderManagedElement}?

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--ro vendorName?                 string

|  +--rw userDefinedState?           string

|  +--ro swVersion?                  string

|  +--rw priorityLabel               uint32

+--rw MeasurementControl* [id] {MeasurementsUnderManagedElement}?

|  +--rw id                   string

|  +--rw attributes

|  |  +--rw pMAdministrativeState?        types3gpp:AdministrativeState

|  |  +--ro pMOperationalState            types3gpp:OperationalState

|  |  +--rw defaultFileLocation?          string

|  |  +--rw defaultFileBasedGP            uint32

|  |  +--rw defaultFileReportingPeriod    uint32

|  |  +--rw defaultStreamTarget?          string {StreamingSupported}?

|  |  +--rw defaultStreamBasedGP          uint32 {StreamingSupported}?

|  +--rw MeasurementReader* [id]

|     +--rw id            string

|     +--rw attributes

|        +--rw measurementTypes*        string

|        +--rw managedObjectDNsBasic*   types3gpp:DistinguishedName

|        +--rw managedObjectDNs*        types3gpp:DistinguishedName

|        +--rw fileLocation?            string

|        +--rw fileBasedGP              uint32

|        +--rw fileReportingPeriod      uint32

|        +--rw streamTarget?            string

|        +--rw streamBasedGP            uint32

+--rw gnbdu3gpp:GNBDUFunction* [id]

|  +--rw gnbdu3gpp:id                      string

|  +--rw gnbdu3gpp:attributes

|  |  +--rw gnbdu3gpp:userLabel?           string

|  |  +--rw gnbdu3gpp:vnfParametersList!

|  |  |  +--rw gnbdu3gpp:vnfInstanceId    string

|  |  |  +--rw gnbdu3gpp:vnfdId?          string

|  |  |  +--rw gnbdu3gpp:flavourId?       string

|  |  |  +--rw gnbdu3gpp:autoScalable     boolean

|  |  +--rw gnbdu3gpp:peeParametersList!

|  |  |  +--rw gnbdu3gpp:siteIdentification    string

|  |  |  +--rw gnbdu3gpp:siteLatitude?         decimal64

|  |  |  +--rw gnbdu3gpp:siteLongitude?        decimal64

|  |  |  +--rw gnbdu3gpp:siteDescription       string

|  |  |  +--rw gnbdu3gpp:equipmentType         string

|  |  |  +--rw gnbdu3gpp:environmentType       string

|  |  |  +--rw gnbdu3gpp:powerInterface        string

|  |  +--rw gnbdu3gpp:priorityLabel        uint32

|  |  +--rw gnbdu3gpp:gNBId                int64

|  |  +--rw gnbdu3gpp:gNBIdLength          int32

|  |  +--rw gnbdu3gpp:gNBDUId              int64

|  |  +--rw gnbdu3gpp:gNBDUName            string

|  +--rw nrsectcarr3gpp:NRSectorCarrier* [id]

|  |  +--rw nrsectcarr3gpp:id            string

|  |  +--rw nrsectcarr3gpp:attributes

|  |     +--rw nrsectcarr3gpp:userLabel?                    string

|  |     +--rw nrsectcarr3gpp:vnfParametersList!

|  |     |  +--rw nrsectcarr3gpp:vnfInstanceId    string

|  |     |  +--rw nrsectcarr3gpp:vnfdId?          string

|  |     |  +--rw nrsectcarr3gpp:flavourId?       string

|  |     |  +--rw nrsectcarr3gpp:autoScalable     boolean

|  |     +--rw nrsectcarr3gpp:peeParametersList!

|  |     |  +--rw nrsectcarr3gpp:siteIdentification    string

|  |     |  +--rw nrsectcarr3gpp:siteLatitude?         decimal64

|  |     |  +--rw nrsectcarr3gpp:siteLongitude?        decimal64

|  |     |  +--rw nrsectcarr3gpp:siteDescription       string

|  |     |  +--rw nrsectcarr3gpp:equipmentType         string

|  |     |  +--rw nrsectcarr3gpp:environmentType       string

|  |     |  +--rw nrsectcarr3gpp:powerInterface        string

|  |     +--rw nrsectcarr3gpp:priorityLabel                 uint32

|  |     +--rw nrsectcarr3gpp:txDirection                   types3gpp:TxDirection

|  |     +--rw nrsectcarr3gpp:configuredMaxTxPower          int32

|  |     +--rw nrsectcarr3gpp:arfcnDL                       int32

|  |     +--rw nrsectcarr3gpp:arfcnUL                       int32

|  |     +--rw nrsectcarr3gpp:bSChannelBwDL                 int32

|  |     +--rw nrsectcarr3gpp:bSChannelBwUL                 int32

|  |     +--rw nrsectcarr3gpp:sectorEquipmentFunctionRef    types3gpp:DistinguishedName

|  +--rw bwp3gpp:BWP* [id]

|  |  +--rw bwp3gpp:id            string

|  |  +--rw bwp3gpp:attributes

|  |     +--rw bwp3gpp:userLabel?           string

|  |     +--rw bwp3gpp:vnfParametersList!

|  |     |  +--rw bwp3gpp:vnfInstanceId    string

|  |     |  +--rw bwp3gpp:vnfdId?          string

|  |     |  +--rw bwp3gpp:flavourId?       string

|  |     |  +--rw bwp3gpp:autoScalable     boolean

|  |     +--rw bwp3gpp:peeParametersList!

|  |     |  +--rw bwp3gpp:siteIdentification    string

|  |     |  +--rw bwp3gpp:siteLatitude?         decimal64

|  |     |  +--rw bwp3gpp:siteLongitude?        decimal64

|  |     |  +--rw bwp3gpp:siteDescription       string

|  |     |  +--rw bwp3gpp:equipmentType         string

|  |     |  +--rw bwp3gpp:environmentType       string

|  |     |  +--rw bwp3gpp:powerInterface        string

|  |     +--rw bwp3gpp:priorityLabel        uint32

|  |     +--rw bwp3gpp:bwpContext           BwpContext

|  |     +--rw bwp3gpp:isInitialBwp         IsInitialBwp

|  |     +--rw bwp3gpp:subCarrierSpacing    uint32

|  |     +--rw bwp3gpp:cyclicPrefix         CyclicPrefix

|  |     +--rw bwp3gpp:startRB              uint32

|  |     +--rw bwp3gpp:numberOfRBs          uint32

|  +--rw nrcelldu3gpp:NRCellDU* [id]

|  |  +--rw nrcelldu3gpp:id            string

|  |  +--rw nrcelldu3gpp:attributes

|  |     +--rw nrcelldu3gpp:userLabel?              string

|  |     +--rw nrcelldu3gpp:vnfParametersList!

|  |     |  +--rw nrcelldu3gpp:vnfInstanceId    string

|  |     |  +--rw nrcelldu3gpp:vnfdId?          string

|  |     |  +--rw nrcelldu3gpp:flavourId?       string

|  |     |  +--rw nrcelldu3gpp:autoScalable     boolean

|  |     +--rw nrcelldu3gpp:peeParametersList!

|  |     |  +--rw nrcelldu3gpp:siteIdentification    string

|  |     |  +--rw nrcelldu3gpp:siteLatitude?         decimal64

|  |     |  +--rw nrcelldu3gpp:siteLongitude?        decimal64

|  |     |  +--rw nrcelldu3gpp:siteDescription       string

|  |     |  +--rw nrcelldu3gpp:equipmentType         string

|  |     |  +--rw nrcelldu3gpp:environmentType       string

|  |     |  +--rw nrcelldu3gpp:powerInterface        string

|  |     +--rw nrcelldu3gpp:priorityLabel           uint32

|  |     +--rw nrcelldu3gpp:cellLocalId             int32

|  |     +--ro nrcelldu3gpp:operationalState?       types3gpp:OperationalState

|  |     +--rw nrcelldu3gpp:administrativeState     types3gpp:AdministrativeState

|  |     +--ro nrcelldu3gpp:cellState?              types3gpp:CellState

|  |     +--rw nrcelldu3gpp:pLMNIdList* [mcc mnc]

|  |     |  +--rw nrcelldu3gpp:mcc    Mcc

|  |     |  +--rw nrcelldu3gpp:mnc    Mnc

|  |     +--rw nrcelldu3gpp:sNSSAIList*             types3gpp:SNssai

|  |     +--rw nrcelldu3gpp:nRPCI                   int32

|  |     +--rw nrcelldu3gpp:nRTAC                   types3gpp:Tac

|  |     +--rw nrcelldu3gpp:arfcnDL                 int32

|  |     +--rw nrcelldu3gpp:arfcnUL                 int32

|  |     +--rw nrcelldu3gpp:arfcnSUL                int32

|  |     +--rw nrcelldu3gpp:bSChannelBwDL           int32

|  |     +--rw nrcelldu3gpp:bSChannelBwUL?          int32

|  |     +--rw nrcelldu3gpp:bSChannelBwSUL?         int32

|  |     +--rw nrcelldu3gpp:ssbFrequency            int32

|  |     +--rw nrcelldu3gpp:ssbPeriodicity          int32

|  |     +--rw nrcelldu3gpp:ssbSubCarrierSpacing    int32

|  |     +--rw nrcelldu3gpp:ssbOffset               int32

|  |     +--rw nrcelldu3gpp:ssbDuration             int32

|  |     +--rw nrcelldu3gpp:nRSectorCarrierRef*     types3gpp:DistinguishedName

|  |     +--rw nrcelldu3gpp:bWPRef*                 types3gpp:DistinguishedName

|  |     +--rw nrcelldu3gpp:nRFrequencyRef*         types3gpp:DistinguishedName

|  +--rw ep3gpp:EP_F1C* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_F1U* [id]

|     +--rw ep3gpp:id            string

|     +--rw ep3gpp:attributes

|        +--rw ep3gpp:userLabel?                  string

|        +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro ep3gpp:measurementType    string

|        |  +--ro ep3gpp:supportedGPs*      uint32

|        +--rw ep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw ep3gpp:ipAddress    inet:ip-address

|        |  +--rw ep3gpp:vlanId       uint16

|        +--rw ep3gpp:remoteAddress               inet:ip-address

+--rw pcf3gpp:PCFFunction* [id]

|  +--rw pcf3gpp:id            string

|  +--rw pcf3gpp:attributes

|  |  +--rw pcf3gpp:userLabel?           string

|  |  +--rw pcf3gpp:vnfParametersList!

|  |  |  +--rw pcf3gpp:vnfInstanceId    string

|  |  |  +--rw pcf3gpp:vnfdId?          string

|  |  |  +--rw pcf3gpp:flavourId?       string

|  |  |  +--rw pcf3gpp:autoScalable     boolean

|  |  +--rw pcf3gpp:peeParametersList!

|  |  |  +--rw pcf3gpp:siteIdentification    string

|  |  |  +--rw pcf3gpp:siteLatitude?         decimal64

|  |  |  +--rw pcf3gpp:siteLongitude?        decimal64

|  |  |  +--rw pcf3gpp:siteDescription       string

|  |  |  +--rw pcf3gpp:equipmentType         string

|  |  |  +--rw pcf3gpp:environmentType       string

|  |  |  +--rw pcf3gpp:powerInterface        string

|  |  +--rw pcf3gpp:priorityLabel        uint32

|  |  +--rw pcf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw pcf3gpp:mcc    Mcc

|  |  |  +--rw pcf3gpp:mnc    Mnc

|  |  +--rw pcf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro pcf3gpp:sBIServiceList*      string

|  |  +--rw pcf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N5* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N7* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N15* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N16* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_Rx* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw af3gpp:AFFunction* [id]

|  +--rw af3gpp:id            string

|  +--rw af3gpp:attributes

|  |  +--rw af3gpp:userLabel?           string

|  |  +--rw af3gpp:vnfParametersList!

|  |  |  +--rw af3gpp:vnfInstanceId    string

|  |  |  +--rw af3gpp:vnfdId?          string

|  |  |  +--rw af3gpp:flavourId?       string

|  |  |  +--rw af3gpp:autoScalable     boolean

|  |  +--rw af3gpp:peeParametersList!

|  |  |  +--rw af3gpp:siteIdentification    string

|  |  |  +--rw af3gpp:siteLatitude?         decimal64

|  |  |  +--rw af3gpp:siteLongitude?        decimal64

|  |  |  +--rw af3gpp:siteDescription       string

|  |  |  +--rw af3gpp:equipmentType         string

|  |  |  +--rw af3gpp:environmentType       string

|  |  |  +--rw af3gpp:powerInterface        string

|  |  +--rw af3gpp:priorityLabel        uint32

|  +--rw cep3gpp:EP_N6* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_Rx* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw udm3gpp:UDMFunction* [id]

|  +--rw udm3gpp:id            string

|  +--rw udm3gpp:attributes

|  |  +--rw udm3gpp:userLabel?           string

|  |  +--rw udm3gpp:vnfParametersList!

|  |  |  +--rw udm3gpp:vnfInstanceId    string

|  |  |  +--rw udm3gpp:vnfdId?          string

|  |  |  +--rw udm3gpp:flavourId?       string

|  |  |  +--rw udm3gpp:autoScalable     boolean

|  |  +--rw udm3gpp:peeParametersList!

|  |  |  +--rw udm3gpp:siteIdentification    string

|  |  |  +--rw udm3gpp:siteLatitude?         decimal64

|  |  |  +--rw udm3gpp:siteLongitude?        decimal64

|  |  |  +--rw udm3gpp:siteDescription       string

|  |  |  +--rw udm3gpp:equipmentType         string

|  |  |  +--rw udm3gpp:environmentType       string

|  |  |  +--rw udm3gpp:powerInterface        string

|  |  +--rw udm3gpp:priorityLabel        uint32

|  |  +--rw udm3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw udm3gpp:mcc    Mcc

|  |  |  +--rw udm3gpp:mnc    Mnc

|  |  +--rw udm3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro udm3gpp:sBIServiceList*      string

|  |  +--rw udm3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N8* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N10* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N13* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw udr3gpp:UDRFunction* [id]

|  +--rw udr3gpp:id            string

|  +--rw udr3gpp:attributes

|  |  +--rw udr3gpp:userLabel?           string

|  |  +--rw udr3gpp:vnfParametersList!

|  |  |  +--rw udr3gpp:vnfInstanceId    string

|  |  |  +--rw udr3gpp:vnfdId?          string

|  |  |  +--rw udr3gpp:flavourId?       string

|  |  |  +--rw udr3gpp:autoScalable     boolean

|  |  +--rw udr3gpp:peeParametersList!

|  |  |  +--rw udr3gpp:siteIdentification    string

|  |  |  +--rw udr3gpp:siteLatitude?         decimal64

|  |  |  +--rw udr3gpp:siteLongitude?        decimal64

|  |  |  +--rw udr3gpp:siteDescription       string

|  |  |  +--rw udr3gpp:equipmentType         string

|  |  |  +--rw udr3gpp:environmentType       string

|  |  |  +--rw udr3gpp:powerInterface        string

|  |  +--rw udr3gpp:priorityLabel        uint32

|  |  +--rw udr3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw udr3gpp:mcc    Mcc

|  |  |  +--rw udr3gpp:mnc    Mnc

|  |  +--rw udr3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro udr3gpp:sBIServiceList*      string

|  |  +--rw udr3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw smf3gpp:SMFFunction* [id]

|  +--rw smf3gpp:id            string

|  +--rw smf3gpp:attributes

|  |  +--rw smf3gpp:userLabel?           string

|  |  +--rw smf3gpp:vnfParametersList!

|  |  |  +--rw smf3gpp:vnfInstanceId    string

|  |  |  +--rw smf3gpp:vnfdId?          string

|  |  |  +--rw smf3gpp:flavourId?       string

|  |  |  +--rw smf3gpp:autoScalable     boolean

|  |  +--rw smf3gpp:peeParametersList!

|  |  |  +--rw smf3gpp:siteIdentification    string

|  |  |  +--rw smf3gpp:siteLatitude?         decimal64

|  |  |  +--rw smf3gpp:siteLongitude?        decimal64

|  |  |  +--rw smf3gpp:siteDescription       string

|  |  |  +--rw smf3gpp:equipmentType         string

|  |  |  +--rw smf3gpp:environmentType       string

|  |  |  +--rw smf3gpp:powerInterface        string

|  |  +--rw smf3gpp:priorityLabel        uint32

|  |  +--rw smf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw smf3gpp:mcc    Mcc

|  |  |  +--rw smf3gpp:mnc    Mnc

|  |  +--ro smf3gpp:nRTACList*           types3gpp:Tac

|  |  +--rw smf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro smf3gpp:sBIServiceList*      string

|  |  +--rw smf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N4* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N7* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N10* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N11* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N16* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_S5C* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw ngeir3gpp:NGEIRFunction* [id]

|  +--rw ngeir3gpp:id            string

|  +--rw ngeir3gpp:attributes

|  |  +--rw ngeir3gpp:userLabel?           string

|  |  +--rw ngeir3gpp:vnfParametersList!

|  |  |  +--rw ngeir3gpp:vnfInstanceId    string

|  |  |  +--rw ngeir3gpp:vnfdId?          string

|  |  |  +--rw ngeir3gpp:flavourId?       string

|  |  |  +--rw ngeir3gpp:autoScalable     boolean

|  |  +--rw ngeir3gpp:peeParametersList!

|  |  |  +--rw ngeir3gpp:siteIdentification    string

|  |  |  +--rw ngeir3gpp:siteLatitude?         decimal64

|  |  |  +--rw ngeir3gpp:siteLongitude?        decimal64

|  |  |  +--rw ngeir3gpp:siteDescription       string

|  |  |  +--rw ngeir3gpp:equipmentType         string

|  |  |  +--rw ngeir3gpp:environmentType       string

|  |  |  +--rw ngeir3gpp:powerInterface        string

|  |  +--rw ngeir3gpp:priorityLabel        uint32

|  |  +--rw ngeir3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw ngeir3gpp:mcc    Mcc

|  |  |  +--rw ngeir3gpp:mnc    Mnc

|  |  +--rw ngeir3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro ngeir3gpp:sBIServiceList*      string

|  |  +--rw ngeir3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N17* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw gnbcucp3gpp:GNBCUCPFunction* [id]

|  +--rw gnbcucp3gpp:id            string

|  +--rw gnbcucp3gpp:attributes

|  |  +--rw gnbcucp3gpp:userLabel?           string

|  |  +--rw gnbcucp3gpp:vnfParametersList!

|  |  |  +--rw gnbcucp3gpp:vnfInstanceId    string

|  |  |  +--rw gnbcucp3gpp:vnfdId?          string

|  |  |  +--rw gnbcucp3gpp:flavourId?       string

|  |  |  +--rw gnbcucp3gpp:autoScalable     boolean

|  |  +--rw gnbcucp3gpp:peeParametersList!

|  |  |  +--rw gnbcucp3gpp:siteIdentification    string

|  |  |  +--rw gnbcucp3gpp:siteLatitude?         decimal64

|  |  |  +--rw gnbcucp3gpp:siteLongitude?        decimal64

|  |  |  +--rw gnbcucp3gpp:siteDescription       string

|  |  |  +--rw gnbcucp3gpp:equipmentType         string

|  |  |  +--rw gnbcucp3gpp:environmentType       string

|  |  |  +--rw gnbcucp3gpp:powerInterface        string

|  |  +--rw gnbcucp3gpp:priorityLabel        uint32

|  |  +--rw gnbcucp3gpp:gNBId                int64

|  |  +--rw gnbcucp3gpp:gNBIdLength          int32

|  |  +--rw gnbcucp3gpp:gNBCUName            string

|  |  +--rw gnbcucp3gpp:pLMNId* [mcc mnc]

|  |     +--rw gnbcucp3gpp:mcc    Mcc

|  |     +--rw gnbcucp3gpp:mnc    Mnc

|  +--rw nrcellcu3gpp:NRCellCU* [id]

|  |  +--rw nrcellcu3gpp:id                         string

|  |  +--rw nrcellcu3gpp:attributes

|  |  |  +--rw nrcellcu3gpp:userLabel?             string

|  |  |  +--rw nrcellcu3gpp:vnfParametersList!

|  |  |  |  +--rw nrcellcu3gpp:vnfInstanceId    string

|  |  |  |  +--rw nrcellcu3gpp:vnfdId?          string

|  |  |  |  +--rw nrcellcu3gpp:flavourId?       string

|  |  |  |  +--rw nrcellcu3gpp:autoScalable     boolean

|  |  |  +--rw nrcellcu3gpp:peeParametersList!

|  |  |  |  +--rw nrcellcu3gpp:siteIdentification    string

|  |  |  |  +--rw nrcellcu3gpp:siteLatitude?         decimal64

|  |  |  |  +--rw nrcellcu3gpp:siteLongitude?        decimal64

|  |  |  |  +--rw nrcellcu3gpp:siteDescription       string

|  |  |  |  +--rw nrcellcu3gpp:equipmentType         string

|  |  |  |  +--rw nrcellcu3gpp:environmentType       string

|  |  |  |  +--rw nrcellcu3gpp:powerInterface        string

|  |  |  +--rw nrcellcu3gpp:priorityLabel          uint32

|  |  |  +--rw nrcellcu3gpp:cellLocalId            int32

|  |  |  +--rw nrcellcu3gpp:pLMNIdList* [mcc mnc]

|  |  |  |  +--rw nrcellcu3gpp:mcc    Mcc

|  |  |  |  +--rw nrcellcu3gpp:mnc    Mnc

|  |  |  +--rw nrcellcu3gpp:sNSSAIList*            types3gpp:SNssai

|  |  |  +--rw nrcellcu3gpp:rRMPolicyType          uint16

|  |  |  +--rw nrcellcu3gpp:rRMPolicy              string

|  |  |  +--rw nrcellcu3gpp:rRMPolicyRatio1List* [groupId]

|  |  |  |  +--rw nrcellcu3gpp:groupId            uint32

|  |  |  |  +--rw nrcellcu3gpp:sNSSAIList*        types3gpp:SNssai

|  |  |  |  +--rw nrcellcu3gpp:rRRMPolicyRatio?   uint8

|  |  |  +--rw nrcellcu3gpp:rRMPolicyRatio2* [groupId]

|  |  |  |  +--rw nrcellcu3gpp:groupId                    uint32

|  |  |  |  +--rw nrcellcu3gpp:sNSSAIList*                types3gpp:SNssai

|  |  |  |  +--rw nrcellcu3gpp:quotaType?                 quotaType

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMaxRatio?         uint8

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMarginMaxRatio?   uint8

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMinRatio?         uint8

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMarginMinRatio?   uint8

|  |  |  +--ro nrcellcu3gpp:nRFrequencyRef?        types3gpp:DistinguishedName

|  |  +--rw eutranfreqrel3gpp:EUtranFreqRelation* [id]

|  |  |  +--rw eutranfreqrel3gpp:id            string

|  |  |  +--rw eutranfreqrel3gpp:attributes

|  |  |     +--rw eutranfreqrel3gpp:userLabel?                    string

|  |  |     +--rw eutranfreqrel3gpp:vnfParametersList!

|  |  |     |  +--rw eutranfreqrel3gpp:vnfInstanceId    string

|  |  |     |  +--rw eutranfreqrel3gpp:vnfdId?          string

|  |  |     |  +--rw eutranfreqrel3gpp:flavourId?       string

|  |  |     |  +--rw eutranfreqrel3gpp:autoScalable     boolean

|  |  |     +--rw eutranfreqrel3gpp:peeParametersList!

|  |  |     |  +--rw eutranfreqrel3gpp:siteIdentification    string

|  |  |     |  +--rw eutranfreqrel3gpp:siteLatitude?         decimal64

|  |  |     |  +--rw eutranfreqrel3gpp:siteLongitude?        decimal64

|  |  |     |  +--rw eutranfreqrel3gpp:siteDescription       string

|  |  |     |  +--rw eutranfreqrel3gpp:equipmentType         string

|  |  |     |  +--rw eutranfreqrel3gpp:environmentType       string

|  |  |     |  +--rw eutranfreqrel3gpp:powerInterface        string

|  |  |     +--rw eutranfreqrel3gpp:priorityLabel                 uint32

|  |  |     +--rw eutranfreqrel3gpp:cellIndividualOffset?         types3gpp:QOffsetRange

|  |  |     +--rw eutranfreqrel3gpp:blackListEntry*               uint16

|  |  |     +--rw eutranfreqrel3gpp:blackListEntryIdleMode*       uint16

|  |  |     +--rw eutranfreqrel3gpp:cellReselectionPriority       int32

|  |  |     +--rw eutranfreqrel3gpp:cellReselectionSubPriority?   uint8

|  |  |     +--rw eutranfreqrel3gpp:pMax                          int32

|  |  |     +--rw eutranfreqrel3gpp:qOffsetFreq?                  int32

|  |  |     +--rw eutranfreqrel3gpp:qQualMin                      int32

|  |  |     +--rw eutranfreqrel3gpp:qRxLevMin                     int32

|  |  |     +--rw eutranfreqrel3gpp:threshXHighP                  int32

|  |  |     +--rw eutranfreqrel3gpp:threshXHighQ                  int32

|  |  |     +--rw eutranfreqrel3gpp:threshXLowP                   int32

|  |  |     +--rw eutranfreqrel3gpp:threshXLowQ?                  int32

|  |  |     +--rw eutranfreqrel3gpp:tReselectionEutra             uint8

|  |  |     +--rw eutranfreqrel3gpp:tReselectionEutraSfHigh       uint8

|  |  |     +--rw eutranfreqrel3gpp:tReselectionEutraSfMedium     uint8

|  |  |     +--rw eutranfreqrel3gpp:eUtranFrequencyRef            types3gpp:DistinguishedName

|  |  +--rw eutrancellrel3gpp:EUtranCellRelation* [id]

|  |  |  +--rw eutrancellrel3gpp:id            string

|  |  |  +--rw eutrancellrel3gpp:attributes

|  |  |     +--rw eutrancellrel3gpp:userLabel?                      string

|  |  |     +--rw eutrancellrel3gpp:vnfParametersList!

|  |  |     |  +--rw eutrancellrel3gpp:vnfInstanceId    string

|  |  |     |  +--rw eutrancellrel3gpp:vnfdId?          string

|  |  |     |  +--rw eutrancellrel3gpp:flavourId?       string

|  |  |     |  +--rw eutrancellrel3gpp:autoScalable     boolean

|  |  |     +--rw eutrancellrel3gpp:peeParametersList!

|  |  |     |  +--rw eutrancellrel3gpp:siteIdentification    string

|  |  |     |  +--rw eutrancellrel3gpp:siteLatitude?         decimal64

|  |  |     |  +--rw eutrancellrel3gpp:siteLongitude?        decimal64

|  |  |     |  +--rw eutrancellrel3gpp:siteDescription       string

|  |  |     |  +--rw eutrancellrel3gpp:equipmentType         string

|  |  |     |  +--rw eutrancellrel3gpp:environmentType       string

|  |  |     |  +--rw eutrancellrel3gpp:powerInterface        string

|  |  |     +--rw eutrancellrel3gpp:priorityLabel                   uint32

|  |  |     +--rw eutrancellrel3gpp:tCI                             uint64

|  |  |     +--rw eutrancellrel3gpp:isRemoveAllowed                 ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isHOAllowed                     ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isICICInformationSendAllowed    ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isLBAllowed                     ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isESCoveredBy                   EnergySavingCoverage

|  |  |     +--rw eutrancellrel3gpp:qOffset                         types3gpp:QOffsetRange

|  |  |     +--ro eutrancellrel3gpp:cellIndividualOffset?           types3gpp:QOffsetRange

|  |  |     +--rw eutrancellrel3gpp:adjacentCell                    types3gpp:DistinguishedName

|  |  +--rw nrcellrel3gpp:NRCellRelation* [id]

|  |  |  +--rw nrcellrel3gpp:id            string

|  |  |  +--rw nrcellrel3gpp:attributes

|  |  |     +--rw nrcellrel3gpp:userLabel?              string

|  |  |     +--rw nrcellrel3gpp:vnfParametersList!

|  |  |     |  +--rw nrcellrel3gpp:vnfInstanceId    string

|  |  |     |  +--rw nrcellrel3gpp:vnfdId?          string

|  |  |     |  +--rw nrcellrel3gpp:flavourId?       string

|  |  |     |  +--rw nrcellrel3gpp:autoScalable     boolean

|  |  |     +--rw nrcellrel3gpp:peeParametersList!

|  |  |     |  +--rw nrcellrel3gpp:siteIdentification    string

|  |  |     |  +--rw nrcellrel3gpp:siteLatitude?         decimal64

|  |  |     |  +--rw nrcellrel3gpp:siteLongitude?        decimal64

|  |  |     |  +--rw nrcellrel3gpp:siteDescription       string

|  |  |     |  +--rw nrcellrel3gpp:equipmentType         string

|  |  |     |  +--rw nrcellrel3gpp:environmentType       string

|  |  |     |  +--rw nrcellrel3gpp:powerInterface        string

|  |  |     +--rw nrcellrel3gpp:priorityLabel           uint32

|  |  |     +--rw nrcellrel3gpp:nRTCI?                  uint64

|  |  |     +--rw nrcellrel3gpp:cellIndividualOffset

|  |  |     |  +--rw nrcellrel3gpp:rsrpOffsetSsb?     types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:rsrqOffsetSsb?     types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:sinrOffsetSsb?     types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:rsrpOffsetCsiRs?   types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:rsrqOffsetCsiRs?   types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:sinrOffsetCsiRs?   types3gpp:QOffsetRange

|  |  |     +--rw nrcellrel3gpp:nRFreqRelationRef       types3gpp:DistinguishedName

|  |  |     +--rw nrcellrel3gpp:adjacentNRCellRef       types3gpp:DistinguishedName

|  |  +--rw nrfreqrel3gpp:NRFreqRelation* [id]

|  |     +--rw nrfreqrel3gpp:id            string

|  |     +--rw nrfreqrel3gpp:attributes

|  |        +--rw nrfreqrel3gpp:userLabel?                    string

|  |        +--rw nrfreqrel3gpp:vnfParametersList!

|  |        |  +--rw nrfreqrel3gpp:vnfInstanceId    string

|  |        |  +--rw nrfreqrel3gpp:vnfdId?          string

|  |        |  +--rw nrfreqrel3gpp:flavourId?       string

|  |        |  +--rw nrfreqrel3gpp:autoScalable     boolean

|  |        +--rw nrfreqrel3gpp:peeParametersList!

|  |        |  +--rw nrfreqrel3gpp:siteIdentification    string

|  |        |  +--rw nrfreqrel3gpp:siteLatitude?         decimal64

|  |        |  +--rw nrfreqrel3gpp:siteLongitude?        decimal64

|  |        |  +--rw nrfreqrel3gpp:siteDescription       string

|  |        |  +--rw nrfreqrel3gpp:equipmentType         string

|  |        |  +--rw nrfreqrel3gpp:environmentType       string

|  |        |  +--rw nrfreqrel3gpp:powerInterface        string

|  |        +--rw nrfreqrel3gpp:priorityLabel                 uint32

|  |        +--rw nrfreqrel3gpp:offsetMO

|  |        |  +--rw nrfreqrel3gpp:rsrpOffsetSsb?     types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:rsrqOffsetSsb?     types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:sinrOffsetSsb?     types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:rsrpOffsetCsiRs?   types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:rsrqOffsetCsiRs?   types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:sinrOffsetCsiRs?   types3gpp:QOffsetRange

|  |        +--rw nrfreqrel3gpp:blackListEntry*               uint16

|  |        +--rw nrfreqrel3gpp:blackListEntryIdleMode*       uint16

|  |        +--rw nrfreqrel3gpp:cellReselectionPriority?      uint32

|  |        +--rw nrfreqrel3gpp:cellReselectionSubPriority?   uint8

|  |        +--rw nrfreqrel3gpp:pMax?                         int32

|  |        +--rw nrfreqrel3gpp:qOffsetFreq?                  types3gpp:QOffsetRange

|  |        +--rw nrfreqrel3gpp:qQualMin?                     int32

|  |        +--rw nrfreqrel3gpp:qRxLevMin                     int32

|  |        +--rw nrfreqrel3gpp:threshXHighP                  int32

|  |        +--rw nrfreqrel3gpp:threshXHighQ                  int32

|  |        +--rw nrfreqrel3gpp:threshXLowP                   int32

|  |        +--rw nrfreqrel3gpp:threshXLowQ                   int32

|  |        +--rw nrfreqrel3gpp:tReselectionNR                int32

|  |        +--rw nrfreqrel3gpp:tReselectionNRSfHigh          uint8

|  |        +--rw nrfreqrel3gpp:tReselectionNRSfMedium        uint8

|  |        +--rw nrfreqrel3gpp:nRFrequencyRef                types3gpp:DistinguishedName

|  +--rw ep3gpp:EP_E1* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_F1C* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_NgC* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_XnC* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_X2C* [id]

|     +--rw ep3gpp:id            string

|     +--rw ep3gpp:attributes

|        +--rw ep3gpp:userLabel?                  string

|        +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro ep3gpp:measurementType    string

|        |  +--ro ep3gpp:supportedGPs*      uint32

|        +--rw ep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw ep3gpp:ipAddress    inet:ip-address

|        |  +--rw ep3gpp:vlanId       uint16

|        +--rw ep3gpp:remoteAddress               inet:ip-address

+--rw sepp3gpp:SEPPFunction* [id]

|  +--rw sepp3gpp:id            string

|  +--rw sepp3gpp:attributes

|  |  +--rw sepp3gpp:userLabel?           string

|  |  +--rw sepp3gpp:vnfParametersList!

|  |  |  +--rw sepp3gpp:vnfInstanceId    string

|  |  |  +--rw sepp3gpp:vnfdId?          string

|  |  |  +--rw sepp3gpp:flavourId?       string

|  |  |  +--rw sepp3gpp:autoScalable     boolean

|  |  +--rw sepp3gpp:peeParametersList!

|  |  |  +--rw sepp3gpp:siteIdentification    string

|  |  |  +--rw sepp3gpp:siteLatitude?         decimal64

|  |  |  +--rw sepp3gpp:siteLongitude?        decimal64

|  |  |  +--rw sepp3gpp:siteDescription       string

|  |  |  +--rw sepp3gpp:equipmentType         string

|  |  |  +--rw sepp3gpp:environmentType       string

|  |  |  +--rw sepp3gpp:powerInterface        string

|  |  +--rw sepp3gpp:priorityLabel        uint32

|  |  +--rw sepp3gpp:pLMNIdList* [mcc mnc]

|  |     +--rw sepp3gpp:mcc    Mcc

|  |     +--rw sepp3gpp:mnc    Mnc

|  +--rw cep3gpp:EP_N32* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_IPX* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw gnbcuup3gpp:GNBCUUPFunction* [id]

|  +--rw gnbcuup3gpp:id            string

|  +--rw gnbcuup3gpp:attributes

|  |  +--rw gnbcuup3gpp:userLabel?           string

|  |  +--rw gnbcuup3gpp:vnfParametersList!

|  |  |  +--rw gnbcuup3gpp:vnfInstanceId    string

|  |  |  +--rw gnbcuup3gpp:vnfdId?          string

|  |  |  +--rw gnbcuup3gpp:flavourId?       string

|  |  |  +--rw gnbcuup3gpp:autoScalable     boolean

|  |  +--rw gnbcuup3gpp:peeParametersList!

|  |  |  +--rw gnbcuup3gpp:siteIdentification    string

|  |  |  +--rw gnbcuup3gpp:siteLatitude?         decimal64

|  |  |  +--rw gnbcuup3gpp:siteLongitude?        decimal64

|  |  |  +--rw gnbcuup3gpp:siteDescription       string

|  |  |  +--rw gnbcuup3gpp:equipmentType         string

|  |  |  +--rw gnbcuup3gpp:environmentType       string

|  |  |  +--rw gnbcuup3gpp:powerInterface        string

|  |  +--rw gnbcuup3gpp:priorityLabel        uint32

|  |  +--rw gnbcuup3gpp:gNBId                int64

|  |  +--rw gnbcuup3gpp:gNBIdLength          int32

|  |  +--rw gnbcuup3gpp:pLMNIdList* [mcc mnc]

|  |     +--rw gnbcuup3gpp:mcc    Mcc

|  |     +--rw gnbcuup3gpp:mnc    Mnc

|  +--rw ep3gpp:EP_E1* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_F1U* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_NgU* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_XnU* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_X2U* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_S1U* [id]

|     +--rw ep3gpp:id            string

|     +--rw ep3gpp:attributes

|        +--rw ep3gpp:userLabel?                  string

|        +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro ep3gpp:measurementType    string

|        |  +--ro ep3gpp:supportedGPs*      uint32

|        +--rw ep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw ep3gpp:ipAddress    inet:ip-address

|        |  +--rw ep3gpp:vlanId       uint16

|        +--rw ep3gpp:remoteAddress               inet:ip-address

+--rw nwdaf3gpp:NWDAFFunction* [id]

|  +--rw nwdaf3gpp:id            string

|  +--rw nwdaf3gpp:attributes

|  |  +--rw nwdaf3gpp:userLabel?           string

|  |  +--rw nwdaf3gpp:vnfParametersList!

|  |  |  +--rw nwdaf3gpp:vnfInstanceId    string

|  |  |  +--rw nwdaf3gpp:vnfdId?          string

|  |  |  +--rw nwdaf3gpp:flavourId?       string

|  |  |  +--rw nwdaf3gpp:autoScalable     boolean

|  |  +--rw nwdaf3gpp:peeParametersList!

|  |  |  +--rw nwdaf3gpp:siteIdentification    string

|  |  |  +--rw nwdaf3gpp:siteLatitude?         decimal64

|  |  |  +--rw nwdaf3gpp:siteLongitude?        decimal64

|  |  |  +--rw nwdaf3gpp:siteDescription       string

|  |  |  +--rw nwdaf3gpp:equipmentType         string

|  |  |  +--rw nwdaf3gpp:environmentType       string

|  |  |  +--rw nwdaf3gpp:powerInterface        string

|  |  +--rw nwdaf3gpp:priorityLabel        uint32

|  |  +--rw nwdaf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw nwdaf3gpp:mcc    Mcc

|  |  |  +--rw nwdaf3gpp:mnc    Mnc

|  |  +--rw nwdaf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro nwdaf3gpp:sBIServiceList*      string

|  |  +--rw nwdaf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw smsf3gpp:SMSFFunction* [id]

|  +--rw smsf3gpp:id            string

|  +--rw smsf3gpp:attributes

|  |  +--rw smsf3gpp:userLabel?           string

|  |  +--rw smsf3gpp:vnfParametersList!

|  |  |  +--rw smsf3gpp:vnfInstanceId    string

|  |  |  +--rw smsf3gpp:vnfdId?          string

|  |  |  +--rw smsf3gpp:flavourId?       string

|  |  |  +--rw smsf3gpp:autoScalable     boolean

|  |  +--rw smsf3gpp:peeParametersList!

|  |  |  +--rw smsf3gpp:siteIdentification    string

|  |  |  +--rw smsf3gpp:siteLatitude?         decimal64

|  |  |  +--rw smsf3gpp:siteLongitude?        decimal64

|  |  |  +--rw smsf3gpp:siteDescription       string

|  |  |  +--rw smsf3gpp:equipmentType         string

|  |  |  +--rw smsf3gpp:environmentType       string

|  |  |  +--rw smsf3gpp:powerInterface        string

|  |  +--rw smsf3gpp:priorityLabel        uint32

|  |  +--rw smsf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw smsf3gpp:mcc    Mcc

|  |  |  +--rw smsf3gpp:mnc    Mnc

|  |  +--rw smsf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro smsf3gpp:sBIServiceList*      string

|  +--rw cep3gpp:EP_N20* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N21* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_MAP_SMSC* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw udsf3gpp:UDSFFunction* [id]

|  +--rw udsf3gpp:id            string

|  +--rw udsf3gpp:attributes

|  |  +--rw udsf3gpp:userLabel?           string

|  |  +--rw udsf3gpp:vnfParametersList!

|  |  |  +--rw udsf3gpp:vnfInstanceId    string

|  |  |  +--rw udsf3gpp:vnfdId?          string

|  |  |  +--rw udsf3gpp:flavourId?       string

|  |  |  +--rw udsf3gpp:autoScalable     boolean

|  |  +--rw udsf3gpp:peeParametersList!

|  |  |  +--rw udsf3gpp:siteIdentification    string

|  |  |  +--rw udsf3gpp:siteLatitude?         decimal64

|  |  |  +--rw udsf3gpp:siteLongitude?        decimal64

|  |  |  +--rw udsf3gpp:siteDescription       string

|  |  |  +--rw udsf3gpp:equipmentType         string

|  |  |  +--rw udsf3gpp:environmentType       string

|  |  |  +--rw udsf3gpp:powerInterface        string

|  |  +--rw udsf3gpp:priorityLabel        uint32

|  |  +--rw udsf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw udsf3gpp:mcc    Mcc

|  |  |  +--rw udsf3gpp:mnc    Mnc

|  |  +--rw udsf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro udsf3gpp:sBIServiceList*      string

|  |  +--rw udsf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw dn3gpp:DNFunction* [id]

|  +--rw dn3gpp:id            string

|  +--rw dn3gpp:attributes

|  |  +--rw dn3gpp:userLabel?           string

|  |  +--rw dn3gpp:vnfParametersList!

|  |  |  +--rw dn3gpp:vnfInstanceId    string

|  |  |  +--rw dn3gpp:vnfdId?          string

|  |  |  +--rw dn3gpp:flavourId?       string

|  |  |  +--rw dn3gpp:autoScalable     boolean

|  |  +--rw dn3gpp:peeParametersList!

|  |  |  +--rw dn3gpp:siteIdentification    string

|  |  |  +--rw dn3gpp:siteLatitude?         decimal64

|  |  |  +--rw dn3gpp:siteLongitude?        decimal64

|  |  |  +--rw dn3gpp:siteDescription       string

|  |  |  +--rw dn3gpp:equipmentType         string

|  |  |  +--rw dn3gpp:environmentType       string

|  |  |  +--rw dn3gpp:powerInterface        string

|  |  +--rw dn3gpp:priorityLabel        uint32

|  +--rw cep3gpp:EP_N6* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw nssf3gpp:NSSFFunction* [id]

|  +--rw nssf3gpp:id            string

|  +--rw nssf3gpp:attributes

|  |  +--rw nssf3gpp:userLabel?           string

|  |  +--rw nssf3gpp:vnfParametersList!

|  |  |  +--rw nssf3gpp:vnfInstanceId    string

|  |  |  +--rw nssf3gpp:vnfdId?          string

|  |  |  +--rw nssf3gpp:flavourId?       string

|  |  |  +--rw nssf3gpp:autoScalable     boolean

|  |  +--rw nssf3gpp:peeParametersList!

|  |  |  +--rw nssf3gpp:siteIdentification    string

|  |  |  +--rw nssf3gpp:siteLatitude?         decimal64

|  |  |  +--rw nssf3gpp:siteLongitude?        decimal64

|  |  |  +--rw nssf3gpp:siteDescription       string

|  |  |  +--rw nssf3gpp:equipmentType         string

|  |  |  +--rw nssf3gpp:environmentType       string

|  |  |  +--rw nssf3gpp:powerInterface        string

|  |  +--rw nssf3gpp:priorityLabel        uint32

|  |  +--rw nssf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw nssf3gpp:mcc    Mcc

|  |  |  +--rw nssf3gpp:mnc    Mnc

|  |  +--rw nssf3gpp:sBIFQDN?             inet:domain-name

|  |  +--rw nssf3gpp:sNSSAIList*          types3gpp:SNssai

|  |  +--rw nssf3gpp:nSIIdListWrap*       types3gpp:NsiId

|  +--rw cep3gpp:EP_N22* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N31* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw ausf3gpp:AUSFFunction* [id]

|  +--rw ausf3gpp:id            string

|  +--rw ausf3gpp:attributes

|  |  +--rw ausf3gpp:userLabel?           string

|  |  +--rw ausf3gpp:vnfParametersList!

|  |  |  +--rw ausf3gpp:vnfInstanceId    string

|  |  |  +--rw ausf3gpp:vnfdId?          string

|  |  |  +--rw ausf3gpp:flavourId?       string

|  |  |  +--rw ausf3gpp:autoScalable     boolean

|  |  +--rw ausf3gpp:peeParametersList!

|  |  |  +--rw ausf3gpp:siteIdentification    string

|  |  |  +--rw ausf3gpp:siteLatitude?         decimal64

|  |  |  +--rw ausf3gpp:siteLongitude?        decimal64

|  |  |  +--rw ausf3gpp:siteDescription       string

|  |  |  +--rw ausf3gpp:equipmentType         string

|  |  |  +--rw ausf3gpp:environmentType       string

|  |  |  +--rw ausf3gpp:powerInterface        string

|  |  +--rw ausf3gpp:priorityLabel        uint32

|  |  +--rw ausf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw ausf3gpp:mcc    Mcc

|  |  |  +--rw ausf3gpp:mnc    Mnc

|  |  +--rw ausf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro ausf3gpp:sBIServiceList*      string

|  |  +--rw ausf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N12* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N13* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw upf3gpp:UPFFunction* [id]

|  +--rw upf3gpp:id            string

|  +--rw upf3gpp:attributes

|  |  +--rw upf3gpp:userLabel?           string

|  |  +--rw upf3gpp:vnfParametersList!

|  |  |  +--rw upf3gpp:vnfInstanceId    string

|  |  |  +--rw upf3gpp:vnfdId?          string

|  |  |  +--rw upf3gpp:flavourId?       string

|  |  |  +--rw upf3gpp:autoScalable     boolean

|  |  +--rw upf3gpp:peeParametersList!

|  |  |  +--rw upf3gpp:siteIdentification    string

|  |  |  +--rw upf3gpp:siteLatitude?         decimal64

|  |  |  +--rw upf3gpp:siteLongitude?        decimal64

|  |  |  +--rw upf3gpp:siteDescription       string

|  |  |  +--rw upf3gpp:equipmentType         string

|  |  |  +--rw upf3gpp:environmentType       string

|  |  |  +--rw upf3gpp:powerInterface        string

|  |  +--rw upf3gpp:priorityLabel        uint32

|  |  +--rw upf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw upf3gpp:mcc    Mcc

|  |  |  +--rw upf3gpp:mnc    Mnc

|  |  +--ro upf3gpp:nRTACList*           types3gpp:Tac

|  |  +--rw upf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N4* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N3* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N9* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_S5U* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  |     +--ro cep3gpp:sBIService*                 string

|  +--rw cep3gpp:EP_N6* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw lmf3gpp:LMFFunction* [id]

|  +--rw lmf3gpp:id            string

|  +--rw lmf3gpp:attributes

|  |  +--rw lmf3gpp:userLabel?           string

|  |  +--rw lmf3gpp:vnfParametersList!

|  |  |  +--rw lmf3gpp:vnfInstanceId    string

|  |  |  +--rw lmf3gpp:vnfdId?          string

|  |  |  +--rw lmf3gpp:flavourId?       string

|  |  |  +--rw lmf3gpp:autoScalable     boolean

|  |  +--rw lmf3gpp:peeParametersList!

|  |  |  +--rw lmf3gpp:siteIdentification    string

|  |  |  +--rw lmf3gpp:siteLatitude?         decimal64

|  |  |  +--rw lmf3gpp:siteLongitude?        decimal64

|  |  |  +--rw lmf3gpp:siteDescription       string

|  |  |  +--rw lmf3gpp:equipmentType         string

|  |  |  +--rw lmf3gpp:environmentType       string

|  |  |  +--rw lmf3gpp:powerInterface        string

|  |  +--rw lmf3gpp:priorityLabel        uint32

|  |  +--rw lmf3gpp:pLMNIdList* [mcc mnc]

|  |     +--rw lmf3gpp:mcc    Mcc

|  |     +--rw lmf3gpp:mnc    Mnc

|  +--rw cep3gpp:EP_NLS* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw amf3gpp:AMFFunction* [id]

|  +--rw amf3gpp:id            string

|  +--rw amf3gpp:attributes

|  |  +--rw amf3gpp:userLabel?           string

|  |  +--rw amf3gpp:vnfParametersList!

|  |  |  +--rw amf3gpp:vnfInstanceId    string

|  |  |  +--rw amf3gpp:vnfdId?          string

|  |  |  +--rw amf3gpp:flavourId?       string

|  |  |  +--rw amf3gpp:autoScalable     boolean

|  |  +--rw amf3gpp:peeParametersList!

|  |  |  +--rw amf3gpp:siteIdentification    string

|  |  |  +--rw amf3gpp:siteLatitude?         decimal64

|  |  |  +--rw amf3gpp:siteLongitude?        decimal64

|  |  |  +--rw amf3gpp:siteDescription       string

|  |  |  +--rw amf3gpp:equipmentType         string

|  |  |  +--rw amf3gpp:environmentType       string

|  |  |  +--rw amf3gpp:powerInterface        string

|  |  +--rw amf3gpp:priorityLabel        uint32

|  |  +--rw amf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw amf3gpp:mcc    Mcc

|  |  |  +--rw amf3gpp:mnc    Mnc

|  |  +--rw amf3gpp:aMFIdentifier!

|  |  |  +--rw amf3gpp:amfRegionId?   AmfRegionId

|  |  |  +--rw amf3gpp:amfSetId?      AmfSetId

|  |  |  +--rw amf3gpp:amfPointer?    AmfPointer

|  |  +--rw amf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro amf3gpp:sBIServiceList*      string

|  |  +--rw amf3gpp:weightFactor?        uint16

|  |  +--rw amf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N2* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N8* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N11* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N12* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N14* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N15* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N17* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N20* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N22* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N26* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_NLS* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_NLG* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw nrf3gpp:NRFFunction* [id]

|  +--rw nrf3gpp:id            string

|  +--rw nrf3gpp:attributes

|  |  +--rw nrf3gpp:userLabel?           string

|  |  +--rw nrf3gpp:vnfParametersList!

|  |  |  +--rw nrf3gpp:vnfInstanceId    string

|  |  |  +--rw nrf3gpp:vnfdId?          string

|  |  |  +--rw nrf3gpp:flavourId?       string

|  |  |  +--rw nrf3gpp:autoScalable     boolean

|  |  +--rw nrf3gpp:peeParametersList!

|  |  |  +--rw nrf3gpp:siteIdentification    string

|  |  |  +--rw nrf3gpp:siteLatitude?         decimal64

|  |  |  +--rw nrf3gpp:siteLongitude?        decimal64

|  |  |  +--rw nrf3gpp:siteDescription       string

|  |  |  +--rw nrf3gpp:equipmentType         string

|  |  |  +--rw nrf3gpp:environmentType       string

|  |  |  +--rw nrf3gpp:powerInterface        string

|  |  +--rw nrf3gpp:priorityLabel        uint32

|  |  +--rw nrf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw nrf3gpp:mcc    Mcc

|  |  |  +--rw nrf3gpp:mnc    Mnc

|  |  +--rw nrf3gpp:sBIFQDN?             inet:domain-name

|  |  +--rw nrf3gpp:nSIIdListWrap*       types3gpp:NsiId

|  |  +--rw nrf3gpp:sNSSAIList*          types3gpp:SNssai

|  |  +--rw nrf3gpp:nFProfileList* [nfInstanceID]

|  |     +--rw nrf3gpp:nfInstanceID                        string

|  |     +--rw nrf3gpp:nfType                              types3gpp:NfType

|  |     +--rw nrf3gpp:nfStatus                            NFStatus

|  |     +--rw nrf3gpp:heartBeatTimer?                     uint16

|  |     +--rw nrf3gpp:plmnList* [mcc mnc]

|  |     |  +--rw nrf3gpp:mcc    Mcc

|  |     |  +--rw nrf3gpp:mnc    Mnc

|  |     +--rw nrf3gpp:sNssais* [sst sd]

|  |     |  +--rw nrf3gpp:sst    uint32

|  |     |  +--rw nrf3gpp:sd     string

|  |     +--rw nrf3gpp:perPlmnSnssaiList* [idx]

|  |     |  +--rw nrf3gpp:idx           uint32

|  |     |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  +--rw nrf3gpp:sNssaiList* [sst sd]

|  |     |     +--rw nrf3gpp:sst    uint32

|  |     |     +--rw nrf3gpp:sd     string

|  |     +--rw nrf3gpp:nsiList*                            string

|  |     +--rw nrf3gpp:fqdn?                               inet:domain-name

|  |     +--rw nrf3gpp:interPlmnFqdn?                      inet:domain-name

|  |     +--rw nrf3gpp:ipv4Addresses*                      inet:ipv4-address

|  |     +--rw nrf3gpp:ipv6Addresses*                      inet:ipv6-address

|  |     +--rw nrf3gpp:allowedPlmns* [mcc mnc]

|  |     |  +--rw nrf3gpp:mcc    Mcc

|  |     |  +--rw nrf3gpp:mnc    Mnc

|  |     +--rw nrf3gpp:allowedNfTypes*                     types3gpp:NfType

|  |     +--rw nrf3gpp:allowedNfDomains*                   string

|  |     +--rw nrf3gpp:allowedNssais* [sst sd]

|  |     |  +--rw nrf3gpp:sst    uint32

|  |     |  +--rw nrf3gpp:sd     string

|  |     +--rw nrf3gpp:priority?                           uint16

|  |     +--rw nrf3gpp:capacity?                           uint16

|  |     +--rw nrf3gpp:load?                               types3gpp:Load

|  |     +--rw nrf3gpp:locality?                           string

|  |     +--rw nrf3gpp:nrfInfo* [idx]

|  |     |  +--rw nrf3gpp:idx               uint32

|  |     |  +--rw nrf3gpp:servedUdrInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID                      string

|  |     |  |  +--rw nrf3gpp:groupId?                          string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:gpsiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:externalGroupIdentifiersRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:supportedDataSets*                DataSetId

|  |     |  +--rw nrf3gpp:servedUdmInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID                      string

|  |     |  |  +--rw nrf3gpp:groupId?                          string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:gpsiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:externalGroupIdentifiersRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:routingIndicators*                string

|  |     |  +--rw nrf3gpp:servedAusfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:groupId?             string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:routingIndicators*   string

|  |     |  +--rw nrf3gpp:servedAmfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID            string

|  |     |  |  +--rw nrf3gpp:amfRegionId?            string

|  |     |  |  +--rw nrf3gpp:amfSetId?               string

|  |     |  |  +--rw nrf3gpp:guamiList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:amfId* [amfRegionId amfSetId amfPointer]

|  |     |  |  |     +--rw nrf3gpp:amfRegionId    AmfRegionId

|  |     |  |  |     +--rw nrf3gpp:amfSetId       AmfSetId

|  |     |  |  |     +--rw nrf3gpp:amfPointer     AmfPointer

|  |     |  |  +--rw nrf3gpp:taiList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tac?      types3gpp:Tac

|  |     |  |  +--rw nrf3gpp:taiRangeList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx             uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tacRangeList* [start end]

|  |     |  |  |     +--rw nrf3gpp:start      string

|  |     |  |  |     +--rw nrf3gpp:end        string

|  |     |  |  |     +--rw nrf3gpp:pattern?   string

|  |     |  |  +--rw nrf3gpp:backupInfoAmfFailure* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:amfId* [amfRegionId amfSetId amfPointer]

|  |     |  |  |     +--rw nrf3gpp:amfRegionId    AmfRegionId

|  |     |  |  |     +--rw nrf3gpp:amfSetId       AmfSetId

|  |     |  |  |     +--rw nrf3gpp:amfPointer     AmfPointer

|  |     |  |  +--rw nrf3gpp:backupInfoAmfRemoval* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:amfId* [amfRegionId amfSetId amfPointer]

|  |     |  |  |     +--rw nrf3gpp:amfRegionId    AmfRegionId

|  |     |  |  |     +--rw nrf3gpp:amfSetId       AmfSetId

|  |     |  |  |     +--rw nrf3gpp:amfPointer     AmfPointer

|  |     |  |  +--rw nrf3gpp:n2InterfaceAmfInfo* [idx]

|  |     |  |     +--rw nrf3gpp:idx                          uint32

|  |     |  |     +--rw (nrf3gpp:address)?

|  |     |  |     |  +--:(nrf3gpp:ipv4EndpointAddress)

|  |     |  |     |  |  +--rw nrf3gpp:ipv4EndpointAddress*   inet:ipv4-address

|  |     |  |     |  +--:(nrf3gpp:ipv6EndpointAddress)

|  |     |  |     |     +--rw nrf3gpp:ipv6EndpointAddress*   inet:ipv6-address

|  |     |  |     +--rw nrf3gpp:amfName?                     string

|  |     |  +--rw nrf3gpp:servedSmfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:sNssaiSmfInfoList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx               uint32

|  |     |  |  |  +--rw nrf3gpp:sNssai* [sst sd]

|  |     |  |  |  |  +--rw nrf3gpp:sst    uint32

|  |     |  |  |  |  +--rw nrf3gpp:sd     string

|  |     |  |  |  +--rw nrf3gpp:dnnSmfInfoList* [dnn]

|  |     |  |  |     +--rw nrf3gpp:dnn    string

|  |     |  |  +--rw nrf3gpp:taiList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tac?      types3gpp:Tac

|  |     |  |  +--rw nrf3gpp:taiRangeList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx             uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tacRangeList* [start end]

|  |     |  |  |     +--rw nrf3gpp:start      string

|  |     |  |  |     +--rw nrf3gpp:end        string

|  |     |  |  |     +--rw nrf3gpp:pattern?   string

|  |     |  |  +--rw nrf3gpp:pgwFqdn?             inet:domain-name

|  |     |  |  +--rw nrf3gpp:accessType*          AccessType

|  |     |  +--rw nrf3gpp:servedUpfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:sNssaiUpfInfoList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx               uint32

|  |     |  |  |  +--rw nrf3gpp:sNssai* [sst sd]

|  |     |  |  |  |  +--rw nrf3gpp:sst    uint32

|  |     |  |  |  |  +--rw nrf3gpp:sd     string

|  |     |  |  |  +--rw nrf3gpp:dnnUpfInfoList* [dnn]

|  |     |  |  |     +--rw nrf3gpp:dnn                string

|  |     |  |  |     +--rw nrf3gpp:dnaiList*          string

|  |     |  |  |     +--rw nrf3gpp:pduSessionTypes*   PduSessionType

|  |     |  |  +--rw nrf3gpp:smfServingArea*      string

|  |     |  |  +--rw nrf3gpp:interfaceUpfInfo* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx                            uint32

|  |     |  |  |  +--rw nrf3gpp:interfaceType                  UPInterfaceType

|  |     |  |  |  +--rw (nrf3gpp:address)?

|  |     |  |  |  |  +--:(nrf3gpp:ipv4EndpointAddresses)

|  |     |  |  |  |  |  +--rw nrf3gpp:ipv4EndpointAddresses*   inet:ipv4-address

|  |     |  |  |  |  +--:(nrf3gpp:ipv6EndpointAddresses)

|  |     |  |  |  |  |  +--rw nrf3gpp:ipv6EndpointAddresses*   inet:ipv6-address

|  |     |  |  |  |  +--:(nrf3gpp:endpointFqdn)

|  |     |  |  |  |     +--rw nrf3gpp:endpointFqdn?            inet:domain-name

|  |     |  |  |  +--rw nrf3gpp:networkInstance?               string

|  |     |  |  +--rw nrf3gpp:iwkEpsInd?           boolean

|  |     |  |  +--rw nrf3gpp:pduSessionTypes*     PduSessionType

|  |     |  +--rw nrf3gpp:servedPcfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID    string

|  |     |  |  +--rw nrf3gpp:dnnList*        string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:rxDiamHost?     string

|  |     |  |  +--rw nrf3gpp:rxDiamRealm?    string

|  |     |  +--rw nrf3gpp:servedBsfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:ipv4AddressRanges* [start end]

|  |     |  |  |  +--rw nrf3gpp:start    inet:ipv4-address

|  |     |  |  |  +--rw nrf3gpp:end      inet:ipv4-address

|  |     |  |  +--rw nrf3gpp:dnnList*             string

|  |     |  |  +--rw nrf3gpp:ipDomainList*        string

|  |     |  |  +--rw nrf3gpp:ipv6PrefixRanges* [start end]

|  |     |  |     +--rw nrf3gpp:start    inet:ipv6-prefix

|  |     |  |     +--rw nrf3gpp:end      inet:ipv6-prefix

|  |     |  +--rw nrf3gpp:servedChfInfo* [nfInstanceID]

|  |     |     +--rw nrf3gpp:nfInstanceID     string

|  |     |     +--rw nrf3gpp:supiRangeList* [start end pattern]

|  |     |     |  +--rw nrf3gpp:start      string

|  |     |     |  +--rw nrf3gpp:end        string

|  |     |     |  +--rw nrf3gpp:pattern    string

|  |     |     +--rw nrf3gpp:gpsiRangeList* [start end pattern]

|  |     |     |  +--rw nrf3gpp:start      string

|  |     |     |  +--rw nrf3gpp:end        string

|  |     |     |  +--rw nrf3gpp:pattern    string

|  |     |     +--rw nrf3gpp:plmnRangeList* [mcc mnc]

|  |     |        +--rw nrf3gpp:mcc    Mcc

|  |     |        +--rw nrf3gpp:mnc    Mnc

|  |     +--rw nrf3gpp:customInfo?                         string

|  |     +--rw nrf3gpp:recoveryTime?                       yang:date-and-time

|  |     +--rw nrf3gpp:nfServicePersistence?               boolean

|  |     +--rw nrf3gpp:nfServices* [serviceInstanceID]

|  |     |  +--rw nrf3gpp:serviceInstanceID                   string

|  |     |  +--rw nrf3gpp:serviceName                         ServiceName

|  |     |  +--rw nrf3gpp:versions* [apiVersionInUri apiFullVersion]

|  |     |  |  +--rw nrf3gpp:apiVersionInUri    string

|  |     |  |  +--rw nrf3gpp:apiFullVersion     string

|  |     |  |  +--rw nrf3gpp:expiry?            yang:date-and-time

|  |     |  +--rw nrf3gpp:scheme                              UriScheme

|  |     |  +--rw nrf3gpp:nfServiceStatus                     NFServiceStatus

|  |     |  +--rw nrf3gpp:fqdn?                               inet:domain-name

|  |     |  +--rw nrf3gpp:interPlmnFqdn?                      inet:domain-name

|  |     |  +--rw nrf3gpp:ipEndPoints* [idx]

|  |     |  |  +--rw nrf3gpp:idx                  string

|  |     |  |  +--rw (nrf3gpp:address)?

|  |     |  |  |  +--:(nrf3gpp:ipv4Address)

|  |     |  |  |  |  +--rw nrf3gpp:ipv4Address?   inet:ipv4-address

|  |     |  |  |  +--:(nrf3gpp:ipv6Address)

|  |     |  |  |  |  +--rw nrf3gpp:ipv6Address?   inet:ipv6-address

|  |     |  |  |  +--:(nrf3gpp:ipv6Prefix)

|  |     |  |  |     +--rw nrf3gpp:ipv6Prefix?    inet:ipv6-prefix

|  |     |  |  +--rw nrf3gpp:transport?           TransportProtocol

|  |     |  |  +--rw nrf3gpp:port?                uint16

|  |     |  +--rw nrf3gpp:apiPrefix?                          string

|  |     |  +--rw nrf3gpp:defaultNotificationSubscriptions* [notificationType]

|  |     |  |  +--rw nrf3gpp:notificationType      NotificationType

|  |     |  |  +--rw nrf3gpp:callbackUri?          inet:uri

|  |     |  |  +--rw nrf3gpp:n1MessageClass?       N1MessageClass

|  |     |  |  +--rw nrf3gpp:n2InformationClass?   N2InformationClass

|  |     |  +--rw nrf3gpp:allowedPlmns* [mcc mnc]

|  |     |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  +--rw nrf3gpp:allowedNfTypes*                     types3gpp:NfType

|  |     |  +--rw nrf3gpp:allowedNfDomains*                   string

|  |     |  +--rw nrf3gpp:allowedNssais*                      types3gpp:SNssai

|  |     |  +--rw nrf3gpp:priority?                           uint16

|  |     |  +--rw nrf3gpp:capacity?                           uint16

|  |     |  +--rw nrf3gpp:load?                               types3gpp:Load

|  |     |  +--rw nrf3gpp:recoveryTime?                       yang:date-and-time

|  |     |  +--rw nrf3gpp:chfServiceInfo* [primaryChfServiceInstance secondaryChfServiceInstance]

|  |     |  |  +--rw nrf3gpp:primaryChfServiceInstance      string

|  |     |  |  +--rw nrf3gpp:secondaryChfServiceInstance    string

|  |     |  +--rw nrf3gpp:supportedFeatures?                  SupportedFeatures

|  |     +--rw nrf3gpp:nfProfileChangesSupportInd?         boolean

|  |     +--rw nrf3gpp:nfProfileChangesInd?                boolean

|  |     +--rw nrf3gpp:defaultNotificationSubscriptions* [notificationType]

|  |        +--rw nrf3gpp:notificationType      NotificationType

|  |        +--rw nrf3gpp:callbackUri?          inet:uri

|  |        +--rw nrf3gpp:n1MessageClass?       N1MessageClass

|  |        +--rw nrf3gpp:n2InformationClass?   N2InformationClass

|  +--rw cep3gpp:EP_N27* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw n3iwf3gpp:N3IWFFunction* [id]

+--rw n3iwf3gpp:id            string

+--rw n3iwf3gpp:attributes

|  +--rw n3iwf3gpp:userLabel?           string

|  +--rw n3iwf3gpp:vnfParametersList!

|  |  +--rw n3iwf3gpp:vnfInstanceId    string

|  |  +--rw n3iwf3gpp:vnfdId?          string

|  |  +--rw n3iwf3gpp:flavourId?       string

|  |  +--rw n3iwf3gpp:autoScalable     boolean

|  +--rw n3iwf3gpp:peeParametersList!

|  |  +--rw n3iwf3gpp:siteIdentification    string

|  |  +--rw n3iwf3gpp:siteLatitude?         decimal64

|  |  +--rw n3iwf3gpp:siteLongitude?        decimal64

|  |  +--rw n3iwf3gpp:siteDescription       string

|  |  +--rw n3iwf3gpp:equipmentType         string

|  |  +--rw n3iwf3gpp:environmentType       string

|  |  +--rw n3iwf3gpp:powerInterface        string

|  +--rw n3iwf3gpp:priorityLabel        uint32

|  +--rw n3iwf3gpp:pLMNIdList* [mcc mnc]

|     +--rw n3iwf3gpp:mcc    Mcc

|     +--rw n3iwf3gpp:mnc    Mnc

+--rw cep3gpp:EP_N2* [id]

|  +--rw cep3gpp:id            string

|  +--rw cep3gpp:attributes

|     +--rw cep3gpp:userLabel?                  string

|     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|     |  +--ro cep3gpp:measurementType    string

|     |  +--ro cep3gpp:supportedGPs*      uint32

|     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|     |  +--rw cep3gpp:ipAddress    inet:ip-address

|     |  +--rw cep3gpp:vlanId       uint16

|     +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw cep3gpp:EP_N3* [id]

+--rw cep3gpp:id            string

+--rw cep3gpp:attributes

+--rw cep3gpp:userLabel?                  string

+--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

+--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  +--ro cep3gpp:measurementType    string

|  +--ro cep3gpp:supportedGPs*      uint32

+--rw cep3gpp:localAddress* [ipAddress vlanId]

|  +--rw cep3gpp:ipAddress    inet:ip-address

|  +--rw cep3gpp:vlanId       uint16

+--rw cep3gpp:remoteAddress               inet:ip-address

module: _3gpp-common-subnetwork

+--rw SubNetwork* [id]

+--rw id                                         string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--rw userDefinedNetworkType?     string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                   types3gpp:Mcc

|  +--rw priorityLabel               uint32

|  +--rw parents*                    -> ../../../SubNetwork/id

|  +--rw containedChildren*          -> ../../../SubNetwork/id

+--rw MeasurementControl* [id] {MeasurementsUnderSubNetwork}?

|  +--rw id                   string

|  +--rw attributes

|  |  +--rw pMAdministrativeState?        types3gpp:AdministrativeState

|  |  +--ro pMOperationalState            types3gpp:OperationalState

|  |  +--rw defaultFileLocation?          string

|  |  +--rw defaultFileBasedGP            uint32

|  |  +--rw defaultFileReportingPeriod    uint32

|  |  +--rw defaultStreamTarget?          string {StreamingSupported}?

|  |  +--rw defaultStreamBasedGP          uint32 {StreamingSupported}?

|  +--rw MeasurementReader* [id]

|     +--rw id            string

|     +--rw attributes

|        +--rw measurementTypes*        string

|        +--rw managedObjectDNsBasic*   types3gpp:DistinguishedName

|        +--rw managedObjectDNs*        types3gpp:DistinguishedName

|        +--rw fileLocation?            string

|        +--rw fileBasedGP              uint32

|        +--rw fileReportingPeriod      uint32

|        +--rw streamTarget?            string

|        +--rw streamBasedGP            uint32

+--rw extservgw3gpp:ExternalServingGWFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extservgw3gpp:id            string

|  +--rw extservgw3gpp:attributes

|     +--rw extservgw3gpp:userLabel?           string

|     +--rw extservgw3gpp:vnfParametersList!

|     |  +--rw extservgw3gpp:vnfInstanceId    string

|     |  +--rw extservgw3gpp:vnfdId?          string

|     |  +--rw extservgw3gpp:flavourId?       string

|     |  +--rw extservgw3gpp:autoScalable     boolean

|     +--rw extservgw3gpp:peeParametersList!

|     |  +--rw extservgw3gpp:siteIdentification    string

|     |  +--rw extservgw3gpp:siteLatitude?         decimal64

|     |  +--rw extservgw3gpp:siteLongitude?        decimal64

|     |  +--rw extservgw3gpp:siteDescription       string

|     |  +--rw extservgw3gpp:equipmentType         string

|     |  +--rw extservgw3gpp:environmentType       string

|     |  +--rw extservgw3gpp:powerInterface        string

|     +--rw extservgw3gpp:priorityLabel        uint32

+--rw extenb3gpp:ExternalENBFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extenb3gpp:id                              string

|  +--rw extenb3gpp:attributes

|  |  +--rw extenb3gpp:userLabel?           string

|  |  +--rw extenb3gpp:vnfParametersList!

|  |  |  +--rw extenb3gpp:vnfInstanceId    string

|  |  |  +--rw extenb3gpp:vnfdId?          string

|  |  |  +--rw extenb3gpp:flavourId?       string

|  |  |  +--rw extenb3gpp:autoScalable     boolean

|  |  +--rw extenb3gpp:peeParametersList!

|  |  |  +--rw extenb3gpp:siteIdentification    string

|  |  |  +--rw extenb3gpp:siteLatitude?         decimal64

|  |  |  +--rw extenb3gpp:siteLongitude?        decimal64

|  |  |  +--rw extenb3gpp:siteDescription       string

|  |  |  +--rw extenb3gpp:equipmentType         string

|  |  |  +--rw extenb3gpp:environmentType       string

|  |  |  +--rw extenb3gpp:powerInterface        string

|  |  +--rw extenb3gpp:priorityLabel        uint32

|  |  +--rw extenb3gpp:eNBId                int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellFDD* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  |  +--rw exteutrancell3gpp:id            string

|  |  +--rw exteutrancell3gpp:attributes

|  |     +--rw exteutrancell3gpp:userLabel?           string

|  |     +--rw exteutrancell3gpp:vnfParametersList!

|  |     |  +--rw exteutrancell3gpp:vnfInstanceId    string

|  |     |  +--rw exteutrancell3gpp:vnfdId?          string

|  |     |  +--rw exteutrancell3gpp:flavourId?       string

|  |     |  +--rw exteutrancell3gpp:autoScalable     boolean

|  |     +--rw exteutrancell3gpp:peeParametersList!

|  |     |  +--rw exteutrancell3gpp:siteIdentification    string

|  |     |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|  |     |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|  |     |  +--rw exteutrancell3gpp:siteDescription       string

|  |     |  +--rw exteutrancell3gpp:equipmentType         string

|  |     |  +--rw exteutrancell3gpp:environmentType       string

|  |     |  +--rw exteutrancell3gpp:powerInterface        string

|  |     +--rw exteutrancell3gpp:priorityLabel        uint32

|  |     +--rw exteutrancell3gpp:pci                  int32

|  |     +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|  |     |  +--rw exteutrancell3gpp:mcc    Mcc

|  |     |  +--rw exteutrancell3gpp:mnc    Mnc

|  |     +--rw exteutrancell3gpp:cellLocalId?         int32

|  |     +--rw exteutrancell3gpp:eNBId                int32

|  |     +--rw exteutrancell3gpp:earfcnDL             int32

|  |     +--rw exteutrancell3gpp:earfcnUL             int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellTDD* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|     +--rw exteutrancell3gpp:id            string

|     +--rw exteutrancell3gpp:attributes

|        +--rw exteutrancell3gpp:userLabel?           string

|        +--rw exteutrancell3gpp:vnfParametersList!

|        |  +--rw exteutrancell3gpp:vnfInstanceId    string

|        |  +--rw exteutrancell3gpp:vnfdId?          string

|        |  +--rw exteutrancell3gpp:flavourId?       string

|        |  +--rw exteutrancell3gpp:autoScalable     boolean

|        +--rw exteutrancell3gpp:peeParametersList!

|        |  +--rw exteutrancell3gpp:siteIdentification    string

|        |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|        |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|        |  +--rw exteutrancell3gpp:siteDescription       string

|        |  +--rw exteutrancell3gpp:equipmentType         string

|        |  +--rw exteutrancell3gpp:environmentType       string

|        |  +--rw exteutrancell3gpp:powerInterface        string

|        +--rw exteutrancell3gpp:priorityLabel        uint32

|        +--rw exteutrancell3gpp:pci                  int32

|        +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|        |  +--rw exteutrancell3gpp:mcc    Mcc

|        |  +--rw exteutrancell3gpp:mnc    Mnc

|        +--rw exteutrancell3gpp:cellLocalId?         int32

|        +--rw exteutrancell3gpp:eNBId                int32

|        +--rw exteutrancell3gpp:earfcn               int32

+--rw amfset3gpp:AMFSet* [id]

|  +--rw amfset3gpp:id            string

|  +--rw amfset3gpp:attributes

|     +--rw amfset3gpp:userLabel?           string

|     +--rw amfset3gpp:vnfParametersList!

|     |  +--rw amfset3gpp:vnfInstanceId    string

|     |  +--rw amfset3gpp:vnfdId?          string

|     |  +--rw amfset3gpp:flavourId?       string

|     |  +--rw amfset3gpp:autoScalable     boolean

|     +--rw amfset3gpp:peeParametersList!

|     |  +--rw amfset3gpp:siteIdentification    string

|     |  +--rw amfset3gpp:siteLatitude?         decimal64

|     |  +--rw amfset3gpp:siteLongitude?        decimal64

|     |  +--rw amfset3gpp:siteDescription       string

|     |  +--rw amfset3gpp:equipmentType         string

|     |  +--rw amfset3gpp:environmentType       string

|     |  +--rw amfset3gpp:powerInterface        string

|     +--rw amfset3gpp:priorityLabel        uint32

|     +--rw amfset3gpp:pLMNIdList* [mcc mnc]

|     |  +--rw amfset3gpp:mcc    Mcc

|     |  +--rw amfset3gpp:mnc    Mnc

|     +--ro amfset3gpp:nRTACList*           types3gpp:Tac

|     +--rw amfset3gpp:sNSSAIList*          types3gpp:SNssai

|     +--rw amfset3gpp:aMFRegion?           instance-identifier

|     +--rw amfset3gpp:aMFSetMemberList*    types3gpp:DistinguishedName

+--rw extgnbcucp3gpp:ExternalGNBCUCPFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extgnbcucp3gpp:id                   string

|  +--rw extgnbcucp3gpp:attributes

|  |  +--rw extgnbcucp3gpp:userLabel?           string

|  |  +--rw extgnbcucp3gpp:vnfParametersList!

|  |  |  +--rw extgnbcucp3gpp:vnfInstanceId    string

|  |  |  +--rw extgnbcucp3gpp:vnfdId?          string

|  |  |  +--rw extgnbcucp3gpp:flavourId?       string

|  |  |  +--rw extgnbcucp3gpp:autoScalable     boolean

|  |  +--rw extgnbcucp3gpp:peeParametersList!

|  |  |  +--rw extgnbcucp3gpp:siteIdentification    string

|  |  |  +--rw extgnbcucp3gpp:siteLatitude?         decimal64

|  |  |  +--rw extgnbcucp3gpp:siteLongitude?        decimal64

|  |  |  +--rw extgnbcucp3gpp:siteDescription       string

|  |  |  +--rw extgnbcucp3gpp:equipmentType         string

|  |  |  +--rw extgnbcucp3gpp:environmentType       string

|  |  |  +--rw extgnbcucp3gpp:powerInterface        string

|  |  +--rw extgnbcucp3gpp:priorityLabel        uint32

|  |  +--rw extgnbcucp3gpp:gNBId                int64

|  |  +--rw extgnbcucp3gpp:gNBIdLength          int32

|  |  +--rw extgnbcucp3gpp:pLMNId* [mcc mnc]

|  |     +--rw extgnbcucp3gpp:mcc    Mcc

|  |     +--rw extgnbcucp3gpp:mnc    Mnc

|  +--rw extnrcellcu3gpp:ExternalNRCellCU* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|     +--rw extnrcellcu3gpp:id            string

|     +--rw extnrcellcu3gpp:attributes

|        +--rw extnrcellcu3gpp:userLabel?           string

|        +--rw extnrcellcu3gpp:vnfParametersList!

|        |  +--rw extnrcellcu3gpp:vnfInstanceId    string

|        |  +--rw extnrcellcu3gpp:vnfdId?          string

|        |  +--rw extnrcellcu3gpp:flavourId?       string

|        |  +--rw extnrcellcu3gpp:autoScalable     boolean

|        +--rw extnrcellcu3gpp:peeParametersList!

|        |  +--rw extnrcellcu3gpp:siteIdentification    string

|        |  +--rw extnrcellcu3gpp:siteLatitude?         decimal64

|        |  +--rw extnrcellcu3gpp:siteLongitude?        decimal64

|        |  +--rw extnrcellcu3gpp:siteDescription       string

|        |  +--rw extnrcellcu3gpp:equipmentType         string

|        |  +--rw extnrcellcu3gpp:environmentType       string

|        |  +--rw extnrcellcu3gpp:powerInterface        string

|        +--rw extnrcellcu3gpp:priorityLabel        uint32

|        +--rw extnrcellcu3gpp:cellLocalId          int32

|        +--rw extnrcellcu3gpp:nRPCI                int32

|        +--rw extnrcellcu3gpp:pLMNIdList* [mcc mnc]

|        |  +--rw extnrcellcu3gpp:mcc    Mcc

|        |  +--rw extnrcellcu3gpp:mnc    Mnc

|        +--rw extnrcellcu3gpp:nRFrequencyRef       types3gpp:DistinguishedName

+--rw extnssf3gpp:ExternalNSSFFunction* [id]

|  +--rw extnssf3gpp:id            string

|  +--rw extnssf3gpp:attributes

|     +--rw extnssf3gpp:userLabel?           string

|     +--rw extnssf3gpp:vnfParametersList!

|     |  +--rw extnssf3gpp:vnfInstanceId    string

|     |  +--rw extnssf3gpp:vnfdId?          string

|     |  +--rw extnssf3gpp:flavourId?       string

|     |  +--rw extnssf3gpp:autoScalable     boolean

|     +--rw extnssf3gpp:peeParametersList!

|     |  +--rw extnssf3gpp:siteIdentification    string

|     |  +--rw extnssf3gpp:siteLatitude?         decimal64

|     |  +--rw extnssf3gpp:siteLongitude?        decimal64

|     |  +--rw extnssf3gpp:siteDescription       string

|     |  +--rw extnssf3gpp:equipmentType         string

|     |  +--rw extnssf3gpp:environmentType       string

|     |  +--rw extnssf3gpp:powerInterface        string

|     +--rw extnssf3gpp:priorityLabel        uint32

|     +--rw extnssf3gpp:pLMNIdList* [mcc mnc]

|        +--rw extnssf3gpp:mcc    Mcc

|        +--rw extnssf3gpp:mnc    Mnc

+--rw extgnbcuup3gpp:ExternalGNBCUUPFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extgnbcuup3gpp:id            string

|  +--rw extgnbcuup3gpp:attributes

|     +--rw extgnbcuup3gpp:userLabel?           string

|     +--rw extgnbcuup3gpp:vnfParametersList!

|     |  +--rw extgnbcuup3gpp:vnfInstanceId    string

|     |  +--rw extgnbcuup3gpp:vnfdId?          string

|     |  +--rw extgnbcuup3gpp:flavourId?       string

|     |  +--rw extgnbcuup3gpp:autoScalable     boolean

|     +--rw extgnbcuup3gpp:peeParametersList!

|     |  +--rw extgnbcuup3gpp:siteIdentification    string

|     |  +--rw extgnbcuup3gpp:siteLatitude?         decimal64

|     |  +--rw extgnbcuup3gpp:siteLongitude?        decimal64

|     |  +--rw extgnbcuup3gpp:siteDescription       string

|     |  +--rw extgnbcuup3gpp:equipmentType         string

|     |  +--rw extgnbcuup3gpp:environmentType       string

|     |  +--rw extgnbcuup3gpp:powerInterface        string

|     +--rw extgnbcuup3gpp:priorityLabel        uint32

|     +--rw extgnbcuup3gpp:gNBId                int64

|     +--rw extgnbcuup3gpp:gNBIdLength          int32

+--rw extnrf3gpp:ExternalNRFFunction* [id]

|  +--rw extnrf3gpp:id            string

|  +--rw extnrf3gpp:attributes

|     +--rw extnrf3gpp:userLabel?           string

|     +--rw extnrf3gpp:vnfParametersList!

|     |  +--rw extnrf3gpp:vnfInstanceId    string

|     |  +--rw extnrf3gpp:vnfdId?          string

|     |  +--rw extnrf3gpp:flavourId?       string

|     |  +--rw extnrf3gpp:autoScalable     boolean

|     +--rw extnrf3gpp:peeParametersList!

|     |  +--rw extnrf3gpp:siteIdentification    string

|     |  +--rw extnrf3gpp:siteLatitude?         decimal64

|     |  +--rw extnrf3gpp:siteLongitude?        decimal64

|     |  +--rw extnrf3gpp:siteDescription       string

|     |  +--rw extnrf3gpp:equipmentType         string

|     |  +--rw extnrf3gpp:environmentType       string

|     |  +--rw extnrf3gpp:powerInterface        string

|     +--rw extnrf3gpp:priorityLabel        uint32

|     +--rw extnrf3gpp:pLMNIdList* [mcc mnc]

|        +--rw extnrf3gpp:mcc    Mcc

|        +--rw extnrf3gpp:mnc    Mnc

+--rw amfr3gpp:AMFRegion* [id]

|  +--rw amfr3gpp:id            string

|  +--rw amfr3gpp:attributes

|     +--rw amfr3gpp:userLabel?           string

|     +--rw amfr3gpp:vnfParametersList!

|     |  +--rw amfr3gpp:vnfInstanceId    string

|     |  +--rw amfr3gpp:vnfdId?          string

|     |  +--rw amfr3gpp:flavourId?       string

|     |  +--rw amfr3gpp:autoScalable     boolean

|     +--rw amfr3gpp:peeParametersList!

|     |  +--rw amfr3gpp:siteIdentification    string

|     |  +--rw amfr3gpp:siteLatitude?         decimal64

|     |  +--rw amfr3gpp:siteLongitude?        decimal64

|     |  +--rw amfr3gpp:siteDescription       string

|     |  +--rw amfr3gpp:equipmentType         string

|     |  +--rw amfr3gpp:environmentType       string

|     |  +--rw amfr3gpp:powerInterface        string

|     +--rw amfr3gpp:priorityLabel        uint32

|     +--rw amfr3gpp:pLMNIdList* [mcc mnc]

|     |  +--rw amfr3gpp:mcc    Mcc

|     |  +--rw amfr3gpp:mnc    Mnc

|     +--ro amfr3gpp:nRTACList*           types3gpp:Tac

|     +--rw amfr3gpp:sNSSAIList*          types3gpp:SNssai

|     +--rw amfr3gpp:aMFRegionId          types3gpp:AmfRegionId

|     +--rw amfr3gpp:aMFSet*              instance-identifier

+--rw extupf3gpp:ExternalUPFFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extupf3gpp:id            string

|  +--rw extupf3gpp:attributes

|     +--rw extupf3gpp:userLabel?           string

|     +--rw extupf3gpp:vnfParametersList!

|     |  +--rw extupf3gpp:vnfInstanceId    string

|     |  +--rw extupf3gpp:vnfdId?          string

|     |  +--rw extupf3gpp:flavourId?       string

|     |  +--rw extupf3gpp:autoScalable     boolean

|     +--rw extupf3gpp:peeParametersList!

|     |  +--rw extupf3gpp:siteIdentification    string

|     |  +--rw extupf3gpp:siteLatitude?         decimal64

|     |  +--rw extupf3gpp:siteLongitude?        decimal64

|     |  +--rw extupf3gpp:siteDescription       string

|     |  +--rw extupf3gpp:equipmentType         string

|     |  +--rw extupf3gpp:environmentType       string

|     |  +--rw extupf3gpp:powerInterface        string

|     +--rw extupf3gpp:priorityLabel        uint32

+--rw nrfreq3gpp:NRFrequency* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw nrfreq3gpp:id            string

|  +--rw nrfreq3gpp:attributes

|     +--rw nrfreq3gpp:userLabel?                  string

|     +--rw nrfreq3gpp:vnfParametersList!

|     |  +--rw nrfreq3gpp:vnfInstanceId    string

|     |  +--rw nrfreq3gpp:vnfdId?          string

|     |  +--rw nrfreq3gpp:flavourId?       string

|     |  +--rw nrfreq3gpp:autoScalable     boolean

|     +--rw nrfreq3gpp:peeParametersList!

|     |  +--rw nrfreq3gpp:siteIdentification    string

|     |  +--rw nrfreq3gpp:siteLatitude?         decimal64

|     |  +--rw nrfreq3gpp:siteLongitude?        decimal64

|     |  +--rw nrfreq3gpp:siteDescription       string

|     |  +--rw nrfreq3gpp:equipmentType         string

|     |  +--rw nrfreq3gpp:environmentType       string

|     |  +--rw nrfreq3gpp:powerInterface        string

|     +--rw nrfreq3gpp:priorityLabel               uint32

|     +--rw nrfreq3gpp:absoluteFrequencySSB        uint32

|     +--rw nrfreq3gpp:sSBSubCarrierSpacing        uint8

|     +--ro nrfreq3gpp:multiFrequencyBandListNR*   uint16

+--rw eutraneteutranfreq3gpp:EUtranFrequency* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw eutraneteutranfreq3gpp:id            string

|  +--rw eutraneteutranfreq3gpp:attributes

|     +--rw eutraneteutranfreq3gpp:userLabel?                string

|     +--rw eutraneteutranfreq3gpp:vnfParametersList!

|     |  +--rw eutraneteutranfreq3gpp:vnfInstanceId    string

|     |  +--rw eutraneteutranfreq3gpp:vnfdId?          string

|     |  +--rw eutraneteutranfreq3gpp:flavourId?       string

|     |  +--rw eutraneteutranfreq3gpp:autoScalable     boolean

|     +--rw eutraneteutranfreq3gpp:peeParametersList!

|     |  +--rw eutraneteutranfreq3gpp:siteIdentification    string

|     |  +--rw eutraneteutranfreq3gpp:siteLatitude?         decimal64

|     |  +--rw eutraneteutranfreq3gpp:siteLongitude?        decimal64

|     |  +--rw eutraneteutranfreq3gpp:siteDescription       string

|     |  +--rw eutraneteutranfreq3gpp:equipmentType         string

|     |  +--rw eutraneteutranfreq3gpp:environmentType       string

|     |  +--rw eutraneteutranfreq3gpp:powerInterface        string

|     +--rw eutraneteutranfreq3gpp:priorityLabel             uint32

|     +--rw eutraneteutranfreq3gpp:earfcnDL                  uint32

|     +--ro eutraneteutranfreq3gpp:multiBandInfoListEutra*   uint16

+--rw extamf3gpp:ExternalAMFFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extamf3gpp:id            string

|  +--rw extamf3gpp:attributes

|     +--rw extamf3gpp:userLabel?           string

|     +--rw extamf3gpp:vnfParametersList!

|     |  +--rw extamf3gpp:vnfInstanceId    string

|     |  +--rw extamf3gpp:vnfdId?          string

|     |  +--rw extamf3gpp:flavourId?       string

|     |  +--rw extamf3gpp:autoScalable     boolean

|     +--rw extamf3gpp:peeParametersList!

|     |  +--rw extamf3gpp:siteIdentification    string

|     |  +--rw extamf3gpp:siteLatitude?         decimal64

|     |  +--rw extamf3gpp:siteLongitude?        decimal64

|     |  +--rw extamf3gpp:siteDescription       string

|     |  +--rw extamf3gpp:equipmentType         string

|     |  +--rw extamf3gpp:environmentType       string

|     |  +--rw extamf3gpp:powerInterface        string

|     +--rw extamf3gpp:priorityLabel        uint32

|     +--rw extamf3gpp:pLMNIdList* [mcc mnc]

|     |  +--rw extamf3gpp:mcc    Mcc

|     |  +--rw extamf3gpp:mnc    Mnc

|     +--rw extamf3gpp:aMFIdentifier!

|        +--rw extamf3gpp:amfRegionId?   AmfRegionId

|        +--rw extamf3gpp:amfSetId?      AmfSetId

|        +--rw extamf3gpp:amfPointer?    AmfPointer

+--rw extgnbdu3gpp:ExternalGNBDUFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

+--rw extgnbdu3gpp:id            string

+--rw extgnbdu3gpp:attributes

+--rw extgnbdu3gpp:userLabel?           string

+--rw extgnbdu3gpp:vnfParametersList!

|  +--rw extgnbdu3gpp:vnfInstanceId    string

|  +--rw extgnbdu3gpp:vnfdId?          string

|  +--rw extgnbdu3gpp:flavourId?       string

|  +--rw extgnbdu3gpp:autoScalable     boolean

+--rw extgnbdu3gpp:peeParametersList!

|  +--rw extgnbdu3gpp:siteIdentification    string

|  +--rw extgnbdu3gpp:siteLatitude?         decimal64

|  +--rw extgnbdu3gpp:siteLongitude?        decimal64

|  +--rw extgnbdu3gpp:siteDescription       string

|  +--rw extgnbdu3gpp:equipmentType         string

|  +--rw extgnbdu3gpp:environmentType       string

|  +--rw extgnbdu3gpp:powerInterface        string

+--rw extgnbdu3gpp:priorityLabel        uint32

+--rw extgnbdu3gpp:gNBId                int64

+--rw extgnbdu3gpp:gNBIdLength          int32

+--rw extgnbdu3gpp:pLMNId* [mcc mnc]

+--rw extgnbdu3gpp:mcc    Mcc

+--rw extgnbdu3gpp:mnc    Mnc

module: _3gpp-nr-nrm-eutranetwork

+--rw EUtraNetwork* [id]

+--rw id                                         string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--rw userDefinedNetworkType?     string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                   types3gpp:Mcc

|  +--rw priorityLabel               uint32

|  +--rw parents*                    -> ../../../EUtraNetwork/id

|  +--rw containedChildren*          -> ../../../EUtraNetwork/id

+--rw extservgw3gpp:ExternalServingGWFunction* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|  +--rw extservgw3gpp:id            string

|  +--rw extservgw3gpp:attributes

|     +--rw extservgw3gpp:userLabel?           string

|     +--rw extservgw3gpp:vnfParametersList!

|     |  +--rw extservgw3gpp:vnfInstanceId    string

|     |  +--rw extservgw3gpp:vnfdId?          string

|     |  +--rw extservgw3gpp:flavourId?       string

|     |  +--rw extservgw3gpp:autoScalable     boolean

|     +--rw extservgw3gpp:peeParametersList!

|     |  +--rw extservgw3gpp:siteIdentification    string

|     |  +--rw extservgw3gpp:siteLatitude?         decimal64

|     |  +--rw extservgw3gpp:siteLongitude?        decimal64

|     |  +--rw extservgw3gpp:siteDescription       string

|     |  +--rw extservgw3gpp:equipmentType         string

|     |  +--rw extservgw3gpp:environmentType       string

|     |  +--rw extservgw3gpp:powerInterface        string

|     +--rw extservgw3gpp:priorityLabel        uint32

+--rw extenb3gpp:ExternalENBFunction* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|  +--rw extenb3gpp:id                              string

|  +--rw extenb3gpp:attributes

|  |  +--rw extenb3gpp:userLabel?           string

|  |  +--rw extenb3gpp:vnfParametersList!

|  |  |  +--rw extenb3gpp:vnfInstanceId    string

|  |  |  +--rw extenb3gpp:vnfdId?          string

|  |  |  +--rw extenb3gpp:flavourId?       string

|  |  |  +--rw extenb3gpp:autoScalable     boolean

|  |  +--rw extenb3gpp:peeParametersList!

|  |  |  +--rw extenb3gpp:siteIdentification    string

|  |  |  +--rw extenb3gpp:siteLatitude?         decimal64

|  |  |  +--rw extenb3gpp:siteLongitude?        decimal64

|  |  |  +--rw extenb3gpp:siteDescription       string

|  |  |  +--rw extenb3gpp:equipmentType         string

|  |  |  +--rw extenb3gpp:environmentType       string

|  |  |  +--rw extenb3gpp:powerInterface        string

|  |  +--rw extenb3gpp:priorityLabel        uint32

|  |  +--rw extenb3gpp:eNBId                int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellFDD* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|  |  +--rw exteutrancell3gpp:id            string

|  |  +--rw exteutrancell3gpp:attributes

|  |     +--rw exteutrancell3gpp:userLabel?           string

|  |     +--rw exteutrancell3gpp:vnfParametersList!

|  |     |  +--rw exteutrancell3gpp:vnfInstanceId    string

|  |     |  +--rw exteutrancell3gpp:vnfdId?          string

|  |     |  +--rw exteutrancell3gpp:flavourId?       string

|  |     |  +--rw exteutrancell3gpp:autoScalable     boolean

|  |     +--rw exteutrancell3gpp:peeParametersList!

|  |     |  +--rw exteutrancell3gpp:siteIdentification    string

|  |     |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|  |     |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|  |     |  +--rw exteutrancell3gpp:siteDescription       string

|  |     |  +--rw exteutrancell3gpp:equipmentType         string

|  |     |  +--rw exteutrancell3gpp:environmentType       string

|  |     |  +--rw exteutrancell3gpp:powerInterface        string

|  |     +--rw exteutrancell3gpp:priorityLabel        uint32

|  |     +--rw exteutrancell3gpp:pci                  int32

|  |     +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|  |     |  +--rw exteutrancell3gpp:mcc    Mcc

|  |     |  +--rw exteutrancell3gpp:mnc    Mnc

|  |     +--rw exteutrancell3gpp:cellLocalId?         int32

|  |     +--rw exteutrancell3gpp:eNBId                int32

|  |     +--rw exteutrancell3gpp:earfcnDL             int32

|  |     +--rw exteutrancell3gpp:earfcnUL             int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellTDD* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|     +--rw exteutrancell3gpp:id            string

|     +--rw exteutrancell3gpp:attributes

|        +--rw exteutrancell3gpp:userLabel?           string

|        +--rw exteutrancell3gpp:vnfParametersList!

|        |  +--rw exteutrancell3gpp:vnfInstanceId    string

|        |  +--rw exteutrancell3gpp:vnfdId?          string

|        |  +--rw exteutrancell3gpp:flavourId?       string

|        |  +--rw exteutrancell3gpp:autoScalable     boolean

|        +--rw exteutrancell3gpp:peeParametersList!

|        |  +--rw exteutrancell3gpp:siteIdentification    string

|        |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|        |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|        |  +--rw exteutrancell3gpp:siteDescription       string

|        |  +--rw exteutrancell3gpp:equipmentType         string

|        |  +--rw exteutrancell3gpp:environmentType       string

|        |  +--rw exteutrancell3gpp:powerInterface        string

|        +--rw exteutrancell3gpp:priorityLabel        uint32

|        +--rw exteutrancell3gpp:pci                  int32

|        +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|        |  +--rw exteutrancell3gpp:mcc    Mcc

|        |  +--rw exteutrancell3gpp:mnc    Mnc

|        +--rw exteutrancell3gpp:cellLocalId?         int32

|        +--rw exteutrancell3gpp:eNBId                int32

|        +--rw exteutrancell3gpp:earfcn               int32

+--rw eutraneteutranfreq3gpp:EUtranFrequency* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

+--rw eutraneteutranfreq3gpp:id            string

+--rw eutraneteutranfreq3gpp:attributes

+--rw eutraneteutranfreq3gpp:userLabel?                string

+--rw eutraneteutranfreq3gpp:vnfParametersList!

|  +--rw eutraneteutranfreq3gpp:vnfInstanceId    string

|  +--rw eutraneteutranfreq3gpp:vnfdId?          string

|  +--rw eutraneteutranfreq3gpp:flavourId?       string

|  +--rw eutraneteutranfreq3gpp:autoScalable     boolean

+--rw eutraneteutranfreq3gpp:peeParametersList!

|  +--rw eutraneteutranfreq3gpp:siteIdentification    string

|  +--rw eutraneteutranfreq3gpp:siteLatitude?         decimal64

|  +--rw eutraneteutranfreq3gpp:siteLongitude?        decimal64

|  +--rw eutraneteutranfreq3gpp:siteDescription       string

|  +--rw eutraneteutranfreq3gpp:equipmentType         string

|  +--rw eutraneteutranfreq3gpp:environmentType       string

|  +--rw eutraneteutranfreq3gpp:powerInterface        string

+--rw eutraneteutranfreq3gpp:priorityLabel             uint32

+--rw eutraneteutranfreq3gpp:earfcnDL                  uint32

+--ro eutraneteutranfreq3gpp:multiBandInfoListEutra*   uint16

module: _3gpp-nr-nrm-nrnetwork

+--rw NRNetwork* [id]

+--rw id                                        string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--rw userDefinedNetworkType?     string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                   types3gpp:Mcc

|  +--rw priorityLabel               uint32

+--rw extgnbcucp3gpp:ExternalGNBCUCPFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extgnbcucp3gpp:id                   string

|  +--rw extgnbcucp3gpp:attributes

|  |  +--rw extgnbcucp3gpp:userLabel?           string

|  |  +--rw extgnbcucp3gpp:vnfParametersList!

|  |  |  +--rw extgnbcucp3gpp:vnfInstanceId    string

|  |  |  +--rw extgnbcucp3gpp:vnfdId?          string

|  |  |  +--rw extgnbcucp3gpp:flavourId?       string

|  |  |  +--rw extgnbcucp3gpp:autoScalable     boolean

|  |  +--rw extgnbcucp3gpp:peeParametersList!

|  |  |  +--rw extgnbcucp3gpp:siteIdentification    string

|  |  |  +--rw extgnbcucp3gpp:siteLatitude?         decimal64

|  |  |  +--rw extgnbcucp3gpp:siteLongitude?        decimal64

|  |  |  +--rw extgnbcucp3gpp:siteDescription       string

|  |  |  +--rw extgnbcucp3gpp:equipmentType         string

|  |  |  +--rw extgnbcucp3gpp:environmentType       string

|  |  |  +--rw extgnbcucp3gpp:powerInterface        string

|  |  +--rw extgnbcucp3gpp:priorityLabel        uint32

|  |  +--rw extgnbcucp3gpp:gNBId                int64

|  |  +--rw extgnbcucp3gpp:gNBIdLength          int32

|  |  +--rw extgnbcucp3gpp:pLMNId* [mcc mnc]

|  |     +--rw extgnbcucp3gpp:mcc    Mcc

|  |     +--rw extgnbcucp3gpp:mnc    Mnc

|  +--rw extnrcellcu3gpp:ExternalNRCellCU* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|     +--rw extnrcellcu3gpp:id            string

|     +--rw extnrcellcu3gpp:attributes

|        +--rw extnrcellcu3gpp:userLabel?           string

|        +--rw extnrcellcu3gpp:vnfParametersList!

|        |  +--rw extnrcellcu3gpp:vnfInstanceId    string

|        |  +--rw extnrcellcu3gpp:vnfdId?          string

|        |  +--rw extnrcellcu3gpp:flavourId?       string

|        |  +--rw extnrcellcu3gpp:autoScalable     boolean

|        +--rw extnrcellcu3gpp:peeParametersList!

|        |  +--rw extnrcellcu3gpp:siteIdentification    string

|        |  +--rw extnrcellcu3gpp:siteLatitude?         decimal64

|        |  +--rw extnrcellcu3gpp:siteLongitude?        decimal64

|        |  +--rw extnrcellcu3gpp:siteDescription       string

|        |  +--rw extnrcellcu3gpp:equipmentType         string

|        |  +--rw extnrcellcu3gpp:environmentType       string

|        |  +--rw extnrcellcu3gpp:powerInterface        string

|        +--rw extnrcellcu3gpp:priorityLabel        uint32

|        +--rw extnrcellcu3gpp:cellLocalId          int32

|        +--rw extnrcellcu3gpp:nRPCI                int32

|        +--rw extnrcellcu3gpp:pLMNIdList* [mcc mnc]

|        |  +--rw extnrcellcu3gpp:mcc    Mcc

|        |  +--rw extnrcellcu3gpp:mnc    Mnc

|        +--rw extnrcellcu3gpp:nRFrequencyRef       types3gpp:DistinguishedName

+--rw extgnbcuup3gpp:ExternalGNBCUUPFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extgnbcuup3gpp:id            string

|  +--rw extgnbcuup3gpp:attributes

|     +--rw extgnbcuup3gpp:userLabel?           string

|     +--rw extgnbcuup3gpp:vnfParametersList!

|     |  +--rw extgnbcuup3gpp:vnfInstanceId    string

|     |  +--rw extgnbcuup3gpp:vnfdId?          string

|     |  +--rw extgnbcuup3gpp:flavourId?       string

|     |  +--rw extgnbcuup3gpp:autoScalable     boolean

|     +--rw extgnbcuup3gpp:peeParametersList!

|     |  +--rw extgnbcuup3gpp:siteIdentification    string

|     |  +--rw extgnbcuup3gpp:siteLatitude?         decimal64

|     |  +--rw extgnbcuup3gpp:siteLongitude?        decimal64

|     |  +--rw extgnbcuup3gpp:siteDescription       string

|     |  +--rw extgnbcuup3gpp:equipmentType         string

|     |  +--rw extgnbcuup3gpp:environmentType       string

|     |  +--rw extgnbcuup3gpp:powerInterface        string

|     +--rw extgnbcuup3gpp:priorityLabel        uint32

|     +--rw extgnbcuup3gpp:gNBId                int64

|     +--rw extgnbcuup3gpp:gNBIdLength          int32

+--rw extupf3gpp:ExternalUPFFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extupf3gpp:id            string

|  +--rw extupf3gpp:attributes

|     +--rw extupf3gpp:userLabel?           string

|     +--rw extupf3gpp:vnfParametersList!

|     |  +--rw extupf3gpp:vnfInstanceId    string

|     |  +--rw extupf3gpp:vnfdId?          string

|     |  +--rw extupf3gpp:flavourId?       string

|     |  +--rw extupf3gpp:autoScalable     boolean

|     +--rw extupf3gpp:peeParametersList!

|     |  +--rw extupf3gpp:siteIdentification    string

|     |  +--rw extupf3gpp:siteLatitude?         decimal64

|     |  +--rw extupf3gpp:siteLongitude?        decimal64

|     |  +--rw extupf3gpp:siteDescription       string

|     |  +--rw extupf3gpp:equipmentType         string

|     |  +--rw extupf3gpp:environmentType       string

|     |  +--rw extupf3gpp:powerInterface        string

|     +--rw extupf3gpp:priorityLabel        uint32

+--rw nrfreq3gpp:NRFrequency* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw nrfreq3gpp:id            string

|  +--rw nrfreq3gpp:attributes

|     +--rw nrfreq3gpp:userLabel?                  string

|     +--rw nrfreq3gpp:vnfParametersList!

|     |  +--rw nrfreq3gpp:vnfInstanceId    string

|     |  +--rw nrfreq3gpp:vnfdId?          string

|     |  +--rw nrfreq3gpp:flavourId?       string

|     |  +--rw nrfreq3gpp:autoScalable     boolean

|     +--rw nrfreq3gpp:peeParametersList!

|     |  +--rw nrfreq3gpp:siteIdentification    string

|     |  +--rw nrfreq3gpp:siteLatitude?         decimal64

|     |  +--rw nrfreq3gpp:siteLongitude?        decimal64

|     |  +--rw nrfreq3gpp:siteDescription       string

|     |  +--rw nrfreq3gpp:equipmentType         string

|     |  +--rw nrfreq3gpp:environmentType       string

|     |  +--rw nrfreq3gpp:powerInterface        string

|     +--rw nrfreq3gpp:priorityLabel               uint32

|     +--rw nrfreq3gpp:absoluteFrequencySSB        uint32

|     +--rw nrfreq3gpp:sSBSubCarrierSpacing        uint8

|     +--ro nrfreq3gpp:multiFrequencyBandListNR*   uint16

+--rw extamf3gpp:ExternalAMFFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extamf3gpp:id            string

|  +--rw extamf3gpp:attributes

|     +--rw extamf3gpp:userLabel?           string

|     +--rw extamf3gpp:vnfParametersList!

|     |  +--rw extamf3gpp:vnfInstanceId    string

|     |  +--rw extamf3gpp:vnfdId?          string

|     |  +--rw extamf3gpp:flavourId?       string

|     |  +--rw extamf3gpp:autoScalable     boolean

|     +--rw extamf3gpp:peeParametersList!

|     |  +--rw extamf3gpp:siteIdentification    string

|     |  +--rw extamf3gpp:siteLatitude?         decimal64

|     |  +--rw extamf3gpp:siteLongitude?        decimal64

|     |  +--rw extamf3gpp:siteDescription       string

|     |  +--rw extamf3gpp:equipmentType         string

|     |  +--rw extamf3gpp:environmentType       string

|     |  +--rw extamf3gpp:powerInterface        string

|     +--rw extamf3gpp:priorityLabel        uint32

|     +--rw extamf3gpp:pLMNIdList* [mcc mnc]

|     |  +--rw extamf3gpp:mcc    Mcc

|     |  +--rw extamf3gpp:mnc    Mnc

|     +--rw extamf3gpp:aMFIdentifier!

|        +--rw extamf3gpp:amfRegionId?   AmfRegionId

|        +--rw extamf3gpp:amfSetId?      AmfSetId

|        +--rw extamf3gpp:amfPointer?    AmfPointer

+--rw extgnbdu3gpp:ExternalGNBDUFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

+--rw extgnbdu3gpp:id            string

+--rw extgnbdu3gpp:attributes

+--rw extgnbdu3gpp:userLabel?           string

+--rw extgnbdu3gpp:vnfParametersList!

|  +--rw extgnbdu3gpp:vnfInstanceId    string

|  +--rw extgnbdu3gpp:vnfdId?          string

|  +--rw extgnbdu3gpp:flavourId?       string

|  +--rw extgnbdu3gpp:autoScalable     boolean

+--rw extgnbdu3gpp:peeParametersList!

|  +--rw extgnbdu3gpp:siteIdentification    string

|  +--rw extgnbdu3gpp:siteLatitude?         decimal64

|  +--rw extgnbdu3gpp:siteLongitude?        decimal64

|  +--rw extgnbdu3gpp:siteDescription       string

|  +--rw extgnbdu3gpp:equipmentType         string

|  +--rw extgnbdu3gpp:environmentType       string

|  +--rw extgnbdu3gpp:powerInterface        string

+--rw extgnbdu3gpp:priorityLabel        uint32

+--rw extgnbdu3gpp:gNBId                int64

+--rw extgnbdu3gpp:gNBIdLength          int32

+--rw extgnbdu3gpp:pLMNId* [mcc mnc]

+--rw extgnbdu3gpp:mcc    Mcc

+--rw extgnbdu3gpp:mnc    Mnc

module: ietf-yang-schema-mount

+--ro schema-mounts

+--ro namespace* [prefix]

|  +--ro prefix    yang:yang-identifier

|  +--ro uri?      inet:uri

+--ro mount-point* [module label]

+--ro module                 yang:yang-identifier

+--ro label                  yang:yang-identifier

+--ro config?                boolean

+--ro (schema-ref)

+--:(inline)

|  +--ro inline!

+--:(shared-schema)

+--ro shared-schema!

+--ro parent-reference*   yang:xpath1.0

# H.7	Mount information

_3gpp-5gc-nrm-affunction.yang

_3gpp-5gc-nrm-amffunction.yang

_3gpp-5gc-nrm-amfregion.yang

_3gpp-5gc-nrm-amfset.yang

_3gpp-5gc-nrm-ausffunction.yang

_3gpp-5gc-nrm-dnfunction.yang

_3gpp-5gc-nrm-ep.yang

_3gpp-5gc-nrm-externalnrffunction.yang

_3gpp-5gc-nrm-externalnssffunction.yang

_3gpp-5gc-nrm-lmffunction.yang

_3gpp-5gc-nrm-n3iwffunction.yang

_3gpp-5gc-nrm-nfprofile.yang

_3gpp-5gc-nrm-nfservice.yang

_3gpp-5gc-nrm-ngeirfunction.yang

_3gpp-5gc-nrm-nrffunction.yang

_3gpp-5gc-nrm-nssffunction.yang

_3gpp-5gc-nrm-nwdaffunction.yang

_3gpp-5gc-nrm-pcffunction.yang

_3gpp-5gc-nrm-seppfunction.yang

_3gpp-5gc-nrm-smffunction.yang

_3gpp-5gc-nrm-smsffunction.yang

_3gpp-5gc-nrm-udmfunction.yang

_3gpp-5gc-nrm-udrfunction.yang

_3gpp-5gc-nrm-udsffunction.yang

_3gpp-5gc-nrm-upffunction.yang

If the above files are mounted the yang files described in clause E.7 shall also be mounted.

Annex I (normative):
XML definitions for network slice

# I.1	General

This annex contains the XML definitions for the network slice NRM, in accordance with network slice NRM Information Model definitions specified in clause 6.

# I.2	Architectural features

The overall architectural feature of network slice information model is specified in clause 6, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations.

# I.3	Mapping

## I.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## I.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# I.4	Solution Set (SS) definitions

## I.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [33].

This annex defines the NRM-specific XML schema sliceNrm.xsd for the network slice Information Model defined in clause 6.

XML schema sliceNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [33].

## I.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## I.4.3	XML schema "sliceNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 network slice Network Resource Model

XML schema definition

sliceNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:sl="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<simpleType name="MobilityLevel">

<restriction base="string">

<enumeration value="STATIONARY"/>

<enumeration value="NOMADIC"/>

<enumeration value="RESTRICTED MOBILITY"/>

<enumeration value="FULLY MOBILITY"/>

</restriction>

</simpleType>

<simpleType name="SharingLevel">

<restriction base="string">

<enumeration value="SHARED"/>

<enumeration value="NON-SHARED"/>

</restriction>

</simpleType>

<complexType name="PerfReq">

<!-- Refer to definitions in TS 22.261-->

<sequence>

<choice minOccurs="0" maxOccurs="1">

<element name="perfReqEMBB" type="sl:PerfReqEmbb"/>

<element name="perfReqUrllc" type="sl:PerfReqUrllc"/>

</choice>

</sequence>

</complexType>

<complexType name="PerfReqEmbb">

<sequence>

<element name="expDataRateDL" type="integer" minOccurs="0"/>

<element name="expDataRateUL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapDL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapUL" type="integer" minOccurs="0"/>

<element name="userDensity" type="integer" minOccurs="0"/>

<element name="activityFactor" type="integer " minOccurs="0"/>

<element name="uESpeed" type="integer" minOccurs="0"/>

<element name="coverage" type="string" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="PerfReqUrllc">

<sequence>

<element name="e2eKatency" type="integer" minOccurs="0"/>

<element name="jitter" type="integer" minOccurs="0"/>

<element name="survivalTime" type="integer" minOccurs="0"/>

<element name="cSAvailability" type="float" minOccurs="0"/>

<element name="reliability" type="float" minOccurs="0"/>

<element name="expDataRate" type="integer " minOccurs="0"/>

<element name="payloadSize" type="integer" minOccurs="0"/>

<element name="trafficDensity" type="string" minOccurs="0"/>

<element name="connDensity" type="float" minOccurs="0"/>

<element name="serviceDimension" type="string " minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ServiceProfile">

<sequence>

<element name="serviceProfileId" type="string"/>

<element name="sNSSAIList" type="ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NrTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="integer" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

<element name="sst" type="ngc:Sst"/>

<element name="availability" type="float" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ServiceProfileList">

<sequence>

<element name="serviceProfile" type="sl:ServiceProfile"/>

</sequence>

</complexType>

<complexType name="SliceProfile">

<sequence>

<element name="sliceProfileId" type="string"/>

<element name="sNSSAIList" type=" ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NrTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="sl: MobilityLevel" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="SliceProfileList">

<sequence>

<element name="sliceProfile" type="sl:SliceProfile"/>

</sequence>

</complexType>

<complexType name="NsInfo">

<!-- Refer to definitions in subclause 8.3.3.2.2 of ETSI NFV IFA013 -->

<sequence>

<element name="nsInstanceId" type="string"/>

<element name="nsName" type="string"/>

<element name="description" type="string"/>

</sequence>

</complexType>

<element name="NetworkSlice" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="serviceProfileList" type="sl:ServiceProfileList"/>					<element name="networkSliceSubnetRef" type="xn:dn"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NetworkSliceSubnet" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="nsInfo" type="sl:NsInfo" minOccurs="0"/>

<element name="sliceProfileList" type="sl:SliceProfileList"/>

<element name="managedFunctionRef" type="xn:dnlist"/>

<element name="networkSliceSubnetRef" type="xn:dnlist"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex J (normative):
JSON definitions for network slice

# J.1	General

This annex contains the JSON definitions for the network slice NRM, in accordance with network slice NRM Information Model definitions specified in clause 6.

# J.2	Architectural features

The overall architectural feature of network slice information model is specified in clause 6, this clause specifies features that are specific to the Schema definitions.

# J.3	Mapping

Refer to TS 32.160 [47].

## Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

ng table.

J.4	Solution Set (SS) definitions

## J.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## J.4.2	Graphical representation

None.

## J.4.3	JSON schema "sliceNrm.json"

{

"openapi": "3.0.1",

"info": {

"title": "3GPP Network Slice NRM",

"version": "15.3.0",

"description": "OAS 3.0.1 specification compatible schema for 3GPP Network Slice NRM"

},

"paths": {},

"components": {

"schemas": {

"MobilityLevel": {

"type": "string",

"enum": [

"STATIONARY",

"NOMADIC",

"RESTRICTED MOBILITY",

"FULLY MOBILITY"

]

},

"SharingLevel": {

"type": "string",

"enum": [

"SHARED",

"NON-SHARED"

]

},

"PerfReq": {

"type": "object",

"properties": {

"prefReqEmbb": {

"$ref": "#/components/schemas/PerfReqEmbb"

},

"prefReqUrllc": {

"$ref": "#/components/schemas/PerfReqUrllc"

}

}

},

"PerfReqEmbb": {

"type": "object",

"properties": {

"expDataRateDL": {

"type": "number"

},

"expDataRateUL": {

"type": "number"

},

"areaTrafficCapDL": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"userDensity": {

"type": "number"

},

"activityFactor": {

"type": "number"

},

"uESpeed": {

"type": "number"

},

"coverage": {

"type": "string"

}

}

},

"PerfReqUrllc": {

"type": "object",

"properties": {

"e2eLatency": {

"type": "number"

},

"jitter": {

"type": "number"

},

"survivalTime": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"cSAvailability": {

"type": "number"

},

"reliability": {

"type": "number"

},

"expDataRate": {

"type": "number"

},

"payloadSize": {

"type": "number"

},

"trafficDensity": {

"type": "string"

},

"connDensity": {

"type": "number"

},

"serviceDimension": {

"type": "string"

}

}

},

"NsInfo": {

"type": "object",

"properties": {

"nsInstanceId": {

"type": "string"

},

"nsName": {

"type": "string"

}

}

},

"NetworkSlice": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

},

{

"type": "object",

"properties": {}

},

{

"type": "object",

"properties": {

"networkSliceSubnetRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

},

"operationalState": {

"$ref": "genericNrm.json#/components/schemas/OperationalState"

},

"administrativeState": {

"$ref": "genericNrm.json#/components/schemas/AdministrativeState"

},

"serviceProfileList": {

"$ref": "#/components/schemas/ServiceProfileList"

}

}

}

]

}

}

}

]

},

"NetworkSliceSubnet": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

},

{

"type": "object",

"properties": {}

},

{

"type": "object",

"properties": {

"managedFunctionRefList": {

"$ref": "genericNrm.json#/components/schemas/DnList"

},

"networkSliceSubnetRefList": {

"$ref": "genericNrm.json#/components/schemas/DnList"

},

"operationalState": {

"$ref": "genericNrm.json#/components/schemas/OperationalState"

},

"administrativeState": {

"$ref": "genericNrm.json#/components/schemas/AdministrativeState"

},

"nsInfo": {

"$ref": "#/components/schemas/NsInfo"

},

"sliceProfileList": {

"$ref": "#/components/schemas/SliceProfileList"

}

}

}

]

}

}

}

]

},

"ServiceProfile": {

"type": "object",

"properties": {

"serviceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "nRNrm.json#/components/schemas/SnssaiList"

},

"plmnIdList": {

"$ref": "nRNrm.json#/components/schemas/PlmnIdList"

},

"perfReq": {

"$ref": "#/components/schemas/PerfReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "ngcNrm.json#/components/schemas/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"$ref": "#/components/schemas/MobilityLevel"

},

"sst": {

"$ref": "nrNrm.json#/components/schemas/Sst"

},

"resourceSharingLevel": {

"$ref": "#/components/schemas/SharingLevel"

},

"availability": {

"type": "number"

}

}

},

"ServiceProfileList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ServiceProfile"

}

},

"SliceProfile": {

"type": "object",

"properties": {

"sliceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "nRNrm.json#/components/schemas/SnssaiList"

},

"plmnIdList": {

"$ref": "nRNrm.json#/components/schemas/PlmnIdList"

},

"perfReq": {

"$ref": "#/components/schemas/PerfReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "ngcNrm.json#/components/schemas/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"$ref": "#/components/schemas/MobilityLevel"

},

"resourceSharingLevel": {

"$ref": "#/components/schemas/SharingLevel"

}

}

},

"SliceProfileList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SliceProfile"

}

}

}

}

}

Annex K :
Void

# nt definitions are listed in the following table.

## definitions are listed in the following table.

Annex L (informative):	
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-09 | SA#81 |  |  |  |  | EdiHelp review | 15.0.1 |
| 2018-12 | SA#82 | SP-181046 | 0001 | 1 | F | Fix issues raised by EditHelp | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0002 | 2 | F | Update NR Stage 2 definition to align with TS 37.340 for MR-DC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0003 | 1 | F | Update NRM Stage 2 defintion to align with TS 23.501 for 5G architecture | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0005 | 1 | F | Update Stage 3 XML definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0006 | 1 | F | Update Stage 3 JSON definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0007 | 1 | F | Update Stage 3 YANG definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0008 | 1 | F | Update Stage 3 XML definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0009 | 1 | F | Update Stage 3 JSON definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0011 | 1 | F | Update stage 3 XML definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0012 | 1 | F | Update Stage 3 JSON definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0013 | 1 | F | Update stage 3 YANG definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0014 | 1 | F | Correct the term sNSSAIList and nRTAClist | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0015 | 1 | F | Update the inheritance hierarchy figure for NR NRM to include BWP IOC and NRSectorCarrier IOC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0016 | 1 | F | Change the term nCGI to nCI | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0019 | 1 | F | Align properties of cell state | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0021 | 1 | F | Add missing attribute definition and condition | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0022 | 1 | F | Add missing detail definition for attribute | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0023 | 1 | F | Adding missing attribute, and correction of reference | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0025 | - | F | Remove NSSF from the abbrevations | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0027 | - | F | Replace symbol for network slice state management | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0031 | 1 | F | Remove the ExternalENBFunction definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0033 | 1 | F | Align the management of external function and cell with TS 28.658 | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0034 | 1 | F | Update NR NRM with Cell Relation | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0038 | 3 | F | RRM Policy enhancements | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0039 | 1 | F | Fix containment issue in YANG definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0040 | - | F | Implement minor corrections | 15.1.0 |
| 2018-12 | SA#82 | SP-181042 | 0041 | - | F | Update Stage 3 NRM for RRM Policy enhancements | 15.1.0 |
| 2019-03 | SA#83 | SP-190121 | 0043 | 1 | F | Align NR attributes definition related to SSB with corresponding NG-RAN IE definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0044 | 1 | F | Correct the use of nCI and PLMN | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0045 | - | F | Remove duplicate definition for ExternalNRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0046 | 2 | F | Correct class diagram for view on external entities | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0047 | 1 | F | Correct the definition for resourceSharingLevel | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0048 | 1 | F | Correction of references | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0052 | 1 | F | Align the term mFIdList and constituentNSSIIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0053 | 1 | F | Correct the definition of nSSIId | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0054 | 1 | F | Add missing attribute constraint for class definition of NSSFFunction | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0055 | 1 | F | Correct attribute constraints for RRMpolicy related attributes in NRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0057 | - | F | Correct cardinality of End Point (EP) to target | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0058 | 0 | F | Correct Import table | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0059 | - | F | Remove ExternalNRCellCU.pLMNIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0060 | - | F | Use 'bS' (not 'bs') to prefix all BS (base station) attributes | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0061 | 1 | F | Correction of State attributes descriptions | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0062 | - | F | Update 5G JSON Solution Set to align with generic NRM | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0063 | 1 | F | Update YANG Solution Set to align with Stage 2 definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0064 | 1 | F | Update Information Service to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0065 | 1 | F | Update Solution Set to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0066 | 1 | F | Add availability in service profile of network slice resource model | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0068 | 1 | F | Add sST attribute to ServiceProfile | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0069 | 1 | F | Update to sST attribute stage 3 | 15.2.0 |
| 2019-03 | SA#83 | SP-190149 | 0073 | 2 | F | Replace CoverageAreaTAList type definition | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0074 | 1 | F | Name datatypes SliceProfile and ServiceProfile | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0075 | 1 | F | Add datatype definition for S-NSSAI | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0076 | 1 | F | Remove incomplete description for TAC | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0079 | 1 | F | Name datatype RRMPolicyRatio2 | 16.0.0 |
| 2019-06 | SA#84 | SP-190374 | 0083 | - | A | Remove attribute availabilityStatus in NRCellDU IOC | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0085 | 1 | F | Correct the definition for nsInfo | 16.1.0 |
| 2019-06 | SA#84 | SP-190374 | 0088 | 1 | A | Update Information Service of NR to fix unclear Note issue | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0096 | 2 | A | Correct the use of plmnIdList | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0098 | 1 | F | Add missing clauses to RRMPolicyRatio2 data type | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0099 | 1 | F | Update RRMPolicyRatio2 data type name in stage 3 | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0102 | - | F | Fix the implementation errors | 16.1.0 |
| 2019-09 | SA#85 | SP-190745 | 0089 | 2 | B | Update 5GC Information Service to align with Managed Service Definition | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0107 | 1 | A | Correct description for NR deployment scenario | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0109 | 1 | A | Correct NR NRM model to be applicable for all NG-RAN architecture | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0114 | 1 | C | Support NF Profile management | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0121 | 1 | A | Clarification of sNSSAIList attribute | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0123 | - | A | Remove pLMNId from GNBDUFunction | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0126 | 2 | A | Update class definition with inheritance information | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0128 | 1 | A | Correct description of NRCellCU and NRCellDU to be applicable for all deployment scenarios | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0130 | - | A | Correct XML solution set for NR | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0132 | - | A | Correct XML solution set for Network slice | 16.2.0 |
| 2019-09 | SA#85 | SP-190750 | 0133 | 1 | F | Clarification on slice model | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0142 | 1 | A | Add YANG mount info | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0143 | - | A | Add YANG solution | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0149 | 1 | F | generate JSON definition for 5GC NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0150 | 1 | A | Fix NR NRM to add missed ID info | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0152 | - | F | XML Solution Set for 5GC | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0154 | - | A | Correct ETSI NFV reference | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0157 | 1 | A | generate JSON definition for Slice NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0158 | 1 | A | generate JSON definition for NR NRM based on new style guideline | 16.2.0 |