| 3GPP TR 28.aaa V0.0.0 (2019-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Heartbeat (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

Annex <X> (informative): Change history	7

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document specifies use cases, requirements, management services and procedures for the heartbeat management service.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[B]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[C]	3GPP TS 32.351: "Telecommunication management; Communication Surveillance (CS) Integration Reference Point (IRP); Requirements".

[D]	3GPP TS 32.352: "Telecommunication management; Communication Surveillance (CS) Integration Reference Point (IRP); Information Service (IS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	General

## 4.1	Overview

# 5	Specification level requirements

## 5.1	Use cases

### 5.1.1	Configuring heartbeat notification periodicity

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure the periodicity at which the heartbeat management service provider shall emit heartbeat notifications to its authorized heartbeat management service consumer. |  |
| Actors and Roles | An authorized consumer of the heartbeat management service. |  |
| Telecom resources | The heartbeat management service provider. |  |
| Assumptions | The heartbeat management service provider and the provider sending notifications are sharing or using the same communication link. |  |
| Pre-conditions | N/A |  |
| Begins when | The heartbeat management service consumer sends a request to the heartbeat management service provider to set the periodicity at which it shall emit heartbeat notifications. |  |
| Step 1 (M) | The heartbeat management service provider receives the request and sets a countdown timer to a value equal to the periodicity requested by the heartbeat management service consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The heartbeat notification periodicity has been configured according to the heartbeat management service consumer request. |  |
| Traceability | REQ-HB-CTRL-FUN-2. |  |

### 5.1.2	Requesting immediate heartbeat notification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To trigger the emission of an immediate heartbeat notification by the heartbeat management service provider. |  |
| Actors and Roles | An authorized consumer of the heartbeat management service. |  |
| Telecom resources | The heartbeat management service provider. |  |
| Assumptions | The heartbeat management service provider and the provider sending notifications are sharing or using the same communication link. There are many types of notifications. This UC assumes all types of notification travel over the same communication link. |  |
| Pre-conditions | N/A |  |
| Begins when | The soliciting heartbeat management service consumer sends a request to the heartbeat management service provider to emit immediately a heartbeat notification. |  |
| Step 1 (M) | The heartbeat management service provider receives the request and sends immediately a heartbeat notification to the soliciting heartbeat management service consumer. |  |
| Step 2 (O) | The heartbeat management service provider sends immediately a heartbeat notification to all the other heartbeat management service consumer(s). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The immediate heartbeat notification has been emitted according to the soliciting heartbeat management service consumer request. |  |
| Traceability | REQ-HB-CTRL-FUN-3, REQ-HB-NOTIF-FUN-2. |  |

### 5.1.3	Emitting periodic heartbeat notifications

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To send periodic heartbeat notifications at the periodicity requested by the heartbeat management service consumer. |  |
| Actors and Roles | An authorized provider of the heartbeat management service. |  |
| Telecom resources | The heartbeat management service consumer. |  |
| Assumptions | The heartbeat notification periodicity has been configured according to the heartbeat management service consumer request. The heartbeat management service provider and the provider sending notifications are sharing or using the same communication link. There are many types of notifications. This UC assumes all types of notification travel over the same communication link. | Configuring heartbeat notification periodicity |
| Pre-conditions | N/A |  |
| Begins when | The countdown timer managed by the heartbeat management service provider has reached the value 0. |  |
| Step 1 (M) | The heartbeat management service provider sends a heartbeat notification to the heartbeat management service consumer. |  |
| Step 2 (M) | The heartbeat management service provider resets its countdown timer to the value initially configured by the heartbeat management service consumer. | Configuring heartbeat notification periodicity |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The periodic heartbeat notification has been emitted to the heartbeat management service consumer at the requested periodicity. |  |
| Traceability | REQ-HB-NOTIF-FUN-1. |  |

## 5.2	Requirements

### 5.2.1	Requirements for controlling heartbeat

### 5.2.2	Requirements for notifying heartbeat

# 6	Heartbeat management services

The heartbeat management services are listed in table 6-1.

Table 6-1: Heartbeat management services

| MnS Name | MnS Component Type A (operations and notifications) | MnS Component Type B (information model) | Note |
| --- | --- | --- | --- |
| HeartbeatControl | Operations defined in TS 28.532 [B]: - getHeartbeatPeriod operation - setHeartbeatPeriod operation - triggerHeartbeat operation | ManagedFunction / ManagedElement defined in TS 28.622 [A]. | This management service enables its consumer to control heartbeat. |
| HeartbeatNotification | Notifications defined in TS 28.532 [B]: - notifyHeartbeat notification | ManagedFunction / ManagedElement defined in TS 28.622 [A]. | This management service enables its consumer to receive heartbeat notifications. |

# 7	Procedures for heartbeat management services

## 7.1	Procedure for configuring heartbeat notification periodicity

Figure 7.1-1 illustrates the procedure for configuring the periodicity of heartbeat notifications.

Figure 7.1-1: Procedure for configuring heartbeat notification periodicity

Editor’s note: the textual description of this procedure is FFS.

## 7.2	Procedure for requesting immediate heartbeat notification

Figure 7.2-1 illustrates the procedure for requesting immediate heartbeat notification.

Figure 7.2-1: Procedure for requesting immediate heartbeat notification

Editor’s note: the textual description of this procedure is FFS.

## 7.3	Procedure for notifying periodic heartbeat notifications

Figure 7.3-1 illustrates the procedure for notifying periodic heartbeat notifications.

Figure 7.3-1: Procedure for notifying periodic heartbeat notifications

Editor’s note: the textual description of this procedure is FFS.

Annex <A> (informative): PlantUML source code

## A.1	Procedure for configuring heartbeat notification periodicity

The following PlantUML source code is used to describe the procedure for configuring heartbeat notification periodicity, as depicted by Figure 7.1-1:

@startuml

title "Configuring heartbeat notification periodicity"

actor "Heartbeat MnS Consumer" as CONS

participant "Heartbeat MnS Provider" as PROV

CONS -> PROV: setHeartbeatPeriod request (Periodicity)

rnote over PROV

Internal Countdown Timer := Periodicity

endrnote

PROV -> CONS: setHeartbeatPeriod response

rnote over PROV

Start countdown immediately

endrnote

@enduml

## A.2	Procedure for requesting immediate heartbeat notification

The following PlantUML source code is used to describe the procedure for requesting immediate heartbeat notification, as depicted by Figure 7.2-1:

@startuml

title "Requesting immediate heartbeat notification"

actor "Heartbeat MnS Consumer" as CONS

participant "Heartbeat MnS Provider" as PROV

CONS -> PROV: triggerHeartbeat request

PROV -> CONS: triggerHeartbeat response

rnote over PROV

Send immediate heartbeat notification

to soliciting Heartbeat MnS Consumer

endrnote

PROV -> CONS: notifyHeartbeat

rnote over PROV

Optional:

send also immediate heartbeat

notifications  to all other

heartbeat management service consumer(s).

endrnote

@enduml

## A.3	Procedure for notifying periodic heartbeat notifications

The following PlantUML source code is used to describe the procedure for notifying periodic heartbeat notifications, as depicted by Figure 7.3-1:

@startuml

title "Notifying periodic heartbeat notifications"

participant "Heartbeat MnS Consumer" as CONS

actor "Heartbeat MnS Provider" as PROV

PROV -> CONS: notifyHeartbeat

rnote over PROV

Reset its countdown timer to

the value initially configured

by the Heartbeat MnS Consumer

endrnote

@enduml

Annex <B> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |