3GPP TSG-<SA5> Meeting # 127	S5-196438

# Sophia Antipolis, France,  14 - 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 00 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add examples |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | REST_SS |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Examples help in implementation and improve interoperability. Currently TS 32.158 has no examples. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Examples are added for numerous use cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability is impacted negatively. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

# Annex X (informative):
Examples

## X.1	Example information model

The following JSON instance document is used for the examples in this chapter.

{ "SubNetwork": { "id": "SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmn-id": { "mcc": 456, "mnc": 789 } }, "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorname": "Company XY", "location": "TV Tower" }, "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }, { "id": "XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ] }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorname": "Company XY", "location": "Grunewald" } } ] } }

Note the following examples do not follow the URI structure specified in clause 4.4 for simplicity reasons. The "data" object in responses is omitted as well.

## X.2	Retrieval of resources

### X.2.1	Retrieval of a single complete resource with HTTP GET

## To retrieve a complete "YxzFunction"resource the MnS Consumer might send the following request.

GET /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Accept: application/json

The response might look like

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "XyzFunction": { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } }

or when using a "data" object

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "data": { "XyzFunction": { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } } }

## X.2.2	Attribute and and attribute field selection on a single resource

## To retrieve only the "userLabel" attribute and the "mcc" attribute field of the "plmn-id" attribute the MnS Consumer might send

GET /SubNetwork=SN1?fields=attributes/userLabel,attributes/plmn-id/mcc HTTP/1.1 Host: example.org Accept: application/json

Alternatively one might send as well

GET /SubNetwork=SN1?attributes=userLabel&fields=attributes/plmn-id/mcc HTTP/1.1 Host: example.org Accept: application/json

The response contains only the selected attribute and attribute field.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "SubNetwork": { "id": "SN1", "attributes": { "userLabel": "Berlin NW", "plmn-id": { "mnc": 789 } } } }

### X.2.3	Retrieval of multiple complete resources using scoping and filtering

The following example selects all "ManageElement" nodes with a "vendorname" of "Company XY".

GET /SubNetwork=SN1?scope=BASE_ALL&\ filter=/SubNetwork/ManagedElement/attributes[vendorname="Company XY"]/parent::node() HTTP/1.1 Host: example.org Accept: application/json

Alternatively, the following XPath expression can be used.

GET /SubNetwork=SN1?scope=BASE_ALL&\ filter=/SubNetwork/ManagedElement[attributes/vendorname="Company XY"] HTTP/1.1 Host: example.org Accept: application/json

The response is

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorname": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorname": "Company XY", "location": "Grunewald" } } ] } }

The following example returns the containment tree only

GET /SubNetwork=SN1?scope=BASE_ALL&attributes= HTTP/1.1 Host: example.org Accept: application/json

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF1" }, { "id": "XYZF2" } ] }, { "id": "ME2" } ] } }

## X.3	Creation of resources

## X.3.1	Creation of a resource with HTTP PUT

In this example a new "XyzFunction" resource is created. The target URI specifies the location of the new resource. The "id" of the new resource is "XYZF1" and created by the MnS Consumer.

PUT /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

The response contains the location header and the complete representation of the new resource.

HTTP/1.1 201 Created Date: Tue, 06 Aug 2019 16:50:26 GMT Location: http://example.org/ SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 Content-Type: application/json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

## X.3.2	Creation of a resource with HTTP POST

When creating a new resource with POST the target URI identifies the parent resource of the new resource to be created. The identifier of the new resource is created by the MnS Producer, hence the "id" is equal to "null" in the POST request. If the "id" carries a value, then the MnS Producer may consider that as a non-binding recommendation by the MnS Consumer.

POST /SubNetwork=SN1/ManagedElement=ME1 HTTP/1.1 Host: example.org Content-Type: application/json { "XyzFunction": [ { "id": "null", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

HTTP/1.1 201 Created Date: Tue, 06 Aug 2019 16:50:26 GMT Location: http://example.org/ SubNetwork=SN1/ManagedElement=ME1/XyzFunction=123e4567-e89b Content-Type: application/json { "XyzFunction": [ { "id": "123e4567-e89b", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

### X.3.3	Creation of a resource with JSON Patch

This example shows the creation of a resource with JSON Patch. The target URI identifies the parent resource of the resource to be created.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "add", "path": "/ManagedElement=ME1", "value": { "id": "ME1", "class": "ManagedElement", "attributes": { "userLabel": " Berlin NW 1", "vendorname": "Company XY", "location": "TV Tower" } } } ]

## X.4	Deletion of a resource

### X.4.1	Deletion of a resource with HTTP DELETE

The following example deletes an instance of "ManagedElement".

DELETE /SubNetwork=SN1/ManagedElement=ME2 HTTP/1.1 Host: example.org

HTTP/1.1 204 No Content Date: Tue, 06 Aug 2019 16:50:26 GMT

### X.4.2	Deletion of a resource with HTTP DELETE

This example deletes both "XyzFunction" resources.

DELETE /SubNetwork=SN1?scopeType= BASE_NTH_LEVEL&scopeLevel=2 HTTP/1.1 Host: example.org

HTTP/1.1 204 No Content Date: Tue, 06 Aug 2019 16:50:26 GMT

### X.4.3	Deletion of a resource with JSON Patch

The following example deletes an instance of "ManagedElement".

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "remove", "path": "/ManagedElement=ME1" } ]

## X.5	Complete update of a resource

## The following example updates a "XyzFunction" resource. Only the "attrA" attribute is updated with a new value. The "attrB" attribute is set to the old value, but still the "attrB" attribute needs to be present.

PUT /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "newValue", "attrB": 551 } } ] }

## X.6	Partial update of a resource

### X.6.1	Partial update of a resource with JSON Merge Patch

The first example shows how the attribute "attrA" of the "XyzFunction with the "id" equal to "YXZF1" is changed from "xyz" to "def" using Enhanced JSON Merge Patch.

PATCH /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "XyzFunction": { "id": "XYZF1", "attributes": { "attrA": "def" } } }

In the second example the "mcc" attribute field of the "plmnId" attribute is updated to "654". The employed patch method is again Enhanced JSON Merge Patch.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "SubNetwork": { "id": "SN1", "attributes": { "plmn-Id": { "mcc": 654 } } } }

### X.6.2	Partial update of a resource with Enhanced JSON Patch

With Enhanced JSON Patch examples look the same as above, except for the media type.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/enhanced-merge-patch+json { "SubNetwork": { "id": "SN1", "attributes": { "plmn-Id": { "mcc": 654 } } } }

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/enhanced-merge-patch+json { "SubNetwork": { "id": "SN1", "attributes": { "plmn-Id": { "mcc": 654 } } } }

### X.6.3	Partial update of a resource with JSON Patch

With JSON Patch the examples look like

PATCH /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/XyzFunction=XYZF1/attributes/attrA", "value": 654 } ]

and

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/SubNetwork=SN1/attributes/plmn-Id/mcc", "value": 654 } ]

## X.7	Manipulating multiple resources

### X.7.1	Manipulating multiple resources with Enhanced JSON Merge Patch

In this example the "userLabel" attribute and the "mcc" attribute field of the "subNetwork" resource is updated. A new "XyzFunction" resource is created as well as a new "ManagedElement" resource.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/enhanced-merge-patch+json { "SubNetwork": { "id": "SN1", "attributes": { "userLabel": "Berlin NW-1", "plmn-id": { "mcc": 456 } }, "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF3", "attributes": { "attrA": "fgh", "attrB": 555 } } ] }, { "id": "ME3", "attributes": { "userLabel": " Berlin NW 3", "vendorname": "Company XY", "location": "Spandau" } } ] } }

In the following example a "XYzFunction" resource is deleted.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/enhanced-merge-patch+json { "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF2", "attributes": null } ] } ] } }

### X.7.2	Manipulating multiple resources with JSON PATCH

## The same resource modifications as in the previous chapter expressed using JSON Patch are given by

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1/attributes/userLabel", "value": "Berlin NW-1" }, { "op": "replace", "path": "/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1/attributes/plmn-id/mcc", "value": 654 }, { "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF3", "value": { "id": "XYZF3", "attributes": { "attrA": "fgh", "attrB": 555 } } }, { "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME3", "value": { "id": "ME3", "attributes": { "userLabel": " Berlin NW 3", "vendorname": "Company XY", "location": "Spandau" } } } ]

End of modificatios