3GPP TSG-SA5 Meeting #128	S5-197043

# Zhuhai , China, 18th Nov 2019 – 22nd Nov 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 00xx | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add definitions of PDCP end user throughput measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IP scheduled end user throughput in E-UTRAN according to 3GPP TS 36.314 assists the network operator with information about the integrity provided to their customers. If an end user often experiences low quality during use of a service, the end-user might change wireless subscription provider, i.e. loss of income for the network operator. Operators are expecting similar measurements to be provided also in 5G. Note: Similar measurement for SCG bearer relevant for LTE-NR Dual Connectivity option 3 is not explicitely mentioned in this measurement proposal due to not standardized DDDS between eNB and CU gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add definitions of PDCP end user throughput are respective. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The volume of PDCP data can’t be monitored by the operators. The monitoring of the flow split in DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

#### 5.1.1.X  Average DL UE buffered Throughput

a)  This measurement provides the average down link buffered UE throughput of NR UE on GNBCUUPFuntion. This measurement is intended for throughput per UE and bearer independent of traffic patterns and packet size.  Initial buffering time in gNB, meant as time interval the first PDCP SDU of the new burst is received in CU until its first part is transmitted over air interface of the DU, is also included.

b)  DER(N=1)

c)  This measurement is obtained by the following formula for a measurement period:

ThroughputVolumeThroughputTime [kbits/s]

where each ThroughputVolume and ThroughputTime is intended to represent one DL burst as explained in the Fig. 5.1.1.X-1 and Fig. 5.1.1.X-2, and Table 5.1.1.X-1 and Table 5.1.1.X-2 for DRB and splitDRB bearer, respectively.

Figure 5.1.1.X-1 Average DL buffered UE throughput per DRB

| T0’ | First PDCP SDU of the new burst arrived to CU and there are not any data in the buffer in RLC entity relevant to the bearer. |
| --- | --- |
| T1’ | First PDCP PDU has been sent to DU after T0. |
| T2’ | The buffer in RLC entity gets empty after T0. |
| ThroughputTime | T2’ – T0’ Note: Contribution of the given PDCP SDU to ThroughputTime, i.e. the time the PDCP PDU related to the PDCP SDU will spend in buffer of RLC enitity is obtained as volume of the PDCP PDU divided with Desired Data Rate for the data radio bearer based on last DDDS feedback [TS 38.425]. Considering the Desired Data Rate includes time intervals when UE has data in the buffer for the given bearer but not scheduled and delay in delivering PDCP PDU from CU to DU it may provide sufficient estimation the time the PDCP PDU related to the PDCP SDU will spend in the buffer of RLC enitity. |
| ThroughptVolume | The PDCP SDU volume in kbits successfully transmitted (acknowledged by) in DL to RLC entity of the DU per UE per bearer and one burst (consisting of PDCP SDU 1, 2 and 3 in example in Fig.1). |

Table 5.1.1.X-1

Figure 5.1.1.X-2  Average DL buffered UE throughput per split DRB

| T0 | First PDCP SDU of the new burst arrived to CU and there are not any data in the buffer in RLC entity for any of the legs relevant to the bearer. |
| --- | --- |
| T1 | First PDCP PDU has been sent to DU of the first leg after T0. |
| T2 | First PDCP PDU has been sent to DU of the second leg after T0. |
| T5 | The buffer in RLC entity of the first leg gets empty after T0. |
| T6 | The buffer in RLC entity of the second leg gets empty after T2 |
| ThroughputTime | T6 – T0 Note: Contribution of the given PDCP SDU to ThroughputTime, i.e. the time the PDCP PDU related to the PDCP SDU will spend in RLC intity buffer of the given leg is obtained as volume of the PDCP PDU divided with Desired Data Rate for the data radio bearer related to the given leg based on last DDDS feedback [TS 38.425]. Considering the Desired Data Rate includes time intervals when UE has data in the buffer for the given bearer but not scheduled and delay in delivering PDCP PDU from CU to DU it may provide sufficient estimation the time the PDCP PDU related to the PDCP SDU will spend in the buffer of RLC enitity. |
| ThroughputVolume | The PDCP SDU volume in kbits successfully transmitted (acknowledged by) in DL to the first and secondary RLC entity per UE per bearer and one burst (consisting of PDCP SDU 1, 2, 3 and 4 in example in Fig.2). |

Table 5.1.1.X-2

d)  Each measurement is a real value representing the throughput in kbits per second. The number of measurements is equal to one. If the optional QoS level measurement is performed, the number of measurements is equal to the number of 5QIs.

e)  The measurement name has the form 
DRB.ThroughputVolume, DRB.ThroughputTime or optionally, DRB.ThroughputVolume.Split, DRB.ThroughputTime.Split, DRB.ThroughputVolume.QOS,  DRB.ThroughputTime.QOS, where QOS identifies the target quality of service class.

f)  NRCellCU for non split bear, NRRelation for split bear.

g)  Valid for packet switched traffic

h)  5GS

i)   One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

End of modifications