3GPP TSG-SA5 Meeting #128	S5-197050

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0138 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change of PSA for UL CL or BP controlled by I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing I-SMF controlled change of PSA and UL CL or BP procedure related charging message flow for local traffic offload ETSUN scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add I-SMF controlled local UPF and local PSA (PSA2) UPF information and local traffic offload usage report charging message flow, based on TS23.502 clause 4.23.9.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support local traffic offload charging upon change of PSA and UL CL or BP controlled by I-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.x.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Should be introduced after S5-197049. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.2.x.9   Change of PSA and UL CL or BP controlled by I-SMF

This clause based on TS23.502 clause 4.23.9.3 describe a simplified change of UL-CL/BP function, e.g. new additional PDU Session Anchor (i.e. PSA2) and releases the existing additional PDU Session Anchor (i.e. PSA0), via modifying IPv6 multi-homing or UL CL rule in the same Branching Point or UL CL under controlled by the same I-SMF, with charging message flow:

Figure 5.2.2.x.8-1: Change of PSA and UL CL or BP controlled by I-SMF

1.	UE has an established PDU Session with a UPF including the PDU Session Anchor 1(controlled by SMF), and the PDU Session Anchor 0 and an I-UPF acting as UL CL or BP (controlled by I-SMF).

1.ch.	Ongoing Charging session already created between SMF and CHF, with I-SMF and the controlled UPF information available in CHF.

2.	At some point the I-SMF decides to establish a new PDU Session Anchor and releases the existing PDU Session Anchor e.g. due to UE mobility. The I-SMF selects a UPF and using N4 establishes the new PDU Session Anchor 2 of the PDU Session.

In the case of IPv6 multi-homing PDU Session, the I-SMF allocates a new IPv6 prefix corresponding to PSA2.

3.	The I-SMF invokes Nsmf_PDUSession_Update Request (Indication of Change of traffic offload, (new allocated IPv6 prefix @PSA2, DNAI(s) supported by PSA2), (Removal of IPv6 prefix @PSA0, DNAI(s) supported by PSA0)) to SMF.

The I-SMF informs the SMF that a change of traffic offload occurred.

The I-SMF provides DNAI(s) no more reachable and the DNAI now reachable to SMF. The SMF generates the corresponding N4 information based on this DNAI(s) information.

In the case of multi-homing, the new allocated IPv6 prefix @PSA2 and removal of IPv6 prefix @PSA0 are also provided to SMF.

4.	The SMF issues a SM Policy Association Modification (in TS 23.501 [2] clause 4.16.5) corresponding to the IP address allocation/release PCRT. The SMF may also send a notification to the AF, as described in in TS 23.501 [2] clause 4.3.6.3.

4.ch-a. All counts are closed in SMF, with I-SMF controlled old UPF information, and a Charging Data Request [Update] message including I-SMF information is sent to CHF, if required by immediate report category trigger. New counts and time stamps for all active service data flows are started in the SMF, with I-SMF controlled new UPF information.

4.ch-b. The CHF updates CDR for this PDU session.

4.ch-c. The CHF acknowledges by sending Charging Data Response [Update] message to the SMF.5.	The SMF responds to I-SMF with Nsmf_PDUSession_Update Response. The information includes N4 information to remove the traffic offload related to the DNAI(s) that are no more reachable and to enable the traffic offload related to the DNAI(s) that are now reachable.

6-8.	Same as step 7-9 in TS 23.501 [2] clause 4.23.9.1

9.	The I-SMF releases via N4 the PSA0 if PSA0 is not collocated with UL CL/BP, or updates the UL CL/BP to remove corresponding rules if it is collocated with UL CL/BP.

End of change