3GPP TSG-SA5 Meeting #128 	S5-197126

# Zhuhai,China, 18-22 November 2019

# Source:	Huawei

# Title:	Add use case for NCR remove blacklisting and whitelisting

# Document for:	Approval

Agenda Item:	6.4.13

1	Decision/action requested

The group is asked to discuss and approve the proposals.

2	References

[1]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks v0.0.0".

[2]             3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage 2".

3	Rationale

ANR function resides in gNB have been described in TS 38.300 [2] which requires O&M to configure three attributes of the NCR, including no remove, no HO and no Xn.  To support the ANR function management, the use case of controlling no remove attribute of the neighbor cell relation should be included.

4	Detailed proposal

1st modified section

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANR	Automatic Neighbour Relation

NCR	Neighbour Cell Relation

NG-RAN	Next Generation Radio Access Network

NCRT	Neighbour Cell Relation Table

2nd modified section

6.4	Use cases

6.4.1	Distributed SON management

6.4.1.3	ANR management

6.4.1.3.X	Prohibiting NCR removal

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to prohitbit a gNB to remove certain neighbour cell relations from the NCRT. |  |
| Actors and Roles | An authorized consumer of the MnS for D-SON management |  |
| Telecom resources | - The producer of the MnS for D-SON management - gNB |  |
| Assumptions | N/A |  |
| Pre conditions | The ANR function is activated |  |
| Begins when | This use case begins when the MnS consumer decides to prohibit the gNB to remove certain neighbour cell relations from NCRT. |  |
| Step 1 (M) | The MnS consumer configures the MnS producer to lock the status of the NCRs so that the corresponding neighbour cell relations are not allowed to be removed. |  |
| Ends when | The use case ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | The gNB is prohibited to remove the neighbour cell relations from NCRT. |  |
| Traceability | REQ-NR-ANR-FUN-X2 |  |

6.4.1.3.X	 Allowing NCR removal

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | The goal is to allow a gNB to remove neighbour cell relation from the NCRT. |  |
| Actors and Roles | An authorized consumer of the MnS for D-SON management |  |
| Telecom resources | - The producer of MnS for D-SON management - gNB |  |
| Assumptions | N/A |  |
| Pre conditions | The ANR function is activated |  |
| Begins when | This use case begins when the MnS consumer decides to allow a gNB to remove certain neighbour cell relations from the NCRT. |  |
| Step 1 (M) | The MnS consumer configures the MnS producer to unlock the status of the NCRs so that the corresponding neighbour cell relations are allowed to be removed. |  |
| Ends when | The use case ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | The gNB is allowed to remove the neighbour cell relations from NCRT. |  |
| Traceability | REQ-NR-ANR-FUN-X1 |  |

End of modified section