3GPP TSG-SA5 Meeting #128	S5-197135

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0771 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 32.298 Correction of Serving Node change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMFTrigger should refer to TS 32.255. According to Table 5.2.1.4.1, Table 5.2.1.6.1, Table 5.2.3.2.2.1 and Table 5.2.3.3.2.1, aMFchange is not one of Change of Charging conditions. In the EPS and 5GS interworking, serving node can be AMF or SGW. aMFchange is an incomplete representation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use ‘servingNodeChange’ instead of ‘aMFChange’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent with 32.255. Charging for interworking is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.5.2	CHF CDRs

This subclause contains the abstract syntax definitions that are specific to the CHF CDR types defined in this document.

.$CHFChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) chfChargingDataTypes (15) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

CauseForRecClosing,

ChargingID,

DataVolumeOctets,

Diagnostics,

EnhancedDiagnostics,

DynamicAddressFlag,

InvolvedParty,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MessageClass,

MessageReference,

MSTimeZone,

NodeAddress,

PLMN-Id,

PriorityType,

RATType,

RecordType,

ServiceSpecificInfo,

Session-Id,

SubscriberEquipmentNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

ChargingCharacteristics,

ChargingRuleBaseName,

ChChSelectionMode,

EventBasedChargingInformation,

PresenceReportingAreaInfo,

RatingGroupId,

ServiceIdentifier

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

OriginatorInfo,

RecipientInfo,

SMMessageType,

SMSResult,

SMSStatus

FROM SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version2 (1)}

;

--

--  CHF RECORDS

--

CHFRecord	::= CHOICE

--

-- Record values 200..201 are specific

--

{

chargingFunctionRecord			[200] ChargingRecord

}

ChargingRecord 	::= SET

{

recordType						[0] RecordType,

recordingNetworkFunctionID		[1] NetworkFunctionName,

subscriberIdentifier			[2] SubscriptionID OPTIONAL,

nFunctionConsumerInformation	[3] NetworkFunctionInformation,

triggers						[4] SEQUENCE OF Trigger OPTIONAL,

listOfMultipleUnitUsage			[5] SEQUENCE OF MultipleUnitUsage OPTIONAL,

recordOpeningTime				[6] TimeStamp,

duration						[7] CallDuration,

recordSequenceNumber			[8] INTEGER OPTIONAL,

causeForRecClosing				[9] CauseForRecClosing,

diagnostics						[10] Diagnostics OPTIONAL,

localRecordSequenceNumber		[11] LocalSequenceNumber OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL,

pDUSessionChargingInformation	[13] PDUSessionChargingInformation OPTIONAL,

roamingQBCInformation			[14] RoamingQBCInformation OPTIONAL,

sMSChargingInformation			[15] SMSChargingInformation OPTIONAL,

chargingSessionIdentifier		[16] ChargingSessionIdentifier OPTIONAL

}

--

-- PDU Session Charging Information

--

PDUSessionChargingInformation 	::= SET

{

pDUSessionChargingID			[0] ChargingID,

userIdentifier					[1] InvolvedParty OPTIONAL,

userEquipmentInfo				[2] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation			[3] OCTET STRING OPTIONAL,

userRoamerInOut					[4] RoamerInOut OPTIONAL,

presenceReportingAreaInfo		[5]	PresenceReportingAreaInfo OPTIONAL,

pDUSessionId					[6] PDUSessionId,

networkSliceInstanceID			[7] NetworkSliceInstanceID OPTIONAL,

pDUType							[8] PDUSessionType OPTIONAL,

sSCMode							[9] SSCMode OPTIONAL,

sUPIPLMNIdentifier				[10] PLMN-Id OPTIONAL,

servingNetworkFunctionID		[11] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

rATType							[12] RATType OPTIONAL,

dataNetworkNameIdentifier		[13] DataNetworkNameIdentifier OPTIONAL,

pDUAddress						[14] PDUAddress OPTIONAL,

authorizedQoSInformation		[15] AuthorizedQoSInformation OPTIONAL,

uETimeZone 						[16] MSTimeZone OPTIONAL,

pDUSessionstartTime				[17] TimeStamp OPTIONAL,

pDUSessionstopTime				[18] TimeStamp OPTIONAL,

diagnostics						[19] Diagnostics OPTIONAL,

chargingCharacteristics			[20] ChargingCharacteristics,

chChSelectionMode				[21] ChChSelectionMode OPTIONAL,

threeGPPPSDataOffStatus			[22] ThreeGPPPSDataOffStatus OPTIONAL,

rANSecondaryRATUsageReport 		[23] SEQUENCE OF NGRANSecondaryRATUsageReport OPTIONAL,

subscribedQoSInformation 		[24] SubscribedQoSInformation OPTIONAL,

authorizedSessionAMBR 			[25] SessionAMBR OPTIONAL,

subscribedSessionAMBR 			[26] SessionAMBR OPTIONAL,

servingCNPLMNID					[27] PLMN-Id OPTIONAL,

sUPIunauthenticatedFlag 		[28] NULL OPTIONAL,

homeProvidedChargingID			[29] ChargingID OPTIONAL,

dnnSelectionMode				[30] DNNSelectionMode OPTIONAL

}

--

-- Roaming QBC Information

--

RoamingQBCInformation 	::= SET

{

multipleQFIcontainer			[0] SEQUENCE OF MultipleQFIContainer OPTIONAL,

uPFID							[1] NetworkFunctionName OPTIONAL,

roamingChargingProfile			[2] RoamingChargingProfile OPTIONAL

}

--

-- SMS Charging Information

--

SMSChargingInformation	::= SET

{

sMSNodeAddress				[0] AddressString,

originatorInfo				[1] OriginatorInfo OPTIONAL,

recipientInfos				[2] SEQUENCE OF RecipientInfo OPTIONAL,

userEquipmentInfo			[3] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation		[4] OCTET STRING OPTIONAL,

uETimeZone 					[5] MSTimeZone OPTIONAL,

rATType						[6] RATType OPTIONAL,

sMSCAddress					[7] AddressString OPTIONAL,

eventtimestamp				[8] TimeStamp,

-- 9 to 19 is for future use

sMDataCodingScheme			[20] INTEGER OPTIONAL,

sMMessageType				[21] SMMessageType OPTIONAL,

sMReplyPathRequested		[22] SMReplyPathRequested OPTIONAL,

sMUserDataHeader			[23] OCTET STRING OPTIONAL,

sMSStatus					[24] SMSStatus OPTIONAL,

sMDischargeTime				[25] TimeStamp OPTIONAL,

sMTotalNumber 				[26] INTEGER OPTIONAL,

sMServiceType				[27] SMServiceType OPTIONAL,

sMSequenceNumber 			[28] INTEGER OPTIONAL,

sMSResult					[29] SMSResult OPTIONAL,

submissionTime				[30] TimeStamp OPTIONAL,

sMPriority					[31] PriorityType OPTIONAL,

messageReference			[32] MessageReference,

messageSize					[33] INTEGER OPTIONAL,

messageClass				[34] MessageClass OPTIONAL,

sMdeliveryReportRequested	[35] SMdeliveryReportRequested OPTIONAL

}

--

-- PDU Container Information

--

PDUContainerInformation 		::= SEQUENCE

{

chargingRuleBaseName				[0] ChargingRuleBaseName OPTIONAL,

afChargingIdentifier				[1] ChargingId OPTIONAL,

timeOfFirstUsage					[2] TimeStamp OPTIONAL,

timeOfLastUsage						[3] TimeStamp OPTIONAL,

qoSInformation						[4] FiveGQoSInformation OPTIONAL,

userLocationInformation				[5] OCTET STRING OPTIONAL,

presenceReportingAreaInfo			[6] PresenceReportingAreaInfo OPTIONAL,

rATType								[7] RATType OPTIONAL,

sponsorIdentity						[8] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[9] OCTET STRING OPTIONAL,

servingNetworkFunctionID			[10] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

uETimeZone 							[11] MSTimeZone OPTIONAL,

threeGPPPSDataOffStatus				[12] ThreeGPPPSDataOffStatus OPTIONAL

}

--

-- QFI Container Information

--

MultipleQFIContainer 		::= SEQUENCE

{

qosFlowId							[0] QoSFlowId OPTIONAL,

triggers							[1] SEQUENCE OF Trigger,

triggerTimeStamp					[2] TimeStamp OPTIONAL,

dataTotalVolume						[3] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[4] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[5] DataVolumeOctets OPTIONAL,

localSequenceNumber					[6] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[8] TimeStamp OPTIONAL,

timeOfLastUsage						[9] TimeStamp OPTIONAL,

qoSInformation						[10] FiveGQoSInformation OPTIONAL,

userLocationInformation				[11] OCTET STRING OPTIONAL,

uETimeZone	 						[12] MSTimeZone OPTIONAL,

presenceReportingAreaInfo			[13] PresenceReportingAreaInfo OPTIONAL,

rATType								[14] RATType OPTIONAL,

reportTime							[15] TimeStamp,

servingNetworkFunctionID			[16] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

threeGPPPSDataOffStatus				[17] ThreeGPPPSDataOffStatus OPTIONAL,

threeGPPChargingID					[18] ChargingID OPTIONAL,

diagnostics					[19] Diagnostics OPTIONAL,

extensionDiagnostics				[20] EnhancedDiagnostics OPTIONAL

}

--

--  CHF CHARGING TYPES

--

FiveGQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi					[1] INTEGER,

aRP						[2] AllocationRetentionPriority,

qoSNotificationControl	[3] BOOLEAN OPTIONAL,

reflectiveQos			[4] BOOLEAN OPTIONAL,

maxbitrateUL			[5] Bitrate OPTIONAL,

maxbitrateDL			[6] Bitrate OPTIONAL,

guaranteedbitrateUL		[7] Bitrate OPTIONAL,

guaranteedbitrateDL		[8] Bitrate OPTIONAL,

priorityLevel 			[9] INTEGER OPTIONAL,

averWindow				[10] INTEGER OPTIONAL,

maxDataBurstVol			[11] INTEGER OPTIONAL,

maxPacketLossRateDL 	[12] INTEGER OPTIONAL,

maxPacketLossRateUL 	[13] INTEGER OPTIONAL

}

AllocationRetentionPriority	::= SEQUENCE

{

priorityLevel 			[1] INTEGER,

preemptionCapability	[2] PreemptionCapability,

preemptionVulnerability	[3] PreemptionVulnerability

}

AMFID	::= OCTET STRING (SIZE(6))

-- See subclause 2.10.1 of 3GPP TS 23.003 [7] for encoding.

ChargingSessionIdentifier	::= OCTET STRING

-- See 3GPP TS 32.290 [57] for details.

AuthorizedQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi				[1] INTEGER,

aRP					[2] AllocationRetentionPriority,

priorityLevel 		[3] INTEGER OPTIONAL,

averWindow			[4] INTEGER OPTIONAL,

maxDataBurstVol		[5] INTEGER OPTIONAL

}

Bitrate	::= OCTET STRING

--

--  See 3GPP TS 29.571 [249] Bitrate data type.

--

DataNetworkNameIdentifier	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of DNN in dot representation.

-- For example, if the complete DNN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- The Identifier is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

MultipleUnitUsage 		::= SEQUENCE

{

ratingGroup							[0] RatingGroupId,

usedUnitContainers					[1] SEQUENCE OF UsedUnitContainer OPTIONAL,

uPFID								[2] NetworkFunctionName OPTIONAL

}

NGRANSecondaryRATType	::= OCTET STRING

--

-- "NR" or "EUTRA"

--

NGRANSecondaryRATUsageReport	::= SEQUENCE

{

nGRANSecondaryRATType			[0] NGRANSecondaryRATType OPTIONAL,

qosFlowsUsageReports			[1] SEQUENCE OF QosFlowsUsageReport OPTIONAL

}

PDUSessionId 		::= INTEGER (0..255)

--

-- See 3GPP TS 29.571 [249] for details

--

QosFlowsUsageReport		::= SEQUENCE

{

qosFlowId						[0] QoSFlowId OPTIONAL,

startTime						[1] TimeStamp,

endTime							[2] TimeStamp,

dataVolumeDownlink				[3] DataVolumeOctets,

dataVolumeUplink				[4] DataVolumeOctets

}

NetworkFunctionInformation	::= SEQUENCE

{

networkFunctionality				[0] NetworkFunctionality,

networkFunctionName					[1] NetworkFunctionName OPTIONAL,

networkFunctionIPv4Address			[2] IPAddress OPTIONAL,

networkFunctionPLMNIdentifier		[3] PLMN-Id OPTIONAL,

networkFunctionIPv6Address			[4] IPAddress OPTIONAL,

networkFunctionFQDN					[5] NodeAddress OPTIONAL

}

NetworkFunctionName	::= IA5String (SIZE(1..20))

-- Shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [410]

NetworkFunctionality	::= ENUMERATED

{

cHF			(0), 	-- this value is not used

sMF			(1),

aMF			(2),

sMSF		(3),

sGW		(4)

-- SGW is only applicable for interworking with EPC scenario when UE is connected to P-GW+SMF via EPC

}

NetworkSliceInstanceID	::= SEQUENCE

-- See S-NSSAI subclause 28.4.2 of TS 23.003 [200] for encoding.

{

sST			[0] SliceServiceType,

sD 			[1] SliceDifferentiator OPTIONAL

}

PartialRecordMethod	::= ENUMERATED

{

default			(0),

individual		(1)

}

PDUAddress 	::= SEQUENCE

{

pDUIPv4Address				[0] IPAddress OPTIONAL,

pDUIPv6AddresswithPrefix	[1] IPAddress OPTIONAL,

iPV4dynamicAddressFlag		[2] DynamicAddressFlag OPTIONAL,

iPV6dynamicPrefixFlag		[3] DynamicAddressFlag OPTIONAL

}

PDUSessionType		::= ENUMERATED

{

iPv4v6			(0),

iPv4			(1),

iPv6			(2),

unstructured	(3),

ethernet		(4)

}

-- See 3GPP TS 29.571 [249] for details.

PreemptionCapability		::= ENUMERATED

{

nOT-PREEMPT			(0),

mAY-PREEMPT			(1)

}

PreemptionVulnerability		::= ENUMERATED

{

nOT-PREEMPTABLE		(0),

pREEMPTABLE			(1)

}

QoSFlowId		::= INTEGER

RatingIndicator	::= BOOLEAN

-- Included if the units have been rated.

RoamingChargingProfile 		::= SEQUENCE

{

roamingTriggers			[0] SEQUENCE OF RoamingTrigger OPTIONAL,

partialRecordMethod		[1] PartialRecordMethod OPTIONAL

}

RoamerInOut	::= ENUMERATED

{

roamerInBound		(0),

roamerOutBound		(1)

}

RoamingTrigger 		::= SEQUENCE

{

trigger					[0] SMFTrigger OPTIONAL,

triggerCategory			[1] TriggerCategory	 OPTIONAL,

timeLimit				[2] CallDuration OPTIONAL,

volumeLimit				[3] DataVolumeOctets OPTIONAL,

maxNbChargingConditions	[4] INTEGER OPTIONAL

}

ServingNetworkFunctionID	::= SEQUENCE

{

servingNetworkFunctionInformation	[0] NetworkFunctionInformation,

aMFIdentifier						[1] AMFID OPTIONAL

}

SessionAMBR	::= SEQUENCE

{

ambrUL				[1] Bitrate,

ambrDL				[2] Bitrate

}

SliceServiceType ::= INTEGER (0..255)

--

-- See subclause 28.4.2 TS 23.003 [200]

--

SliceDifferentiator		::= OCTET STRING (SIZE(3))

--

-- See subclause 28.4.2 TS 23.003 [200]

--

SSCMode	::= INTEGER

{

sSCMode1				(1),

sSCMode2				(2),

sSCMode3				(3)

}

-- See 3GPP TS 29.501 [248] for details.

SMdeliveryReportRequested ::= ENUMERATED

{

yes		(0),

no		(1)

}

SMFTrigger				::= INTEGER

{

startOfPDUSession							(1),

atartOfServiceDataFlowNoSession			(2),

-- Change of Charging conditions

qoSChange									(100),

userLocationChange							(101),

servingNodeChange									(102),

presenceReportingAreaChange				(103),

threeGPPPSDataOffStatusChange				(104),

tariffTimeChange							(105),

uETimeZoneChange							(106),

pLMNChange									(107),

rATTypeChange								(108),

sessionAMBRChange							(109),

additionOfUPF								(110),

removalOfUPF 								(111),

-- Limit per PDU session

pDUSessionExpiryDataTimeLimit				(200),

pDUSessionExpiryDataVolumeLimit				(201),

pDUSessionExpiryDataEventLimit				(202),

pDUSessionExpiryChargingConditionChanges	(203),

-- Limit per Rating group

ratingGroupDataTimeLimit					(300),

ratingGroupDataVolumeLimit					(301),

ratingGroupDataEventLimit					(302),

-- Quota management

timeThresholdReached						(400),

volumeThresholdReached						(401),

unitThresholdReached						(402),

timeQuotaExhausted							(403),

volumeQuotaExhausted						(404),

unitQuotaExhausted							(405),

expiryOfQuotaValidityTime					(406),

reAuthorizationRequest						(407),

startOfServiceDataFlowNoValidQuota			(408),

-- Others

terminationOfServiceDataFlow				(500),

managementIntervention						(501),

unusedQuotaTimerExpiry						(502),

endOfPDUSession								(503),

cHFResponseWithSessionTermination			(504),

cHFAbortRequest								(505),

-- Limit per QoS Flow

qoSFlowExpiryDataTimeLimit					(600),

qoSFlowExpiryDataVolumeLimit				(601),

-- interworking with EPC

eCGIChange									(700),

tAIChange									(701),

handoverCancel								(702),

handoverStart								(703),

handoverComplete							(704)

}

-- See TS 32.255 [15] for details.

SMServiceType 	::= INTEGER

{

-- 0 to 10 VAS4SMS Short Message, see TS TS 22.142 [x] for details

contentProcessing					(0),

forwarding							(1),

forwardingMultipleSubscriptions		(2),

filtering 							(3),

receipt								(4),

networkStorage						(5),

toMultipleDestinations				(6),

virtualPrivateNetwork				(7),

autoreply							(8),

personalSignature					(9),

deferredDelivery					(10)

-- 11 to 99	Reserved for 3GPP defined SM services

-- 100 to 199 Vendor specific SM services

}

SMReplyPathRequested	::= ENUMERATED

{

noReplyPathSet 			(0),

replyPathSet			(1)

}

SubscribedQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi				[1] INTEGER,

aRP					[2] AllocationRetentionPriority OPTIONAL,

priorityLevel 		[3] INTEGER OPTIONAL

}

Trigger	::= CHOICE

{

sMFTrigger		[0] SMFTrigger

}

TriggerCategory	::= ENUMERATED

{

immediateReport		(0),

deferredReport		(1)

}

UsedUnitContainer 		::= SEQUENCE

{

serviceIdentifier					[0] ServiceIdentifier OPTIONAL,

time								[1] CallDuration OPTIONAL,

triggers							[2] SEQUENCE OF Trigger,

triggerTimeStamp					[3] TimeStamp OPTIONAL,

dataTotalVolume						[4] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[5] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[6] DataVolumeOctets OPTIONAL,

serviceSpecificUnits				[7] INTEGER OPTIONAL,

eventTimeStamp						[8] TimeStamp OPTIONAL,

localSequenceNumber					[9] LocalSequenceNumber OPTIONAL,

ratingIndicator						[10] RatingIndicator OPTIONAL,

pDUContainerInformation				[11] PDUContainerInformation OPTIONAL,

quotaManagementIndicator			[12] BOOLEAN OPTIONAL

}

DnnSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.502 [250] for more information

--

{

uEorNetworkProvidedSubscriptionVerified				(0),

uEProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

.#END

End of change