3GPP TSG-SA5 Meeting # 128	S5-197152

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0203 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add attribute to describe different types of isolation for network slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2019-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Isolation is one of the key expectations of network slicing. A network slice instance may be fully or partly, logically and/or physically, isolated from another network slice instance. Isolation Level Support is one of GST attributes which is defined in GSMA NG.116 for Network Slicing requirement.It is important to define some attributes for network slice isolation level, which would protect sensitive data inside a relative network slice |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define isolationLevel attribute to describe different types of isolation for a network slice in ServiceProfile class and SliceProfile class |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without specifying different types of isolation, the risk of a network slice sercurity will be more higher.Weak network slice isolation and connection may compromise the entire 5G security,sensitive data managed inside a network slice, could be exposed to applications running in other network slices services, through side channel attacks |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2, 6.3.3.3, 6.3.4.2, 6.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of Change *****

6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | M | T | T | F | T |
| isolationLevel | O | T | T | F | T |
| physicalIsolation | CM | T | T | F | T |
| logicalIsolation | CM | T | T | F | T |

***** Next Change *****

6.3.3.3	Attribute constraints

.

| Name | Definition |
| --- | --- |
| physicalIsolation | Condition: This parameter must be present when “IsolationLevel” is set to 1. |
| logicalIsolation | Condition: This parameter must be present when “IsolationLevel” is set to 2. |

***** Next Change *****

6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| isolationLevel | O | T | T | F | T |
| physicalIsolation | CM | T | T | F | T |
| logicalIsolation | CM | T | T | F | T |

***** Next Change *****

6.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| physicalIsolation | Condition: This parameter must be present when “IsolationLevel” is set to 1. |
| logicalIsolation | Condition: This parameter must be present when “IsolationLevel” is set to 2. |

***** End of Changes *****